----------------------------------------------------------------------------

| * SETPRN *  A printer utility for the IBM PC & PC-XT.  V. T. Bly  12/11/83 |

----------------------------------------------------------------------------

SUMMARY

-------

SETPRN provides an easy method of setting printer parameters from the

DOS command level or a batch file.  SETPRN is similar to the BASIC LPRINT

command, except that control codes are enclosed in square brackets ("[]")

instead of the "CHR$()" used in BASIC.  Also, text strings should not be

enclosed in quotes.  Like LPRINT, the normal carriage return/line feed can

be suppressed by a trailing semicolon (";") or comma (",").  A trailing

semicolon will leave the print head at the end of the just printed text,

while a comma will advance the print head to the next tab stop.

EXAMPLES

--------

The following examples assume that the file SETPRN.COM is on the

default disk drive.  Note that "<enter>" means that you should press

the enter/return key after typing the command.

SETPRN Testing 0123456789 <enter>

This command will simply print the string "Testing 0123456789" on

the printer and drop down to the next line.

SETPRN Merry Christmas; <enter>

This command will print the string "Merry Christmas" on the printer,

but NOT drop down to the next line.  Notice the trailing ";" which

suppresses the carriage return/line feed.  On some printers, "Merry

Christmas" may not actually be printed until an output is sent to

the printer which contains a carriage return.

SETPRN [27]E; <enter>

This command will send Escape E to the printer.  This sequence turns

on emphasized printing on the IBM and Epson dot matrix printers.

SETPRN [27]!Boldface Title[27]" <enter>

This command will print the string "Boldface Title" in boldface on

the C. Itoh and NEC dot matrix printers and drop down to the next

line.  With these printers, Escape ! starts boldface printing and

Escape " stops boldface printing.  Note that the quote symbol (") may

be included in the SETPRN command line, just like any other character.

NOTES

-----

Any printable character may be included in the SETPRN command line

except the left square bracket ("["), which is used as the prefix to a

control code.  This character may be included by enclosing its ASCII code

(91) between square brackets; that is "[91]".

SETPRN should work correctly on any IBM compatible computer operating

under MS-DOS.  It should work with any version of MS-DOS, including 1.0,

1.05, 1.1, 2.0, and 2.1.

Please send your comments to:  Vincent Bly

Post Office Box 409

Ft. Belvoir, VA  22060####################################################################