SMART R8 CONTROL

version 1.02�

Copyright (c) 1992,93 FineWare

This document serves as a preliminary 'manual' to the beta release of

Smart R8 Control v1.02.  The system requirements for operation of this

program are the Drake R8 Communications Receiver and an MS-DOS based

Computer running at least MS-DOS version 3.3, with a minimum of 450k

free base RAM and one available serial port.  Operation of Smart R8

Control v1.02� under the OS/2 operating system (even in DOS Compati-

bility mode) should be avoided.  This text serves as a tutorial to

show what the software is intended to do, how to set it up, and fi-

nally how to run it.  The distribution file SMCON102.ZIP should con-

tain the following:

SC.EXE         - The Smart R8 Control v1.02� Program

SC.DOC         - This Text

SWBCGIDE.DAT   - Shortwave frequency database file

STATIONS.DAT   - Shortwave stations database file

DXSHOWS.TXT    - Selected shortwave programming

*    SCR.EXE        - Smart R8 Control Printing Utility v1.02�

*    SCT.EXE        - Smart R8 Control Database Utility v1.02�

*    SCP.EXE        - Smart R8 Control Propagation Prediction

Utility v1.02�

*    SCUTIL.DOC     - Documentation for SCR, SCT and SCP

Note:  The files SCR.EXE, SCT.EXE, SCP.EXE and SCUTIL.DOC are not con-

tained in the shareware distribution file SMCON102.ZIP and are re-

served for registered copies only.

Although fundamental tuning techniques are covered herein, this docu-

mentation assumes that the reader has some working knowledge of the

Drake R8 Communications Receiver.  However, it is suggested that the

user print out this text (SC.DOC) for future reference in the opera-

tion of Smart R8 Control and its companion utility programs.

WARRANTY, LICENSING AND REGISTRATION.

This and all of the other files contained in the distribution file SM-

CON102.ZIP are provided as shareware to be used and disseminated in an

unmodified form.  The author requests that no fee be charged by any

agent other than FineWare (Mark J. Fine), for the enjoyment or use of

this software.  Neither the author nor his associates are responsible

for any results of using this software or hardware modification recom-

mendations included in this document on any computer system or radio

receiver, whether used or performed either properly or improperly.

Smart R8 Control  version 1.02�                                   Page 1 of 17

The user hereby agrees that he is using this software/modifying his

hardware at his own risk.

This and all of the other files contained in the file SMCON102.ZIP are

provided as is, with no warranties, express or implied, including but

not limited to the implied warranties of fitness for a particular pur-

pose, and all such warranties are expressly and specifically dis-

claimed.  In no event should the author or any of his associates be

responsible for any indirect, special, incidental or consequential

damages or lost profits to you or any other person or entity regard-

less of the legal theory, even if the author or any of his associates

have been advised of the possibility of such damage.

No person or entity is authorized to expand or alter either these war-

ranties or this agreement; any such representation will not bind the

author or his associates.

The user is licensed to operate the software free of charge, for the

purposes of inspection, for a period of sixty (60) days.  After the 60

day period expires, the user is expected to pay the nominal registra-

tion charge of twenty-five United States dollars (US$25).  Checks or

money orders may be made payable to MARK J. FINE and can be sent to

the following address:

MARK J. FINE

c/o FineWare

11252 Cardinal Drive

Remington, Virginia 22734-9684

Support of this shareware product ensures its continued development

and dissemination.  As a bonus for registering Smart R8 Control, the

user will receive future updates to the software, free of charge.  As

an extra added bonus, the registering party will receive three free

companion utilities as follows:

SCT.EXE - Allows the user to update and compile the shortwave database

with user data and frequency schedule changes.

SCR.EXE - Allows the user to print the full shortwave database in a

neatly collated two-column newspaper style format.

SCP.EXE - Allows the user to predict HF propagation conditions to a

particular station or area of the world.

SCUTIL.DOC - Documentation and special instructions for operating the

above three companion utilities.

INTRODUCTION.

Because most shortwave broadcast stations change frequencies during

the day, it's often frustrating have to double check mounds of refer-

Smart R8 Control  version 1.02�                                   Page 2 of 17

ence material while trying to listen at the same time.  For example,

consider the following:

Scenario 1:  Upon settling in for a relaxing session of twirling the

dial, an unfamiliar station pops out of the ether.  While waiting for

the station to ID, you fumble through pages of reference material try-

ing to get a heads-up on what the station might be.

Scenario 2:  You have an urge to listen to a particular station such

as the Radio Sweden, Radio Netherlands, Radio Moscow, or Radio Aus-

tralia (if the conditions are right).  Again, you fumble through your

references to find out what frequency to tune to.  Not only have you

found the station's programming schedule, but the station won't be on

for at least another hour.

For these and other cases, the Smart R8 Control program is designed to

both simplify and enhance the shortwave listening hobby.  Smart R8

Control integrates a database of frequency information with the power

of remotely controlling the shortwave receiver.  Smart R8 Control emu-

lates most of the R8's front panel controls, adding the capabilities

of tuning by user selected increments (i.e., 5 kHz) and controlling an

attached tape recorder; while adding the name of the station you're

listening to and a dual 24-hour clock (local/UTC) to the continuous

display.

Aside from displaying the station you're tuned to, Smart R8 Control

allows smart queries to the integrated frequency database.  Using an

internally designed windowing environment, Smart R8 Control allows the

user to choose from a list of stations broadcasting at that time.

Once a station is selected, a separate window is created containing

that station's list of current frequencies and target areas.  The user

can now automatically tune the receiver by selecting a frequency from

the list.  A marking feature is included, to allow the user to high-

light particularly reliable frequencies in the list (i.e., the 5975,

7325 and 9915 kHz evening frequencies for the BBC to North America).

The frequency database can also be queried to display a station's full

frequency schedule, when selected programs are on that particular day,

as well as a query to find what other stations use the frequency the

receiver is tuned to.  Other ancillary features include a logging fa-

cility and a 'reminder' display of the shortwave frequency bands.

INSTALLATION.

Unarchive the entire distribution file SMCON102.ZIP into a single di-

rectory.  This is important, because the main program, SC.EXE, expects

to find all of the individual files and programs it creates and uses

in a single disk directory.  If this is the initial use of Smart R8

Control, please pay close attention to the following 'SETUP' and

'RUNNING' sections.

Smart R8 Control  version 1.02�                                   Page 3 of 17

If this version is to be used as an upgrade from a previous version,

Smart R8 Control v1.02� should be able to use all of the original

setup files.  In this case, no additional installation procedures are

required.  The program is ready to run as before.

SETUP.

First, it is suggested that most terminate but stay resident programs

(TSRs) are removed from the environment and that the following com-

mands are included in the user's CONFIG.SYS file:

DEVICE=[drive:][path]ANSI.SYS

FILES=25

BUFFERS=20

It is necessary (but not essential) that the computer's real time

clock be accurately set.  If no battery back up clock is used in the

computer, the DOS TIME and DATE commands should be used prior to run-

ning Smart R8 Control.  The importance of this will be detailed later.

It is essential, though, that the user know the UTC-to-local time off-

set, which of the computer's serial ports the receiver is connected

to, and which of the receiver's two antenna ports is connected to the

shortwave antenna.  This information is only required the first time

the program is run.

RUNNING.

Upon first starting Smart R8 Control, the user is prompted to input

the number of hours required to convert to Universal Coordinated Time

(UTC).  The main window of Smart R8 Control contains a real-time 24-

hour UTC/local time clock both for the user's information, and for use

in some of the program's other functions.  The user should respond

with the number of hours required to be added to his local time to

achieve UTC.  For example, the user should respond to this prompt with

a '5' for winter time on the East Coast.

The second piece of information required by Smart R8 Control is the

COMM port number to which the receiver is connected.  The user should

respond with a '1' for COM1:, or up to '4' for COM4:.  The program can

only recognize ports 1 through 4.

Lastly, the program will prompt for the preferred shortwave antenna

port.  An entry of '1' will select the receiver's 50 Ohm (SO-259) con-

nector, and a '2' will select the receiver's 300 Ohm (spring-clip)

connector.  This option serves as a default for those users who have

installed the optional VHF converter, and preempts the receiver from

automatically switching to port 1 when returning to HF, if this is not

desired.

Smart R8 Control  version 1.02�                                   Page 4 of 17

These three pieces of information will then be stored in an initial-

ization file, in the default directory, named 'SMARTCON.DAT'.  Upon

future runs of the program, this file will be automatically read at

start-up.  Since these options can change, the user can enter these

options independently, upon further runs of the program, at the com-

mand line:

SC G5               (Changes the UTC offset to '5')

SC R1               (Sets the receiver to COM1:)

SC A2               (Sets the preferred antenna to port '2')

Because the serial interface requires a hardware handshake, it is im-

perative that the receiver be hooked up to the computer before the

program is run.  If Smart R8 Control is to be run for database access

only, or if the receiver is unavailable for any reason, the program

can be started in 'Demo Mode'.  This will start the program without

the hardware handshake engaged, and is invoked by using a 'D' switch

on the command line:

SC D

After all initial prompts are answered, Smart R8 Control will read the

frequency database and go to it's main screen.  The main screen's pri-

mary component is the radio's faceplate window, located in the upper

left hand corner.  The layout of the faceplate is similar to that of

the R8, except that it also displays the current tuning step size, the

local and UTC times, a carrier present indicator, and space for up to

four co-channel station names.  A typical faceplate will look similar

to the following:

*D----------[ Smart R8 Control v1.02� ]-----------|

| ERROR     7435.00 kHz  MEM  19 STEP(kHz) LOCAL  |

| Carrier  NB W  Notch  Scan  LC    5.00   20:33t |

| VFO  ANT   RF   AGC   BW  MODE  SYNCHRO   UTC   |

|  B    2   Off  Slow  6.0   AM      *     01:33w |

| 0100-0200 smtwhfa WWCR Nashville TN          na |

|                                                 |

|                                                 |

|                                                 |

|------------------- F1 - Help -------------------|

Upper Left Corner:

Space is reserved for a database status indicator.  This space will

display a '*' only if the database has changed.

Space has been reserved for a Demo Mode indicator.  This space will

display a 'D' only if the program is in Demo Mode.

Line 1:

Space is reserved for a status indicator.  The space will display

'ERROR' for bad entries, or if a command was unsuccessfully sent

to the receiver.  A 'WAIT' indication will display during

database or tuning operations.  Additional keystrokes may be

buffered while either indicator is showing.

Smart R8 Control  version 1.02�                                   Page 5 of 17

The frequency is displayed in kHz for frequencies under 30 MHz, or in

MHz for tuning with the receiver's optional VHF Converter.  If

the receiver is off, this space will display 'O F F'.

Space is reserved for the memory mode indicator 'MEM' if memory mode

is active.  The number of the memory channel currently selected

will be continuously displayed.

Line 2:

Space is reserved for a carrier indicator.  If a signal is present and

is at a level above the squelch threshold, 'Carrier' will dis-

play, as shown.

Space is reserved for the status of the noise blanker.  If the noise

blanker is engaged, the indicator will show 'NB N' or 'NB W' for

the narrow or wide settings, respectively.

Space is reserved for the status of the notch filter.  If the notch

filter is engaged, the indicator will display 'Notch', as shown.

Space is reserved for a scanning indicator.  If the receiver is in

scanning mode the indicator will display 'Scan', as shown

The scanning method is shown as letters.  The first letter refers to

'M' for Memory, 'L' for List, or 'A' for A-B modes.  The second

letter refers to 'S' for Seek, 'T' for Time, or 'C' for Carrier

Detect types.  If these values are not set, this space will dis-

play '??'.

The tuning step size is displayed in kHz.

The local time is displayed in 24-hour format.  Next to the time, the

day of the week is displayed as a single letter.  'h' is used for

Thursday, and 'a' is used for Saturday.  All other letters per-

tain to the first letter of that day.

Line 4:

Line 4 contains indicators for the VFO, antenna port, RF amplifier,

AGC, bandwidth, and choice of demodulation.

Space is reserved for the status of the synchronous detector.  If the

synchronous detector is engaged, the indicator will display a

'*', as shown.

Universal Coordinated Time is displayed in 24-hour format.  Next to

the time, the UTC day of the week is displayed as a single let-

ter.  The same convention is used for that of the local time

clock.

Lines 5-8:

The bottom of the faceplate will display the name of the station cur-

rently tuned to.  In the event that a few stations may be occupy-

ing this frequency, up to four stations will be listed.  If the

frequency has been previously marked for a particular station,

that station's name will be highlighted.  The format of the

shortwave database, hence the information displayed, is based on

the Shortwave Guide section of Grove Enterprises' Monitoring

Times magazine.  For those unfamiliar with this format, it is de-

scribed as follows:

The first field in each line will contain the time period this fre-

quency will be occupied by the station.  This period will be dis-

played to reflect a portion within the one-hour block.

Smart R8 Control  version 1.02�                                   Page 6 of 17

The next field contains the days of the week for which this broadcast

would occur.  This field uses the same convention described for

the local and UTC clocks.  For simplicity, if the broadcast is

made daily, this space will be empty, otherwise, the particular

day will display in the designated space, as shown above.

The next two fields will display the name or location of the station

and target area, if known.  Typical target areas are 'do' for do-

mestic service, 'om' for omnidirectional pattern, 'na' for North

America, 'am' for the Americas, 'ca' for Central America, 'sa'

for South America, 'eu' for Europe, 'af' for Africa, 'as' for

Asia, 'pa' for the Pacific, 'au' for Australia, or 'va' for some

combination of target areas.  Some feeders may display a 'uf' or

'lf' in this area to designate that the transmission is in upper

or lower sideband.

If the station is a mediumwave station (AM Broadcast Band), the name

and target area field is expanded to show the station's output

power in kW and the approximate times of operation, if known.

The codes used for this are 'an' for all night, 'ar' for all

night-reduced power at night, 'nh' for normal hours, 'nr' for

normal hours-reduced power at night, 'dr' for day time only-re-

duced power during dusk/dawn, and 'da' for daytime operation

only.

RADIO CONTROLS.

Esc - Power.

Arrows - Stepped Tuning.

Numbers - Direct Access Tuning.

Alt-Z - Step Increments.

The receiver can be turned on and off with the keyboard 'Esc' key.

The receiver can be tuned in a stepping fashion with the left and

right cursor keys.  If desired, the receiver can be direct-access

tuned by typing in the frequency numerically, and hitting return.  The

stepping increment defaults to 5 kHz upon program start-up.  This can

be changed by pressing Alt-Z.  This will prompt for the increment to

change.  The increment is entered in kHz and is valid for a range of

.01 to 99999.99 kHz.

When using the direct-access tuning, typing 15, 15.000 or 15000 will

yield the same result: 15 MHz.  This only works between the frequency

range of .176 through 55 MHz.  Frequencies below .176 MHz and above

108 MHz must be entered in MHz (i.e., .153 for 153 kHz or 147 for 147

MHz).  Since the receiver only tunes between .100 - 30,35 - 55, and

108 - 175 MHz, any other frequency will yield an error message.  If

the optional VHF Converter is not installed, tuning in the latter two

bands will yield an error message.

All of these key positions were specifically chosen because of the

layout of the 101 style keyboard.  On the 101, the 'Esc' key is in the

upper left hand corner of the keyboard, which keeps it nicely out of

the way.  Also on the 101 keyboard, are separate cursor and numeric

Smart R8 Control  version 1.02�                                   Page 7 of 17

keypads.  If the 'NumLock' is depressed, it is extremely easy to punch

in a direct-access frequency or incrementally tune.

Other R8 features can be accessed as follows:

Alt-V - VFO.

The VFO can be toggled from A to B and vice versa by pressing Alt-V.

Alt-A - ANT.

The antenna port can be changed by pressing Alt-A and entering '1' for

port 1, '2' for port 2, or 'Conv' for the VHF Converter.

Alt-R - RF.

The RF Amplifier can be changed by pressing Alt-R and entering 'Att'

for the attenuator, 'Pre' to engage the preamplifier, or 'Off' for no

RF enhancement.

Alt-G - AGC.

The AGC setting can be changed by pressing Alt-G and entering

'Off','Slow', or 'Fast'.

Alt-B - BW.

The IF bandwidth can be changed by pressing Alt-B and entering '0.5',

'1.8', '2.3', '4.0' or '6.0' for the respective filter setting in kHz.

Alt-M - MODE.

The demodulation mode can be switched by pressing Alt-M and entering

'AM', 'FM', 'CW', 'RTTY', 'LSB' or 'USB'.

Alt-K - NB.

The noise blanker can be switched by pressing Alt-K and entering

'Narr', 'Wide' or 'Off'.

Alt-N - NOTCH.

The notch filter can be toggled on and off by pressing Alt-N.

Alt-S - SYNCHRO.

The synchronous detector can be toggled on and off by pressing Alt-S.

Note:  For those above controls that require further user entry, the

selection can be abbreviated by using the first character of the re-

sponse (i.e., the user can type 'R' instead of 'RTTY' when responding

to the Mode control, Alt-M).  Unfortunately, there are no provisions

to remotely control receiver's tone, notch, squelch, passband offset,

volume or RF control knobs; nor the internal clock or timer settings.

There are provisions to remotely control the tuning knob step display,

but because of the features provided by this program, the point is

moot.

Alt-E - Memory Mode Operation.

Alt-Y - Jump To Memory Location.

Alt-I - Insert Memory Location.

Memory mode tuning can be toggled on and off by pressing Alt-E.  When

in memory mode, the 'MEM' indicator will light and the receiver will

Smart R8 Control  version 1.02�                                   Page 8 of 17

tune to the memory number continuously displayed on the faceplate.

Pressing Alt-E again will download the memory settings to the current

VFO, which is analogous to the receiver's M-V key.  In memory mode,

using the right and left cursor keys will step up and down the memory

slots, respectively.  An alternate method for accessing a memory num-

ber is to press Alt-Y and enter the memory number.  The receiver may

be tuned by direct-access without having to leave memory mode.  Smart

R8 Control will handle this automatically.

A memory number can be loaded (inserted) with the current settings by

pressing Alt-I and entering the memory number.  Memory insertion can

not be performed while operating within memory mode.  This last fea-

ture is analogous to the V-M key on the receiver, and only locations 0

through 99 are valid.

Alt-C - Scanning Mode Operation.

Alt-P - Scanning Parameters.

Scanning mode can be accessed by pressing Alt-C.  This function does

not work very well on the shortwave bands and is only recommended

while using the FM demodulator.  The only exception is for scanning in

the AM Broadcast Band while using the AM demodulator.  Using the left

and right cursor keys during scanning mode will allow the user to

change scanning to increment in that direction, respectively.  If the

scanning parameters have not been set, indicated by a '??' on the

faceplate, Smart R8 Control will prompt for them.

The parameters may be changed later by pressing Alt-P.  Scanning modes

are entered as 'M' for Memory (scans all memories), 'L' for List

(scans specific memory banks), or 'A' for A-B (scans between the fre-

quencies in the A and B VFOs).  If list scanning is selected, the pro-

gram will prompt for the banks to be scanned (i.e., typing '123' will

scan banks 1, 2 and 3).  The program will then prompt for the scanning

type.  Scanning types are entered as 'S' for Seek (stop scanning on

carrier found), 'T' for Time (stop for a while on carrier found), or

'C' for Carrier Detect (stop on carrier found, continue when dropped).

If memory or list scanning is used, the user must remember to enter

memory mode and initially tune to a memory location within the speci-

fied memory list.  Failure to do has been known to confuse the re-

ceiver.

Alt-T - Tape VOX Control.

Smart R8 Control provides for a Tape VOX control.  If a tape recorder

is connected to the line audio and timer connections in the back of

the receiver, pressing Alt-T will activate the timer relay in the re-

ceiver.  This is provided as a handy 'footswitch' for a controlling a

tape recorder in order to capture radio programs or updates to the

program schedules.  The Tape VOX control, once activated, will only

roll the tape if a carrier is present.  This way, if the receiver is

being used on the VHF Service frequencies (i.e., Police, Fire), 'dead

air' will not be recorded, thus compressing tape usage.

Smart R8 Control  version 1.02�                                   Page 9 of 17

DATABASE QUERIES.

F5 - What Shortwave Stations Are On Now.

Shift-F5 - What BCB/CB  Stations Are On.

Alt-F5 - What Utility Stations Are On.

This feature will query the database to provide a selectable list of

stations that are on at that time.  This list will be shown in a

scrollable window in the bottom half of the full screen.  The stations

are listed from left to right by order of start time, end time, and

country name.  Certain stations, such as the BBC, will jump around the

window upon subsequent calls due to the fact that most of the BBC en-

tries are indexed in 15 minute blocks.  This is despite the fact that

the BBC broadcasts around the clock.

If any of the frequencies that the station uses have been previously

marked by the user, the station name will be highlighted.  This will

alert the user that this station has been heard previously, during

that time (see 'F6 - Find Station Frequencies', below).  The list of

station names can be indexed with a cursor bar.  If the station list

contains more stations than is shown in the visible window, arrows

will be displayed inside the left edge of the window.  Navigation of

the cursor bar can be accomplished with all of the normal cursor keys

(arrows, page keys, home, and end).  Once a station is highlighted

with the cursor bar, it may be selected by pressing the 'Enter' key.

This has the same effect as pressing F6 to find the stations broadcast

frequencies (see 'F6', below).  If a frequency list is already show-

ing, the user can automatically go back to that window by using the

'Tab' key.

With version 1.02� of Smart R8 Control, the user may now query to this

window by three separate classes of HF (and MW) broadcasts.  Pressing

F5, alone, will display all scheduled and 24 hour shortwave stations.

Pressing Shift and F5 together, will provide the user a display of all

AM Broadcast Band (BCB) and Citizen Band entries in the database.

Pressing the Alt and F5 keys together will provide the user a display

of all utility stations (e.g., Amateur nets, military, maritime, time,

press, etc.) in the database.

F6 - Find Station Frequencies.

A shortcut to finding a particular station's frequencies is to use

this direct query.  After pressing the F6 function key, Smart R8 Con-

trol will prompt the user to provide the name of the station to query.

Once the string is entered, and a matching station is on at that time,

a window containing a frequency list will appear along the right edge

of the full screen.  Each frequency will be associated with the time

and days of broadcast (within the one-hour block) and the intended

target area.  Typically, on the East Coast of North America, frequen-

cies targeted for Asia and the Pacific can be avoided during certain

times of the day.  If any of the frequencies in the list have been

previously marked by the user, the frequency will be highlighted.  It

is anticipated that this will be used to alert the user that this sta-

tion has been previously heard on this frequency during that time pe-

riod.

Smart R8 Control  version 1.02�                                   Page 10 of 17

The list of frequencies can be indexed with a pointer.  If the fre-

quency list contains more frequencies than is shown in the visible

window, arrows will be displayed inside the top edge of the window.

Navigation of the pointer can be accomplished with the up and down

cursor, up and down page, home, and end keys.  Once a frequency is

highlighted with the pointer, it may be selected by pressing the

'Enter' key.  This will subsequently tune the radio to that frequency.

If the station list window is present at the bottom of the full

screen, the user can automatically go back to that window by using the

'Tab' key.

The frequency can be marked for future reference by pointing to it and

by pressing the 'Ctrl' and 'Enter' keys, simultaneously.  This feature

is a useful way to draw attention to frequencies audible at the lis-

teners location.  A marked frequency will be evidenced by a high-

lighted entry in this window, as well as the station name in the face-

plate window if the receiver is currently tuned to it.

With version 1.02� of Smart R8 Control, the user can make minor

changes to the database through this window.  A frequency entry can be

modified by pointing to it and by pressing the 'Insert' (or 'Ins')

key.  Similarly, the user may wish to add a new frequency to the list,

this may be done by pointing to the approximate location the entry is

to be inserted and pressing either the 'Ctrl' and 'Home' or 'Ctrl' and

'End' keys.  'Ctrl-Home' will add the entry above the pointer, and

'Ctrl-End' will add the entry below the pointer.

Whether the user is modifying or adding a frequency to the list, the

software will then prompt the user to enter the new frequency.  If the

user simply presses 'Enter' in response to this prompt, the frequency

that the Drake R8 receiver is currently tuned to will serve as the de-

fault input frequency.  The software will then prompt the user for the

new target area for this frequency, if known.  The user may elect to

leave this blank.  After completing input of all the modification pa-

rameters, the software will prompt the user if he wishes to go ahead

with the modification, as a safety measure.

Also, with version 1.02�, the user may delete a frequency in the list.

This is done by pointing to the frequency entry and pressing the

'Delete' or ('Del') key.  As before, the software will then prompt the

user if he wishes to go ahead with the frequency entry deletion, as a

safety measure.

Whether the user is marking, modifying, adding or deleting a frequency

entry in this window, the database will be changed.  This will be evi-

denced by '*' in the upper left hand corner of the faceplate window.

The database change will not become permanent until it is saved (F9)

to disk.

F7 - Station's Full Program Schedule.

A particular station's full 24-hour broadcast schedule may be queried

by using the F7 function key.  As with the F6 function key, Smart R8

Control will prompt the user to provide the name of the station to

query.  Once the station name is entered, a scrollable window contain-

Smart R8 Control  version 1.02�                                   Page 11 of 17

ing the full broadcast schedule will appear along the bottom half of

the full screen.  The format of the information displayed will be sim-

ilar to that found in the Shortwave Guide section of Grove Enter-

prises' Monitoring Times magazine.  Frequencies previously marked are

highlighted in the list to allow for planning purposes.  If the sched-

ule contains more listings than are shown in the visible window, ar-

rows will be displayed inside the left edge of the window.  Navigation

of the schedule window can be accomplished with the up and down cur-

sor, up and down page, home, and end keys.  If a file copy of the

schedule is desired, the query can be made by pressing the 'Shift' and

F7 function keys, simultaneously.  The schedule may be printed in the

same manner, by either pressing 'Enter' in response to the file name

prompt, or by using the file name 'PRN'.  Since printed and file

copies of the schedule cannot highlight marked frequencies, per se,

marked frequencies will be followed by a '*' instead.

F8 - Who Else Uses This Frequency.

The database typically contains entries for broadcasts in English.  If

foreign language broadcasts are encountered, this will usually be made

on a frequency used for English broadcasts as well.  A listing of all

stations that will use the currently tuned frequency at any time of

the day may be accessed by pressing the F8 function key.  This will

produce a scrollable window containing a list of the these stations,

their times of operation on that frequency (in English), and the tar-

get area for the broadcast.  The window will appear along the bottom

left half of the full screen.  Stations with a previously marked fre-

quency are highlighted in the list.  If the listing contains more sta-

tions than are shown in the visible window, arrows will be displayed

inside the left edge of the window.  Navigation of the schedule window

can be accomplished with the up and down cursor, up and down page,

home, and end keys.

F9 - Save Changed Database.

Marking frequencies will change the database, as evidenced by a '*'

displayed in the upper left corner of the faceplate window.  It is im-

portant to note that the entire shortwave database is contained in

memory (rather than direct disk access) to make operations on it more

efficient.  During some operations, a changed database will automati-

cally be saved to disk (e.g., a shell to DOS and exiting the program).

This can be done manually, by pressing the F9 function key.  While the

database is being saved, a small bar scale display will show the rela-

tive progress of the operation.

Alt-F9 - Dump Database To Disk (Registered Users Only).

Major modifications to the database are performed by using a text edi-

tor.  In order to make major modifications to the database, the

database must be dumped in a text file format.  Pressing the Alt-F9

dumps the database to disk in a text format.  This feature is re-

stricted to registered users that have the Smart R8 Control Database

Utility, which then compiles this text file back into a run-time for-

mat.

Smart R8 Control  version 1.02�                                   Page 12 of 17

F3 - List Selected DX Shows.

The R8 Communications Receiver is especially designed for listening to

programs.  Pressing the F3 function key displays a scrollable window

along the bottom half of the full screen, containing a list of se-

lected programming for that UTC day.  Each entry displays the name of

the station and UTC time that the program is on.  If the station hap-

pens to be on at the time displayed in the faceplate window it will be

highlighted in the list, regardless of whether the show is actually

on.  This was implemented in this manner, because stations often shift

program times during the schedule season.  If the listing contains

more programs than are shown in the visible window, arrows will be

displayed inside the left edge of the window.  Navigation of the pro-

gram window can be accomplished with the up and down cursor, up and

down page, home and end keys.

The database of programs is contained in the file 'DXSHOWS.TXT'.  As

an example, DXSHOWS.TXT mostly contains listings for radio oriented

programming, such as Glen Hauser's 'World of Radio'.  This file is a

field-delimited flat file that may be edited with simple text editor.

The user may edit this file, provided that the general format of the

examples are adhered.  Another restriction is that the station name

must match exactly what is in the main shortwave database.

Note:  For queries prompting for a station name, the user may enter

the full or partial name of the station.  The entry must consist of a

discriminating string.  For example, the user could enter 'BBC' or

'London' to query 'BBC, London'.  However, using terms like 'Radio' or

'Voice' alone should be avoided, because there are hundreds of sta-

tions that use these terms.  This would result in a listing of thou-

sands of entries, for which the software cannot support.

OTHER FUNCTIONS.

F1 - Display Help Screen.

Most of the key commands used in Smart R8 Control will be easily memo-

rized.  In the event that some reminder is required, a help screen

containing all of the key commands is provided.  The help screen is

organized by keys pertaining to radio operation, database query and

other utilities.  In the event that the F1 key cannot be associated

with displaying the help screen, a small reminder is positioned at the

bottom of the receiver's faceplate window.

F2 - Display Shortwave Band Plan.

Despite the rulings of the World Administrative Radio Conference

(WARC) and the International Telecommunications Union (ITU), there are

a plethora of stations broadcasting outside of the authorized broad-

cast bands.  By pressing the F2 function key, a fixed window of the de

facto band plan will be displayed in the lower right corner of the

full screen.  The frequency limits displayed are not necessarily hard

and fast.  The VOIRI, Iran, still broadcasts on the odd ball frequency

of 9022 kHz, which is nowhere near the limits of the 31 meter band.

However, the limits displayed represent where the majority of short-

Smart R8 Control  version 1.02�                                   Page 13 of 17

wave broadcasters may be found.  It serves as a helpful reminder of

what frequencies to search while using the stepped method of tuning

within Smart R8 Control.

Alt-F - Run HF Propagation Prediction (Registered Users Only).

HF propagation is a varying medium.  Of the frequency entries provided

in the database, many are not to the listener's target area but may be

audible there.  In order to better predict which frequency will be the

best audible in a particular area, a link to the Smart R8 Control HF

Propagation Prediction Utility is provided within the main program.

This utility can be invoked by pressing the 'Alt-F' key combination.

If 'Alt-F' is pressed when the user is either pointing to a particular

station in one of the 'Available Stations' windows, the HF Propagation

Prediction Utility will attempt to find that station, for an expedient

calculation.  If 'Alt-F' is pressed while in the 'Frequency List' win-

dow, the utility will point to this station, as well.

This feature is restricted to registered users that have the Smart R8

Control HF Propagation Prediction Utility.

Alt-L - Enter Log Information.

One of the mundane tasks of the listening hobby is the logging of ac-

tivity.  Smart R8 Control automatically stamps the time, date and fre-

quency on all entries.  Upon pressing Alt-L, Smart R8 Control prompts

the user for a log entry.  Any additional information may be typed

here, such as service, call, etc.  The entry is concluded with the

'ENTER' key.  All entries are posted in the file 'SMARTCON.LOG'.

F4 - Display Log Information.

It may be desirable to re-display old log information for additional

reference.  By pressing the F4 function key, a scrollable window is

displayed, containing all information accumulated in the log file

'SMARTCON.LOG'.

Alt-D - Demo Mode.

Because the serial interface requires a hardware handshake, it is im-

perative that the receiver be hooked up to the computer during program

run time.  If Smart R8 Control is to be run for database access only,

the program can be put into 'Demo Mode' from within the program.  This

will disengage the hardware handshake, and is invoked by pressing Alt-

D.  The receiver must be off and the program must recognize that the

receiver is off before this function can be invoked.

Alt-Q - Clear Screen.

Sometimes it is desirable to unclutter the screen.  Pressing Alt-Q

will clear all windows from the screen, with the exception of the re-

ceiver faceplate window.

F10 - Shell to DOS.

Pressing the F10 function key will automatically save the database (if

it has changed) and temporarily put the user in hands of DOS.  This

allows the user to edit the log file with a small text editor of

his/her choosing.  Pressing F10 could also be used to free up the se-

Smart R8 Control  version 1.02�                                   Page 14 of 17

rial line to use a terminal unit (TU) or terminal node controller

(TNC) with a separate terminal program.  It is recommended that the

user remember that this is only a child process of DOS and Smart R8

Control is still dominating a chunk of memory.  While shelled out to

DOS, the user must type 'EXIT' to return to Smart R8 Control.  A small

reminder of this has been inserted into the DOS prompt while in the

DOS shell.

Alt-X - Exit Program.

Pressing Alt-X will automatically save the database (if it has

changed), save the current settings to 'SMARTCON.DAT' and return the

user to DOS.  Since this program alters certain computer parameters,

it is important that the user ONLY exit the program in this manner.

If for some reason the program hangs, the best solution is to reboot

the machine with Ctrl-Alt-Del.  Pressing Ctrl-Break to 'break' out of

Smart R8 Control should be avoided.

EXPECTED FUTURE ENHANCEMENTS.

The formal release of Smart R8 Control v1.1 is expected to occur some-

time in the April-May 1993 time frame.  Some of the features expected

for the formal release are as follows:

Capability to display a station's frequencies, when selected within

the 'DX Shows' window.

Compatibility with TRS Consultant's 'English Language SWBC Schedules,'

as a source of SWBC frequency information.

Compatibility with a mouse, to access Smart R8 Control functions (per

Charles Smiley).

Compatibility with the OS/2 operating system (per Raymond Pfaff).

CONCLUSION.

The author would appreciate any comments regarding the improvement of

this software or any bug reports (nobody's perfect).  The author may

be contacted on the following bulletin board system:

The Cats Meow BBS, Midland, Virginia         1-703-439-3060

(Name used on BBSs: 'MARK FINE')

He may also be contacted via any bulletin board system currently par-

ticipating in the following national BBS conference echo:

RIME (RelayNet) SCANNERS echo

(route using '->MEOW', R/O capable, public key 26936)

Smart R8 Control  version 1.02�                                   Page 15 of 17

The author may also be contacted by mail at his home address (phone

calls are discouraged):

MARK J. FINE

c/o FineWare

11252 Cardinal Drive

Remington, Virginia 22734-9684

Comments should be constructive and should contain detailed descrip-

tions of what the user would like to see included in this software.

If a bug report is in order, please include information such as com-

puter make and model, version and make of DOS used, a hard copy of the

user's AUTOEXEC.BAT and CONFIG.SYS files, as well as detailed descrip-

tions of the nature of each problem.  All constructive comments and

bug reports will result in a prompt, courteous response.

VERSION HISTORY.

v1.02� - Released 27 March 1993 - Enhanced beta release.

Added interface to HF Propagation Prediction program (SCP.EXE).  SCP

is now invoked by pressing Alt-F.  If the user is in the WHAT or

FIND windows, SCP will automatically set the Station 2 pointer to

match that station name, if data is available.  (Registered users

only).

Added separate key to create a dump of the frequency database.  This

is now invoked by pressing the Alt-F9 keys.  (Registered users

only).

Added capability to insert or delete individual frequencies within the

'Find' window.  Dumping, altering and then compiling the database

is now only required for major changes to the database.  The in-

sert key is used to modify the frequency record in the list.  The

delete key is used to delete the record from the list. Ctrl-Home

and Ctrl-End are used to insert frequencies above or below the

frequency pointed to in the list, respectively.

Added capability to search utility stations, as well as shortwave sta-

tions. By pressing Alt-F5, the station window switches to search

for all of the strict utility stations (time, maritime, WEFAX,

etc.) in the database.  Similarly, all AM Broadcast Band (BCB)

stations in the database, as well as a table of CB channel fre-

quencies can be searched by pressing Shift-F5.

Redesigned and updated the input windows and help screen to be more

functional.  The input windows now have more room for log en-

tries.

Fixed bug that only allowed the user to set the radio interface to ei-

ther COM1: or COM2:.  Program now correctly allows up to and in-

cluding COM4:.

Fixed bug that incorrectly tried to compile an unchanged database.

When a databse dump is created, and the user shells into DOS, the

program recompiled the database even if it wasn't changed.  This

has been corrected.  (Registered users only)

Fixed bug that mishandled the size of information displayed in the

windows.  Smart R8 Control was not correctly trapping the size of

Smart R8 Control  version 1.02�                                   Page 16 of 17

frequencies, stations, log entries, etc. in their respective win-

dows.  If more than the expected amount of information was re-

trieved, a run-time error would occur.  This has been corrected.

Fixed documentation. Documentation erroneously stated the command line

switch to set the serial port was "P".  This has been corrected

to read "R".

v1.01� - Released 01 February 1993 - Follow up beta release.

Extended Band Plan to better reflect the set of frequencies in the

database.

Added display of registration number in the faceplate of the main

screen.

Fixed bugs in the companion Printing and Database Utility software.

Updated documentation.

v1.0� - Released 02 January 1993 - Initial beta release.

ACKNOWLEDGEMENTS.

* The Drake R8 Communications Receiver is a product of the R. L. Drake

Company (Drake), Miamisburg, Ohio 45342.

* The format of the shortwave database used in Smart R8 Control is

based upon the Shortwave Guide column found in Monitoring Times maga-

zine.  Monitoring Times is a monthly publication of Grove Enterprises,

Inc., 140 Dog Branch Road, Brasstown, North Carolina, 28902-0098.

Special credit is given to Bob Grove, Publisher, and Bill Grove, SYSOP

of the Grove Systems BBS, for their permission to reproduce this for-

mat.  The Grove Systems BBS can be accessed, Monday through Friday

from 6:30PM to 8:00AM EST and 24 hours on weekends, on the following

telephone numbers:

(704) 837-7081  9600/14400 BPS

(704) 837-9200   300/ 1200/ 2400 BPS

(704) 837-5957   300/ 1200/ 2400 BPS

* English Language SWBC Schedules is a product of TRS Consultants, PO

Box 2275, Vincentown, NJ 08088-2275.

Smart R8 Control  version 1.02�                                   Page 17 of 17