April 1994

FCC Frequency Allocations

---------------------------

A summary of the FCC Table of Frequency Allocations, based on

the Oct '91 Code of Federal Regulations  -  47 CFR 2.106

30 - 50 MHz:  FM @ 20 kHz steps

30.010 -  30.550  US Government

30.560 -  31.980  Business / Industry / Forestry

31.990 -  32.000  Public Safety

32.010 -  33.000  US Government

33.000 -  33.100  Public Safety

33.120 -  33.400  Business / Petroleum

33.420 -  34.000  Fire

34.000 -  35.000  US Government

35.020 -  36.000  Business / Paging

36.000 -  37.000  US Government

37.020 -  37.420  Police / Public Service

37.460 -  37.860  Power, Water, Pipeline

37.900 -  38.000  Highway Maint / Special Emergency

38.000 -  39.000  US Government

39.020 -  40.000  Police / Public Service

40.000 -  42.000  US Government

42.020 -  42.940  State Police

42.960 -  43.680  Business / Paging

43.700 -  44.600  Transportation - bus, truck

44.620 -  45.060  State Police / Forestry Conservation

45.080 -  45.860  Police / Local Govt / Highway Maint

45.900 -  46.040  Police / Emergency

46.060 -  46.500  Fire

46.520 -  46.580  Local Govt

46.610 -  46.970  Cordless Phones - base        (30 kHz steps)

47.020 -  47.400  Highway Maint

47.440 -  47.680  Industry / Emergency

47.700 -  49.580  Industry

49.610 -  49.970  Cordless Phones - handset     (30 kHz steps)

50 - 150 MHz

50.000 -  54.000  Amateur (6-meters)

54.000 -  72.000  Broadcast TV   chs 2-4        (6 MHz steps - FMw)

72.000 -  76.000  (various)

76.000 -  88.000  Broadcast TV   chs 5-6        (6 MHz steps - FMw)

88.000 - 108.000  FM Broadcast                  (200 kHz steps - FMw)

108.000 - 118.000  Aero - navigation

118.000 - 136.000  Aero - communications         (25 kHz steps - AM)

136.000 - 138.000  Satellite

138.000 - 144.000  US Government

144.000 - 148.000  Amateur (2-meters)

148.000 - 150.800  US Government

150 - 162 MHz:  FM @ 15 kHz steps

150.815 - 150.965  Auto Emergency

150.995 - 151.595  Highway / Forestry / Industry

151.625 - 151.955  Business                      (30 kHz steps)

152.030 - 152.240  Mobile Telephone (Base)       (30 kHz steps)

152.270 - 152.450  Taxi (Base)

152.495 - 152.855  Mobile Telephone (Base)       (30 kHz steps)

152.870 - 153.725  Industry

153.740 - 154.445  Fire / Govt (mobile)

154.452 - 154.481  Govt / Industry (telemetry)   (7.5 kHz steps)

154.490 - 154.625  Industry

154.650 - 156.240  Police / Govt / Emrgncy / Hwy

156.025 - 157.425  Maritime (ship)               (25 kHz steps)

157.470 - 157.515  Auto Emergency

157.530 - 157.710  Taxi (mobile) / Business

157.770 - 158.100  Mobile Telephone (mobile)     (30 kHz steps)

158.130 - 158.460  Industry

158.490 - 158.670  Mobile Telephone (mobile)     (30 kHz steps)

158.730 - 158.210  Police / Govt / Highway

159.225 - 159.465  Forestry Conservation

159.495 - 160.200  Transportation - bus, truck

160.215 - 161.610  Railroad

160.625 - 160.950  Maritime - Coast              (25 kHz steps)

161.640 - 161.760 {Broadcast Pickups

161.500 - 162.025 {Maritime - Coast              (25 kHz steps)

162 - 450 MHz

162.025 - 174.000  (various, mainly US Government)

174.000 - 216.000  Broadcast TV  chs 7-13        (6 MHz steps - FMw)

216.000 - 220.000  Maritime                      (25 kHz steps)

220.000 - 222.000  (Private land Mobile)

222.000 - 225.000  Amateur

225.000 - 400.000  US Government - Aero          (AM)

400.000 - 406.000  US Govt - Meteorlogical / Space

406.000 - 420.000  US Government

420.000 - 450.000  Amateur / military radar

450 - 460 MHz:  FM @ 25 kHz steps  (450-455 base, 455-460 mobile)

450.050 - 450.925  Auxiliary Broadcasting

451.025 - 451.275  Power / Industry

451.300 - 451.500  Telephone Maint / Industry

451.525 - 452.025  Industry / Special Industrial

452.050 - 452.500  Taxi / Industry / Transport

452.525 - 452.600  Automobile Emergency

452.625 - 452.950  Transportation - Trucks / Railroad

452.975 - 453.000  Relay Press

453.025 - 453.975  Local Govt / Public Safety

454.025 - 454.650  Mobile Telephone

454.675 - 454.975  Mobile Telephone Air

455.050 - 455.925  Auxiliary Broadcasting

456.025 - 456.275  Power / Industry

456.300 - 456.500  Telephone Maint / Industry

456.525 - 457.025  Industry / Special Industrial

457.050 - 457.500  Taxi / Industry / Transport

457.525 - 457.600 {Maritime - shipboard repeater

{Business - low power

457.625 - 457.950  Transportation - Trucks / Railroad

457.975 - 458.000  Relay Press

458.025 - 458.975  Public Safety / Local Govt

459.025 - 459.650  Mobile Telephone

459.675 - 459.975  Mobile Telephone Air

460 - 470 MHz:  FM @ 25 kHz steps  (460-465 base, 465-470 mobile)

460.025 - 460.550  Police / Public Safety

460.575 - 460.625  Fire

460.650 - 460.875  Business - Airport use

460.900 - 461.000  Business - Central Alarms

461.025 - 462.175  Business

462.200 - 462.525  Manufacturers / Industry

462.550 - 462.725  GMRS

462.750 - 462.925  Business (paging)

462.950 - 463.175  MED (Ambulance/Hospital)

463.200 - 465.000  Business

465.025 - 465.550  Police / Public Safety

465.575 - 465.625  Fire

465.650 - 465.875  Business - Airport use

465.900 - 466.000  Business - Central Alarms

466.025 - 467.175  Business

467.200 - 467.525  Manufacturers / Industry

467.550 - 467.725  GMRS

467.750 - 467.925 {Business (low power)

467.750 - 467.825 {Maritime - shipboard

467.950 - 468.175  MED  (Ambulance/Hospital)

468.200 - 469.975  Business

470 - 806 MHz:  6 MHz per channel, wide FM audio

470.000 - 512.000 {Broadcast TV, chs 14-20

{Large Metro Public Safety     (25 MHz steps - FM)

512.000 - 806.000  Broadcast TV, Chs 21-69

806 - 896 MHz:  FM @ 25 kHz steps  (mobile 806-851, base 851-896)

806.000 - 809.750  General - conventional

809.750 - 816.000  General - trunked

816.000 - 821.000  SMR - trunked

821.000 - 824.000  Public Safety - trunked       (12.5 kHz steps)

824.040 - 834.360  Cellular Telephone            (30 kHz steps)

834.390 - 835.620  Cellular Telephone (data)     (30 kHz steps)

835.650 - 848.970  Cellular Telephone            (30 kHz steps)

849.000 - 851.000  Aircraft Telephone            (5 kHz steps - AM)

851.000 - 854.750  General - conventional

854.750 - 861.000  General - trunked

861.000 - 866.000  SMR - trunked

866.000 - 869.000  Public Safety - trunked       (12.5 kHz steps)

869.040 - 879.360  Cellular Telephone            (30 kHz steps)

879.390 - 880.620  Cellular Telephone (data)     (30 kHz steps)

880.650 - 893.970  Cellular Telephone            (30 kHz steps)

894.000 - 896.000  Aircraft Telephone            (5 kHz steps - AM)

896 - 941 MHz

896.000 - 901.000  Business / Industry / SMR     (12.5 kHz steps)

901.000 - 902.000  General Purpose Mobile Radio

902.000 - 935.000  ()

935.000 - 940.000  Business / Industry / SMR     (12.5 kHz steps)

940.000 - 941.000  General Purpose Mobile Radio