--------------------------- PCTIMER 1.2 --------------------------

Program written by: Chih-Hao Tsai (c-tsai4@uiuc.edu)       1/29/95

------------------------------------------------------------------

This  program  is  FREE. Feel free to use, re-distribute or modify

it.  However, you should recognize that this program comes without

any  warranty.  It  is  recommended that you examine and test this

program  before  actually use it. If you find this program useful,

or if you find a bug, please send an e-mail to me to let me know.

------------------------------------------------------------------

1. PCTIMER

PCTIMER  is  a  high-resolution timing routine in the form of C

functions. You can compile and link PCTIMER to your programs in

which millisecond (or even higher) accuracy timing is required.

History

11/5/94 Version 1.0

The  first  freeware  release of PCTIMER. With source codes for

GNU C/C++ and Microsoft C/C++ compilers.

1/16/95 Version 1.1

(1)  In  version  1.0, gccint8.c only intercepts protected mode

int  8h,  which may cause loss of ticks when programs switch to

real  mode.  I  revised  gccint8.c that it would intercept both

protected mode and real mode int8h's. (I would like to thank DJ

Delorie,  Eli  Zaretskii,  and  Long  Doan  for  their  helpful

suggestions and discussions.)

(2) I added sources for Turbo C/C++ compiler.

(3) The delay8h function had been slightly modified.

1/29/95 Version 1.2

Revision  (and  correction)  of  gccint8.c. PCTIMER for GCC was

first  developed  under  gcc  2.6.0,  and it was found that the

following  instructions:  "r.x.cs  = rm_old_handler.rm_segment;

r.x.ip   =  rm_old_handler.rm_offset;  r.x.ss  =  r.x.sp  =  0;

_go32_dpmi_simulate_fcall_iret(&r);"     could     not     call

rm_old_handler.rm_segment:rm_old_handler.rm_offset   correctly.

Therefore,  my  solution  in PCTIMER 1.1 was to call f000:fea5,

which  is  the  default  handler  of int 8h in IBM PC and fully

compatible  BIOSes.  Since the bug has been fixed in gcc 2.6.3,

the  real  mode interrupt handler "rm_new8h" in gccint8.c could

now  be  revised. As a result, PCTIMER for GCC now requires GCC

version 2.6.3 or higher.

2. RATIONALE FOR WRITING AND USING PCTIMER

Although  the  timing  routines  of  most programming languages

always  report times to centisecond, the resolution of standard

timer  of  IBM-PCs is actually only 1/18.2 seconds. That is, if

you   use   the   standard  timing  routine  provided  by  your

programming  language,  the  precision  of  your measurement is

actually  55  milliseconds  with  a  plus/minus 27.5 seconds of

measurement  error!  I  wrote  PCTIMER  to provide IBM-PCs with

high-resolution timing ability.

3. FEATURES

(1) Precise: Basically, PCTIMER is designed to provide a timing

routine  with millisecond resolution. If such a resolution does

not  satisfy  you,  you  can  also  modify  it  to  get  higher

resolutions.  However, in most cases, millisecond resolution is

precise enough.

(2)  Stable:  PCTIMER  sets  new  parameters  to  the  8254  IC

(Programmable  Interrupt  Timer)  to  get  high  frequencies of

SYSTEM  TIMER  Interrupt (IRQ0, or INT8h) whose default is 18.2

times  per  second.  However,  many hardware routines depend on

IRQ0.  Abnormal IRQ0 frequencies may cause the system unstable.

To  maintain  the  PC's stability, PCTIMER intercepts the INT8,

only calls the original INT8 at the frequency of 18.2 times per

second.   Consequently,   high-resolution   timing  and  system

stability are simultaneously achieved.

(3)  Flexible:  Originally  I  wrote  PCTIMER to do millisecond

resolution  timing.  If millisecond precision in unnecessary to

you,  you  can  also  set  PCTIMER  to  do  centisecond timing.

Resolution  higher  than  millisecond is also possible, but you

have  to  modify  a  few  parts  of the program by yourself. It

should be an easy task.

(4) Portable: Most public-domain timing routines are written in

assembly  codes, or in C codes with in-line assembly codes. The

assembly  codes  make those routine difficult to understand and

use  if  one  does  not  have  good  background of the assembly

language.  And  the  format of in-line assembly varies from one

compiler  to  another. Worst of all, several C compilers do not

support in-line assembly. PCTIMER does not have these problems,

because  all  functions  are  written  in C codes. There are no

assembly codes at all.

4. SYSTEM AND COMPILER REQUIREMENTS

(1)  PCTIMER  is  a  tiny  timing  routine  that  does not have

specific memory and CPU speed limitations. The only requirement

is  that  it  only  runs on IBM-AT compatible PCs. An XT is not

appropriate  because  an  XT  does not have an CMOS IC. PCTIMER

need the CMOS IC to obtain correct system time.

(2)  PCTIMER  comes with three versions, one for GNU C/C++, one

for  Microsoft  C/C++, and one for Turbo C/C++. The GCC version

was  developed  under  GCC 2.6.3, the MSC version was developed

under  MSC  6.0,  and  the  Turbo C version was developed under

Turbo C/C++ 2.0. There should be no problems if you are using a

different  version  of  Turbo C or MSC. However, the version of

GCC *does* matter. Please do not use GCC older than 2.6.3.

5. USING PCTIMER

(1)  At  the beginning of your program, include the header file

of PCTIMER:

#include "gccint8.h"

/* "mscint8.h" for MSC; "tcint8.h" for Turbo C */

(2)  Declare  three  unsigned  long  integers (of course, their

names are arbitrary):

unsigned long int tick_1, tick_2, elapsedtime;

(3)  In  your program, use the following function and parameter

to initiate the timer:

init8h(1000); /* init8h(100) for centisecond resolution */

ticks_8h = 0;

(4) Timing

tick_1 = ticks_8h;

/* .... do some stuff here .... */

tick_2 = ticks_8h;

elapsedtime = time8h(tick_1, tick_2);

/*  The  function  "time8h" always returns millisecond unit

digits.  That  is,  if you use init8h(1000), then the value

returned  by  time8h  is  exactly  the time elapsed between

tick_1  and  tick_2.  However,  if you use init8h(100), the

value   returned   by  time8h  is  still  in  the  unit  of

millisecond.  The  difference is that the value returned by

time8h  in  the  init8h(1000)  condition is measured in the

precision  of  one millisecond, while the value returned by

time8h  in  the  init8h(100)  condition  is measured in the

precision of 10 milliseconds. */

(5) How to turn off the timer:

quit8h();

/*  Be  sure  to  turn  off  the  timer before your program

terminates.  Also  turn  off the timer before you set a new

parameter for init8h. */

(6) delay8h and sound8h

/*  The  "delay8h" is a high-resolution delay function. The

parameter  is  always in the unit of millisecond, no matter

what you put in the "init8h". However, the precision of the

delay  duration  depends on the parameter of "init8h". Here

is an example for a 150 milliseconds delay: */

delay8h(150);

/* the "sound8h" takes advantage of "delay8h" to generate a

sound  with  a  specific duration. Here is an example for a

800 Hz sound with 200 ms duration: */

sound(800, 200);

6. LIMITATION

PCTIMER  uses  an unsigned long integer to count the cumulative

frequency  of  INT8h interrupts. Assuming you set init8h(1000),

ticks_8h  =  0, PCTIMER will work 4294967296/1000 = 4294967.296

seconds,  or  1193.05 hours without encountering the problem of

overflow. I hope that the 1193.05 hours would be pretty enough.

If  you do need to continuously use PCTIMER for a period longer

than  1193.05  hours,  just  reset  the  ticks_8h  to  zero  at

appropriate location of your program.

7. FILES

pctimer.doc    This file

gccint8.h      PCTIMER: Header file for GCC

gccint8.c      PCTIMER: Source code for GCC

mscint8.h      PCTIMER: Header file for Microsoft C

mscint8.c      PCTIMER: Source code for Microsoft C

tcint8.h      PCTIMER: Header file for Turbo C

tcint8.c      PCTIMER: Source code for Turbo C

test8.c      Source code for test8.exe

test8.exe    Test program (Note 1)

maketest.bat    Simple batch file to create test8.exe (Note 2)

Note  1.  This test program was compiled by GCC 2.6.3. That is,

it requires PCs with 32-bit CPUs and go32.exe.

Note  2. This batch file is also for GCC. Please make necessary

changes if you do not use GCC.

------------------------------------------------------------------