CARBON COPY MODEM DESCRIPTOR UTILITY

Table of Contents

-----------------

OVERVIEW

RELATIVE SOFTWARE VERSIONS

MODEM DESCRIPTOR TABLE

Creating and Maintaining Descriptors

Menu Options

EDIT MODEM DEFINITION MENU

Edit Options

Data Field Descriptions

ADD ACTION SYMBOL MENU

Edit Options

Symbol Descriptions

HARDWARE and SOFTWARE RELATIONSHIPS

Hardware Leads and Software Commands

Software Register Values

Error-Correcting Modems

Result Code Values

OVERVIEW

The MODEM DESCRIPTOR is the file MODEM.DSC which is distributed with the Carbon Copy PLUS and CCExpress software packages.  The following is a description of the utility CCMODEM.EXE which is used to create and maintain the modem descriptor file.

The MODEM.DSC file contains not only modem entries, such as MICROCOM and Hayes, but also communciation-switching system entries, such as ROLM and GANDALF.   The Carbon Copy PLUS or CCExpress software in which the modem entries are displayed are the CCINSTAL.EXE and EXINSTAL.EXE utilities.

The entries of the descriptor file are displayed within these utilities

for the end user to select a "MODEM TYPE" from the list.  The list represents the modem descriptor created and maintained by this utility.  Each modem entry is composed of data fields which dictate the performance relationship Carbon Copy PLUS and/or CCExpress software.  These data fields and their impact on performance will be described under the "EDIT MODEM DEFINITION" section.

RELATIVE SOFTWARE VERSIONS

--------------------------

There have been several versions of Carbon Copy with copies of the CCMODEM.EXE utility of the same version number.  There are several options contained in the current CCMODEM.EXE utility which are not functional for earlier versions of Carbon Copy or CCExpress.  It is assumed that any customer needing this documentation is a recent customer having Carbon Copy PLUS version 5.0 or an existing customer having a version of Carbon Copy 4.0 or CCExpress version 2.0.  This document assumes that the Carbon Copy version 3.0 and earlier will have the Carbon Copy version 4.0 UPGRADE software.

Throughout this manual there will be references to Carbon Copy, Carbon Copy PLUS and CCExpress versions, the versions have software code relationships which cause some data fields and action symbols to be version dependent. We will attempt to explain the level of behavior which can be expected from earlier software versions or whether the feature is supported in earlier versions.  This document is written focusing on the communication features of Carbon Copy PLUS version 5.0 and CCExpress version 2.0.

The modem descriptor table did not exist in Carbon Copy version 2.14 or 2.15. Carbon Copy version 3.0 was its beginning, yet only 30 modem entries were possible with version 3.0.  The modem types were displayed in the CCINSTAL.EXE utility by toggling through the list one-by-one.  In Carbon Copy version 4.0 also referred to as Carbon Copy PLUS, the modem table in CCINSTAL.EXE became a point-and-chose list with 60 modem entries. CCExpress became a product of the logic based on Carbon Copy PLUS, therefore, it uses the same modem table and performed in exactly the same manner with modems and data-switches.  The modem performance became more flexible with Carbon Copy version 5.0 which required several additional features of the CCMODEM.EXE utility.

MODEM DESCRIPTOR TABLE

----------------------

Creating and Maintaining Descriptors

There are 58 modem entry locations which can be used for building modem descriptors or building data-switch descriptors.  The 59th location is reserved for use exclusively by MERIDIAN Technology.  The 59th location should not be used; it is tied to a special area in the software code which performs "RING INDICATOR" logic used exclusively or switch-related behaviors.  Placing a modem descriptor in this location may cause errors in code performance.

The first modem entry "NO MODEM" is for use exclusively for communication through a NULL-MODEM cabling device to another PC or a RS-232 cable directly to a host system.  Since the first modem entry is not EDITable, it is not included in the modem location numbers.  All specific performance criterion

defined for the "NO MODEM" entry are coded within the software and can not be changed by the end user.  If some specific behavior is required with the use of the direct connection, then a specific modem descriptor must be

created for controlling that performance.

There are several locations at the end of the list which are "Not Defined" and can be used to create special modem descriptors.

Since CCMODEM.EXE does not create a backup copy of the MODEM.DSC file, it is strongly recommended that you COPY the file to a backup file name.  Any changes made to the modem descriptors during the run of CCMODEM.EXE will be lost in the case of a power disruption or system re-boot; the MODEM.DSC file is updated following the selection of the X-eXit option from the CCMODEM.EXE menu screen.

Menu Options@

The following describes the options used on this menu:

# - MODEM NUMBER to edit the modem descriptor

When entering the number of the modem descriptor's location the

"EDIT MODEM DESCRIPTOR" screen will appear, displaying the appropraite modem's data.

A - ALPHABETIZE

This option alphabetizes or sorts the modems in ascending lexigraphical order (ascii character set order),  starting with

0 thru 10, then 'A' thru 'Z' followed by  'a' thru 'z'.  This makes it necessary to be observant when mixing upper and lower case characters in the modem's NAME field.

C - COPY

When selecting this option, a prompt will appear

FROM: _ _

requesting the source modem descriptor number FROM which the data

field definitions will be copied.  After entering the second digit

of the descriptor number or press <RETURN> following a single digit

descriptor, a second prompt will appear

TO: _ _

This prompt requests the destination modem location TO which the descriptor's definitions are to be copied.  Since the modem description is copied exactly FROM one location TO the other, the two descriptors will have the same name.  It is suggested that the name of one of the descriptors be altered since only the modem names appear on the CCINSTAL.EXE modem screen.

To clear a descriptor entry, you could copy a modem descriptor reading  "Not Defined" into the descriptor location to be blanked. There is a second method of clearing described under the MODEM NAME field in the "Data Field Description" section.

Q - QUIT

This command will cause the CCMODEM utility to return to DOS with no updates recorded in the MODEM.DSC file.

X - EXIT

This command will cause the CCMODEM utility to return to DOS, saving all changes permanently in the MODEM.DSC file; since these updates are permanent, a backup copy is suggested prior to EDITing any modem descriptor.

EDIT MODEM DESCRIPTION MENU

---------------------------

After selecting a modem location the "EDIT MODEM DEFINITION" screen

will be displayed.  The data fields appearing in this screen will be

empty as you are editing a modem descriptor which is "Not Defined".

The cursor will appear at the beginning of the MODEM NAME field.  If you are editing a modem descriptor which has been defined (NAMED), then thedata previously defined will be displayed.  The cursor will appear on the first character of the NAME field and the reverse video or color bar will be displayed across the data field.  The cursor bar indicates the data field which is CURRENT for editing.

To enter data in the current data field, type any character on the keyboard or select an ACTION SYMBOL to be inserted in the data field. To blank a descriptor, use the DEL key to delete all characters until the reverse video or color bar disappears, then "RETURN to MENU".  The location previously selected for edit will then state "Not Defined". Each field in the modem descriptor location has been cleared. To move from data field to data field use the appropriate scroll key.

The following is an example of a MODEM descriptor as it appears in the EDIT MODEM DEFINITION screen:

insert example of MICROCOM AX-AT modem here

Edit options

There are several special edit keys for use in editing the modem's definition:

[F1] Add Action Symbol -- You may go in and out of the "ADD ACTION SYMBOL"

screen selecting symbols to be placed within the  descriptor field.  The action symbol screen is  described with more detail in the next section.  The  data field currently being edited is not completed until the "RETURN" key, the down-arrow key, or [F10] function key have been pressed.

[F10] Return  to  Menu -- The [F10] option performs different update behaviors  for CCMODEM.EXE version 5.0 and earlier, than the version 5.1

With versions 5.0 and earlier, the [F10] option exits to the main menu and all changes to the data field being edited are lost.  With version 5.1, the currently edited field is saved when the [F10] option is chosen.

[ESC] the  ESCape  key -- restores the data field currently being edited to its  previous values.

[INS] the  INSert  key -- inserts the ascii space character at the current cursor location and moves the remaining characters in data field one space to the right.

[DEL] the  DELete  key -- deletes the character at the current cursor location and moves the remaining characters in the data field one character to the left.

Arrow-Editing keys     -- The RIGHT and LEFT arrow keys move the current-edit  cursor position across the current character to next character in the data field.

The UP and DOWN arrow keys move the cursor bar either up or down to the next line field to be edited.  All edits performed to the current data line are saved if  either of these keys are pressed. The <ESC> can be  pressed to restore the data field if edits were erroneously entered.  If the data line being edited is the last "RESPONSE STRING" field and the DOWN arrow key is pressed, then the software returns to the main menu, "Carbon Copy Modem Descriptor Program", and the edits are saved in the modem descriptor table in memory.

Data Field Descriptions

References below to code or software imply the Carbon Copy code. The data field

descriptions concentrate on modem behaviors.  If a data-switch descriptor is to

be built, then one must be very careful to consider the types of calls which

the data-switch performs.  The "data-switch to data-switch" calls as well as

"data-switch through modem pool calls" should be provided for within the same

descriptor.  If this is not possible it is suggested that a descriptor for each

be built.

The following describes the definition of each of the data fields within the

"EDIT MODEM DEFINITION" screen:

MODEM NAME  (19 characters)

The name entered in this field will be the name which appears on the main

menu of this utility as well as the MODEM TABLE menu of CCINSTAL.EXE.

RESET STR 1   (63 characters)

RESET STR 2   (63 characters)

Reset string #1 and #2 are sent to the modem in one continuous stream.

They are sent once when the Carbon Copy executable is loaded, then again

when disconnecting from the other user.  The reset strings are also sent

whenever the option under DATA LINK MAINTAINENCE for RESET THE DATA LINK

is selected.

Note: the maximum command-line length (number of characters between

carriage returns) by most modems is 40 characters, excluding spaces.

When building a descriptor one must insert carriage returns followed

by a delay, keeping command-line length within character limit.

There are several modem descriptors whose reset strings are broken into

several seperate "AT" commands.  The reasons for these seperations maybe

one of the following:

a) if the command is needed for one of the manufacturers modems but

may cause an error to be returned from other types of manufacturers

modems, then it is seperated from the other commands by a carriage

return and delay symbols.  This will cause the command causing the

error to be discarded by modems with can not perform the command

but allow them to perform all other commands within the reset

string.  For each of the command-lines (command data from AT to the

carriage return) the modem only returns one result code.

b) if the reset string one command-line extends through to reset

string 2, and the total number of characters since the previous

carriage return or start of the commandline may be only forty

characters.

DIAL PREFIX STRING   (31 characters)

This string is sent prior to the phone number in each call placed.

At the time in the Carbon Copy code when this string is sent, the code

assumes that the modem is still in the command state from the reset

string execution.  This string is typically the modem's DIAL command.

Note for PBX: if the user must call through a PBX using "two stage"

dialing, this would be the location where the modem pool's

phone number would be located. Otherwise, this would be

the location for requesting the switch's attention to

initiate calling another extension in the switch.

ATTENTION STRING  (19 characters)

The attention string is represented by the diamond symbol.  This string

contains the breaks or escape character sequence necessary to get the

modem's attention and force it into the command mode.  Since the modem

can only process commands in the command state, the diamond symbol which

represents ATTENTION STRING performance should be placed in the front of

the RESET STR 1.

The ATTENTION STRING has two uses; for versions of Carbon Copy 3.0, 4.0,

and 5.0.  In Carbon Copy version 5.0 where "FULL" reset is choosen and all

performances of versions 3.0 and 4.0, the attention string is used to

send the attention/escape characters followed by the remainder of the

RESET STRING. In Carbon Copy 5.0 where the "PARTIAL" reset is chosen

only the ATTENTION STRINGS contents are sent to the modem; the RESET

STRINGS are only sent when the executable loads into memory and when the

"RESET the DATA LINK" option is performed (described under RESET STRING).

AUTO-ANSWER STRING   (19 characters)

This string is sent when the "Auto-Answer mode" of Carbon Copy PLUS

is toggled to the  ON  state (ENABLE Auto-Answer).  This option is

within the [F8] DATA LINK MAINTENANCE submenu.

NO AUTO-ANSWER STRING   (19 characters)

This string is sent when the "Auto-Answer mode" of Carbon Copy

is toggled to the  OFF  state (DISABLE Auto-Answer).  This option

is within the DATA LINK MAINTENANCE submenu.  It is also sent following

the RESET STRINGS when the "Optional Configuration Parameter" from the

CCINSTAL.EXE or EXINSTAL.EXE screen is set to

NORMAL ANSWER MODE ... NO ANSWER.

CC VOICE to DATA  STRING   (19 characters)

This string is sent to the modem when the system running CC.EXE has

previously chosen the option, "SWITCH DATA to VOICE MODE", from the

main control screen and is returning to the DATA mode.  For the user

to return to data modem he must select the option, "SWITCH VOICE to

DATA MODE", from the main control screen menu.

This string commands the modem to go to  ANSWER  mode as the other

modem is instructed to  ORIGINATE  the carrier negotiation.  ANSWER

and ORIGINATE are modem terms for LISTENING and TALKING on the phone line;

one modem must be setup for listening while the other, for talking.

CCHELP VOICE to DATA  STRING   (19 characters)

This string is sent by the system running CCHELP.EXE under similar

circumstances to the data string described above; this system's modem

is instructed to ORIGINATE the carrier negotiation between the modems.

Frequently, the modem descriptors will have an "X1" command embedded in

this string due to the "X4" (FULL call progress monitoring) command

which is the preferred command in the reset string. The progress

monitoring may report result codes when placing the call; this will

confuse the Carbon Copy code which is testing the result code data

against the CONNECT STRINGs.  The Carbon Copy code will start comparing

the result code returned during the call against the CONNECT STRINGS

instead of the result code returned after carrier is negotiated.  This

will prevent a successful SWITCH back to the data mode.

INITIAL BAUD RATE

This is the initial baud rate which will appear in the "BAUD RATE" field

on the CCINSTAL.EXE "SYSTEM PARAMETER" screen when this modem type is

selected. The baud rate value must be one of the ACTION SYMBOLS.  The

corresponding value is stored in the last character of the modem's name

field (twentieth character) in the modem descriptor file.

This feature is not supported in Carbon Copy version 3.0 and should be

left empty for every modem descriptor in the list when executing the

3.0 version.

CONNECT STRING   (19 characters)

There are eight of these fields which must hold each of the modem's

result codes.  Carbon Copy examines each of the connect strings for

a match after receiving the result code from the modem.  The carriage

return character can not be examined, therefore data having an embedded

carriage return can not be used in the connect string.  If the result

code has an embedded carriage return only the first characters up to the

carriage return character should be placed in the CONNECT STRING.  Make

sure that these characters define a unique result code.  If the

characters occur in more than one result-code string, then find a command

for the modem which will change the result codes reported to be unique

result codes (for example: from verbose result codes to non-verbose

codes where instead of words you will have digits).

To find the RESULT CODEs returned by your modem or data-switch look

in the modem manufacturer's documentation under RESULT CODES or

RETURN CODES.  Data-switches do not always return the connecting speed

to the user.  If the connecting speed is not returned a manual SWITCH

dialing process should be set-up which will be described under

DATA_SWITCH DESCRIPTIONS.

Most of the modem descriptors contain each of the result codes

indicating a CONNECT SPEED as well as the phone line BUSY result code.

If there are more than eight result codes (usually high-speed and error-

correcting modem descriptors), then the result codes stored in the

CONNECT STRINGs are selected by the following criteria:

a) Error correcting modems all error-free connecting speeds are

listed,

b) Standard settings of 2400, 1200, then 300  (600 also included

when space permits),

c) On modems maintaining a fixed DTE rate, where all baud codes

can not be examined, use a VERBOSE setting in the reset string

and as many unique characters to represent the other connections;

example the word CONNECT could represent CONNECT 2400 and

CONNECT 1200.

The following rule must be implemented when storing the connect strings

in the data fields:

a) Non-Unique entries must be ordered with the longest character

string stored in the CONNECT STRING above the shorter string.

example

Connect String:  CONNECT 2400/REL

Connect String:  CONNECT 2400

Connect String:  CONNECT             represents all other

connections 1200, 600,

and 300

another example,

Connect String: 10            represents connect 2400

Connect String: 5             represents connect 1200

Connect String: 1             represents connect 300 with

the X1 or X4 used in reset str

the connect 2400 string must be above the connect 300

b) When cursor bar is on the field, the reverse video must end on

the last character; otherwise, a space character will be considered

a part of the result code.  Since all connect strings are ended

in the modem descriptor file with a null character (indicating

end of string at the Null), the test for a match will fail since

the space character will also be tested.

RESPONSE STRING   (19 characters)

There are several standards which are implemented in the RESPONSE

STRINGs.  If the COMM port's baud rate on the PC must change to match

the connecting speed of the modem, then four .5-second delays followed

by the baud rate ACTION SYMBOL should be stored in the RESPONSE STRING.

The baud rate symbol must match the same rate that the corresponding

CONNECT STRING reports from the connection.  The preceeding delays are

to allow the modems to be quiet before the software exchanges data.

NOTE to leased line and PBX users:

Since leased lines and PBXs often maintain Carrier Detect high, it

maybe necessary to place the Carrier Detect symbol in the front

of the response string.  This causes the software to check for the

carrier before proceeding. (Carbon Copy version 5.0 software only)

NOTE for ERROR CORRECTING MODEMS

With these modems the error-free link symbol should be inserted in

the front of the response string followed by 5 of the .5 second

delays.  This allows enough synchronization time before the software

begins the login process.

If the modems correctly negotiate carrier (meaning white tones complete),

following which they immediately drop the call without having completed

the connection, then a special symbol "NO RTSOFF on Sync-up" should be

inserted in the front of the response string.  The modem may be going

back "on-hook" due to Carbon Copy software performing the sync-up pulls

the RTS lead to low immediately after the password is sent.  This symbol

is available for use with version 5.0 only; there is a patch which can be

performed against version 4.0 of the software.

ADD ACTION SYMBOL

-----------------

There are currently 21 action symbols which can appear as embedded characters

within the modem descriptors.  Some of the symbols are only relevant for

version 5.0 Carbon Copy software and later, meaning that they may cause other

software versions to perform incorrectly. The action symbol descriptions

follow in the order in which they appear on the sub-menu,

"EDIT MODEM DEFINITION ACTION SYMBOLS". Each of the following elements

are described with their related software behaviors.  The element's storage

within the string maps to a special symbol for use by the software and in

no way reflects the ascii character which the element may be named.

CARRIAGE RETURN -- this embeds a carriage return in the data stream.

.5 SECOND DELAY -- this causes the software to perform a half-second delay

during which data retention or discard is dependent on the

location in the code.  Delays performed during the

response string cause incoming data from the modem to be

ignored.

SEND BREAK -- This performs a 200 millisecond break to the data stream.

SEND ATTENTION -- this sends the data contained within the ATTENTION STRING

data field from the EDIT MODEM DEFINITON menu's data field.

This symbol is typically the first or second character in

RESET STR 1.

SET ERROR FREE LINE -- This informs the software that the Carbon Copy

proprietary protocol will not be used and no checksum

verifications are to occur on the data packets sent

during a connection.  This setting should only be used

when the modems or data-units linking the PCs are

"ERROR CORRECTING".  Error-correction is assumed to

be performed between the two connecting units which

would guarantee that the data passed to the receiving

PC is exactly what was sent from the transmitting

system.

This symbol should be inserted in the RESPONSE STRING

which corresponds to RELIABLE or PROTECTED result codes.

The symbol should be followed immediately by five .5

second delay symbols. This delay is necessary. If this

symbol is placed in the RESET STRING, then the software

will assume that every connection is "ERROR FREE" unless

the RESET ERROR FREE LINE symbol is placed in non-ERROR

FREE RESPONSE STRINGS.

RESET ERROR FREE LINE -- This informs the software that the Carbon Copy

proprietary protocol will be used to check for data

line errors. The default for both the Carbon Copy and

CCExpress software is not an error free line it

assumes error checking is necessary.

CONNECT SEQUENCE -- This instructs the software to send the Carbon Copy

proprietary connecting sequence. This is used only in the

RESPONSE STRING.

NO CONNECT SEQUENCE --  This instructs the software to inform the user that

the connection could not be made.  This is used in

the RESPONSE STRINGS where circumstances from the

modem indicate that carrier could not be successfully

made.

SELECT ALT. DIVISOR TABLE -- This instructs the software to use an alternative

UART clock divisor when performing data throughput

to the modem.  This is used in the beginning of

the RESET STRING before the attention symbol is

inserted.

SUBSTITUTE RING COUNT -- This instructs the software to use the RING COUNT set

by the CCINSTAL.EXE utility and replace any succeeding

digits with that count when setting up the modem.

*LINE FEED -- This instructs the software to send the ASCII LINE FEED character

in the data stream.  There is a similar symbol which can be

inserted by the CCINSTAL.EXE utility in the CALL TABLE's phone

number field.

*NO RTSOFF ON SYNC-UP -- This instructs the software to maintain the RTS lead

immediately following the completed carrier

negotiation.  The default behavior in the software is

to pull the lead LOW for .5 seconds following the

sending of the password or LOGON packet from the CC

side.  This symbol should be used only as the first

character in the RESPONSE STRING where applicable.

The RTS sync-up process was necessary for version

2.14 and 2.15 of Carbon Copy.

*WAIT FOR CARRIER DETECT --  This instructs the software to test the

CARRIER DETECT lead for HIGH, indicating that

carrier is present, then continue processing. If

carrier is not present, then WAIT FOR for CARRIER

DETECT lead to become high.  This is used in the

RESET STRINGs or RESPONSE STRINGs.  There is a

similar symbol which can be inserted by the

CCINSTAL.EXE utility in the CALL TABLE's phone

number field.

*SET BAUD 38,400 -- Change PC RS232 port speed to 38,400 bits per second

SET BAUD 19,200 -- Change PC RS232 port speed to 19,200 bits per second

SET BAUD   9600 -- Change PC RS232 port speed to 9600 bits per second

SET BAUD   4800 -- Change PC RS232 port speed to 4800 bits per second

SET BAUD   2400 -- Change PC RS232 port speed to 2400 bits per second

SET BAUD   1200 -- Change PC RS232 port speed to 1200 bits per second

SET BAUD    600 -- Change PC RS232 port speed to 600 bits per second

SET BAUD    300 -- Change PC RS232 port speed to 300 bits per second

*  These action symbols may only be in descriptors built for use with

Carbon Copy PLUS version 5.0 and later and CCExpress version later than

version 2.0.

SETUP REQUIREMENTS FOR BOTH HARDWARE SWITCH OR SOFTWARE PROGRAMMABLE CONTROL

----------------------------------------------------------------------------

DTR    Data Terminal Ready  should follow the DTE device (PC system)

&D2                         standard software setting

DCD    Carrier Detect       should follow true carrier from the telephone line

&C1                         standard software setting

DSR    Data Set Ready       should always be held high by the DCE device(modem)

&S0                         standard software setting (usualy a default)

RI     Ring Indicator 	    should come high on the incoming call ring and

remain low when carrier is high.  Some modems have

an optional setting.

X4     Extended Result      the extended result code set must be set to at

Code                 least X1 in order to allow the standard 2400 and

1200 baud modems to give baud rate related result

codes.  The preferred setting is  X4 which is

full "CALL PROGRESS MONITORING". The X1 and X4

settings detail the true connecting speed by

returning "CONNECT nnnn" where nnnn is the speed.

When the nonverbose setting is selected, then this

returning connection speeds must be digits

E0     Echo OFF             The local echo is turned OFF due to the length

of time the modem's typically require to return

the echo.  If this feature is ON, it tends to

cause many modems to lose characters, causing

an error in its command processing.  If the echo

feature is necessary in TERMINAL EMULATION use,

then the "DUPLEX" feature should be set to "HALF"

in the "SETUP CONFIGURATION" screen and/or the

"EMULATION ENTRY" screen within CCINSTAL.EXE's

CALL TABLE.

Q0     Quiet OFF            The QUIET mode refers to the call progress

monitoring and/or the return of the result codes.

It is critical for the result codes to be returned;

this is the way the Carbon Copy software

can determine the baud rate of the connection and

the protocol link type for error-correcting modems.

Full progress reporting, however, is desired.

V0     NONverbose           The NON verbose format is typically used.

The exceptions for which the verbose format is

used have been described above under the

"CONNECT STRING" definitions.

Registers

---------

S0=?1  Answer Ring Count    The question mark in answer ring count represents

a symbol character which is the "Substitute Ring

Count".  This symbol is always followed by a one.

If the ring count which is setup by the CCINSTAL

utility is "DEFAULT", then a ring count of  1

is used (no substitution is made).  Likewise, if

six is setup as the ring count, then the number

six will be substituted when the Carbon Copy

software sends the RESET STRing issuing this

command.

S7=255 Wait for Carrier     The Carbon Copy software beginning with version 5.0

After Dialing        has a "DIALING TIMEOUT" field in its configuration

(set by CCINSTAL.EXE). This command allows the

Carbon Copy software to use the configuration

setting for determining dialing time limit, after

which it pulls the DTR lead low (halts the

dialing process).   It is recommended to set the

S7 register value to the modem's maximum timeout

limit.

S10=40 Delay before         This limit is the maximum time the modem can

Hanging-Up on Loss   stay "ON-Line" when the carrier has been lost or

of Carrier           interrupted.  If one has "CALL WAITING" on the

phone line or telephone line noise, then carrier

can be interrupted.  Some Error-Correcting modems

have Error-correction problems with a timeout

exceeding 3.5 seconds.  For standard modems the

4 second timeout is sufficent.

There are standard settings which we have chosen with error correcting modems

for use within the modem descriptors.

1) Chose Verbose Extended Result Codes where a modem manufacturer's modems

can return more than seven speed differences and a BUSY result code;

otherwise, chose Numeric Extended Result Codes.

2) Select a setting for fixed DTE baud rate.  This allows the modem

to perform buffering and data compression to its maximum efficiency.

3) MNP flow control must use hardware bidirectional flow control.

Carbon Copy uses bidirectional flow control in all communications.

The XON/XOFF logic within modems must be set to PASS-THROUGH logic

in order that the software receive these characters in both file

transfer and remote communications.

4) Use AUTO-RELIABLE link negotiation to allow the modem to connect

to both the error correcting modems and standard compatibles.

Setting a modem for either NORMAL or RELIABLE connections,

restricts connections to one kind of connection or the other.

The circumstances where this restriction should be performed would be

where the carrier negotiations can not be completed with auto-reliable

answertones.  Two modem descriptors should be setup for the end user.

There should be a descriptor providing access to the error-correcting

modems using the same protocol with a RELIABLE descriptor, and a

descriptor for use with those modems using NORMAL carrier negotiations.

The limitations are that the operator must know that the MODEM TYPE at

both the calling end and answering end is setup compatibly by the

CCINSTAL utility.

5) Hayes V-Series can not operate in an error-free link, the result codes

returned for 2400 and 1200 settings have embedded carriage returns.

RESULT CODES

------------

Hayes  Link Type      MNP Compatible   Baud Rate   Link Type

-----  ---------      --------------   ---------   ---------

14     PROTECTED      33               19.2        RELIABLE

12     PROTECTED      33               9600        RELIABLE

?               7200        RELIABLE

11     PROTECTED      30               4800        RELIABLE

28               4000        RELIABLE

27               3200        RELIABLE

10     PROT/UnPROT    23               2400        RELIABLE

5     PROT/UNPROT    22               1200        RELIABLE

1     PROT/UnPROT    20                300        RELIABLE

10     UnPROTECTED    10               2400        NonRELIABLE

9     UnPROTECTED     9                600        NonRELIABLE

5     UnPROTECTED     5               1200        NonRELIABLE

1     UnPROTECTED     1                300        NonRELIABLE

other result codes

7     BUSY            7

6     No DIALTONE     6

4     ERROR           4

3     No CARRIER      3

As you can see by the chart, Hayes does not report protected and unprotected

links by a single result code.  They have an extended reporting mode which

allows several codes to be returned identifying the LAP protocol chosen as

well as the speed of the connection. However, this reporting mode has

embedded carriage returns which the Carbon Copy code accepts the first code

returned.  This would not pose a problem if the Hayes V-Series had provided

for a command to set a fixed DTE rate.

@DATA_SWITCH DESCRIPTIONS@

There are several considerations for DATA-SWITCHES which direct Carbon Copy

performance. When placing calls in a switch the following behaviors may be

observed:

CCHELP CALLING CC

a) Switch requires an extension once connection is established, the

extension number should be placed in the password field of the call

table entry or the call can not be completely automated, this allows

the extension to be reached. Carbon Copy will be waiting for a password

entry. The password prompt screen stating "SENDING PASSWORD TO CC" will

appear; the user should enter his password at this time. If garbage

characters appear in the remote operator's dialogue fox of the "Chat

Screen", meaning illegible dialogue, then the response strings on

the switches side are incorrect. Either more delays are required in the

response strings or the baud rate should change to reflect the correct

speed.

CC CALLING CCHELP

b) Possible with version 5.0 of the software. If an extension is required,

then the CC system, from within the CCINSTAL utility can use the phone

number field to contact the CCHELP system. There is one stipulation,

the speed of the CC modem must be exactly the anticipated speed on

the CCHELP side. The CC side will not receive the baud rate connect

message until after the carrier negotiation has been completed. Within

the phone number field of the call table entry, the phone number should

be placed first, followed by the "CARRIER DETECT" symbol. Once the

phone number is dialed through the modem, the software will pause until

the carrier has been established with both modems having generated

connection result codes which the CC side will discard at this point

in the code. Since the CC side will still be performing the dialing

process it will use the result code and it will not be searching for a

match against the connect strings now, follolwing the CARRIER DETECT

symbol should be the extension number which the CCHELP call table's

phone number field should contain next. The modem descriptor's connect

strings are not being checked at this point in the software.