WATCHDCD

Version 0.1

by Gary Chanson

(C) 1994

WATCHDCD   is  Copyright  1994   by  Gary  Chanson  and

Starquest  Systems.  No restrictions  are placed on the

non-commercial  use of this  program.  This program and

its  documentation  may  be  copied  and  redistributed

provided  that it is not changed  and no fee is charged

for  it.   An  exception  is  granted to not-for-profit

user's groups, which are authorized to charge a nominal

fee  for materials and handling.  Starquest Systems and

Gary  Chanson assume no liability for damages caused by

the use of these programs.

description:

WATCHDCD  is a DESQview  utility which monitors  the serial port

status  of  the  specified  COM  port and restarts the specified

DESQview  task  if  a  disconnect  is  detected.   Disconnect is

assumed if DCD drops and stays low for the specified delay time.

During  operation,  WATCHDCD  looks  for  messages  sent  to its

mailbox   (named  "WATCH:<name>"  where  <name>  is  the  string

specified  as the mailbox parameter passed to WATCHDCD).  If the

message   "STOP"  is   received,  WATCHDCD   terminates  without

affecting  the  target  program.   If  "CLEAR"  is received, any

pending  restart is  canceled and  WATCHDCD's state  is reset to

off-line.

If  a log  file is  specified with  the -L option, all character

output  is  sent  to  the  specified  file  (default log file is

WATCHDCD.LOG).

usage:

WATCHDCD port mailbox dvp-file [-options]

port     - serial port as COMx or a port address

mailbox  - name assigned to the mailbox of the BBS node

dvp-file - task to start if triggered

options:

-Dn1        reboot Delay in ms (65,535 max)

-L[file]    Send output to Log file

-Q          Quiet mode

messages:

"CLEAR"     cancel pending restart and reset to off-line

"STOP"      terminate

error levels returned:

0           Normal exit

1           Unrecognized option

2           Specified mailbox not found

3           Not running under DESQview

4           Task terminated and restarted successfully

5           Task terminated and restart failed

-----------------------------------------------------------------------

Please forward all comments and bug reports to:

Gary Chanson

31 Chester Avenue

Waltham, MA  02154

Voice:  617-899-4771

Email:  Gary.Chanson@Channel1.com

-----------------------------------------------------------------------

For on-line support, call our BBS!

Arcane Incantations

617-899-6672

-----------------------------------------------------------------------