CoreView(tm)

A Window Into Your PC Memory

Version 1.0

Copyright (c) 1989-1991 RolySoft, Denmark

What is CoreView?

-----------------

CoreView is a small but fast memory  viewer.  With help of CoreView you

may  look through  the  memory of  your  computer.   CoreView also  has

facilities  for   searching  memory  for   specific  strings   or  BIOS

extensions.

CoreView is  particular a great  tool if you  are going to  install new

hardware in your computer.  Often you have to set jumbers on the add-on

card to install its BIOS into the right address space.  But WHAT is the

right address space? With CoreView you  may browse through ROM at C000h

thru F000h and look for empty slots.

CoreView is also  capable of viewing memory  in real time.  What  do we

mean with  that? You  may enable  a function  we call  "timed display".

While this function is on, CoreView continously updates the screen with

data from memory.  If  any bits change, you will see  it immediately on

the screen.

Q quick tour through the CoreView functions

-------------------------------------------

CoreView has many functions.  As all functions are easy to use, we will

only use little  space to explain them.   You may call the  help screen

inside  CoreView by  hitting Alt-H  from  the main  screen.  This  help

screen summarises most of the functions.

First of all  you may scroll through  memory.  To do this,  you use the

arrow keys, Home, End etc.  All PCs  are capable of addressing at least

1 Mb of memory.  With CoreView you may inspect this 1 Mb address space.

By using the arrow  keys, PgUp, PgDn, Home, End and  many control keys,

you may browse through  this 1 Mb address space.  At  the bottom of the

PC  memory there  is  always  RAM installed.   When  you  go to  higher

addresses,  the PC  has  ROM installed.   RAM is  memory  which may  be

changed by the CPU.  ROM is Read Only memory, which means it may NOT be

changed.  If CoreView  encounters a ROM area,  you will see a  star (*)

right after the  address.  This star tells you, that  you are currently

looking at ROM locations.

If you are using a colour monitor, you may have noticed that some bytes

are high-lighted while others aren't.  All standard figures and letters

are shown in a  bright colour by CoreView to make it  easier for you to

recognize human-readable strings.

If you  want to  search for a  specific string, you  may do  that.  Hit

Alt-S to enter the  search input screen.  Enter the string  you want to

search for.   In the upper  input window  you may enter  all characters

between 20h  and FFh.  If  you also need to  search any of  the control

characters, you should  hit the Tab key once, and  enter the characters

as two-digit hex numbers.  Hit Enter to  begin the search or hit Esc to

abort.

You may choose  to search for an exact  match, or you may  search for a

match, which is case-insensitive, eg.  upper- and lowercase letters are

treated equal.  Use Alt-C in the main screen to change the search case.

If CoreView finds your search string, and you want to repeat the search

for the next copy, then hit Alt-R to repeat.

CoreView displays  a search message  in the  lower right corner  of the

screen, while it's searching.  The  actual search is NOT interruptable.

On 386s a 1 Mb search takes  approximately 2 seconds.  On XTs and other

relatively slow  computers, the search may  take 15 seconds or  more to

finish.

If you want to jump to a total different address, you may enter it into

the address window.   When you hit Enter,  CoreView is set to  this new

address.  You may also pick up the  current address you are looking at,

and put it into the input box.  Hit Ctrl-Enter to read the address from

the current  window position.  If you  want to normalize  this address,

just  hit  Alt-N.   What  do  we  mean  with  'normalized  address'?  A

normalized address,  is an address  with an  offset part always  in the

range 0000  thru 000Fh.

CoreView provides you with two functions to specify how data are shown.

If you press Alt-W memory is displayed in wide format.  This allows you

to see more  data on a single  screen.  To switch back  to the standard

hex display,  hit Alt-W  once more.  Alt-A  acts as  a filter.   If you

press Alt-A (ASCII), only ASCII  characters are displayed.  This filter

is useful  when you are  looking into text  strings and don't  want the

code displayed.  Most code contains opcodes outside the ASCII range.

A final word...

---------------

The CoreView program and its documentation have been donated the public

domain.

Please report  any problems to ROLAND  LYNGVIG at the  Josti-BBS, phone

+45 47 38 05 24 (DANE TECH 2:230/31 or SDNet/Work: #551.13).

Thank you for using CoreView...

RolySoft, Denmark