CORELOOK.EXE

PC Memory Scan and Search

This program is a simple tool for viewing the contents of the

PC's memory.  It will display any memory address within the 1024 K

bytes that are addressable in the Intel segmented addressing scheme.

Thus includes the 640 K bytes of user memory and the 360 K bytes

defined for system and hardware support functions.

This program also has a string search function that will find

the memory locations that contain a string of up to five characters

that you define.

When CORELOOK is loaded, a screen frame appears that includes

the viewing "window" into the memory contents as well as the

address/string input fields and all required help information.

MEMORY CONTENTS DISPLAY

CORELOOK displays the Hex values of 256 bytes of memory in the

viewing "window", arranged in 16 rows of 16 bytes, similar to DOS's

DEBUG utility.  Each row is preceded by the 16-bit segment and

offset components of the 20-bit segmented address of the first byte

in the row.  At the end of each row, the ASCII representations of

the 16 bytes are displayed.  Unprintable low-ASCII and extended

characters are shown as blank spaces, much as DEBUG displays

periods.

When CORELOOK first starts, the viewing window displays the

memory beginning at address 0000:0000.  You can then change to other

addresses by using the several scanning options, as follows.

>0000:0000<  ENTER BEGINNING DISPLAY ADDRESS

This option provides an input field to specify the segment

and offset of the beginning of the desired block of memory.

The cursor starts under the leftmost position: enter valid

hex values (0-9 and A-F) across the field, skipping over

unchanged positions with the <Right> cursor.  The Backspace

and <Left> cursor do not work, but entry will wrap around

to the beginning if you need to correct a value.  At any

place in the field, pressing Enter will put the indicated

memory block in the viewing window.

F7 - BACK PAGE

Changes the contents of the viewing window to begin at an

address 256 bytes lower than the current beginning address.

F8 - FORWARD PAGE

Changes the contents of the viewing window to begin at an

address 256 bytes greater than the current beginning

address.

F9 - BACK LINE

Changes the contents of the viewing window to begin at an

address 16 bytes lower than the current beginning address.

F10 - FORWARD LINE

Changes the contents of the viewing window to begin at an

address 16 bytes greater than the current beginning

address.

When these function key commands are used to scroll the viewing

window, the address in the BEGINNING DISPLAY ADDRESS input field is

updated automatically, and will always be consistent with the first

memory location displayed in the viewing window.

The viewing window scroll functions also have a wrap capability

at the ends of the addressable memory.  If the viewing window

beginning address is 0000:0000 and you use F7 to move back one page,

the new viewing window will begin at the address of FFF0:0000.

STRING SEARCHING

CORELOOK provides a string search feature that will find memory

locations that hold byte values matching a five character ASCII

string.  The match must be exact, including leading and trailing

spaces that you have in the input field.

Alt F - ENTER FIND STRING

Press ALT F to access the string search function.  The

cursor will move to the input field where you type the five

characters to be matched.  If the search string is less

than five characters, you need to enter leading and

trailing spaces exactly as you expect them to be located.

This may take a little experimentation if you are not sure

how the search string is delimited in its memory storage.

When the input field is to your liking, press <Enter> to

begin the search.

F5 - REPEAT FIND

This will search for the next occurrence of the string

currently displayed in the input field.  Memory addresses

are searched in ascending order.

PROGRAM TERMINATION

Pressing <ESC> at any time will terminate CORELOOK and take you

back to DOS.

Rev 0 (docs only)

TAM 9/3/88