VGA Convergence Check Ver. 3.2

----------------------------------------------------------------------

VGACC is a test-picture which will help you to evaluate the

convergence of your VGA monitor. Convergence determines how

accurately the 3 electron beams (red, green, blue) are are aligned.

Misaligned electron beams result in fuzzy images and/or colored

"shadows" around white images.

The color white consists of all three beams fired up simultaneously,

so white is the best color to use when you check the convergence of

your monitor.

Convergence errors usually get more pronounced in the corners of the

screen, so you should pay extra attention there.

For a real "acid" test, turn on high intensity white (upper case W) and

check the lines up close. Any colored shades? There shouldn't be.

Also check the text screen (T) for sharpness.

The color bar is composed of 8 low intensity colors with high

intensity borders, and 8 high intensity colors with low intensity

borders. Borders are 1 pixel. There is 1 pixel of black between

adjacent colors. Can you distinguish the border on all sides of

all colors? How about the brown border around high intensity yellow?

Move the color bar and text box around to check the various parts of

your screen. Use the center circle to make sure that the aspect ratio

of the screen is correct so the circle looks like a circle, not an

ellipse. Make sure the picture is stable and has no line movements

like "jitter" (<� sek), "swim" (�10 sek) or "drift" (>60 sek). Also use

VGACC to check the focus of each beam by turning on red, green and blue

respectively.

ACTION                       KEY

------------------------     ---------------------

Low intensity grid colors... Lower case b,g,c etc.

High intensity grid colors.. Upper case B,G,C etc.

Move text................... Arrow keys

Move color bar.............. Shift-arrow keys

Flip color bar.............. F

Switch to text screen....... T

Coarser grid................ - (minus)

Finer grid.................. + (plus)

Reset colors and grid....... *

End......................... Esc

Finally: The quality of monitors vary wildly from unit to unit.

Insist on trying several monitors of the same type and choose only

the best. There are a LOT of garbage monitors out there, even among

the megabuck models. Few importers/dealers bother to realign their

monitors after shipping. Don't let them get away with it!

Informatic Software

Tom Sivesind

Svenskerud 15

3408 Tranby

Norway

Tel: +473-852200

Fax: +473-852828