Notes on upgrading Color/Graphics card to 64K RAMs      20 JAN 83

Features:

-- compatible with all software written for IBM 16K board

-- allows 640 x 700 (interlaced) or 640 x 350 (non-interlaced)

graphics on IBM monchrome monitor

-- allows 640 x 400 interlaced color display on TV

-- allows 44 line character mode display on monochrome monitor

Warnings:

Removing the 16K RAMs which come in the card is NOT easy, as

the leads are clinched over before soldering and the pads are

extremely delicate.  Be careful!  Also be prepared to rewire any

traces or pads you might damage.

I suggest that you have a copy of the IBM technical reference

manual so that you can understand what is being changed and fix

it if it goes wrong.

Although the upgrade is compatible, it is not supported by IBM,

so you must write your own driver for your application.  I'll

maintain a library of routines written for the upgrade and

submitted to me for distribution.  (Send me a floppy and return

postage for its mailer and I'll add your routine to the library

and return a copy of the current library to you.)

IBM will likely use the graphics memory space in a different

way (but you'll have a jump on high resolution software).

To use the upgrade, you must reprogram the 6845 registers.  If

done improperly this can cause a monitor to burn up.  Be extremely

careful!

The upgrade works on Color/Graphics adapters with schematics

approximately as shown in the August 1981 edition of the Technical

Reference manual.  If the U numbers on your board don't seem to

match the IC types given, STOP! and let me know so I can update

this writeup.  Note that the XT Technical Reference manual correctly

shows bit 3 of the Status Register (3DA) as Vertical Sync (very

useful for synchronizing display updates).

Upgrade Procedure:

-- Carefully remove the 8 16K RAMs (2118s U50 thru U57) and

replace with 16 pin sockets.

-- Connect pin 9 of U50 thru U57 and then to pin 19 of U58, U59,

U60 and U61.

-- Connect pin 18 of U58 (LS374) to pin 17 of U38 (6845 MA13).

-- Connect pin 18 of U59 (LS374) to U15 (LS00) pin 6.

-- Connect pin 18 of U60 (LS374) to P5 pin A15 (+A16).

-- Connect pin 18 of U61 (LS374) to P5 pin A17 (+A14).

-- Connect pin 5 of U15 to U38 pin 37 (RA1).

-- Connect pin 4 of U15 to pin 12 of U15 (640x200 mode).

-- Cut connection to pin 2 of U19 (+A16).

-- Connect pin 2 of U19 to pin 16 of U19 (+5V).

-- Install 150 ns 2164s in sockets U50 thru U57.

Now the Color/Graphics memory will look as follows:

Character modes:

B8000 to BFFFF:  16K character/attribute pairs

(can scroll by setting R12 and R13 of 6845)

A8000 to AFFFF:  32K general purpose buffer memory.

Graphics modes:

B8000 to B9FFF and BC000 to BDFFF:  lines with RA=0

BA000 to BBFFF and BE000 to BFFFF:  lines with RA=1

A8000 to A9FFF and AC000 to ADFFF:  lines with RA=2

AA000 to ABFFF and AE000 to AFFFF:  lines with RA=3

To use extended resolution modes, set 6845 R12 to 10H.  (This

sets MA12 high so that MA13 goes high at the proper point.)

To use RA=2 and 3 (for 640 x 700 mode) set 6845 R9 (Max Scan Line)

to 3.

Connection to IBM monochrome display:

-- Connect a 22 ohm resistor from the emmitter of Q1 (2N3904 -

composite video) to pin 7 of the 9 pin D connector (video out)

-- Install a subminiature DPDT switch thru the back of the RFI

shield.  (To allow switching back to a normal monitor.)

-- Cut trace to U67 pin 13 (LS244 - VSYNC) and make this connection

thru the switch instead (U67 pin 13 to pole A).

-- Connect U65 pin 13 (LS02) to the other throw of pole A.

-- Cut trace to U67 pin 11 (HSYNC) and make this connection thru

the switch instead (U67 pin 11 to pole B).

-- Connect U65 pin 6 (+HSYNCDLY) to the other throw of pole B.

-- Connect U65 pin 9 to U65 pins 11 and 12 (+VSYNCDLY).

This provides the proper (inverted) vertical sync and non-serrated

horizontal sync to the IBM monitor.

Sample 6845 register settings are given in the BASIC program:

GRPH64K.BAS

Hal Sampson

501-D Vandell Way

Campbell, CA  95008