���������������������������͸

� On The Topic of 450 Baud! �

���������������������������;

by Scott Loftesness

May  28, 1983

Many users have asked about using 450 baud with this

RBBS system.  Typically, they are asking why such an option

exists (the N option on the main RBBS menu) when their modem and

IBM-PC BASIC don't appear to support such a speed.

They are correct about IBM-PC BASIC not supporting 450

baud. In fact, the ROM BIOS routines in the PC don't support 450

baud as an option either.  It's not that IBM or Microsoft skipped

the speed when they developed the PC, but the fact that 450 baud

is not a standard communications speed for the RS-232-C

interface.  The next higher 'standard' speed above 300 baud is

600 baud, followed by 1200 baud, etc.  As a result, 450 baud is

not supported on most personal computer systems.

As it turns out, however, most 300 baud modems will run

at somewhat higher speeds - up to, guess what!, 450 baud in most

cases. Although a somewhat higher error rate can be expected when

you "push" a 300 baud modem to 450 baud, the error rate is

typically still low enough to not be any kind of serious problem

to file transfer and message passing operations.  In fact, using

the XMODEM file transfer protocol, errors which do occur are

normally detected by the check sum error detection scheme used by

XMODEM and simply force a re-transmission of the block originally

received in error.

Since the speed advantage of 450 over 300 baud can be

substantial, and since we are all interested in doing the most

work in the least amount of time with the equipment at hand, many

users have implemented 450 baud as an option which can used with

their "home-grown" communications programs.  (Note that standard

IBM-PC communications packages don't support 450 baud either -

such as PC-Talk III). I'll describe how 450 baud can be

implemented from a programming standpoint shortly. But first,

let's get rid of a few erroneous impressions about modem

operation.

Your Hayes 300 baud SmartModem (or a Hayes 1200 running

in 300 baud mode) is capable of receiving modem commands and data

at up to 1200 baud.  What this means is that the microprocessor

in the Hayes is programmed to automatically look for the AT

command sequence - and performs automatic baud rate detection at

the speed you happen to be sending the command sequence to the

modem. This capability, for example, is used in the RBBS code to

send the various modem commands at 1200 baud - to minimize the

time spent sending them!

Your Hayes 300 will even try to send data across the

line at 1200 baud - if you open your communications line at that

speed and attempt to go to data mode. However, the 300 is not

capable of handling data at that speed - in fact, it can only

transmit and receive data at up to slightly more than 450 baud -

with any reasonable error rate. Since the Hayes doesn't care at

what rate is receives data (anything from 1 to 1200 baud is

acceptable), it is possible to transmit data to the modem and have

the modem transmit it to the phone line at any of those speeds.

However, 450 baud is the reasonable upper limit for data

communications with the Hayes 300.

Now that we understand the workings of the Hayes modem,

let's talk about how we can get the PC to send data from its

RS-232-C Asynchronous Communications Adapter at 450 baud. As

mentioned earlier, 450 is not an acceptable option to either the

BASIC OPEN statement or to the direct BIOS calls for asychronous

communications. However, as always, almost *anything* is possible

using software! The IBM ACA uses an 8250 chip which is capable of

running at speeds from 1 to 19,600 baud (officially, only up to

9600 baud, but that's another discussion!). The 8250 includes an

on-chip software programmable baud rate generator - which

actually determines the speed used to send and receive data

across the RS-232-C interface. This baud rate generator can be

programmed to run at *any* baud rate between 1 and 19,600 baud.

The IBM-PC ROM BIOS routines, however, were programmed to only

support the "standard" RS-232-C speeds. Since BASIC uses the BIOS

routines, BASIC only supports those same "standard" speeds.

However, it is possible for you to program the 8250 to

run at any speed you like. In this case, we are only interested

in programming it to run at 450 baud. Programming the baud rate

generator in the 8250 consists of loading the baud rate generator

on the chip with a "baud rate divisor" - which takes the clock

frequency and divides it by the number you provide to come up

with the baud rate itself. (Those of you with Technical Reference

manuals for the PC can read all about this on pages 2-135 to

2-137). The divisor is a two byte value - which is stored into

two one byte registers within the 8250 chip. First, we need to

determine what the baud rate divisor should be for 450 baud

operation. Take it from me that the value is X'0100'.  Next, we

need to get this value from a BASIC program into the 8250 itself.

Fortunately, BASIC includes the OUT statement - which allows us

to send data to the various I/O ports on the system. The 8250

uses several of these ports - for the baud rate divisior, modem

control, and for actually sending and receiving the data across

the RS-232-C interface.

The baud rate divisor ports are X'3F8' for the least

significant byte and X'3F9' for the most significant byte - when

using the COM1 port. (COM2 addresses are X'2F8' and X'2F9'

respectively. Before we can load these divisor ports, however, we

must load another value to another port to indicate that we are

setting the baud rate divisor. This third port is the Line

Control Register and is port X'3FB' (X'2FB' for COM2). The high

order bit (X'80') of the LCR controls access to the baud rate

divisor registers. When this bit is 1, we can load the baud rate

divisors. When it is 0, the 8250 resumes operation with the baud

rate loaded. The following sequence of BASIC statements will load

the baud rate divisor registers:

OUT &H3FB, (INP(&H3FB) AND &H80) 'Enable divisor registers.

OUT &H3F8,0 'Low order byte of X'0100'

OUT &H3F9,1 'High order byte of X'0100'

OUT &H3FB,(INP(&H3FB) OR &H80) 'Disable divisor registers.

Adding this code to a BASIC program will cause the COM1

port to immediately switch to 450 baud operation.

Switching to other speeds is just as easy. The only

thing you need to know is the baud rate divisor required for that

speed. Here are a few of the common divisors (see the full table

in the Tech. Ref.):

300     X'0180'

450     X'0100'

1200    X'0060'

I hope this discussion has helped to clarify the

subject of 450 baud operation. Perhaps with this help, you can

make some simple changes to whatever communications program

you're running which will allow you to use your Hayes 300 at a

speed which will allow up to 50 per cent faster communications!

####################################################################################################################