REAL TIME CLOCK ROUTINES

========================

The  files  CLOCKSET.COM,  CLOCKGET.COM and SETCLOCK.BAT were  developed  to

overcome  some of the limitations of the Real Time Clock(RTC) programs  that

utilize the MM58167 RTC chip.   This chip unfortunately does not have a YEAR

counter  and does not correct for leap years (i.e.  the 29th of Feb does not

exist).   The above programs allow the RTC to be set from the system  clock,

it automatically rolls over the year counter (the program TIMER has this bug

in  it)  and it automatically adjusts for leap years.   A feature  of  these

programs  is that they automatically search all the commonly used  addresses

of the RTC until one is found.   e.g.  These programs work with the  Persyst

Time Spectum card and most of the cheap RTC cards available.

SETTING THE REAL-TIME CLOCK

---------------------------

Enter SETCLOCK and press RETURN.  You will be asked to set the systems date

and  then time.   The CLOCKSET.COM program will automatically set your  Real

Time Clock (RTC) - if you have one!

SETTING THE SYSTEM TIME/DATE AUTOMATICALLY

------------------------------------------

You may already have an AUTOEXEC.BAT file on your System disk, if not create

one using the COPY command or Editor (i.e. EDLIN).  Insert the word CLOCKGET

into the file, this will automatically set the systems time/date to the Real

Time  Clocks when you start-up the system.   If you already have an AUTOEXEC

file then delete DATE and TIME and insert CLOCKGET.  You will no longer need

to set the time and date AND it will always be correct when you power up.

SPECIAL NOTE

------------

The  RTC  may  need to be re-adjusted again for DAY-LIGHT  SAVING  time  and

occationally  when  it is corrupted if the system crashes.   Simply run  the

SETCLOCK Batch(BAT) program again to reset it.   If your time and date  from

the RTC is corrupted often, try changing the cards address.

SYSTEM FILES

============

The   CLOCK.SYS  device  driver  is  provided  to  allow  the  DATE  to   be

incremented   when  the  TIME clocks over to 00:00:00.00.    This   is  done

already in DOS version 3 but not in version 2.    It is also useful  in  DOS

version 3 because some  programs  (i.e.   on-screen clocks  etc) may 'steal'

the timer flag indicating that the clock has passed 24 hours.   Also,  since

this driver monitors the system timer  continually,   date increments cannot

be  lost when the time is  not  read for 24 hours.    To install the  driver

simply   copy CLOCK.SYS  to  your boot disk and include DEVICE =   CLOCK.SYS

in your CONFIG.SYS file.  If one does not exist then copy the one on

this disk to your boot disk.

(C) COPYRIGHT 1988, PROCON SOFTWARE, P.O. BOX 43, ESSENDON, AUSTRALIA.

#########################################################################################################################