ABOUTPC.DOC

This section has been written in response to requests by many users who just

don't quite see how to get started.  It's OK to be confused.  We originally

wrote this program for printed circuit experts who were also quite familiar

with programming and IBM DOS.   Most of our first users liked the idea of a

new cave to explore, and took it as a challenge to figure out how to do

everything.

We would now like to be a little more friendly.  So this section will tell you

something about how printed circuits are made, then LEARNSYM.DOC will lead you

step by step through a simple circuit.

When I first started making printed circuit boards in a large government

laboratory, we used paint pencils and fresh copper clad board.  It was awful.

With great care, lines could be made 1/8" wide, but they always had bad spots.

Then again, almost anything would work with tube circuits, and parts were so

large that the wide traces were not much of a problem.  When the industry

started using photographic techniques, things got better fast.   The process

was to make a drawing of the desired circuit at a large scale, reduce it to

the correct size by photography, and use it to expose a photoresist film on a

copper clad board.  The layout was made on large sheets of poster board, later

grided drawing paper, finally ghost gird polyester.  We used strips of black

drafting tape and pre-cut doughnut shapes for the pads.  Much later we could

get whole shapes, like multi-pin ic packages as a single stick on unit.

For a while we dipped or sprayed on the photoresist.  It was really hard to

get a uniform coat and much material was wasted.  Photoresist is a material

that hardens under exposure to ultra-violet light.  By using a negative of the

desired circuit, the clear lines caused the photoresist to harden where the

board traces were to go.  Next a solvent was used to strip off the unhardened

resist.  Now the board could be dipped in a solution of ferric chloride which

disolves the copper where it is exposed.  Pads were made with an open spot in

the center which was later used as a center spot for a drill.

Things have progressed a lot since the early days.  We will attempt to

describe current small shop practice, realizing that there are simpler and

more exotic techniques in use.

We assume that you send your printed circuit vendor 1 to 1 positive art work.

That is, the film is black where you want copper.  We further assume that you

are requesting a double sided plated through board, i.e., traces on both sides

of the board, with holes through the board that have plating inside them to

connect the two sides of the board.  If you send the vendor something

different, say a 4 to 1 taped master, he will first photographically reduce

the art work.  Not long ago the vendors insisted on "right reading, emulsion

side down" film.  I find that today they are not so fussy and will figure out

how to do your job given any reasonable chance.  When film is exposed with

light from a photoplotter and then developed, the exposed area shows up as a

blackened area on the emulsion side of the film which is the up facing side

when plotted.  "Right reading emulsion side down" means that when the film is

placed emulsion side down on the surface of the final board that any legend

will be readable, and the circuit will be as you want it.  Note that if you

use a CAD system like ours (or most other systems) that view all layers from

the same side, then when the film is plotted, one side will need to be

mirrored in order for the emulsion side of the film to be on the surface of

the board.  Further, writing on the side that is not mirrored will need to be

placed on the job mirrored.  Confused?  We will go through this again with an

example.

Suppose we follow our standard and make a two sided board where we define

level one to be the parts side, and level two to be the solder or wire side.

These are old names, these days boards are made that have traces on both sides

and even in the middle and can have parts on both sides of the board.  The

names carry over from single sided board days.  When we lay out the board, we

look at it from the parts side.  If we visualize the traces, with the final

film emulsion side down on the board, we note that we are looking at the

emulsion side of level two, the bottom side of the board.  Thus the plotter

should plot level two just the way we look at it when we lay it out.  Note

however, that writing on level two must be mirrored in x if it is to appear

correctly when we look at it from the top (the only way we can) when it is

laying against the printed circuit board in its final position.

Level one, the parts side, is just the opposite.  It is created with normal

writing.  Since film is plotted with the film emulsion face up, it must be

plotted mirrored in x.  Thus, the writing comes out correctly when the

emulsion side of the film is placed face down against the board.

Any hand writing that you place on the art work to guide the vendor should

follow these conventions or confusion will result.  Or follow your vendors

guide lines.  Most vendors have unintentionally made many boards backwards

from their customer's desire, so they are quite specific as to what they want.

The vendor will likely follow your writing, even if the emulsion is on the

wrong side.  Except for very fine line high precision work, the emulsion side

makes little difference.

The vendor will normally make a 1 to 1 contact print of your film or the film

reduction he makes from your art work.  This will then be inspected carefully.

At this point you get phone calls from the vendor when lines are found that go

nowhere or if lines almost touch something.  Most vendors expect to make small

corrections.  Small line breaks caused by hair line faults in the photography

will normally be fixed without question or additional cost.

For plated through holes, the vendor expects to get art work with solid pads.

That is, there should be no holes in the doughnut.  So how does the vendor

figure out where to drill the holes?  He expects another piece of film called

a drill schedule.  This is a precision film, just like the art work layers,

with an appropriate mark and legend where each hole is to be drilled.

Normally most holes are the same size so that it is appropriate to mark the

film "drill all holes 0.031 except A=0.125, B=0.100."  Penned  circles or

triangles, etc. around the exceptions with a legend will be good enough.  If

you follow our recommendation and make your pads as symbols, then you can draw

suitable marks on the drill schedule layer to show the exceptionally sized

holes.  If you do not send a drill schedule, most vendors will make one from

your layout.  Problems arise when you do not put an obvious pad at every

location you expect to be drilled.  Good practice these days is to put a pad

on every layer, even if there is no connection.  If you have some pads on one

side, some on the other, and some on both sides and don't send a drill

schedule, don't expect the vendor to figure out what you want.  There is

little chance that he will do the correct thing.

Now comes the ludicrous part.  You have used a computer to generate the art

work.  The coordinates of every drill hole are known to the computer to great

precision.  All this information is thrown away and the vendor remeasures your

film to determine the drill hole location.  Some day we will all get together

on standards.  A lot of work is done direct machine to machine, but if you are

a small guy, today in 1988, trying to get a dozen boards made by the lowest

cost vendor, this is the most likely way your boards will be done.  The vendor

also needs a final board outline.  A good place to put this is on the drill

drawing.  The same operator will trace the board outline for use in a final

routing operation which will cut the board to size.

The vendor makes a copy of your drill schedule or uses your layout if you

don't send one.  The vendor prepares a layout where a continuous line is drawn

with a colored pen through all the holes of a single size.  A second color is

used for the next size and the process is continued until all the holes are

marked.  Locations for alignment holes are also marked at this time.  Next the

artwork is placed on a motor driven table which is controlled by a joy stick

with a TV camera looking down on the work.  The operator guides the work under

the camera with the joy stick and hits the trigger each time a drill spot is

centered on the cross hair.  When he hits the trigger, a paper tape record of

the coordinate and hole size is punched.  Paper tape you say?  In 1988?  Well,

possibly it goes to floppy disk in more modern plants.

Now a suitable material size is selected for your job.  This is most commonly

1/16" epoxy fiber glass laminate, commonly called G-10 or some other similar

designation.  It will likely be 1 oz. copper clad on both sides.  This means

that there is one ounce of copper per square foot of board area on each side

of the board.  This is roughly 0.001" thick.  If the board size is small, the

vendor may elect to place several copies on a single work piece, or he may

combine several small jobs.  If there are a lot of boards, the boards may be

stacked several deep for some operations, like drilling and routing.

The drill tape is now used to control a drilling machine.  These machines can

change drills automatically so they have no problem with a variety of drill

sizes.  They also keep track of drill wear and change drills as needed to make

clean holes.  The boards are now removed from the drill machine and some

complicated cleaning operations take place.  The holes have to be clean, and

their surface prepared in just the right way if the plated through hole

operation is to be successful.

The board is now put in a special plating tank.  This tank contains a contact

plate which attaches one molecule thick to any exposed surface.  When this

process is complete, there is just enough of a conduction path through each

hole to allow standard electro-plating.  There may be some additional plating

to assure that the plated through hole is strongly bonded at this time.

Now the boards have a special plastic film laminated on each side of the

board.  This film is again about 0.001" thick and comes in large rolls of

various widths.  It is laminated to the boards between heated pressure rollers

so that the final product is very tightly bonded to the board.  Film positives

of the circuit (dark where there are to be circuit traces) are now aligned to

the copper board.  Some vendors do this by eye, others use alignment holes

punched to match the board alignment holes.  The board is exposed to ultra-

violet light for a few minutes.  The plastic is hardened where it is exposed

to the light, and stays soft where it is shielded by the dark conductor paths.

The boards are now "developed".  This means that they are put in a washing

machine which contains a solvent which disolves the soft plastic.

The boards are now moved to a copper plating tank and another ounce of copper

is plated to the exposed surfaces.  At this point for the final traces we have

one ounce of original copper, one ounce added, and one ounce thickness through

the holes.  We also have one ounce of copper everywhere else with a plastic

film covering it.  Next the board is moved to a tin-lead plating tank and a

thin layer of "solder" is added to the exposed conductor.

Now the board is placed in a second solvent tank and the hardened film is

washed away.  When we look at the board we see solder where the traces are to

be, copper everywhere else, and solder plated copper through the holes.

Next comes an etch bath.  The etchant disolves copper, but leaves solder

alone.  Now what is left is solder plated traces where we want them.  This is

pretty much what is needed electronically, but there are a few more steps.

One is to pass the board under a heat lamp.  This causes the solder plating to

"reflow" to make a coating that is easy to solder (for a while).  Most boards

made today have a solder mask added.  This is a plastic film (often green or

blue) placed on each side of the board which masks everything but the pads

from the soldering operation.  This reduces the chance of solder bridging

between two circuits when the board is passed through a flow soldering

machine.  Last the board is silk screened with a legend to show the location

of each part, and possibly a part identifier or value.  Note that you will

need to supply artwork if you want the vendor to perform these last two

silk screen operations.

We have not mentioned inspection.  Most vendors inspect the work at many

stages in the process.  A typical process will have twenty to thirty steps in

the process counting inspections.  This is why the normal turn around for a

two sided plated through board is about two weeks.  Yes, an automated plant

will be faster, and a ma and pa operation will do fewer steps, but two weeks

turn around, $100 for setup charges and say $10 each for ten small boards is

about the norm (August, 1988).

Part of the reason for going through the above steps is to help you understand

what art work the vendor will need.  For a two sided board he will need art

work for the two sides.  By our convention, these will be drawn on level 1 for

the parts side, and level 2 for the solder side.  A drill layout is prepared

on layer 3.  We use layer 4 for the parts outline, which the vendor uses to

prepare the parts location silk screen, and layer 5 for the solder mask.

Many of you want to do baord fabrication work at home, or in a small shop.

You can do fair single or two sided board work at home using the kits

available from Kepro.  See the Newark Electonics catalog (Newark Electronics,

4801 N. Ravenswood Street, Chicago, IL 60640 Tel. (312) 941-7200) for the

materials.  They will supply pre-coated material which makes it relatively

easy to make boards.  Plated through hole boards are another matter.

Electroplating is still pretty much a black art.  Plating solutions do not

always work.  Your commercial vendor will probably throw away a few hundred

boards when he starts up his process, and he keeps it going by making

continuous performance tests and corrections.  Further, the plating operations

tend to involve rather nasty chemicals, like cyanides which are lethal if

handled improperly and lots of acid.  The ferric chloride used to etch the

pre-coated boards in the Kepro kits is relatively benign, but must be

discarded properly or the EPA will get you.

In LEARNSYM.DOC we will start the step by step production of a sample piece

of artwork for a two sided plated through board.