FIRST.DOC

Copyright 1986, 1987, 1988  by Thomas F. Droege, All Rights Reserved.

Version 1.86 21 August, 1988

*******************************

*  BE SURE TO READ UPDATE.DOC *

*******************************

DROEGE

Design Robot for Origination of Exacting Graphic Engineering

DROEGE is a general purpose program for the design of printed circuit

photomasters.  Is was created as a demonstration program for our low cost

precision photoplotter.  As it was developed it became more and more useful as

a design tool, so eventually the circuit boards and the mechanical drawings

for our photoplotter were done using this program.

This is a manual CAD system.  That is, it does nothing automatically like

schematic capture, wire listing, routing, parts lists, etc..  It does provide

a computer aided equivalent of a taped master or drawing board with the

advantage that things are always in perfect registration and one can work on

selected layers.

General features are a 65 x 65 inch working area 12 layers deep.  Resolution

over this area is 0.001 inch.  At least this is the way it is labeled - the

output can be scaled to anything so what is really provided is a 65000 X 65000

point working space.  The layers can be turned on and off and assigned to any

available color.  Lines, Pads (something a photoplotter can do - a small

object drawn all at once), Symbols (a named collection of other objects),

Arcs, and Labels can be entered.  The Symbol feature is particularly powerful

since Symbols can be nested within Symbols to many levels (20) allowing

construction of very complex objects with a few keystrokes.

The program being distributed through free channels is very powerful, but is

missing some of the convenience features of the commercial version.  The main

differences are lower display resolution, CGA versus EGA; reduced editing

commands; and less available memory.  We have been able to create very large

boards with this program, even in its limited form.  If you end up using it a

for professional work you will want the commercial version as it will pay for

itself in reduced labor costs.

Depending on how clever one is in conserving space, circuits can be designed

with ten to twenty chips or about 100 components.  As can be seen from the

demonstration files,  fairly elaborate designs can be created with the "free"

version.  The program with full storage allocation will accommodate designs of

about 300 chip equivalents.  It also allows you to view more of a circuit at a

time through use of the EGA.  Also supported in the pay version is cursor

movement by a mouse, and much improved object editing with move, copy, insert

and move point, and change.

You will find no appeal to guilt to get you to buy the registered version.  If

you really use it, it will be in your interest to buy a license.  If you try

this demonstration version  on something small to test its features you are

earning what you get by your work,  and helping us sell the product - a fair

exchange we think.  Print PROD.SUM for details of the advanced software and

the photoplotter.  Print PRICE.DOC for a price list for preparation of plots

from the output of this program.

Print DEMO.DOC for instructions for a quick tour around the facilities.

TUTOR.DOC directs you through a few simple tasks.  LEARNSYM.DOC works through

the steps needed to prepare simple printed circuit boards in great detail.  Do

not underestimate the effort required to learn to use this or any other CAD

system.  As a member of a CAD selection committee for a large laboratory the

author has reviewed many systems, it is an awful job.  My current view is that

one just begins to know the features needed after six months use.

Note in editing: This above was written in 1986.  My group in the laboratory

where I work is currently (1988) attempting to purchase a new CAD system.  I

am mercifully not involved, since I like what I have.  But some who work for

me are.  While standing around the water cooler yesterday, we added up $10,000

worth of my group's time just visiting vendors and trying to understand their

systems.  We are planning to spend about $250,000.  We realize that both the

evaluation time and the cost of the system are small compared to the cost of

using it.    My group has been brain washed by using my system so they want

the powerful symbol features of this $10 program to be available in the

$250,000 system.   These are really capable people, several have PhD's.  But

after spending all that time they just don't know what the systems will do.

The marketing people are really clever.  Their demonstration programs are

designed to make it look like the programs can do anything.  It is only after

you come back with your problem job to try to do it yourself that the

limitations are uncovered.  Use this "free" system a while and you will be a

lot more expert at finding the limitations of the fancy systems.  These big

commercial systems are really difficult to learn.  We expect that a good

designer (some of our designers hold PhD degrees in physics) will just start

to be able to be facile with the system after six months of 12 hour days.

While they auto-route, auto-place, schematic capture, rubber band, and

simulate the circuit; they may not be able to do symbols as this program can.

The big expensive systems come with a wall full of manuals.  More is better -

right?  One system we like because it is powerful, requires that you talk to

it in Unix macro language.  If you know what that means you know what it

means.  If you don't, then it is worse than your most imaginative nightmare.

So when you write me letters complaining about how hard this system is to

learn, and that I don't provide enough documentation, be thankful that you

have not spent $250,000 for it, that most people can get it running without

spending two weeks at the factory, and that it did now come with 50 manuals

that you must read.

***** SPECIAL OFFER *****

For a limited time, if you follow the procedure in PHOTOOL.DOC we will produce

a phototooling set for your job.  We will plot anything you can get on an 8" x

10" area of 10" by 12" film, for $25.00 per layer (Since we don't know when

you will read this, this offer is subject to confirmation, see PHOTOOL.DOC for

the procedure).

The following lists the documentation on this disk with a brief description of

what each file contains.  We suggest you do DEMO.DOC and TUTOR.DOC and don't

bother to look at much else until you have tried to do a few things.

ABOUTPC.DOC tells how printed circuits are made.  LEARNSYM.DOC works through

the symbol process in great detail.  KEYTREE.DOC will help show you where you

are going in the program:

The last distribution contained PC-Write page breaks.  We removed them in this

version so run your printer with whatever page break feature you like.

Otherwise you will get continuous listings.  It is enough for us to try to

write this stuff without writing a print program to make it look pretty.

FIRST.DOC

This document.

MANUAL.BAT - PRINTS A MANUAL CONSISTING OF THE FOLLOWING SECTIONS:

(You may print individual sections if you wish)

INTRO.DOC

Introduction and design philosophy.

DEMO.DOC

This leads you through a demonstration of what the system will do.  You

do not get to do anything, just look.  This is our "gee whiz" demo.

TUTOR.DOC

This leads you step by step through a few simple tasks.  We have tried

to start with excruciating detail and gradually tell you less and less

as you become familiar with our general approach.

ABOUTPC.DOC

This section tell how printed circuit boards are made.  Please, those of you

who are experts, don't write me letters telling me the procedure is not up to

date.  It was written to give the novice some idea of why the various photo

tooling items are needed.

LEARNSYM.DOC

This section is for real beginners.  If you are an old hand at printed circuit

layout and are used to computers, the DEMO and TUTOR sections and the COMMANDS

appendix may be enough for you.  You can operate on the assumption that the

program will do what you want it to do and just look up the desired operation

in the COMMANDS section.

QUICK.DOC

Quick summaries of how to perform the most common tasks.

HOWTO.DOC

How to do some more complex operations.

APPENDIX.BAT - PRINTS AN APPENDIX TO THE MANUAL CONSISTING OF THE FOLLOWING

SECTIONS (Or you can print them separately):

COMMANDS.DOC

An alphabetical listing of the command keys.  This is the primary reference

document for all the operations that are available.

KEYTREE.DOC

Operation of this program is by use of the function keys.  Each key press

typically reassigns the keys to a new set of values.  KEYTREE.DOC is a "map of

the cave" showing the route through the menu keys with very brief summaries of

key operation.  The key menu is presently fourteen layers deep and has

hundreds of entries, many of them repeated.  A number of the key operations

are reentrant so that much deeper entry is possible.  The design allows quick

movement to a menu that will do what you want.  It may be useful to print this

out and hang it on the wall when you first start using this program to remind

you of what is available and as an index for COMMANDS.DOC.  We thought this

would be very useful.  We find it almost impossible to read, but we include it

because we went to the work to prepare it and it might fit someone's taste.

ERRORS.DOC

Error messages and their meaning.

STANDARD.DOC

This document suggests a set of standards to get you started.  It pays to

approach a job in a consistent manner.  This file gives a set of standard

procedures for producing a design for a two sided plated through hole board.

If you follow this standard we will be able to plot your job for you with

minimal hassle.

PLOTOUT.DOC

How to generate a plot output file with sample file output.

THE FOLLOWING MISCELLANEOUS DOCUMENTS CAN ALL BE PRINTED WITH MISC.BAT

OR THEY MAY BE PRINTED SEPARATELY:

PHOTOOL.DOC

How to get a set of phototooling prepared from your design.  Read this before

you prepare a job for us to plot.

PRICE.DOC

We can prepare phototooling sets from the output of this program.  This list

allows you to estimate the cost.  We can charge a Visa or Master Card.

PROD.DOC

A summary of the Environmental Optics products.  Specifications are included

for our photoplotter, and the the two current versions of the program.  We

also include some of our future software plans.

USERS.DOC

We will collect and distribute useful libraries.  This is a form to

make a submission.

REGISTER.DOC

We do not expect a registration fee.  If you find this program to be useful,

and feel guilty, then buy the commercial version, $100 post paid.  You will

enjoy the additional features more than the loss of guilt.  Send this form

without money and we will keep you advised of new versions.

UPDATE.DOC

Changes and bug fixes by date.  *********Be sure to read this file.*********

New goodies are described here.

PRINTALL.BAT

Prints everything.

ORDER.DOC

Order form for photoplotting.

Please forgive inconsistency in this documentation.  It is a big job to edit

all these words when we make an improvement.  So if words point to something

that you cannot find, perhaps it was there once.