INTRO.DOC

INTRODUCTION

This program is designed to aid the manual preparation of printed circuit

phototooling.

It runs on an IBM PC, XT, AT and anything else that runs PC DOS we think.  A

minimum configuration is one disk,  320k memory, and a color card.  The free

version uses 320 k memory.  The licensed version will make use of 640k on a

PC.  While it will run with a mono monitor on a color card,  the program is not

very useful without a color monitor.  An Epson compatible printer and an

additional disk are useful.

We describe this as a manual system because there are now many programs which

purport to automatically generate phototooling from circuit diagrams or logic

statements.  We have been designing mostly analog circuits for thirty years and

are yet to know enough about design to direct a skilled layout professional,

much less to design a computer program for the task.  We would not know where

to begin to set up design rules.  We often redesign the circuit during layout

to overcome a problem introduced by the topology of the corner we have worked

ourselves into.  We do not see how a program could be designed to make such

opportunistic decisions.

If you have not used a computer aided layout system before you have an

important decision to make.  Will such a system save you time (money) in the

long run?  There is no doubt that it will be more expensive at first to go to a

computer aided system.  Even one like this that is free! (Ha!).  Everyone

underestimates the effort it requires to become productive.  Your friendly

marketeer will talk about factors like 5 in productivity.  You will be lucky to

make .5 the first year.

In our design group, the engineers do their own layout.  We have watched some

of the fancy automatic systems in operation and we sometimes wonder why anyone

would want to work so hard to do a layout.  We watch with awe the problems

associated with the introduction of a new component into an automatic system.

I have completed an entire design while my neighbor in the CAD room was trying

for several weeks to properly specify a new component.  If you want a machine

to be really automatic you have to tell it everything about its components.

This is tough enough with digital chips, as my CAD neighbor discovered.  For

analog chips or for new concepts unanticipated by the CAD designers, it is very

difficult indeed.

We are not so reactionary as to think that computers will never be able to

perform this task.  We do feel that with the aid of a simple design tool like

this program that today we can outperform the computers on all but the most

routine digital layout tasks.  Marketeers extol the virtues of such features as

rubberbanding - yet try it on a reasonable sized circuit and it is almost

impossible to make any sense out of the jumble.

Computer aided systems win big if there is repetition.  Some of the boards we

design contain 32 identical amplifiers, and some control circuitry.  Typically

a thousand mixed analog and digital components.  These designs are very cost

effective for this design system.  When the customer comes back for a

different design we can produce a whole layout just by changing the amplifier

symbol. (Horrors, we have just discovered (Sept, 1988) that the largest

systems - big names, $100,000 up, cannot do this.  If you have 32 identical

amplifiers they will be wired 32 different ways.  This is what auto-routing

does for you.  Imagine trouble shooting such circuitry.  If you want to have

the identical circuits wired alike, you get to do each one by hand yourself.)

We have set out to design a program to aid us overcome the drudgery of the

layout task.  It has been written while under constant use by several very

senior designers.  The general plan has been to add the feature most griped

about next.  This will explain to CAD professionals the otherwise mysterious

selection of features.  For example, there is no mouse or joystick input

because we were able to work just fine without it.  There was no marketing

department to force us to add features for marketing purposes only.  Before you

decide to pass up this system because it does not have some favored feature,

give it a chance.  You may find it workable.  We have used the full version of

this program to produce 13" x 17" printed circuit boards containing over one

thousand components and 3500 holes.  The boards are mostly mixed analog-digital

designs where we are concerned with microvolt signal levels and nanosecond rise

times.

Some of the designs have been done on a team basis.  It is important to us to

be able to pass parts of a design between work stations with several designers

working at the same time.  This is accomplished by the extensive and flexible

symbol features of this program.  We have not yet networked our terminals but

only because it is still easier to walk across the hall with a floppy disk.

A symbol is anything that might be put down as part of a printed circuit

master.  A very simple symbol might be a pad for the location of a component.

This program allows nesting of symbols so that once, for example,  a symbol for

a printed circuit component pad is created it can be duplicated to form an

integrated circuit package outline.  At the next level a group of package

symbols might be combined with wiring to form a memory symbol.  Some examples

of this are included in the demonstrations.  We are gradually accumulating a

library of such symbols so that we can rapidly produce rather large designs.

Designers who are used to working with pre-printed stick-ons for the layout of

designs will find the concepts of this system familiar.  It is easy to create

a new stick-on, we call such an item a symbol.  Since this system allows

nesting of symbols (symbols can contain symbols) very powerful stick-ons can

be easily created.  An example could be a nine chip memory block.

The program allows entry of lines, pads, symbols, labels, and arcs on 12

different layers and on a 65000 x 65000 point grid.  Lines correspond to

printed circuit tape which is used to interconnect components.  Lines come in

fifteen widths.  Pads correspond to the doughnut shaped cutout used to indicate

that a drill hole was to be made to mount a component.  Symbols are arbitrary

collections of everything else which can be located and deleted as a single

object.  A familiar symbol would be a 14 pin IC layout stick-on.  But they

can be much more complex, in fact it is recommended that the whole job be

made a symbol so it can be manipulated as a single item.  Labels are

lettering.  Arcs allow drawing circles and partial circles.

While the program is labeled as a 65.000" square grid, there is nothing special

about the label and the output files can be scaled arbitrarily.

Data is entered by selecting an object type and locating it on the grid through

use of a screen display and the arrow keys.  We had planned for a mouse,

(available in the pay version) but our users never complained.  A color display

is provided so that objects can be distinguished on different levels ( our

users absolutely demanded color) though the "free" version shows only the last

color entered for intersecting objects.

Three different colors can be assigned to the layers.  This is usually enough,

since more is just confusing.  The pay version supports the EGA with more

colors but early work indicates that the resolution is more interesting than

the additional colors.  We seldom work on more than two or three layers at

once, even when doing boards with many more layers.

Parameter sets can be stored for frequently used combinations of working

conditions.  One set allows instant redraw to any preselected part of the

layout.  Another set allows switching between entered object types.  For

example a few key strokes are all that is needed to switch between entering red

100 mil wide lines on level 3 to green 15 mil wide lines on level 7 if these

parameters have been preset.

Our goal has been a simple system that works on a minimum PC configuration.  We

have resisted making special monitor calls or doing anything that was not

available on any system with a color card and color monitor, even if it would

make operation better.  This further explains why there is no mouse.  The

program was written originally in compiled BASIC though we are gradually

converting sections to assembly language.  We have run it on a variety of IBM

PC, XT, and AT computers, and a number of clones.  Except for saving and

loading jobs, and creating plot files, the program runs entirely in memory.

When we started this development several years ago, we made the decision that

by the time that we would get our program written, memory would be cheap

enough to hold it.  It is almost an axiom that memory will be cheap enough to

hold any program you can conceive of writing by the time you can get it

written.

This version is being distributed free, primarily to advertise our

photoplotter, which was the reason for it being written.  As distributed it has

restricted memory, enough for about a thirty chip circuit board.  One of the

demos, ALRM, is a 1000+ component board where advantage is taken of the

repetitive nature of the circuits to save memory.  (Sorry this design has

been removed from v1.86 to allow room for additional documentation.  We

include several large example designs on the commercial version.)  Print

PROD.SUM for the specifications of the extended version of this program which

is available for $100.00.  Work started on the free version of the program is

transferable to the pay version which has a number of additional features

which serious users will want.  We hope to get desperate letters asking us to

rush the larger version as users run out of memory.  Plan ahead and buy now.