LEARNSYM.DOC

HOW TO WORK WITH SYMBOLS

This exercise tells you a little about symbols, and how they are unique to

this program.  We then lead you step by step in the creation of a symbol,

which is the normal way to prepare a set of artwork.  We show you how to put a

symbol on the job or in another symbol.  We show you how to get a plot on a

printer, how to make plotter files, and how to save your work for later

additions.

The symbol feature of this program is very powerful.  We find that systems

which cost $50,000 and up cannot nest symbols as we do.  This is because they

have to do schematic capture, simulation, and other wonderful things that you

may not need to do.  Keeping track of everything through layers of symbols is

too tough a job for even the big systems.  We have just spent hundreds of

group man hours reviewing systems from (July 1988) big names like Cadnetix,

Hewlett Packard, Mentor Graphics, and others and none can nest symbols.

We are presently involved in the design of a high voltage power supply system

which has eight high voltage modules which plug into a VME mother board.  We

designed the plug in board manually using this system as a two layer board.

Another group designed the VME mother board.  It was done by a big design

system and uses ten layers.  There are awful errors, like the power bus is 10

mils wide in several spots, and even a moderately good layout designer would

have done the job with four layers.  That is what happens when you turn a big

design system loose with some rules.  When you see the result, you wish you

had provided a different set of rules, but a live designer would have changed

his rules when he saw something bad.  But the real problem is that the 8

identical circuits are laid out eight different ways.  Now we have to test it

and find that we have eight different sets of problems.  We have discovered

that there is no real way, on even the largest systems, to duplicate part of a

layout when it is repeated.

My guess is that the other design group will do more work than I before the

job is done.  The big systems do have some nice features.  They make you put

all the wires in to match your schematic.  People like me that change their

designs on the fly must keep going back to the schematic section of the

program if they want to make a change, in the end you are absolutely forced to

have a correct diagram.  Still, I would prefer the pain of having to keep

track of which wires I have routed to the helplessness of having the machine

put things where it wants to.

For this system, a symbol is any collection of work.  This means lines, pads,

lettering, arcs, and even other symbols.  The only limitation is that you

cannot put a symbol in itself.  We sometimes allow ourselves to do this for

test, the result is like standing between two mirrors, eventually the computer

runs out of something and dies.

We find it useful to make a symbol library of common things.  Not just the

basic symbols for common sized R, C, and IC outlines found on the distribution

disk as ELEC.LIB, but also common circuits we use, such as amplifiers, timing

circuits, memory layouts, etc.

We in fact recommend that you make your entire project a symbol.

In order to locate a symbol on the job or in another symbol, it must be loaded

into memory.  If in doubt, you can list the symbols in memory as we do in the

following example.  You will need to have a printer.

To start the program put distribution disk 1 in the default drive and type:

DROEGE (ENTER)  That is type DROEGE followed by the ENTER key.

After you have read the sign on  message hitting any key should get a nice

blue screen with function key labels across the bottom of the screen and other

information (see DEMO.DOC) across the top, including the cursor position at

the top right.  The cursor appears as a blinking square which can be moved by

the arrow keys.

Most operations are done by hitting function keys at the left of the keyboard.

DEMO.DOC and TUTOR.DOC introduce these keys.  This exercise assumes you are

now familiar with the general ideas.

Now press the function keys as indicated.  We list here the entire label as it

shows on the key.  You just have to push the numbered key at the left of the

keyboard:

F2FILE          Because that is where we happened to put the command to list

symbols.

If you have not already done so, this is a good place to get a listing of the

setups.  You will need a printer for the following step.  If you do not have a

printer skip this step.

F5LSTSET        A two page table is printed.  Hang this where it is available

for reference.

F2GETFIL        We want to get something from a file

F3LSTSYM        To list the symbols in memory on the printer.

If you don't have a printer the system will say something awful.  Like fatal

error and some other garbage.  But don't be concerned, it is hard to kill this

program.  From a freshly started program you will find that you have no

symbols.  The printer will list one funny symbol called JOB$$SAV.  This is

where we store the work you do on the job.  We don't automatically load

symbols because they use up space and we want to give you as much space as

possible.  To load some representative symbols, for example the library

ELEC.LIB push function key 2.

F2LODLIB

To the question answer:

ELEC (ENTER)

Do not type the extension .LIB or again you will get a nasty error message.

Just follow the instructions and try again.

If all goes well, the disk light will come on and symbols will be loaded.

F3LSTSYM

Will now get a list of about 15 symbols on the printer.  If you don't have a

printer, you can still do this exercise, but you will have to fly blind.

Now use the F9 key to get back to the starting menu.

F9BACK

F9BACK

The bottom menu line should now read:

1ENTER   2FILE   3PLOT   4SETUP   5PATH

If it does not read as above, hitting F9 enough times should get the above

menu.  If you get lost you can always get to this menu by hitting F9 enough

times.  Answer "y" when asked if you want to leave a symbol, but "n" when

asked if you want to leave the job.  If against all our precautions, you

manage to "break" the program, hitting key pad #9 with caps lock on and num

lock on and the rest of the special keys off, will usually get you back to

this starting position.  This is useful if you have done a lot of work without

periodic saves and then produce a hang.  Honest folks, we have not seen a real

hang in this version of the program in over a year.  Still, we provide a

parachute when you fly with us.

Now let us play with symbols.

F1ENTER           To go towards entering data

F3SYMBOL          To do something with symbols

F3CREATE          To do something creative with symbols - I had to name it

something.

F5OPEN            To open an existing symbol to change it.

In answer to the question:

IC14 (ENTER) - The name of the symbol we want to work on followed by the

ENTER key.

Symbol names are case sensitive.  That is Ic14 is a different symbol from

IC14.  If you type wrong you will get a symbol not available message.  Hit F5

and try again.  We recommend that you run this exercise with the CAPS LOCK key

on.  This will reduce the chance for a typing error.  In general, unless you

like naming things in mixed upper and lower case we suggest you pick one case

and stick to it.  We always run in upper case.

The system will draw an IC outline at a scale which fills most of the screen.

We try to follow a convention when we make symbols.  For the most part, the

symbols in ELEC.LIB were made so they extend horizontally with pin one at the

lower left, or 0,0 position.

This symbol is more complicated than it looks.  It has information on five

different layers.  By using the prepared setups, we can look at the layers

(levels) one at a time.  Looking at the chart we printed, we see that setup 1

contains a color assignment for only level 1.  The color assignment column

indicates what will show when we redraw.  We are presently drawing with setup

0 which contains color assignments for most layers, so nearly everything is

showing.   To change the setup assignment we need to find a menu which has a

4SETUP key label.  This is repeated many places, usually where needed.  But it

is not on the current menu.  We hit a key to find it:

1LINE

The new menu contains 4SETUP so:

4SETUP

The top line now shows that we are in setup 0, and some information about

setup 0.  There is not enough room on the top line for everything.  The

advanced version uses the EGA features to display the entire setup table, but

we will have to be content with our listing.  We want to change the setup so:

1EDSET

We get a new menu which contains only UP and DOWN  and a key 5 label which

reads 5*SETUP to indicate that we can change the setup.  Since we want to go

to setup 1 we go up once.  Many downs would also get there.

1UP

The second line now reads SETUP 1, LEVEL 1, COLOR GREEN, APER OFF.  This tells

us that we are now in setup 1, that we are looking at level one of the setup 1

table, that level one of setup one is displayed in green, and that if we

attempt to draw a line or pad, nothing will happen as there is no aperture

selected.

We can redraw the screen at any time.  This is done with the F10 key.  The

first press lists a table of previously selected places to look.  We can use

the menu to select a new area or we can redraw the present area with a second

F10.  We want to redraw the present area with the new setup so:

F10REDRAW

F10REDRAW

The IC is now drawn in green.  Just the pads are drawn.  And we notice they

are smaller.  When the system last drew the IC it noticed that there were

things on several layers.  To show this it drew in the "OR" of the colors and

the size of the largest object.  Since this IC has an eighty mil solder mask

on level 5, it drew the pads as eighty mils square.  Now it is only showing

level 1 where the pads are sixty-two mils on a side.  Notice also that the

package outline drawing disappeared.  It was on level 4.  STANDARD.DOC lists

our standards for design.  It is not necessary that you follow them, but it

helps avoid confusion if you send us work.  Otherwise, it is helpful to you to

set up standard ways of doing a design.

Let us look at another layer.

F1UP

This time the second line shows:

SETUP 2, LEVEL 1, COLOR OFF, APER OFF

This is OK as far as it goes.  If we look at the print out we indeed see that

level one does not have a color assigned to it.  But level 2 does and that

does not show on the status line.  There is just not enough room to show

everything.  By backing down one menu level and selecting level for edit, one

can UP - DOWN through the levels for this setup.  The whole process is awkward

but it is the best we can do.  Some day we will do this with pull down menus

and a mouse.  The advanced version is better as you can see everything at

once.

F10REDRAW

F10REDRAW

And the pads redraw in red.  This is what is on level two.  Now go to setup 3

and redraw:

F1UP

F10REDRAW

F10REDRAW

Looking at the setup listing, we see that setup 3 draws level 3 and 4 at the

same time.  We put drill spots on level 3 and the package outline on level 4.

We can easily see both as the drill spots are slightly larger than the line

used for the package outline.  The drill spots are displayed in green and the

outline in brown according to the color selections for the two layers showing.

F1UP

F10REDRAW

F10REDRAW

One more time gets us the outline only.

F2DWN

F2DWN

F2DWN

F2DWN

Puts us back where we started, at setup 0, where all levels are showing.

Now that we have seen what this symbol looks like, we will close it and look

at another symbol.

F9BACK    -  four times.

To leave the symbol.  The system asks "enter y to exit symbol" answer:

Y (ENTER).

The system tells us it has closed the symbol.  It also does a calculation of

the size of the symbol at this time so that the next time the symbol is opened

it can draw it to fill the screen.

F5OPEN            To open another symbol.

answer C (ENTER) to look at the capacitor symbol.

Again the system draws the capacitor symbol to fill the screen.  Again back

out with an F9.

F9BACK  answer Y (ENTER)

Now that we have seen what a couple of symbols look like, we will put several

symbols in a symbol to simulate creating a full piece of work.

F1NEW

We are asked to enter a symbol name.  There is no real limit to the entry, but

sticking to eight letters or less will facilitate later making a library  (DOS

file) with the same name.  Being an old Smokey Stover fan we name the symbol:

FOO (ENTER)

We are now asked to locate the symbol origin.  Wherever the cursor is when F3

is hit will become the 0,0 reference point for this symbol.  When we locate

the symbol we are creating in the job or another symbol, the work we enter now

will appear in the same orientation to the point located as it does to the

origin.  This may seem vague now but a few examples will demonstrate the

process.  Symbols can be located rotated at 90 degree steps.  Symbols rotate

around their origin.  So it it best design symbols so you can remember their

origin and their original orientation.  Sorry, but we have not been consistent

in the various examples.  ELEC.LIB is mostly done with pin one at the lower

left for IC outlines, and R's and C's horizontal.  Other libraries we have

done standing up.  You will develop your own preference.  It does not take

long to make a library in a style that is pleasing to you.

Move the cursor near the lower left corner of the screen.

F3ORIGIN

Note that the value of the cursor position is now 0,0.  We notice that the

capacitor symbol is still there.  The screen is normally erased only when you

redraw.  We do that now, though sometimes it is useful to have an old symbol

left on the screen.  For example, we sometimes make use of this feature when

we want to make two symbols which connect to each other.

F10REDRAW

F10REDRAW

We now have a nice clean space and a new symbol "FOO" to create.  We will put

an IC outline, a bypass capacitor, and a couple of connection lines to make a

bypassed IC symbol.

First we get the IC14 symbol that we just looked at and locate it standing up

at 0,0 where we can remember how it looks.

We notice that we have the familiar line:

1LINE   2PAD   3SYMBOL   4LABEL   5OTHER

At the bottom of the screen.  When we are in the mode where we can add

material to a symbol, we get this line just before we can back out of a

symbol.  Again, if you get lost in the command tree you can always hit F9

until you get the "enter y to exit symbol" message.  Then hit anything but a y

and you will know where you are.  We are working on our new symbol "FOO" which

is at present blank.  We want to add a symbol to it so we select symbol.

3SYMBOL

We will have to tell the program what symbol we want to enter in our symbol.

We also need to tell it how we want it oriented.

F2NAME

We need to name a symbol.  We are asked to enter a symbol name.

IC14  (ENTER)

We notice the second line now reads "enter symbol IC14" to show that we have

selected a symbol which can be entered into our new symbol FOO.  We could do

some other things, like rotate our symbol.  For now we will try entering this

symbol into the new symbol FOO.

1LOCSYM

We now notice that the bottom line reads:

1FIND   2DELETE   3*MODFY  4SETUP   5REJECT

Whenever we are in a position to enter data, this line appears.  We also

notice a 7, a triangle pointing to the right, and a happy face in the upper

left hand corner of the screen.  The 7 is our level in the key tree.  The

right pointing triangle indicates that the locate key is enabled to locate

something, and the happy face indicates that we are locating work in a symbol.

We will locate the library symbol IC14 in our new symbol FOO.

F7LOCATE

And the IC14 outline appears.  Depending on where the cursor was placed when

the symbol origin was located, the outline may be partially off the screen.

Do not worry, it is not located where we want it anyway.

F1FIND

We need to find the object just entered to delete it.  With the advanced

version objects can be moved and copied.  It is enough to do work to "find",

"delete" and re "locate" objects.  The advanced version is just more

convenient.

The information line reads "symbol IC14 found @ above x,y".  The position x,y

reads 0,0 which is where we just put the symbol so we have found the right

one.  Later when you have a big job and don't remember where everything is,

entered objects can be found by positioning the cursor near where the object

is and noting if the right one is found.  If the wrong object is found,

hitting the F5REJECT key will cause the search to continue until the desired

object is found.  Note that sometimes there will be several objects located at

the same position.

This is the only object on the screen and it is in the wrong place so:

F2DELETE

And the IC14 symbol disappears.  Actually it is redrawn in background.  This

means that if several objects were drawn on top of each other then part of

some of the overlap might disappear.  Two F10REDRAW key hits will always

redraw what is there.  At the moment our symbol FOO is empty so there is

nothing to redraw.

We want to draw our symbol standing up, so we need to get back to the menu

which allows this.

F9BACK

Now for our instructional purposes, we want to draw the IC symbol standing up

with the notch at the top.   The 5ROT=  key allows the symbol to be entered

rotated in 90 degree increments, starting from how it was created.  Since the

notch was to the left in the symbol as it is stored, we want to rotate is 3

steps of 90 degree counterclockwise.  Why counterclockwise?  I hear you ask.

That is the way mathematicians think of angles.  It could have been done

either way, but we followed normal math conventions.

F5ROT=0

F5ROT=1

F5ROT=2

After pushing the F5 key the third time it ends up reading F5ROT=3 which is

where we want it.  It is also necessary at this point to rename the symbol we

want to enter.  Sorry, but a peculiarity of this system is that the current

symbol name is canceled when a delete operation is performed.  We thought we

had a good reason at the time, but now the logic is so imbeded that it would

take a major rewrite (which is in progress) to change it.

F2NAME

Answer IC14

F1LOCSYM

We are now ready to locate the symbol.  Move the cursor to x=0.0, y=+0.6.  Why

this location?   Because I am designing this symbol.  You get to design your

symbols the way you want to.  I have thought out in my mind how I want it to

look.   I am going to make an IC symbol with a bypass cap at the top and pin 7

at the lower left hand corner, which will be at zero, zero.  At least the way

I did it this location is off the screen.  Many systems do not let you locate

items off the screen.  We often do.  It sometimes saves trying to reposition

the work on the screen at just the right scale.  We now locate the IC outline.

F7LOCATE

Part of the IC outline appears.   The rest is off the screen.  No matter.

Move the cursor to 0,0 which should be the lower left hand pin or pin 7.  This

is a mess.  We would like our work nicely centered on the screen.   We will

now create a work area of a scale we like.  The 4SETUP key gets us to menus

which allow changing all sorts of things about the screen setup.

F4SETUP

F5SCREN         To work on the screen appearance.

F4REGION        To change the region of the screen we look at.

F3AUTO          To do it in automatic mode.

We have not done things in the right order but purposefully so to instruct

you.  We are asked for a screen width in inches.  We will enter 2

2 (ENTER)

The screen redraws.  We briefly see part of one of the pads drawn at the

screen top before it is overwritten with the status lines.  The screen is now

blank.  I guess we didn't do it right.  The way auto works, is to draw the

screen at the entered width with the present cursor position at the upper left

of the new screen.  We will try again.  We first move the cursor to x=-0.5,

y=1.2.  Again, I am doing it the way I want to.  You might pick a different

position for your work space.

F3AUTO

1.8  (ENTER)

This puts things where I want them.  We might like to remember this location

so we do not have to fuss the next time we work on this symbol.  We will use

the memory feature of the redraw command to store this redraw area.

F10REDRAW

We notice that some of the areas have names, and some are blank.  We could use

any of them but we will use the up key to select 7 which is presently blank.

F1UP 6

We note the key label changes to 1UP 7 so we are now selecting the space we

want.

F5CHANGE

This will store the present screen view in location 7.  We are asked to enter

a label for this setup.  Enter anything that is descriptive to you that will

fit on the screen.  We enter "Nice view of symbol FOO".  We notice that this

label appears opposite the number 7.  To show how this works, we will redraw

the screen using one of the old setups.  Use the 2DWN key to get to setup 5.

F2DWN 7

F2DWN 6

And the key label reads 2DWN 5 to show that we are now pointing at the stored

view 5.  We select this view by the select key.

F3SELECT

The screen redraws with the indicated area of 22 by 17 inches.  This is much

too large to do any work.  We can barely see our IC outline.  So let us go

back to our stored view.

F10REDRAW

This gets the list of redraw areas.  We again use the UP DOWN keys to pick the

area we just made.

F1UP 5

F1UP 6

And the key now reads F1UP 7 to show that we are pointing to the nice view of

our symbol.  So we redraw this view.

F3SELECT

And we are back to the "nice view".  In general this is the way this system is

designed to move around the work area.  Use the AUTO feature when you want to

generate a new work view.  Save it as above when you know that you will want

to come back to it.

Now we will add to our symbol.  It would be nice to have a bypass capacitor.

F9BACK

F9BACK

F9BACK

F9BACK

Gets us to the menu that lets us select a symbol name.  We know the menus are

a little hard to get around at first.  By the time you do a little real work

you will find that you are moving around the keys with little effort.  If you

think this system is hard to learn, try one of the big expensive systems where

similar moves around the system are done with UNIX macro strings.  NO hope to

learn without going to a school for a long time.

F2NAME

C (ENTER)

We note that rotation is still at three which is a 270 degree rotation.

Since this is not where we want it we rotate it one more time.

5ROT=3

And note that it changes to 5ROT=0

In general, special conditions like rotate are reset to a standard initial

mode when we leave an operation.  This has advantages and disadvantages.

Sometimes the system does not remember the current symbol, rotation, etc. when

we would like it to.  Sometimes it remembers an old condition when we would

rather it would reset.  We will fix all thin in version 3.  But for now, and

the free version probably forever, you will have to put up with strange

behavior.  So if you go to enter a symbol and nothing happens, go back and

rename the symbol.  We will now enter a bypass capacitor.

F1LOCSYM

And we see that we have the standard data entry menu.   We move the cursor to

x=0.050, y=0.8.  To do this we have to change the cursor movement speed.  It

was moving 0.1 inch per step.  Since we want to move to a .050 step we need to

change the movement.

F6SPEED

F6SPEED

F6SPEED

F6SPEED

F6SPEED

F6SPEED

If we have both counted right, the entry ahead of the % sign on the top line

now reads M  50 to show we are now moving in 50 mil steps.  Use the F6 key to

go around again if you miss it the first time.  The smallest number in this

sequence is set by the GRID key.  It can be set at any number.  The SPEED key

then works on a 1,2,4,5,10,20 multiple of the grid.  The grid is presently set

at 25 mils.

Now that we are at the desired location we locate a capacitor.

F7LOCATE

And the capacitor symbol appears where we want it.  It happens to have been

created lying horizontally so we do not need to rotate it.

We will now connect the capacitor between pins 7 and 14 with wires on level 1

and level 2.  Looking at our chart we see that setup 8 will enter 20 mil wires

on level 1.  We change the setup to 8.

4SETUP

1EDSET

1 UP  pushed 8 times gets us to setup 8 as shown on the second screen line.

F9BACK pushed 4 times gets us to the line, pad, symbol, label, other menu.

We want to enter lines so we pick:

1LINE         To enter lines.

1LOCLINE      To distinguish from entering points in existing lines

And we now have the familiar find, delete, modfy, setup, reject menu that

tells us we can enter data.

Move the cursor to the right end of the capacitor, x=.250, y=.8.  Hit locate

to start a line.

F7LOCATE

A tick sound is heard to indicate a line has been started.  Now move to

x=.3, y=.8 and locate an intermediate line point.

F7LOCATE

A green 20 mil line segment appears between the starting point and the current

cursor position.  Now move to the IC pin 14 x=.3,y=.6 and end the line.

F8END

A lower pitch sound distinguishes a line end from a line start.  We now have a

line connecting the right end of the capacitor to the IC pin 14.  For fun, we

will put the next wire on the other side.  We notice that setup 9 is setup for

20 mil lines on level 2.  So we select setup 2.  Notice that if none of the

setups had the line width and level combination that we want, we can edit any

one of the setups for the desired combination.  We generally try to pre edit

the setups so we have available the ones we usually want.  We save one,

usually setup 0, for use when we want something special.  See documentation

elsewhere for the edit procedure, or just muck around till you figure out how

to do it.

F4SETUP

F1EDSET

F1 UP

And we now have setup 9.

F9BACK  twice and we are back at the data entry menu.

Move the cursor to the left end of the capacitor.  x=.05, y=.8 and start a

line.

F7LOCATE

Again a tick is heard.  Move to x=.1, y=.8 and locate an intermediate point.

F7LOCATE

A tick is heard and a red line appears indicating a line on level 2.  Now move

to x=.1, y=0.0 and place another line segment.

F7LOCATE

Finally move to x=0, y=0 and end the line.

F8END

A lower pitch sound again indicates a line end.

This does not seem like a very complicated object but it illustrates most of

the concepts needed to make very complex boards.  Back out of the symbol now

by hitting F9 three times and answering y.

F9BACK three times, y (ENTER)

The system says "symbol closed".  We can now use this symbol.  But first we

will open it to see what we have made.  Note we created it with "new" but now

that it is an existing symbol we have to open it.

F5OPEN

FOO (ENTER) to the question.

Notice that it is now drawn in a scale that fits it to the available screen

space as best the system can.  Use the redraw command and select "nice view"

to observe that the system picked its own view of this symbol.  Now use F9 to

back out of the symbol and answer y to close it.  You now have your very own

original symbol.   Let us make something even more complicated.

F1NEW

GOO (ENTER) to create the new symbol goo.

F3ORIGIN  at some convenient place.

While the new GOO symbol space is empty, our old symbol is still visible.  We

sometimes make use of this when we want to make two symbols that have to fit

together.  For now we will clean up the space by hitting F10 twice to redraw

the clear area.  The cursor is now sitting at 0,0.

F10REDRAW

F10REDRAW

We will put several copies of the FOO symbol in the GOO symbol.  Suppose, for

example, that the FOO symbol were all the lines necessary for a memory chip,

as we indicated in the TUTOR exercise.  We could now make a multi chip memory

section.  We will put down three of the FOO symbols since that is enough to

illustrate the idea.

F3SYMBOL

F2NAME       FOO (ENTER)

F1LOCSYM

F7LOCATE

The FOO symbol appears at 0,0.  Move to 0.5,0 and locate a second copy.

F7LOCATE

Again a third copy at 1.0, 0

F7LOCATE

This is enough for illustration.  Close the symbol by backing out with F9's

and a y.

Now get a large working space.

F10REDRAW

F2DWN    six times untli it reads 1

F3SELECT  to select the 10 by 8 inch work space

With this system, you can work with symbols until you have completed your

task.  For output purposes, the work must be put on something called the

"job".  The difference between a symbol and the job is that while a symbol can

be moved, saved and deleted as a unit, the job is a special part of the work

area which is used for the generation of output files.  The next version of

this program will allow everything to be done from a symbol which will remove

one level of confusion.  We just did not think to do it this way at first and

it is now too hard to change without a complete rewrite.

We will treat the symbol GOO like it is our complete task.  We will first

locate it on the job area.  Then we will make a check plot of it and prepare

files for photoplotting.  First use F9 to back out to the line, pad, symbol,

label, other menu.  Note that when we do this we are at key tree level 2 as

shown by the upper left hand corner screen number.  We are no longer working

in a symbol, work we do now goes in the job.  Move the cursor to 0,0.  Select

line.  We will first draw a line around the work area so that we can see where

to put our work.

F9BACK

F9BACK

F1LINE         To go to enter lines

F1LOCLIN       To locate a line

F4SETUP        We will make this line a different width for practice.

F1EDSET        To pick a different setup.

We note that setup 15 draws a line on four levels at once.  This will

conveniently put a border on all the layers.  F1 UP six times gets us to setup

15.

F1 UP          Six times

F9BACK         Twice gets us to the find, delete, ... menu.

We are still at a move speed of 50 mils.  5 F6SPEED will get us to 1 inch

moves.  We are already at 0,0.  Let us move to 1,1 and draw a border 1 inch

inside a possible 10 by 8 inch piece of film.

Move to x=1, y=1

F7LOCATE

A tick is heard indicating that a line has been started.  Move to x=1, y=7.

F7LOCATE

A tick is heard and a line appears between 1,1 and 1,7.  Continue the border.

Move to x=9, y=7.

F7LOCATE

Move to x=9, y=1

F7LOCATE

Move back to x=1, y=1

F8END

And the border is complete.

F9BACK      twice to get to the menu which allows symbol selection.

F3SYMBOL    to select symbol entry

F2NAME      to choose a symbol

GOO (ENTER) to pick the symbol we made to simulate a real piece of work.

F1LOCSYM    to locate the selected symbol on the job.

Now we move to x=4, y=3 to locate the symbol GOO.

F7LOCATE    A tick sound is heard and the three chip outlines are drawn.

We now have a job with some stuff on it.  It is not much, but does illustrate

how a job might be made.  We can now make a check print to inspect our work.

Three F9's will get us back to the very first menu where we can select the

plot menu.

F9BACK  three times.

F3PLOT  to select the plot menu.

We will first make a plot on the Epson printer.  Since this can take a long

time, we will select only the part of the layout where there is something of

interest.  This is the area of the three chips.  First use F6 to select .1

inch steps so we can accurately bracket the area of interest.  Then move to

x=3.8, y=4.1.  Now we select the Epson plot mode.

F2ESPLOT

We are asked to place the cursor at the upper left hand corner and to hit F7.

F7LOCATE

We are immediately asked to locate the lower right hand corner of the area to

be plotted.  We move to x=5.5, y=2.8 and:

F7LOCATE

We are now asked for a scale factor.  Entry of 1 would print the design at

labeled scale.  It would be hard to see the detail.  We pick 2 to get a 2 to

one plot.  The part of the drawing selected is printed to a new scale on the

screen and the printer and the printer starts (slowly) printing the screen.

It there is no printer, or if the printer is not compatible with Epson

graphics mode 5 there will be a nasty error message but no work will be lost.

When the print out is complete we notice that the various levels are printed

out on top of each other.  We can't tell which level is which.  To get

separate levels, we need to select a setup which prints only one level at a

time.  We note that Setup 1 is prepared to print only level 1, the parts side

while Setup 2 is arranged to print only level 2, the wire side.  Setup 3 has

been arranged to print level 3, the drill spots, and level 4 the parts

outlines together.  This makes a useful combination.  Repeat the check print

process with each of the setups selected to get a working set.

The last thing we will do for this exercise is to make a set of plot files and

save the work.

We should still be in the plot menu.

F3PLTFIL

The system asks for a plot file name.  Our convention is that we name the

files ending in 1,2,3,... to indicate the level we are plotting.  The normal

procedure for a two sided plated through hole board would be to make 5 plots.

They are:

Level 1, the parts side.

Level 2, the wire or solder side.

Level 3, the drill spots.

Level 4, the parts outline, used for a silk screen and parts location.

Level 5, the solder mask.

We answer

GOO1 (ENTER)

We are asked for a level - we answer

1 (ENTER)

We are asked for the plotter steps per inch.  For 1 to 1 scale on most

plotters enter 1000.  Here is your chance to change the scale.  If your

plotter makes 400 steps per inch as do some low cost pen plotters you would

answer 400.  If you happen to know that your plotter has a scale factor error,

then a correction could be made here.  We will assume you are sending work to

us and enter 1000.

1000 (ENTER)

Now we are asked for offsets in x and y.  You could enter values here in

thousandths of an inch if you know that you want to relocate the drawing on

the plotting table.  For example, by giving each plot level a different offset

you can arrange to have all the levels plotted on a single piece of film.

Before these last entries are made be sure you have enough room on the default

disk to hold the plot files.  If in doubt, a fresh formatted disk is in order.

The system can die a horrible death if it cannot find enough room.  Some

versions of DOS die in a way that does not let you recover.  Best to be sure

you have enough room.

0 (ENTER)

0 (ENTER)

The disk whirrs and a file is created.  Later you will see that it is named

FOO1.PLT.  Repeat the above process naming the files FOO2 for level 2, FOO3

for level 3 etc..

Finally F9BACK get the menu with 2FILE.

F9BACK

F2FILE

F1PUTFIL

F1SAVJOB

And we are asked for a file name.  We will call this effort GOO.

GOO (ENTER)

The system advises us that it did not remember loading this job.  We are given

a chance to change our mind.  Entering anything but c will cause the present

work to be saved.  If we later reload this job we will be back where we are

now.  Any symbol sets loaded are saved with this job.  Remember we loaded the

library ELEC.  We will not have to reload it when we restart this job as it

will be saved as a part of this job.

(ENTER)

The disk whirrs and the job is saved.  Again, be sure there is room on the

disk.  Late versions of DOS give you another chance on a full disk.  Earlier

versions did sometimes.

Now that the work is saved, three F9BACK will get the question "are you sure"

We are, and enter y to quit.

An examination of the disk directory should show 7 new files.  There will be

five files with names like GOO1.PLT, GOO2.PLT,... These are the plot files.

Send them and money to us if you want photoplots.  There will be a pair of

files GOO.PCD, and GOO.PCS which are the files that save your work.  The PCD

file save all the working conditions, like the redraw areas you have named.

The PCS file contains all the data you have entered.

It has taken a lot of words to describe this very simple operation.  But that

is the way words work.  Most of you will get tired and quit following the step

by step operations as you pick up on the general idea of this system.  The

detail is here to help those that are lost .