STANDARD.DOC

This suggests a set of standards for a simple two sided board with plated

through holes.  The symbols in ELEC.LIB follow this standard and will save

some time in getting started.

Level 1

Used for the component side of the board.  We look at the board from the

component side and see through to the solder side.  We show this side in

green/cyan depending on the Palette switch setting.  Label this level "Parts

Side" or some such with normal lettering.

Level 2

Used for the solder side of the board.  We show this side in red/magenta

depending on the Palette switch setting.   Label this level "Solder Side" or

some similar legand but enter the label mirrored in x.

Level 3

Used to show drill spots.  We show the drill spots in green/cyan.  We suggest

that a different sized spot be used for each drill size.  We normally use 15

mil spots for most holes, which are actually drilled 0.031" for average work.

Other sized spots and a legend on the border will allow the vendor to make a

drill tape.

Level 4

Used to show component outlines.  We show the components in brown/white.

Level 5

Used for the solder mask.  We show the solder mask in green/cyan.

While you are free to set up your own standards, if you want us to plot a

demonstration file for you it will reduce the chance for confusion if you

follow the above.

We suggest that you name your plot files with names ending in 1,2,3,etc.. so

that they come out looking like MYPLOT1.PLT etc..  This way we will be less

likely to mirror the wrong plot if we plot for you.  We will normally plot side

1 mirrored, and side 2 direct.  This way the two sides will register emulsion

to emulsion for the least error.  This explains the mirroring of the labels.

When side 1 is plotted, it will be plotted mirrored so that when it is turned

over to put the emulsion which was plotted facing up, down against the board,

then the lettering which we are now viewing through the board will look

correct.  If you plot all the levels on one film, as described in the HOW TO

section, then we will not be able to mirror side one.  We will just plot it as

you send it.  This is OK unless you are trying to do very fine line work.

If you have not sent work to us before, it pays to send the job (.PCD, .PCS

files ) with the plot files.  Then we will probably be able to straighten out

any problem.

We find that everyone has different choices for colors.  There seems to be

great variations in the way each of us perceive color.  When we can, we will

allow  multi-color selection for the EGA version of this program.  If you

hate the IBM color set choices this is one more reason to buy the licensed

version.  Note that you can switch between the two IBM color choices (1,1,3,5).