BOOTP - resident BOOTP service.

Network managers will love this program as they can simply set a PC's IP number

once, perhaps in the autoexec.bat, and then every application which supports

BOOTP is automatically configured!  We use it here extensively.

This program is a 912 byte TSR that watches for any packet driver application

sending out a BOOTP request to a class 1 (Ethernet) packet driver.  When it

sees one, this program fakes an upcalls to make it seem like a real BOOTP

response.

Usage:

BOOTP options...

where options can be:

QUIET                  - so that is loads without screen activity

MYIP=ip_number

MY=ip_number

PC_IP=ip_number        - these 3 do the same thing, that is, set your local

IP address

SERV=ip_number         - sets IP address of the server supposedly responding

SIP=ip_number          - sets IP address of a gateway

HW=hardware_address    - physical address of the server supposedly responding

MASK=subnet_mask       - sets the subnet mask on magic cookie (63.82.53.63)

BOOTPs

NAME=ip_number         - sets the name server on magic cookie (63.82.53.63)

BOOTPs

GATE=ip_number         - sets the gateway on magic cookie (63.82.53.63) BOOTPs

EXTRA=xx,xx,xx,xx etc  - fills in the vendor field with more stuff on magic

cookie (63.82.53.63) BOOTPs

@filename              - a filename full of more options like above

(not recursive!)

- multiple @filename options can be specified on the

command line

The ip_number, hardware_address, and subnet_mask are specified

as nnn.nnn.nnn.nnn or a hexadecimal address like x628462FE

Other issues: The upcalls are done during the transmit.  So if an application

sends a BOOTP and immediately clears all received packets, it will not see the

BOOTP response.  This could be fixed by deferring the upcalls to a timer tick

mechanism, however, it works for everything I've tried so far anyway.

Example usage:

bootp my=129.97.20.102 @defaults

where the file 'defaults' contains:

serv=129.97.20.1

sip=129.97.20.1

hw=xc800c700

gate=129.97.20.1

name=129.97.129.140

name=129.97.128.152

mask=255.255.254.0

extra=8,4,129.97.128.152

Notice how 2 name servers are given.  Also, a quote server has been given

using the "extra" option.

You may use environment variables, eg.

set ip=129.97.20.102

bootp  my=%ip% @defaults

This software has been tested with:

CUTCP

WATTCP  (latest revision)

but it should work with most TCPs.

See RFC1084 and RFC951 for more information on BOOTP

Source code freely available.  Distribute freely.  Make any changes.

Please leave my name on it though.  Our update site is sunee.uwaterloo.ca

in pub/wattcp/bootp.zip

Written by Bruce Campbell

WATSTAR Office

Engineering Computing

University of Waterloo

bruce@development.watstar.uwaterloo.ca