STANDARD MICROSYSTEMS CORPORATION

Novell NetWare OS/2 Workstation Requester

For Ether 10/100 Fast Ethernet EISA Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  Preinstallation hardware requirements.

o  Adapter installation notes.

o  How to use EZStart to copy SMC's NetWare OS/2 ODI workstation

driver from the SMC SuperDisk to your PC.

o  How to install the driver using Novell's OS/2 requester

INSTALL program.

o  How to install multiple adapters.

o  Error messages.

PREINSTALLATION HARDWARE REQUIREMENTS

Hardware Requirements:

o  An IBM-compatible personal computer with 80386 or higher

microprocessor and PCI bus.

o  An EtherPower 10/100 Fast Ethernet, EtherPower Ethernet, or EtherPower^2

Dual Ethernet PCI adapter for that bus.

Firmware Requirements:

o  Fully compliant Rev. 2.0 or above PCI BIOS supplied with the PC.

ADAPTER INSTALLATION NOTES

Refer to the EtherPower 10/100, EtherPower, or EtherPower^2 user guides for

complete installation instructions.

When installing any of the supported Ethernet PCI adapter in

a PCI computer, make sure the slot you select supports bus master

operations.

o  Motherboards with bus master jumpers and interrupt jumpers for

PCI slots:

1. Enable bus master operations to the selected PCI slot using the

appropriate motherboard jumper.

2. Select interrupt to the selected PCI slot using the appropriate

motherboard jumper.

3. Enter your computer's setup menu and:

- Enable bus master on the EtherPower 10/100 PCI slot.

- Enable I/O on the EtherPower 10/100 PCI slot.

- Define interrupt number on the EtherPower 10/100 PCI slot.

o Motherboards with automatic PCI parameterization:

1. No specific setup is needed.

COPYING THE DRIVER FILES

From the EZStart Network Drivers and Documentation Selection

screen, select 'Novell NetWare OS/2 ODI Driver'. Then, when

the Network Drivers and Documentation screen appears, select

'Copy Files' to copy all the files used by this driver from

the SuperDisk to the Destination Directory recommended by

EZStart, or to a different directory that you specify.

LOADING THE DRIVER

Use the following instructions with those in Novell's NetWare

Requester for OS/2 manual:

1. Load the NetWare Requester INSTALL program according to the

instructions in the NetWare manual.

2. Select "Requester on workstation" from the menu.

3. Specify the source and target directories.

4. Select "Edit CONFIG.SYS and Copy Files".

5. Select "SMCPWRS.SYS" from the list. If the SMC driver is not in

the list, type: SMCPWRS.SYS

6. When prompted as to whether this is a third-party NIC driver,

select "Yes".

7. Select or deselect any remaining options, then select "Save".

In the "Save File As:" field, type: CONFIG.SYS

8.  In the Copy Requester Files window, select "Copy".

9.  Proceed with the instructions in the NetWare Workstation for

OS/2 manual until you complete the installation.

NET.CFG PARAMETERS

Edit NET.CFG file and check that the line speed, cable type, and

network protocol type settings are correct for your network.

Note that there is no need to specify port numbers or interrupt numbers.

You only need to specify PCI slot numbers if you are installing more than

one board.

INSTALLING MULTIPLE ADAPTERS

If two adapters are installed in the same workstation, modify the

NET.CFG file in the \NETWARE subdirectory on your PC. To modify the

file, use any text editor to open the file and add the following

lines:

LINK DRIVER SMCPWRS

SLOT X

LINK DRIVER SMCPWRS

SLOT X

In the above lines, X is the logical PCI slot number of the adapter starting

from 16. EISA slots are counted from 1 to 15. The first PCI adapter installed

in your computer will be treated as if it is installed in slot 16, the next

one will be in slot 17, and so on.

After you modify the file, copy it to the \NETWARE subdirectory or to

the directory where the SMCPWRS.SYS OS/2 ODI driver resides.

ERROR MESSAGES

"Adapter hardware failure detected (initialization)"

The adapter did not pass the initialization process.

Check the adapter for configuration conflicts.

"Adapter hardware failure detected (self-test)"

The adapter did not pass the initialization self-test process

due to a hardware problem. Check the adapter for configuration

conflicts.

"No Ethernet Adapters Found"

The driver could not detect an adapter installed in the system.

Check the adapter is inserted properly into the slot connector.

"Invalid Ethernet node address specified"

The node address specified in the NET.CFG file was illegal.

Broadcast and multicast addresses are not allowed as valid

node addresses. Use the node address override with caution,

as this can cause problems on your network.

"Cannot access adapter RAM properly"

The adapter could read or write the adapter RAM. Verify that

another adapter does not share the same memory space as the

LAN adapter.

"Multiple media types defined."

More than one media type (e.g. BNC, TP) are defined in the NET.CFG

file under the Link Driver SMCPWRS section. The last media type

keyword listed will be used in this case.

"Invalid line speed defined."

A value other than 100 or 10 is used for the Line_Speed keyword in

the NET.CFG file under the Link Driver SMCPWRS section. The driver

will load as if no Line_Speed keyword is specified in this case.

"Line Speed ignored when AUTODETECT is defined."

Both the Line_Speed and the AUTODETECT keyword are used in the

NET.CFG file under the Link Driver SMCPWRS section. The Line_Speed

keyword will be ignored in this case.

"The media type defined is not supported at 100Mbps."

A media type keyword such as BNC or AUI is defined together with

Line_Speed 100 in the NET.CFG file under the Link Driver SMCPWRS

section. The driver will not load since such media type is not

supported at 100Mbps. Change the media type or the Line_Speed keywords

in the NET.CFG file and try again.

"Adapter does not support Media type or Line speed defined in net.cfg."

The media type and/or Line_speed defined in the net.cfg file is not

supported by this adapter. Change the media type or Line_speed to

the proper value or use AUTODETECT.

"SROM contains invalid configuration data."

Contact technical support if you see this message.

This commercial computer software and documentation is provided

with RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of

the Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge, NY  11788.

SMC and Standard Microsystems are registered trademarks; and

EtherPower 10/100, EZStart, and SuperDisk are trademarks of Standard

Microsystems Corporation. Other products and company names are

registered trademarks or trademarks of their respective companies.