STANDARD MICROSYSTEMS CORPORATION

Packet Driver

For EtherPower, EtherPower^2 (Dual Channel)

and EtherPower 10/100

PCI Network Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  Basic packet driver use.

o  Software interrupts.

o  How to use the packet driver to connect to a NetWare 2.2 server

using the BYU packet driver shell.

o  How to use the packet driver to connect to a NetWare server

using the Intel packet driver shell.

o  Using the packet driver with SMC/WD adapters.

SUPPORTED ADAPTERS

o     EtherPower Ethernet PCI adapters (8432 board types)

o     EtherPower^2 Dual Channel Ethernet PCI adapters (8434 board types)

o     EtherPower 10/100 Fast Ethernet PCI adapters (9332 board types)

BASIC PACKET DRIVER USE

The following section describes basic packet driver use. Since

different networks have different requirements, you can include the

following switches in your command lines as needed.

-d    This switch delays the adapter's initialization until the first

time the packet driver is accessed. Use this switch if your

workstation loads the operating system over the network using a

Boot ROM on the SMC adapter.

-n    This switch configures the driver to change 8137-formatted

packets seen by the client into IEEE802.3 packets.  Unless you

use the BYU's NetWare packet driver shell, you can omit this

switch. If you want to use 8137 packets, use "econfig" to

configure Intel's PDIPX103 packet driver shell.

-p    This switch disables promiscuous mode. A small level of security

can be achieved by disabling promiscuous mode; however,

disabling promiscuous mode should not be mistaken for real

security.

-w    This switch is used with Microsoft Windows for nonresident DOS

TCP stacks, such as NCSA Telnet and PC-Gopher. Alternatively,

you can use the Windows freely copyable utility, winpkt.

-i    The SMC packet driver can be used with both Ethernet_II and

IEEE 802.3 frame types. The packet driver specification, however,

only allows a driver to report one class which, by default, is

Ethernet_II. If you use the packet driver with client software

that expects to find an IEEE 802.3 packet driver instead of an

Ethernet_II packet driver, use this switch to configure the

driver to use IEEE 802.3-formatted packets.

SOFTWARE INTERRUPTS

A packet driver provides services for client programs. These programs

access the packet driver by issuing software interrupts.

Any software interrupt between the hexadecimal addresses of 0x60

through 0x80 can be used with the packet driver, with the following

exceptions:

o  0x67 is reserved for the EMS specification.

o  0x70 through 0x77 are reserved for hardware IRQ lines 8 through 15.

After you choose a software interrupt, assign that interrupt to the

packet driver, and install the packet driver. The client programs

will then automatically locate the packet driver by searching for

the packet driver's signature at software interrupts from 0x60

through 0x80.

CONNECTING TO A NetWare 2.2 SERVER USING THE BYU PACKET DRIVER SHELL

If you want to connect to a NetWare 2.2 server that, by default, is

configured to use IEEE 802.3-type packets, use the following command

line examples.

PKTPWR -n 0x7e

BYUIPX

NETX

In this command line:

o  PKTPWR denotes the SMCPower adapter.

o  -n configures the driver to change 8137-formatted packets seen by

the client into IEEE802.3 packets.

o  Software interrupt setting 0x7e is the hexadecimal address 7E,

which allows packet driver clients to use interrupt 7E to

communicate with the packet driver.

CONNECTING TO A NetWare SERVER USING THE INTEL PACKET DRIVER SHELL

The following command line can be used to connect the SMC EtherPower adapters to  a NetWare 3.11, 3.12, 4.0, 4.01 or 4.10 server:

PKTPWR -i <0xPacket_int_no>  [<slot_num>]

[TP | BNC | AUI | TP_FULL_DUPLEX | TP_NO_LINK_TEST]

PDIPX

NETX

In this example:

o  PKTPWR denotes the SMCPower adapter.

o  -i configures the driver to use IEEE 802.3 formatted packets,

which the PDIPX protocol expects to receive.

o  <0xPacket_int_no> is the adapter's software interrupt setting,

and is prefaced by "0x" to indicate a hexadecimal number.

o  [<slot_num>] is the optional parameter for the slot number

where the adapter is placed in the machine.

o [TP | BNC | AUI | TP_FULL_DUPLEX | TP_NO_LINK_TEST] is

the optional parameter to specify the cable type.

A typical command line might resemble:

PKTPWR -i 0x7e

PDIPX

NETX

Note:   If you are using Ethernet II frame types, you must use

"econfig" to configure the PDIPX driver before loading it.

"econfig" is a step used to configure a shell.  For more

information, refer to our Novell Netware documentation.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherPower,

EtherPower^2, EtherPower 10/100, EZStart, and SuperDisk are

trademarks of Standard Microsystems Corporation. Other product and company

names are registered trademarks or trademarks of their respective holders.