STANDARD MICROSYSTEMS CORPORATION

Microsoft Windows for Workgroups v3.11 NDIS v3.0 Driver

For EtherPower, EtherPower^2, and EtherPower 10/100

PCI Network Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide:

o  Provides an overview.

o  Describes preinstallation hardware and software requirements.

o  Describes supplied files.

o  Provides adapter installation notes.

o  Describes how to use EZStart to copy the DOS NDIS3 driver files

for Microsoft Windows for Workgroups v3.11 from the SuperDisk to

your PC.

o  Describes how to install the driver.

IMPORTANT PROTOCOL.INI PARAMETERS

Configuration information is stored in the PROTOCOL.INI file. Two

important PROTOCOL.INI parameters are SIA_MODE, which defines the serial

interface connection (including full-duplex or half-duplex operation),

and SLOT, which specifies the PCI slot number location of the adapter.

Refer to the section in NDIS.DOC titled PROTOCOL.INI PARAMETERS

for further details.

EtherPower 10/100 Fast Ethernet Users Only

Ensure that the line speed (10 Mbps vs. 100 Mbps) configured

for the adapter matches the speed of the hub to which it is

attached. Set the adapter at the correct line speed

(10/100 Mbps) using the PROTOCOL.INI parameter DATA_RATE.

SUPPORTED ADAPTERS

o  EtherPower Ethernet PCI adapters (8432 board types)

o  EtherPower^2 Dual-Channel PCI Ethernet adapters (8434 board types)

o  EtherPower 10/100 Fast Ethernet PCI adapters (9332 board types)

OVERVIEW

This driver conforms to the Network Driver Interface Specification

(NDIS) v3.0 defined by Microsoft/3Com.

PREINSTALLATION HARDWARE AND SOFTWARE REQUIREMENTS

Hardware Requirements:

- An IBM-compatible personal computer with 80386 or higher

microprocessor and PCI bus.

- An EtherPower, EtherPower^2, or EtherPower 10/100 adapter

for that bus.

Software Requirements:

- Windows for Workgroups v3.11

SUPPLIED FILES

General files

o  RELEASE.DOC      Release notes for this version of the driver.

o  WIN_WKGP.DOC     This file.

Files to install the driver on Windows for Workgroups

o  SMCPWRW.386      NDIS3 driver.

o  OEMSETUP.INF     Network information file.

ADAPTER INSTALLATION NOTES

Refer to the user guide for complete adapter installation instructions.

When installing an SMC PCI adapter in a PCI machine, make sure the

slot you select supports bus master operations.

o  Motherboards with bus master jumpers and interrupt jumpers for

PCI slots:

1. Enable bus master operations to the selected PCI slot using

the appropriate motherboard jumper.

2. Select interrupt to the selected PCI slot using the appropriate

motherboard jumper.

3. Enter your computer's setup menu and:

- Enable bus master on the PCI slot.

- Enable I/O on the PCI slot.

- Define interrupt number on the PCI slot.

o Motherboards with automatic PCI parameterization:

1. No specific setup is needed.

COPYING THE DRIVER FILES

From the EZStart Driver Installation screen, select 'Copy Files' to

copy all of the files used by the Microsoft Windows for Workgroups

v3.11 driver from the SuperDisk to the Destination Directory

recommended by EZStart, or to a different directory that you specify.

The EZStart default Destination Directory is a:\NDIS.  Please make

a backup copy of your SuperDisk before EZStart copies files to the

SuperDisk to create an installation diskette.

INSTALLING THE DRIVER FILES

The following section describes how to install Windows for

Workgroups v3.11 along with the SMC driver file. If you already

have Windows for Workgroups installed and only want to install

the SMC driver, proceed to "Installing the SMC Driver Only".

Installing Windows for Workgroups and the SMC Driver

The following procedure describes how to install Windows for

Workgroups v3.11 along with the driver files. This procedure

is designed to expedite the driver installation. For information

on customizing your installation and Windows desktop, refer to

your Windows for Workgroups manual.

Note: To install an ODI or NDIS3 driver, you will need the

NETWARE.DRV file. You can download the current version

of this file from Novell's NetWire Forum on CompuServe.

1. Insert the Windows for Workgroups Disk 1 Setup diskette in your

computer's disk drive.

2. Change to the drive where you inserted the disk in step 1.

Then, at the DOS prompt, type: SETUP

3. Press the Enter key to start the setup procedure. Typically,

this procedure takes from 10 to 15 minutes.

4. o  Follow the screen prompts until the Network Setup window

appears.

o  Select "Advanced"

o  Select "Networks" and "Install Microsoft Windows Network".

o  Select "No additional network" or "Other", as necessary.

Refer to your Windows for Workgroups manual for information.

o  Select "Sharing" and specify whether you want other users to

be able to share your files and printer(s). Then select

"OK".

o  Select "Real Mode"

o  Select "Drivers".

o  Select "Add Adapter". The Add Network Adapter dialog box

appears.

o  Select "Unlisted or Updated Network Adapter" followed by

"OK".

5. When the Install Driver window appears, insert the SuperDisk

into your computer's disk drive.

6. In the Install Driver window, type the path where the

driver resides (for example, A:\).

7. Select "OK". The Unlisted or Updated Network Adapter window

appears.

8. Select the appropriate SMC adapter (8432 - EtherPower,

8434 - EtherPower^2, 9332 - EtherPower 10/100) you are using

followed by "OK".

9. If you are prompted for the adapter's Base I/O setting,

specify it and select "OK".

10. o  When the Network Drivers window appears, you can use

"Add Adapter" and/or "Add Protocol" to add more adapters

(up to four) and protocols. Refer to your Windows for

Workgroups manual for more information.

o  To change the Base I/O setting you specified in step 9

and/or select a driver type, select "Setup" and make the

appropriate selections.

o  Select "Real Mode".

o  Select "Advanced". You will be prompted with several options,

including Data Rate, Burst Length, and Bus Number. Click on

"Set" for each option. Click on "OK" to close the dialog box.

o  Select "Close" (or "Cancel") followed by "OK" (or "Continue").

o  In the Microsoft Windows Networks Names box, select "OK".

o  Follow the screen prompts to complete the driver

installation.

o  You will be prompted to provide the NETWARE.DRV file. Specify

the path where this file is located. If the file resides on a

floppy diskette, insert the diskette into an available disk

drive. Then press Enter.

11. At the end of the installation, you will be prompted whether

you want to accept all the options you selected and restart

your computer or return to DOS. Restart your computer so

your options can take effect.

Installing the SMC Driver Only

The following procedure describes how to install the SMC driver

and assumes that Windows for Workgroups v3.11 is already installed.

1.  Double-click on the Network Setup icon in the Network window.

2.  When the Network Setup dialog box appears, refer to "Installing

Windows for Workgroups and the SMC Driver".

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherPower,

EtherPower^2, EtherPower 10/100, EZStart, and SuperDisk are trademarks

of Standard Microsystems Corporation. Other product and company names

are registered trademarks or trademarks of their respective holders.