STANDARD MICROSYSTEMS CORPORATION

Microsoft Windows 95 NDIS v3.0 Driver

For EtherPower, EtherPower^2, and EtherPower 10/100

PCI Network Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  An overview.

o  Preinstallation hardware and software requirements.

o  Supplied files.

o  Adapter installation notes.

o  How to use EZStart to copy the Microsoft Windows 95

NDIS spec 3.0 driver from the SuperDisk to your PC.

o  How to install the driver.

o  Driver parameters.

SUPPORTED ADAPTERS

o  EtherPower Ethernet PCI adapters (8432 board types)

o  EtherPower^2 Dual-Channel PCI Ethernet adapters (8434 board types)

o  EtherPower 10/100 Fast Ethernet PCI adapters (9332 board types)

OVERVIEW

This file explains how to install the Windows 95 driver for x86-based

platforms.

PREINSTALLATION HARDWARE AND SOFTWARE REQUIREMENTS

Hardware Requirements:

- An IBM-compatible personal computer with 80386 or higher

microprocessor and PCI bus.

- An EtherPower, EtherPower^2, or EtherPower 10/100 PCI

adapter for that bus.

Software Requirements:

- Windows 95.

SUPPLIED FILES

General files

o  WN95INST.TXT     This file.

o  RELEASE.DOC      Release notes for this version of the driver.

Files to install the driver on Windows 95

o  SMCPWRN.SYS      The driver.

o  NETPCI.INF       The driver's setup file.

o  DISK.ID          A tag file for Windows 95, make sure it is in the

root directory from which you are installing.

O  NDIS.VXD         The NDIS wrapper.

ADAPTER INSTALLATION NOTES

Refer to the user guide for complete installation instructions.

When installing an SMC PCI adapter in a PCI computer, make sure

the slot you select supports bus master operations.

o  Motherboards with bus master jumpers and interrupt jumpers for

PCI slots:

1. Enable bus master operations to the selected PCI slot using the

appropriate motherboard jumper.

2. Select interrupt to the selected PCI slot using the appropriate

motherboard jumper.

3. Enter your computer's setup menu and:

- Enable bus master on the PCI slot.

- Enable I/O on the PCI slot.

- Define interrupt number on the PCI slot.

o Motherboards with automatic PCI parameterization:

1. No specific setup is needed.

INSTALLING THE ADAPTER

To ensure the correct loading of the SMC 8432 enhanced mode adapter

and driver within Windows 95, the following steps should be taken.

The driver's network NETPCI.INF file on the SuperDisk must be substituted

for the network NETPCI.INF file that is included on your Windows 95 CD.

Do these steps before installing your 8432 adapter.

1) Start up Windows 95 WITHOUT the SMC adapter installed.

2) a. Click Start.

b. Move the mouse pointer to Programs.

c. When the Programs subtitles appear, move the mouse pointer to

MS-DOS Prompt and click.

3) A DOS box will appear with the cursor in the windows directory.

Type CD\INF to enter the INF directory.

4) Rename the NETPCI.INF file with the command

REN NETPCI.INF NETPCI.SAV.

5) Type exit to leave the DOS box. Click on Start. Click on shutdown.

A box with the highlighted message "Shut down the computer?"

should appear. Click on yes. Turn off the machine when the

message to do so appears.

6) Install the SMC 8432 enhanced board in a PCI slot, attach network

cable, and power up the machine. Windows 95 starts to boot up.

When the message "Starting Windows 95" appears, hit F8 key. A

startup menu will appear. Choose option 6.

COPYING THE DRIVER FILES

A) Insert the SuperDisk into the floppy drive. Type A:EZSTART.

Ezstart will show the present configuration of the 8432 adapter.

B) Select Custom Configuration.

C) Select Drivers/Documents.

D) Select Microsoft for the network operating system vendor.

E) Select Windows 95.

F) Choose an appropriate destination directory to place the

uncompressed files. The recommended destination directory is

"A:\". Select Copy files.

G) When the file copying is complete, the message "To install the

driver...." appears, select OK and EXIT. After exiting Ezstart,

type WIN to enter Windows 95.

INSTALLING THE SMCPWRN 95 DRIVER

7) The 8432 adapter will be detected. A message box will appear.

Be sure the message "Driver from disk provided by manufacturer"

is highlighted. Click okay. Type path to driver and inf file and

press return key.

8) A message "Please insert the disk labeled 'PCI Netcard Drivers

Directory" will appear. Click OK.

9) The driver will now load. If an SMCPWR.DOS driver is available, it

will load. If not, click Skip File when prompted to do so.

10) The message "Please insert the disk labeled 'Windows 95 CD-ROM'

and then click o.k." will appear. Click o.k.

11) Windows 95 will now begin to load the network files. If the

message that a certain file cannot be found appears, type the

path to that file. It will be the path to the installation

files (the CD). When all files are loaded, remove all

diskettes, click yes at the restart message. The system will

restart and boot up into Windows 95. A log on message will

appear.

12) Restart the machine. When the message Starting Windows 95 appears,

enter F8 key. Choose option 6, Command Prompt. Copy NDIS.VXD from the

SuperDisk to the following directory C:\WINDOWS\SYSTEM. Click YES when

prompted.

13) Restart the machine.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherPower,

EtherPower^2, EtherPower 10/100, EZStart, and SuperDisk are trademarks

of Standard Microsystems Corporation. Other product and company names are

registered trademarks or trademarks of their respective holders.