STANDARD MICROSYSTEMS CORPORATION

DOS ODI Driver

For EtherPower, EtherPower^2, and EtherPower 10/100

PCI Network Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  Preinstallation hardware and software requirements.

o  Supplied files.

o  Adapter installation notes.

o  How to use EZStart to copy the SMCPWR.COM DOS ODI driver from the

SuperDisk to your PC.

o  How to setup an SMC Ethernet workstation to communicate with

both Novell NetWare 386 and IBM AS/400 networks concurrently.

o  How to modify the line speed (EtherPower 10/100 only).

For custom installations involving multiple frame types,

refer to your Novell NetWare ODI Shell for DOS manual.

SUPPORTED ADAPTERS

o  EtherPower Ethernet PCI adapters (8432 board types)

o  EtherPower^2 Dual-Channel PCI Ethernet adapters (8434 board types)

o  EtherPower 10/100 Fast Ethernet PCI adapters (9332 board types)

ETHERPOWER 10/100 USERS:

Ensure that the line speed (10 Mbps vs. 100 Mbps) configured

for the adapter matches the speed of the hub to which it is

attached. The EtherPower 10/100 adapter is default configured

to operate at 100 Mbps.

Configuration information is stored in the NET.CFG file.

It is important to set the adapter at the correct line speed

(10/100 Mbps) using the Line_Speed parameter. Refer to the

MODIFYING THE LINE SPEED section later in this chapter.

PREINSTALLATION HARDWARE AND SOFTWARE REQUIREMENTS

Hardware Requirements:

o  An IBM-compatible personal computer with 80386 or higher

microprocessor and PCI bus.

o  An EtherPower, EtherPower^2, or EtherPower 10/100

PCI adapter for that bus.

Firmware Requirements:

o  Fully compliant Rev. 2.0 or above PCI BIOS supplied with the PC.

Software Revision Requirements:

o  MS-DOS v3.0 or above.

o  Novell LSL (provided on this SuperDisk).

o  Novell IPXODI (available from Novell).

o  Novell NETX (available from Novell).

or

o  Novell VLM (available from Novell).

SUPPLIED FILES

Files provided in the Novell NetWare DOS ODI Client Directory

include:

o  INSTALL.DOC     This file.

o  RELEASE.DOC     Provides a description of updates to the driver.

o  SMCPWR.COM      SMCPWR Novell NetWare ODI Client driver.

o  NET.CFG         Novell network configuration file for the driver.

o  LSL.COM         Needed for Novell NetWare ODI DOS protocol stack.

o  PROTOCOL.INI    Needed for Novell NetWare ODINSUP protocol support.

ADAPTER INSTALLATION NOTES

Refer to the user guide for complete installation instructions.

When installing an SMC PCI adapter in a PCI machine, make sure the

slot you select supports bus master operations.

o  Motherboards with bus master jumpers and interrupt jumpers for

PCI slots:

1. Enable bus master operations to the selected PCI slot using

the appropriate motherboard jumper.

2. Select interrupt to the selected PCI slot using the

appropriate motherboard jumper.

3. Enter the setup menu of your machine and:

- Enable bus master on the PCI slot.

- Enable I/O on the PCI slot.

- Define interrupt number on the PCI slot.

o Motherboards with automatic PCI parameterization:

1. No specific setup is needed.

COPYING THE DRIVER FILES

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files used by this driver from the SuperDisk to the

Destination Directory recommended by EZStart, or to a different

directory that you specify.

Note: You can also find the NetWare DOS ODI shell drivers on the

Novell supplied DOS ODI workstation diskette.

SETUP PROCEDURE

1. Copy the following files to your workstation:

LSL.COM

ODINSUP.COM

IPXODI.COM

NETX.COM

These files may be acquired from Novell Technical Support or

from the Novell section on CompuServe (type GO NOVELL at the

CompuServe prompt, currently the DOSUP7.ZIP download).  When

copying these files to your workstation, it does not matter

under which subdirectory you place them.  For convenience,

copy all these files to the same directory.  \LSP is the

subdirectory that we recommend.

2. Check to see that the IBM LAN Support Program has correctly

installed the following files in the \LSP subdirectory of

your workstation:

NETBIND.EXE

PROTMAN.DOS

DXMEOMOD.SYS

DXMAOMOD.SYS

If they have not been installed, copy them from the LAN Support

Program diskette to your workstation.  The DXMEOMOD.SYS and

DXMAOMOD.SYS files should be v1.3 or higher.

3. Ensure that the following lines are correct in your system

NET.CFG under the Protocol and Link Driver sections:

Protocol ODINSUP

Bind SMCPWR

BUFFERED

Link Driver SMCPWR

Frame Ethernet_802.3

Frame Ethernet_802.2

Frame Ethernet_II

Frame Ethernet_SNAP

Protocol IPX 0 Ethernet_802.3

You need all four frame types for the system to load properly.

The Protocol IPX line is only necessary when the 802.3 frame

type is not the first entry.

4. Using a text editor, open the PROTOCOL.INI file located on your

workstation.  Usually, this file is in the \LSP directory.

Make a backup of the current PROTOCOL.INI file and then edit

PROTOCOL.INI to add the following lines:

[PROTMAN_MOD]

DriverName = PROTMAN$

[DXMEO_NIF]

DriverName = DXMEO$

Bindings = SMCPWR

5. Using a text editor, edit the CONFIG.SYS file on your

workstation to add the following lines:

device=\lsp\protman.dos

device=\lsp\dxma0mod.sys 001

device=\lsp\dxme0mod.sys

device=\lsp\eimpcs.sys

device=\lsp\ecyddx.sys files=8 drives=6

6. Using a text editor, edit the AUTOEXEC.BAT file on your

workstation to add the following lines:

cls

cd\ (add the directory name where SMCPWR resides)

lsl

SMCPWR

ipxodi

odinsup

netbind

netx

f:

7. Reboot the workstation.

The workstation displays the F: prompt.

Your workstation can now communicate with both Novell NetWare 386

and AS/400 networks.

MODIFYING THE LINE SPEED (ETHERPOWER 10/100 ONLY)

The NET.CFG file contains various configuration parameters for use

with the adapter. The default line speed for the EtherPower 10/100

adapter is 100 Mbps. Therefore, make no changes to the Line_Speed

parameter in the NET.CFG file if you wish to attach to a 100 Mbps hub.

To operate at 10 Mbps, remove the comment (;) indicator from the line:

Line_Speed=10

to make 10 Mbps the active line speed selection. Other parameters,

such as the active serial interface, are also specified using this

file.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherPower,

EtherPower^2, EtherPower 10/100, EZStart, and SuperDisk are trademarks

of Standard Microsystems Corporation. Other product and company names are

registered trademarks or trademarks of their respective holders.