STANDARD MICROSYSTEMS CORPORATION

DOS and OS/2 NDIS2 Drivers

For EtherPower, EtherPower^2, and EtherPower 10/100

PCI Network Adapters

RELEASE INFORMATION

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

Version 2.00         22 June 1995

This release information for the SMCPWR NDIS2 Driver describes:

o  Interrupt sharing.

o  The version history of the DOS and OS/2 NDIS drivers.

The General Installation Guide (NDIS.DOC) can be found in the NDIS2

directory of the SuperDisk. Specific installation instructions for

various network operating systems can be found in the appropriate

subdirectories under the NDIS2 directory.

SUPPORTED ADAPTERS

o  EtherPower Ethernet PCI adapters (8432 board types)

o  EtherPower^2 Dual-Channel PCI Ethernet adapters (8434 board types)

o  EtherPower 10/100 Fast Ethernet PCI adapters (9332 board types)

INTERRUPT SHARING

This version of the driver does not support interrupt sharing.  Do not

install an SMC PCI adapter together with another adapter that uses the

same interrupt level.

VERSION HISTORY

-- Version 2.00 (Current Version) --

The driver now supports Win95, IBM Lan Server versions 3.0 and 4.0,

and IBM OS/2 V3.0 (WARP).

Added an autosensing mechanism for either 10Mb or 100Mb for the

9332.

Added a PROTOCOL.INI keyword "TX_THRESHOLD".

-- Version 1.04 --

Added multiple adapter support.

-- Version 1.03 --

Added support for the new 8432 & 9332 with updated EEROMs.

Supports serial ROM format version 1.00.

Driver can now support promiscuous mode.  It can be invoked by the

this driver using the SetPacketFilter NDIS2 call.

Added a PROTOCOL.INI keyword, "TURBO", to be used with the INTEL

host-to-PCI bridges.  This may improve networking performance.

Removed PROTOCOL.INI keywords which enable the user to set values

to the DC21140 (pass 1.1) general purpose register.  These will

now be stored in the new serial ROM.

Fixed a problem with MS TCP/IP for windows.

-- Version 1.02 --

Added two PROTOCOL.INI keywords which enable the user to define

the number of transmit and receive buffers the driver will

allocate and use.

Added a PROTOCOL.INI keyword which will enable the user to set

values to the DC21140 (pass 1.1) general purpose register.

Modified the driver reset mechanism in order to avoid problems in

the windows environment.

-- Version 0.32 --

Initial release.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherPower,

EtherPower^2, EtherPower 10/100 and SuperDisk are trademarks of

Standard Microsystems Corporation. Other product and company names are

registered trademarks or trademarks of their respective holders.