SMCPWRW fullmac driver for Windows for Workgroups [tm] 3.11

Version v2.42

24-Aug-1995

1. Copyright Notice:

First Edition, April 1994

Updated, December 1994

Updated, February 1995

Updated, April 1995

Digital Equipment Corporation makes no representations that

the use of its products in the manner described in this

publication will not infringe on existing or future patent

rights, nor do the descriptions contained in this

publication imply the granting of licenses to make, use, or

sell equipment or software in accordance with the

description.

Copyright Digital Equipment Corporation 1994.

All rights reserved.

Do not redistribute the driver files.

Intel is a trademark of Intel Corporation.

Windows and Windows for Workgroups are trademarks of

Microsoft Corporation.

Microsoft, MS, MS-DOS are registered trademarks of

Microsoft Corporation.

DEC, DECchip, DECnet, Digital, PATHWORKS,

ThinWire, and the DIGITAL logo, are  trademarks of

Digital Equipment Corporation.

All other trademarks and registered trademarks are the

property of their respective holders.

2. Overview:

This file explains how to install the Windows for Workgroups

driver for x86-based platforms.  The driver is currently supported

for SMC's EtherPower Ethernet and Fast Ethernet adapters

3. Requirements:

A.  Hardware Requirements:

80386 or higher microprocessor based computer

with an EISA or PCI bus.

EtherPower PCI adapter

B.  Software Revision Requirements:

Windows for Workgroups Version 3.11 or later

4. Files and directories:

Table 1:  The Server Directory Files

---------------------------------------------------------------------

INSTALL.DOC       This file.

---------------------------------------------------

SMCPWRW.386       The driver.

---------------------------------------------------

OEMSETUP.INF      The driver's setup file.

---------------------------------------------------------------------

5. Installing the Hardware Boards:

A. Installing SMC's EtherPower PCI Ethernet Controller:

Refer to SMC's EtherPower the board's User Guide.

6. Installing the SMCPWRW Driver on Windows for Workgroups:

1. In the Windows for Workgroups Network Group, open the Network Setup option.

2. In the Network Setup applet, press the [Drivers...] button.

3. In the Network Drivers applet, press the [Add_Adapter...] button.

4. Select the [Unlisted or Updated Network Adapter] option and click

[OK]

5. When the Insert Driver dialog box prompts you for a disk, insert the

driver installation diskette into the drive, press the [Browse...]

button and select the i386\wfw sub_directory.

6. Select the adapter that matches your hardware

from the Network Adapters list

[SMC8432 EtherPower PCI Adapter      ]

[SMC8432e EtherPower PCI Adapter     ]

[SMC8434 EtherPower2 PCI Adapter     ]

[SMC9332 EtherPower10/100 PCI Adapter]

7. For PCI adapters, the driver's [Setup..] option allows the user

to change the following driver's properties:

o BurstLength:    Specify the length of the DMA data transfer within

a PCI burst (1dword = 4 bytes)

1_dword

2_dwords

4_dwords

8_dwords  (default)

16_dwords

32_dwords

o Bus Number:     Specify the PCI Bus Number the adapter is plugged in.

Should be modified only for adapter plugged over a Pci

to Pci bridge.

0  (default)

1

2

o DataRate: Specify the selected Media port

------------------------------------------------------------------------

SMC 8432

-------------------------------------------------------------------------

o AutoDetect                  In AutoDetect mode the driver

checks first the Twisted Pair port.

If the Link Test fails the driver

checks the BNC port. If no carrier is

detected the driver switches to the AUI

port.

o Twisted_Pair (10BaseT)

o Twisted_Pair Full_Duplex    To connect to Full Duplex hubs

o Twisted_Pair No_Link_Test   To connect hubs not supporting

the Link Integrity Test

o BNC (10Base2)               BNC connector

o AUI (10Base5)              15_pin connector

------------------------------------------------------------------------

SMC 9332

-------------------------------------------------------------------------

o AutoDetect                 In AutoDetect mode the driver

checks the 100BaseTx and 10BaseT ports.

If no link is detected the driver

selects the 100BaseTx port.

o 100Mbps                    To connect 100BaseTx Ethernet hubs

o 100Mbps Full_Duplex        To connect 100BaseTx Full Duplex hubs

o 10Mbps                     To connect 10BaseT Ethernet hubs

o PciMechanism:   Specify the mechanism access to the PCI

configuration space.

o AutoDetect                  Detect the system's PCI mechanism

automatically

o Mechanism 1                       Force PCI mechanism #1 accesses to the

PCI configuration space

o Mechanism 2                       Force PCI mechanism #2 accesses to the

PCI configuration space

o ReceiveBuffers:  Specify the size of the receive buffer's ring

8 .. 64  (default 16)

o MapRegisters = REG_DWORD {4,8,16}   (default: 8)

Number of map registers allocated

by the adapter.

o SnoozeMode = REG_DWORD { 0 | 1 }  (DC21041 only)

CFDA<SM>    0 Disabled (default)

1 Enabled

o StoreAndForward = REG_DWORD { 0 | 1}  (DC21140 only)

Select the StoreAndForward mode if a low PCI

bandwith causes Transmit Underruns

CSR6<SF>    1 StoreAndForward mode

o TransmitThreshold = REG_DWORD { 72 | 96  | 128 | 160 |

256 | 512 | 1024        }

CSR6<TTM,TR>  72 bytes

96 bytes (DC2104X's default)

128 bytes

160 bytes (DC21140's default)

256 bytes (DC21140 only)

512 bytes (DC21140 only)

1024 bytes (DC21140 only)

8. Click [Close] and [OK] to complete the driver installation.

9. Shutdown and reboot the system.

10. Installing multiple PCI boards:

Repeat the procedure above.

7. Error messages

The error messages are logged in the Entry Log.

Each error message has a standard Ndis error code and an driver's additional code.

The additional codes are:

DC21X4_ERRMSG_INIT_INTERRUPT      0x01

DC21X4_ERRMSG_CREATE_FILTER       0x02

DC21X4_ERRMSG_ALLOC_MEMORY        0x03

DC21X4_ERRMSG_INITIAL_INIT        0x04

DC21X4_ERRMSG_ALLOC_OPEN          0x05

DC21X4_ERRMSG_NETWORK_ADDRESS     0x06

DC21X4_ERRMSG_SYSTEM_ERROR        0x07

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT  0x08

DC21X4_ERRMSG_OPEN_DB             0x09

DC21X4_ERRMSG_DEVICE_ID           0x0A

DC21X4_ERRMSG_INIT_DEVICE         0x0B

DC21X4_ERRMSG_SROM                0X0C

7.1 Driver initialization errors:

o NDIS_ERROR_CODE_ADAPTER_NOT_FOUND

DC21X4_ERRMSG_DEVICE_ID

No board matching the AdapterCFID Registry's key value was found

plugged into the PCI bus

o NDIS_ERROR_CODE_OUT_OF_RESOURCES

DC21X4_ERRMSG_ALLOC_MEMORY

Not enough memory to allocate space for the Adapter data block

o NDIS_ERROR_CODE_INVALID_VALUE_FROM_ADAPTER

DC21X4_ERRMSG_SROM

Invalid data read from the adapter's serial ROM

o NDIS_ERROR_CODE_NETWORK_ADDRESS

DC21X4_ERRMSG_NETWORK_ADDRESS

No readable burnt_in or software configured station address

o NDIS_ERROR_CODE_OUT_OF_RESOURCES

DC21X4_ERRMSG_CREATE_FILTER

Ethernet Filter initialization failed

o NDIS_ERROR_CODE_INTERRUPT_CONNECT

DC21X4_ERRMSG_INIT_INTERRUPT

Interrupt initilization failed

o NDIS_ERROR_CODE_TIMEOUT

DC21X4_ERRMSG_INIT_DEVICE

DC21x4 Cam initialization failed

o NDIS_ERROR_CODE_OUT_OF_RESOURCES

DC21X4_ERRMSG_ALLOC_OPEN

Not enough memory space to allocate space for the open binding.

o NDIS_ERROR_CODE_OUT_OF_RESOURCES

DC21X4_ERRMSG_OPEN_DB

Not enough memory space to allocate space for the Ethernet filter

7.2 Driver run time errors:

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_SYSTEM_ERROR

System_Error interrupt (bus parity error)

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT

Transmit Jabber timer expired