STANDARD MICROSYSTEMS CORPORATION

Artisoft's LANtastic Network Software v6.0 Driver

For EtherPower, EtherPower^2, and EtherPower 10/100

PCI Network Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide:

o  Provides an overview.

o  Describes supplied files.

o  Describes how to use EZStart to copy the driver files for

Artisoft's LANtastic Network Software v6.0 from the SuperDisk

to your PC.

o  Describes how to install the driver files for Artisoft's LANtastic

Network Software v6.0.

o  Provides sample file contents after LANtastic Network Software

v6.0 is installed.

NOTE: Refer also to the NDIS.DOC file located in the NDIS2 directory

for complete information regarding DOS and OS/2 NDIS2 drivers.

NDIS.DOC describes:

o  An overview of the NDIS2 driver.

o  Preinstallation hardware and software requirements.

o  Adapter installation notes.

o  Basic driver installation (for any network operating system).

o  Installing multiple adapters.

o  How to define line speed, mode of operation, PCI slot

number, and other parameters via PROTOCOL.INI.

IMPORTANT PROTOCOL.INI PARAMETERS

Configuration information is stored in the PROTOCOL.INI file. Two

important PROTOCOL.INI parameters are SIA_MODE, which defines the serial

interface connection (including full-duplex or half-duplex operation),

and SLOT, which specifies the PCI slot number location of the adapter.

Refer to the section in NDIS.DOC titled PROTOCOL.INI PARAMETERS

for further details.

EtherPower 10/100 Fast Ethernet Users Only

Ensure that the line speed (10 Mbps vs. 100 Mbps) configured

for the adapter matches the speed of the hub to which it is

attached. Set the adapter at the correct line speed

(10/100 Mbps) using the PROTOCOL.INI parameter DATA_RATE.

SUPPORTED ADAPTERS

o  EtherPower Ethernet PCI adapters (8432 board types)

o  EtherPower^2 Dual-Channel PCI Ethernet adapters (8434 board types)

o  EtherPower 10/100 Fast Ethernet PCI adapters (9332 board types)

OVERVIEW

This driver conforms to the Network Driver Interface Specification

(NDIS) v2.0.1 defined by Microsoft/3Com. The driver has been tested

with Microsoft LAN Manager, 3Com 3+Open, IBM LAN Server, IBM PC LAN

Program, DEC Pathworks, and other NDIS-compatible network operating

systems and protocol stacks.

SUPPLIED FILES

General files

o  RELEASE.DOC      Release notes for this version of the driver.

o  LNTSTC60.DOC     This file.

Files to install the driver.

o  SMCPWR.DOS       DOS NDIS2 driver.

o  PROTOCOL.INI     Includes driver binding information and

operational parameters.

COPYING THE DRIVER FILES

From the Driver Installation screen, select 'Copy Files' to copy all

the files used by this driver from the SuperDisk to the Destination

Directory recommended by EZStart, or to a different directory that

you specify.

Make a backup copy of your SuperDisk before EZStart copies files to

the SuperDisk.

INSTALLING THE DRIVER FILES

The following section describes how to install Artisoft's LANtastic

Network Software v6.0 along with the SMC driver file. If you

already have Artisoft's LANtastic Network Software v6.0 installed

and only want to install the SMC driver, proceed to "Installing the

SMC Driver Only".

Installing Artisoft's LANtastic Network Software v6.0 and the

SMC Driver

The following procedure describes how to install Artisoft's LANtastic

Network Software v6.0 along with the driver files. This procedure is

designed to expedite the driver installation.

1. Edit the PROTOCOL.INI file you copied from EZStart so it

resembles the following:

[SMCPWR_NIF]

Drivername = SMCPWR$

2. Insert Artisoft's LANtastic Network Software version 6.0

Disk 1 of 4 in your computer's disk drive.

3. Type:  a:\install

4. Press the Enter key to start the setup procedure.

5. Follow the on-screen prompts.

6. When the Select Network Adapter Window prompts you for a LAN

driver, select "NDIS Support for Network Adapters" followed

by "OK".

7. When you are prompted for the manufacturer's driver diskette,

insert the SuperDisk in your computer's disk drive. Then

specify the path where the modified PROTOCOL.INI resides on the

SuperDisk.

Note: If you used EZStart to copy the driver to a different

Destination Directory, specify that directory's path in step

7. If that directory resides on a diskette other than the

SuperDisk, insert that diskette in your computer's disk

drive.

8. Select "OK".

9. Proceed with the rest of the LANtastic Network Software

v6.0 driver installation procedure.

10. At the end of the installation procedure, reboot your computer.

Installing the SMC Driver Only

The following procedure describes how to install the SMC driver

and assumes that Artisoft's LANtastic Network Software v6.0 is

already installed.

1.  Double-click on the Control Panel icon in the Main window.

2.  Double-click on the Network icon.

3.  Select the Adapters icon in the Options area.

4.  Select the adapter you want to remove followed by "Remove".

5.  When a message asks whether you are sure you want to remove

the adapter, select "Yes".

6.  Select "Add". The Install Network Adapter dialog box appears.

7.  Select the SMC adapter you want to install followed by "OK".

The Install Driver dialog box appears.

8.  Type the path where the driver resides (for example, A:\)

and select "OK". The Unlisted or Updated Network Adapter

dialog box appears.

9.  Select "OK" when the next two dialog boxes appear.

10. Select "Close". The Network Settings dialog box appears.

11. Select "OK". You will be prompted whether you want to accept

all the options you selected and restart your computer or

return to DOS. Restart your computer so your options can take

effect.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherPower,

EtherPower^2, EtherPower 10/100, EZStart, and SuperDisk are trademarks

of Standard Microsystems Corporation. Other product and company names are

registered trademarks or trademarks of their respective holders.