STANDARD MICROSYSTEMS CORPORATION

Microsoft Windows 95 NDIS v3.0 Driver

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the driver files for Microsoft Windows

95 from the SuperDisk.

o  How to update the Windows 95 NETSMC.INF file with the

NETSMC.INF file provided on the SuperDisk.

o  How to install the driver files under Windows 95.

Note: This driver supports all 8216 (Ultra), 80xx (Elite),

80xx (PLUS) Series, and 8416 (EtherEZ) ISA and Micro

Channel Ethernet adapters.

COPYING THE DRIVER FILES USING EZSTART

Note: It is recommended that you make a backup copy of your

original SuperDisk and use the backup copy for the installation.

1. Boot the computer to the DOS prompt.

2. Insert the SuperDisk into a floppy disk drive.

3. Change to the floppy drive where the SuperDisk is installed.

For example, if the disk is in drive A, type "A:" and press Enter.

4. From the DOS prompt, type "EZStart" (or "EZStart /t" for text

mode) and press Enter.

5. Select the "Custom" button from the EZStart main screen.

6. Select the "Basic" button to run the basic adapter tests.  After

successful completion, select the "Previous" button to get back

to the custom installation.

7 Select the "Drivers/Documents" button.

8. At the Network Drivers and Documentation Selection screen,

select the appropriate vendor depending on the network operating system

you are using (make sure the selection you want is highlighted).

Then press the Enter key or select "OK".

9. Select the appropriate network driver to copy to your system.

10. A default destination directory and a list of files to be copied

are displayed.  To accept the default destination directory select

"Copy Files".

11. Select "Exit" to exit EZStart.

12.  At the DOS prompt change to the C:\WINDOWS\INF directory with the

command

C:> CD C:\WINDOWS\INF

13. Rename the NETSMC.INF file with the command

C:> REN NETSMC.INF NETSMC.SAV.

14. Copy the new NETSMC.INF file from the SuperDisk with the command

C:> COPY A:\NETSMC.INF

15. Remove the SuperDisk from the floppy disk drive and turn off the

machine.

16. Install the SMC 8416 adapter, attach network cable, and power

up the machine. Windows 95 starts to boot up.  Make sure your

Windows 95 CD ROM is in the CD ROM drive.

INSTALLING THE DRIVER FILES

The following procedure describes how to install the SMC driver

and assumes that Windows 95 is already installed.  For information

on customizing your installation and Windows desktop, refer to

your Windows 95 manual.

For Plug and Play Adapters:

1. Windows 95 will notify you that it has detected "New Hardware..."

and begins installing files from the Windows 95 CD ROM.

2. When prompted for an SMC driver, insert the SuperDisk into the floppy

disk drive.

3. At the "Copy files from" prompt, enter the drive letter where the disk is inserted

For example: A:\

4. When prompted to restart the computer to finish the installation,

click on the "Yes" button.

For Non-Plug and Play Adapters:

1.  Double-click on the My Computer icon in the Main window.

2.  Double-click on the Control Panel icon.

3.  Double-click on the Add New Hardware icon.

4.  Click on the Next button.

5. Select "Yes" to let Windows 95 search for the new adapter and

click on the Next button.

6. Click on Finish to finish installing the new adapter.

7. Windows 95 begins installing files from the Windows 95 CD ROM.

8. When prompted for an SMC driver, insert the SuperDisk into the floppy

disk drive.

9. At the "Copy files from" prompt, enter the drive letter where the disk is inserted

For example: A:\

10. When prompted to restart the computer to finish the installation,

click on the "Yes" button.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, EliteView, EZStart, SuperDisk, and UltraChip

are trademarks of Standard Microsystems Corporation.  Other product

and company names are registered trademarks or trademarks of their

respective holders.