STANDARD MICROSYSTEMS CORPORATION

Microsoft Windows NT v3.5 and v3.51 NDIS v3.0 Driver

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the Microsoft Windows NT v3.5

and v3.51 NDIS spec 3.0 driver from the SuperDisk to

your PC.

o  How to install the driver.

Note: This driver supports all 8216 (Ultra), 80xx (Elite),

80xx (PLUS), and 8416 (EtherEZ) Series ISA,

and Micro Channel Ethernet adapters.

COPYING THE DRIVER FILE USING EZSTART

1. Boot the computer to the DOS prompt.

2. Insert the SuperDisk into a floppy disk drive.

3. Change to the floppy drive where the SuperDisk is installed.

For example, if the disk is in drive A, type "A:" and press Enter.

4. From the DOS prompt, type "EZStart" (or "EZStart /t" for text

mode) and press Enter.

5. Select "Automatic" from the EZStart main screen to perform the

Basic Adapter Test.

6. Select "Save" to save the selected configuration and continue.

7. At the Network Drivers and Documentation Selection screen,

select the appropriate driver you want to install (make sure the

selection you want is highlighted).  Then press the Enter key or

select "OK".

8. At the Network Drivers and Documentation screen, the default

destination directory and a list of files to be copied to the

default directory are displayed.  To accept the default

destination directory select "Copy Files".  Otherwise, enter a

destination directory at the prompt and select "Copy Files".

9. Follow the prompts to continue the installation.

10. Remove the Superdisk from the floppy disk drive.

11. Select "Exit" to exit EZStart.

This completes the driver installation procedure.

MULTIPLE SMC ETHERNET ADAPTERS

If you have more than one SMC adapter in your system, EZStart

will display each adapter with its configuration.  Select an

individual adapter by double-clicking on it, or by highlighting it

and either choosing "Select" or pressing the Enter key.  The Ezstart

Main screen will appear..

The EZStart default Destination Directory is A:\. Please make

a backup copy of your SuperDisk before EZStart copies files to the

SuperDisk to create an installation diskette.

Note:  If you change the Destination Directory, make sure that the

DISK.ID file is in the root directory of the disk that you

are using, and that the OEMSETUP.INF and SMC8000N.SYS files

reside in the root directory or a subdirectory on the disk

you are using.

INSTALLING THE DRIVER

1. Boot your computer from Windows NT.

2. From the Program Manager's "Main" window, double-click on the

"Control Panel" icon.

3. Double-click on "Network". The Network Settings dialog box

appears.

4. Select "Add Adapter".

5. Select "<Other> Requires disk from manufacturer".

6. Select "Continue".

7. When prompted, insert the SuperDisk.

8. Type the Destination Directory where EZStart copied the

driver files (for example, A:\WIN_NT). If you are not

sure where the Destination Directory resides, look for

the SMC8000N.SYS file. This file was one of the files

copied by EZStart to the Destination Directory. When you

find the directory where this file resides, you will have

found the Destination Directory.

9. Select "OK".

10. When the next dialog box appears, select the SMC EtherEZ

adapter followed by "OK".

11. You will be presented with the Adapter Card Setup dialog box.

NOTE: If you have previously selected the wrong adapter

(step 10) Windows NT will fail to locate the adapter

and you will be presented with default settings.

If PnP is enabled (EtherEZ default), Windows NT will

fail to locate the adapter and you will be presented

with default settings.

12. Choose the IRQ Level, Memory Base address, and I/O Base

address. Then, click on OK.

13. When the next two dialog boxes appear, select "OK" for

each one.

14. When prompted, select Restart Now.

NOTE:  When performing a Primary Installation (installing

Windows NT for the first time), you will be prompted

with a Network Adapter Card Detection dialog box. Select

Do Not Detect to manually detect the EtherEZ and then

select Continue.

When installing a second adapter and prompted with the

Network Adapter Card Detection dialog box, select

Automatic Detection and then select Continue.

In the unlikely event you encounter any device driver errors

after Windows NT boots, check the event log in the

Administrative Tools icon to see whether the error is related

to your network adapter.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, EliteView, EZStart, SuperDisk, and UltraChip

are trademarks of Standard Microsystems Corporation.  Other product

and company names are registered trademarks or trademarks of their

respective holders.