# APPENDIX B CDMA CALL FLOW EXAMPLES

This appendix contains examples of call flow.  The diagrams follow these conventions:

o All messages are received without error

o Receipt of messages is not shown except in the handoff examples

o Acknowledgements are not shown

o Optional authentication procedures are not shown

o Optional private long code transitions are not shown

| Mobile Station |  |  |  | Base Station |
| --- | --- | --- | --- | --- |
| o Detects user-initiated call |  |  |  |  |
| o Sends Origination Message | - | Access Channel | - | o Sets up Traffic Channel |
|  |  |  |  | o Begins sending null Traffic Channel data |
| o Sets up Traffic Channel | = | Paging Channel | = | o Sends Channel Assignment Message |
| o Receives N5m consecutive valid frames |  |  |  |  |
| o Begins sending the Traffic Channel preamble | - |  | - | o Acquires the Reverse Traffic Channel |
| o Begins transferring primary traffic packets to and from the Service Option 1 application | = | Forward Traffic Channel | = | o Sends Base Station Acknowledgement Order |
| Optional |  |  |  | Optional |
| o Sends Origination Continuation Message | - | Reverse Traffic Channel | - |  |
| Optional |  |  |  | Optional |
| o Applies ring back in audio path | = | Forward Traffic Channel | = | o Sends Alert With Information Message (ring back tone) |
| Optional |  |  |  | Optional |
| o Removes ring back from audio path | = | Forward Traffic Channel | = | o Sends Alert With Information Message (tones off) |
| (User conversation) |  |  |  | (User conversation) |

Figure B-1.  Simple Call Flow, Mobile Station Origination Example Using Default Service Optiontc  \l 8 "Figure B-1.  Simple Call Flow, Mobile Station Origination Example Using Default Service Option"§

| Mobile Station |  |  |  | Base Station |
| --- | --- | --- | --- | --- |
|  | = | Paging Channel | = | o Sends Page Message or Slotted Page Message |
| o Sends Page Response Message | - | Access Channel | - | o Sets up Traffic Channel o Begins sending null Traffic Channel data |
| o Sets up Traffic Channel | = | Paging Channel | = | o Sends Channel Assignment Message |
| o Receives N5m consecutive valid frames |  |  |  |  |
| o Begins sending the Traffic Channel preamble | - |  | - | o Acquires the Reverse Traffic Channel |
| o Begins transferring received primary traffic packets to the Service Option 1 application and sending null Traffic Channel data | = | Forward Traffic Channel | = | o Sends Base Station Acknowledgement Order |
| o Starts ringing | = | Forward Traffic Channel | = | o Sends Alert With Information Message (ring) |
| o User answers call |  |  |  |  |
| o Stops ringing |  |  |  |  |
| o Sends Connect Order | - | Reverse Traffic Channel | - |  |
| o Begins sending primary traffic packets from the Service Option 1 application |  |  |  |  |
| (User conversation) |  |  |  | (User conversation) |

Figure B-2.  Simple Call Flow, Mobile Station Termination Example Using Default Service Optiontc  \l 8 "Figure B-2.  Simple Call Flow, Mobile Station Termination Example Using Default Service Option"§

| Mobile Station |  |  |  | Base Station |
| --- | --- | --- | --- | --- |
| o Detects user-initiated disconnect |  |  |  |  |
| o Sends Release Order | - | Reverse Traffic Channel | - |  |
|  | = | Forward Traffic Channel | = | o Sends Release Order |
| o Enters the System Determination Substate of the Mobile Station Initialization State |  |  |  |  |

Figure B-3.  Simple Call Flow, Mobile Station Initiated Call Disconnect Exampletc  \l 8 "Figure B-3.  Simple Call Flow, Mobile Station Initiated Call Disconnect Example"§

| Mobile Station |  |  |  | Base Station |
| --- | --- | --- | --- | --- |
|  |  |  |  | o Detects call disconnect |
|  | = | Forward Traffic Channel | = | o Sends Release Order |
| o Sends Release Order | - | Reverse Traffic Channel | - |  |
| o Enters the System Determination Substate of the Mobile Station Initialization State |  |  |  |  |

Figure B-4.  Simple Call Flow, Base Station Initiated Call Disconnect Exampletc  \l 8 "Figure B-4.  Simple Call Flow, Base Station Initiated Call Disconnect Example"§

| Mobile Station |  |  |  | Base Station |
| --- | --- | --- | --- | --- |
| (User conversation) |  |  |  | (User conversation) |
| o Detects request for third party to be added to conversation |  |  |  |  |
| o Sends Flash With Information Message (dialed digits) | - | Reverse Traffic Channel | - | o MTSO mutes speech |
| Optional |  |  |  | Optional |
| o Applies ring back in audio path | = | Forward Traffic Channel | = | o Sends Alert With Information Message (ring back tone) |
|  |  |  |  | (Called party answers) |
| Optional |  |  |  | Optional |
| o Removes ring back tone from audio path | = | Forward Traffic Channel | = | o Sends Alert With Information Message (tones off) |
|  |  |  |  | o MTSO unmutes speech from added party |
| (Two-way conversation with added party; original party held) |  |  |  | (Two-way conversation with added party; original party held) |
| o Detects user request to establish three-way conversation |  |  |  |  |
| o Sends Flash With Information Message | - | Reverse Traffic Channel | - | o MTSO reconnects original party |
| (Three-way conversation) |  |  |  | (Three-way conversation) |

Figure B-5.  Simple Call Flow, Three-Party Calling Exampletc  \l 8 "Figure B-5.  Simple Call Flow, Three-Party Calling Example"§

| Mobile Station |  |  |  | Base Station |
| --- | --- | --- | --- | --- |
| (User conversation with first party) |  |  |  | (User conversation with first party) |
|  |  |  |  | o Detects incoming call |
| Optional |  |  |  | Optional |
| o Applies call waiting tone in audio path | = | Forward Traffic Channel | = | o Sends Alert With Information Message (call waiting tone) |
| o Detects user request to change parties |  |  |  |  |
| o Sends Flash With Information Message | - | Reverse Traffic Channel | - | o MTSO mutes speech path to first party, connects second party |
| (User conversation with second party; first party held) |  |  |  | (User conversation with second party; first party held) |
| o Detects user request to change parties |  |  |  |  |
| o Sends Flash With Information Message | - | Reverse Traffic Channel | - | o MTSO mutes speech path to second party, connects first party |
| (User conversation with first party; second party held) |  |  |  | (User conversation with first party; second party held) |

Figure B-6.  Simple Call Flow, Call-Waiting Exampletc  \l 8 "Figure B-6.  Simple Call Flow, Call-Waiting Example"§

Figure B-7 illustrates call processing operations during a soft handoff from base station A to base station B.  Figure B-8 illustrates call processing operations during a sequential soft handoff in which the mobile station is transferred from a pair of base stations A and B through a pair of base stations A and C to base station C.

| Mobile Station |  |  |  | Base Station |
| --- | --- | --- | --- | --- |
| (User conversation using A) |  |  |  | (User conversation using A) |
| o Pilot B strength exceeds T_ADD |  |  |  |  |
| o Sends Pilot Strength Measurement Message | - | Reverse Traffic Channel | - | o A receives Pilot Strength Measurement Message |
|  |  |  |  | o B begins transmitting traffic on the Forward Traffic Channel and acquires the Reverse Traffic Channel |
| o Receives Handoff Direction Message | = | Forward Traffic Channel | = | o A and B send Handoff Direction Message to use A and B |
| o Acquires B; begins using Active Set {A,B} |  |  |  |  |
| o Sends Handoff Completion Message | - | Reverse Traffic Channel | - | o A and B receive Handoff Completion Message |
| o Handoff drop timer of pilot A expires |  |  |  |  |
| o Sends Pilot Strength Measurement Message | - | Reverse Traffic Channel | - | o A and B receive Pilot Strength Measurement Message |
| o Receives Handoff Direction Message | = | Forward Traffic Channel | = | o A and B send Handoff Direction Message to use B only |
| o Stops diversity combining; begins using Active Set {B} |  |  |  |  |
| o Sends Handoff Completion Message | - | Forward Traffic Channel | - | o A and B receive Handoff Completion Message |
|  |  |  |  | o A stops transmitting on the Forward Traffic Channel and receiving on the Reverse Traffic Channel |
| (User conversation using B) |  |  |  | (User conversation using B) |

Figure B-7.  Call Processing During Soft Handofftc  \l 8 "Figure B-7.  Call Processing During Soft Handoff"§

| Mobile Station |  |  |  | Base Station |
| --- | --- | --- | --- | --- |
| (User conversation using A and B) |  |  |  | (User conversation using A and B) |
| o Handoff drop timer of pilot A expires and pilot C strength exceeds T_ADD |  |  |  |  |
| o Sends Pilot Strength Measurement Message | - | Reverse Traffic Channel | - | o A and B receive Pilot Strength Measurement Message, determine that new Active Set should contain B and C |
|  |  |  |  | o C begins transmitting traffic on the Forward Traffic Channel and acquires the Reverse Traffic Channel |
| o Receives Handoff Direction Message | = | Forward Traffic Channel | = | o A, B, and C send Handoff Direction Message to use B and C |
| o Stops diversity combining A and B; starts diversity combining B and C |  |  |  |  |
| o Sends Handoff Completion Message | - | Reverse Traffic Channel | - | o A, B, and C receive Handoff Completion Message |
|  |  |  |  | o A stops transmitting on the Forward Traffic Channel and receiving on the Reverse Traffic Channel |
| o Handoff drop timer of pilot B expires |  |  |  |  |
| o Sends Pilot Strength Measurement Message | - | Reverse Traffic Channel | - | o B and C receive Pilot Strength Measurement Message |
| (Continues) |  |  |  | (Continues) |
| [ |  |  |  | [ |

tc  \l 8 "Figure B-8.  Call Processing During Sequential Soft Handoff"§ (Part 1 of 2)

| Mobile Station |  |  |  | Base Station |
| --- | --- | --- | --- | --- |
| (Continues) |  |  |  | (Continues) |
| [ |  |  |  | [ |
| o Receives Handoff Direction Message | = | Forward Traffic Channel | = | o B and C send Handoff Direction Message to use C only |
| o Stops diversity combining; begins using Active Set {C} |  |  |  |  |
| o Sends Handoff Completion Message | - | Forward Traffic Channel | - | o B and C receive Handoff Completion Message |
|  |  |  |  | o B stops transmitting on the Forward Traffic Channel and receiving on the Reverse Traffic Channel |
| (User conversation using C) |  |  |  | (User conversation using C) |

Figure B-8.  Call Processing During Sequential Soft Handoff (Part 2 of 2)