# APPENDIX D CDMA CONSTANTS

The following tables provide values for identifiers given in the text:

Table D-1.  Time Limits

Table D-2.  Other Constants

tc  \l 9 "Table D-1.  Time Limits "§(Part 1 of 3)

| Time Limit | Description | Value | References |
| --- | --- | --- | --- |
| T1m | Maximum time the mobile station waits for an acknowledgement | 0.4 s | 6.6.4.1.3.1.1 |
| T2m | Maximum time allowed for the mobile station to send an acknowledgement | 0.2 s | 6.6.4.1.3.1.2 |
| T3m | Period in which two messages arriving on the Forward Traffic Channel, not requiring an acknowledgement, and carrying the same sequence numbers, are considered duplicates | 0.32 s | 6.6.4.1.3.2 |
| T4m | Period in which two messages arriving on the same Paging Channel and carrying the same sequence numbers are considered duplicates | 2.2 s | 6.6.2.1.2 |
| T5m | Limit of the Forward Traffic Channel fade timer | 5 s | 6.4.4 |
| T20m | Maximum time to remain in the Pilot Channel Acquisition Substate | 15 s | 6.6.1.2 |
| T21m | Maximum time to receive a valid Sync Channel message | 1 s | 6.6.1.3 |
| T30m | Maximum time to receive a valid Paging Channel message | 3 s | 6.4.3 |
| T31m | Maximum time for which overhead messages arriving on the Paging Channel are considered valid | 600 s | 6.6.2.2 |
| T32m | Maximum time to respond to an SSD Update Message, BS Challenge Confirmation Order, and Authentication Challenge Message | 5 s | 6.6.2.4 |
| T33m | Maximum time to respond to messages received while in the Mobile Station Idle State (except authentication messages) | 0.3 s | 6.6.2.3 6.6.5.6.2.3 |

Table D-1.  Time Limits (Part 2 of 3)

| Time Limit | Description | Value | References |
| --- | --- | --- | --- |
| T40m | Maximum time to receive a valid Paging Channel message before aborting an access attempt | 1 s | 6.4.3 |
| T41m | Maximum time to obtain updated overhead messages arriving on the Paging Channel | 4 s | 6.6.3.2 |
| T42m | Maximum time to receive a delayed layer 3 response following the receipt of an acknowledgement for an access probe | 12 s | 6.6.3.3 |
| T50m | Maximum time to obtain N5m consecutive good Forward Traffic Channel frames when in the Traffic Channel Initialization Substate | 0.2 s | 6.6.4.2 |
| T51m | Maximum time for the mobile station to receive a Base Station Acknowledgement Order when in the Traffic Channel Initialization Substate | 2 s | 6.6.4.2 |
| T52m | Maximum time to receive a message in the Waiting for Order Substate that transits the mobile station to a different substate or state | 5 s | 6.6.4.3.1 |
| T53m | Maximum time to receive a message in the Mobile Station Answer Substate that transits the mobile station to a different substate or state | 65 s | 6.6.4.3.2 |
| T54m | Maximum time for the mobile station to send an Origination Continuation Message upon entering the Conversation Substate | 0.2 s | 6.6.4.4 |
| T55m | Maximum time to receive a message in the Release Substate that transits the mobile station to a different substate or state | 2 s | 6.6.4.5 |
| T56m | Default maximum time to respond to a received message or order on the Forward Traffic Channel | 0.2 s | 6.6.4 6.6.6 |
| T57m | Limit of the power-up registration timer | 20 s | 6.6.5.1.1 |
| T60m | Maximum time to execute a hard handoff involving a new frequency assignment using the same base station | 0.06 s | 6.6.6.2.8 |
| T61m | Maximum time to execute a hard handoff involving a new frequency assignment using a different base station | 0.08 s | 6.6.6.2.8 |

Table D-1.  Time Limits (Part 3 of 3)

| Time Limit | Description | Value | References |
| --- | --- | --- | --- |
| T62m | Maximum time to execute a hard handoff involving the same frequency assignment | 0.02 s | 6.6.6.2.8 |
| T63m | Maximum time to execute a CDMA to Analog handoff | 0.1 s | 6.6.6.2.9 |
| T1b | Maximum period between subsequent transmissions of an overhead message on the Paging Channel by the base station | 1 s | 7.6.2.2 |
| T2b | Maximum time for the base station to send a Release Order after receiving a Release Order | 0.8 s | 7.6.4.4 |
| T3b | Minimum time the base station continues to transmit on a code channel after sending or receiving a Release Order | 0.3 s | 7.6.4.5 |
| T4b | Maximum time for the base station to respond to a service option request when it does not support the requested service option | 10 s | 7.6.4.1.2.2.1 |

Table D-2.  Other Constantstc  \l 9 "Table D-2.  Other Constants"§

| Constant | Description | Value | References |
| --- | --- | --- | --- |
| N1m | Maximum number of times that a mobile station transmits a message requiring an acknowledgement on the Reverse Traffic Channel | 3 | 6.6.4.1.3.1.1 |
| N2m | Number of received consecutive bad Forward Traffic Channel frames before a mobile station must disable its transmitter | 12 | 6.4.4 |
| N3m | Number of received consecutive good Forward Traffic Channel frames before a mobile station is allowed to re-enable its transmitter after disabling its transmitter | 2 | 6.4.4 6.6.6.2.8 |
| N4m | Number of frames after the transmission of a response to a Long CodeTransition Request Order determining the 80 ms boundary for the private long code transition | 20 | 6.6.4 7.6.4.1.5 |
| N5m | Number of received consecutive good Forward Traffic Channel frames before a mobile station is allowed to enable its transmitter after entering the Traffic Channel Initialization Substate of the Mobile Station Control on the Traffic Channel State | 2 | 6.6.4.2 |
| N6m | Minimum supported Active Set size | 6 | 6.6.6.2.6.1 |
| N7m | Minimum supported Candidate Set size | 6 | 6.6.6.2.6.2 |
| N8m | Minimum supported Neighbor Set size | 20 | 6.6.6.2.6.3 |
| N9m | Minimum supported zone list size | 7 | 6.6.5 |
| N10m | Minimum supported SID/NID list size | 4 | 6.6.5 |