# 6 REQUIREMENTS FOR MOBILE STATION CDMA OPERATION xe "mobile station, transmitter"§

This section only details requirements that are specific to CDMA mobile station equipment and operation.  See Section 2 and Section 4 for analog mobile station requirements.

## 6.1 Transmitter

### 6.1.1 Frequency Parameters xe "mobile station, transmitter, frequency parameters"§

#### 6.1.1.1 Channel Spacing and Designation xe "mobile station, transmitter, channel spacing and designation"§

Channel spacing and designation for the dual-mode mobile station transmissions shall be as specified in 2.1.1.1.  The mobile station shall support CDMA operations on channel numbers 1013 through 1023, and 1 through 777 inclusive, as shown in Table 6.1.1.1-1.

The xe "CDMA frequency assignment"§CDMA frequency assignment in MHz corresponding to the xe "CDMA Channel number shown in Table 6.1.1.1-1 (expressed as N) is calculated as shown in Table 6.1.1.1-2."§CDMA Channel number shown in Table 6.1.1.1-1 (expressed as N) is calculated as shown in Table 6.1.1.1-2.

#### 6.1.1.2 Frequency Tolerance

When operating in the CDMA transmission mode, the mobile station transmit carrier frequency shall be 45.0 MHz ±300 Hz lower than the frequency of the base station transmit signal as measured at the mobile station receiver.

### 6.1.2 Power Output Characteristics xe "mobile station, transmitter, power output characteristics"§

All power levels are referenced to the mobile station antenna connector unless otherwise specified.

#### 6.1.2.1 Maximum Output Power

The absolute maximum xe "effective radiated power "§effective radiated power (xe "ERP"§ERP) with respect to a half-wave dipole for any class mobile station transmitter shall be 8 dBW (6.3 Watts).  ERP measured during a transmitted power control group (see 6.1.3.1.7.1) for each mobile station class when commanded to maximum output power shall be within the limits given in Table 6.1.2.1-1.  Transmission at maximum power shall not degrade the spurious emission levels as specified in 6.1.4.2.  An inoperative antenna assembly shall not degrade spurious emission levels as specified in 6.1.4.2.

tc "Table 6.1.1.1-1.  CDMA Channel Numbers and Corresponding Frequencies"\c 	§Table 6.1.1.1-1.  CDMA Channel Numbers and Corresponding Frequencies

| System |  | xe "Valid CDMA Frequency"§Valid CDMA Frequency Assignments |  | Analog Channel Count | CDMA Channel Number | Transmitter Frequency Assignment (MHz) Mobile Base |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 991 | 824.040 | 869.040 |
|  |  | ///////// |  | 22 |  |  |  |
| A" |  |  |  |  | 1012 | 824.670 | 869.670 |
| (1 MHz) |  |  |  |  | 1013 | 824.700 | 869.700 |
|  |  | CDMA |  | 11 |  |  |  |
|  |  |  |  |  | 1023 | 825.000 | 870.000 |
|  |  |  |  |  | 1 | 825.030 | 870.030 |
| A |  | CDMA |  | 333 |  |  |  |
| (10 MHz) |  |  |  |  |  |  |  |
|  |  |  |  |  | 333 | 834.990 | 879.990 |
|  |  |  |  |  | 334 | 835.020 | 880.020 |
| B |  |  |  |  |  |  |  |
| (10 MHz) |  | CDMA |  | 333 |  |  |  |
|  |  |  |  |  | 666 | 844.980 | 889.980 |
|  |  |  |  |  | 667 | 845.010 | 890.010 |
| A' |  |  |  |  |  |  |  |
| (1.5 MHz) |  | CDMA |  | 50 |  |  |  |
|  |  |  |  |  | 716 | 846.480 | 891.480 |
|  |  |  |  |  | 717 | 846.510 | 891.510 |
| B' |  | CDMA |  | 61 |  |  |  |
| (2.5 MHz) |  |  |  |  |  |  |  |
|  |  |  |  |  | 777 | 848.310 | 893.310 |
|  |  |  |  |  | 778 | 848.340 | 893.340 |
|  |  | ///////// |  | 22 |  |  |  |
|  |  |  |  |  | 799 | 848.970 | 893.970 |

Frequencies in shaded (///////) regions are not valid for xe "CDMA frequency assignment"§CDMA frequency assignments.

tc "Table 6.1.1.1-2.  CDMA Channel Number to CDMA Frequency Assignment Correspondence"\c 	§Table 6.1.1.1-2.  CDMA Channel Number to CDMA Frequency Assignment Correspondence

| Transmitter | CDMA Channel Number | CDMA Frequency Assignment, MHz |
| --- | --- | --- |
| Mobile Station | 1 _ N _ 777 | 0.030 N + 825.000 |
|  | 1013 _ N _ 1023 | 0.030 (N-1023) + 825.000 |
| Base Station | 1 _ N _ 777 | 0.030 N + 870.000 |
|  | 1013 _ N _ 1023 | 0.030 (N-1023) + 870.000 |

tc "Table 6.1.2.1-1.  Effective Radiated Power at Maximum Output Power"\c 	§Table 6.1.2.1-1.  Effective Radiated Power at Maximum Output Power

| Mobile Station Class | ERP at Maximum Output Shall Exceed | ERP at Maximum Output Shall not Exceed |
| --- | --- | --- |
| I | 1 dBW (1.25 watts) | 8 dBW (6.3 watts) |
| II | -3 dBW (0.5 watts) | 4 dBW (2.5 watts) |
| III | -7 dBW (0.2 watts) | 0 dBW (1.0 watts) |

#### 6.1.2.2 Output Power Limits xe "mobile station, transmitter, carrier on/off conditions"§

##### 6.1.2.2.1 Minimum Controlled Output Power

With both closed loop and open loop power control functions set to minimum (see 6.1.2.3), the mean output power of the mobile station shall be less than -50 dBm/1.23 MHz (-111 dBm/Hz) for all frequencies within ±615 kHz of the center frequency.

##### 6.1.2.2.2 Gated Output Power

When operating in variable data rate transmission mode (see 6.1.3.1.7.1), the mobile station shall transmit at nominal controlled power levels only during gated-on periods.  During gated-off periods, the mobile station shall reduce its ERP either by at least 20 dB with respect to the power of the most recent gated-on period, or to the transmitter noise floor, whichever is greater.  The transmitter noise floor should be less than -60 dBm/1.23 MHz and shall be less than -54 dBm/1.23 MHz.  The duration of a single gated-on power control group shall be at least 1.25 ms.  The transition time from either the gated-on level to the gated-off level or from the gated-off level to the gated-on level shall be within the limits shown in Figure 6.1.2.2.2-1.

tc "Figure 6.1.2.2.2-1.  Transmission Envelope Mask (Single Gated-on Power Control Group)"\c §Figure 6.1.2.2.2-1.  Transmission Envelope Mask (Single Gated-on Power Control Group)

##### 6.1.2.2.3 Standby Output Power

The mobile station shall disable its transmitter except when transmitting an access probe when in the System Access State or when in the Mobile Station Control on the Traffic Channel State (see 6.6.3 and 6.6.4).  When the transmitter is disabled, the output noise density of the mobile station shall be less than -60 dBm/1.23 MHz for all frequencies within the mobile station’s transmit band between 824 and 849 MHz.

#### 6.1.2.3 Controlled Output Power

When operating in the CDMA transmission mode, mobile stations shall provide two independent means for output power adjustment:  open loop estimation, solely a mobile station operation, and closed loop correction, involving both the mobile station and the base station.

Accuracy requirements on the controlled range of mean output power (see 6.1.2.3 and 6.1.2.4) need not apply for the following three cases:  mean output power levels exceeding the minimum ERP at the maximum output power for the corresponding mobile station class (see Table 6.1.2.1-1); mean output power levels less than the minimum controlled output power (see 6.1.2.2.1); or mean input power levels exceeding -25 dBm.

##### 6.1.2.3.1 Estimated Open Loop Output Power

In the following equations, mean power is referenced to the nominal CDMA Channel bandwidth of 1.23 MHz.

For open loop probing on the xe "Access Channel"§Access Channel (with closed loop correction inactive) the mobile station shall transmit the first probe at a xe "mean output power"§mean output power level defined by

mean output power (dBm)  =	- mean input power (dBm) 
		- 73
		+ 0.5 ¥ xe "NOM_PWR "§NOM_PWR (dB) 
		+ 0.5 ¥ xe "INIT_PWR "§INIT_PWR (dB).

Subsequent probes in an xe "access probe sequence"§access probe sequence are sent at increased power levels until a response is obtained or the sequence ends (see 6.6.3.1).

The initial transmission on the xe "Reverse Traffic Channel"§Reverse Traffic Channel shall be at a xe "mean output power"§mean output power defined by

mean output power (dBm)  =	- mean input power (dBm) 
		- 73 
		+ 0.5 ¥ xe "NOM_PWR "§NOM_PWR (dB) 
		+ 0.5 ¥ xe "INIT_PWR "§INIT_PWR (dB) 
		+ the sum of all xe "access probe corrections"§access probe corrections (dB).

Once the first xe "power control bit "§power control bit has been received after initializing xe "Reverse Traffic Channel"§Reverse Traffic Channel transmissions, the xe "mean output power"§mean output power shall be defined by

mean output power (dBm)  =	- mean input power (dBm) 
		- 73 
		+ 0.5 ¥ xe "NOM_PWR "§NOM_PWR (dB) 
		+ 0.5 ¥ xe "INIT_PWR "§INIT_PWR (dB) 
		+ the sum of all xe "access probe corrections"§access probe corrections (dB)
		+ the sum of all xe "closed loop power control "§closed loop power control corrections (dB).

The values for xe "NOM_PWR"§NOM_PWR and xe "INIT_PWR"§INIT_PWR are system parameters, specified in the xe "Access Parameters Message"§Access Parameters Message (see 7.7.2.3.2) and acquired by the mobile station prior to transmitting.  The range of the xe "NOM_PWR"§NOM_PWR parameter is ±64 dB, with a nominal value of 0 dB.  The range of the xe "INIT_PWR"§INIT_PWR parameter is ±16 dB, with a nominal value of 0 dB.

Prior to application of access probe corrections, closed loop power control corrections, and with NOM_PWR set to zero, the mobile station’s estimated open loop mean output power should be within ±6 dB and shall be within ±9 dB of the value determined by the following relationship:

mean output power (dBm)  =	- mean input power (dBm) 
		-73 
		+ 0.5 ¥ xe "INIT_PWR "§INIT_PWR (dB).

This requirement shall be met over the full range of INIT_PWR (from -16 to +16).

##### 6.1.2.3.2 Closed Loop Output Power

For closed loop correction on the xe "Reverse Traffic Channel"§Reverse Traffic Channel (with respect to the open loop estimate), the mobile station shall adjust its xe "mean output power"§mean output power level in response to each valid xe "power control bit "§power control bit (see 7.1.3.1.7) received on the xe "Forward Traffic Channel"§Forward Traffic Channel.  A power control bit shall be considered valid if it is received in a 1.25 ms timeslot (see 6.1.3.1.7.1) that is the second timeslot following a timeslot in which the mobile station transmitted.  When the mobile station is transmitting at the full data rate (9600 bps), the change in mean output power level per single power control bit shall be 0.5 dB.  When the mobile station is transmitting at less than the full data rate, the change in the xe "mean output power"§mean output power level for each valid power control bit shall vary according to the number of timeslots since the last valid xe "power control bit "§power control bit was received, in accordance with Table 6.1.2.3.2-1.  The total changed closed loop mean output power shall be the accumulation of the level changes.  The mobile station shall lock the accumulation of level changes and shall ignore received power control bits when the transmitter is disabled.

After accumulating the power control steps (with 0.25 dB resolution), the mobile station may truncate the mean output power but shall preserve at least 0.5 dB resolution.  That is, if the total accumulation is 1.0 dB and 2 timeslots have elapsed since the last valid xe "power control bit"§power control bit, then the mean output power may increase by 0.5 dB instead of 0.75 dB; if the total accumulation is 1.25 dB and two timeslots have elapsed since the last valid xe "power control bit"§power control bit, then the xe "mean output power"§mean output power may increase by 1.0 dB.  After the allowed truncation, the change in mean output power per single power control bit shall be within ±0.3 dB of the values specified by Table 6.1.2.3.2-1, and the change in mean output power level per 10 valid power control bits of the same sign and level change shall be within ±20% of 10 times the values specified in Table 6.1.2.3.2-1.  A ‘0’ xe "power control bit "§power control bit implies an increase in transmit power; a ‘1’ xe "power control bit "§power control bit implies a decrease in transmit power.

The mobile station shall provide a closed loop adjustment range greater than ±24 dB around its open loop estimate.

See 6.6.6.2.7.2 for combining power control bits received from different multipath components or from different base stations during handoff.

tc "Table 6.1.2.3.2-1.  Power Level Change per Power Control Bit"\c 	§Table 6.1.2.3.2-1.  Power Level Change per Power Control Bit

| Timeslots since Last Valid xe "Power Control Bit"§Power Control Bit | Level Change per Power Control Bit |
| --- | --- |
| 1 | 0.50 dB |
| 2 | 0.75 dB |
| 3 | 0.75 dB |
| 4 | 1.00 dB |
| 5 | 1.00 dB |
| 6 | 1.25 dB |
| 7 | 1.25 dB |
| 8 | 1.50 dB |
| 9 | 1.50 dB |
| 10 | 1.50 dB |
| 11 | 1.75 dB |
| 12 | 1.75 dB |
| 13 | 1.75 dB |
| 14 | 1.75 dB |
| 15 | 1.75 dB |

#### 6.1.2.4 Power Transition Characteristics xe "mobile station, transmitter, power transition time limits"§

##### 6.1.2.4.1 Open Loop Estimation

Following a step change in mean input power, the xe "mean output power"§mean output power of the mobile station shall transition between levels within the limits shown in Figure 6.1.2.4.1-1.  These limits shall apply for step changes in mean input power of ±30 dB or less.  As an example, if the mean input power change step is -30 dB, the expected change in mean output power is +30 dB.  The upper limit on the change is 1.2 ¥ 30 dB = 36 dB, and the lower limit is 0.8 ¥ 30 dB = 24 dB, as shown in Figure 6.1.2.4.1-1.

tc "Figure 6.1.2.4.1-1.  Open Loop Power Transition Mask"\c §Figure 6.1.2.4.1-1.  Open Loop Power Transition Mask

##### 6.1.2.4.2 Closed Loop Correction

Following the reception of a valid closed loop xe "power control bit"§power control bit, the xe "mean output power"§mean output power of the mobile station shall be within 0.3 dB of final value in less than 500 ms.

### 6.1.3 Modulation Characteristics xe "mobile station, transmitter, modulation characteristics"§

#### 6.1.3.1 Reverse CDMA Channel Signals xe "mobile station, transmitter, modulation characteristics, Reverse CDMA Channel"§

The xe "Reverse CDMA Channel"§Reverse CDMA Channel is composed of xe "Access Channel"§Access Channels and xe "Reverse Traffic Channel"§Reverse Traffic Channels.  These channels shall share the same xe "CDMA frequency assignment"§CDMA frequency assignment using direct-sequence CDMA techniques.  Figure 6.1.3.1-1 shows an example of all of the signals received by a base station; on the Reverse CDMA Channel.  Each Traffic Channel or Access Channel is identified by a distinct user long code.  Multiple Reverse CDMA Channels may be used by a base station in a frequency multiplexed manner.

The modulation process for the xe "Reverse CDMA Channel"§Reverse CDMA Channel is as shown in Figure 6.1.3.1-2.  Data transmitted on the i.Reverse CDMA Channel; is grouped into 20 ms frames.  All data transmitted on the xe "Reverse CDMA Channel"§Reverse CDMA Channel is convolutionally encoded, block interleaved, modulated by the 64-ary orthogonal modulation, and direct-sequence spread prior to transmission.

µ §

tc "Figure 6.1.3.1-1.  Example of Logical Reverse CDMA Channels Received at a Base Station"\c §Figure 6.1.3.1-1.  Example of Logical Reverse CDMA Channels Received at a Base Station

tc "Figure 6.1.3.1-2.  Reverse CDMA Channel Modulation Process"\c §Figure 6.1.3.1-2.  Reverse CDMA Channel Modulation Process

Data frames may be transmitted on the xe "Reverse CDMA Channel"§Reverse CDMA Channel at a data rate of 9600, 4800, 2400, or 1200 bps.  The xe "Reverse Traffic Channel"§Reverse Traffic Channel may use any of these data rates for transmission.  The transmission duty cycle on the Reverse Traffic Channel varies with the transmission data rate.  Specifically, the transmission duty cycle for 9600 bps frames is 100 percent, the transmission duty cycle for 4800 bps frames is 50 percent, the transmission duty cycle for 2400 bps frames is 25 percent, and the transmission duty cycle for 1200 bps frames is 12.5 percent as shown in Table 6.1.3.1.1-1.  As the duty cycle for transmission varies proportionately with the data rate, the actual burst transmission rate is fixed at 28,800 code symbols per second.  Since six code symbols are modulated as one of 64 xe "Walsh symbol"§Walsh symbols for transmission, the Walsh symbol transmission rate is fixed at 4800 Walsh symbols per second.  This results in a fixed xe "Walsh chip "§Walsh chip rate of 307.2 kcps.  The rate of the spreading PN sequence is fixed at 1.2288 Mcps, so that each xe "Walsh chip "§Walsh chip is spread by four PN chips.  Table 6.1.3.1.1-1 defines the signal rates and their relationship for the various transmission rates on the xe "Reverse Traffic Channel"§Reverse Traffic Channel.

The numerology is identical for the xe "Access Channel"§Access Channel except that the transmission rate is fixed at 4800 bps, each code symbol is repeated once, and the transmission duty cycle is 100 percent.  Table 6.1.3.1.1-2 defines the signal rates and their relationship on the xe "Access Channel"§Access Channel.

##### 6.1.3.1.1 Modulation Parameters

The modulation parameters for the Reverse Traffic Channel and the Access Channel are shown in Table 6.1.3.1.1-1 and Table 6.1.3.1.1-2, respectively.

tc ""\c 	§

Table 6.1.3.1.1-1.  Reverse Traffic Channel Modulation Parameters

|  | Data Rate (bps) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | 9600 | 4800 | 2400 | 1200 | Units |
| PN Chip Rate | 1.2288 | 1.2288 | 1.2288 | 1.2288 | Mcps |
| Code Rate | 1/3 | 1/3 | 1/3 | 1/3 | bits/code sym |
| TX Duty Cycle | 100.0 | 50.0 | 25.0 | 12.5 | % |
| Code Symbol Rate | 28,800 | 28,800 | 28,800 | 28,800 | sps |
| Modulation | 6 | 6 | 6 | 6 | code sym/Walsh sym |
| xe "Walsh Symbol "§Walsh Symbol Rate | 4800 | 4800 | 4800 | 4800 | sps |
| xe "Walsh Chip"§Walsh Chip Rate | 307.20 | 307.20 | 307.20 | 307.20 | kcps |
| Walsh Symbol | 208.33 | 208.33 | 208.33 | 208.33 | µs |
| PN Chips/Code Symbol | 42.67 | 42.67 | 42.67 | 42.67 | PN chip/code sym |
| PN Chips/xe "Walsh Symbol"§Walsh Symbol | 256 | 256 | 256 | 256 | PN chip/Walsh sym |
| PN Chips/xe "Walsh Chip"§Walsh Chip | 4 | 4 | 4 | 4 | PN chips/Walsh chip |

tc "Table 6.1.3.1.1-2.  Access Channel Modulation Parameters"\c 	§Table 6.1.3.1.1-2.  Access Channel Modulation Parameters

|  | Data Rate (bps) |  |
| --- | --- | --- |
| Parameter | 4800 | Units |
| PN Chip Rate | 1.2288 | Mcps |
| Code Rate | 1/3 | bits/code sym |
| Code Symbol Repetition | 2 | symbols/code sym |
| TX Duty Cycle | 100.0 | % |
| Code Symbol Rate | 28,800 | sps |
| Modulation | 6 | code sym/Walsh sym |
| xe "Walsh Symbol "§Walsh Symbol Rate | 4800 | sps |
| xe "Walsh Chip"§Walsh Chip Rate | 307.20 | kcps |
| Walsh Symbol | 208.33 | µs |
| PN Chips/Code Symbol | 42.67 | PN chip/code sym |
| PN Chips/xe "Walsh Symbol"§Walsh Symbol | 256 | PN chip/Walsh sym |
| PN Chips/xe "Walsh Chip"§Walsh Chip | 4 | PN chips/Walsh chip |

##### 6.1.3.1.2 Data Rates xe "mobile station, transmitter, Reverse CDMA Channel"§

Data shall be transmitted on the i.Reverse CDMA Channel; by frames.  Each frame shall be 20 ms in length.

The mobile station shall transmit data frames on the Reverse Traffic Channel at a variable data rate of 9600, 4800, 2400, or 1200 bps.  The data rate on the Reverse Traffic Channel shall be selected on a frame-by-frame basis.

The mobile station shall transmit data frames on the Access Channel at a fixed data rate of 4800 bps.

##### 6.1.3.1.3 Convolutional Encoding xe "mobile station, transmitter, Reverse CDMA Channel, convolutional encoding"§

The mobile station shall convolutionally encode the data transmitted on the xe "Reverse Traffic Channel"§Reverse Traffic Channel and the xe "Access Channel"§Access Channel prior to interleaving.  The xe "convolutional code"§convolutional code shall be rate 1/3 and has constraint length (K) of 9.  The generator functions for this code shall be g0 equals 557 (octal), g1 equals 663 (octal), and g2 equals 711 (octal).  Since this is a rate 1/3 code, three code symbols are generated for each data bit input to the encoder.  These code symbols shall be output so that the code symbol (c0) encoded with generator function g0 shall be output first, the code symbol (c1) encoded with generator function g1 shall be output second, and the code symbol (c2) encoded with generator function g2 shall be output last.  The state of the convolutional encoder, upon initialization, shall be the all-zero state.  The first code symbol output after initialization shall be a code symbol encoded with generator function g0.

Convolutional encoding involves the modulo-2 addition of selected taps of a serially time-delayed data sequence.  The length of the data sequence delay is equal to K-1, where K is the constraint length of the code.  Figure 6.1.3.1.3-1 illustrates the encoder for the code specified in this section.

##### 6.1.3.1.4 Code Symbol Repetition xe "mobile station, transmitter, Reverse CDMA Channel, code symbol repetition"§

Code symbols output from the convolutional encoder are repeated before being interleaved when the data rate is lower than 9600 bps.

Code symbol repetition on the Reverse Traffic Channel is only used as an expedient method for describing the operation of the block interleaver specified in 6.1.3.1.5 and the data burst randomizer specified in 6.1.3.1.7.2.  Implementations other than code symbol repetition that achieve the same result are allowed.

µ §

tc "Figure 6.1.3.1.3-1.  K = 9, Rate 1/3 Convolutional Encoder"\c §Figure 6.1.3.1.3-1.  K = 9, Rate 1/3 Convolutional Encoder

The code symbol repetition rate on the xe "Reverse Traffic Channel"§Reverse Traffic Channel varies with data rate.  Code symbols shall not be repeated for the 9600 bps data rate.  Each code symbol at the 4800 bps data rate shall be repeated 1 time (each symbol occurs 2 consecutive times).  Each code symbol at the 2400 bps data rate shall be repeated 3 times (each symbol occurs 4 consecutive times).  Each code symbol at the 1200 bps data rate shall be repeated 7 times (each symbol occurs 8 consecutive times).  For all of the data rates (9600, 4800, 2400, and 1200 bps), this results in a constant code symbol rate of 28800 code symbols per second.  On the xe "Reverse Traffic Channel"§Reverse Traffic Channel these repeated code symbols shall not be transmitted multiple times.  Rather, the repeated code symbols shall be input to the block interleaver function, and all but one of the code symbol repetitions shall be deleted prior to actual transmission due to the variable transmission duty cycle.

Each code symbol on the xe "Access Channel"§Access Channel, which has a fixed data rate of 4800 bps, shall be repeated 1 time (each symbol occurs 2 consecutive times).  On the xe "Access Channel"§Access Channel both repeated code symbols shall be transmitted.

##### 6.1.3.1.5 Block Interleaving xe "mobile station, transmitter, Reverse CDMA Channel, block interleaving"§

The mobile station shall interleave all code symbols on the xe "Reverse Traffic Channel"§Reverse Traffic Channel and the xe "Access Channel"§Access Channel prior to modulation and transmission.  A xe "block interleaver"§block interleaver spanning 20 ms shall be used.  The interleaver shall be an array with 32 rows and 18 columns (i.e., 576 cells).  Code symbols (repeated code symbols when at data rates lower than 9600 bps) shall be written into the interleaver by columns filling the complete 32 ¥ 18 matrix.  Tables 6.1.3.1.5-1 through 6.1.3.1.5-4 illustrate the ordering of write operations of code symbols (or repeated code symbols) into the interleaver array for transmission data rates of 9600, 4800, 2400, and 1200 bps, respectively.

Reverse Traffic Channel code symbols shall be output from the xe "interleaver"§interleaver by rows.  The interleaver rows shall be output in the following order:

At 9600 bps:

1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32

At 4800 bps:

1 3 2 4 5 7 6 8 9 11 10 12 13 15 14 16 17 19 18 20 21 23 22 24 25 27 26 28 29 31 30 32

At 2400 bps:

1 5 2 6 3 7 4 8 9 13 10 14 11 15 12 16 17 21 18 22 19 23 20 24 25 29 26 30 27 31 28 32

At 1200 bps:

1 9 2 10 3 11 4 12 5 13 6 14 7 15 8 16 17 25 18 26 19 27 20 28 21 29 22 30 23 31 24 32

xe "Access Channel"§Access Channel code symbols shall be output from the xe "interleaver"§interleaver by rows.  The interleaver rows shall be output in the following order:

1 17 9 25 5 21 13 29 3 19 11 27 7 23 15 31 2 18 10 26 6 22 14 30 4 20 12 28 8 24 16 32

tc "Table 6.1.3.1.5-1.  Reverse Traffic Channel Interleaver Memory (Write Operation) (9600 bps)"\c 	§Table 6.1.3.1.5-1.  Reverse Traffic Channel Interleaver Memory (Write Operation) (9600 bps)

tc "Table 6.1.3.1.5-2.  Reverse Traffic Channel or Access Channel Interleaver Memory (Write Operation) (4800 bps)"\c 	§Table 6.1.3.1.5-2.  Reverse Traffic Channel or Access Channel Interleaver Memory (Write Operation) (4800 bps)

tc "Table 6.1.3.1.5-3.  Reverse Traffic Channel Interleaver Memory (Write Operation) (2400 bps)"\c 	§Table 6.1.3.1.5-3.  Reverse Traffic Channel Interleaver Memory (Write Operation) (2400 bps)

tc "Table 6.1.3.1.5-4.  Reverse Traffic Channel Interleaver Memory (Write Operation) (1200 bps)"\c 	§Table 6.1.3.1.5-4.  Reverse Traffic Channel Interleaver Memory (Write Operation) (1200 bps)

##### 6.1.3.1.6 Orthogonal Modulation xe "mobile station, transmitter, Reverse CDMA Channel, orthogonal modulation"§

Modulation for the xe "Reverse CDMA Channel"§Reverse CDMA Channel shall be 64-ary orthogonal modulation.  One of 64 possible xe "modulation symbol"§modulation symbols is transmitted for each six code symbols.  The modulation symbol shall be one of 64 mutually orthogonal waveforms generated using xe "Walsh function"§Walsh functions.  These xe "modulation symbol"§modulation symbols are given in Table 6.1.3.1.6-1 and are numbered 0 through 63.  The modulation symbols shall be selected according to the following formula:

xe "Modulation symbol"§Modulation symbol number = c0 + 2c1 + 4c2 + 8c3 + 16c4 + 32c5,

where c5 shall represent the last (or most recent) and c0 the first (or oldest) binary valued (‘0’ and ‘1’) code symbol of each group of six code symbols that form a xe "modulation symbol"§modulation symbol.

The 64 by 64 matrix shown in Table 6.1.3.1.6-1 can be generated by means of the following recursive procedure:

H = 0,						H = ,			
			H = ,				H = , N is a power of 2.

The period of time required to transmit a single xe "modulation symbol"§modulation symbol is referred to as a Walsh symbol interval and shall be equal to 1/4800 second (= 208.333... µs).  The period of time associated with one-sixty-fourth of the modulation symbol is referred to as a xe "Walsh chip"§Walsh chip and shall be equal to 1/307200 second (= 3.255...  µs).

Within a Walsh symbol, Walsh chips shall be transmitted in the order of 0, 1, 2, ..., 63.

tc "Table 6.1.3.1.6-1.  64-ary Orthogonal Symbol Set"\c 	§Table 6.1.3.1.6-1.  64-ary Orthogonal Symbol Set

xe "Walsh Chip"§xe "Walsh Symbol"§

##### 6.1.3.1.7 Variable Data Rate Transmission xe "mobile station, transmitter, Reverse CDMA Channel, variable data rate"§

###### 6.1.3.1.7.1 Rates and Gating xe "mobile station, transmitter, Reverse CDMA Channel, variable data rate"§

Prior to transmission, the Reverse Traffic Channel xe "interleaver "§interleaver output stream is gated with a time filter that allows transmission of certain interleaver output symbols and deletion of others.  This process is illustrated in Figure 6.1.3.1.7.1-1.  As shown in the figure, the duty cycle of the transmission gate varies with the transmit data rate.  When the transmit data rate is 9600 bps, the transmission gate allows all interleaver output symbols to be transmitted.  When the transmit data rate is 4800 bps, the transmission gate allows one-half of the xe "interleaver"§interleaver output symbols to be transmitted, and so forth.  The gating process operates by dividing the 20 ms frame into 16 equal length (i.e., 1.25 ms) periods, called xe "power control group"§power control groups.  Certain power control groups are gated-on (i.e., transmitted), while other groups are gated-off (i.e., not transmitted).

The assignment of gated-on and gated-off groups, referred to as the xe "data burst randomizing"§data burst randomizing function, is specified in 6.1.3.1.7.2.  The gated-on xe "power control group"§power control groups are pseudorandomized in their positions within the frame.  The data burst randomizer ensures that every code symbol input to the repetition process is transmitted exactly once.  During the gated-off periods, the mobile station shall comply with the requirement in 6.1.2.2.2, thus reducing the interference to other mobile stations operating on the same xe "Reverse CDMA Channel"§Reverse CDMA Channel.

When transmitting on the Access Channel, the code symbols are repeated once (each symbol occurs twice) prior to transmission.  The data burst randomizer is not used when the mobile station transmits on the xe "Access Channel"§Access Channel.  Therefore, both copies of the repeated code symbols are transmitted as shown in Figure 6.1.3.1.7.1-2.

###### 6.1.3.1.7.2 Data Burst Randomizing Algorithm

The xe "data burst randomizer"§data burst randomizer generates a masking pattern of ‘0’s and ‘1’s that randomly masks out the redundant data generated by the code repetition.  The masking pattern is determined by the data rate of the frame and by a block of 14 bits taken from the xe "long code"§long code.  These 14 bits shall be the last 14 bits of the long code used for spreading in the second last power control group of the previous frame.  In other words, these are the 14 bits which occur exactly one xe "power control group "§power control group (1.25 ms) before each xe "Reverse Traffic Channel frame "§Reverse Traffic Channel frame boundary.  These 14 bits are denoted as

b  b  b  b  b  b  b b  b  b  b  b  b  b,

where b0 represents the oldest bit, and b13 represents the latest bit.

tc "Figure 6.1.3.1.7.1-1.  Reverse CDMA Channel Variable Data Rate Transmission Example"\c §Figure 6.1.3.1.7.1-1.  Reverse CDMA Channel Variable Data Rate Transmission Example

tc "Figure 6.1.3.1.7.1-2.  Access Channel Transmission Structure"\c §Figure 6.1.3.1.7.1-2.  Access Channel Transmission Structure

Each 20 ms xe "Reverse Traffic Channel frame"§Reverse Traffic Channel frame shall be divided into 16 equal length (i.e., 1.25 ms) xe "power control group"§power control groups numbered from 0 to 15 as shown in Figure 6.1.3.1.7.1-1.  The xe "data burst randomizer"§data burst randomizer algorithm shall be as follows:

Data Rate Selected:  9600 bps

Transmission shall occur on xe "power control group"§power control groups numbered:

0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15.

Data Rate Selected:  4800 bps

Transmission shall occur on xe "power control group"§power control groups numbered:

b , 2 + b, 4 + b , 6 + b , 8 + b , 10 + b , 12 + b, 14 + b.

Data Rate Selected:  2400 bps

Transmission shall occur on xe "power control group"§power control groups numbered:

b  if  b = 0,			or	2 + bif  b = 1;

4 + b  if  b = 0,		or	6 + b if  b = 1;

8 + b  if  b = 0 ,	or	10 + b if  b = 1;

12 + b if  b= 0,	or	14 + b if  b = 1.

Data Rate Selected:  1200 bps

Transmission shall occur on xe "power control group"§power control groups numbered:

b  if  (b = 0 and  b = 0), 		or	2 + b if  (b = 1 and  b = 0),

or	4 + b  if  (b = 0 and  b = 1), 	or	6 + b  if  (b = 1 and  b = 1);

8 + b  if  (b = 0 and b = 0),	or	10 + b  if  (b = 1 and b = 0),

or	12 + b  if  (b= 0 and b = 1), or	14 + b  if  (b = 1 and b = 1).

##### 6.1.3.1.8 Direct Sequence Spreading

Prior to transmission, the Reverse Traffic Channel and the xe "Access Channel"§Access Channel shall be direct sequence spread by the long code.  This spreading operation involves modulo-2 addition of the data burst randomizer output stream and the long code.

This long code shall be periodic with period 242-1 chips and shall satisfy the linear recursion specified by the following characteristic polynomial:

p(x) = x42 + x35 + x33 + x31 + x27 + x26 + x25 + x22 + x21 + x19 +
x18 + x17 + x16 + x10 + x7 + x6 + x5 + x3 + x2 + x1 + 1.

Each PN chip of the xe "long code"§long code shall be generated by the modulo-2 inner product of a 42-bit mask and the 42-bit state vector of the sequence generator as shown in Figure 6.1.3.1.8-1.

The time alignment of the long code generator shall be as shown in Figure 1.2-1.

The mask used for the long code varies depending on the channel type on which the mobile station is transmitting.  See Figure 6.1.3.1.8-2.  Specifically, when transmitting on the xe "Access Channel"§Access Channel, the mask shall be as follows:  M41 through M29 shall be set to ‘1100011001101’; M28 through M24 shall be set to the Access Channel number chosen (see 6.6.3.1.1.2); M23 through M21 shall be set to the xe "code channel"§code channel number for the associated xe "Paging Channel"§Paging Channel (the range is 1 through 7), M20 through M9 shall be set to the xe "REG_ZONE"§REG_ZONE value (see 7.7.2.3.2.1) for the current base station; and M8 through M0 shall be set to the xe "PILOT_PN"§PILOT_PN value for the current xe "CDMA Channel"§CDMA Channel (see 7.7.1.3).

When transmitting on the xe "Reverse Traffic Channel"§Reverse Traffic Channel, the mobile station shall use one of two xe "long code"§long code masks unique to that mobile station:  a xe "public long code"§public long code mask unique to the mobile station’s xe "ESN"§ESN or a private long code mask unique for each xe "MIN"§MIN.  The public long code mask shall be as follows:  M41 through M32 shall be set to ‘1100011000’, and M31 through M0 shall be set to a permutation of the mobile station’s xe "ESN"§ESN bits.  This permutation is specified as follows:

ESN =	(E31, E30, E29, E28, E27, E26, E25, . . . E2, E1, E0)

Permuted ESN = 	(E0, E31, E22, E13, E4, E26, E17, E8, E30, E21, E12, E3, E25, E16, E7, E29, E20, E11, E2, E24, E15, E6, E28, E19, E10, E1, E23, E14, E5, E27, E18, E9).

The xe "private long code"§private long code mask shall be as follows:  M41 through M40 shall be set to ‘01’, and M39 through M0 shall be set as specified in 6.3.12.3.

tc "Figure 6.1.3.1.8-1.  Long Code Generator"\c §Figure 6.1.3.1.8-1.  Long Code Generator

xe "Long Code Mask"§

tc "Figure 6.1.3.1.8-2.  Long Code Mask Format"\c §Figure 6.1.3.1.8-2.  Long Code Mask Format

##### 6.1.3.1.9 Quadrature Spreading

Following the direct sequence spreading, the Reverse Traffic Channel and Access Channel are spread in quadrature as shown in Figure 6.1.3.1-2.  The sequences used for this spreading shall be the zero-offset I and Q pilot PN sequences used on the Forward CDMA Channel (see 7.1.3.2.1).  These sequences are periodic with period 215 chips and shall be based on the following characteristic polynomials, respectively:

PI(x) = x15 + x13 + x9 + x8 + x7 + x5 + 1

(for the in-phase (I) sequence)

and

PQ(x) = x15 + x12 + x11 + x10 + x6 + x5 + x4 + x3 + 1

(for the quadrature-phase (Q) sequence).

The maximum length linear feedback shift register sequences, {i(n)} and {q(n)}, based on the above polynomials are of period 215-1 and can be generated by using the following linear recursions:

i(n) = i(n-15) _ i(n-10) _ i(n-8) _ i(n-7) _ i(n-6) _ i(n-2)

(based on PI(x) as the characteristic polynomial)

and

q(n) = q(n-15) _ q(n-12) _ q(n-11) _ q(n-10) _ q(n-9) _ q(n-5) _ q(n-4) _ q(n-3)

(based on PQ(x) as the characteristic polynomial),

where i(n) and q(n) are binary-valued (‘0’ and ‘1’) and the additions are modulo-2.  In order to obtain the I and Q pilot PN sequences (of period 215), a ‘0’ is inserted in {i(n)} and {q(n)} after 14 consecutive ‘0’ outputs (this occurs only once in each period).  Therefore, the pilot PN sequences have one run of 15 consecutive ‘0’ outputs instead of 14.

The mobile station shall align the I and Q pilot PN sequences such that the first chip on every even second mark as referenced to the transmit time reference (see 6.1.5.1) is the ‘1’ after the 15 consecutive ‘0’s (see Figure 1.2-1).

The pilot PN sequences repeat every 26.666... ms (= 215/1228800 seconds).  There are exactly 75 repetitions in every 2 seconds.

The spreading modulation shall be offset quadrature phase shift keying (OQPSK).  The data spread by the Q pilot PN sequence is delayed by half a chip time (406.901 ns) with respect to the data spread by the I pilot PN sequence.

The binary data (‘0’s and ‘1’s) I and Q shown in Figure 6.1.3.1-2 shall be mapped into phase according to Table 6.1.3.1.9-1.

tc "Table 6.1.3.1.9-1.  Reverse CDMA Channel I and Q Mapping"\c 	§Table 6.1.3.1.9-1.  Reverse CDMA Channel I and Q Mapping

| I | Q | Phase |
| --- | --- | --- |
| 0 | 0 | _/4 |
| 1 | 0 | 3_/4 |
| 1 | 1 | -3_/4 |
| 0 | 1 | -_/4 |

The resulting signal constellation and phase transition are shown in Figure 6.1.3.1.9-1.

tc "Figure 6.1.3.1.9-1.  Reverse CDMA Channel Signal Constellation and Phase Transition"\c §Figure 6.1.3.1.9-1.  Reverse CDMA Channel Signal Constellation and Phase Transition

##### 6.1.3.1.10 Baseband Filtering

Following the spreading operation, the I and Q data are applied to the inputs of the I and Q baseband filters as shown in Figure 6.1.3.1-2.  The baseband filters shall have linear phase.  The baseband filters shall have a frequency response S(f) that satisfies the limits given in Figure 6.1.3.1.10-1.  Specifically, the normalized frequency response of the filter shall be contained within ±d1 in the passband 0 _ f _ fp and shall be less than or equal to d2 in the stopband f _ fs.  The numerical values for the parameters are d1 = 1.5 dB, d2 = -40 dB, fp = 590 kHz, and fs = 740 kHz.

tc "Figure 6.1.3.1.10-1.  Baseband Filters Frequency Response Limits"\c §Figure 6.1.3.1.10-1.  Baseband Filters Frequency Response Limits

Let s(t) be the impulse response of the baseband filter.  Then s(t) shall satisfy the following equation:

Mean Squared Error = - h(k)] _ 0.03,

where the constants a and t are used to minimize the mean squared error.  The constant Ts is equal to 203.451... ns, which equals one quarter of a PN chip.  The values of coefficients h(k) are given in Table 6.1.3.1.10-1.  Note that h(k) equals h(47 - k).

tc "Table 6.1.3.1.10-1.  Coefficients h(k)"\c 	§Table 6.1.3.1.10-1.  Coefficients h(k)

| k | h(k) |
| --- | --- |
| 0, 47 | -0.025288315 |
| 1, 46 | -0.034167931 |
| 2, 45 | -0.035752323 |
| 3, 44 | -0.016733702 |
| 4, 43 | 0.021602514 |
| 5, 42 | 0.064938487 |
| 6, 41 | 0.091002137 |
| 7, 40 | 0.081894974 |
| 8, 39 | 0.037071157 |
| 9, 38 | -0.021998074 |
| 10, 37 | -0.060716277 |
| 11, 36 | -0.051178658 |
| 12, 35 | 0.007874526 |
| 13, 34 | 0.084368728 |
| 14, 33 | 0.126869306 |
| 15, 32 | 0.094528345 |
| 16, 31 | -0.012839661 |
| 17, 30 | -0.143477028 |
| 18, 29 | -0.211829088 |
| 19, 28 | -0.140513128 |
| 20, 27 | 0.094601918 |
| 21, 26 | 0.441387140 |
| 22, 25 | 0.785875640 |
| 23, 24 | 1.0 |

#### 6.1.3.2 Access Channel xe "mobile station, transmitter, modulation characteristics, Access Channel"§

The xe "Access Channel"§Access Channel is used by the mobile station to initiate communication with the base station and to respond to Paging Channel messages.  An Access Channel transmission is a coded, interleaved, and modulated spread-spectrum signal.  The Access Channel is random-access (see 6.6.3.1.1).  Access Channels are uniquely identified by their xe "long code"§long codes (see 6.1.3.1.8).

##### 6.1.3.2.1 Access Channel Time Alignment and Modulation Rate xe "mobile station, transmitter, Access Channel, time alignment/modulation rates"§

The mobile station shall transmit information on the xe "Access Channel"§Access Channel at a fixed data rate of 4800 bps.  An xe "Access Channel"§Access Channel frame shall be 20 ms in length.  An xe "Access Channel"§Access Channel frame shall begin only when System Time (see Figure 1.2-1) modulo 20 ms equals zero.

The synchronization, timing, and structure of the xe "Access Channel"§Access Channel are specified in 6.6.3.1.1 and 6.7.1.1.

The xe "Reverse CDMA Channel"§Reverse CDMA Channel may contain up to 32 Access Channels numbered 0 through 31 per supported xe "Paging Channel"§Paging Channel.  At least one Access Channel exists on the Reverse CDMA Channel for each xe "Paging Channel"§Paging Channel on the corresponding xe "Forward CDMA Channel"§Forward CDMA Channel.  Each Access Channel is associated with a single Paging Channel.

##### 6.1.3.2.2 Access Channel Convolutional Encoding

The xe "Access Channel"§Access Channel data shall be convolutionally encoded prior to transmission as specified in 6.1.3.1.3.

When generating Access Channel data, the encoder shall be initialized to the all zero state at the end of each 20 ms frame.

##### 6.1.3.2.3 Access Channel Code Symbol Repetition

Each code symbol output from the convolutional encoder on the xe "Access Channel"§Access Channel shall be repeated once (each symbol occurs two consecutive times) as specified in 6.1.3.1.4.

##### 6.1.3.2.4 Access Channel Interleaving xe "mobile station, transmitter, Access Channel, interleaver"§

The repeated code symbols on the xe "Access Channel"§Access Channel shall be interleaved as specified in 6.1.3.1.5.

##### 6.1.3.2.5 Access Channel Modulation xe "mobile station, transmitter, Access Channel, orthogonal modulation"§

The xe "Access Channel"§Access Channel data shall be modulated as specified in 6.1.3.1.6.

##### 6.1.3.2.6 Access Channel Gating xe "mobile station, transmitter, Access Channel, gating"§

The mobile station shall not gate off any power control group while transmitting on the xe "Access Channel"§Access Channel as specified in 6.1.3.1.7.1.

##### 6.1.3.2.7 Access Channel Direct Sequence Spreading xe "mobile station, transmitter, Access Channel, direct sequence spreading"§

The Access Channel shall be spread by the long code as specified in 6.1.3.1.8.

##### 6.1.3.2.8 Access Channel Quadrature Spreading xe "mobile station, transmitter, Access Channel, quadrature spreading"§

The Access Channel shall be quadrature spread by the pilot PN sequences as specified in 6.1.3.1.9.

##### 6.1.3.2.9 Access Channel Baseband Filtering xe "mobile station, transmitter, Access Channel, baseband filtering"§

The Access Channel shall be filtered as specified in 6.1.3.1.10.

##### 6.1.3.2.10 Access Channel Frame Structure

Each xe "Access Channel"§Access Channel frame contains 96 bits (20 ms frame at 4800 bps).  Each xe "Access Channel"§Access Channel frame shall consist of 88 information bits and eight xe "Encoder Tail Bits (see Figure 6.1.3.2.10-1).  The 8-bit CRC used for Reverse Traffic Channel frames at 4800 bps shall not be used. "§Encoder Tail Bits (see Figure 6.1.3.2.10-1).  The 8-bit CRC used for Reverse Traffic Channel frames at 4800 bps shall not be use

tc "Figure 6.1.3.2.10-1.  Access Channel Frame Structure"\c §Figure 6.1.3.2.10-1.  Access Channel Frame Structure

#### 6.1.3.3 Reverse Traffic Channel xe "mobile station, transmitter, modulation characteristics, Reverse Traffic Channel"§

The xe "Reverse Traffic Channel"§Reverse Traffic Channel is used for the transmission of primary traffic, secondary traffic, and signaling traffic to the base station during a call.

##### 6.1.3.3.1 Reverse Traffic Channel Time Alignment and Modulation Rates xe "mobile station, transmitter, Reverse Traffic Channel, time alignment"§ xe "mobile station, transmitter, Reverse Traffic Channel, modulation rates"§

The xe "mobile station shall transmit information on the Reverse Traffic Channel"§mobile station shall transmit information on the Reverse Traffic Channel at a variable data rate of 9600, 4800, 2400, or 1200 bps.  A xe "Reverse Traffic Channel frame "§Reverse Traffic Channel frame shall be fixed at 20 ms in length.

A zero-offset Reverse Traffic Channel frame shall begin only when System Time (see Figure 1.2-1) modulo 20 ms equals zero.  A zero-offset Traffic Channel frame shall have the xe "FRAME_OFFSET"§FRAME_OFFSET parameter (see the xe "Channel Assignment Message"§Channel Assignment Message in 7.7.2.3.2.8) set to zero.  A mobile station shall support staggered Traffic Channel frames.  A staggered frame shall begin 1.25 ¥ FRAME_OFFSET ms later than the zero-offset Traffic Channel frame.  The Reverse Traffic Channel interleaver block ;shall be aligned with the xe "Reverse Traffic Channel frame"§Reverse Traffic Channel frame.

##### 6.1.3.3.2 Reverse Traffic Channel Convolutional Encoding xe "mobile station, transmitter, Reverse Traffic Channel, convolutional encoder"§

The xe "Reverse Traffic Channel"§Reverse Traffic Channel data shall be convolutionally encoded prior to transmission as specified in 6.1.3.1.3.

When generating Reverse Traffic Channel data, the encoder shall be initialized to the all zero state at the end of each 20 ms frame.

##### 6.1.3.3.3 Reverse Traffic Channel Code Symbol Repetition

Reverse Traffic Channel code symbol repetition shall be as specified in 6.1.3.1.4.

##### 6.1.3.3.4 Reverse Traffic Channel Interleaving xe "mobile station, transmitter, Reverse Traffic Channel, interleaver"§

The code symbols (or repeated code symbols when a data rate lower than 9600 bps is used) on the xe "Reverse Traffic Channel"§Reverse Traffic Channel shall be interleaved as specified in 6.1.3.1.5.

##### 6.1.3.3.5 Reverse Traffic Channel Modulation xe "mobile station, transmitter, Reverse Traffic Channel, orthogonal modulation"§

The xe "Reverse Traffic Channel"§Reverse Traffic Channel data shall be modulated as specified in 6.1.3.1.6.

##### 6.1.3.3.6 Reverse Traffic Channel Gating xe "mobile station, transmitter, Reverse Traffic Channel, gating, data burst randomizing"§

The mobile station shall perform the data burst randomizing function as specified in 6.1.3.1.7 while transmitting on the xe "Reverse Traffic Channel"§Reverse Traffic Channel.

##### 6.1.3.3.7 Reverse Traffic Channel Direct Sequence Spreading xe "mobile station, transmitter, Reverse Traffic Channel, direct sequence spreading"§

The Reverse Traffic Channel shall be spread by the long code as specified in 6.1.3.1.8.

##### 6.1.3.3.8 Reverse Traffic Channel Quadrature Spreading xe "mobile station, transmitter, Reverse Traffic Channel, quadrature spreading"§

The Reverse Traffic Channel shall be quadrature spread by the pilot PN sequences as specified in 6.1.3.1.9.

##### 6.1.3.3.9 Reverse Traffic Channel Baseband Filtering xe "mobile station, transmitter, Reverse Traffic Channel, baseband filtering"§

The Reverse Traffic Channel shall be filtered as specified in 6.1.3.1.10.

##### 6.1.3.3.10 Reverse Traffic Channel Frame Structure xe "mobile station, transmitter, Reverse Traffic Channel, structure"§

Reverse Traffic Channel frames sent at the 9600 bps transmission rate shall consist of 192 bits.  These 192 bits shall be composed of 172 information bits followed by 12 frame quality indicator ;(xe "CRC"§CRC) bits and eight xe "Encoder Tail Bits "§Encoder Tail Bits as shown in Figure 6.1.3.3.10-1.

Reverse Traffic Channel frame;s sent at the 4800 bps transmission rate shall consist of 96 bits.  These 96 bits shall be composed of 80 information bits followed by eight frame quality indicator ;(xe "CRC"§CRC) bits and eight xe "Encoder Tail Bits "§Encoder Tail Bits as shown in Figure 6.1.3.3.10-1.

Reverse Traffic Channel frame;s sent at the 2400 bps transmission rate shall consist of 48 bits.  These 48 bits shall be composed of 40 information bits followed by eight xe "Encoder Tail Bits "§Encoder Tail Bits as shown in Figure 6.1.3.3.10-1.

Reverse Traffic Channel frame;s sent at the 1200 bps transmission rate shall consist of 24 bits.  These 24 bits shall be composed of 16 information bits followed by eight xe "Encoder Tail Bits "§Encoder Tail Bits as shown in Figure 6.1.3.3.10-1.

The number of bits in the frame and the frame format (see 6.1.3.3.11) shall be variable on a frame-by-frame basis.

tc "Figure 6.1.3.3.10-1.  Reverse Traffic Channel Frame Structure"\c §Figure 6.1.3.3.10-1.  Reverse Traffic Channel Frame Structure

###### 6.1.3.3.10.1 Reverse Traffic Channel Frame Quality Indicator xe "mobile station, transmitter, Reverse Traffic Channel, frame quality indicator"§

Each 9600 bps and 4800 bps frame shall include a xe "frame quality indicator.  This frame quality indicator is a CRC"§frame quality indicator.  This frame quality indicator is a CRC.  No frame quality indicator is used for the 2400 bps and 1200 bps transmission rates.

For both the 9600 bps and 4800 bps rates, the xe "frame quality indicator shall be calculated on all bits within the frame, except the frame quality indicator itself and the .i.Encoder Tail Bits"§frame quality indicator shall be calculated on all bits within the frame, except the frame quality indicator itself and the Enco.  The 9600 bps transmission rate shall use a 12-bit frame quality indicator.  The generator polynomial for this frame quality indicator shall be as follows:

g(x) = x12 + x11 + x10 + x9 + x8 + x4 + x + 1.

The 4800 bps transmission rate shall use an 8-bit xe "frame quality indicator.  The generator polynomial for this frame quality indicator shall be as follows:"§frame quality indicator.  The generator polynomial for this frame quality indicator shall be as follows

g(x) = x8 + x7 + x4 + x3 + x + 1.

The xe "CRC"§CRCs shall be computed according to the following procedure using the logic shown in Figures 6.1.3.3.10.1-1 and 6.1.3.3.10.1-2:

•	Initially, all shift register elements shall be set to logical one and the switches shall be set in the up position.

•	The register shall be clocked 172 times (for 192-bit frame) or 80 times (for 96-bit frame) with the information bits as input.

•	The switches shall be set in the down position, and the register shall be clocked an additional 12 times (for 192-bit frame) or 8 times (for 96-bit frame).  The 12 or 8 additional output bits shall be the check bits.

•	The bits shall be transmitted in the order calculated.

###### 6.1.3.3.10.2 Reverse Traffic Channel Encoder Tail Bits xe "mobile station, transmitter, Reverse Traffic Channel, encoder tail bits"§

The last 8 bits of each xe "Reverse Traffic Channel frame "§Reverse Traffic Channel frame are called the xe "Encoder Tail Bits"§Encoder Tail Bits.  These 8 bits shall be set to ‘0’.  The mobile station shall not include these bits in the computation of the 9600 and 4800 bps frame quality indicators.

tc "Figure 6.1.3.3.10.1-1.  Reverse Traffic Channel Frame Quality Indicator Calculation at the 9600 bps Rate"\c §Figure 6.1.3.3.10.1-1.  Reverse Traffic Channel Frame Quality Indicator Calculation at the 9600bps Rate

tc "Figure 6.1.3.3.10.1-2.  Reverse Traffic Channel Frame Quality Indicator Calculation at the 4800 bps Rate"\c §Figure 6.1.3.3.10.1-2.  Reverse Traffic Channel Frame Quality Indicator Calculation at the 4800bps Rate

###### 6.1.3.3.10.3 Traffic Channel Preamble

The Traffic Channel preamble shall consist of frames of 192 zeros that are transmitted at the 9600 bps rate.  The Traffic Channel preamble shall not include the frame quality indicator.

The Traffic Channel preamble is transmitted to aid the base station in performing initial acquisition of the Reverse Traffic Channel.  The Traffic Channel preamble transmission occurs after the mobile station’s transmitter has been enabled during the xe "Traffic Channel Initialization Substate"§Traffic Channel Initialization Substate of the xe "Mobile Station Control on the Traffic Channel State and before receipt of the first valid message on the Forward Traffic Channel (see 6.6.4.2)."§Mobile Station Control on the Traffic Channel State and before receipt of the first valid message on the Forward Traffic Channel

###### 6.1.3.3.10.4 Null Traffic Channel Data

Null Traffic Channel data shall consist of frames of 16 ones followed by 8 zeros (the Encoder Tail Bits) sent at the 1200 bps rate.

The mobile station transmits null Traffic Channel data when no service option is active.  Typically this occurs for a mobile station terminated call when the mobile station is in either the Waiting for Order Substate or the Waiting for Mobile Station Answer Substate of the xe "Mobile Station Control on the Traffic Channel State"§Mobile Station Control on the Traffic Channel State.  Null Traffic Channel data serves as a “keep-alive” operation so that the base station can maintain synchronization with the mobile station.

##### 6.1.3.3.11 Multiplex Option 1 Information Bits xe "mobile station, transmitter, Reverse Traffic Channel, frame format"§

Multiplex Option 1 is also referred to as the default multiplex option.  It provides for the transmission of primary traffic and signaling traffic.  Signaling traffic may be transmitted via blank-and-burst with the signaling traffic using all of the frame or via dim-and-burst with the primary traffic and signaling traffic sharing the frame.  Multiplex Option 1 also supports the transmission of secondary traffic.  When primary traffic is active, secondary traffic is transmitted via dim-and-burst with the primary traffic and secondary traffic sharing the frame.  When primary traffic is not active, secondary traffic is transmitted via blank-and-burst with the secondary traffic using all of the frame.  The information bit structures for primary and signaling traffic are specified in 6.1.3.3.11.1; the information bit structures for secondary traffic are specified in 6.1.3.3.11.2.  Table 6.1.3.3.11-1 shows the information bit structures supported by Multiplex Option 1.

The mobile station shall support Multiplex Option 1.  The mobile station shall support the transmission of primary traffic and signaling traffic using the information bit structures specified in 6.1.3.3.11.1.  The mobile station may support secondary traffic, and if so, the mobile station shall also use the information bit structures specified in 6.1.3.3.11.2.

tc "Table 6.1.3.3.11-1.  Reverse Traffic Channel Information Bits for Multiplex Option 1"\c 	§Table 6.1.3.3.11-1.  Reverse Traffic Channel Information Bits for Multiplex Option 1

|  | Format Bits |  |  |  | Primary Traffic |  | Signaling Traffic |  | Secondary Traffic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmit Rate (bits/sec) | Mixed Mode (MM) | Burst Format (BF) | Traffic Type (TT) | Traffic Mode (TM) | bits/ frame | bits/ sec | bits/ frame | bits/ sec | bits/ frame | bits/ sec |
|  | 0 | – | – | – | 171 | 8550 | 0 | 0 | 0 | 0 |
|  | 1 | 0 | 0 | 00 | 80 | 4000 | 86 | 4300 | 0 | 0 |
|  | 1 | 0 | 0 | 01 | 40 | 2000 | 126 | 6300 | 0 | 0 |
|  | 1 | 0 | 0 | 10 | 16 | 800 | 150 | 7500 | 0 | 0 |
|  | 1 | 1 | 0 | – | 0 | 0 | 168 | 8400 | 0 | 0 |
| 9600 * | 1 | 1 | 1 | – | 0 | 0 | 0 | 0 | 169 | 8450 |
| * | 1 | 0 | 1 | 00 | 80 | 4000 | 0 | 0 | 87 | 4350 |
| * | 1 | 0 | 1 | 01 | 40 | 2000 | 0 | 0 | 127 | 6350 |
| * | 1 | 0 | 1 | 10 | 16 | 800 | 0 | 0 | 151 | 7550 |
| 4800 | – | – | – | – | 80 | 4000 | 0 | 0 | 0 | 0 |
| 2400 | – | – | – | – | 40 | 2000 | 0 | 0 | 0 | 0 |
| 1200 | – | – | – | – | 16 | 800 | 0 | 0 | 0 | 0 |
| Notes: Secondary traffic structures, marked with *, are optional. Signaling traffic bits do not include the SOM bit. |  |  |  |  |  |  |  |  |  |  |

###### 6.1.3.3.11.1 Primary and Signaling Traffic with Multiplex Option 1 xe "mobile station, transmitter, Reverse Traffic Channel, frame format"§

The mobile station shall support the information bit structures described in Table 6.1.3.3.11-1 and Figure 6.1.3.3.11.1-1.

For signaling traffic, the first bit of the frame’s signaling portion shall be a SOM (Start of Message) Bit.  The SOM Bit shall be a ‘1’ if a Reverse Traffic Channel message (signaling message) begins at the following bit.  A Reverse Traffic Channel message capsule shall not begin anywhere else in the frame.  The SOM Bit shall be a ‘0’ if a message does not begin at the following bit.  This occurs if the frame contains part of a message that began in a previous frame.

tc "Figure 6.1.3.3.11.1-1.  Information Bits for Primary Traffic and Signaling Traffic"\c §Figure 6.1.3.3.11.1-1.  Information Bits for Primary Traffic and Signaling Traffic
(Part 1 of 2)

Figure 6.1.3.3.11.1-1.  Information Bits for Primary Traffic and Signaling Traffic
(Part 2 of 2)

###### 6.1.3.3.11.2 Secondary Traffic with Multiplex Option 1 xe "mobile station, transmitter, Reverse Traffic Channel, frame format"§

If the mobile station supports secondary traffic, the mobile station shall use the information bit structures described in Table 6.1.3.3.11-1 and Figure 6.1.3.3.11.2-1.

tc "Figure 6.1.3.3.11.2-1.  Information Bits for Secondary Traffic"\c §Figure 6.1.3.3.11.2-1.  Information Bits for Secondary Traffic

###### 6.1.3.3.11.3 Use of Various Information Bit Formats xe "mobile station, transmitter, Reverse Traffic Channel, frame format"§

When neither a primary traffic service option nor a secondary traffic service option is active, the mobile station shall transmit signaling traffic using only blank-and-burst frames.  When not transmitting signaling traffic, the mobile station shall transmit only null Traffic Channel data frames.

When a primary traffic service option is active and a secondary traffic service option is not active, the mobile station shall use the information formats specified in 6.1.3.3.11.1.  The mobile station shall not transmit null Traffic Channel data.  The mobile station should use the dim-and-burst information format specified in 6.1.3.3.11.1 for signaling traffic.

When a primary traffic service option is not active and a secondary traffic service option is active, the mobile station shall use the information formats specified in 6.1.3.3.11.2 to transmit secondary traffic.  The mobile station shall use the blank-and-burst format specified in 6.1.3.3.11.1 for signaling traffic.  The mobile station shall transmit null Traffic Channel data when neither secondary traffic nor signaling traffic is to be sent.

When both a primary traffic service option and a secondary traffic service option are active, the mobile station shall use the information formats specified in 6.1.3.3.11.1 and 6.1.3.3.11.2.  The mobile station shall not transmit null Traffic Channel data.  The mobile station should use the dim-and-burst information format specified in 6.1.3.3.11.1 for signaling traffic.

###### 6.1.3.3.11.4 Control of Service Options

Multiplex Option 1 controls the number of bits that the connected service options supplies for a frame (see Appendix A).

The mobile station shall use the following rules when a primary traffic service option is active:  If signaling traffic is to be transmitted in a frame, Multiplex Option 1 shall either restrict the primary traffic service option to generate zero bits (for a blank-and-burst frame) or to generate less than 171 bits (for a dim-and-burst frame).  If secondary traffic is to be transmitted in a frame, Multiplex Option 1 may restrict the primary traffic service option to generate less than 171 bits but shall allow the primary traffic service option to generate at least 16 bits.  In all other cases, Multiplex Option 1 shall allow the primary traffic service option to generate either 16, 40, 80, or 171 bits for a frame.

### 6.1.4 Limitations on Emissions xe "mobile station, transmitter, emission limits"§

#### 6.1.4.1 Bandwidth Occupied xe "mobile station, transmitter, bandwidth"§

Modulation products in a bandwidth of 30 kHz centered ±750 kHz from the channel center frequency should be at least 45 dB and shall be at least 42 dB below the xe "mean output power"§mean output power level.

#### 6.1.4.2 Conducted Spurious Emissions xe "mobile station, transmitter, conducted spurious emissions"§

##### 6.1.4.2.1 Suppression Inside Cellular Band

When transmitting on any CDMA Channel, spurious emission levels in the mobile station transmit band between 824 and 849 MHz shall be less than the limits specified in Table 6.1.4.2.1-1.

In addition, spurious emissions in each 1.23 MHz band located anywhere in the mobile station receive band between 869 and 894 MHz shall be less than -80 dBm.  These requirements shall apply to measurements made at the mobile station antenna connector.

tc "Table 6.1.4.2.1-1.  Spurious Emission Limits When Transmitting"\c 	§Table 6.1.4.2.1-1.  Spurious Emission Limits When Transmitting

| For Frequency f, with ¦f - Center Frequency¦ | Greater than 780 kHz | Greater than 1.98 MHz |
| --- | --- | --- |
| Spurious emission levels | (a) -42 dBc/30 kHz | (a) -54 dBc/30 kHz |
| shall be less than either (a), or both (b) and (c) | (b) -60 dBm/30 kHz (c) -54 dBm/1.23 MHz |  |
| Spurious emission levels | (a) -45 dBc/30 kHz | (a) -60 dBc/30 kHz |
| should be less than either (a), or both (b) and (c) | (b) -66 dBm/30 kHz (c) -60 dBm/1.23 MHz |  |

Note:	All frequencies in the measurement bandwidth shall satisfy the restrictions 
on |f - Center Frequency|.

##### 6.1.4.2.2 Suppression Outside Cellular Band

Current FCC rules shall apply.

#### 6.1.4.3 Radiated Spurious Emissions xe "mobile station, transmitter, radiated spurious emissions"§

Radiated spurious emissions (from sources other than the antenna connector) shall meet levels corresponding to the conducted spurious requirements listed in 6.1.4.2.

### 6.1.5 Synchronization and Timing xe "mobile station, transmitter, sync and timing"§

#### 6.1.5.1 Time Reference

Figure 1.2-1 illustrates the nominal relationship between the mobile station and base station transmit and receive time references.  The mobile station shall establish a time reference which is used to derive timing for the transmit chip, symbol, frame slot, and system timing.  The mobile station time reference shall be synchronized to the time of occurrence, as measured at the mobile station antenna connector, of the earliest arriving multipath component being used for demodulation.  If another multipath component (belonging to the same Pilot Channel or to a different Pilot Channel) becomes the earliest arriving multipath component to be used, the mobile station time reference shall track to the new component.

When receiving the Forward Traffic Channel, the mobile station time reference shall be used to determine the transmit timing of the Reverse Traffic Channel.  When receiving the xe "Paging Channel"§Paging Channel, the mobile station time reference shall be used to determine the transmit timing of the xe "Access Channel"§Access Channel.

The mobile station transmit time (see Figure 1.2-1) as measured at the mobile station antenna connector shall be maintained within ±1 µs of the time of occurrence, as measured at the mobile station antenna connector, of the earliest arriving multipath component being used for demodulation.

If a new multipath component becomes the earliest arriving multipath component used for demodulation, the mobile station time reference shall be corrected by no faster than 1/4 chip (203.451 ns) in any 200 ms period and no slower than 300 ns per second.

#### 6.1.5.2 PN Time Tolerance xe "mobile station, transmitter, PN time tolerance"§

The time error between the xe "long code"§long code timing and the 64-ary xe "Walsh symbol "§Walsh symbol sequence timing shall be less than ±50 ns.

### 6.1.6 Transmitter Performance Requirements

System performance is predicated on transmitters meeting the requirements set forth in the document titled “Recommended Minimum Performance Standards for 800 MHz Wideband Spread Spectrum Dual-Mode Mobile Stations.”

## 6.2 Receiver xe "mobile station, receiver"§

### 6.2.1 Frequency Parameters xe "mobile station, receiver, frequency parameters"§

#### 6.2.1.1 Channel Spacing and Designation xe "mobile station, receiver, channel spacing and designation"§

Channel spacing and designation for the mobile station reception shall be as specified in 2.1.1.1.  Valid channels for CDMA operations shall be as specified in 6.1.1.1.

### 6.2.2 Demodulation Characteristics

#### 6.2.2.1 Processing

The mobile station demodulation process shall perform complementary operations to the base station modulation process on the Forward CDMA Channel (see 7.1.3).

#### 6.2.2.2 Forward Traffic Channel Frame Categorization

The xe "multiplex sublayer"§multiplex sublayer in the mobile station shall classify received xe "Forward Traffic Channel frame"§Forward Traffic Channel frames into the following 14 categories (see 7.1.3.5.11):

1.	9600 bps frame, xe "primary traffic "§primary traffic only

2.	9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/2 primary and xe "signaling traffic"§signaling traffic

3.	9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/4 primary and xe "signaling traffic"§signaling traffic

4.	9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/8 primary and xe "signaling traffic"§signaling traffic

5.	9600 bps frame, xe "blank-and-burst"§blank-and-burst with xe "signaling traffic"§signaling traffic only

6.	4800 bps frame, xe "primary traffic "§primary traffic only

7.	2400 bps frame, xe "primary traffic "§primary traffic only

8.	1200 bps frame, xe "primary traffic "§primary traffic or null data only

9.	9600 bps frame, xe "primary traffic "§primary traffic only, with probable bit errors

10.	Frame quality insufficient to decide upon rate

11.	9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/2 primary and xe "secondary traffic"§secondary traffic

12.	9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/4 primary and xe "secondary traffic"§secondary traffic

13.	9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/8 primary and xe "secondary traffic"§secondary traffic

14.	9600 bps frame, xe "blank-and-burst"§blank-and-burst with xe "secondary traffic"§secondary traffic only

Frames in categories 9 and 10 are bad frames; all frames otherwise categorized are considered good frames.

The Forward Traffic Channel frame classification shall be done for each frame via xe "frame quality indicator"§frame quality indicator (xe "CRC"§CRC checks, available for 9600 and 4800 bps frames), decoder quality information, and the xe "Mixed Mode Bit"§Mixed Mode Bit, xe "Burst Format Bit"§Burst Format Bit, xe "Traffic Type Bit, and Traffic Mode Bits"§Traffic Type Bit, and Traffic Mode Bits.  If xe "primary traffic "§primary traffic is active and xe "secondary traffic"§secondary traffic is not active, then the mobile station shall categorize the received frames into one of categories 1 through 10.  If primary traffic is not active and secondary traffic is active, then the mobile station shall categorize the received frames into one of categories 5, 8, 10 and 14.  If neither primary traffic nor secondary traffic is active, then the mobile station shall categorize the received frames into one of categories 5, 8, and 10.  Mobile stations that do not implement secondary traffic services are not required to implement categories 11 through 14.

#### 6.2.2.3 Forward Traffic Channel Time Alignment

The Forward Traffic Channel frame time alignment is specified in 7.1.3.5.1.  A mobile station shall support staggered Forward Traffic Channel frames.

### 6.2.3 Limitations on Emissions xe "mobile station, receiver, emission limits"§

#### 6.2.3.1 Conducted Spurious Emissions xe "mobile station, receiver, spurious emissions"§

##### 6.2.3.1.1 Suppression Inside Cellular Band

Total spurious emissions in each 1.23 MHz band located in the mobile station’s receive band between 869 and 894 MHz shall not exceed -80 dBm.  Total spurious emissions in each 1.23 MHz band located in the mobile station’s transmit band between 824 and 849 MHz shall not exceed -60 dBm.  These requirements shall apply to measurements made at the mobile station antenna connector, with the transmitter disabled.

##### 6.2.3.1.2 Suppression Outside Cellular Band

Current FCC rules shall apply.

#### 6.2.3.2 Radiated Spurious Emissions

Current FCC rules shall apply.

### 6.2.4 Receiver Performance Requirements

System performance is predicated on receivers meeting the requirements set forth in the document titled “Recommended Minimum Performance Standards for 800 MHz Wideband Spread Spectrum Dual-Mode Mobile Stations.”

## 6.3 Security and Identification xe "mobile station, security and identification"§

### 6.3.1 Mobile Station Identification Number xe "mobile station, identification number"§

See 2.3.1.

### 6.3.2 Electronic Serial Number xe "mobile station, electronic serial number"§

See 2.3.2.

### 6.3.3 Station Class Mark xe "mobile station, station class mark"§

See 2.3.3.

### 6.3.4 Registration Memory xe "mobile station, registration memory"§

See 2.3.4 for registration memory when operating in the analog mode.

The mobile station shall have memory to store one element in the zone-based registration; list xe "ZONE_LISTs-p (see 6.6.5.1.5).  This stored element shall include both REG_ZONE and the corresponding (.i.SID"§ZONE_LISTs-p (see 6.6.5.1.5).  This stored element shall include both REG_ZONE and the corresponding (SID, xe "NID"§NID) pair.  The data retention time under power-off conditions shall be at least 48 hours.  If, after 48 hours, the data integrity cannot be guaranteed, then the entry in ZONE_LISTs-p shall be deleted upon power-on.

The mobile station shall have memory to store one element in the system/network registration; list xe "SID_NID_LISTs-p"§SID_NID_LISTs-p (see 6.6.5.1.5).  The data retention time under power-off conditions shall be at least 48 hours.  If, after 48 hours, the data integrity cannot be guaranteed, then the entry in SID_NID_LISTs-p shall be deleted upon power-on.

The mobile station shall have memory to store the xe "distance-based registration"§distance-based registration variables xe "BASE_LAT_REGs-p"§BASE_LAT_REGs-p, xe "BASE_LONG_REG"§BASE_LONG_REGs-p;, and xe "REG_DIST_REG"§REG_DIST_REGs-p; (see 6.6.5.1.4).  The data retention time under power-off conditions shall be at least 48 hours.  If, after 48 hours, the data integrity cannot be guaranteed, then REG_DIST_REG;s-p shall be set to zero upon power-on.

### 6.3.5 Access Overload Class xe "mobile station, access overload class"§

See 2.3.5.

### 6.3.6 Reserved

### 6.3.7 Reserved

### 6.3.8 Home System and Network Identification xe "home system identification"§ xe "network identification"§

In addition to the HOME_SIDp parameter that the mobile station stores for each MIN that can be associated with the mobile station (see 2.3.8), the mobile station shall provide memory to store at least one home (SIDp, NIDp) pair for each MIN that can be associated with the mobile station.  The mobile station shall also provide memory to store the 1-bit parameters MOB_TERM_HOMEp, MOB_TERM_FOR_SIDp, and MOB_TERM_FOR_NIDp for each MIN that can be associated with the mobile station (see 6.6.5.3).

### 6.3.9 Local Control Option xe "mobile station, local control option"§

See 2.3.9.

### 6.3.10 Preferred Operation Selection xe "mobile station, preferred operation selection"§

#### 6.3.10.1 Preferred System xe "mobile station, preferred system selection"§

See 2.3.10.1.

#### 6.3.10.2 Preferred CDMA or Analogxe "mobile station, CDMA/analog preference"§

See 2.3.10.2.

### 6.3.11 Discontinuous Reception

The mobile station shall provide memory to store the preferred slot cycle index, SLOT_CYCLE_INDEXp (see 6.6.2.1.1.3).

### 6.3.12 Authentication, Encryption of Signaling Information/User Data and Voice Privacyxe "mobile station, authentication"§

#### 6.3.12.1 Authentication

Authentication is the process by which information is exchanged between a mobile station and base station for the purpose of confirming the identity of the mobile station.  The intent of the authentication process is to demonstrate that the mobile station and base station possess identical sets of Shared Secret Data (SSD).  During the authentication process, the mobile station processes all messages received, whether related to authentication or not.

##### 6.3.12.1.1 Shared Secret Data (SSD)

SSD is a 128-bit quantity that is stored in semi-permanent memory in the mobile station, as specified in 2.3.12.1.1.

SSD_A is used to support the authentication procedures and SSD_B is used to support voice privacy and message encryption.  SSD is generated according to the procedure specified in 2.3.12.1.8 or 6.3.12.1.9.

##### 6.3.12.1.2 Random Challenge Memory (RAND)

See 2.3.12.1.2.

##### 6.3.12.1.3 Call History Parameter (COUNT)

See 2.3.12.1.3.

##### 6.3.12.1.4 Authentication of Mobile Station Registrations

The following authentication procedures shall be performed when the AUTH field of the Access Parameters Message is set to ‘01’ (standard authentication mode), and the mobile station attempts to register (by sending an Initial Registration Message on the Access Channel) or to update its registration (by sending a Registration Update Message on the Access Channel):

The mobile station shall initialize the authentication algorithm (CAVE) as illustrated in Figure 6.3.12.1.4-1.  If more than one MIN is specified in the message, the CAVE algorithm shall be initialized using the first MIN in the message, whether added or deleted.  If the message contains no MIN, the CAVE algorithm shall be initialized using the least significant 24 bits of the ESN in place of MIN1.

The mobile station shall then execute the CAVE algorithm (see 2.3.12.1.9).  The 18-bit output of CAVE shall be used to fill the AUTHR field of the Initial Registration Message or Registration Update Message, as appropriate.  The RANDC (eight most significant bits of RAND) and COUNT fields of the message shall be filled with the current values stored in the mobile station.

The base station computes the value of AUTHR using its internally stored value of SSD_A.  The base station compares its computed value of AUTHR to the value received from the mobile station.

The base station also compares the received value of RANDC to the most significant eight bits of its internally stored value of RAND.

The base station may also compare the received value of COUNT with its internally stored value associated with the received MIN/ESN.

If any of the comparisons fail, the base station may deem the registration attempt unsuccessful, initiate the Unique Challenge-Response Procedure (see 6.3.12.1.5) or commence the process of updating SSD (see 6.3.12.1.9).

tc "Figure 6.3.12.1.4-1.  Computation of AUTHR for Authentication of Mobile Station Registrations"\c §Figure 6.3.12.1.4-1.  Computation of AUTHR for Authentication of Mobile Station Registrations

##### 6.3.12.1.5 Unique Challenge-Response Procedure

The Unique Challenge-Response Procedure is initiated by the base station and can be carried out over either on the Paging Channel and Access Channel, or on the Forward Traffic Channel and Reverse Traffic Channel.  The procedure is as follows:

The base station generates the 24-bit quantity RANDU and sends it to the mobile station in the Authentication Challenge Message on either the Paging Channel or the Forward Traffic Channel.  Upon receipt of the Authentication Challenge Message, the mobile station shall initialize the authentication algorithm (CAVE) as illustrated in Figure 6.3.12.1.5-1.  On the Paging Channel, the Authentication Challenge Message contains the MIN and ESN of the mobile station being challenged.  On the Forward Traffic Channel, the Authentication Challenge Message does not contain the MIN and ESN; the MIN and ESN used by both the base station and the mobile station shall be the MIN and ESN used to establish the call.

The mobile station shall then execute the CAVE algorithm (see 2.3.12.1.9).  The 18-bit output of CAVE shall be used to fill the AUTHU field of the Authentication Challenge Response Message, which shall be sent to the base station.

The base station computes the value of AUTHU using its internally stored value of SSD_A.  The base station compares its computed value of AUTHU to the value received from the mobile station.  If the comparison fails, the base station may deny further access attempts by the mobile station, drop the call in progress, or initiate the process of updating SSD (see 6.3.12.1.9).

tc "Figure 6.3.12.1.5-1.  Computation of AUTHU for the Unique Challenge-Response Procedure"\c §Figure 6.3.12.1.5-1.  Computation of AUTHU for the Unique Challenge-Response Procedure

##### 6.3.12.1.6 Authentication of Mobile Station Originations

When the AUTH field of the Access Parameters Message sent on the Paging Channel is set to ‘01’ (standard authentication mode), and the mobile station attempts to originate a call (by sending an Origination Message on the Access Channel), the following authentication procedures shall be performed:

The mobile station shall initialize the authentication algorithm (CAVE) as illustrated in Figure 6.3.12.1.6-1.  The DIGITS field shall contain the least significant four bits of each of the last six digits contained in the Origination Message.  If fewer than six digits are included in the Origination Message, the most significant bits of MIN1 shall be used to replace the missing digits.

The mobile station shall then execute the CAVE algorithm (see 2.3.12.1.9).  The 18-bit output of CAVE shall be used to fill the AUTHR field of the Origination Message.  The RANDC (eight most significant bits of RAND) and COUNT fields of the message shall be filled with the current values stored in the mobile station.

The base station computes the value of AUTHR using its internally stored value of SSD_A.  The base station compares its computed value of AUTHR to the value received from the mobile station.

The base station also compares the received value of RANDC to the most significant eight bits of its internally stored value of RAND.

The base station may also compare the received value of COUNT with its internally stored value associated with the received MIN/ESN.

If the comparisons executed at the base station are successful, the base station initiates the appropriate channel assignment procedures.  After channel assignment, the base station may issue a Set Parameters Message on the Forward Traffic Channel, modifying the value of COUNT in the mobile station.

If any of the comparisons fail, the base station may deny service, initiate the Unique Challenge-Response Procedure (see 6.3.12.1.5) or commence the process of updating SSD (see 6.3.12.1.9).

tc "Figure 6.3.12.1.6-1.  Computation of AUTHR for Authentication of Mobile Station Originations"\c §Figure 6.3.12.1.6-1.  Computation of AUTHR for Authentication of Mobile Station Originations

##### 6.3.12.1.7 Authentication of Mobile Station Terminations

When the AUTH field of the Access Parameters Message sent on the Paging Channel is set to ‘01’ (standard authentication mode), and the mobile station responds to a page (by sending a Page Response Message on the Access Channel), the following authentication procedures shall be performed:

The mobile station shall initialize the authentication algorithm (CAVE) as illustrated in Figure 6.3.12.1.7-1.  MIN1 shall be as specified in the Page Response Message.

The mobile station shall then execute the CAVE algorithm (see 2.3.12.1.9).  The 18-bit output of CAVE shall be used to fill the AUTHR field of the Page Response Message.  The RANDC (eight most significant bits of RAND) and COUNT fields of the message shall be filled with the current values stored in the mobile station.

The base station computes the value of AUTHR using its internally stored value of SSD_A.  The base station compares its computed value of AUTHR to the value received from the mobile station.

The base station also compares the received value of RANDC to the eight most significant bits of its internally stored value of RAND.

The base station may also compare the received value of COUNT with its internally stored value associated with the received MIN/ESN.

If the comparisons executed at the base station are successful, the base station may initiate the appropriate channel assignment procedures.  After channel assignment, the base station may issue a Set Parameters Message on the Forward Traffic Channel, modifying the value of COUNT in the mobile station.

If any of the comparisons fail, the base station may deny service, initiate the Unique Challenge Response Procedure (see 6.3.12.1.5) or commence the process of updating SSD (see 6.3.12.1.9).

tc "Figure 6.3.12.1.7-1.  Computation of AUTHR for Authentication of Mobile Station Terminations"\c §Figure 6.3.12.1.7-1.  Computation of AUTHR for Authentication of Mobile Station Terminations

##### 6.3.12.1.8 Authentication of Mobile Station Data Bursts

When the AUTH field of the Access Parameters Message sent on the Paging Channel is set to ‘01’ (standard authentication mode), and the mobile station attempts to send a Data Burst Message on the Access Channel, the following authentication procedures shall be performed:

The mobile station shall initialize the authentication algorithm (CAVE) as illustrated in Figure 6.3.12.1.8-1.  The CHARS field shall contain the least significant four bits of each of the last six characters contained in the Data Burst Message.  If fewer than six characters are included in the Data Burst Message, the most significant bits of MIN1 shall be used to replace the missing characters.

The mobile shall then execute the CAVE algorithm (see 2.3.12.1.9).  The 18-bit output of CAVE shall be used to fill the AUTHR field of the Data Burst Message.  The RANDC (eight most significant bits of RAND) and COUNT fields of the message shall be filled with the current values stored in the mobile station.

The base station computes the value of AUTHR using its internally stored value of SSD_A.  The base station compares its computed value of AUTHR to the value received from the mobile station.

The base station also compares the received value of RANDC to the most significant eight bits of its internally stored value of RAND.

The base station may also compare the received value of COUNT with its internally stored value associated with the received MIN/ESN.

If the comparisons are successful, the base station processes the message.

If any of the comparisons fail, the base station may ignore the message, initiate the Unique Challenge-Response Procedure (see 6.3.12.1.5) or commence the process of updating SSD (see 6.3.12.1.9).

tc "Figure 6.3.12.1.8-1.  Computation of AUTHR for Authentication of Mobile Station Data Bursts"\c §Figure 6.3.12.1.8-1.  Computation of AUTHR for Authentication of Mobile Station Data Bursts

##### 6.3.12.1.9 Updating the Shared Secret Data (SSD)

SSD is updated using CAVE (see 2.3.12.1.9), initialized with mobile station specific information, random data and the mobile station’s A-key.  The A-key is 64 bits long.  It is assigned to the mobile station and is stored in the mobile station’s permanent security and identification memory.  The A-key is known only to the mobile station and to its associated Home Location Register/Authentication Center (HLR/AC) (see EIA/TIA/IS-41).  See “Wideband Spread Spectrum Dual-Mode Cellular System Authentication and Privacy Procedures” for details of A-key entry into the mobile station.

The SSD update procedure is performed as follows:

The base station sends an SSD Update Message on either the Paging Channel or the Forward Traffic Channel.  The RANDSSD field of the SSD Update Message contains the same value used for the HLR/AC computation of SSD.  On the Paging Channel, the SSD Update Message contains the MIN and ESN of the mobile station being addressed.  On the Forward Traffic Channel, the SSD Update Message does not contain the MIN and ESN; the MIN and ESN used by both the base station and the mobile station for initializing CAVE shall be the MIN and ESN used to establish the call.

Upon receipt of the SSD Update Message the mobile station shall initialize CAVE as shown in Figure 6.3.12.1.9-1.  The mobile station shall then execute the CAVE algorithm (see 2.3.12.1.9).  The 64 most significant bits of the CAVE algorithm output are designated SSD_A_NEW and the 64 least significant bits of the output are designated SSD_B_NEW.

The mobile station shall then select a 32-bit random number RANDBS and shall send it to the base station in a Base Station Challenge Order on the Access Channel or Reverse Traffic Channel.

Both the mobile station and the base station shall then initialize CAVE as shown in Figure 6.3.12.1.9-2 and shall execute the CAVE algorithm.  For transactions on the Paging Channel and Access Channel, the value of MIN1 to be used shall be the same as the value specified in the SSD Update Message sent by the base station.  For Traffic Channel transactions, the value of MIN1 to be used shall be the MIN1 used to establish the call.  The 18-bit result is designated AUTHBS.  The base station sends its computed value of AUTHBS to the mobile station in a Base Station Challenge Confirmation Order on the Paging Channel or the Forward Traffic Channel.

Upon receipt of the Base Station Challenge Confirmation Order the mobile station shall compare the received value of AUTHBS to its internally computed value.  (If the mobile station receives a Base Station Challenge Confirmation Order without having previously received an SSD Update Message, the mobile station shall respond with an SSD Update Rejection Order.)

If the comparison is successful, the mobile station shall set SSD_A equal to SSD_A_NEW and shall set SSD_B equal to SSD_B_NEW.  The mobile station shall then send an SSD Update Confirmation Order to the base station, indicating successful completion of the SSD update.

If the comparison is not successful, the mobile station shall discard SSD_A_NEW and SSD_B_NEW.  The mobile station shall then send an SSD Update Rejection Order to the base station, indicating unsuccessful completion of the SSD update.

Upon receipt of the SSD Update Confirmation Order, the base station sets SSD_A and SSD_B to the values received from the HLR/AC (see EIA/TIA/IS-41).

tc "Figure 6.3.12.1.9-1.  Computation of Shared Secret Data (SSD)"\c §Figure 6.3.12.1.9-1.  Computation of Shared Secret Data (SSD)

tc "Figure 6.3.12.1.9-2.  Computation of AUTHBS"\c §Figure 6.3.12.1.9-2.  Computation of AUTHBS

#### 6.3.12.2 Signaling Message Encryption

In an effort to enhance the authentication process and to protect sensitive subscriber information (such as PINs), a method is provided to encrypt certain fields of selected Traffic Channel signaling messages.

The standard message encryption algorithm is identical to the CMEA algorithm specified in Appendix A of EIA/TIA/IS-54-B.

The availability of encryption algorithm information is governed under the U. S. International Traffic and Arms Regulation (ITAR) and the Export Administration Regulations.  TIA acts as the focal point and facilitator for making such information available.  See EIA/TIA/IS-54-B for further details.

Messages shall not be encrypted if authentication is not performed (AUTH field equal to ‘00’ in the Access Parameters Message).

Signaling message encryption is controlled for each call individually.  The initial encryption mode for the call is established by the value of the ENCRYPT_MODE field in the Channel Assignment Message.  If ENCRYPT_MODE is set to ‘0000’, message encryption is off.  To turn encryption on after channel assignment, the base station sends one of the following messages to the mobile station:

•	Handoff Direction Message with the ENCRYPT_MODE field set to ‘0001’

•	Analog Handoff Direction Message with the MEM field set to ‘1’

•	Message Encryption Mode Order with the ENCRYPT_MODE field set to ‘0001’

Regardless of the time at which encryption is turned on, the data used to initialize the algorithm shall be computed using CAVE parameter values in effect at the end of the computation of AUTHR for the Origination Message or Page Response Message (see 6.3.12.1.6 and 6.3.12.1.7, respectively).

Encryption shall apply only to the part of the message body following the ENCRYPTED field, including any RESERVED field at the end of the message.  The message CRC shall never be encrypted.

A message containing only one octet following the ENCRYPTED field cannot be encrypted.

When encryption is off, all fields of all signaling messages sent by the mobile station and base station are unencrypted.

When encryption is on (ENCRYPT_MODE equal to binary ‘0001’), the encryptable fields of the following Forward Traffic Channel messages are encrypted.  All other Forward Traffic Channel messages are unencrypted.

•	Alert With Information Message

•	Flash With Information Message

•	Data Burst Message

•	Send Burst DTMF Message

•	Continuous DTMF Tone Order

•	Retrieve Parameters Message

•	Set Parameters Message

When encryption is on (ENCRYPT_MODE equal to binary ‘0001’) the encryptable fields of the following Reverse Traffic Channel messages shall always be encrypted.  All other Reverse Traffic Channel messages shall be unencrypted.

•	Origination Continuation Message

•	Flash With Information Message

•	Data Burst Message

•	Send Burst DTMF Message

•	Continuous DTMF Tone Order

•	Parameters Response Message

To turn signaling message encryption off, the base station sends one of the following Forward Traffic Channel messages to the mobile station

•	Handoff Direction Message with the ENCRYPT_MODE field set to ‘0000’.

•	Analog Handoff Direction Message with the MEM field set to ‘0’.

•	Message Encryption Mode Order with the ENCRYPT_MODE field set to ‘0000’.

#### 6.3.12.3 Voice Privacy

Voice privacy is provided in the CDMA system by means of the private long code mask used for PN spreading (see 6.1.3.1.8).

The generation and application of the private long code mask is specified in “Wideband Spread Spectrum Dual-Mode Cellular System Authentication and Privacy Procedures.”

Voice privacy control is provided on the Traffic Channels only.  All calls are initiated using the public long code mask for PN spreading (see 6.1.3.1.8).

The transition to private long code mask shall not be performed if authentication is not performed (AUTH field set to ‘00’ in the Access Parameters Message or mobile station unable to perform authentication).

To initiate a transition to the private or public long code mask, either the base station or the mobile station sends a Long Code Transition Request Order on the Traffic Channel.  The mobile station actions in response to receipt of this order are specified in 6.6.4, and the base station actions in response to receipt of this order are specified in 7.6.4.

The base station can also cause a transition to the public long code mask by sending the Handoff Direction Message with the PRIVATE_LCN bit set appropriately.

### 6.3.13 Lock Orders

The mobile station shall have memory to store the lock reason codes (LCKRSNs-p and LCKRSN_Ps-p) received in the Lock Order and Lock Until Power-Cycled Order.;  The data retention time under power-off conditions shall be at least 48 hours.

### 6.3.14 Mobile Station Revision Identification

The mobile station shall provide memory to store the following parameters sent in the Status Message (Terminal Information information record):

•	CAI revision number (MOB_CAI_REVp)

•	Manufacturer’s model number (MOB_MODELp)

•	Firmware revision number (MOB_FIRM_REVp)

## 6.4 Supervision xe "mobile station, supervision"§

This section details the supervision mechanisms in CDMA.  The time and numerical constant values (e.g. T30m and N2m) are given in Appendix D.

### 6.4.1 Pilot Channel xe "mobile station, pilot strength measurements"§

The mobile station shall monitor the Pilot Channel at all times.  The mobile station shall measure the strength of the Pilot Channel as specified in 6.6.6.2.2.

### 6.4.2 Sync Channel

(reworded old sections 2.4.3 (below) and 2.4.1 (above))

xe "The mobile station shall check the CRC of all received Sync Channel messages (see 7.7.1.2.2).  The mobile station shall ignore any message that has a CRC that does not check."§The mobile station shall check the CRC of all received Sync Channel messages (see 7.7.1.2.2).  The mobile station shall ignore a

### 6.4.3 Paging Channel

(reworded old sections 2.4.3 and 2.4.1 (above); words from 2.6.3 (below))

(reworded old sections 2.4.3 and 2.4.1 (above); words from 2.6.3 (below))

The mobile station shall check the CRC of all received Paging Channel messages (see 7.7.2.2.2).  The mobile station shall ignore the body of a message that has a CRC that does not check.

If the mobile station is operating in the non-slotted mode in the Mobile Station Idle State, it shall monitor the Paging Channel at all times.  The mobile station shall reset a timer for T30m seconds whenever a valid message is received on the Paging Channel, whether addressed to the mobile station or not.  If the timer expires, the mobile station shall declare a loss of the Paging Channel.

If the mobile station is operating in the slotted mode in the Mobile Station Idle State, it shall set a timer for T30m seconds at the start of the first slot in which the Paging Channel is monitored, and shall reset the timer for T30m seconds whenever a valid message is received, whether addressed to the mobile station or not.  If the timer expires while the mobile station is monitoring the Paging Channel, the mobile station shall declare a loss of the Paging Channel.  The mobile station shall disable the timer when not monitoring the Paging Channel.

When in the System Access State, the mobile station shall monitor the Paging Channel at all times.  The mobile station shall reset a timer for T40m seconds whenever a valid message is received on the Paging Channel, whether addressed to the mobile station or not.  If the timer expires, the mobile station shall declare a loss of the Paging Channel.

### 6.4.4 Forward Traffic Channel

The mobile station shall check the CRC of all received Forward Traffic messages (see 7.7.3.2.2).  The mobile station shall ignore any message that has a CRC that does not check.

When in the Mobile Station Control on the Traffic Channel State, the mobile station shall monitor the Forward Traffic Channel at all times.  If the mobile station receives xe "N2m"§N2m consecutive bad frames on the xe "Forward Traffic Channel (see 6.2.2.2), it shall disable its transmitter.  Thereafter, if .i.the mobile station receives N3m"§Forward Traffic Channel (see 6.2.2.2), it shall disable its transmitter.  Thereafter, if the mobile station receives N3m consecutive good framesfrafr, the mobile station should re-enable its transmitter.

The mobile station shall establish a Forward Traffic Channel fade timer.  The timer shall be enabled when the mobile station first enables its transmitter when in the Traffic Channel Initialization Substate of the Mobile Station Control on the Traffic Channel State.  The fade timer shall be reset for T5m seconds whenever N3m; consecutive xe "good frames are received on the Forward Traffic Channel.  If the timer expires, the mobile station shall disable its transmitter and declare a loss of the Forward Traffic Channel.  The mobile station also resets this timer when it re-enables its transmitter when performing a CDMA to CDMA hard handoff (see 6.6.6.2.8)."§good frames are received on the Forward Traffic Channel.  If the timer expires, the mobile station shall disable its transmitter

### 6.4.5 Accumulated Statistics

(nothing)

#### 6.4.5.1 Accumulated Access Channel Statistics

The mobile station shall maintain the counters shown in Table 6.4.5.1-1;.  Each counter shall be 16 bits long.  The mobile station shall initialize each counter described herein to zero upon power-on; the mobile station shall not re-initialize any counter described herein at any other time.

The mobile station shall increment the ACC_1 counter for each Access Channel request message it generates.  The mobile station shall increment the ACC_2 counter for each Access Channel response message;s it generates.  The mobile station shall increment the ACC_i counter during the i-1 transmission of an access probe in the access attempt, for i equal 3 to 7.  The mobile station shall increment ACC_10 if the xe "access attempt"§access attempt is unsuccessful due to the transmission of xe "MAX_REQ_SEQ"§MAX_REQ_SEQ or xe "MAX_RSP_SEQ"§MAX_RSP_SEQ probe sequences.

tc "Table 6.4.5.1-1.  Accumulated Access Channel Statistics"\c 	§Table 6.4.5.1-1.  Accumulated Access Channel Statistics

| Counter Identifier | Length (bits) | Description |
| --- | --- | --- |
| xe "ACC_1"§ACC_1 | 16 | Number of xe "Access Channel request message"§Access Channel request messages generated by xe "layer 3"§layer 3 |
| xe "ACC_2"§ACC_2 | 16 | Number of xe "Access Channel response message"§Access Channel response messages generated by xe "layer 3"§layer 3 |
| xe "ACC_3"§ACC_3 | 16 | Number of times that an xe "access probe"§access probe was transmitted at least twice |
| xe "ACC_4"§ACC_4 | 16 | Number of times that an access probe was transmitted at least three times |
| xe "ACC_5"§ACC_5 | 16 | Number of times that an access probe was transmitted at least four times |
| xe "ACC_6"§ACC_6 | 16 | Number of times that an access probe was transmitted at least five times |
| xe "ACC_7"§ACC_7 | 16 | Number of times that an access probe was transmitted at least six times |
| xe "ACC_8"§ACC_8 | - | Reserved |
| xe "ACC_9"§ACC_9 | - | Reserved |
| xe "ACC_10"§ACC_10 | 16 | Number of unsuccessful xe "access attempt"§access attempts |

#### 6.4.5.2 Accumulated Reverse Traffic Channel Statistics

The mobile station shall maintain the counters shown in Table 6.4.5.2-1 when supporting Multiplex Option 1;.  Each counter shall be 32 bits long.  The mobile station shall initialize each counter described herein to zero upon power-on; the mobile station shall not re-initialize any counter described herein at any other time.

Each time a Multiplex Option 1 Reverse Traffic Channel frame is transmitted, the mobile station shall increment the counter corresponding to the type of frame.

tc "Table 6.4.5.2-1.  Accumulated Reverse Traffic Channel Statistics"\c 	§Table 6.4.5.2-1.  Accumulated Reverse Traffic Channel Statistics

| Counter Identifier | Length (bits) | Type of Frame |
| --- | --- | --- |
| xe "MUX1_REV_1"§MUX1_REV_1 | 32 | 9600 bps frame, xe "primary traffic "§primary traffic only |
| xe "MUX1_REV_2"§MUX1_REV_2 | 32 | 9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/2 primary and xe "signaling traffic"§signaling traffic |
| xe "MUX1_REV_3"§MUX1_REV_3 | 32 | 9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/4 primary and xe "signaling traffic"§signaling traffic |
| xe "MUX1_REV_4"§MUX1_REV_4 | 32 | 9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/8 primary and xe "signaling traffic"§signaling traffic |
| xe "MUX1_REV_5"§MUX1_REV_5 | 32 | 9600 bps frame, xe "blank-and-burst"§blank-and-burst with xe "signaling traffic"§signaling traffic only |
| xe "MUX1_REV_6"§MUX1_REV_6 | 32 | 4800 bps frame, xe "primary traffic "§primary traffic only |
| xe "MUX1_REV_7"§MUX1_REV_7 | 32 | 2400 bps frame, xe "primary traffic "§primary traffic only |
| xe "MUX1_REV_8"§MUX1_REV_8 | 32 | 1200 bps frame, xe "primary traffic "§primary traffic or null Traffic Channel data only |
| MUX1_REV_9 | - | Reserved |
| MUX1_REV_10 | - | Reserved |
| xe "MUX1_REV_11"§MUX1_REV_11 | 32 | 9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/2 primary and xe "secondary traffic"§secondary traffic |
| xe "MUX1_REV_12"§MUX1_REV_12 | 32 | 9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/4 primary and xe "secondary traffic"§secondary traffic |
| xe "MUX1_REV_13"§MUX1_REV_13 | 32 | 9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/8 primary and xe "secondary traffic"§secondary traffic |
| xe "MUX1_REV_14"§MUX1_REV_14 | 32 | 9600 bps frame, xe "blank-and-burst"§blank-and-burst with xe "secondary traffic"§secondary traffic only |

#### 6.4.5.3 Accumulated Paging Channel Statistics

The mobile station shall maintain the counters shown in Table 6.4.5.3-1;.  The counters shall have the length specified in the Table 6.4.5.3-1.  The mobile station shall initialize each counter described herein to zero upon power-on; the mobile station shall not re-initialize any counter described herein at any other time.

The mobile station shall increment the counter PAG_1 for each Paging Channel message CRC that it tests.  The mobile station shall increment the counter xe "PAG_2 "§PAG_2 for each valid Paging Channel message.  The mobile station shall increment the counter xe "PAG_3 "§PAG_3 for each record or message that it receives addressed to the mobile station’s ESN or to an active MIN at the mobile station.  The PAG_3 counter shall not be incremented for messages detected as duplicates or for acknowledgements.  The mobile station shall increment the counter xe "PAG_4 each time that it has determined "§PAG_4 each time that it has determined that synchronization was lost on the xe "Paging Channel "§Paging Channel during the time that the mobile station is active.  xe "PAG_4 "§PAG_4 shall not count the initial synchronization performed by a xe "slotted mobile station"§slotted mobile station for each of its assigned timeslots.  xe "The mobile station shall increment the counter PAG_5 each time that it declares a loss of the Paging Channel (see 6.4.3).  The mobile station shall increment the counter PAG_6 for each idle handoff it performs."§The mobile station shall increment the counter PAG_5 each time that it declares a loss of the Paging Channel (see 6.4.3).  The m

tc "Table 6.4.5.3-1.  Accumulated Paging Channel Statistics"\c 	§Table 6.4.5.3-1.  Accumulated Paging Channel Statistics

| Counter Identifier | Length (bits) | Description |
| --- | --- | --- |
| xe "PAG_1"§PAG_1 | 32 | Number of xe "Paging Channel "§Paging Channel messages the mobile station attempted to receive |
| xe "PAG_2"§PAG_2 | 32 | Number of xe "Paging Channel "§Paging Channel messages the mobile station received with a good xe "CRC"§CRC |
| xe "PAG_3"§PAG_3 | 16 | Number of xe "Paging Channel message"§Paging Channel messages or records the mobile station received that were addressed to it |
| xe "PAG_4"§PAG_4 | 16 | Number of times that the mobile station had to resynchronize to the xe "Paging Channel "§Paging Channel |
| xe "PAG_5"§PAG_5 | 16 | Number of times that the mobile station declared a loss of the xe "Paging Channel "§Paging Channel |
| xe "PAG_6"§PAG_6 | 16 | Number of xe "mobile station idle handoff"§mobile station idle handoffs |

#### 6.4.5.4 Accumulated Forward Traffic Channel Statistics

The mobile station shall maintain the counters shown in Table 6.4.5.4-1 when supporting xe "Multiplex Option 1"§Multiplex Option 1.  Each counter shall be 32 bits long.  The mobile station shall initialize each counter described herein to zero upon power-on; the mobile station shall not re-initialize any counter described herein at any other time.

Each time a mobile station categorizes a received xe "Multiplex Option 1 Forward Traffic Channel frame"§Multiplex Option 1 Forward Traffic Channel frame (see 6.2.2.2), the mobile station shall increment the counter corresponding to the type of frame.  The accumulation shall start when the mobile station enables its transmitter while in the xe "Traffic Channel Initialization Substate"§Traffic Channel Initialization Substate of the xe "Mobile Station Control on the Traffic Channel State "§Mobile Station Control on the Traffic Channel State (see 6.6.4.2).  The accumulation shall stop when the mobile station exits the Mobile Station Control on the Traffic Channel State.

tc "Table 6.4.5.4-1.  Accumulated Forward Traffic Channel Statistics"\c 	§Table 6.4.5.4-1.  Accumulated Forward Traffic Channel Statistics

| Counter Identifier | Length (bits) | Type of Frame |
| --- | --- | --- |
| xe "MUX1_FOR_1"§MUX1_FOR_1 | 32 | 9600 bps frame, xe "primary traffic "§primary traffic only |
| xe "MUX1_FOR_2"§MUX1_FOR_2 | 32 | 9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/2 primary and xe "signaling traffic"§signaling traffic |
| xe "MUX1_FOR_3"§MUX1_FOR_3 | 32 | 9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/4 primary and xe "signaling traffic"§signaling traffic |
| xe "MUX1_FOR_4"§MUX1_FOR_4 | 32 | 9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/8 primary and xe "signaling traffic"§signaling traffic |
| xe "MUX1_FOR_5"§MUX1_FOR_5 | 32 | 9600 bps frame, xe "blank-and-burst"§blank-and-burst with xe "signaling traffic"§signaling traffic only |
| xe "MUX1_FOR_6"§MUX1_FOR_6 | 32 | 4800 bps frame, xe "primary traffic "§primary traffic only |
| xe "MUX1_FOR_7"§MUX1_FOR_7 | 32 | 2400 bps frame, xe "primary traffic "§primary traffic only |
| xe "MUX1_FOR_8"§MUX1_FOR_8 | 32 | 1200 bps frame, xe "primary traffic "§primary traffic or null Traffic Channel data only |
| xe "MUX1_FOR_9"§MUX1_FOR_9 | 32 | 9600 bps frame, xe "primary traffic "§primary traffic only, probable bit errors |
| xe "MUX1_FOR_10"§MUX1_FOR_10 | 32 | Frame quality insufficient to decide upon rate |
| xe "MUX1_FOR_11"§MUX1_FOR_11 | 32 | 9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/2 primary and xe "secondary traffic"§secondary traffic |
| xe "MUX1_FOR_12"§MUX1_FOR_12 | 32 | 9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/4 primary and xe "secondary traffic"§secondary traffic |
| xe "MUX1_FOR_13"§MUX1_FOR_13 | 32 | 9600 bps frame, xe "dim-and-burst"§dim-and-burst with Rate 1/8 primary and xe "secondary traffic"§secondary traffic |
| xe "MUX1_FOR_14"§MUX1_FOR_14 | 32 | 9600 bps frame, xe "blank-and-burst"§blank-and-burst with xe "secondary traffic"§secondary traffic only |

#### 6.4.5.5 Accumulated Layer Two Statistics

The mobile station shall maintain the counters shown in Table 6.4.5.5-1;.  Each counter shall be 16 bits long.  The mobile station shall initialize each counter described herein to zero upon power-on; the mobile station shall not re-initialize any counter described herein at any other time.

When the mobile station transmits a Reverse Traffic Channel message requiring an acknowledgement for the ith time, for i equal 1 to 3, it shall increment the counter LAYER2_RTCi.

The mobile station shall increment the counter xe "LAYER2_RTC5"§LAYER2_RTC5 each time it aborts using the Traffic Channel because the timeout expired after the xe "N1m"§N1m transmission of a xe "message requiring an acknowledgement"§message requiring an acknowledgement.

The mobile station shall increment the counter xe "LAYER2_RTC6"§LAYER2_RTC6 for each transmission of a message not requiring an acknowledgement on the xe "Reverse Traffic Channel"§Reverse Traffic Channel.  This count shall include all transmissions, including those that were repeated multiple times or those carrying an identical layer 3 content.

tc "Table 6.4.5.5-1.  Accumulated Layer 2 Statistics"\c 	§Table 6.4.5.5-1.  Accumulated Layer 2 Statistics

| Counter Identifier | Length (bits) | Description |
| --- | --- | --- |
| xe "LAYER2_RTC1"§LAYER2_RTC1 | 16 | Number of messages requiring acknowledgement that were transmitted at least once on the xe "Reverse Traffic Channel"§Reverse Traffic Channel |
| xe "LAYER2_RTC2"§LAYER2_RTC2 | 16 | Number of messages requiring acknowledgement that were transmitted at least twice on the xe "Reverse Traffic Channel"§Reverse Traffic Channel |
| xe "LAYER2_RTC3"§LAYER2_RTC3 | 16 | Number of messages requiring acknowledgement that were transmitted at least three times on the xe "Reverse Traffic Channel"§Reverse Traffic Channel |
| xe "LAYER2_RTC4"§LAYER2_RTC4 | 16 | Reserved |
| xe "LAYER2_RTC5"§LAYER2_RTC5 | 16 | Number of times that the mobile station aborted a call as a result of the timeout expiring after the xe "N1m"§N1m transmission of a message requiring acknowledgement |
| xe "LAYER2_RTC6"§LAYER2_RTC6 | 16 | Number of times a xe "message not requiring an acknowledgement"§message not requiring an acknowledgement was sent on the xe "Reverse Traffic Channel"§Reverse Traffic Channel |

#### 6.4.5.6 Other Monitored Quantities and Statistics

The mobile station shall store the value described in Table 6.4.5.6-1.

tc "Table 6.4.5.6-1.  Other Monitored Quantities and Statistics"\c 	§Table 6.4.5.6-1.  Other Monitored Quantities and Statistics

| Quantity Identifier | Length (bits) | Description |
| --- | --- | --- |
| xe "OTHER_SYS_TIME"§OTHER_SYS_TIME | 36 | The xe "SYS_TIME"§SYS_TIME field from the first xe "Sync Channel"§Sync Channel Message received after power-on |

## 6.5 Malfunction Detection xe "mobile station, malfunction detection"§

The mobile station shall include methods to detect malfunctions that would cause degradation to system capacity.  Once a failure has been detected, transmission from the mobile station shall remain disabled until the failed test is successfully performed on two consecutive test attempts.

### 6.5.1 Malfunction Timer xe "mobile station, malfunction timer"§

The mobile station shall have a malfunction timer which meets the requirements of 2.5.1.

### 6.5.2 Transmit Waveform Quality xe "mobile station, false transmission"§

To ensure that the majority of mobile stations transmit spread spectrum signals which do not adversely affect system capacity, the transmit waveform of each mobile station shall exceed a specified waveform quality with a specified probability, called the availability.  This transmit waveform quality, r, is defined as

where x(t; ø) is the transmit waveform with phase ø, y(t; q) is the waveform defined in 6.1 with phase q, and T is the integration interval.  During the entire service life of the mobile station, the transmit waveform quality shall exceed the waveform quality value given in Table 6.5.2-1 with the given availability value or the transmitter shall be disabled.  Refer to “Recommended Minimum Performance Standards for 800 MHz Wideband Spread Spectrum Dual-Mode Mobile Stations.”

tc "Table 6.5.2-1.  Required Availability of Transmit Waveform Quality"\c 	§Table 6.5.2-1.  Required Availability of Transmit Waveform Quality

| Transmit Waveform Quality, r | Transmit Waveform Degradation | Availability |
| --- | --- | --- |
| _ 0.89 | _ 0.5 dB | _ 98% |
| _ 0.45 | _ 3.5 dB | _ 99.9% |
| _ 0.1 | _ 10.0 dB | _ 99.999% |

## 6.6 Call Processing

This section describes mobile station call processing.  It contains frequent references to the messages that flow between the mobile station and base station.  While reading this section, it may be helpful to refer to the message formats (see 6.7 and 7.7), and to the message flow examples (see Appendix B).

The values for the time and numerical constants used in this section (e.g., T20m, N4m) are specified in Appendix D.

As illustrated in Figure 6.6-1, mobile station call processing consists of the following states:

•	xe "Mobile Station Initialization State - In this state, the mobile station selects and acquires a system."§Mobile Station Initialization State - In this state, the mobile station selects and acquires a system.

•	xe "Mobile Station Idle State - In this state, the mobile station monitors messages on the Paging Channel."§Mobile Station Idle State - In this state, the mobile station monitors messages on the Paging Channel.

•	xe "System Access State - In this state, the mobile station sends messages to the base station on the Access Channel."§System Access State - In this state, the mobile station sends messages to the base station on the Access Channel.

•	xe "Mobile Station Control on the Traffic Channel State - In this state, the mobile station communicates with the base station using the Forward and Reverse Traffic Channels."§Mobile Station Control on the Traffic Channel State - In this state, the mobile station communicates with the base station using

After power is applied to the mobile station, it shall enter the xe "System Determination Substate of the Mobile Station Initialization State.i."§System Determination Substate of the Mobile Station Initialization State with a power-up indication.

tc "Figure 6.6-1.  Mobile Station Call Processing States"\c §Figure 6.6-1.  Mobile Station Call Processing States

### 6.6.1 Mobile Station Initialization State xe "mobile station, Mobile Station Initialization State"§

xe "In this state, the mobile station first selects a system to use.  If the selected system is a CDMA system, the mobile station proceeds to acquire and then synchronize to the CDMA system.  If the selected system is an analog system, the mobile station begins analog mode operation.  "§In this state, the mobile station first selects a system to use.  If the selected system is a CDMA system, the mobile station pr

As illustrated in Figure 6.6.1-1, the xe "Mobile Station Initialization State"§Mobile Station Initialization State  consists of the following substates:

•	xe "System Determination Substate - In this substate, the mobile station selects which system to use."§System Determination Substate - In this substate, the mobile station selects which system to use.

•	xe "Pilot Channel"§Pilot Channel Acquisition Substate - In this substate, the mobile station acquires the Pilot Channel of a CDMA system.

•	xe "Sync Channel"§Sync Channel Acquisition Substate - In this substate, the mobile station obtains system configuration and timing information for a CDMA system.

•	xe "Timing Change Substate - In this substate, the mobile station synchronizes its timing to that of a CDMA system."§Timing Change Substate - In this substate, the mobile station synchronizes its timing to that of a CDMA system.

While in the Mobile Station Initialization State, the mobile station shall update all active registration timers as specified in 6.6.5.6.1.2.

tc "Figure 6.6.1-1.  Mobile Station Initialization State"\c §Figure 6.6.1-1.  Mobile Station Initialization State

#### 6.6.1.1 System Determination Substate xe "mobile station, Mobile Station Initialization State, System Determination Substate"§

In this substate, the mobile station selects the system to use.  The precise process for system selection is left to the mobile station manufacturer.  It is typically influenced by a set of expressed user preferences, such as the following:

•	System A (or B) only

•	System A (or B) preferred

•	CDMA (or analog) system only

•	CDMA (or analog) system preferred

•	CDMA Primary (or Secondary) Channel preferred

Upon entering the System Determination Substate, the mobile station shall initialize registration parameters as specified in 6.6.5.6.1.1.  If the mobile station enters the System Determination Substate with a power-up indication, the mobile station shall set the First-Idle ID status to enabled (see 2.6.3.11), and the RANDs variable to 0 (see 2.3.12.1.2).

In the xe "System Determination Substate"§System Determination Substate, the mobile station shall perform the following:

•	The mobile station shall determine which system to use.

•	If the mobile station is to use System A, it shall set SERVSYSs to SYS_A.  If the mobile station is to use System B, it shall set SERVSYS;s to SYS_B.

•	If the mobile station is to use an analog system, it shall enter the Initialization Task (see 2.6.1).

•	If the mobile station is to use a CDMA xe "system"§system, it shall set CDMACHs either to the Primary or Secondary xe "CDMA Channel number for the selected serving system (SERVSYSs).  The mobile station shall enter the .i.Pilot Channel Acquisition Substate."§CDMA Channel number for the selected serving system (SERVSYSs).  The mobile station shall enter the Pilot Channel Acquisition Su

If the mobile station fails to acquire a CDMA system on the first CDMA Channel it tries, the mobile station should attempt to acquire on the alternate CDMA Channel (Primary or Secondary) before performing the system selection process again.

#### 6.6.1.2 Pilot Channel Acquisition Substate

In this substate, the mobile station acquires the xe "Pilot Channel"§Pilot Channel of the selected CDMA system.

Upon entering the Pilot Channel Acquisition Substate, the mobile station shall tune to the CDMA Channel number equal to CDMACHs and set its code channel for the Pilot Channel.

The mobile station shall search for the Pilot Channel.  If the mobile station acquires the Pilot Channel within T20m seconds, the mobile station shall enter the xe "Sync Channel Acquisition Substate"§Sync Channel Acquisition Substate.

If the mobile station does not acquire the Pilot Channel within T20m seconds, the mobile station shall enter the System Determination Substate.

#### 6.6.1.3 Sync Channel Acquisition Substate xe "mobile station, Mobile Station Initialization State, Sync Channel Acquisition Substate"§

In this substate, the mobile station receives and processes the Sync Channel Message to obtain system configuration and timing information.

Upon entering the Sync Channel Acquisition Substate , the mobile station shall set its code channel for the Sync Channel.

xe "Sync Channel Message"§xe "Pilot PN Sequence Offset"§If the mobile station does not receive a valid xe "Sync Channel Message"§Sync Channel Message (see 6.4.2) within xe "T21m"§T21m seconds, the mobile station shall enter the System Determination Substate.

If the mobile station receives a valid Sync Channel Message within T21m seconds but the CAI revision level supported by mobile station (xe "MOB_CAI_REV"§MOB_CAI_REVp) is less than the minimum CAI revision level supported by the base station ;,(MIN_CAI_REVr), the mobile station shall enter the System Determination Substate.

If the mobile station receives a valid Sync Channel Message within T21m seconds and the CAI revision level supported by the mobile station (xe "MOB_CAI_REV"§MOB_CAI_REVp) is greater than or equal to the minimum CAI revision level;, supported by the base station (MIN_CAI_REVr), the mobile station shall store the following information from the message:

•	Common Air Interface revision level (xe "CAI_REV"§CAI_REVs = CAI_REVr)

•	Minimum Common Air Interface revision level (xe "MIN_CAI_REV"§MIN_CAI_REVs = MIN_CAI_REVr)

•	System identification (xe "SID"§SIDs = SIDr)

•	Network identification (xe "NID"§NIDs = NIDr)

•	Pilot PN sequence offset (xe "PILOT_PN"§PILOT_PNs = PILOT_PNr)

•	xe "Long code"§Long code state (xe "LC_STATE"§LC_STATEs = LC_STATEr)

•	xe "Paging Channel"§Paging Channel data rate (xe "PRAT"§PRATs = PRATr)

•	xe "System Time"§System Time (xe "SYS_TIME"§SYS_TIMEs = SYS_TIMEr)

The mobile station may store the following information from the message:

•	Number of leap seconds that have occurred since the start of xe "System Time (.i.LP_SEC"§System Time (LP_SECs = LP_SECr)

•	Offset of local time from xe "System Time (.i.LTM_OFFssss = LTM_OFFr)"§System Time (LTM_OFFs = LTM_OFFr)

•	Daylight savings time indicator (xe "DAYLTssss = DAYLTr)"§DAYLTs = DAYLTr)

The mobile station shall enter the Timing Change Substate.

#### 6.6.1.4 Timing Change Substatexe "mobile station, Mobile Station Initialization State, Timing Change Substate"§

In this substate, the mobile station synchronizes its long code timing and system timing to those of the CDMA system, using the LC_STATEs and SYS_TIMEs values obtained from the received Sync Channel Message.  SYS_TIMEs is equal to the System Time corresponding to 320 ms past the end of the last 80 ms superframe (see Figure 7.7.1.1-1) of the received Sync Channel Message minus the pilot PN offset.  LC_STATEs is equal to the system long code state corresponding to SYS_TIMEs (see Figure 6.6.1.4-1).

In the Timing Change Substate, the mobile station shall synchronize its long code timing to the CDMA system long code timing derived from LC_STATEs, and synchronize its system timing to the CDMA system timing derived from SYS_TIMEs.

The mobile station shall set PAGECHs to the Primary CDMA Paging Channel number and perform registration initialization as specified in 6.6.5.6.1.3.  The mobile station shall enter the Mobile Station Idle State.

tc "Figure 6.6.1.4-1.  Mobile Station Internal Timing"\c §Figure 6.6.1.4-1.  Mobile Station Internal Timing

### 6.6.2 Mobile Station Idle State xe "mobile station, Mobile Station Idle State"§

In this state, the mobile station monitors the Paging Channel.  The mobile station can receive messages, receive an incoming call (mobile station terminated call), initiate a call (xe "mobile station originated call"§mobile station originated call), initiate a xe "registration"§registration request, or initiate a message transmission.

Upon entering the Mobile Station Idle State, the mobile station shall set its code channel to PAGECHs, set the Paging Channel data rate as determined by PRATs and shall perform Paging Channel supervision as specified in 6.4.3.

At any time, the mobile station may exit the xe "Mobile Station Idle State "§Mobile Station Idle State and enter the xe "System Determination Substate"§System Determination Substate of the xe "Mobile Station Initialization State."§Mobile Station Initialization State.

While in the xe "Mobile Station Idle State"§Mobile Station Idle State, the mobile station shall perform the following procedures:

•	The mobile station shall perform Paging Channel monitoring procedures as specified in 6.6.2.1.1.

•	The mobile station shall perform message acknowledgement procedures as specified in 6.6.2.1.2.

•	The mobile station shall perform registration procedures as specified in 6.6.2.1.3.

•	The mobile station shall perform idle handoff procedures as specified in 6.6.2.1.4.

•	The mobile station shall perform the xe "Response to Overhead Information Operation"§Response to Overhead Information Operation as specified in 6.6.2.2 whenever the mobile station receives a system overhead message (System Parameter Message, CDMA Channel List Message, Neighbor List Message or Access Parameters Message).

•	The mobile station shall perform the xe "Mobile Station Page Match Operation"§Mobile Station Page Match Operation as specified in 6.6.2.3 whenever it receives a Page Message or xe "Slotted Page Message"§Slotted Page Message.

•	The mobile station shall perform the xe "Mobile Station Order and Message Processing Operation"§Mobile Station Order and Message Processing Operation as specified in 6.6.2.4 whenever the mobile station receives a message other than a system overhead message or a page message.

•	The mobile station shall perform the Mobile Station Origination Operation as specified in 6.6.2.5 if directed by the user to initiate a call.

•	If the mobile station supports message transmission, it shall perform the Mobile Station Message Transmission Operation as specified in 6.6.2.6 if directed by the user to transmit a message.

•	The mobile station shall perform the Mobile Station Power-Down Operation as specified in 6.6.2.7 if directed by the user to power down.

#### 6.6.2.1 Idle Procedures

##### 6.6.2.1.1 Paging Channel Monitoring Procedures

###### 6.6.2.1.1.1 General Overview

The xe "Paging Channel"§Paging Channel is divided into 200 ms time slots called xe "Paging Channel slot"§Paging Channel slots.  Paging and control messages for a mobile station operating in the non-slotted mode can be received in any of the Paging Channel slots.  Therefore, the non-slotted mode of operation requires the mobile station to monitor all slots.

The xe "Paging Channel"§Paging Channel protocol also provides for scheduling the transmission of messages for a specific mobile station in certain assigned time slots.  Support of this feature is optional and may be enabled independently for each mobile station.  A mobile station that monitors the Paging Channel only during its assigned timeslot is referred to as operating in the slotted mode.  During the time slots in which the Paging Channel is not being monitored, the mobile station can stop or reduce its processing for power conservation.  A mobile station may not operate in the slotted mode in any state except the Mobile Station Idle State.

A mobile station operating in the slotted mode generally monitors the Paging Channel for one or two slots per slot cycle.  The mobile station can control the length of the slot cycle through the SLOT_CYCLE_INDEX field in the xe "Initial Registration Message"§Initial Registration Message or xe "Registration Update Message"§Registration Update Message.  The length of the slot cycle, T, in seconds, is given by

T = 2xe "SLOT_CYCLE_INDEX"§SLOT_CYCLE_INDEX.

There are 5 ¥ T slots in a slot cycle.

The SLOT_NUM field of the Slotted Page Message is the Paging Channel slot number, modulo the maximum length slot cycle (640 slots).  That is, the value of SLOT_NUM is

SLOT_NUM = Ît/10_ mod 640,

where t is the xe "System Time"§System Time in frames.  For each mobile station, the starting times of its slot cycles are offset from the slot in which SLOT_NUM equals zero by a fixed, randomly selected number of slots as specified in 6.6.2.1.1.3.  A mobile station operating in the slotted mode can use the SLOT_NUM field to determine its slot cycle time alignment with the base station.

Figure 6.6.2.1.1.1-1 shows an example for a slot cycle length of two seconds, in which the computed value of PGSLOT (see 6.6.2.1.1.3) is equal to 6, so that one of the mobile station’s slot cycles begins when SLOT_NUM equals 6.  The mobile station begins monitoring the Paging Channel at the start of the slot in which SLOT_NUM equals 6.  The next slot in which the mobile station must begin monitoring the Paging Channel is 10 slots later, i.e., the slot in which SLOT_NUM is (6 + 5 ¥ T) modulo 640, which is 16.

tc "Figure 6.6.2.1.1.1-1.  Mobile Station Idle Slotted Mode Structure Example (see text)"\c §Figure 6.6.2.1.1.1-1.  Mobile Station Idle Slotted Mode Structure Example (see text)

Slotted Page Messages contain a field called MORE_PAGES which, when set to ‘0’ during a mobile station’s assigned slot, indicates that the remainder of the slot will contain no more messages addressed to that mobile station.  This allows a mobile station operating in the slotted mode to stop monitoring the Paging Channel as soon as possible.

If a Slotted Page Message containing the MORE_PAGES field equal to ‘0’ is not received in the assigned slot, the mobile station continues to monitor the Paging Channel for one additional slot.  This allows the base station to carry over a message begun in the assigned slot into the following slot if necessary.

###### 6.6.2.1.1.2 Non-Slotted Mode Requirements

A mobile station operating in the non-slotted mode shall monitor the Paging Channel at all times.  If the mobile station declares loss of the Paging Channel (see 6.4.3), the mobile station shall enter the System Determination Substate of the Mobile Station Initialization State.

When a mobile station monitors the Paging Channel in any state other than the Mobile Station Idle State, it shall operate in the non-slotted mode.

###### 6.6.2.1.1.3 Slotted Mode Requirements

A mobile station operating in the slotted mode is not required to monitor the Paging Channel at all times.

If the mobile station declares loss of the Paging Channel (see 6.4.3), the mobile station shall enter the System Determination Substate of the Mobile Station Initialization State.

If the mobile station’s xe "slot cycle"§slot cycle is modified, the mobile station performs parameter-change registration (see 6.6.5.1.6).

If the mobile station user configures a mobile station for slotted mode operation, the mobile station shall specify its preferred slot cycle index in the SLOT_CYCLE_INDEX field of the xe "Initial Registration Message"§Initial Registration Message or a Registration Update Message.  Operation with the modified slot cycle shall begin only after an acknowledgement of the registration has been received.  A mobile station equipped for slotted operation may disable slotted operation by setting bit 5 of the station class mark to ‘0’ (see 2.3.3).

If any of the following conditions is true, the mobile station shall operate either in the non-slotted mode or in the slotted mode with either a 1 or 2 second xe "slot cycle"§slot cycle (xe "SLOT_CYCLE_INDEX"§SLOT_CYCLE_INDEX equal to 0 or 1):

•	The system configuration parameters are not current (see 6.6.2.2); or

•	If the current base station’s SID and NID (SIDs and NIDs, as stored from the System Parameters Message) do not match any entry of SID_NID_LISTs.

If the mobile station’s stored system configuration parameters are current (see 6.6.2.2) and SID_NID_LISTs contains an entry corresponding to SIDs and NIDs, then the mobile station should use the value of SLOT_CYCLE_INDEX sent in the last registration message to determine its assigned slot.  If the mobile station does not use this value, it shall operate either in the non-slotted mode or in the slotted mode with either a 1 or 2 second xe "slot cycle"§slot cycle (xe "SLOT_CYCLE_INDEX"§SLOT_CYCLE_INDEX equal to 0 or 1).

A mobile station operating in the slotted mode shall use the hash function specified in 6.6.7.1 to select a number PGSLOT in the range 0 to 639 (spanning the maximum slot cycle length, which is 128 seconds).  The mobile station’s assigned slots shall be those slots in which

(Ît/10 _ - PGSLOT) mod (5 ¥ T) = 0,

where t is the xe "System Time"§System Time in frames and T is the slot cycle length in seconds given by

T = 2SLOT_CYCLE_INDEX.

The mobile station shall begin monitoring the Paging Channel in time to receive the first bit of each assigned slot.

If a mobile station operating in the slotted mode determines that the configuration information is not current, as specified in 6.6.2.2, it shall continue to monitor the Paging Channel until either the configuration information has been updated and is current or the mobile station declares loss of the Paging Channel (see 6.4.3).  When the configuration information is current, the mobile station shall continue to monitor the Paging Channel until one of the following conditions is true:

•	A Slotted Page Message has been received during the assigned slot with the MORE_PAGES bit equal to ‘0’; or

•	The mobile station has monitored the Paging Channel for two time slots, starting with the assigned slot, and has received at least one valid message (see 6.4.3); or

•	The mobile station declares loss of the Paging Channel (see 6.4.3).

##### 6.6.2.1.2 Acknowledgement Procedures

Acknowledgement procedures facilitate the reliable exchange of messages between the base station and the mobile station.  The mobile station uses the fields ACK_SEQ xe "ACK_SEQ "§(acknowledgement sequence number), MSG_SEQxe "MSG_SEQ"§ (message sequence number), ACK_REQ xe "ACK_REQ "§(acknowledgement required), and VALID_ACK xe "VALID_ACK "§(valid acknowledgement) to support this mechanism.  These fields are referred to as layer 2 fields, and the acknowledgement procedures are referred to as layer 2 procedures.  All other message fields and the processing thereof are referred to as pertaining to layer 3.  (See Appendix C for further discussion of layering.)

Acknowledgements of messages received on the Paging Channel shall be sent on the Access Channel (see 6.6.3).

When sending a message that includes an acknowledgement, the mobile station shall set the VALID_ACK field to ‘1’ and shall set the ACK_SEQ field equal to the MSG_SEQ field of the message being acknowledged.  For acknowledgement of a Page Message or Slotted Page Message, the mobile station shall set the ACK_SEQ field equal to the MSG_SEQ field of the record containing the MIN that matched one of the mobile station’s active MINs.

When sending a message that does not include an acknowledgement, the mobile station shall set the VALID_ACK field to ‘0’ and shall set the ACK_SEQ field equal to the ACK_SEQ field of the previous message sent.  If no previous message has been sent, the mobile station shall set the ACK_SEQ field to ‘00’.

Unless otherwise specified in the requirements for processing a specific message, the mobile station shall transmit an acknowledgement in response to any message received that is addressed to the mobile station and that has the ACK_REQ field set to ‘1’.  The mobile station shall transmit a Page Response Message including an acknowledgement in response to each record of a Page Message or Slotted Page Message addressed to one of the mobile station’s active MINs.  If a specific message is required in response to any other message requiring acknowledgement, the acknowledgement shall be included with the response.  If no specific message is required to be transmitted in response to a received message requiring acknowledgement, the mobile station shall include the acknowledgement in a Mobile Station Acknowledgement Order (see 6.7.3).

If no message requiring acknowledgement has been received, the mobile station shall not include an acknowledgement in any transmitted message until a message is received that requires acknowledgement.  After a message including an acknowledgement has been sent, the mobile station shall not include an acknowledgement in any subsequent transmitted message until another message is received that requires acknowledgement.

The mobile station shall detect duplicate received messages by the following rules.

The mobile station shall consider two messages (or order records) addressed to the mobile station’s ESN (all directed messages except Page Messages and Slotted Page Messages) to be duplicates if all of the following are true:

•	The messages (records) were received on the same Paging Channel; and

•	The messages (records) contain the same values in both the MSG_SEQ and ACK_REQ fields; and

•	The messages (records) were received within T4m seconds (see Appendix D) of each other (see Figure 6.6.2.1.2-1); and

The mobile station shall consider two page records (addressed by MIN, as contained in Page Messages and Slotted Page Messages) to be duplicates if all of the following are true:

•	The records were received on the same Paging Channel; and

•	The records contain the same values in the MSG_SEQ field; and

•	The records were received in messages received within T4m seconds of each other (see Figure 6.6.2.1.2-1), or in the same message; and

•	Both records are addressed to the same MIN.

The mobile station shall discard, without further processing, any message or page record that is a duplicate of one previously received.

Paging Channels shall be considered different if they are transmitted by different base stations, if they are transmitted on different code channels or if they are transmitted on different CDMA Channels.

The mobile station shall consider messages to be different if they are not duplicates according to the rules given above.  The mobile station shall process all messages that are considered to be different.

tc "Figure 6.6.2.1.2-1.  Time Interval for Duplicate Message Detection"\c §Figure 6.6.2.1.2-1.  Time Interval for Duplicate Message Detection

##### 6.6.2.1.3 Registration

While in the Mobile Station Idle State, the mobile station shall maintain all active registration timers (see 6.6.5.5), and shall perform the registration procedures specified in 6.6.5.6.2.1.

##### 6.6.2.1.4 Idle Handoff

###### 6.6.2.1.4.1 Pilot Search

An idle handoff occurs when a mobile station has moved from the area of coverage of one base station into the coverage of a second base station during the xe "Mobile Station Idle State"§Mobile Station Idle State.  The mobile station determines that an idle handoff should occur when it detects a sufficiently strong xe "Pilot Channel"§Pilot Channel signal other than that of the current base station’s Pilot Channel signal.

Pilot Channels are identified by their offsets relative to the zero offset pilot PN sequence (see 7.1.3.2.1).  Pilot offsets are grouped into sets describing their status with regard to pilot searching.

The following sets of pilot offsets are defined for a mobile station in the Mobile Station Idle State.  Each pilot offset is a member of only one set.

_	xe "Active Set:  The pilot offset of the Forward CDMA Channel whose Paging Channel is being monitored."§Active Set  The pilot offset of the Forward CDMA Channel whose Paging Channel is being monitored.

_	Neighbor Set:  The offsets of the Pilot Channels that are likely candidates for xe "idle handoff"§idle handoff.  The members of the Neighbor Set are specified in the Neighbor List Message.

_	xe "Remaining Set:  The set of all possible pilot offsets in the current system (integer multiples of PILOT_INCs) on the current CDMA frequency assignment, excluding the pilots in the .i.Neighbor Set"§Remaining Set  The set of all possible pilot offsets in the current system (integer multiples of PILOT_INCs) on the current CDMA and the xe "Active Set"§Active Set.

The mobile station shall be capable of storing the information from the Neighbor List Message for at least N8m members of the neighbor set (see Appendix D).

In the xe "Mobile Station Idle State"§Mobile Station Idle State, the mobile station shall continuously search for the strongest xe "Pilot Channel"§Pilot Channel signal on the current xe "CDMA"§CDMA frequency assignment (xe "CDMACH"§CDMACHs).  Search performance criteria are defined in the “Recommended Minimum Performance Standards for 800 MHz Wideband Spread Spectrum Dual-Mode Mobile Stations.”

This search should be governed by the following:

_	xe "Active Set"§Active Set:  The search window size for each pilot in the Active Set should be the number of PN chips specified in Table 6.6.6.2.1-1 corresponding to xe "SRCH_WIN_A"§SRCH_WIN_As.  The mobile station should center the search window for each pilot of the Active Set around the earliest arriving usable multipath component of the pilot.

_	xe "Neighbor Set"§Neighbor Set:  The search window size for each pilot in the Neighbor Set should be the number of PN chips specified in Table 6.6.6.2.1-1 corresponding to SRCH_WIN_N;s.  The mobile station should center the search window for each pilot in the Neighbor Set around the pilot’s PN sequence offset using timing defined by the mobile station’s time reference (see 6.1.5.1).

_	xe "Remaining Set"§Remaining Set:  The search window size for each pilot in the Remaining Set should be the number of PN chips specified in Table 6.6.6.2.1-1 corresponding to SRCH_WIN_R;s.  The mobile station should center the search window for each pilot in the Remaining Set around the pilot’s PN sequence offset using timing defined by the mobile station’s time reference (see 6.1.5.1).

If the mobile station determines that one of the Neighbor Set or Remaining Set Pilot Channel signals is sufficiently stronger than the Pilot Channel of the Active Set, the mobile station should perform idle handoff as specified in 6.6.2.1.4.2.

###### 6.6.2.1.4.2 Idle Handoff Procedures

While performing an idle handoff, the mobile station shall operate in the non-slotted mode until the mobile station has received at least one valid message on the new Paging Channel.

If the new base station is listed in the xe "Neighbor List Message"§Neighbor List Message, the mobile station shall use the 3-bit xe "NGHBR_CONFIG "§NGHBR_CONFIG field to determine the actions required to transition to the new base station.  If the new base station is not listed in the Neighbor List Message, the mobile station shall perform the handoff operation using the same procedure as for a pilot in the list with the NGHBR_CONFIG field set to ‘011’.

If the xe "NGHBR_CONFIG"§NGHBR_CONFIG  field is ‘000’, the mobile station shall set ACC_MSG_SEQs to NULL (see 6.6.2.2) and shall set PILOT_PNs to the pilot offset index of the base station transmitting the new Paging Channel.  If the mobile station has not stored configuration parameters for the new Paging Channel, or if the stored information is not current (see 6.6.2.2), the mobile station shall set CONFIG_MSG_SEQs, SYS_PAR_MSG_SEQs, NGHBR_LST_MSG_SEQs, and CHAN_LST_MSG_SEQs to NULL.  The mobile station shall ;begin monitoring the Paging Channel of the new base station, using the same Code Channel and CDMA Channel.

If the xe "NGHBR_CONFIG "§NGHBR_CONFIG field is ‘001’, the mobile station shall set ACC_MSG_SEQs to NULL and shall set PILOT_PNs to the pilot offset index of the base station transmitting the new Paging Channel.  If the mobile station has not stored configuration parameters for the Primary Paging Channel of the new base station, or if the stored information is not current (see 6.6.2.2), the mobile station shall set CONFIG_MSG_SEQs, SYS_PAR_MSG_SEQs, NGHBR_LST_MSG_SEQs, and CHAN_LST_MSG_SEQs to NULL.  The ;mobile station shall begin monitoring the Primary Paging Channel of the new base station, using the same CDMA Channel.

If the xe "NGHBR_CONFIG "§NGHBR_CONFIG field is ‘010’, the mobile station shall set ACC_MSG_SEQs to NULL and shall set PILOT_PNs to the pilot offset index of the base station transmitting the new Paging Channel.  If the mobile station has not stored configuration parameters for the Primary Paging Channel of the new base station, or if the stored information is not current (see 6.6.2.2), the mobile station shall set CONFIG_MSG_SEQs, SYS_PAR_MSG_SEQs, NGHBR_LST_MSG_SEQs and CHAN_LST_MSG_SEQs to NULL.  The mobile station shall tune to the first CDMA Channel given in the xe "CDMA Channel List Message"§CDMA Channel List Message for the new base station and begin monitoring the Primary Paging Channel of the new base station.

If the xe "NGHBR_CONFIG"§NGHBR_CONFIG field is ‘011’, the mobile station shall enter the xe "Sync Channel Acquisition Substate"§Sync Channel Acquisition Substate of the xe "Mobile Station Initialization State"§Mobile Station Initialization State.

#### 6.6.2.2 Response to Overhead Information Operation

The xe "Response to Overhead Information Operation"§Response to Overhead Information Operation is performed whenever the mobile station receives an xe "overhead message"§overhead message (System Parameters Message, CDMA Channel List Message, Neighbor List Message or Access Parameters Message).  The mobile station updates internally stored information from the received message’s data fields.

Configuration information and access parameters are received in the configuration messages and the Access Parameters Message.  The configuration messages are:

•	System Parameters Message

•	CDMA Channel List Message

•	Neighbor List Message

Associated with the set of configuration messages sent on each Paging Channel is a configuration message sequence number (CONFIG_MSG_SEQ).  When the contents of one or more of the configuration messages change, the configuration message sequence number is incremented.  For each of the configuration messages received, the mobile station stores the configuration message sequence number contained in the configuration message (SYS_PAR_MSG_SEQs, CHAN_LIST_MSG_SEQs, NGHBR_LIST_MSG_SEQs).  The mobile station also stores the most recently received configuration message sequence number (CONFIG_MSG_SEQs) contained in any message.  (See 6.6.2.2.1, 6.6.2.2.3, 6.6.2.2.4 and 6.6.2.3.)  The mobile station examines the stored values of the configuration message sequence numbers to determine whether the configuration parameter values stored by the mobile station are current.

The configuration message sequence number is also included in the Page Message and the Slotted Page Message.  This allows the mobile station to determine whether the stored configuration parameters are current without waiting for a configuration message.

Access Parameters Messages are independently sequence-numbered by the ACC_MSG_SEQ field.  The mobile station stores the most recently received Access Parameters Message sequence number (ACC_MSG_SEQs).

Paging Channels shall be considered different if they are transmitted by different base stations, if they are transmitted on different code channels, or if they are transmitted on different CDMA Channels.  Configuration and access parameters from one Paging Channel shall not be used while monitoring a different Paging Channel.  The mobile station shall ignore any overhead message whose PILOT_PNr field is not equal to the pilot offset index (PILOT_PNs) of the base station whose Paging Channel is being monitored.

The mobile station may store the xe "configuration message"§configuration message parameters from Paging Channels it has recently monitored.  When a mobile station performs an idle handoff; (see 6.6.2.1.4.2) or other transition to a Paging Channel that it has recently monitored, the mobile station can determine whether the stored parameters are current by examining the xe "CONFIG_MSG_SEQs in a configuration message, in a Slotted Page Message"§CONFIG_MSG_SEQs in a configuration message, in a Slotted Page Message, or in a xe "Page Message"§Page Message.

The mobile station shall define a special value, NULL, to be stored in place of sequence numbers for messages that have not been received or are marked as not current.  The special value NULL shall be unequal to any valid message sequence number.

The mobile station shall consider the stored configuration parameters to be current only if all the following conditions are true:

•	All three stored configuration message sequence numbers (SYS_PAR_MSG_SEQs, CHAN_LIST_MSG_SEQs, NGHBR_LIST_MSG_SEQs) are equal to CONFIG_MSG_SEQs; and

•	CONFIG_MSG_SEQs is not equal to NULL; and

•	No more than T31m seconds (see Appendix D) have elapsed since the mobile station last received a valid message on the Paging Channel for which the parameters were stored.

##### 6.6.2.2.1 System Parameters Message

Whenever a System Parameters Message is received on the xe "Paging Channel"§Paging Channel, the configuration message sequence number, CONFIG_MSG_SEQr, shall be compared to that stored in SYS_PAR_MSG_SEQs.  If the comparison results in a mismatch, then the mobile station shall process the remaining fields in the message as described in 6.6.2.2.1.1, 6.6.2.2.1.2, 6.6.2.2.1.3, 6.6.2.2.1.4, and 6.6.2.2.1.5.

###### 6.6.2.2.1.1 Stored Parameters

The mobile station shall store the following parameters:

•	xe "Configuration message sequence number"§Configuration message sequence number (xe "CONFIG_MSG_SEQs = CONFIG_MSG_SEQr, SYS_PAR_MSG_SEQs = .i.CONFIG_MSG_SEQ"§CONFIG_MSG_SEQs=CONFIG_MSG_SEQr, SYS_PAR_MSG_SEQs=CONFIG_MSG_SEQr)

•	xe "System identification "§System identification (xe "SID"§SIDs = SIDr)

•	xe "Network identification "§Network identification (xe "NID"§NIDs = NIDr)

•	xe "Registration zone "§Registration zone (xe "REG_ZONE"§REG_ZONEs = REG_ZONEr)

•	Number of registration zones to be retained (TOTAL_ZONESs = TOTAL_ZONESr)

•	xe "Zone timer length "§Zone timer length (xe "ZONE_TIMERs = ZONE_TIMERr)"§ZONE_TIMERs=ZONE_TIMERr)

•	xe "Base station identification "§Base station identification (xe "BASE_IDs = BASE_IDr)"§BASE_IDs=BASE_IDr)

•	xe "Base station class"§Base station class (xe "BASE_CLASS"§BASE_CLASSs = BASE_CLASSr)

•	xe "Home registration indicator "§Home registration indicator (xe "HOME_REG"§HOME_REGs = HOME_REGr)

•	xe "NID roamer registration indicator "§NID roamer registration indicator (xe "FOR_NID_REG"§FOR_NID_REGs = FOR_NID_REGr)

•	xe "SID roamer registration indicator "§SID roamer registration indicator (xe "FOR_SID_REG"§FOR_SID_REGs = FOR_SID_REGr)

•	xe "Power-up registration indicator "§Power-up registration indicator (xe "POWER_UP_REG"§POWER_UP_REGs = POWER_UP_REGr)

•	xe "Power-down registration indicator "§Power-down registration indicator (xe "POWER_DOWN_REG"§POWER_DOWN_REGs = POWER_DOWN_REGr)

•	xe " Parameter-change registration indicator "§Parameter-change registration indicator (xe "PARAMETER_REG"§PARAMETER_REGs = PARAMETER_REGr)

•	xe "Registration period "§Registration period (xe "REG_PRD"§REG_PRDs = REG_PRDr)

•	xe "Base station latitude"§Base station latitude (xe "BASE_LAT"§BASE_LATs = BASE_LATr)

•	xe "Base station longitude"§Base station longitude (xe "BASE_LONG"§BASE_LONGs = BASE_LONGr)

•	xe "Registration distance"§Registration distance (xe "REG_DIST"§REG_DISTs = REG_DISTr)

•	xe "Search window size for the Active Set and Candidate Set"§Search window size for the Active Set and Candidate Set (xe "SRCH_WIN_A"§SRCH_WIN_As = SRCH_WIN_Ar)

•	xe "Search window size for the Neighbor Set "§Search window size for the Neighbor Set (xe "SRCH_WIN_N"§SRCH_WIN_Ns = SRCH_WIN_Nr)

•	xe "Search window size for the Remaining Set "§Search window size for the Remaining Set (xe "SRCH_WIN_R"§SRCH_WIN_Rs = SRCH_WIN_Rr)

•	xe "Maximum age for retention of .i.Neighbor Set"§Maximum age for retention of Neighbor Set members ;(xe "NGHBR_MAX_AGEs = NGHBR_MAX_AGEr)"§NGHBR_MAX_AGEs=NGHBR_MAX_AGEr)

•	xe "Power control reporting frame count "§Power control reporting frame count (xe "PWR_REP_FRAMESs = PWR_REP_FRAMESr)"§PWR_REP_FRAMESs=PWR_REP_FRAMESr)

•	xe "Power control reporting threshold"§Power control reporting threshold (xe "PWR_REP_THRESHs = PWR_REP_THRESHr)"§PWR_REP_THRESHs=PWR_REP_THRESHr)

•	xe "Power report mode"§Power report mode indicator (xe "PWR_REP_MODEs = PWR_REP_MODEr)"§PWR_REP_MODEs=PWR_REP_MODEr)

•	xe "Power report delay"§Power report delay (xe "PWR_REP_DELAYs = PWR_REP_DELAYr)"§PWR_REP_DELAYs=PWR_REP_DELAYr)

•	xe "Pilot detection threshold"§Pilot detection threshold (xe "T_ADDs  = T_ADDr)"§T_ADDs =T_ADDr)

•	xe "Pilot drop threshold"§Pilot drop threshold (xe "T_DROPs = T_DROPr)"§T_DROPs=T_DROPr)

•	xe "Active Set versus Candidate Set comparison threshold"§Active Set versus Candidate Set comparison threshold (xe "T_COMPs = T_COMPr)"§T_COMPs=T_COMPr)

•	xe "Drop timer value "§Drop timer value (xe "T_TDROPs = T_TDROPr)"§T_TDROPs=T_TDROPr)

###### 6.6.2.2.1.2 Paging Channel Assignment Change

If the xe "number of Paging Channels"§number of Paging Channels specified in the message (xe "PAGE_CHANr) is different from PAGE_CHANs, the mobile station shall use the hash algorithm specified in 6.6.7.1 to select a new Paging Channel number in the range 1 to PAGE_CHANr.  The mobile station shall store the new Paging Channel number as PAGECHs.  The mobile station shall then set PAGE_CHANs to PAGE_CHANr.  The mobile station shall set ACC_MSG_SEQs to NULL.  If the mobile station has not stored configuration parameters for the new Paging Channel, or if the stored parameters are not current (see 6.6.2.2), the mobile station shall set CONFIG_MSG_SEQs, SYS_PAR_MSG_SEQs, NGHBR_LST_MSG_SEQs, and CHAN_LST_MSG_SEQs to NULL.  The mobile station shall then begin monitoring the new Paging Channel as specified in 6.6.2.1.1."§PAGE_CHANr) is different from PAGE_CHANs, the mobile station shall use the hash algorithm specified in 6.6.7.1 to select a new P

###### 6.6.2.2.1.3 RESCAN Parameter

If the xe "RESCANr field in the .i.System Parameters Message"§RESCANr field in the System Parameters Message equals ‘1’, the mobile station shall enter the xe "System Determination Substate "§System Determination Substate of the xe "Mobile Station Initialization State"§Mobile Station Initialization State.

###### 6.6.2.2.1.4 Roaming Status

The mobile station shall determine the roaming status for each of the MINs active at the mobile station (see 6.6.5.3).  The mobile station should indicate to the user whether any MIN active at the mobile station is roaming.

###### 6.6.2.2.1.5 Registration

The mobile station shall update stored variables and perform other registration procedures as specified in 6.6.5.6.2.2.

##### 6.6.2.2.2 Access Parameters Message

Whenever an Access Parameters Message is received on the Paging Channel;, the sequence number, ACC_MSG_SEQr, shall be compared to ACC_MSG_SEQs.  If the comparison results in a mismatch, then the mobile station shall process the remaining fields in the message as follows.

The mobile station shall store the following parameters:

•	xe "Access Parameters Message sequence number"§Access Parameters Message sequence number (xe "ACC_MSG_SEQ"§ACC_MSG_SEQs = ACC_MSG_SEQr)

•	xe "Number of Access Channels"§Number of Access Channels (xe "ACC_CHAN"§ACC_CHANs = ACC_CHANr)

•	xe "Nominal transmit power offset"§Nominal transmit power offset (xe "NOM_PWR"§NOM_PWRs = NOM_PWRr)

•	xe "Initial power offset for access"§Initial power offset for access (xe "INIT_PWR"§INIT_PWRs = INIT_PWRr)

•	xe "Power increment"§Power increment (xe "PWR_STEP"§PWR_STEPs = PWR_STEPr)

•	xe "Number of access probes"§Number of access probes (xe "NUM_STEP"§NUM_STEPs = NUM_STEPr)

•	xe "Maximum Access Channel message capsule size"§Maximum Access Channel message capsule size (xe "MAX_CAP_SZ"§MAX_CAP_SZs = MAX_CAP_SZr)

•	xe "Access Channel preamble length"§Access Channel preamble length (xe "PAM_SZ"§PAM_SZs = PAM_SZr)

•	xe "Persistence"§Persistence modifier for Access Channel attempts for registrations which are not responses to the Registration Request Order (xe "REG_PSIST"§REG_PSISTs = REG_PSISTr)

•	xe "Persistence"§Persistence modifier for Access Channel attempts for message transmissions ; (xe "MSG_PSIST"§MSG_PSISTs = MSG_PSISTr)

•	xe "Time randomization"§Time randomization for Access Channel probes (xe "PROBE_PN_RAN"§PROBE_PN_RANs = PROBE_PN_RANr)

•	xe "Acknowledgement timeout "§Acknowledgement timeout (xe "ACC_TMO"§ACC_TMOs = ACC_TMOr)

•	xe "Access Channel probe backoff "§Access Channel probe backoff range (xe "PROBE_BKOFF"§PROBE_BKOFFs = PROBE_BKOFFr)

•	xe "Access Channel probe sequence backoff "§Access Channel probe sequence backoff range (xe "BKOFF"§BKOFFs = BKOFFr)

•	xe "Maximum number of probe sequences for an Access Channel request "§Maximum number of probe sequences for an Access Channel request (xe "MAX_REQ_SEQ"§MAX_REQ_SEQs = MAX_REQ_SEQr)

•	xe "Maximum number of probe sequences for an Access Channel response "§Maximum number of probe sequences for an Access Channel response (xe "MAX_RSP_SEQ"§MAX_RSP_SEQs = MAX_RSP_SEQr)

•	xe "Authentication mode"§Authentication mode (xe "AUTH"§AUTHs = AUTHr)

•	xe "Random challenge value"§Random challenge value (xe "RAND"§RANDs = RANDr)

For each MIN active at the mobile station, the mobile station shall record the xe "persistence"§persistence parameter number that corresponds to the MIN’s xe "overload class"§overload class as follows:

•	xe "Persistence value for access .i.overload class"§Persistence value for access overload class (xe "PSIST"§PSISTs = PSISTr)

##### 6.6.2.2.3 Neighbor List Message

Whenever a valid Neighbor List Message is received on the current xe "Paging Channel"§Paging Channel (PAGECHs), the configuration message sequence number, CONFIG_MSG_SEQr, shall be compared to that stored in NGHBR_LST_MSG_SEQs.  If the comparison results in a mismatch, then the mobile station shall process the remaining fields in the message as follows.

The mobile station shall store the following parameters:

•	xe "Configuration message sequence number"§Configuration message sequence number (xe "CONFIG_MSG_SEQ"§CONFIG_MSG_SEQs = CONFIG_MSG_SEQr, NGHBR_LST_MSG_SEQs = xe "CONFIG_MSG_SEQ"§CONFIG_MSG_SEQr)

•	Pilot PN sequence offset increment (PILOT_INCs = PILOT_INCr)

For each of the neighboring base stations contained in the xe "Neighbor List Message"§Neighbor List Message, the mobile station shall store the following:

•	xe "Neighbor configuration (NGHBR_CONFIG"§Neighbor configuration (NGHBR_CONFIGs = NGHBR_CONFIGr)

•	xe "Neighbor pilot PN sequence offset (NGHBR_PN"§Neighbor pilot PN sequence offset (NGHBR_PNs = NGHBR_PNr)

The mobile station shall update the idle handoff Neighbor Set and Remaining Set (see 6.6.2.1.4) so that the Neighbor Set consists only of pilot offsets listed in the Neighbor List Message.  If the Neighbor List Message contains more pilot offsets than the mobile station can store, the mobile station shall store the pilot offsets beginning at the start of the Neighbor List Message, up to the limits of the mobile station’s Neighbor Set storage capacity.

##### 6.6.2.2.4 CDMA Channel List Message

Whenever a CDMA Channel List Message is received on the xe "Paging Channel, the configuration message sequence number, CONFIG_MSG_SEQr, shall be compared to that stored in CHAN_LST_MSG_SEQs.  If the comparison results in a mismatch, then the mobile station shall process the remaining fields in the message as follows."§Paging Channel, the configuration message sequence number, CONFIG_MSG_SEQr, shall be compared to that stored in CHAN_LST_MSG_SEQ

The mobile station shall store the following parameters:

•	xe "Configuration message sequence number"§Configuration message sequence number (xe "CONFIG_MSG_SEQ"§CONFIG_MSG_SEQs = CONFIG_MSG_SEQr, CHAN_LST_MSG_SEQs = CONFIG_MSG_SEQ;r)

The mobile station shall use the hash algorithm specified in 6.6.7.1 and the number of channels listed in the CDMA Channel List Message to determine the CDMA Channel (frequency assignment) for its Paging Channel.  If the CDMA frequency assignment has changed (the computed CDMA Channel is different from CDMACHs), the mobile station shall perform the following actions:

•	Set CDMACHs to the new CDMA Channel.

•	Set PAGECHs to the Primary Paging Channel.

•	Set CONFIG_MSG_SEQs, SYS_PAR_MSG_SEQs, NGHBR_LST_MSG_SEQs, CHAN_LST_MSG_SEQs, and ACC_MSG_SEQs to NULL.

•	Tune to the new CDMA Channel.

#### 6.6.2.3 Mobile Station Page Match Operation xe "mobile station, Mobile Station Idle State, Mobile Station Page Match Operation"§

The xe "Mobile Station Page Match Operation"§Mobile Station Page Match Operation is performed whenever the mobile station receives a xe "Page Message "§Page Message or a xe "Slotted Page Message"§Slotted Page Message.  The mobile station searches each message to determine whether it contains one of the mobile station’s active xe "MIN"§MINs.  If so, the mobile station transmits a response to the Page Message or the Slotted Page Message on the xe "Access Channel"§Access Channel.

The mobile station shall compare the configuration message sequence number, CONFIG_MSG_SEQr, to CONFIG_MSG_SEQs.  If the comparison results in a mismatch, then the mobile station shall set CONFIG_MSG_SEQs to CONFIG_MSG_SEQr.  The mobile station shall also compare the Access Parameters Message sequence number, ACC_MSG_SEQr, with that stored in ACC_MSG_SEQs.  If the comparison results in a mismatch, then the mobile station shall set ACC_MSG_SEQs to NULL (see 6.6.2.2).

The mobile station shall compare each of its active MINs with the MIN in each record of the xe "Page Message"§Page Message or xe "Slotted Page Message"§Slotted Page Message.  If both MIN1 and MIN2 are present in a page record and both MIN1 and MIN2 match MIN1 and MIN2 for an active MIN at the mobile station, then a match shall be declared.  If MIN1 but not MIN2 is present in a page record, MIN1 matches MIN1 for an active MIN at the mobile station, and a home (SID, NID) pair for the same active MIN matches the SID and NID of the base station, then a match shall be declared (see 6.6.5.3).  Any other combination shall be considered a mismatch.

If a match is declared, and the MIN is configured to receive mobile station terminated calls in its present roaming status (see 6.6.5.3), the mobile station shall enter the xe "Update Overhead Information Substate"§Update Overhead Information Substate of the xe "System Access State"§System Access State (see 6.6.3) with a page response indication within T33m seconds after the page message is received.

If the MIN is not configured to receive mobile station terminated calls in its present roaming status, the mobile station may ignore the page record.

#### 6.6.2.4 Mobile Station Order and Message Processing Operation xe "mobile station, Mobile Station Idle State, Mobile Station Order and Message Processing Operation"§

During the xe "Mobile Station Order and Message Processing Operation"§Mobile Station Order and Message Processing Operation, the mobile station processes mobile station directed messages (addressed by ESN), including order messages.  Directed messages include all messages except overhead messages and page messages.

The mobile station shall compare its ESN to the ESN field of the message.  If the ESN matches, the mobile station shall process the message; otherwise the mobile station shall ignore the message.

The following cases occur for messages received on the xe "Paging Channel"§Paging Channel whose ESN field matches the mobile station’s ESN:

•	If the message requires acknowledgement, and is not the xe "Lock Order"§Lock Order, the xe "Lock Until Power-Cycled Order"§Lock Until Power-Cycled Order, or the xe "Unlock Order"§Unlock Order, the mobile station shall acknowledge the message as specified in 6.6.2.1.2.  The mobile station shall enter the xe "Update Overhead Information Substate"§Update Overhead Information Substate of the xe "System Access State"§System Access State with an order/message response indication within T33m seconds, unless otherwise specified for a particular message.

•	If the message does not require acknowledgement, the mobile station shall transmit a response only if it is required by the message or order.  If a response is required, the mobile station shall enter the xe "Update Overhead Information Substate"§Update Overhead Information Substate of the xe "System Access State"§System Access State with an order/message response indication within T33m seconds, unless otherwise specified for a particular message.

•	If the message is a message that cannot be processed by the mobile station, the mobile station shall respond with a Mobile Station Reject Order with the ORDQ field set to indicate the reason for rejection.  The mobile station shall enter the xe "Update Overhead Information Substate"§Update Overhead Information Substate of the xe "System Access State"§System Access State with an order/message response indication within T33m seconds, unless otherwise specified for a particular message.

The following directed messages and orders can be received.  If any field value of the message is outside its permissible range, the mobile station shall send a Mobile Station Reject Order with ORDQ equal to ‘00000100’ (message field not in valid range) or ‘00000010’ (message not accepted in this state), as appropriate.

1.	xe "Abbreviated Alert Order"§Abbreviated Alert Order:  The mobile station shall compare each of its active MINs with the MIN specified in the xe "Abbreviated Alert Order.  If the received MIN1 and MIN2 match the MIN1 and MIN2 for an active MIN at the mobile station, then the mobile station may alert the user."§Abbreviated Alert Order.  If the received MIN1 and MIN2 match the MIN1 and MIN2 for an active MIN at the mobile station, then th

2.	xe "Added MIN Rejected Order"§Added MIN Rejected Order; No requirements.

3.	xe "Authentication Challenge Message"§Authentication Challenge Message:  The mobile station shall process the message and shall respond with an Authentication Challenge Response Message as specified in 6.3.12.1.5.  The mobile station shall enter the xe "Update Overhead Information Substate"§Update Overhead Information Substate of the xe "System Access State"§System Access State with an order/message response indication within T32m seconds

4.	xe "Base Station Acknowledgement Order"§Base Station Acknowledgement Order:  No requirements.

5.	xe "Base Station Challenge Confirmation Order"§Base Station Challenge Confirmation Order:  The mobile station shall process the message and shall respond with an xe "SSD Update Confirmation Order"§SSD Update Confirmation Order or xe "SSD Update Rejection Order"§SSD Update Rejection Order as specified in 6.3.12.1.9.  The mobile station shall enter the xe "Update Overhead Information Substate"§Update Overhead Information Substate of the xe "System Access State"§System Access State with an order/message response indication within T32m seconds.

6.	xe "Channel Assignment Message"§Channel Assignment Message:  If the message specifies a xe "Paging Channel "§Paging Channel assignment (xe "ASSIGN_MODE"§ASSIGN_MODE equal to ‘001’), the mobile station shall store the specified xe "CDMA Channel"§CDMA Channel (CDMACHs = CDMA_FREQr) and xe "Paging Channel"§Paging Channel (PAGECHs = CODE_CHANr).  The mobile station shall set CONFIG_MSG_SEQs, SYS_PAR_MSG_SEQs, NGHBR_LST_MSG_SEQs, CHAN_LST_MSG_SEQs, and ACC_MSG_SEQs to NULL and begin monitoring the new Paging Channel.

If the ASSIGN_MODE field is any value other than ‘001’, the mobile station shall respond with a Mobile Station Reject Order; with xe "ORDQ"§ORDQ equal to ‘00000010’ (message not accepted in this state).

7.	xe "Data Burst Message"§Data Burst Message:  No requirements.

8.	xe "Local Control Order"§Local Control Order:  No requirements.

9.	xe "Lock Order"§Lock Order:  The mobile station shall record the reason for the Lock Order in the mobile station’s semi-permanent memory (xe "LCKRSNs-p = ORDQ"§LCKRSNsp = ORDQr).  After a mobile station receives this order, it shall not enter the System Access State (see 6.6.3) until it has received an xe "Unlock Order"§Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.  The mobile station should notify the user of the locked condition.  The mobile station may exit the Mobile Station Idle State and enter the System Determination Substate of the Mobile Station Initialization State.

10.	xe "Lock Until Power-Cycled Order"§Lock Until Power-Cycled Order:  The mobile station shall record the reason for the Lock Order in the mobile station’s semi-permanent memory (xe "LCKRSN_Ps-p = ORDQ"§LCKRSN_Psp = ORDQr).  After a mobile station receives this order, it shall not enter the System Access State (see 6.6.3) until it has received an xe "Unlock Order"§Unlock Order or until after power-cycling the mobile station (i.e., after the next mobile station power-up).  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.  The mobile station should notify the user of the locked condition.  The mobile station may exit the Mobile Station Idle State and enter the System Determination Substate of the Mobile Station Initialization State.

11.	xe "Message Waiting Order"§Message Waiting Order:  No requirements.

12.	xe "Registration Accepted Order"§Registration Accepted Order:  No requirements.

13.	xe "Registration Rejected Order"§Registration Rejected Order:  No requirements.

14.	xe "Registration Request Order"§Registration Request Order:  The mobile station shall process the message and perform registration procedures as specified in 6.6.5.6.2.3.

15.	xe "SSD Update Message"§SSD Update Message:  The mobile station shall process the message and shall respond with a xe "Base Station Challenge Order"§Base Station Challenge Order as specified in 6.3.12.1.9.  The mobile station shall enter the xe "Update Overhead Information Substate"§Update Overhead Information Substate of the xe "System Access State"§System Access State with an order/message response indication within T32m seconds.

16.	xe "Unlock Order"§Unlock Order:  After receiving this order, the mobile station is no longer locked.  The mobile station should notify the user that the locked condition has been removed.  The mobile station shall enter the xe "System Determination Substate"§System Determination Substate of the xe "Mobile Station Initialization State"§Mobile Station Initialization State.

If any message or order is received that is not listed above, the mobile station shall respond with a xe "Mobile Station Reject Order"§Mobile Station Reject Order with xe "ORDQ"§ORDQ equal to ‘00000010’ (message not accepted in this state).

#### 6.6.2.5 Mobile Station Origination Operation xe "mobile station, Mobile Station Idle State, Mobile Station Origination Operation"§

The Mobile Station Origination Operation is performed when the mobile station is directed by the user to initiate a call.

The mobile station shall enter the Update Overhead Information Substate; of the xe "System Access State"§System Access State (see 6.6.3) with a call origination indication within T33m seconds.

#### 6.6.2.6 Mobile Station Message Transmission Operation

Support of this operation is optional.  If the mobile station supports the Mobile Station Message Transmission Operation, the operation is performed when the user directs the mobile station to transmit a message.

If the mobile station supports this operation, the mobile station shall enter the Update Overhead Information Substate; of the xe "System Access State"§System Access State (see 6.6.3) with a message transmission indication within T33m seconds.

#### 6.6.2.7 Mobile Station Power-Down Operation

The Mobile Station Power-Down Operation is performed when the user directs the mobile station to power down.

The mobile station shall update stored parameters and perform other registration procedures as specified in 6.6.5.6.2.4.

If no power-down registration is performed (see 6.6.5.6.2.4), the mobile station may power down.

### 6.6.3 System Access State xe "mobile station, System Access State"§

In this state, the mobile station sends messages to the base station on the Access Channel(s) and receives messages from the base station on the Paging Channel.

As illustrated in Figure 6.6.3-1, the System Access State consists of the following substates:

• 	Update Overhead Information Substate - In this substate, the mobile station monitors the Paging Channel until it has received a current set of configuration messages.

•	Mobile Station Origination Attempt Substate - In this substate, the mobile station sends an Origination Message to the base station.

•	Page Response Substate - In this substate, the mobile station sends a Page Response Message to the base station.

•	Mobile Station Order/Message Response Substate - In this substate, the mobile station sends a response to a message received from the base station.

•	Registration; Access Substate - In this substate, the mobile station sends a Registration Message or Registration Update Message to the base station.

•	Mobile Station Message Transmission Substate - In this substate, the mobile station sends a Data Burst Message to the base station.

tc "Figure 6.6.3-1.  System Access State"\c §Figure 6.6.3-1.  System Access State

#### 6.6.3.1 Access Procedures xe "mobile station, System Access State, Access Procedures"§

##### 6.6.3.1.1 Access Attempts

###### 6.6.3.1.1.1 Overview

The mobile station transmits on the Access Channel using a random access protocol.  Many parameters of the random access protocol are supplied by the base station in the xe "Access Parameters Message"§Access Parameters Message.  The mobile station transmits a single message repeatedly until it receives an acknowledgement from the base station on the Paging Channel.  Each transmission is called an access probe.  The entire process of sending one message and receiving (or failing to receive) an acknowledgement for that message is called an access attempt (see Figures 6.6.3.1.1.1-1 and 6.6.3.1.1.1-2).

Within an access attempt, access probes are grouped into access probe sequences.  Each access probe sequence consists of up to NUM_STEP + 1 access probes, all transmitted on the same Access Channel.  The Access Channel used for each access probe sequence is chosen pseudorandomly from among all the Access Channels associated with the current Paging Channel.  The first access probe of each access probe sequence is transmitted at a specified power level relative to the nominal open loop power level.  Each subsequent access probe is transmitted at a power level a specified amount higher than the previous access probe (see 6.1.2.3.1).

The timing of access probes and access probe sequences is expressed in terms of Access Channel slots (see 6.7.1.1).  Each slot is the length of one Access Channel transmission (access probe) and each access probe occurs in a particular slot.

The timing of the start of each access probe sequence is determined pseudorandomly.  Different procedures are used for sending a response message (one that is a response to a base station message) and for sending a request message (one that is sent autonomously by the mobile station).  For every access probe sequence except the first access probe sequence of a request access attempt, a backoff delay RS of between 0 and BKOFF + 1 slots is generated pseudorandomly.  For a response access probe sequence, the first access probe of the sequence begins after RS slots.  For request access probe sequences only, an additional delay is imposed by the use of a persistence test.  For each slot after the backoff delay RS, the mobile station performs a pseudorandom test, with parameters that depend on the reason for the access attempt and the access overload class (ACCOLCp) of the mobile station.  If the test passes, the first access probe of the sequence begins in that slot.  If the test fails, the access probe sequence is deferred until at least the next slot.

Timing between access probes of an access probe sequence is also generated pseudorandomly.  After transmitting each access probe, the mobile station waits a specified period TA = ACC_TMO ¥ 80 ms to receive an acknowledgement from the base station.  If an acknowledgement is received, the access attempt ends.  If no acknowledgement is received, the next access probe is transmitted after an additional backoff delay RT of between 0 and PROBE_BKOFF + 1 slots.

The precise timing of the Access Channel transmissions in an access attempt is determined by a procedure called PN randomization.  For each access attempt, the mobile station computes a delay RN of from 0 to 2xe "PROBE_PN_RAN"§PROBE_PN_RAN - 1 PN chips using a (non-random) hash function that depends on its ESN.  The mobile station delays its transmit timing by RN PN chips.  This transmit timing adjustment includes delay of the direct sequence spreading long code and of the quadrature spreading I and Q pilot PN sequences, so it effectively increases the apparent range from the mobile station to the base station.

tc "Figure 6.6.3.1.1.1-1A.  Access Channel Request and Response Attempts"\c §Figure 6.6.3.1.1.1-1A.  Access Channel Request and Response Attempts

xe "Access Probe"§xe "Backoff Randomization "§xe "PROBE_PN_RAN"§xe "Acknowledgement Response Timeout"§

tc "Figure 6.6.3.1.1.1-1B.  Access Channel Request and Response Attempts"\c §Figure 6.6.3.1.1.1-1B.  Access Channel Request and Response Attempts

xe "Access Probe"§xe "Backoff Randomization"§xe "PROBE_PN_RAN"§xe "Acknowledgement Response Timeout"§

tc "Figure 6.6.3.1.1.1-2.  Access Procedure"\c §Figure 6.6.3.1.1.1-2.  Access Procedure

###### 6.6.3.1.1.2 Requirements

Each time the mobile station performs an access attempt, it shall compute a number, RN, from 0 to 2PROBE_PN_RAN - 1, using the hashing technique described in 6.6.7.1.  For the duration of this access attempt, the mobile station shall delay its transmit timing, including long code direct sequence spreading (see 6.1.3.2.7) and I and Q pilot PN sequence quadrature spreading (see 6.1.3.2.8), by RN PN chips.

When the mobile station performs an access attempt, it shall transmit one or more access probe sequences.  If the xe "access attempt"§access attempt is an xe "Access Channel request"§Access Channel request, the mobile station shall transmit no more than xe "MAX_REQ_SEQ"§MAX_REQ_SEQs xe "access probe sequence"§access probe sequences; if the xe "access attempt"§access attempt is an xe "Access Channel response"§Access Channel response, the mobile station shall transmit no more than xe "MAX_RSP_SEQ"§MAX_RSP_SEQs access probe sequences.

Before transmitting each xe "access probe sequence"§access probe sequence, the mobile station shall generate a random number, RA, from 0 to xe "ACC_CHAN"§ACC_CHANs using the procedure described in 6.6.7.2.  The mobile station shall use this random number, RA, as the Access Channel number for all access probes in that access probe sequence (see 6.1.3.1.8).

Before transmitting each access probe sequence of a response attempt and before each access probe sequence other than the first access probe sequence of a request attempt, the mobile station shall generate a random number, RS, from 0 to (BKOFFs + 1), using the procedure described in 6.6.7.2.  The mobile station shall delay the transmission of the access probe sequence for RS slots.

If the xe "access attempt"§access attempt is an xe "Access Channel request"§Access Channel request, then before transmitting the first xe "access probe"§access probe in each xe "access probe sequence, and after the delay of RS if applicable, the mobile station shall perform a persistence test for each .i.Access Channel slot "§access probe sequence, and after the delay of RS if applicable, the mobile station shall perform a persistence test for each Accuntil the test passes.  When the test passes, the mobile station shall transmit the first access probe in the corresponding slot.  To perform the persistence test, the mobile station shall generate a random number RP, 0 _ RP _ 1, using the technique described in 6.6.7.2.  The persistence test is said to pass when RP is less than the current value of P for the type of this access attempt.  If P equals 0, the access attempt fails, and the mobile station shall end the access attempt, update its registration variables as specified in 6.6.5.6.3.2, and enter the System Determination Substate of the Mobile Station Initialization State.

If the Access Channel request is a registration, P shall be computed by

P =  		n = 0, 1, ..., 9

P =  		n = 10, 11, ..., 15

where n is the highest overload class (ACCOLCp) assigned to a MIN being actively supported by the mobile station.

If the Access Channel request is a message transmission, P shall be computed by

P =  		n = 0, 1, ..., 9

P =  		n = 10, 11, ..., 15

where n is the overload class assigned to the MIN sending the message.

If the Access Channel request is other than a registration or a message transmission, P shall be computed by

P =  		n = 0, 1, ..., 9

P =  		n = 10, 11, ..., 15

where n is the overload class assigned to a MIN associated with the request.

The mobile station shall transmit the first probe in each xe "access probe sequence"§access probe sequence at the power level specified in 6.1.2.3.1.  The mobile station shall transmit each subsequent probe in the access probe sequence at a power level PWR_STEPs ¥ 0.5 dB greater than that of the previous probe.  Between access probes, the mobile station shall disable its transmitter.

After transmitting each probe, the mobile station shall wait TA = ACC_TMOs ¥ 80 ms from the end of the xe "Access Channel"§Access Channel slot.  If no acknowledgement is received within TA seconds, the mobile station shall perform the following:

•	If NUM_STEPs or fewer access probes have been transmitted in this access probe sequence, the mobile station shall generate a random number, RT, from 0 to PROBE_BKOFF + 1, using the procedure described in 6.6.7.2.  The mobile station shall delay RT additional Access Channel slots, and then transmit the next access probe.

•	Otherwise, if fewer than MAX_REQ_SEQ (for a request access) or MAX_RSP_SEQ (for a response access) access probe sequences have been transmitted in this access attempt, the mobile station shall begin the randomization procedures for another access probe sequence.

•	Otherwise, the mobile station shall update its registration variables as specified in 6.6.5.6.3.2 and enter the System Determination Substate of the Mobile Station Initialization State.

##### 6.6.3.1.2 Acknowledgement Procedures xe "mobile station, System Access State"§

The acknowledgement procedures facilitate the reliable exchange of messages between the base station and the mobile station.  The mobile station uses the fields ACK_SEQ (acknowledgement sequence number), MSG_SEQ (message sequence number), ACK_REQ (acknowledgement required), and VALID_ACK (valid acknowledgement) to support this mechanism.  These fields are referred to as layer 2 fields, and the acknowledgement procedures are referred to as layer 2 procedures.  All other message fields and the processing thereof are referred to as pertaining to layer 3.  (See Appendix C for further discussion of layering.)

The mobile station shall set the ACK_SEQ and VALID_ACK fields of all messages sent on the Access Channel as specified in 6.6.2.1.2.

The mobile station shall generate a single set of MSG_SEQ numbers for messages sent on the Access Channel.  The mobile station shall set the MSG_SEQ field of the first message sent on the Access Channel after powering on to 0.  The mobile station shall increment MSG_SEQ, modulo 4, for each new access attempt, even if the contents of the new message are identical to those of the previous message.

The mobile station shall monitor the Paging Channel while in the System Access State.  When the mobile station receives a message with the VALID_ACK field set to ‘1’ and the ACK_SEQ field set to the MSG_SEQ number of the message currently being sent, the mobile station shall consider the current message to have been acknowledged and shall end the access attempt.

The mobile station shall not begin a new access attempt until the previous access attempt has ended.

##### 6.6.3.1.3 Registration xe "mobile station, System Access State"§

When an access attempt ends by the receipt of an acknowledgement from the base station, the mobile station shall update its registration variables as specified in 6.6.5.6.3.1.

##### 6.6.3.1.4 Handoffs

While in the System Access State, the mobile station should continue its pilot search (see 6.6.2.1.4.1), but shall not perform idle handoffs.

#### 6.6.3.2 Update Overhead Information Substatexe "mobile station, System Access State, Update Overhead Information Substate"§

In this substate, the mobile station monitors the Paging Channel until it has received the current configuration messages. The mobile station compares sequence numbers to determine whether all the configuration messages are up to date.  To make sure it has the latest access parameters, the mobile station receives at least one message containing the ACC_MSG_SEQ field (except in case of a page response, since the initiating Page Message or Slotted Page Message contains ACC_MSG_SEQ), and waits, if necessary, for an Access Parameters Message.

Upon entering the xe "Update Overhead Information Substate"§Update Overhead Information Substate, the mobile station shall set the xe "System Access State timer"§System Access State timer to a value of T41m seconds. The mobile station shall set PAGED to NO. If the Update Overhead Information Substate was entered with a page response indication, the mobile station shall set CURR_ACC_MSG_SEQ to ACC_MSG_SEQs; otherwise, it shall set CURR_ACC_MSG_SEQ to NULL.

If the state timer expires while in this substate, the mobile station shall enter the xe "System Determination Substate"§System Determination Substate of the xe "Mobile Station Initialization State"§Mobile Station Initialization State.

While in the Update Overhead Information Substate, the mobile station shall monitor the Paging Channel.  If the mobile station declares a loss of the Paging Channel (see 6.4.3), it shall enter the Mobile Station Idle State.  If the mobile station receives any of the following messages or orders that contain the mobile station’s xe "ESN"§ESN, the mobile station shall process the message as follows:

1.	xe "Local Control Order"§Local Control Order:  No requirements;.

2.	xe "Lock Order"§Lock Order:  The mobile station shall record the reason for the Lock Order in the mobile station’s semi-permanent memory (LCKRSNs-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

3.	xe "Lock Until Power-Cycled Order"§Lock Until Power-Cycled Order:  The mobile station shall record the reason for the Lock Until Power-Cycled Order in the mobile station’s semi-permanent memory (LCKRSN_Ps-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until after the next mobile station power-up or until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

If the mobile station receives any of the following messages, it shall process the message as follows:

1.	System Parameters Message:  The mobile station shall store the parameters from the message (see 6.6.2.2.1.1).

2.	Access Parameters Message:  The mobile station shall store the parameters from the message (see 6.6.2.2.2).

3.	Neighbor List Message:  The mobile station shall store the parameters from the message (see 6.6.2.2.3).

4.	CDMA Channel List Message:  The mobile station shall store the parameters from the message (see 6.6.2.2.4).

5.	Slotted Page Message:  The mobile station shall set CURR_ACC_MSG_SEQ to ACC_MSG_SEQr.  If this substate was not entered with an origination or page response indication, the mobile station shall compare each of its active MINs with the MIN in each record of the message.  If a match is declared (see 6.6.2.3), the mobile station shall set PAGED to YES.

6.	Page Message:  The mobile station shall set CURR_ACC_MSG_SEQ to ACC_MSG_SEQr.  If this substate was not entered with an origination or page response indication, the mobile station shall compare each of its active MINs with the MIN in each record of the message.  If a match is declared (see 6.6.2.3), the mobile station shall set PAGED to YES.

7.	Any other message or order:  The mobile station may ignore any other order or message.

When the stored configuration parameters are current (see 6.6.2.2) and CURR_ACC_MSG_SEQ and ACC_MSG_SEQs are equal and are not NULL, the mobile station shall disable the System Access State timer and do one of the following:

•	If PAGED is equal to YES or if this substate was entered with a page response indication, the mobile station shall enter the xe "Page Response Substate"§Page Response Substate.

•	If this substate was entered with an origination indication, the mobile station shall enter the xe "Mobile Station Origination Attempt Substate"§Mobile Station Origination Attempt Substate.

•	If this substate was entered with an order/message response indication, the mobile station shall enter the xe "Mobile Station Order/Message Response Substate"§Mobile Station Order/Message Response Substate.

•	If this substate was entered with a registration indication, the mobile station shall enter the Registration Access Substate.

•	If this substate was entered with a message transmission indication, the mobile station shall enter the xe "Mobile Station Message Transmission Substate."§Mobile Station Message Transmission Substate.

#### 6.6.3.3 Page Response Substate xe "mobile station, System Access State, Page Response Substate"§

In this substate, the mobile station sends a Page Response Message in response to a Page Message or Slotted Page Message from the base station.  If the base station responds with an authentication request, the mobile station responds in this substate.

Upon entering the xe "Page Response Substate"§Page Response Substate, the mobile station shall send a Page Response Message, using the access procedures specified in 6.6.3.1.1.2.

While in this substate, the mobile station shall monitor the Paging Channel.  If the mobile station declares a loss of the Paging Channel (see 6.4.3), it shall disable its transmitter and enter the Mobile Station Idle State.  If the mobile station receives an acknowledgement to any message sent by the mobile station in this substate, the mobile station shall end the access attempt and update the registration variables as specified in 6.6.3.1.3.  If the acknowledgement was not included in a xe "Channel Assignment Message"§Channel Assignment Message, Authentication Challenge Message, Base Station Challenge Confirmation Order, or SSD Update Message, the mobile station shall set the System Access State timer to T42m.

If the System Access State timer expires while in this substate, the mobile station shall enter the Mobile Station Idle State.

If the mobile station receives any of the following messages and ESNr matches the mobile station’s ESN, then the mobile station shall process the message as described.

1.	xe "Authentication Challenge Message"§Authentication Challenge Message:  If the mobile station receives this message while an access attempt is in progress, the mobile station shall ignore the message.  If the mobile station receives this message after, or included in the same message as, the acknowledgement to any message sent by the mobile station in this substate, the mobile station shall disable the System Access State timer and respond to the message as specified in 6.3.12.1.5, using the access procedures specified in 6.6.3.1.1.2.

2.	xe "Base Station Challenge Confirmation Order"§Base Station Challenge Confirmation Order:  If the mobile station receives this message while an access attempt is in progress, the mobile station shall ignore the message.  If the mobile station receives this message after, or included in the same message as, the acknowledgement to any message sent by the mobile station in this substate, the mobile station shall disable the System Access State timer and respond to the message as specified in 6.3.12.1.9, using the access procedures specified in 6.6.3.1.1.2.

3.	xe "Channel Assignment Message"§Channel Assignment Message:  The mobile station shall terminate any access attempt in progress.  It shall then process the message as follows:

If ASSIGN_MODEr equals ‘000’, the mobile station shall store the Forward Traffic Channel code channel (CODE_CHANs = CODE_CHANr), the frame offset (FRAME_OFFSETs = FRAME_OFFSETr), the message encryption mode indicator (ENCRYPT_MODEs = ENCRYPT_MODEr), and, if FREQ_INCLr equals ‘1’, the frequency assignment (CDMACHs = CDMA_FREQr), and then enter the xe "Traffic Channel Initialization Substate"§Traffic Channel Initialization Substate of the xe "Mobile Station Control on the Traffic Channel State"§Mobile Station Control on the Traffic Channel State.

If ASSIGN_MODEr equals ‘001’, the mobile station shall store the assigned Paging Channel (PAGECHs = CODE_CHANr).  If FREQ_INCLr equals ‘1’, the mobile station shall store the frequency assignment (CDMACHs = CDMA_FREQr).  The mobile station shall then enter the xe "Mobile Station Idle State"§Mobile Station Idle State.

If ASSIGN_MODEr equals ‘010’ and RESPONDr equals ‘1’, the mobile station shall enter the Initialization Task with a page response indication (see 2.6.1).

If ASSIGN_MODEr equals ‘010’ and RESPONDr equals ‘0’, the mobile station shall enter the Initialization Task (see 2.6.1).

If ASSIGN_MODEr equals ‘011’, the mobile station shall store the system identification (SIDs = SIDr), voice mobile station attenuation code (VMACs = VMACr), voice channel number (ANALOG_CHANs = ANALOG_CHANr), SAT color code (SCCs = SCCr), and message encryption mode indicator (MEMs = MEMr) and enter the Confirm Initial Voice Channel Task (see 2.6.4.2) with a page response indication.

4.	xe "Local Control Order"§Local Control Order:  No requirements;.

5.	xe "Lock Order"§Lock Order:  The mobile station shall disable its transmitter and record the reason for the Lock Order in the mobile station’s semi-permanent memory (LCKRSNs-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

6.	xe "Lock Until Power-Cycled Order"§Lock Until Power-Cycled Order:  The mobile station shall disable its transmitter and record the reason for the Lock Until Power-Cycled Order in the mobile station’s semi-permanent memory (LCKRSN_Ps-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until after the next mobile station power-up or until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

7.	xe "Release Order"§Release Order:  The mobile station shall enter the xe "Mobile Station Idle State"§Mobile Station Idle State or the xe "System Determination Substate "§System Determination Substate of the xe "Mobile Station Initialization State"§Mobile Station Initialization State.

8.	xe "SSD Update Message"§SSD Update Message:  If the mobile station receives this message while an access attempt is in progress, the mobile station shall ignore the message.  If the mobile station receives this message after, or included in the same message as, the acknowledgement to any message sent by the mobile station in this substate, the mobile station shall disable the System Access State timer and respond to the message as specified in 6.3.12.1.9, using the access procedures specified in 6.6.3.1.1.2.

9.	Any other message or order;:  The mobile station may ignore any other order or message.

#### 6.6.3.4 Mobile Station Order/Message Response Substate xe "mobile station, System Access State, Mobile Station Order/Message Response Substate"§

In this substate, the mobile station sends a message that is a response to a message received from the base station.  If the base station responds with an authentication request, the mobile station responds in this substate.

Upon entering the Mobile Station Order/Message Response Substate, the mobile station shall send the response message using the access procedures specified in 6.6.3.1.1.2.

While in this substate, the mobile station shall monitor the Paging Channel.  If the mobile station declares a loss of the Paging Channel (see 6.4.3), it shall enter the Mobile Station Idle State.  If the mobile station receives an acknowledgement to any message sent by the mobile station in this substate, it shall end the access attempt and update the registration variables as specified in 6.6.3.1.3.  If the acknowledgement was not included in an Authentication Challenge Message, Base Station Challenge Confirmation Order, or SSD Update Message, the mobile station shall enter the Mobile Station Idle State.

If the mobile station receives any of the following messages and ESNr matches the mobile station’s ESN, then the mobile station shall process the message as follows:

1.	xe "Authentication Challenge Message"§Authentication Challenge Message:  If the mobile station receives this message while an access attempt is in progress, the mobile station shall ignore the message.  If the mobile station receives this message included in the acknowledgement to any message sent by the mobile station in this substate, the mobile station shall respond to the message as specified in 6.3.12.1.5, using the access procedures specified in 6.6.3.1.1.2.

2	xe "Base Station Challenge Confirmation Order"§Base Station Challenge Confirmation Order:  If the mobile station receives this message while an access attempt is in progress, the mobile station shall ignore the message.  If the mobile station receives this message included in the acknowledgement to any message sent by the mobile station in this substate, the mobile station shall respond to the message as specified in 6.3.12.1.9, using the access procedures specified in 6.6.3.1.1.2.

3.	xe "Local Control Order"§Local Control Order:  No requirements;.

4.	xe "Lock Order"§Lock Order:  The mobile station shall disable its transmitter and record the reason for the Lock Order in the mobile station’s semi-permanent memory (LCKRSNs-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

5.	xe "Lock Until Power-Cycled Order"§Lock Until Power-Cycled Order:  The mobile station shall disable its transmitter and record the reason for the Lock Until Power-Cycled Order in the mobile station’s semi-permanent memory (LCKRSN_Ps-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until after the next mobile station power-up or until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

6.	xe "SSD Update Message"§SSD Update Message:  If the mobile station receives this message while an access attempt is in progress, the mobile station shall ignore the message.  If the mobile station receives this message included in the acknowledgement to any message sent by the mobile station in this substate, the mobile station shall respond to the message as specified in 6.3.12.1.9, using the access procedures specified in 6.6.3.1.1.2.

7.	Any other message or order:  The mobile station may ignore any other message or order.

#### 6.6.3.5 Mobile Station Origination Attempt Substate xe "mobile station, System Access State, Mobile Station Origination Attempt Substate"§

In this substate, the mobile station sends an xe "Origination Message"§Origination Message.  If the base station responds with an authentication request, the mobile station responds in this substate.

Upon entering the xe "Mobile Station Origination Attempt Substate"§Mobile Station Origination Attempt Substate, the mobile station shall send the Origination Message using the access procedures specified in 6.6.3.1.1.2.  The mobile station shall include in the Origination Message as many of the dialed digits as possible without exceeding the message capsule size.

While in this substate, the mobile station shall monitor the Paging Channel.  If the mobile station declares a loss of the Paging Channel (see 6.4.3), it shall enter the Mobile Station Idle State.  If the mobile station receives an acknowledgement to any message sent by the mobile station in this substate, it shall end the access attempt and update the registration variables as specified in 6.6.3.1.3.  If the acknowledgement was not included in a xe "Channel Assignment Message"§Channel Assignment Message, Authentication Challenge Message, Base Station Challenge Confirmation Order, or SSD Update Message, the mobile station shall set the System Access State timer to T42m.

If the System Access State timer expires while in this substate, the mobile station shall enter the Mobile Station Idle State.

If the mobile station is directed by the user to disconnect the call, the mobile station shall abort any access attempt in progress and enter the System Determination Substate of the Mobile Station Initialization State.

If the mobile station receives any of the following messages and ESNr matches the mobile station’s ESN, then the mobile station shall process the message as follows:

1.	xe "Authentication Challenge Message"§Authentication Challenge Message:  If the mobile station receives this message while an access attempt is in progress, the mobile station shall ignore the message.  If the mobile station receives this message after, or included in the same message as, the acknowledgement to any message sent by the mobile station in this substate, the mobile station shall disable the System Access State timer and respond to the message as specified in 6.3.12.1.5, using the access procedures specified in 6.6.3.1.1.2.

2.	xe "Base Station Challenge Confirmation Order"§Base Station Challenge Confirmation Order:  If the mobile station receives this message while an access attempt is in progress, the mobile station shall ignore the message.  If the mobile station receives this message after, or included in the same message as, the acknowledgement to any message sent by the mobile station in this substate, the mobile station shall disable the System Access State timer and respond to the message as specified in 6.3.12.1.9, using the access procedures specified in 6.6.3.1.1.2.

3.	xe "Channel Assignment Message"§Channel Assignment Message:  The mobile station shall terminate any access attempt in progress.  It shall then process the message as follows:

If ASSIGN_MODEr equals ‘000’, the mobile station shall store the Forward Traffic Channel code channel (CODE_CHANs = CODE_CHANr), the frame offset (FRAME_OFFSETs = FRAME_OFFSETr), the message encryption mode indicator (ENCRYPT_MODEs = ENCRYPT_MODEr), and, if FREQ_INCLr equals ‘1’, the frequency assignment (CDMACHs = CDMA_FREQr), and then enter the xe "Traffic Channel Initialization Substate"§Traffic Channel Initialization Substate of the xe "Mobile Station Control on the Traffic Channel State"§Mobile Station Control on the Traffic Channel State.

If ASSIGN_MODEr equals ‘001’, the mobile station shall store the assigned Paging Channel (PAGECHs = CODE_CHANr).  If FREQ_INCLr equals ‘1’, the mobile station shall store the frequency assignment (CDMACHs = CDMA_FREQr).  The mobile station shall then enter the xe "Mobile Station Idle State"§Mobile Station Idle State.

If ASSIGN_MODEr equals ‘010’ and RESPONDr equals ‘1’, the mobile station shall enter the Initialization Task with a page response indication (see 2.6.1).

If ASSIGN_MODEr equals ‘010’ and RESPONDr equals ‘0’, the mobile station shall enter the Initialization Task (see 2.6.1).

If ASSIGN_MODEr equals ‘011’, the mobile station shall store the system identification (SIDs = SIDr), voice mobile station attenuation code (VMACs = VMACr), voice channel number (ANALOG_CHANs = ANALOG_CHANr), SAT color code (SCCs = SCCr), and message encryption mode indicator (MEMs = MEMr) and enter the Confirm Initial Voice Channel Task (see 2.6.4.2) with a page response indication.

4.	xe "Intercept Order"§Intercept Order:  The mobile station shall terminate any access attempt in progress and enter the xe "Mobile Station Idle State"§Mobile Station Idle State.

5.	xe "Local Control Order"§Local Control Order:  No requirements;.

6.	xe "Lock Order"§Lock Order:  The mobile station shall disable its transmitter and record the reason for the Lock Order in the mobile station’s semi-permanent memory (LCKRSNs-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

7.	xe "Lock Until Power-Cycled Order"§Lock Until Power-Cycled Order:  The mobile station shall disable its transmitter and record the reason for the Lock Until Power-Cycled Order in the mobile station’s semi-permanent memory (LCKRSN_Ps-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until after the next mobile station power-up or until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

8.	xe "Release Order"§Release Order:  The mobile station shall enter the xe "Mobile Station Idle State"§Mobile Station Idle State or the xe "System Determination Substate "§System Determination Substate of the xe "Mobile Station Initialization State"§Mobile Station Initialization State.

9.	xe "Reorder Order"§Reorder Order:  The mobile station shall terminate any access attempt in progress and enter the xe "Mobile Station Idle State"§Mobile Station Idle State.

10.	xe "SSD Update Message"§SSD Update Message:  If the mobile station receives this message while an access attempt is in progress, the mobile station shall ignore the message.  If the mobile station receives this message after, or included in the same message as, the acknowledgement to any message sent by the mobile station in this substate, the mobile station shall disable the System Access State timer and respond to the message as specified in 6.3.12.1.9, using the access procedures specified in 6.6.3.1.1.2.

11.	Any other message or order;:  The mobile station may ignore any other message or order.

#### 6.6.3.6 Registration Access Substate xe "mobile station, System Access State, Registration Access Substate"§

In this substate, the mobile station sends an Initial Registration Message, a sequence of one or more Registration Update Messages, or both.  If the base station responds with an authentication request, the mobile station responds in this substate.

Upon entering the Registration Access Substate, the mobile station shall send the first registration message, using the access procedures specified in 6.6.3.1.1.2.

While in this substate, the mobile station shall monitor the Paging Channel.  If the mobile station declares a loss of the Paging Channel (see 6.4.3), it shall enter the Mobile Station Idle State.  If the mobile station receives an acknowledgement to any message sent by the mobile station in this substate, it shall end the access attempt and update the registration variables as specified in 6.6.3.1.3.  If the acknowledgement was not included in an Authentication Challenge Message, Base Station Challenge Confirmation Order, SSD Update Message, or Release Order, or if the registration access was initiated due to a user direction to power down, the mobile station shall do one of the following:

•	If there are further registration messages to be sent, the mobile station shall send the next registration message using the access procedures specified in 6.6.3.1.1.2.

•	Otherwise, if the registration access was initiated due to a user direction to power down, the mobile station shall update registration variables as specified in 6.6.5.6.3.3 and may power down.

•	Otherwise, the mobile station shall enter the Mobile Station Idle State.

If the mobile station is directed by the user to originate a call, the mobile station may abort any access attempt in progress and enter the Mobile Station Origination Attempt Substate.

If the mobile station receives a Page Message or a Slotted Page Message, the mobile station may compare each of its active MINs with the MIN in each record of the message.  If a match is declared (see 6.6.2.3), the mobile station shall abort any access attempt in progress and enter the Page Response Substate.

If the mobile station receives any of the following messages and ESNr matches the mobile station’s ESN, then the mobile station shall process the message as described.

1.	xe "Added MIN Rejected Order"§Added MIN Rejected Order:  No requirements.

2.	xe "Authentication Challenge Message"§Authentication Challenge Message:  If the mobile station receives this message while an access attempt is in progress, or if the registration access was initiated due to a user direction to power down, the mobile station shall ignore the message.  If the mobile station receives this message after, or included in, the acknowledgement to any message sent by the mobile station in this substate, the mobile station shall respond to the message as specified in 6.3.12.1.5, using the access procedures specified in 6.6.3.1.1.2.

3.	xe "Base Station Challenge Confirmation Order"§Base Station Challenge Confirmation Order:  If the mobile station receives this message while an access attempt is in progress, or if the registration access was initiated due to a user direction to power down, the mobile station shall ignore the message.  If the mobile station receives this message after, or included in, the acknowledgement to its Page Response Message, the mobile station shall respond to the message as specified in 6.3.12.1.9, using the access procedures specified in 6.6.3.1.1.2.

4.	xe "Local Control Order"§Local Control Order:  No requirements;.

5	xe "Lock Order"§Lock Order:  The mobile station shall disable its transmitter and record the reason for the Lock Order in the mobile station’s semi-permanent memory (LCKRSNs-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

6.	xe "Lock Until Power-Cycled Order"§Lock Until Power-Cycled Order:  The mobile station shall disable its transmitter and record the reason for the Lock Until Power-Cycled Order in the mobile station’s semi-permanent memory (LCKRSN_Ps-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until after the next mobile station power-up or until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

7.	xe "Registration Accepted Order"§Registration Accepted Order:  No requirements.

8.	xe "Registration Rejected Order"§Registration Rejected Order:  No requirements.

9.	xe "Release Order"§Release Order:  The mobile station shall enter the xe "Mobile Station Idle State"§Mobile Station Idle State or the xe "System Determination Substate "§System Determination Substate of the xe "Mobile Station Initialization State"§Mobile Station Initialization State.

10.	xe "SSD Update Message"§SSD Update Message:  If the mobile station receives this message while an access attempt is in progress, or if the registration access was initiated due to a user direction to power down, the mobile station shall ignore the message.  If the mobile station receives this message after, or included in, the acknowledgement to its Page Response Message, the mobile station shall respond to the message as specified in 6.3.12.1.9, using the access procedures specified in 6.6.3.1.1.2.

11.	Any other message or order:  The mobile station may ignore any other message or order.

#### 6.6.3.7 Mobile Station Message Transmission Substate

In this substate, the mobile station sends a Data Burst Message.  If the base station responds with an authentication request, the mobile station responds in this substate.

Support of this substate is optional.

Upon entering the Mobile Station Message Transmission Substate, the mobile station shall transmit the xe "Data Burst Message"§Data Burst Message using the access procedures specified in 6.6.3.1.1.2.

While in this substate, the mobile station shall monitor the Paging Channel.  If the mobile station declares a loss of the Paging Channel (see 6.4.3), it shall enter the Mobile Station Idle State.  If the mobile station receives an acknowledgement to any message sent by the mobile station in this substate, it shall end the access attempt and update the registration variables as specified in 6.6.3.1.3.  If the acknowledgement was not included in an Authentication Challenge Message, Base Station Challenge Confirmation Order, or SSD Update Message, the mobile station shall enter the Mobile Station Idle State.

If the mobile station receives a Page Message or a Slotted Page Message, the mobile station may compare each of its active MINs with the MIN in each record of the message.  If a match is declared (see 6.6.2.3), the mobile station shall abort any access attempt in progress and enter the Page Response Substate.  The mobile station may store the message for later transmission.

If the mobile station receives any of the following messages and ESNr matches the mobile station’s ESN, then the mobile station shall process the message as described.

1.	xe "Authentication Challenge Message"§Authentication Challenge Message:  If the mobile station receives this message while an access attempt is in progress, the mobile station shall ignore the message.  If the mobile station receives this message included in the acknowledgement to any message sent by the mobile station in this substate, the mobile station shall respond to the message as specified in 6.3.12.1.5, using the access procedures specified in 6.6.3.1.1.2.

2.	xe "Base Station Challenge Confirmation Order"§Base Station Challenge Confirmation Order:  If the mobile station receives this message while an access attempt is in progress, the mobile station shall ignore the message.  If the mobile station receives this message included in the acknowledgement to any message sent by the mobile station in this substate, the mobile station shall respond to the message as specified in 6.3.12.1.9, using the access procedures specified in 6.6.3.1.1.2.

3.	xe "Local Control Order"§Local Control Order:  No requirements;.

4.	xe "Lock Order"§Lock Order:  The mobile station shall disable its transmitter and record the reason for the Lock Order in the mobile station’s semi-permanent memory (LCKRSNs-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

5.	xe "Lock Until Power-Cycled Order"§Lock Until Power-Cycled Order:  The mobile station shall disable its transmitter and record the reason for the Lock Until Power-Cycled Order in the mobile station’s semi-permanent memory (LCKRSN_Ps-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until after the next mobile station power-up or until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

6.	xe "Data Burst Message"§Data Burst Message:  No requirements.

7.	xe "SSD Update Message"§SSD Update Message:  If the mobile station receives this message while an access attempt is in progress, the mobile station shall ignore the message.  If the mobile station receives this message included in the acknowledgement to any message sent by the mobile station in this substate, the mobile station shall respond to the message as specified in 6.3.12.1.9, using the access procedures specified in 6.6.3.1.1.2.

8.	Any other message or order;:  The mobile station may ignore any other message or order.

### 6.6.4 Mobile Station Control on the Traffic Channel Statexe "mobile station, Mobile Station Control on the Traffic Channel State"§

In this state, the mobile station communicates with the base station using the Forward and Reverse Traffic Channels.

As illustrated in Figure 6.6.4-1, the xe "Mobile Station Control on the Traffic Channel State consists of the following substates:"§Mobile Station Control on the Traffic Channel State consists of the following substates

•	xe "Traffic Channel Initialization Substate - In this substate, the mobile station verifies that it can receive the Forward Traffic Channel and begins transmitting on the Reverse Traffic Channel."§Traffic Channel Initialization Substate - In this substate, the mobile station verifies that it can receive the Forward Traffic

•	xe "Waiting for Order Substate - In this substate, the mobile station waits for an Alert With Information Message."§Waiting for Order Substate - In this substate, the mobile station waits for an Alert With Information Message.

•	xe "Waiting for Mobile Station Answer Substate - In this substate, the mobile station waits for the user to answer the call."§Waiting for Mobile Station Answer Substate - In this substate, the mobile station waits for the user to answer the call.

•	xe "Conversation Substate - In this substate, the mobile station’s primary service option application exchanges primary traffic packets with the base station."§Conversation Substate - In this substate, the mobile stations primary service option application exchanges primary traffic packe

•	xe "Release Substate - In this substate, the mobile station disconnects the call."§Release Substate - In this substate, the mobile station disconnects the call.

tc "Figure 6.6.4-1.  Mobile Station Control on the Traffic Channel State"\c §Figure 6.6.4-1.  Mobile Station Control on the Traffic Channel State

#### 6.6.4.1 Special Functions and Actions

The mobile station performs the following special functions and actions in one or more of the substates of the Mobile Station Control on the Traffic Channel State.

##### 6.6.4.1.1 Forward Traffic Channel Power Control

To support Forward Traffic Channel power control, the mobile station reports frame error rate statistics to the base station.  If the base station enables periodic reporting, the mobile station reports frame error rate statistics at specified intervals.  If the base station enables threshold reporting, the mobile station reports frame error rate statistics when the frame error rate reaches a specified threshold.

The mobile station shall maintain a counter (TOT_FRAMESs) for the total number of received frames and a counter (BAD_FRAMESs) for the number of received bad frames (see 6.2.2.2).

The mobile station shall perform the following for each received frame:

•	The mobile station shall increment TOT_FRAMESs by 1.

•	If the received frame is bad, the mobile station shall increment BAD_FRAMESs by 1.

•	If PWR_REP_THRESHs is not zero and BAD_FRAMESs is equal to PWR_REP_THRESHs, the mobile station shall send a Power Measurement Report Message and set TOT_FRAMESs and BAD_FRAMESs to zero.  After sending the message, the mobile station shall not increment either counter for a period of PWR_REP_DELAYs ¥ 4 frames.

•	If PWR_REP_MODEs is equal to ‘0’ and TOT_FRAMESs is equal to Î(2(PWR_REP_FRAMESs/2) ¥ 5)_ , the mobile station shall set TOT_FRAMESs and BAD_FRAMESs to zero.

•	If PWR_REP_MODEs is equal to ‘1’ and TOT_FRAMESs is equal to Î(2(PWR_REP_FRAMESs/2) ¥ 5)_ , the mobile station shall send a Power Measurement Report Message and set TOT_FRAMESs and BAD_FRAMESs to zero.

###### 6.6.4.1.1.1 Forward Traffic Channel Power Control Initialization

To initialize Forward Traffic Channel power control, the mobile station shall set TOT_FRAMESs and BAD_FRAMESs to zero.

###### 6.6.4.1.1.2 Processing the Power Control Parameters Message

The mobile station shall store the following parameters from the Power Control Parameters Message:

•	Power control reporting threshold (PWR_REP_THRESHs = PWR_REP_THRESHr)

•	Power control reporting frame count (PWR_REP_FRAMESs = PWR_REP_FRAMESr)

•	Power report mode indicator (PWR_REP_MODEs = PWR_REP_MODEr)

•	Power report delay (PWR_REP_DELAYs = PWR_REP_DELAYr)

The mobile station shall set TOT_FRAMESs and BAD_FRAMESs to zero.

##### 6.6.4.1.2 Service Options

###### 6.6.4.1.2.1 Overview xe "mobile station, Mobile Station Control on the Traffic Channel State, Service Options"§

While operating on the Traffic Channel, the mobile station and base station can use the default service option, Service Option 1, or request an alternative service option.  The mobile station can request an alternative service option at the time of call origination or during operation on the Traffic Channel.  If the service option request is acceptable to the base station, the mobile station and the base station begin using the new service option.

If the mobile station requests a service option that is not acceptable to the base station, the base station can reject the requested service option or request an alternative service option.  If the base station requests an alternative service option, the mobile station can accept or reject the base station’s alternative service option, or request another service option.  This process, called service option negotiation, ends when the mobile station and base station find a mutually acceptable service option, or when the mobile station rejects a service option request from the base station or the base station rejects a service option request from the mobile station.

The mobile station and base station use the Service Option Request Order to request a service option, and Service Option Response Order to accept or reject a service option request.  In addition, the mobile station can request a service option in the Origination Message.

The mobile station uses a variable (SO_REQs) to record the number of the service option for which the mobile station has sent a Service Option Request Order and is expecting a Service Option Response Order.  SO_REQs is set to a special value NULL when the mobile station is not expecting a Service Option Response Order.

###### 6.6.4.1.2.2 Requirements

6.6.4.1.2.2.1 Processing the Service Option Request Order

When the mobile station receives a Service Option Request Order, it shall perform the following:

•	If the mobile station supports and accepts the requested service option, the mobile station shall set SO_REQs to NULL and send a Service Option Response Order accepting the requested service option within the time limit required by the requested service option.  The mobile station shall begin using the requested service option in accordance with the requirements for the requested service option.

•	If the mobile station supports the requested service option and has an alternative service option to request, the mobile station shall set SO_REQs to the alternative service option number and send a Service Option Request Order requesting the alternative service option within the time limit required by the service option specified in the received order.

•	If the mobile station supports the requested service option, does not accept the requesed service option, and does not have an alternative service option to request, the mobile station shall set SO_REQs to NULL and send a Service Option Response Order to reject the request within the time limit required by the requested service option.

•	If the mobile station does not support the requested service option and has an alternative service option to request, the mobile station shall set SO_REQs to the alternative service option number and send a Service Option Request Order to request the alternative service option within T56m seconds.

•	If the mobile station does not support the requested service option and does not have an alternative service option to request, the mobile station shall set SO_REQs to NULL and send a Service Option Response Order to reject the request within T56m seconds.

6.6.4.1.2.2.2 Processing the Service Option Response Order

When the mobile station receives a Service Option Response Order, it shall perform the following:

•	If the service option number specified in the order is equal to SO_REQs, the mobile station shall set SO_REQs to NULL and begin using the specified service option in accordance with the requirements for the service option.

•	If the order indicates a service option rejection, the mobile station shall set SO_REQs to NULL.

•	If the order does not indicate a service option rejection and the service option specified in the order is not equal to SO_REQs, the mobile station shall set SO_REQs to NULL and send a Mobile Station Reject Order (ORDQ = ‘00000100’) within T56m seconds.

6.6.4.1.2.2.3 Service Option Request Initialization

The mobile station shall set SO_REQs to the selected service option number.

##### 6.6.4.1.3 Acknowledgement Procedures

The acknowledgement procedures facilitate the reliable exchange of messages between the base station and the mobile station.  The mobile station uses the fields ACK_SEQ xe "ACK_SEQ "§(acknowledgement sequence number), MSG_SEQxe "MSG_SEQ"§ (message sequence number) and ACK_REQ xe "ACK_REQ "§(acknowledgement required indicator) to detect duplicate messages and provide a reference for acknowledgements.  These message fields are referred to as layer 2 fields, and the acknowledgement procedures are referred to as layer 2 procedures.  All other message fields are referred to as layer 3 fields, and the processing of layer 3 fields is referred to as layer 3 processing.  (See Appendix C for further discussion of layering.)

On both the Forward Traffic Channel and the Reverse Traffic Channel, the procedure for messages requiring acknowledgement is a selective repeat scheme in which a message is retransmitted only if an acknowledgement for it is not received.

###### 6.6.4.1.3.1 Messages Requiring Acknowledgement

A Traffic Channel message requires acknowledgement when the ACK_REQ field is set to ‘1’.

6.6.4.1.3.1.1 Transmitting a Message and Receiving an Acknowledgement

If the mobile station requires that the base station receive one message before another message, the mobile station shall wait for an acknowledgement to the first message before transmitting the second.  Otherwise, the mobile station may transmit up to four different messages requiring acknowledgement before receiving an acknowledgement for the first message.

The mobile station shall store the next message sequence number to be transmitted for messages requiring acknowledgement (MSG_SEQ_ACKs).  The mobile station shall store an acknowledgement status indicator (ACK_WAITINGs[n], where n is 0 through 7) for each possible value of the Reverse Traffic Channel message MSG_SEQ field.  The mobile station shall not send a new message requiring acknowledgement when ACK_WAITINGs[(MSG_SEQ_ACKs + 4) mod 8] is equal to YES.

The mobile station shall perform the following procedures:

•	When the mobile station receives any message on the Forward Traffic Channel, it shall set ACK_WAITINGs[ACK_SEQr] to NO.

•	When the mobile station sends a new message requiring acknowledgement on the Reverse Traffic Channel, it shall set ACK_WAITINGs[MSG_SEQ_ACKs] to YES and shall set the MSG_SEQ field of the message to MSG_SEQ_ACKs.  The mobile station shall then increment MSG_SEQ_ACKs, modulo 8.

The mobile station shall not retransmit a message for which it has received an acknowledgement.

If the mobile station has not received an acknowledgement within T1m seconds after transmitting the message, the mobile station shall retransmit the message (see Figure 6.6.4.1.3.1.1-1).  If the mobile station retransmits a message, the mobile station shall use the same MSG_SEQxe "MSG_SEQ"§ number for the retransmission.  The mobile station shall not retransmit a message sooner than T1m seconds after the previous transmission.

The mobile station shall store a retransmission counter (RETRY_COUNTs) for each transmitted message requiring acknowledgement.  The mobile station shall set RETRY_COUNTs to zero prior to the first transmission of the message.  After each transmission of the message, the mobile station shall increment RETRY_COUNTs if no acknowledgement is received.  When RETRY_COUNTs is equal to N1m, the mobile station shall declare an acknowledgement failure.

tc "Figure 6.6.4.1.3.1.1-1.  Time Limit for Acknowledgement"\c §Figure 6.6.4.1.3.1.1-1.  Time Limit for Acknowledgement

6.6.4.1.3.1.2 Receiving a Message and Returning an Acknowledgement

Messages received on the Forward Traffic Channel contain MSG_SEQ fields that are incremented by the same rules as messages transmitted on the Reverse Traffic Channel.  Separate sequence numbers are maintained for Forward Traffic Channel Messages that require acknowledgement and for messages that do not require acknowledgement.

The mobile station acknowledges a received message by transmitting a message with the ACK_SEQ field set equal to the MSG_SEQ field of the received message.  A message transmitted with the ACK_SEQ field set in this manner is referred to as including an acknowledgement of the received message.

Whenever a message requiring acknowledgement is received, the mobile station shall set the ACK_SEQ field of subsequent Reverse Traffic Channel messages to MSG_SEQr.

After receiving a message requiring acknowledgement,xe "message requiring acknowledgement"§ the mobile station shall transmit a message including an acknowledgement within T2mxe "T2m"§ seconds as shown in Figure 6.6.4.1.3.1.2-1.

When a received message requires acknowledgement and no message is available within T2m seconds after the message is received, the mobile station shall transmit a Mobile Station Acknowledgement Order including the acknowledgement.  The Mobile Station Acknowledgement Order shall be sent as a message not requiring acknowledgement.

tc "Figure 6.6.4.1.3.1.2-1.  Time Limit for Acknowledgement"\c §Figure 6.6.4.1.3.1.2-1.  Time Limit for Acknowledgement

For duplicate message detection, the mobile station shall store a received status indicator for each possible value of the Forward Traffic Channel message MSG_SEQ field (MSG_SEQ_RCVDs[n], where n is 0 through 7).  The mobile station shall perform the following procedures:

•	When a message requiring acknowledgement is received and MSG_SEQ_RCVDs[MSG_SEQr] is equal to NO, the mobile station shall process the message as a new message.  The mobile station shall then set MSG_SEQ_RCVDs[MSG_SEQr] to YES, and shall set MSG_SEQ_RCVDs[(MSG_SEQr + 4) mod 8] to NO.

•	When a message requiring acknowledgement is received and MSG_SEQ_RCVDs[MSG_SEQr] is equal to YES, the mobile station shall acknowledge the message but shall not perform any further processing of the message.

###### 6.6.4.1.3.2 Messages Not Requiring Acknowledgement

A Traffic Channel message does not require acknowledgement when the ACK_REQ field is set to ‘0’.

The mobile station shall store the next message sequence number to be transmitted for messages not requiring acknowledgement (MSG_SEQ_NOACKs).  For each new message sent that does not require acknowledgement, the mobile station shall set the MSG_SEQ field of the message to MSG_SEQ_NOACKs and shall then increment MSG_SEQ_NOACKs, modulo 8.

The mobile station shall consider all messages received within T3mxe "T3m"§ seconds that do not require acknowledgement and have the same MSG_SEQxe "MSG_SEQ"§ number to be duplicates, as shown in Figure 6.6.4.1.3.2-1.  If the mobile station receives multiple copies of a message as determined by the MSG_SEQxe "MSG_SEQ"§ number, it shall discard the duplicate copies.

tc "Figure 6.6.4.1.3.2-1.  Time Window for Detecting Duplicate Messages not Requiring Acknowledgement"\c §Figure 6.6.4.1.3.2-1.  Time Window for Detecting Duplicate Messages not Requiring Acknowledgement

###### 6.6.4.1.3.3 Acknowledgement Procedures Reset

The mobile station shall reset the acknowledgement procedures as follows:

•	Message sequence number reset.  The mobile station shall set MSG_SEQ_ACKs to 0, MSG_SEQ_NOACKs to 0, and shall set ACK_WAITINGs[n] to NO for all values of n from 0 to 7.

•	Retransmission counter reset.  The mobile station shall set the retransmission counter (RETRY_COUNTs) to zero.

•	Acknowledgement sequence number reset.  The mobile station shall set the ACK_SEQ field of all Reverse Traffic Channel messages to 7 until the first message requiring acknowledgement is received.

•	Duplicate detection reset.  The mobile station shall set MSG_SEQ_RCVDs[n] to NO for all values of n from 0 to 7.

##### 6.6.4.1.4 Processing the In-Traffic System Parameters Message

The mobile station shall store the following parameters from the In-Traffic System Parameters Message:

•	System identification (SIDs = SIDr)

•	Network identification (NIDs = NIDr)

•	xe "Search window size for the Active Set and the Candidate Set"§Search window size for the Active Set and the Candidate Set (xe "SRCH_WIN_A"§SRCH_WIN_As = SRCH_WIN_Ar)

•	xe "Search window size for the Neighbor Set"§Search window size for the Neighbor Set (xe "SRCH_WIN_N"§SRCH_WIN_Ns = SRCH_WIN_Nr)

•	xe "Search window size for the Remaining Set (.i.SRCH_WIN_R"§Search window size for the Remaining Set (SRCH_WIN_Rs = SRCH_WIN_Rr)

•	xe "Pilot detection threshold"§Pilot detection threshold (T_ADDs = T_ADDr)

•	xe "Pilot drop threshold .i.T_DROP"§Pilot drop threshold T_DROPs = T_DROPr)

•	xe "Active Set versus Candidate Set comparison threshold (.i.T_COMP"§Active Set versus Candidate Set comparison threshold (T_COMPs = T_COMPr)

•	xe "Drop timer value"§Drop timer value (xe "T_TDROP"§T_TDROPs = T_TDROPr)

•	xe "Maximum age for retention of .i.Neighbor Set"§Maximum age for retention of Neighbor Set members ;(xe "NGHBR_MAX_AGE"§NGHBR_MAX_AGEs = NGHBR_MAX_AGEr)

##### 6.6.4.1.5 Message Action Times

A message without a USE_TIME field or with a USE_TIME field set to ‘0’ has an implicit action time.  A message whose USE_TIME field is set to ‘1’ has an explicit action time which is specified in the ACTION_TIME field of the message.  A message with a future action time is called a pending message.

Unless otherwise specified, a message having an implicit action time shall take effect as soon as it is processed, and a message with an explicit action time shall take effect when System Time (in 80 ms units) modulo 64 becomes equal to the message’s ACTION_TIME.

The mobile station shall support one pending message at any given time, not including pending Service Option Control Orders.  The number of pending Service Option Control Orders that the mobile station is required to support is specific to the service option (see Appendix A).

#### 6.6.4.2 Traffic Channel Initialization Substate xe "mobile station, Mobile Station Control on the Traffic Channel State, Traffic Channel Initialization Substate"§

In this substate, the mobile station verifies that it can receive the Forward Traffic Channel and begins transmitting on the Reverse Traffic Channel.

Upon entering the Traffic Channel Initialization Substate, the mobile station shall perform the following:

•	The mobile station shall perform registration initialization as specified in 6.6.5.6.4.1.

•	The mobile station shall reset the acknowledgement procedures as specified in 6.6.4.1.3.3.

•	The mobile station shall initialize Forward Traffic Channel power control as specified in 6.6.4.1.1.1.

•	If the Origination Message requests a special service option, the mobile station shall perform service option request initialization as specified in 6.6.4.1.2.2.3 using the requested service option number; otherwise, the mobile station shall perform service option request initialization using NULL.

While in the Traffic Channel Initialization Substate, the mobile station shall perform the following:

•	The mobile station shall perform pilot strength measurements as specified in 6.6.6.2.2, but shall not send Pilot Strength Measurement Messages.

•	The mobile station shall perform registration timer maintenance as specified in 6.6.5.6.4.2.

If the mobile station does not support the assigned CDMA Channel (see 6.2.1.1) or the assigned Forward Traffic code channel, the mobile station shall enter the System Determination Substate of the Mobile Station Initialization State.

If the mobile station supports the assigned CDMA Channel and the assigned Forward Traffic code channel, the mobile station shall perform the following:

•	The mobile station shall tune to the assigned CDMA Channel.

•	The mobile station shall set its code channel for the assigned Forward Traffic code channel.

•	The mobile station shall set its Forward and Reverse Traffic Channel frame offsets to the assigned frame offset.

•	The mobile station shall set its Forward and Reverse Traffic Channel long code masks to the public long code mask (see 6.1.3.1.8).

If the mobile station does not receive N5m consecutive good frames within T50m seconds, the mobile station shall enter the System Determination Substate of the Mobile Station Initialization State.

If the mobile station receives N5m consecutive good frames within T50m seconds, the mobile station shall perform the following additional functions while it remains in the Traffic Channel Initialization Substate:

•	The mobile station shall perform Forward Traffic Channel supervision as specified in 6.4.4.  If a loss of the Forward Traffic Channel is declared, the mobile station shall enter the System Determination Substate of the Mobile Station Initialization State.

•	The mobile station shall adjust its transmit power as specified in 6.1.2.3.2.

•	The mobile station shall transmit the Traffic Channel preamble as specified in 6.1.3.3.10.3.

•	The mobile station shall perform the acknowledgement procedures as specified in 6.6.4.1.3.  If an acknowledgement failure is declared, the mobile station shall disable its transmitter and enter the System Determination Substate of the Mobile Station Initialization State.

If the mobile station does not receive a Base Station Acknowledgement Order within T51m seconds, the mobile station shall disable its transmitter and enter the System Determination Substate of the Mobile Station Initialization State.

If the mobile station receives a Base Station Acknowledgement Order within T51m seconds, the mobile station shall perform the following:

•	If the call is mobile station terminated, the mobile station shall enter the Waiting for Order Substate.

•	If the call is mobile station originated, the mobile station shall enter the Conversation Substate.

#### 6.6.4.3 Alerting

##### 6.6.4.3.1 Waiting for Order Substate xe "mobile station, Mobile Station Control on the Traffic Channel State, Waiting for Order Substate"§

In this substate, the mobile station waits for an Alert With Information Message.

Upon entering the Waiting for Order Substate, the mobile station shall set the substate timer for xe "T52m"§T52m seconds.

While in the Waiting for Order Substate, the mobile station shall perform the following:

•	If the substate timer expires, the mobile station shall disable its transmitter and enter the xe "System Determination Substate "§System Determination Substate of the xe "Mobile Station Initialization State"§Mobile Station Initialization State.

•	The mobile station shall perform Forward Traffic Channel supervision as specified in 6.4.4.  If a loss of the Forward Traffic Channel is declared, the mobile station shall enter the System Determination Substate of the Mobile Station Initialization State.

•	The mobile station shall adjust its transmit power as specified in 6.1.2.3.2.

•	The mobile station shall perform Forward Traffic Channel power control as specified in 6.6.4.1.1.

•	The mobile station shall perform handoff processing as specified in 6.6.6.

•	The mobile station shall transfer the received primary traffic packets to the primary traffic service option application.

•	The mobile station shall transmit null Traffic Channel data on the Reverse Traffic Channel (see 6.1.3.3.10.4).

•	The mobile station shall perform registration timer maintenance as specified in 6.6.5.6.4.2.

•	If the mobile station is directed by the user to transmit a message, the mobile station shall send a Data Burst Message.

•	If the mobile station is directed by the user to request a service option, the mobile station shall perform service option request initialization as specified in 6.6.4.1.2.2.3, and shall send a Service Option Request Order (ORDQ = requested service option number).

•	If the mobile station is directed by the user to request a private long code transition and has the long code mask (see 6.3.12.3), the mobile station shall send a Long Code Transition Request Order (ORDQ = ‘00000001’) as a message requiring acknowledgement.

•	If the mobile station is directed by the user to request a public long code transition, the mobile station shall send a Long Code Transition Request Order (ORDQ = ‘00000000’) as a message requiring acknowledgement.

•	If the mobile station is directed by the user to operate in analog mode, the mobile station shall send the Request Analog Service Order as a message requiring acknowledgement.

•	If the mobile station is directed by the user to power down, the mobile station shall enter the Release Substate with a power-down indication (see 6.6.4.5).

•	The mobile station shall perform the acknowledgement procedures as specified in 6.6.4.1.3.  If an acknowledgement failure is declared, the mobile station shall disable its transmitter and enter the System Determination Substate of the Mobile Station Initialization State.

•	If the mobile station receives a message which is included in the following list and every message field value is within its permissible range, the mobile station shall process the message as described below and in accordance with the message’s action time (see 6.6.4.1.5).

1.	xe "Alert With Information Message"§Alert With Information Message:  The mobile station should alert the user and shall enter the xe "Waiting for Mobile Station Answer Substate"§Waiting for Mobile Station Answer Substate (see 6.6.4.3.2).

2.	xe "Analog Handoff Direction Message"§Analog Handoff Direction Message:  The mobile station shall process the message as specified in 6.6.6.2.9, and enter the Waiting For Order Task (see 2.6.4.3.1) with a handoff from CDMA indication.

3.	xe "Authentication Challenge Message"§Authentication Challenge Message:  The mobile station shall reset the substate timer for T52m seconds.  The mobile station shall then process the message and respond as specified in 6.3.12.1.5 within T32m seconds.

4.	xe "Base Station Acknowledgement Order"§Base Station Acknowledgement Order:  No requirements.

5.	xe "Base Station Challenge Confirmation Order"§Base Station Challenge Confirmation Order:  The mobile station shall reset the substate timer for T52m seconds.  The mobile station shall then process the message and respond with an SSD Update Confirmation Order or SSD Update Rejection Order as specified in 6.3.12.1.9 within T32m seconds.

6.	DData Data Burst Message;:  No requirements.

7.	xe "Handoff Direction Message"§Handoff Direction Message:  The mobile station shall process the message as specified in 6.6.6.2.5.1.  The mobile station shall reset the substate timer for T52m seconds.

8.	xe "In-Traffic System Parameters Message"§In-Traffic System Parameters Message:  The mobile station shall process the message as specified in 6.6.4.1.4.

9.	xe "Local Control Order"§Local Control Order:  No requirements.

10.	xe "Lock Order"§Lock Order:  The mobile station shall disable its transmitter and record the reason for the Lock Order in the mobile station’s semi-permanent memory (LCKRSNs-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

11.	xe "Lock Until Power-Cycled Order"§Lock Until Power-Cycled Order:  The mobile station shall disable its transmitter and record the reason for the Lock Until Power-Cycled Order in the mobile station’s semi-permanent memory (LCKRSN_Ps-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until after the next mobile station power-up or until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

12.	xe "Long Code Transition Request Order"§Long Code Transition Request Order:  If the Long Code Transition Request Order requests a private long code transition, the mobile station is able to generate the private long code (see 6.3.12.3), and the mobile station accepts the request, the mobile station shall send a Long Code Transition Response Order (ORDQ = ‘00000011’) within T56m seconds.  The mobile station shall use the private long code on both the xe "Forward Traffic Channel"§Forward Traffic Channel and the Reverse Traffic Channel.  If xe "USE_TIME"§USE_TIME equals ‘0’, the mobile station shall begin using the private xe "long code"§long code at the first 80 ms boundary (relative to the start of xe "System Time"§System Time) after N4m frames from the end of the response transmission.  The mobile station should turn on the voice privacy indicator.  If the Long Code Transition Request Order requests a private long code transition, and the mobile station is not able to generate the private long code or the mobile station does not accept the request, the mobile station shall send a Long Code Transition Response Order (ORDQ = ‘00000010’) within T56m seconds.

If the Long Code Transition Request Order requests a public long code transition and the mobile station accepts the request, the mobile station shall send a Long Code Transition Response Order (ORDQ = ‘00000010’) within T56m seconds.  The mobile station should turn off the voice privacy indicator.  If the Long Code Transition Request Order requests a public long code transition, and the mobile station does not accept the request, the mobile station shall send a Long Code Transition Response Order (ORDQ = ‘00000011’) within T56m seconds.

13.	xe "Maintenance Order"§Maintenance Order:  The mobile station shall enter the xe "Waiting for Mobile Station Answer Substate"§Waiting for Mobile Station Answer Substate.

14.	xe "Message Encryption Mode Order"§Message Encryption Mode Order:  The mobile station shall process the message as specified in 6.3.12.2.

15.	xe "Message Waiting Order"§Message Waiting Order:  No requirements.

16.	xe "Mobile Station Registered Message:  The mobile station shall process the message as specified in 6.6.5.6.4.3."§Mobile Station Registered Message  The mobile station shall process the message as specified in 6.6.5.6.4.3.

17.	xe "Neighbor List Update Message"§Neighbor List Update Message:  The mobile station shall process the message as specified in 6.6.6.2.6.3.

18.	xe "Pilot Measurement Request Order"§Pilot Measurement Request Order:  The mobile station shall process the order as specified in 6.6.6.2.5.1.

19.	xe "Power Control Parameters Message"§Power Control Parameters Message:  The mobile station shall process the message as specified in 6.6.4.1.1.2.

20.	xe "Release Order"§Release Order:  The mobile station shall enter the xe "Release Substate"§Release Substate with a base station release indication; (see 6.6.4.5).

21.	xe "Retrieve Parameters Message"§Retrieve Parameters Message:  If the mobile station can supply values for all of the parameters specified by the PARAMETER_ID ;fields in the message, the mobile station shall send, within T56m seconds, a Parameters Response Message; otherwise, the mobile station shall send, within T56m seconds, a Mobile Station Reject Order;.

22.	xe "Service Option Control Order"§Service Option Control Order:  The mobile station shall transfer the Service Option Control Order to the primary traffic service option application.

23.	xe "Service Option Request Order"§Service Option Request Order:  The mobile station shall process the message as specified in 6.6.4.1.2.2.1.

24.	xe "Service Option Response Order"§Service Option Response Order:  The mobile station shall process the message as specified in 6.6.4.1.2.2.2.

25.	xe "Set Parameters Message"§Set Parameters Message:  If the mobile station can set all of the parameters specified by the PARAMETER_ID ;fields in the message, the mobile station shall set them; otherwise, the mobile station shall send, within T56m seconds, a Mobile Station Reject Order;.

26.	xe "SSD Update Message"§SSD Update Message:  The mobile station shall reset the substate timer for T52m seconds.  The mobile station shall then process the message and respond with a Base Station Challenge Order as specified in 6.3.12.1.9 within T32m seconds.

27.	xe "Status Request Order"§Status Request Order:  The mobile station shall send, within T56m seconds, a xe "Status Message"§Status Message to the base station.

•	If the mobile station receives a message which is included in the above list and any message field value is not within its permissible range, the mobile station shall discard the message and send a Mobile Station Reject Order (ORDQ = ‘00000100’) within T56m seconds.

•	If the mobile station receives a message which is not included in the above list, the mobile station shall send the Mobile Station Reject Order (ORDQ = ‘00000010’) within T56m seconds.

##### 6.6.4.3.2 Waiting for Mobile Station Answer Substate xe "mobile station, Mobile Station Control on the Traffic Channel State, Waiting for Mobile Station Answer Substate"§

In this substate, the mobile station waits for the user to answer the xe "mobile station terminated call"§mobile station terminated call.

Upon entering the Waiting for Mobile Station Answer Substate , the mobile station shall set the substate timer for T53m seconds.

While in the Waiting for Mobile Station Answer Substate, the mobile station shall perform the following:

•	If the substate timer expires, the mobile station shall disable its transmitter and enter the xe "System Determination Substate "§System Determination Substate of the xe "Mobile Station Initialization State"§Mobile Station Initialization State.

•	The mobile station shall perform Forward Traffic Channel supervision as specified in 6.4.4.  If a loss of the Forward Traffic Channel is declared, the mobile station shall enter the System Determination Substate of the Mobile Station Initialization State.

•	The mobile station shall adjust its transmit power as specified in 6.1.2.3.2.

•	The mobile station shall perform Forward Traffic Channel power control as specified in 6.6.4.1.1.

•	The mobile station shall perform handoff processing as specified in 6.6.6.

•	The mobile station shall transfer the received primary traffic packets to the primary traffic service option application.

•	The mobile station shall transmit null Traffic Channel data on the Reverse Traffic Channel (see 6.1.3.3.10.4).

•	The mobile station shall perform registration timer maintenance as specified in 6.6.5.6.4.2.

•	If the mobile station is directed by the user to answer the call, the mobile station shall send a xe "Connect Order"§Connect Order to the base station as a xe "message requiring acknowledgement"§message requiring acknowledgement.  The mobile station shall enter the xe "Conversation Substate"§Conversation Substate.

•	If the mobile station is directed by the user to transmit a message, the mobile station shall send a Data Burst Message.

•	If the mobile station is directed by the user to request a service option, the mobile station shall perform service option request initialization as specified in 6.6.4.1.2.2.3, and shall send a Service Option Request Order (ORDQ = requested service option number).

•	If the mobile station is directed by the user to request a private long code transition and has the long code mask (see 6.3.12.3), the mobile station shall send a Long Code Transition Request Order (ORDQ = ‘00000001’) as a message requiring acknowledgement.

•	If the mobile station is directed by the user to request a public long code transition, the mobile station shall send a Long Code Transition Request Order (ORDQ = ‘00000000’) as a message requiring acknowledgement.

•	If the mobile station is directed by the user to operate in analog mode, the mobile station shall send the Request Analog Service Order as a message requiring acknowledgement.

•	If the mobile station is directed by the user to power down, the mobile station shall enter the Release Substate with a power-down indication (see 6.6.4.5).

•	The mobile station shall perform the acknowledgement procedures as specified in 6.6.4.1.3.  If an acknowledgement failure is declared, the mobile station shall disable its transmitter and enter the System Determination Substate of the Mobile Station Initialization State.

•	If the mobile station receives a message which is included in the following list and every message field value is within its permissible range, the mobile station shall process the message as described below and in accordance with the message’s action time (see 6.6.4.1.5).

1.	xe "Alert With Information Message"§Alert With Information Message:  The mobile station shall reset the substate timer for T53m seconds.

2.	xe "Analog Handoff Direction Message"§Analog Handoff Direction Message:  The mobile station shall process the message as specified in 6.6.6.2.9 and enter the Waiting For Answer Task (see 2.6.4.3.2) with a handoff from CDMA indication.

3.	xe "Authentication Challenge Message"§Authentication Challenge Message:  The mobile station shall process the message and respond as specified in 6.3.12.1.5 within T32m seconds.

4.	xe "Base Station Acknowledgement Order"§Base Station Acknowledgement Order:  No requirements.

5.	xe "Base Station Challenge Confirmation Order"§Base Station Challenge Confirmation Order:  The mobile station shall process the message and respond with an SSD Update Confirmation Order or SSD Update Rejection Order as specified in 6.3.12.1.9 within T32m seconds.

6.	xe ".i.Data Burst Message"§Data Burst Message:  No requirements.

7.	xe "Handoff Direction Message"§Handoff Direction Message:  The mobile station shall process the message as specified in 6.6.6.2.5.1.

8.	xe "In-Traffic System Parameters Message"§In-Traffic System Parameters Message:  The mobile station shall process the message as specified in 6.6.4.1.4.

9.	xe "Local Control Order"§Local Control Order:  No requirements.

10.	xe "Lock Order"§Lock Order:  The mobile station shall disable its transmitter and record the reason for the Lock Order in the mobile station’s semi-permanent memory (LCKRSNs-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

11.	xe "Lock Until Power-Cycled Order"§Lock Until Power-Cycled Order:  The mobile station shall disable its transmitter and record the reason for the Lock Until Power-Cycled Order in the mobile station’s semi-permanent memory (LCKRSN_Ps-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until after the next mobile station power-up or until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

12.	xe "Long Code Transition Request Order"§Long Code Transition Request Order:  If the Long Code Transition Request Order requests a private long code transition, the mobile station is able to generate the private long code (see 6.3.12.3), and the mobile station accepts the request, the mobile station shall send a Long Code Transition Response Order (ORDQ = ‘00000011’) within T56m seconds.  The mobile station shall use the private long code on both the xe "Forward Traffic Channel"§Forward Traffic Channel and the Reverse Traffic Channel.  If xe "USE_TIME"§USE_TIME equals ‘0’, the mobile station shall begin using the private xe "long code"§long code at the first 80 ms boundary (relative to the start of xe "System Time"§System Time) after N4m frames from the end of the response transmission.  The mobile station should turn on the voice privacy indicator.  If the Long Code Transition Request Order requests a private long code transition, and the mobile station is not able to generate the private long code or the mobile station does not accept the request, the mobile station shall send a Long Code Transition Response Order (ORDQ = ‘00000010’) within T56m seconds.

If the Long Code Transition Request Order requests a public long code transition and the mobile station accepts the request, the mobile station shall send a Long Code Transition Response Order (ORDQ = ‘00000010’) within T56m seconds.  The mobile station should turn off the voice privacy indicator.  If the Long Code Transition Request Order requests a public long code transition, and the mobile station does not accept the request, the mobile station shall send a Long Code Transition Response Order (ORDQ = ‘00000011’) within T56m seconds.

13.	xe "Maintenance Order"§Maintenance Order:  The mobile station shall reset the substate timer for T53m seconds.

14.	xe "Message Encryption Mode Order"§Message Encryption Mode Order:  The mobile station shall process the message as specified in 6.3.12.2.

15.	xe "Message Waiting Order"§Message Waiting Order:  No requirements.

16.	xe "Mobile Station Registered Message:  The mobile station shall process the message as specified in 6.6.5.6.4.3."§Mobile Station Registered Message  The mobile station shall process the message as specified in 6.6.5.6.4.3.

17.	xe "Neighbor List Update Message"§Neighbor List Update Message:  The mobile station shall process the message as specified in 6.6.6.2.6.3.

18.	xe "Pilot Measurement Request Order"§Pilot Measurement Request Order:  The mobile station shall process the order as specified in 6.6.6.2.5.1.

19.	xe "Power Control Parameters Message"§Power Control Parameters Message:  The mobile station shall process the message as specified in 6.6.4.1.1.2.

20.	xe "Release Order"§Release Order:  The mobile station shall enter the xe "Release Substate"§Release Substate with a base station release indication; (see 6.6.4.5).

21.	xe "Retrieve Parameters Message"§Retrieve Parameters Message:  If the mobile station can supply values for all of the parameters specified by the PARAMETER_ID ;fields in the message, the mobile station shall send, within T56m seconds, a Parameters Response Message; otherwise, the mobile station shall send, within T56m seconds, a Mobile Station Reject Order;.

22.	xe "Service Option Control Order"§Service Option Control Order:  The mobile station shall transfer the Service Option Control Order to the primary traffic service option application.

23.	xe "Service Option Request Order"§Service Option Request Order:  The mobile station shall process the message as specified in 6.6.4.1.2.2.1.

24.	xe "Service Option Response Order"§Service Option Response Order:  The mobile station shall process the message as specified in 6.6.4.1.2.2.2.

25.	xe "Set Parameters Message"§Set Parameters Message:  If the mobile station can set all of the parameters specified by the PARAMETER_ID ;fields in the message, the mobile station shall set them; otherwise, the mobile station shall send, within T56m seconds, a Mobile Station Reject Order;.

26.	xe "SSD Update Message"§SSD Update Message:  The mobile station shall process the message and respond with a Base Station Challenge Order as specified in 6.3.12.1.9 within T32m seconds.

27.	xe "Status Request Order"§Status Request Order:  The mobile station shall send, within T56m seconds, a xe "Status Message"§Status Message.

•	If the mobile station receives a message which is included in the above list and any message field value is not within its permissible range, the mobile station shall discard the message and send a Mobile Station Reject Order (ORDQ = ‘00000100’) within T56m seconds.

•	If the mobile station receives a message which is not included in the above list, the mobile station shall send the Mobile Station Reject Order (ORDQ = ‘00000010’) within T56m seconds.

#### 6.6.4.4 Conversation Substate xe "mobile station, Mobile Station Control on the Traffic Channel State, Conversation Substate"§

In this substate, the mobile station’s primary traffic service option application exchanges primary traffic packets with the base station.

While in the Conversation Substate, the mobile station shall perform the following:

•	The mobile station shall perform Forward Traffic Channel supervision as specified in 6.4.4.  If a loss of the Forward Traffic Channel is declared, the mobile station shall enter the System Determination Substate of the Mobile Station Initialization State.

•	The mobile station shall adjust its transmit power as specified in 6.1.2.3.2.

•	The mobile station shall perform Forward Traffic Channel power control as specified in 6.6.4.1.1.

•	The mobile station shall perform handoff processing as specified in 6.6.6.

•	The mobile station shall transfer the received primary traffic packets to the primary traffic service option application.

•	The mobile station shall transmit primary traffic packets from the primary traffic service option application.

•	The mobile station shall perform registration timer maintenance as specified in 6.6.5.6.4.2.

•	If the mobile station originated the call and did not send all the dialed digits in the xe "Origination Message"§Origination Message, the mobile station shall send the remaining dialed digits to the base station in the xe "Origination Continuation Message"§Origination Continuation Message.  The mobile station shall send the Origination Continuation Message as a xe "message requiring acknowledgement"§message requiring acknowledgement within T54m seconds after entering the xe "Conversation Substate"§Conversation Substate.

•	If the mobile station is directed by the user to transmit a message, the mobile station shall send a Data Burst Message.

•	If the mobile station is directed by the user to request a service option, the mobile station shall perform service option request initialization as specified in 6.6.4.1.2.2.3, and shall send a Service Option Request Order (ORDQ = requested service option number).

•	If the mobile station is directed by the user to request a private long code transition and has the long code mask (see 6.3.12.3), the mobile station shall send a Long Code Transition Request Order (ORDQ = ‘00000001’) as a message requiring acknowledgement.

•	If the mobile station is directed by the user to request a public long code transition, the mobile station shall send a Long Code Transition Request Order (ORDQ = ‘00000000’) as a message requiring acknowledgement.

•	If the mobile station is directed by the user to issue a flash, the mobile station shall build a xe "Flash With Information Message"§Flash With Information Message with the collected digits and send the message to the base station as a xe "message requiring acknowledgement"§message requiring acknowledgement.

•	If the mobile station is directed by the user to send burst DTMF digits, the mobile station shall build the xe "Send Burst DTMF Message"§Send Burst DTMF Message with the dialed digits and send the message as a message requiring acknowledgement.

•	If the mobile station is directed by the user to send a continuous DTMF digit, the mobile station shall build the xe "Continuous DTMF Tone Order"§Continuous DTMF Tone Order with the dialed digit and send the order as a xe "message requiring acknowledgement"§message requiring acknowledgement.  When the mobile station is directed by the user to cease sending the continuous DTMF digit, the mobile station shall send the xe "Continuous DTMF Tone Order (ORDQ = ‘11111111’) as a message requiring acknowledgement."§Continuous DTMF Tone Order (ORDQ = 11111111) as a message requiring acknowledgement.

•	If the mobile station is directed by the user to operate in analog mode, the mobile station shall send the Request Analog Service Order as a message requiring acknowledgement.

•	If the mobile station is directed by the user to disconnect the call, the mobile station shall enter the Release Substate with a mobile station release indication (see 6.6.4.5).

•	If the mobile station is directed by the user to power down, the mobile station shall enter the Release Substate with a power-down indication (see 6.6.4.5).

•	The mobile station shall perform the acknowledgement procedures as specified in 6.6.4.1.3.  If an acknowledgement failure is declared, the mobile station shall disable its transmitter and enter the System Determination Substate of the Mobile Station Initialization State.

•	If the mobile station receives a message which is included in the following list and every message field value is within its permissible range, the mobile station shall process the message as described below and in accordance with the message’s action time (see 6.6.4.1.5).

1.	xe "Alert With Information Message"§Alert With Information Message:  If the message contains a signal information record with the SIGNAL_TYPE field set to ‘01’ or ‘10’, the mobile station shall enter the Waiting For Mobile Station Answer Substate.

2.	xe "Analog Handoff Direction Message"§Analog Handoff Direction Message:  The mobile station shall process the message as specified in 6.6.6.2.9 and enter the Conversation Task (see 2.6.4.4) with a handoff from CDMA indication.

3.	xe "Authentication Challenge Message"§Authentication Challenge Message:  The mobile station shall process the message and respond as specified in 6.3.12.1.5 within T32m seconds.

4.	xe "Base Station Acknowledgement Order"§Base Station Acknowledgement Order:  No requirements.

5.	xe "Base Station Challenge Confirmation Order"§Base Station Challenge Confirmation Order:  The mobile station shall process the message and respond with an SSD Update Confirmation Order or SSD Update Rejection Order as specified in 6.3.12.1.9 within T32m seconds.

6.	Continuous DTMF Tone Order;:  No requirements.

7.	xe ".i.Data Burst Message"§Data Burst Message:  No requirements.

8.	xe "Flash with Information Message"§Flash with Information Message:  No requirements.

9.	xe "Handoff Direction Message"§Handoff Direction Message:  The mobile station shall process the message as specified in 6.6.6.2.5.1.

10.	xe "In-Traffic System Parameters Message"§In-Traffic System Parameters Message:  The mobile station shall process the message as specified in 6.6.4.1.4.

11.	xe "Local Control Order"§Local Control Order:  No requirements.

12.	xe "Lock Order"§Lock Order:  The mobile station shall disable its transmitter and record the reason for the Lock Order in the mobile station’s semi-permanent memory (LCKRSNs-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

13.	xe "Lock Until Power-Cycled Order"§Lock Until Power-Cycled Order:  The mobile station shall disable its transmitter and record the reason for the Lock Until Power-Cycled Order in the mobile station’s semi-permanent memory (LCKRSN_Ps-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until after the next mobile station power-up or until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

14.	xe "Long Code Transition Request Order"§Long Code Transition Request Order:  If the Long Code Transition Request Order requests a private long code transition, the mobile station is able to generate the private long code (see 6.3.12.3), and the mobile station accepts the request, the mobile station shall send a Long Code Transition Response Order (ORDQ = ‘00000011’) within T56m seconds.  The mobile station shall use the private long code on both the xe "Forward Traffic Channel"§Forward Traffic Channel and the Reverse Traffic Channel.  If xe "USE_TIME"§USE_TIME equals ‘0’, the mobile station shall begin using the private xe "long code"§long code at the first 80 ms boundary (relative to the start of xe "System Time"§System Time) after N4m frames from the end of the response transmission.  The mobile station should turn on the voice privacy indicator.  If the Long Code Transition Request Order requests a private long code transition, and the mobile station is not able to generate the private long code or the mobile station does not accept the request, the mobile station shall send a Long Code Transition Response Order (ORDQ = ‘00000010’) within T56m seconds.

If the Long Code Transition Request Order requests a public long code transition and the mobile station accepts the request, the mobile station shall send a Long Code Transition Response Order (ORDQ = ‘00000010’) within T56m seconds.  The mobile station should turn off the voice privacy indicator.  If the Long Code Transition Request Order requests a public long code transition, and the mobile station does not accept the request, the mobile station shall send a Long Code Transition Response Order (ORDQ = ‘00000011’) within T56m seconds.

15.	xe "Maintenance Order"§Maintenance Order:  The mobile station shall enter the Waiting for Mobile Station Answer Substate.

16.	xe "Message Encryption Mode Order"§Message Encryption Mode Order:  The mobile station shall process the message as specified in 6.3.12.2.

17.	xe "Message Waiting Order"§Message Waiting Order:  No requirements.

18.	xe "Mobile Station Registered Message:  The mobile station shall process the message as specified in 6.6.5.6.4.3."§Mobile Station Registered Message  The mobile station shall process the message as specified in 6.6.5.6.4.3.

19.	xe "Neighbor List Update Message"§Neighbor List Update Message:  The mobile station shall process the message as specified in 6.6.6.2.6.3.

20.	xe "Pilot Measurement Request Order"§Pilot Measurement Request Order:  The mobile station shall process the order as specified in 6.6.6.2.5.1.

21.	xe "Power Control Parameters Message"§Power Control Parameters Message:  The mobile station shall process the message as specified in 6.6.4.1.1.2.

22.	xe "Release Order"§Release Order:  The mobile station shall enter the xe "Release Substate"§Release Substate with a base station release indication; (see 6.6.4.5).

23.	xe "Retrieve Parameters Message"§Retrieve Parameters Message:  If the mobile station can supply values for all of the parameters specified by the PARAMETER_ID ;fields in the message, the mobile station shall send, within T56m seconds, a Parameters Response Message; otherwise, the mobile station shall send, within T56m seconds, a Mobile Station Reject Order;.

24.	xe "Send Burst DTMF Message"§Send Burst DTMF Message:  No requirements.

25.	xe "Service Option Control Order"§Service Option Control Order:  The mobile station shall transfer the Service Option Control Order to the primary traffic service option application.

26.	xe "Service Option Request Order"§Service Option Request Order:  The mobile station shall process the message as specified in 6.6.4.1.2.2.1.

27.	xe "Service Option Response Order"§Service Option Response Order:  The mobile station shall process the message as specified in 6.6.4.1.2.2.2.

28.	xe "Set Parameters Message"§Set Parameters Message:  If the mobile station can set all of the parameters specified by the PARAMETER_ID ;fields in the message, the mobile station shall set them; otherwise, the mobile station shall send, within T56m seconds, a Mobile Station Reject Order;.

29.	xe "SSD Update Message"§SSD Update Message:  The mobile station shall process the message and respond with a Base Station Challenge Order as specified in 6.3.12.1.9 within T32m seconds.

30.	xe "Status Request Order"§Status Request Order:  The mobile station shall send, within T56m seconds, a xe "Status Message"§Status Message.

•	If the mobile station receives a message which is included in the above list and any message field value is not within its permissible range, the mobile station shall discard the message and send a Mobile Station Reject Order (ORDQ = ‘00000100’) within T56m seconds.

•	If the mobile station receives a message which is not included in the above list, the mobile station shall send the Mobile Station Reject Order (ORDQ = ‘00000010’) within T56m seconds.

#### 6.6.4.5 Release Substate xe "mobile station, Mobile Station Control on the Traffic Channel State, Release Substate"§

In this substate, the mobile station confirms the call disconnect.

Upon entering the Release Substate, the mobile station shall perform the following:

•	The mobile station shall set the substate timer for xe "T55m"§T55m seconds.

•	If the mobile station enters the Release Substate with a power-down indication, the mobile station shall send a Release Order (ORDQ = ‘00000001’), and perform registration power-down (see 6.6.5.6.4.3).

•	If the mobile station enters the Release Substate with a mobile station release indication, the mobile station shall send a Release Order (ORDQ = ‘00000000’).

•	If the mobile station enters the Release Substate with a base station release indication, the mobile station shall send a Release Order (ORDQ = ‘00000000’).  The mobile station shall disable its transmitter and enter the System Determination Substate of the Mobile Station Initialization State.

While in the Release Substate, the mobile station shall perform the following:

•	If the substate timer expires, the mobile station shall disable its transmitter and enter the xe "System Determination Substate of the .i.Mobile Station Initialization State."§System Determination Substate of the Mobile Station Initialization State.

•	The mobile station shall perform Forward Traffic Channel supervision as specified in 6.4.4.  If a loss of the Forward Traffic Channel is declared, the mobile station shall enter the System Determination Substate of the Mobile Station Initialization State.

•	The mobile station shall adjust its transmit power as specified in 6.1.2.3.2.

•	The mobile station shall perform Forward Traffic Channel power control as specified in 6.6.4.1.1.

•	The mobile station shall perform handoff processing as specified in 6.6.6.

•	The mobile station shall transmit null Traffic Channel data on the Reverse Traffic Channel (see 6.1.3.3.10.4).

•	The mobile station shall perform registration timer maintenance as specified in 6.6.5.6.4.2.

•	The mobile station shall perform the acknowledgement procedures as specified in 6.6.4.1.3.  If an acknowledgement failure is declared, the mobile station shall disable its transmitter and enter the System Determination Substate of the Mobile Station Initialization State.

•	If the mobile station receives a message which is included in the following list and every message field value is within its permissible range, the mobile station shall process the message as described below and in accordance with the message’s action time (see 6.6.4.1.5).

1.	xe "Alert With Information Message"§Alert With Information Message:  The mobile shall enter the xe "Waiting for Mobile Station Answer Substate"§Waiting for Mobile Station Answer Substate.

2.	xe "Base Station Acknowledgement Order"§Base Station Acknowledgement Order:  No requirements.

3.	xe ".i.Data Burst Message"§Data Burst Message:  No requirements.

4.	xe "Handoff Direction Message"§Handoff Direction Message:  The mobile station shall process the message as specified in 6.6.6.2.5.1.

5.	xe "In-Traffic System Parameters Message"§In-Traffic System Parameters Message:  The mobile station shall process the message as specified in 6.6.4.1.4.

6.	xe "Local Control Order"§Local Control Order:  No requirements.

7.	xe "Lock Order"§Lock Order:  The mobile station shall disable its transmitter and record the reason for the Lock Order in the mobile station’s semi-permanent memory (LCKRSNs-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

8.	xe "Lock Until Power-Cycled Order"§Lock Until Power-Cycled Order:  The mobile station shall disable its transmitter and record the reason for the Lock Until Power-Cycled Order in the mobile station’s semi-permanent memory (LCKRSN_Ps-p = ORDQr).  The mobile station should notify the user of the locked condition.  The mobile station shall enter the System Determination Substate of the Mobile Station Initialization State, and shall not enter the System Access State again until after the next mobile station power-up or until it has received an Unlock Order.  This requirement shall take precedence over any other mobile station requirement specifying entry to the System Access State.

9.	xe "Message Waiting Order"§Message Waiting Order:  No requirements.

10.	xe "Mobile Station Registered Message:  The mobile station shall process the message as specified in 6.6.5.6.4.3."§Mobile Station Registered Message  The mobile station shall process the message as specified in 6.6.5.6.4.3.

11.	xe "Neighbor List Update Message"§Neighbor List Update Message:  The mobile station shall process the message as specified in 6.6.6.2.6.3.

12.	xe "Power Control Parameters Message"§Power Control Parameters Message:  The mobile station shall process the message as specified in 6.6.4.1.1.2.

13.	xe "Release Order"§Release Order:  The mobile station shall disable its transmitter.  If the mobile station enters the Release Substate with a power-down indication, the mobile station may power down; otherwise, the mobile station shall enter the System Determination Substate; of the xe "Mobile Station Initialization State"§Mobile Station Initialization State.

14.	xe "Retrieve Parameters Message"§Retrieve Parameters Message:  If the mobile station can supply values for all of the parameters specified by the PARAMETER_ID ;fields in the message, the mobile station shall send, within T56m seconds, a Parameters Response Message; otherwise, the mobile station shall send, within T56m seconds, a Mobile Station Reject Order;.

•	If the mobile station receives a message which is included in the above list and any message field value is not within its permissible range, the mobile station shall discard the message and send a Mobile Station Reject Order (ORDQ = ‘00000100’) within T56m seconds.

•	If the mobile station receives a message which is not included in the above list, the mobile station shall send the Mobile Station Reject Order (ORDQ = ‘00000010’) within T56m seconds.

### 6.6.5 Registration

#### 6.6.5.1 Forms of Registration

Registration is the process by which the mobile station notifies the base station of its location, status, identification, slot cycle, station class, and other characteristics.  The mobile station informs the base station of its location and status so that the base station can efficiently page the mobile station when establishing a mobile terminated call.  The mobile station supplies the base station with the set of xe "MIN"§MINs that are active.  This allows the base station to know which mobile station to contact when handling a mobile terminated call.  If operating in the slotted mode, the mobile station supplies the xe "SLOT_CYCLE_INDEX "§SLOT_CYCLE_INDEX parameter so that the base station knows which slots the mobile station is monitoring.  The mobile station supplies the station class mark and CAI revision number so that the base station knows the capabilities of the mobile station. xe "mobile station registration"§

The CDMA xe "system"§system supports nine different forms of registration: xe "registration, forms of"§

1.	xe "Power-up registration"§Power-up registration.  The mobile station registers when it powers on, switches from using the alternate serving system, or switches from using the analog system.

2.	xe "Power-down registration"§Power-down registration.  The mobile station registers when it powers off if previously registered in the current serving system.

3.	xe "Timer-based registration"§Timer-based registration.  The mobile station registers when a timer expires.

4.	xe "Distance-based registration"§Distance-based registration.  The mobile station registers when the distance between the current base station and the base station in which it last registered exceeds a threshold.

5.	xe "Zone-based registration"§Zone-based registration.  The mobile station registers when it enters a new zone.

6.	xe "Parameter-change registration"§Parameter-change registration.  The mobile station registers when certain of its stored parameters change.

7.	xe "Ordered registration"§Ordered registration.  The mobile station registers when the base station requests it.

8.	xe "Implicit registration"§Implicit registration.  When a mobile station successfully uses the Access Channel, the base station can infer the mobile station's location, causing an implicit registration.

9.	xe "Traffic Channel registration"§Traffic Channel registration.  Whenever the base station has registration information for a mobile station that has been assigned to a Traffic Channel, the base station can notify the mobile station that it is registered.

The first six forms are called xe "autonomous registrations "§autonomous registrations since the mobile station initiates the registration in response to an event, (i.e., without being explicitly directed to register by the base station).  The first five forms of registration, as a group, are called xe "class C registration"§class C registration and are enabled by roaming status for any MIN (see 6.6.5.3).  Autonomous parameter-change registration is independent of MIN roaming status.  Ordered registration is initiated by the base station through an xe "Order Message"§Order Message.  Implicit registration does not involve the exchange of any registration messages between the base station and the mobile station.  While a mobile station is assigned a Traffic Channel, the base station can obtain registration information by using the Status Request Order to obtain Status Messages from the mobile station.  The mobile station can be notified that it is registered through the Mobile Station Registered Message.

Any of the various forms of xe "autonomous registration"§autonomous registration can be enabled or disabled.  The forms of registration that are enabled and the corresponding xe "registration parameters "§registration parameters are communicated in the xe "System Parameters Message"§System Parameters Message.

In addition, the mobile station may enable or disable class C registration for each MIN for each type of roaming described in 6.6.5.3.

##### 6.6.5.1.1 Power-Up Registration xe "registration, power-up"§

xe "Power-up registration"§Power-up registration is performed when the mobile station is turned on.  So that a mobile station is not continually registering when power is quickly turned on and off, the mobile station delays T57m seconds before registering after entering the Mobile Station Idle State.

The mobile station shall maintain a power-up/initialization timer.  While the power-up/initialization timer is active, the mobile station shall not make registration access attempts.

When power-up registration is performed, the mobile station shall perform an initial registration (see 6.6.5.4).

##### 6.6.5.1.2 Power-Down Registration xe "registration, power-down"§

xe "Power-down registration"§Power-down registration is performed when the user directs the mobile station to power off.  If power-down registration is performed, the mobile station does not power down until after completing the registration attempt.

The mobile station shall not perform power-down registration if it has not previously registered with the current base station.

When power-down registration is performed, the mobile station shall perform a registration update (see 6.6.5.4).  The Registration Update Message shall not contain any added or deleted MINs.

##### 6.6.5.1.3 Timer-Based Registration xe "registration, timer-based"§

xe "Timer-based registration"§Timer-based registration causes the mobile station to register at regular intervals.  Its use also allows the system to automatically deregister mobile stations that did not perform a successful xe "power-down registration"§power-down registration.  Timer-based registration uses a Paging Channel slot counter (equivalent to a timer with time increments of 200 ms).  Timer-based registration is performed when the counter reaches a maximum value (REG_COUNT_MAXs) that is controlled by the base station via the REG_PRD field of the System Parameters Message.  The base station disables timer-based registration by setting REG_PRD to zero.

The counter is reset on power-up and when switching from analog or alternate serving systems.  The counter is also reset after each successful system access (implicit registration).

The mobile station shall maintain a timer-based registration counter (REG_COUNTs).  The mobile station shall compute and store the timer expiration count (REG_COUNT_MAXs) as

xe "REG_COUNT_MAX"§REG_COUNT_MAXs = Î2REG_PRD/4_.

The mobile station shall maintain an indicator of timer-based registration timer enable status (COUNTER_ENABLEDs).

Whenever the mobile station changes COUNTER_ENABLEDs from NO to YES, it shall set REG_COUNTs to a pseudorandom value between 0 and xe "REG_COUNT_MAX"§REG_COUNT_MAXs - 1, using the pseudorandom number generator specified in 6.6.7.2.

If the mobile station is operating in the non-slotted mode, it shall increment the timer-based registration counter once per 200 ms whenever COUNTER_ENABLEDs equals YES.  If the mobile station is operating in slotted mode, it may increment the timer-based registration counter when it begins to monitor the Paging Channel (see 6.6.2.1.1.3).  A mobile station operating in the slotted mode shall increment the counter by the same amount that the counter would have been incremented if the mobile station had been operating in the non-slotted mode.

##### 6.6.5.1.4 Distance-Based Registration xe "registration, distance-based"§

xe "Distance-based registration"§Distance-based registration causes a mobile station to register when the distance between the current base station and the base station in which it last registered exceeds a threshold.  The mobile station determines that it has moved a certain distance by computing a distance measure based on the difference in latitude and longitude between the current base station and the base station where the mobile station last registered.  If this distance measure exceeds the threshold value, the mobile station registers.

The mobile station stores the base station latitude (xe "BASE_LAT_REG"§BASE_LAT_REGs-p), the base station longitude (xe "BASE_LONG_REG"§BASE_LONG_REGs-p) and the registration distance (xe "REG_DIST_REG"§REG_DIST_REGs-p), of the base station whose Access Channel was used for the mobile station’s last registration (see 6.3.4).  The mobile station shall compute the current base station’s distance from the last registration point (DISTANCE) as:

DISTANCE = Î  _,

where

_lat = xe "BASE_LAT"§BASE_LATs - xe "BASE_LAT_REGs-p"§BASE_LAT_REGsp

_long = (xe "BASE_LONG"§BASE_LONGs - BASE_LONG_REGs-p) ¥ cos (_/180 ¥ BASE_LAT_REGs-p/14400).

The mobile station shall compute DISTANCE with an error of no more than ±5% of its true value when |xe "BASE_LAT_REGs-p"§BASE_LAT_REGsp/14400| is less than 60 and with an error of no more than ±7% of its true value when |BASE_LAT_REGs-p/14400| is between 60 and 70.

##### 6.6.5.1.5 Zone-Based Registration xe "registration, zone-based"§

Zones are groups of base stations within a given system and network.  A base station’s zone assignment is identified by the REG_ZONE field of the System Parameters Message.

xe "Zone-based registration"§Zone-based registration causes a mobile station to register whenever it moves into a new zone not on its internally stored list of visited registration zones.  A zone is added to the list whenever a registration (including implicit registration) occurs, and is deleted upon expiration of a timer.  After a system access, timers are enabled for every zone except one that was successfully registered by the access.  Timers are also enabled at the start of a call.

A mobile station can be registered in more than one zone.  Zones are uniquely identified by a zone number (REG_ZONE) plus the SID and NID of the zone.

The mobile station shall store a list of the zones in which the mobile station has registered (xe "ZONE_LISTs"§ZONE_LISTs).  Each entry in xe "ZONE_LISTs shall include the zone number (REG_ZONE) and the (SID, NID) pair for the zone.  The mobile station shall be capable of storing at least N9m entries in ZONE_LISTs.  A base station shall be considered to be in ZONE_LISTs only if the base station’s REG_ZONE, SID and NID are found in an entry in ZONE_LISTs.  The mobile station provides storage for one entry of ZONE_LISTs in semi-permanent memory, ZONE_LISTs-p (see 6.3.4)."§ZONE_LISTs shall include the zone number (REG_ZONE) and the (SID, NID) pair for the zone.  The mobile station shall be capable o

The mobile station shall maintain a zone list entry timer for each entry in ZONE_LISTs.  When an entry in ZONE_LISTs is removed from the list, the corresponding zone list entry timer shall be disabled.  The timer duration shall be as determined from the stored value of ZONE_TIMERs using Table 7.7.2.3.2.1-1.  The mobile station shall provide a means to examine each timer’s value while the timer is active, so that the age of list entries can be compared.

The base station controls the maximum number of zones in which a mobile station may be considered registered, by means of the TOTAL_ZONES field of the System Parameters Message.  When an entry is added to the zone list, or if TOTAL_ZONES is decreased, the mobile station removes entries from the zone list if there are more entries than allowed by the setting of TOTAL_ZONES.

Whenever ZONE_LISTs contains more than TOTAL_ZONESs entries, the mobile station shall delete the excess entries according to the following rules:

•	If TOTAL_ZONESs is equal to zero, the mobile station shall delete all entries.

•	If TOTAL_ZONESs is not equal to zero, the mobile station shall delete those entries having active zone list entry timers (excluding any such entry selected to be retained), starting with the oldest entry, as determined by the timer values, and continuing in order of decreasing age until no more than TOTAL_ZONESs entries remain.

The mobile station shall store a list of the systems/networks in which the mobile station has registered (xe "SID_NID_LISTs"§SID_NID_LISTs).  Each entry in xe "SID_NID_LISTs shall include the (SID, NID) pair for the system/network.  The mobile station shall be capable of storing at least N10m entries in SID_NID_LISTs.  A base station shall be considered to be in the SID_NID_LISTs only if the base station’s SID and NID are found in an entry in SID_NID_LISTs.  The mobile station shall provide storage for one entry of SID_NID_LISTs in semi-permanent memory (SID_NID_LISTs-p)."§SID_NID_LISTs shall include the (SID, NID) pair for the system/network.  The mobile station shall be capable of storing at least

The mobile station shall maintain a SID/NID list entry timer for each entry in SID_NID_LISTs.  When an entry in SID_NID_LISTs is removed from the list, the corresponding SID/NID list entry timer shall be disabled.  The timer duration shall be as determined from the stored value of ZONE_TIMERs using Table 7.7.2.3.2.1-1.  The mobile station shall provide a means to examine each timer’s value while the timer is active, so that the age of list entries can be compared.

Whenever SID_NID_LISTs contains more than N10m entries, the mobile station may delete the excess entries according to the following rule:

•	The mobile station shall delete those entries having active SID/NID list entry timers, starting with the oldest entry, as determined by the timer values, and continuing in order of decreasing age.

When zone-based registration is performed, the mobile station may perform a registration update (see 6.6.5.4) if SID_NID_LISTs contains an entry with the current base station’s SID and NID (SIDs and NIDs); otherwise, the mobile station shall perform an initial registration (see 6.6.5.4).

##### 6.6.5.1.6 Parameter-Change Registration xe "registration, parameter-change"§

Parameter-change registration is performed when a mobile station modifies any of the following stored parameters:

•	The preferred slot cycle index

•	The station class mark

•	An active MIN’s home SID

•	The list of active MINs (adding or deleting a MIN)

Parameter-change registration is independent of the roaming status of any MINs associated with the mobile station.

Whenever a parameter changes, the mobile station shall delete all entries from SID_NID_LISTs.  When parameter-change registration is performed, the mobile station shall perform an initial registration (see 6.6.5.4).

##### 6.6.5.1.7 Ordered Registration xe "registration, ordered"§

The base station can command the mobile station to register by sending a xe "Registration Request Order"§Registration Request Order.  Ordered registration is performed in the Mobile Station Order and Message Processing Operation (6.6.2.4).  Requirements are specified in 6.6.5.6.2.3.

##### 6.6.5.1.8 Implicit Registration xe "registration, implicit"§

Whenever an access is completed, the base station can infer the location of the mobile station.  This is treated as an implicit registration.  Requirements are specified in 6.6.5.6.3.

##### 6.6.5.1.9 Traffic Channel Registration xe "registration, Traffic Channel"§

While a mobile station is assigned a Traffic Channel, the mobile station is notified that it is registered through the Mobile Station Registered Message.  Requirements are specified in 6.6.5.6.4.3.

#### 6.6.5.2 Systems and Networks xe "registration, in systems and networks"§

A base station is a member of a cellular system and a xe "network"§network.  A network is a subset of a system.

Systems are labeled with an identification called the system identification or SID; xe "network"§networks within a system are given a network identification or NID.  A network is uniquely identified by the pair (xe "SID"§SID, xe "NID"§NID).  The NID number 0 is a reserved value indicating all base stations that are not included in a specific network.  The NID number 65535 (216-1) is a reserved value the mobile station may use for roaming status determination (see 6.6.5.3) to indicate that the mobile station considers the entire SID (regardless of NID) as home.

Figure 6.6.5.2-1 shows an example of systems and networks.  SID i contains three networks labeled t, u, and v.  A base station in xe "system"§system i that is not in one of these three networks is in NID 0.

tc "Figure 6.6.5.2-1.  Systems and Networks Example"\c §Figure 6.6.5.2-1.  Systems and Networks Example

#### 6.6.5.3 Roaming MINs xe "registration, roaming MINs"§

The concept of roaming applies individually to each mobile station identification number (xe "MIN"§MIN) that is active in a mobile station.  The mobile station, identified by the electronic serial number (xe "ESN"§ESN), may have one or more MINs associated with it.  In addition, the same MIN can be entered into multiple mobile stations.  The mobile station informs the base station of these MINs during the registration process.  Since neither the ESN nor the MIN directly provide a MIN’s home system, a means is provided for identifying the home SID, where basic parameters for a particular MIN can be obtained.  This SID is given by the HOME_SID parameter, which is transmitted in the xe "Initial Registration Message"§Initial Registration Message and the xe "Registration Update Message"§Registration Update Message.

For each MIN associated with the mobile station, the mobile station has a list of one or more home (SID, NID) pairs.  A MIN is roaming if the stored (SIDs, NIDs) pair (received in the System Parameters Message) does not match one of the MIN’s home (SID, NID) pairs.  Two types of roaming are defined:  A MIN is a foreign NID roamer if the MIN is roaming and there is some (SID, NID) pair in the MIN’s (SID, NID) list for which SID is equal to SIDs.  A MIN is a foreign SID roamer if there is no (xe "SID"§SID, xe "NID"§NID) pair in the MIN’s (SID, NID) list for which SID is equal to SIDs.  The mobile station may use the special NID value 65535 to indicate that the mobile station considers all NIDs within a SID to be home (i.e., that the mobile station is not roaming when operating with any base station in that system).

For each MIN associated with the mobile station, the mobile station shall store three 1-bit parameters in its permanent memory (see 6.3.8).  These parameters are MOB_TERM_HOMEp, MOB_TERM_FOR_SIDp, and MOB_TERM_FOR_NIDp.  The mobile station shall set MOB_TERM_HOMEp to ‘1’ if the mobile station is configured to receive mobile station terminated calls when using a home (SID, NID) pair; otherwise MOB_TERM_HOMEp shall be set to ‘0’.  The mobile station shall set MOB_TERM_FOR_SIDp to ‘1’ if the mobile station is configured to receive mobile station terminated calls when it is a foreign SID roamer; otherwise MOB_TERM_FOR_SIDp shall be set to ‘0’.  The mobile station shall set MOB_TERM_FOR_NIDp to ‘1’ if the mobile station is configured to receive mobile station terminated calls when it is a foreign NID roamer; otherwise MOB_TERM_FOR_NIDp shall be set to ‘0’.

The mobile station determines the registration status for each MIN using these parameters and the HOME_REG, FOR_NID_REG, and FOR_SID_REG fields of the System Parameters Message.

The mobile station shall store a mobile station call termination enabled indicator, MOB_TERMs, for each active MIN.  The mobile station shall set MOB_TERMs to YES if any of the following conditions is met for the MIN; otherwise xe "MOB_TERM"§MOB_TERMs shall be set to NO:

•	The MIN is not roaming, and MOB_TERM_HOMEp is equal to ‘1’; or

•	The MIN is a foreign NID roamer and MOB_TERM_FOR_NIDp is equal to ‘1’; or

•	The MIN is a foreign SID roamer and MOB_TERM_FOR_SIDp is equal to ‘1’.

The mobile station shall store a registration status indicator, xe "MIN_REG_ENABLED"§MIN_REG_ENABLEDs, for each active MIN.  The indicator xe "MIN_REG_ENABLED"§MIN_REG_ENABLEDs for a MIN shall be set to YES if any of the following conditions is met for the MIN; otherwise xe "MIN_REG_ENABLED"§MIN_REG_ENABLEDs shall be set to NO:

•	The MIN is not roaming, and both HOME_REGs and MOB_TERM_HOMEp are  equal to ‘1’; or

•	The MIN is a foreign NID roamer and both FOR_NID_REGs and MOB_TERM_FOR_NIDp are equal to  ‘1’; or

•	The MIN is a foreign SID roamer and both FOR_SID_REGs and MOB_TERM_FOR_SIDp are equal to ‘1’.

The mobile station performs class C registrations if xe "MIN_REG_ENABLED"§MIN_REG_ENABLEDs is YES for any of the mobile station’s active MINs.

The mobile station shall store the indicator xe "C_REG_ENABLED"§C_REG_ENABLEDs.  The mobile station shall set xe "C_REG_ENABLED"§C_REG_ENABLEDs to YES if MIN_REG_ENABLEDs is YES for any active MIN.

#### 6.6.5.4 Registration Messages xe "registration, messages"§

The mobile station sends two types of xe "registration messages "§registration messages to the base station:  the xe "Initial Registration Message "§Initial Registration Message and the xe "Registration Update Message"§Registration Update Message.  The Initial Registration Message is sent when performing an initial registration.  An initial registration is performed whenever it is necessary to inform the base station of the complete configuration of the mobile station.  Any registration that is not an initial registration is a registration update.  The Registration Update Message is sent for a registration update, such as when the mobile station adds or deletes a MIN or registers without changing the list of MINs.  The Registration Update Message is also sent as a continuation of an initial registration when the complete list of MINs does not fit into a single Initial Registration Message.

The first registration sent after power-up or after the serving system is changed shall be an initial registration.

When the mobile station performs an initial registration, all active MINs shall be included in the registration.  The first registration message sent shall be an Initial Registration Message.  If the mobile station cannot fit the entire set of MINs into the Initial Registration Message, it shall send the Initial Registration Message, followed by one or more Registration Update Messages with the additional MINs.  If an initial registration is incomplete (i.e., if not all MINs are registered), the next registration performed by the mobile station, if not a power-down registration, shall also be an initial registration

When the mobile station performs a registration update, it shall send a Registration Update Message.  If the mobile station adds or deletes multiple MINs and cannot fit the entire set of MIN changes into one Registration Update Message, it shall send additional Registration Update Messages as needed.

#### 6.6.5.5 Registration Timers and Indicators

The mobile station shall provide the following registration timers:

•	Power-up/initialization timer (see 6.6.5.1.1).

•	Timer-based registration timer (see 6.6.5.1.3).

•	Zone list entry timers (see 6.6.5.1.5).

•	SID/NID list entry timers (see 6.6.5.1.5).

The mobile station shall provide a means of enabling and disabling each timer.  When a timer is disabled, it shall not be considered expired.  A timer that has been enabled is referred to as active.

#### 6.6.5.6 Registration Procedures

##### 6.6.5.6.1 Actions in the Mobile Station Initialization State

###### 6.6.5.6.1.1 Power-up or Serving System Change

Upon power-up, the mobile station shall perform the following actions:

•	Delete all entries of ZONE_LISTs.

•	If ZONE_LISTs-p contains an entry, copy the entry to ZONE_LISTs and disable the corresponding entry timer.

•	Delete all entries of SID_NID_LISTs.

•	If SID_NID_LISTs-p contains an entry, copy the entry to SID_NID_LISTs and disable the corresponding entry timer.

Upon power-up or after switching from analog or the alternate CDMA serving system, the mobile station shall perform the following actions:

•	Set timer-based registration enable status (COUNTER_ENABLEDs) to NO.

•	Set class C registration enable status (C_REG_ENABLEDs) to NO.

•	Disable all registration timers (see 6.6.5.5).

###### 6.6.5.6.1.2 Timer Maintenance

While in the Mobile Station Initialization State, the mobile station shall update all active registration timers (see 6.6.5.5).  If any timer expires while in this state, the mobile station shall preserve the expiration status so that further action can be taken in the Mobile Station Idle State.

###### 6.6.5.6.1.3 Entering the Mobile Station Idle State

Before entering the Mobile Station Idle State, the mobile station shall perform the following actions:

•	Set the stored message sequence numbers CONFIG_MSG_SEQs, SYS_PAR_MSG_SEQs, NGHBR_LST_MSG_SEQs, CHAN_LST_MSG_SEQs, and ACC_MSG_SEQs variables to NULL (see 6.6.2.2).

•	If SID_NID_LISTs is empty, enable the power-up/initialization timer with an expiration time of T57m seconds (see 6.6.5.1.1).

##### 6.6.5.6.2 Actions in the Mobile Station Idle State

Requirements in this section and its subsections apply only when the mobile station is in the Mobile Station Idle State.

###### 6.6.5.6.2.1 Idle Registration Procedures

These procedures are performed whenever the mobile station is in the Mobile Station Idle State (see 6.6.2.1.3).

While in the Mobile Station Idle State, the mobile station shall update all active registration timers (see 6.6.5.5).

If the power-up/initialization timer has expired or is disabled, the mobile station shall perform the following actions in the order given; except that if any action causes a registration to be performed, the remaining actions, if any, shall not be performed and the mobile station shall enter the Update Overhead Information Substate of the System Access State (see 6.6.3) with a registration indication.

1.	The timer-based registration timer shall be enabled (COUNTER_ENABLEDs = YES) and the timer count (REG_COUNTs) shall be set to a pseudorandom number as specified in 6.6.5.1.3, if the following conditions are met:

a.	COUNTER_ENABLEDs is equal to NO; and

b.	The stored configuration parameters are current (see 6.6.2.2); and

c.	C_REG_ENABLEDs is equal to YES; and

d.	REG_PRDs is not equal to zero.

2.	If any zone list entry timer (see 6.6.5.1.5) has expired, the mobile station shall delete the corresponding entry from ZONE_LISTs.

3.	If any SID/NID list entry timer (see 6.6.5.1.5) has expired, the mobile station shall delete the corresponding entry from SID_NID_LISTs.

4.	The mobile station shall perform power-up registration, as specified in 6.6.5.1.1, if all the following conditions are met:

a.	POWER_UP_REGs is equal to ‘1’; and

b.	The stored configuration parameters are current (see 6.6.2.2); and

c.	SID_NID_LISTs is empty; and

d.	C_REG_ENABLEDs is equal to YES.

5.	The mobile station shall perform parameter-change registration (see 6.6.5.1.6) if all the following conditions are met:

a.	PARAMETER_REGs is equal to ‘1’; and

b.	The stored configuration parameters are current (see 6.6.2.2); and

c.	There is no entry of SID_NID_LISTs whose SID and NID fields match the stored SIDs and NIDs.

6.	The mobile station shall perform timer-based registration (see 6.6.5.1.3) if all the following conditions are met:

a.	COUNTER_ENABLEDs is equal to YES; and

b.	The stored configuration parameters are current (see 6.6.2.2); and

c.	C_REG_ENABLEDs is equal to YES; and

d.	REG_COUNTs is greater than or equal to REG_COUNT_MAXs.

7.	The mobile station shall perform distance-based registration (see 6.6.5.1.4) if all the following conditions are met:

a.	REG_DISTs is not equal to zero; and

b.	The stored configuration parameters are current (see 6.6.2.2); and

c.	C_REG_ENABLEDs is equal to YES; and

d.	The current base station’s distance from the base station in which the mobile station last registered (see 6.6.5.1.4) is greater than or equal to REG_DIST_REGs-p.

8.	The mobile station shall perform zone-based registration (see 6.6.5.1.5) if all the following conditions are met:

a.	TOTAL_ZONESs is not equal to zero; and

b.	The stored configuration parameters are current (see 6.6.2.2); and

c.	C_REG_ENABLEDs is equal to YES; and

d.	There is no entry of ZONE_LISTs whose SID, NID and REG_ZONE fields match the stored SIDs, NIDs and REG_ZONEs.

###### 6.6.5.6.2.2 Processing the Registration Fields of the System Parameters Message

When the mobile station processes the System Parameters Message, it shall perform the following actions:

1.	If REG_PRDs is equal to zero, the mobile station shall set COUNTER_ENABLEDs to NO.

2.	If REG_PRDs is not equal to zero, the mobile station shall set REG_COUNT_MAXs as specified in 6.6.5.1.3.

3.	The mobile station shall update MIN roaming status and set C_REG_ENABLEDs as specified in 6.6.5.3.

4.	If ZONE_LISTs contains more than TOTAL_ZONESs entries, the mobile station shall delete the excess entries according to the rules specified in 6.6.5.1.5.

###### 6.6.5.6.2.3 Ordered Registration

Ordered registration is performed after receiving a Registration Request Order while in the Mobile Station Order and Message Processing Operation (see 6.6.2.4).

If the ORDQ field is ‘00000001’, then the mobile station shall perform an initial registration (see 6.6.5.4).  If the ORDQ field is ‘00000010’, then the mobile station shall perform a registration update (see 6.6.5.4).  The mobile station shall enter the xe "Update Overhead Information Substate"§Update Overhead Information Substate of the xe "System Access State"§System Access State with a registration indication within T33m seconds after the Registration Request Order is received.

###### 6.6.5.6.2.4 Power Down

These procedures are performed when the mobile station is directed by the user to power down.

If POWER_UP_REGs is equal to ‘0’, the mobile station shall perform the following actions:

•	If an entry of ZONE_LISTs does not have an active timer, copy that entry to ZONE_LISTs-p; otherwise, delete any entry in ZONE_LISTs-p.

•	If an entry of SID_NID_LISTs does not have an active timer, copy that entry to SID_NID_LISTs-p; otherwise, delete any entry in SID_NID_LISTs-p.

If POWER_UP_REGs is equal to ‘1’, the mobile station shall delete all entries from ZONE_LISTs-p and SID_NID_LISTs-p.

The mobile station shall perform power-down registration, as specified in 6.6.5.1.2, if all the following conditions are met:

•	C_REG_ENABLEDs equals YES; and

•	POWER_DOWN_REGs equals ‘1’; and

•	There is an entry of SID_NID_LISTs for which the SID and NID fields are equal to SIDs and NIDs; and

•	The power-up/initialization timer (see 6.6.5.1.1) is disabled or has expired.

##### 6.6.5.6.3 Actions in the System Access State

Requirements in this section and its subsections apply only when the mobile station is in the System Access State.

###### 6.6.5.6.3.1 Successful Access

These procedures are performed after the mobile station receives an acknowledgement for a probe sent on the Access Channel (see 6.6.3.1.2).

The mobile station shall perform the following actions:

•	Disable the power-up/initialization timer (see 6.6.5.1.1).

•	Set the First-Idle ID status to enabled (see 2.6.1.1).

•	Set REG_COUNTs to zero.

•	Add REG_ZONEs, SIDs, and NIDs to ZONE_LISTs if not already in the list.

•	Disable the zone list entry timer for the entry of ZONE_LISTs containing REG_ZONEs, SIDs, and NIDs.  For any other entry of ZONE_LISTs whose entry timer is not active, enable the entry timer with the duration specified by ZONE_TIMERs (see 6.6.5.1.5).

•	If ZONE_LISTs contains more than TOTAL_ZONESs entries, delete the excess entries according to the rules specified in 6.6.5.1.5.

•	Add SIDs and NIDs to SID_NID_LISTs if not already in the list.

•	Disable the SID/NID list entry timer for the entry of SID_NID_LISTs containing SIDs, and NIDs.  For any other entry of SID_NID_LISTs whose entry timer is not active, enable the entry timer with the duration specified in 6.6.5.1.5.

•	If SID_NID_LISTs contains more than N10m entries, the mobile station may delete the excess entries according to the rules specified in 6.6.5.1.5.

•	Set the stored location of last registration (xe "BASE_LAT_REG"§BASE_LAT_REGs-p and xe "BASE_LONG_REG"§BASE_LONG_REGs-p) to the current base station’s location (xe "BASE_LAT"§BASE_LATs and xe "BASE_LONG"§BASE_LONGs).  Set the stored registration distance (REG_DIST_REGs-p) to the current base station’s registration distance (REG_DISTs).

•	Update MIN roaming status and set C_REG_ENABLEDs and MOB_TERMs as specified in 6.6.5.3.

###### 6.6.5.6.3.2 Unsuccessful Access

These procedures are performed when an access attempt fails.

The mobile station shall perform the following actions:

•	If there is an entry of ZONE_LISTs whose entry timer is not active, enable the entry timer with the duration specified by ZONE_TIMERs (see 6.6.5.1.5).

•	If there is an entry of SID_NID_LISTs whose entry timer is not active, enable the entry timer with the duration specified in 6.6.5.1.5.

###### 6.6.5.6.3.3 Power Down

These procedures are performed when the mobile station is directed by the user to power down.

If POWER_UP_REGs is equal to ‘0’, the mobile station shall perform the following actions:

•	If an entry of ZONE_LISTs does not have an active timer, copy that entry to ZONE_LISTs-p; otherwise, delete any entry in ZONE_LISTs-p.

•	If an entry of SID_NID_LISTs does not have an active timer, copy that entry to SID_NID_LISTs-p; otherwise, delete any entry in SID_NID_LISTs-p.

If POWER_UP_REGs is equal to ‘1’, the mobile station shall delete all entries from ZONE_LISTs-p and SID_NID_LISTs-p.

##### 6.6.5.6.4 Actions in the Mobile Station Control on the Traffic Channel State

Requirements in this section and its subsections apply only when the mobile station is in the Mobile Station Control on the Traffic Channel State.

###### 6.6.5.6.4.1 Call Initiation

Upon entering the Mobile Station Control on the Traffic Channel State, the mobile station shall set COUNTER_ENABLEDs to NO.

###### 6.6.5.6.4.2 Timer Maintenance

While in the Mobile Station Control on the Traffic Channel State, the mobile station shall update all active registration timers.

If a zone list entry timer expires, the mobile station shall delete the corresponding entry from ZONE_LISTs.  If a SID/NID list entry timer expires, the mobile station shall delete the corresponding entry from SID_NID_LISTs.

###### 6.6.5.6.4.3 Processing the Mobile Station Registered Message

The mobile station receives the Mobile Station Registered Message on the Forward Traffic Channel when the mobile station is considered registered for the base station whose location and other parameters are included in the message.

The mobile station shall store the following parameters:

•	xe "System identification "§System identification (xe "SID"§SIDs = SIDr)

•	xe "Network identification "§Network identification (xe "NID"§NIDs = NIDr)

•	xe "Registration zone "§Registration zone (xe "REG_ZONE"§REG_ZONEs = REG_ZONEr)

•	Number of registration zones to be retained (TOTAL_ZONESs = TOTAL_ZONESr)

•	xe "Zone timer length "§Zone timer length (xe "ZONE_TIMERs = ZONE_TIMERr)"§ZONE_TIMERs=ZONE_TIMERr)

•	xe "Base station latitude (.i.BASE_LAT"§Base station latitude (BASE_LATs = BASE_LATr)

•	xe "Base station longitude (.i.BASE_LONG"§Base station longitude (BASE_LONGs = BASE_LONGr)

•	xe "Registration distance"§Registration distance (xe "REG_DIST"§REG_DISTs = REG_DISTr)

The mobile station shall perform the following actions:

•	Set the First-Idle ID status to enabled (see 2.6.2.1).

•	Add REG_ZONEs, SIDs, and NIDs to ZONE_LISTs if not already in the list.

•	Disable the zone list entry timer for the entry of ZONE_LISTs containing REG_ZONEs, SIDs, and NIDs.  For any other entry of ZONE_LISTs whose entry timer is not active, enable the entry timer with the duration specified by ZONE_TIMERs (see 6.6.5.1.5).

•	If ZONE_LISTs contains more than TOTAL_ZONESs entries, delete the excess entries according to the rules specified in 6.6.5.1.5.

•	Add SIDs and NIDs to SID_NID_LISTs if not already in the list.

•	Disable the SID/NID list entry timer for the entry of SID_NID_LISTs containing SIDs, and NIDs.  For any other entry of SID_NID_LISTs whose entry timer is not active, enable the entry timer with the duration specified in 6.6.5.1.5.

•	If SID_NID_LISTs contains more than N10m entries, the mobile station may delete the excess entries according to the rules specified in 6.6.5.1.5.

•	Set the stored location of last registration (xe "BASE_LAT_REG"§BASE_LAT_REGs-p and xe "BASE_LONG_REG"§BASE_LONG_REGs-p) to the base station’s location (xe "BASE_LAT"§BASE_LATs and xe "BASE_LONG"§BASE_LONGs).  Set the stored registration distance (REG_DIST_REGs-p) to the base station’s registration distance (REG_DISTs).

•	Update MIN roaming status and set MOB_TERMs as specified in 6.6.5.3.

###### 6.6.5.6.4.4 Power Down

These procedures are performed when the mobile station is directed by the user to power down.

If POWER_UP_REGs is equal to ‘0’, the mobile station shall perform the following actions:

•	If an entry of ZONE_LISTs does not have an active timer, copy that entry to ZONE_LISTs-p; otherwise, delete any entry in ZONE_LISTs-p.

•	If an entry of SID_NID_LISTs does not have an active timer, copy that entry to SID_NID_LISTs-p; otherwise, delete any entry in SID_NID_LISTs-p.

If POWER_UP_REGsis equal to ‘1’, the mobile station shall delete all entries from ZONE_LISTs-p and SID_NID_LISTs-p.

### 6.6.6 Handoff Procedures

This section presents an overview and mobile station requirements for handoffs occurring while the mobile station is in the Mobile Station Control on the Traffic Channel State (see 6.6.4).  Mobile station requirements for handoffs occurring while the mobile station is in the xe "Mobile Station Idle State "§Mobile Station Idle State are specified in 6.6.2.1.4.

#### 6.6.6.1 Overview

##### 6.6.6.1.1 Types of Handoff

The mobile station supports the following three handoff procedures while in the xe "Mobile Station Control on the Traffic Channel State"§Mobile Station Control on the Traffic Channel State:

_	xe "Soft Handoff:"§Soft Handoff  .A handoff in which the mobile station commences communications with a new base station without interrupting communications with the old base station.  Soft handoff can only be used between xe "CDMA Channel"§CDMA Channels having identical frequency assignments.  Soft handoff provides diversity of Forward Traffic Channels and Reverse Traffic Channel paths on the boundaries between base stations.

_	xe "CDMA to CDMA Hard Handoff:"§CDMA to CDMA Hard Handoff  .A handoff in which the mobile station is transitioned between disjoint sets of base stations, different frequency assignments, or different frame offsets.

_	xe "CDMA to Analog Handoff:"§CDMA to Analog Handoff  .A handoff in which the mobile station is directed from a Forward Traffic Channel to an analog voice channel.

##### 6.6.6.1.2 xe "Pilot Sets"§Pilot Sets

In the following, the term pilot refers to a Pilot Channel identified by a pilot sequence offset (see 7.1.3.2.1) and a frequency assignment (see 7.1.1.1).  A pilot is associated with the Forward Traffic Channels in the same Forward CDMA Channel.

Soft handoffs and CDMA to CDMA hard handoffs using the same frequency assignment are typically initiated by the mobile station.  The mobile station searches for pilots to detect the presence of CDMA Channels and to measure their strengths.  When the mobile station detects a pilot of sufficient strength that is not associated with any of the Forward Traffic Channels assigned to it, it sends a Pilot Strength Measurement Message to the base station.  The base station can then assign a Forward Traffic Channel associated with that pilot to the mobile station and direct the mobile station to perform a handoff.

The pilot search parameters and the rules for Pilot Strength Measurement Message transmission are expressed in terms of the following sets of pilots:

_	xe "Active Set"§Active Set:  The pilots associated with the Forward Traffic Channels assigned to the mobile station.

_	xe "Candidate Set"§Candidate Set:  The pilots that are not currently in the Active Set but have been received by the mobile station with sufficient strength to indicate that the associated Forward Traffic Channels could be successfully demodulated.

_	xe "Neighbor Set"§Neighbor Set:  The pilots that are not currently in the Active Set or the Candidate Set but are likely candidates for xe "handoff"§handoff.

_	xe "Remaining Set"§Remaining Set:  The set of all possible pilots in the current system on the current CDMA frequency assignment, excluding the pilots in the xe "Neighbor Set, the Candidate Set, and the Active Set."§Neighbor Set, the Candidate Set, and the Active Set.

#### 6.6.6.2 Requirements

##### 6.6.6.2.1 Pilot Search

The base station specifies for each of the above pilot sets the search window (range of PN offsets) in which the mobile station is to search for usable multipath components (i.e., multipath components that the mobile station can use for demodulation of the associated Forward Traffic Channel) of the pilots in the set.

Search performance criteria are defined in the “Recommended Minimum Performance Standards for 800 MHz Wideband Spread Spectrum Dual-Mode Mobile Stations.”

This search shall be governed by the following:

_	xe "Active Set"§Active Set  and xe "Candidate Set"§Candidate Set:  The search procedures for pilots in the Active Set and Candidate Set shall be identical.  The search window size for each pilot in the Active Set and Candidate Set shall be the number of PN chips specified in Table 6.6.6.2.1-1 corresponding to SRCH_WIN_A;s.  The mobile station should center the search window for each pilot of the Active Set and Candidate Set around the earliest arriving usable multipath component of the pilot.

tc "Table 6.6.6.2.1-1.  Searcher Window Sizes"\c 	§Table 6.6.6.2.1-1.  Searcher Window Sizes

| SRCH_WIN_A SRCH_WIN_N SRCH_WIN_R | Window Size (PN chips) | SRCH_WIN_A SRCH_WIN_N SRCH_WIN_R | Window Size (PN chips) |
| --- | --- | --- | --- |
| 0 | 2 | 8 | 40 |
| 1 | 4 | 9 | 56 |
| 2 | 6 | 10 | 80 |
| 3 | 8 | 11 | 114 |
| 4 | 10 | 12 | 160 |
| 5 | 14 | 13 | 226 |
| 6 | 20 | 14 | 320 |
| 7 | 28 | 15 | 452 |

_	xe "Neighbor Set"§Neighbor Set:  The search window size for each pilot in the Neighbor Set shall be the number of PN chips specified in Table 6.6.6.2.1-1 corresponding to SRCH_WIN_N;s.  The mobile station should center the search window for each pilot in the Neighbor Set around the pilot’s PN sequence offset using timing defined by the mobile station’s time reference (see 6.1.5.1).

_	xe "Remaining Set"§Remaining Set:  The search window size for each pilot in the Remaining Set shall be the number of PN chips specified in Table 6.6.6.2.1-1 corresponding to SRCH_WIN_R;s.  The mobile station should center the search window for each pilot in the Remaining Set around the pilot’s PN sequence offset using timing defined by the mobile station’s time reference (see 6.1.5.1).

##### 6.6.6.2.2 Pilot Strength Measurements

The mobile station assists the base station in the handoff process by measuring and reporting the strengths of received pilots.

The mobile station shall compute the strength of a pilot by adding the ratios of received pilot energy per chip, Ec, to total received spectral density (noise and signals), I0, of at most k usable multipath components, where k is the number of demodulating elements supported by the mobile station.

##### 6.6.6.2.3 Handoff Drop Timer

The mobile station shall maintain a handoff drop timer for each pilot in the Active Set and Candidate Set.  The mobile station shall start the timer whenever the strength of the corresponding pilot becomes less than T_DROPs.  The mobile station shall reset and disable the timer if the strength of the corresponding pilot exceeds T_DROPs.  If T_TDROPs equals zero, the mobile station shall consider the timer expired within 100 ms of enabling it.  Otherwise, the mobile station shall consider the timer expired within 10% of the timer expiration value shown in Table 6.6.6.2.3.-1 corresponding to T_TDROPs.  If T_TDROPs changes, the mobile station shall begin using the new value within 100 ms.

tc "Table 6.6.6.2.3-1.  Handoff Drop Timer Expiration Values"\c 	§Table 6.6.6.2.3-1.  Handoff Drop Timer Expiration Values

| T_TDROP | Timer Expiration (seconds) | T_TDROP | Timer Expiration (seconds) |
| --- | --- | --- | --- |
| 0 | 0 | 8 | 27 |
| 1 | 1 | 9 | 39 |
| 2 | 2 | 10 | 55 |
| 3 | 4 | 11 | 79 |
| 4 | 6 | 12 | 112 |
| 5 | 9 | 13 | 159 |
| 6 | 13 | 14 | 225 |
| 7 | 19 | 15 | 319 |

##### 6.6.6.2.4 Pilot PN Phase

The mobile station shall measure the arrival time, PILOT_ARRIVAL, for each pilot reported to the base station.  The pilot arrival time shall be the time of occurrence, as measured at the mobile station antenna connector, of the earliest arriving usable multipath component of the pilot.  The arrival time shall be measured relative to the mobile station’s time reference (see 6.1.5.1) in units of PN chips.  The mobile station shall compute the reported pilot PN phase, PILOT_PN_PHASE, as

PILOT_PN_PHASE = (PILOT_ARRIVAL + (64 ¥ PILOT_PN)) mod 215,

where PILOT_PN is the PN sequence offset index of the pilot (see 7.1.3.2.1).

##### 6.6.6.2.5 Handoff Messages

###### 6.6.6.2.5.1 Processing of Forward Traffic Channel Handoff Messages

If the mobile station receives any of the following messages, then the mobile station shall process the message as described.

1.	Pilot Measurement Request Order: The mobile station shall send, within T56m seconds, a xe "Pilot Strength Measurement Message"§Pilot Strength Measurement Message.

2.	Handoff Direction Message: The message shall take effect at the following time:

_	If FRAME_OFFSETr is not equal to FRAME_OFFSETs and xe "USE_TIMEr"§USE_TIMEr equals ‘0’, then the mobile station shall process the message at the first 80 ms boundary (relative to System Time) occurring at least 80 ms after the end of the frame containing the last bit of the message.

_	Otherwise,; the message shall take effect at the xe "action time of the message as specified in 6.6.4.1.5."§action time of the message as specified in 6.6.4.1.5.

When the message takes effect, the mobile station shall perform the following:

_	Update the Active Set in accordance with the Handoff Direction Message (see 6.6.6.2.6.1).

_	Discontinue use of all Forward Traffic Channels associated with pilots not listed in the Handoff Direction Message.

_	If FRAME_OFFSETr is not equal to FRAME_OFFSETs, change the frame offset on both the Forward Traffic Channel and the xe "Reverse Traffic Channel."§Reverse Traffic Channel.

_	If the RESET_L2r is equal to ‘1’, reset the acknowledgement procedures as specified in 6.6.4.1.3.3, and reset the Forward Traffic Channel power control counters as specified in 6.6.4.1.1.1.

_	Use the long code mask specified by the PRIVATE_LCMr (see 6.3.12.3).

_	Process the ENCRYPT_MODE field as specified in 6.3.12.2.

_	Store the following parameters from the Handoff Direction Message:

_	Handoff Direction Message sequence number (HDM_SEQs = HDM_SEQr)

_	xe "Pilot detection threshold"§Pilot detection threshold (T_ADDs = T_ADDr)

_	xe "Pilot drop threshold (.i.T_DROPs = T_DROPr)"§Pilot drop threshold (T_DROPs = T_DROPr)

_	xe "Active Set versus Candidate Set comparison threshold (.i.T_COMPs = T_COMPr)"§Active Set versus Candidate Set comparison threshold (T_COMPs = T_COMPr)

_	xe "Drop timer"§Drop timer value (xe "T_TDROP"§T_TDROPs = T_TDROPr)

_	Frame offset (FRAME_OFFSETs = FRAME_OFFSETr)

xe "33.	Analog Handoff Direction Message:  The mobile station shall process the message"§3.Analog Handoff Direction Message  The mobile station shall process the message as specified in 6.6.6.2.9.

xe "4.	Neighbor List Update Message:  The mobile station shall process the message"§4.Neighbor List Update Message  The mobile station shall process the message as specified in 6.6.6.2.6.3.

###### 6.6.6.2.5.2 Processing of Reverse Traffic Channel Handoff Messages

The mobile station sends the following messages on the xe "Reverse Traffic Channel"§Reverse Traffic Channel in support of handoff when its transmitter is enabled and following the receipt of the first Base Station Acknowledgement Order on the Forward Traffic Channel:

1.	Pilot Strength Measurement Message:  The mobile station shall send an autonomous xe "Pilot Strength Measurement Message"§Pilot Strength Measurement Message whenever any of the following events occur:

_	The strength of a Candidate Set, Neighbor Set, or Remaining Set pilot is found to be above T_ADDs and a Pilot Strength Measurement Message carrying this information has not been sent since the last Handoff Direction Message was received.

_	The strength of a Candidate Set pilot exceeds the strength of an Active Set pilot by T_COMPs dB and a Pilot Strength Measurement Message carrying this information has not been sent since the last Handoff Direction Message was received.

_	The handoff drop timer of an Active Set pilot has expired and a Pilot Strength Measurement Message carrying this information has not been sent since the last Handoff Direction Message was received.

2.	Handoff Completion Message:  The mobile station shall send the Handoff Completion Message within T56m seconds of the action time of a received Handoff Direction Message.

##### 6.6.6.2.6 Set Maintenance

###### 6.6.6.2.6.1 Maintenance of the Active Set

The mobile station shall support an Active Set size of at least N6m pilots.

When the mobile station is first assigned a Forward Traffic Channel, the xe "mobile station shall initialize the Active Set"§mobile station shall initialize the Active Set to contain only the pilot associated with the assigned Forward Traffic Channel.  When the mobile station processes a Handoff Direction Message it shall replace the Active Set with the pilots listed in the message.

###### 6.6.6.2.6.2 Maintenance of the Candidate Set

The mobile station shall support a Candidate Set size of at least N7m pilots.

When the mobile station is first assigned a Forward Traffic Channel, the xe "mobile station shall initialize the Candidate Set to contain no pilots.  The mobile station shall adjust the Candidate Set whenever any of the following events occur:"§mobile station shall initialize the Candidate Set to contain no pilots.  The mobile station shall adjust the Candidate Set whene

_	If the mobile station detects that the strength of a Neighbor Set pilot or a Remaining Set pilot exceeds T_ADDs, the mobile station shall add the pilot to the Candidate Set.

_	If the mobile station processes a Handoff Direction Message which does not list a pilot in the current Active Set, and the handoff drop timer corresponding to that pilot has not expired, the mobile station shall add the pilot to the Candidate Set.

_	If the mobile station processes a Handoff Direction Message which lists a pilot in the current Candidate Set, the mobile station shall delete the pilot from the Candidate Set.

_	If the handoff drop timer corresponding to a Candidate Set pilot expires, the mobile station shall delete the pilot from the Candidate Set.

_	If the mobile station adds a pilot to the Candidate Set and the resulting Candidate Set size exceeds the size supported by the mobile station, the mobile station shall delete from the Candidate Set the pilot whose handoff drop timer is closest to expiration.  If more than one such pilot exists, the mobile station shall delete one such pilot that has the lowest strength.  If no pilot in the Candidate Set has an enabled handoff drop timer, the mobile station shall delete from the Candidate Set one of the pilots that has the lowest strength.

###### 6.6.6.2.6.3 Maintenance of the Neighbor Set

The mobile station shall support a Neighbor Set size of at least N8m pilots.

When the mobile station is first assigned a Forward Traffic Channel, the mobile station shall initialize the xe "Neighbor Set"§Neighbor Set to contain the pilots specified in the most recently received xe "Neighbor List Message."§Neighbor List Message.

The mobile station shall maintain a counter, AGEs, for each pilot in the Neighbor Set.  The mobile station shall initialize this counter to zero when it moves the pilot from the Active Set or the Candidate Set to the Neighbor Set.  The mobile station shall initialize this counter to NGHBR_MAX_AGEs when it moves the pilot from the Remaining Set to the Neighbor Set.  The mobile station shall increment AGEs for each pilot in the Neighbor Set upon receipt of a Neighbor List Update Message.

The mobile station shall adjust the Neighbor Set whenever any of the following events occur:

_	If the mobile station receives a xe "Neighbor List Update Message"§Neighbor List Update Message, it shall perform the following:

_	Increment AGEs for each pilot in the Neighbor Set.

_	Delete from the Neighbor Set all pilots whose AGEs exceeds NGHBR_MAX_AGEs.

_	Add to the xe "Neighbor Set"§Neighbor Set each pilot named in the message, if it is not already a pilot of the Candidate Set or Neighbor Set.  If the mobile station can store in the Neighbor Set only k additional pilots and more than k new pilots were sent in the Neighbor List Update Message, the mobile station shall store the first k new pilots listed in the message.

_	If the handoff drop timer of a pilot in the Candidate Set expires, the mobile station shall add the pilot to the Neighbor Set.

_	If the mobile station processes a Handoff Direction Message in which a pilot in the Active Set is not listed and the handoff drop timer corresponding to the pilot has expired, the mobile station shall add the pilot to the Neighbor Set.

_	If the mobile station adds a pilot to the Candidate Set and the resulting Candidate Set size exceeds the size supported by the mobile station, the mobile station shall add the deleted Candidate Set pilot to the Neighbor Set (see 6.6.6.2.6.2).

_	If the mobile station detects that the strength of a Neighbor Set pilot exceeds T_ADDs, the mobile station shall delete the pilot from the Neighbor Set.

_	If the mobile station processes a Handoff Direction Message which lists a pilot in the current Neighbor Set, the mobile station shall delete the pilot from the Neighbor Set.

_	If the mobile station adds a pilot to the Neighbor Set and the resulting Neighbor Set size exceeds the size supported by the mobile station, the mobile station shall delete from the Neighbor Set the pilot whose AGEs is largest.  If more than one such pilot exists, the mobile station shall delete one such pilot that has the lowest strength.

##### 6.6.6.2.7 Soft Handoff

###### 6.6.6.2.7.1 Forward Traffic Channel Processing

All Forward Traffic Channels associated with pilots in the Active Set of the mobile station carry identical modulation symbols with the exception of the power control subchannel (see 7.1.3.1.7 and 7.6.6.2.4.2).

When the Active Set contains more than one pilot, the mobile station should provide diversity combining of the associated Forward Traffic Channels.  The mobile station shall provide for differential propagation delays from zero to at least 150 µs.

###### 6.6.6.2.7.2 Reverse Traffic Channel Power Control During Soft Handoff

The xe "Handoff Direction Message "§Handoff Direction Message identifies sets of Forward Traffic Channels that carry identical closed loop power control subchannels.

In each power control group containing valid power control bits (see 6.1.2.3.2), the mobile station should provide diversity combining of the identical closed loop power control subchannels and shall obtain at most one power control bit from each set of identical closed loop power control subchannels.  If the power control bits obtained from all sets are equal to ‘0’, the mobile station shall increase its power as specified in 6.1.2.3.2.  If the power control bit obtained from any set is equal to ‘1’, the mobile station shall decrease its power as specified in 6.1.2.3.2.

###### 6.6.6.2.7.3 Timing During Soft Handoff

The mobile station shall comply with the requirements specified in 6.1.5.1 during xe "soft handoff"§soft handoff.

##### 6.6.6.2.8 CDMA to CDMA Hard Handoff xe "mobile station, call processing, handoff, CDMA-to-CDMA hard handoff"§

When the mobile station processes a xe "Handoff Direction Message specifying a CDMA to CDMA hard handoff, it shall disable its transmitter, reset the fade timer specified in 6.4.4, and tune to the assigned Forward Traffic Channel.  The mobile station shall not enable its transmitter until it receives at least N3m consecutive good frames on the assigned .i.Forward Traffic Channel."§Handoff Direction Message specifying a CDMA to CDMA hard handoff, it shall disable its transmitter, reset the fade timer specifi

If the Handoff Direction Message specifies a CDMA frequency assignment different from the current CDMA frequency assignment and an Active Set containing pilots with pilot PN sequence offsets identical to those of the pilots in the current Active Set, the mobile station shall begin monitoring the assigned Forward Traffic Channel within T60m seconds.

If the Handoff Direction Message specifies a CDMA frequency assignment different from the current CDMA frequency assignment and an Active Set containing a pilot with pilot PN sequence offset not equal to that of any pilot in the current Active Set, the mobile station shall begin monitoring the assigned Forward Traffic Channel within T61m seconds.

If the Handoff Direction Message specifies a CDMA to CDMA hard handoff using the current CDMA frequency assignment, the mobile station shall begin monitoring the assigned Forward Traffic Channel within T62m seconds.

##### 6.6.6.2.9 CDMA to Analog Handoff

The base station directs the mobile station to perform a CDMA to Analog handoff by sending an xe "Analog Handoff Direction Message"§Analog Handoff Direction Message.

The mobile station shall store the following parameters from the Analog Handoff Direction Message:

•	System identification (SIDs = SIDr)

•	Voice mobile station attenuation code (VMACs = VMACr)

•	Analog voice channel number (ANALOG_CHANs = ANALOG_CHANr)

•	SAT color code (SCCs = SCCr)

•	Message encryption mode indicator (MEMs = MEMr)

At the action time specified by the Analog Handoff Direction Message (see 6.6.4.1.5), the mobile station shall disable its transmitter.  The mobile station shall enable its transmitter on the analog voice channel within T63m seconds.

#### 6.6.6.3 Examples

The following examples illustrate typical message exchanges between the mobile station and the base station during handoff.  Refer to Appendix B for examples of call processing during handoff.

Figure 6.6.6.3-1 shows an example of the messages exchanged between the mobile station and the base station during a typical handoff process.

(1)	Pilot strength exceeds T_ADD.  Mobile station sends a Pilot Strength Measurement Message and transfers pilot to the Candidate Set.

(2)	Base station sends a Handoff Direction Message.

(3)	Mobile station transfers pilot to the Active Set and sends a Handoff Completion Message.

(4)	Pilot strength drops below T_DROP.  Mobile station starts the handoff drop timer.

(5)	Handoff drop timer expires.  Mobile station sends a Pilot Strength Measurement Message.

(6)	Base station sends a Handoff Direction Message.

(7)	Mobile station moves pilot from the Active Set to the Neighbor Set and sends a Handoff Completion Message.

tc "Figure 6.6.6.3-1.  Handoff Threshold Example"\c §Figure 6.6.6.3-1.  Handoff Threshold Example

Figure 6.6.6.3-2 illustrates the messaging triggered by a pilot of the Candidate Set as its strength gradually rises above the strength of each pilot of the Active Set.  Note that the mobile station reports that a Candidate Set pilot is stronger than an Active Set pilot only if the difference between their respective strengths is at least T_COMP.

Candidate Set:  Pilot P0

Active Set:  Pilots P1, P2

t0 — Pilot Strength Measurement Message sent, P0 > T_ADD

t1 — Pilot Strength Measurement Message sent, P0 > P1 + T_COMP

t2 — Pilot Strength Measurement Message sent, P0 > P2 + T_COMP

tc "Figure 6.6.6.3-2.  Pilot Strength Measurements Triggered by a Candidate Pilot"\c §Figure 6.6.6.3-2.  Pilot Strength Measurements Triggered by a Candidate Pilot

### 6.6.7 Hash Functions and Randomization xe "mobile station, call processing, hash function"§

#### 6.6.7.1 Hash Function

Certain procedures require a uniform distribution of mobile stations among N resources.  The following function returns an integer, using as arguments the mobile station’s 32-bit ESN, the number of resources N, and a modifier DECORR.  The modifier serves to decorrelate the values obtained for the various applications from the same xe "ESN"§ESN.

Define:

_	Word L to be bits 0-15 of the xe "ESN"§ESN

_	Word H to be bits 16-31 of the xe "ESN"§ESN

where bit 0 is the least significant bit of the ESN.  The hash value;  is computed as follows:

R = ÎN ¥ ((40503 ¥ (L _ H _ DECORR)) mod 216 ) / 216_.

The mobile station shall choose the range N and the 16-bit modifier xe "DECORR"§DECORR according to the application as shown in Table 6.6.7.1-1.  In the table, xe "ESN"§ESN [0...11] denotes the 12 least significant bits of the ESN.

tc "Table 6.6.7.1-1.  Hash Function Modifier"\c 	§Table 6.6.7.1-1.  Hash Function Modifier

| Application | N | xe "DECORR"§DECORR | Return Value |
| --- | --- | --- | --- |
| xe "CDMA Channel Number"§CDMA Channel Number | Number of channels in last xe "CDMA Channel List Message"§CDMA Channel List Message (up to 10) | 0 | R + 1 |
| xe "Paging Channel"§Paging Channel Number | xe "PAGE_CHAN"§PAGE_CHANs from xe "System Parameters Message"§System Parameters Message (up to 7) | 2 ¥ xe "ESN"§ESN [0…11] | R + 1 |
| Paging Slot Number | 640 | 6 ¥ ESN [0…11] | R |
| xe "Access Channel"§Access Channel xe "PN Randomization"§PN Randomization | 2PROBE_PN_RANs where xe "PROBE_PN_RAN"§PROBE_PN_RAN is from xe "Access Parameters Message"§Access Parameters Message (up to 512) | 14 ¥ ESN [0…11] | R |

#### 6.6.7.2 Pseudorandom Number Generator xe "mobile station, call processing, pseudorandom number generator"§

Where pseudorandom numbers are needed in the CDMA cellular protocols, a linear congruential generator shall be used.  The mobile station shall implement the linear congruential generator defined by:

zn = a ¥ zn-1 mod m

where a = 75 = 16807 and m = 231 - 1 = 2147483647.  zn is the output of the generator.

During the xe "Mobile Station Initialization State"§Mobile Station Initialization State, the mobile station shall seed its generator with

z0 = (xe "ESN"§ESN _ RANDOM_TIME) mod m

where RANDOM_TIME shall be the least-significant 32-bits of SYS_TIMEs stored from the Sync Channel Message.  If the initial value so produced is found to be zero, it shall be replaced with one. The mobile station shall compute a new zn for each subsequent use.

The mobile station shall use the value un = zn / m for those applications that require a binary fraction un, 0 < un < 1.

The mobile station shall use the value kn = ÎN ¥ zn / m_ for those applications that require a small integer kn, 0 £ kn £ N-1.

## 6.7 Signaling Formats xe "mobile station, signaling formats"§

This section describes the messages sent by the mobile station.

Some bits in the following message formats are marked as xe "RESERVED"§RESERVED.  These bits allow for extensions to the basic message for future features and capabilities.  The mobile station sets all reserved bits to ‘0’.

All messages have a set of acknowledgement fields.  These fields are xe "ACK_SEQ"§ACK_SEQ, xe "MSG_SEQ"§MSG_SEQ, xe "ACK_REQ"§ACK_REQ, and xe "VALID_ACK"§VALID_ACK for xe "Access Channel message"§Access Channel messages and ACK_SEQ, MSG_SEQ, and ACK_REQ for xe "Reverse Traffic Channel message"§Reverse Traffic Channel messages.

In any multi-bit field of a signaling message, the most significant bit shall be transmitted first.

### 6.7.1 xe "Access Channel"§Access Channel

This section describes the messages sent by the mobile station on the Access Channel (see 6.1.3.2).

#### 6.7.1.1 Access Channel Structure xe "mobile station, signaling formats, Access Channel, slot structure"§

An i.Access Channel slot; are (3 + xe "MAX_CAP_SZ"§MAX_CAP_SZ) + (1 + xe "PAM_SZ"§PAM_SZ) xe "Access Channel frame"§Access Channel frames in length.  An xe "Access Channel slot "§Access Channel slot begins and ends on an Access Channel frame boundary.  Access Channel slots begin at Access Channel frames in which

t mod (4 + xe "MAX_CAP_SZ"§MAX_CAP_SZ + xe "PAM_SZ"§PAM_SZ) = 0,

where t is the SSSystem Time; in frames.  Note that all Access Channels associated with a particular Paging Channel have the same slot size and that all of the slots begin at the same time.  Figure 6.7.1.1-1 shows an example of Access Channel slots.  Figure 6.7.1.1-2 shows the Access Channel structure.

The xe "Access Channel slot length"§Access Channel slot length may differ from base station to base station.  A mobile station shall determine the beginning and length of the xe "Access Channel slot "§Access Channel slot prior to transmission.

An xe "Access Channel transmission "§Access Channel transmission consists of the xe "Access Channel preamble"§Access Channel preamble and the xe "Access Channel message capsule"§Access Channel message capsule.  An xe "Access Channel transmission"§Access Channel transmission shall be an integer number of tc "Access Channel frame"§Access Channel frames in length and shall not exceed 4 + xe "MAX_CAP_SZ"§MAX_CAP_SZ + xe "PAM_SZ"§PAM_SZ Access Channel frames in length.

On each xe "Access Channel transmission"§Access Channel transmission, the mobile station shall transmit a preamble consisting of continuous xe "Walsh symbol "§Walsh symbol zero starting at the beginning of the slot (plus PN randomization as specified in 6.6.3.1.1.2) and 1 + xe "PAM_SZ "§PAM_SZ Access Channel frames in length.  The mobile station shall transmit an xe "Access Channel message capsule "§Access Channel message capsule immediately following the preamble.

tc "Figure 6.7.1.1-1.  Example of Access Channel Slot Structure"\c §Figure 6.7.1.1-1.  Example of Access Channel Slot Structure

tc "Figure 6.7.1.1-2.  Access Channel Structure "\c §Figure 6.7.1.1-2.  Access Channel Structure

#### 6.7.1.2 Access Channel Message Structure xe "mobile station, signaling formats, Access Channel, message structure"§

An Access Channel message capsule consists of an Access Channel message and padding, as shown in Figure 6.7.1.2-1.  The length of the Access Channel message capsule shall be an integer number of Access Channel frames given by

CAP_SZ =

where Èx_ denotes smallest integer equal to or exceeding x.

Each xe "Access Channel message"§Access Channel message shall consist of a length field, a message body, and a xe "CRC"§CRC, in that order.  The message body size shall be selected so that CAP_SZ does not exceed 3 + xe "MAX_CAP_SZ"§MAX_CAP_SZ.  The mobile station shall transmit the Access Channel message immediately following the preamble.

The mobile station shall transmit xe "padding "§padding consisting of zero or more ‘0’ bits immediately following the Access Channel message.  The length of the padding shall be such that

8 + Message Body Length + 30 + xe "Padding"§Padding Length = 88 ¥ CAP_SZ.

tc "Figure 6.7.1.2-1.  Access Channel Message Structure"\c §Figure 6.7.1.2-1.  Access Channel Message Structure

##### 6.7.1.2.1 Access Channel MSG_LENGTH Field xe "mobile station, signaling formats, Access Channel, MSG_LENGTH field"§

The mobile station shall set the xe "MSG_LENGTH "§MSG_LENGTH field of each Access Channel signaling message to the length of the message in octets, including the MSG_LENGTH field, the xe "message body"§message body, and the xe "CRC"§CRC, but not including the preamble or the xe "padding"§padding.  The MSG_LENGTH field shall be 8 bits in length.  The maximum xe "Access Channel message length"§Access Channel message length is therefore 255 octets, or 2040 bits.

##### 6.7.1.2.2 Access Channel Message CRC xe "mobile station, signaling formats, Access Channel, signaling message CRC"§

A 30-bit xe "CRC"§CRC shall be computed for each Access Channel signaling message.  The CRC shall include the xe "MSG_LENGTH"§MSG_LENGTH field and the xe "message body"§message body.  The generator polynomial for the CRC shall be as follows:

g(x) = x30 + x29 + x21 + x20 + x15 + x13 + x12 + x11 + x8 + x7 + x6 + x2 + x + 1.

The CRC shall be the value computed by the following procedure and the logic shown in Figure 6.7.1.2.2-1:

•	All shift register elements shall be initialized to logical one.

•	The switches shall be set in the up position.

•	The information bit count k shall be defined as 8 + message body length in bits.

•	The register shall be clocked k times, with the length and message body of the message as the k input bits.

•	The switches shall be set in the down position.

•	The register shall be clocked an additional 30 times.

•	The 30 additional output bits shall be the check bits.

•	The bits shall be transmitted in the order in which they are calculated.

tc "Figure 6.7.1.2.2-1.  Access Channel CRC Calculation"\c §Figure 6.7.1.2.2-1.  Access Channel CRC Calculation

#### 6.7.1.3 Access Channel Message Body Format xe "mobile station, signaling formats, Access Channel"§

The messages sent on the xe "Access Channel "§Access Channel are summarized in Table 6.7.1.3-1;.

tc "Table 6.7.1.3-1.  Access Channel Messages "\c 	§Table 6.7.1.3-1.  Access Channel Messages

| Message Name | Message Type (binary) |
| --- | --- |
| xe "Initial Registration Message"§Initial Registration Message | 00000001 |
| xe "Order Message"§Order Message | 00000010 |
| xe "Data Burst Message"§Data Burst Message | 00000011 |
| xe "Origination Message"§Origination Message | 00000100 |
| xe "Page Response Message"§Page Response Message | 00000101 |
| xe "Registration Update Message"§Registration Update Message | 00000110 |
| i.Authentication Challenge Response Message | 00000111 |

##### 6.7.1.3.1 Common Fields

###### 6.7.1.3.1.1 Common Fields xe "mobile station, signaling formats, Access Channel"§

All Access Channel messages share the same first five fields:

xe "ACK_SEQ"§ACK_SEQ	-	Acknowledgement sequence number.

The mobile station shall set this field to the value of the MSG_SEQ field from the most recently received Paging Channel message requiring acknowledgement.  If no such message has been received, the mobile station shall set this field to ‘111’.  See 6.6.2.1.2.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

The mobile station shall set this field to the message sequence number for this message.  See 6.6.3.1.2.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.  This field indicates whether this message requires an acknowledgement.  The mobile station shall set this field to ‘1’.

xe "VALID_ACK"§VALID_ACK	-	Valid acknowledgement indicator.

To acknowledge a Paging Channel message, the mobile station shall set this field to ‘1’.  Otherwise, the mobile station shall set this field to ‘0’.  See 6.6.2.1.2.

xe "ESN"§ESN	-	Mobile station’s xe "electronic serial number"§electronic serial number.

The mobile station shall set this field to its electronic serial number.  See 2.3.2.

###### 6.7.1.3.1.2 Common Authentication Fields xe "mobile station, signaling formats, Access Channel, Authentication"§

Most Access Channel messages share the same four fields related to authentication:

xe "AUTH_MODE"§AUTH_MODE	-	Authentication mode.

If authentication information is not available, or if the base station has indicated that authentication is not required (by setting the xe "AUTH"§AUTH field in the xe "Access Parameters Message "§Access Parameters Message to ‘00’), the mobile station shall set this field to ‘00’.  If authentication is required by the base station and authentication information is available, the mobile station shall set this field to ‘01’.  All other values are reserved.

xe "AUTHR"§AUTHR	-	Authentication data.

If the AUTH_MODE field is set to ‘01’, the mobile station shall set this field as specified in 6.3.12.1.  If the AUTH_MODE field is set to any other value, the mobile station shall omit this field.

xe "RANDC"§RANDC	-	Random challenge value.

If the AUTH_MODE field is set to ‘01’, the mobile station shall set this field as specified in 6.3.12.1.  If the AUTH_MODE field is set to any other value, the mobile station shall omit this field.

xe "COUNT"§COUNT	-	Call history parameter.

If the AUTH_MODE field is set to ‘01’, the mobile station shall set this field to the current value of the COUNTs-p parameter.  If the AUTH_MODE field is set to any other value, the mobile station shall omit this field.

##### 6.7.1.3.2 Message Body Contents xe "mobile station, signaling formats, Access Channel, message body"§

The following sections specify the contents of the message body for each message that may be sent on the Access Channel.

###### 6.7.1.3.2.1 Initial Registration Message xe "mobile station, signaling formats, Access Channel, Initial Registration Message"§

When the mobile station sends an xe "Initial Registration Message"§Initial Registration Message, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE "§MSG_TYPE (‘00000001’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 2 |
| xe "MSG_SEQ"§MSG_SEQ | 2 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "VALID_ACK"§VALID_ACK | 1 |
| xe "ESN"§ESN | 32 |
| xe "AUTH_MODE"§AUTH_MODE | 2 |
| xe "AUTHR"§AUTHR | 0 or 18 |
| xe "RANDC"§RANDC | 0 or 8 |
| xe "COUNT"§COUNT | 0 or 6 |
| xe "REG_TYPE"§REG_TYPE | 4 |
| xe "MORE_MINS"§MORE_MINS | 1 |
| xe "SLOT_CYCLE_INDEX"§SLOT_CYCLE_INDEX | 3 |
| xe "MOB_CAI_REV"§MOB_CAI_REV | 8 |
| xe "SCM"§SCM | 8 |
| One or more occurrences of the following record: |  |
| xe "ADD_DEL"§ADD_DEL | 1 |
| xe "MIN1"§MIN1 | 24 |
| xe "MIN2"§MIN2 | 10 |
| xe "HOME_SID"§HOME_SID | 15 |
| MOB_TERM | 1 |
| xe "RESERVED"§RESERVED | 0 - 7 (as needed) |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00000001’.

xe "ACK_SEQ"§ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.1.3.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.1.3.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.1.3.1.

xe "VALID_ACK"§VALID_ACK	-	Valid acknowledgement indicator.

See 6.7.1.3.1.

xe "ESN"§ESN	-	Mobile station’s xe "electronic serial number"§electronic serial number.

See 6.7.1.3.1.

xe "AUTH_MODE"§AUTH_MODE	-	Authentication mode.

See 6.7.1.3.2.

xe "AUTHR"§AUTHR	-	Authentication data.

See 6.7.1.3.2.

xe "RANDC"§RANDC	-	Random challenge value.

See 6.7.1.3.2.

xe "COUNT"§COUNT	-	Call history parameter.

See 6.7.1.3.2.

xe "REG_TYPE"§REG_TYPE	-	Registration type.

This field indicates which type of event generated the registration attempt.

The mobile station shall set this field to the REG_TYPE value shown in Table 6.7.1.3.2.1-1 corresponding to the event that caused this registration to occur (see 6.6.5.1).

tc "Table 6.7.1.3.2.1-1.  Registration Type (REG_TYPE) Codes"\c 	§Table 6.7.1.3.2.1-1.  Registration Type (REG_TYPE) Codes

| REG_TYPE (binary) | Type of Registration |
| --- | --- |
| 0000 | Timer-based (see 6.6.5.1.3) |
| 0001 | Power-up (see 6.6.5.1.1) |
| 0010 | Zone-based (see 6.6.5.1.5) |
| 0011 | Power-down (see 6.6.5.1.2) |
| 0100 | Parameter-change (see 6.6.5.1.6) |
| 0101 | Ordered (see 6.6.5.1.7) |
| 0110 | Distance-based (see 6.6.5.1.4) |
| All other REG_TYPE values are reserved. |  |

xe "MORE_MINS"§MORE_MINS	-	More MINs to follow indicator.

This indicator specifies whether additional MINs are to be sent in later xe "Registration Update Messages"§Registration Update Messages.

If all MINs actively supported by the mobile station are included in this message, the mobile station shall set this field to ‘0’.  If not, the mobile station shall set this field to ‘1’.

xe "SLOT_CYCLE_INDEX"§SLOT_CYCLE_INDEX	-	Slot cycle mode.

If the mobile station is operating in non-slotted mode, it shall set this field to ‘000’.  If the mobile station is operating in slotted mode, it shall set this field to the SLOT_CYCLE_INDEX value in use (see 6.6.2.1.1).

xe "MOB_CAI_REV"§MOB_CAI_REV	-	CAI revision of the mobile station.

The mobile station shall set this field to ‘00000001’.

xe "SCM"§SCM	-	Station class mark.

The mobile station shall set this field to its xe "station class mark.  See 2.3.3."§station class mark.  See 2.3.3.

The mobile station shall include one occurrence of the following five-field record for each MIN included in this message.

xe "ADD_DEL"§ADD_DEL	-	Add/Delete indicator.

This field indicates whether the MIN in this record is to be added to or deleted from the list of active MINs for this mobile station.  In this message, all MINs are to be added to the list.

The mobile station shall set this field to ‘1’.

xe "MIN1"§MIN1	-	First part of the xe "mobile station identification number"§mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN1 value for the MIN specified by this record.

xe "MIN2"§MIN2	-	Second part of the xe "mobile station identification number"§mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN2 value for the MIN specified by this record.

xe "HOME_SID"§HOME_SID	-	Home xe "system identification"§system identification.

The mobile station shall set this field to the xe "SID "§SID of the home cellular system for the MIN specified by this record.

MOB_TERM	-	Mobile terminated calls accepted indicator.

If the MIN specified by this record is configured to accept mobile terminated calls while operating with the current roaming status (see 6.6.5.3), the mobile station shall set this bit to ‘1’.  Otherwise, the mobile station shall set this bit to ‘0’.

.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall add reserved bits as needed in order to make the length of the entire message equal to an integer number of octets.  The mobile station shall set these bits to ‘0’.

###### 6.7.1.3.2.2 Order Message xe "mobile station, signaling formats, Access Channel, Order Messages"§

When the mobile station sends an xe "Order Message "§Order Message on the Access Channel, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00000010’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 2 |
| xe "MSG_SEQ"§MSG_SEQ | 2 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "VALID_ACK"§VALID_ACK | 1 |
| xe "ESN"§ESN | 32 |
| xe "RESERVED"§RESERVED | 2 |
| xe "ORDER"§ORDER | 6 |
| xe "ADD_RECORD_LEN"§ADD_RECORD_LEN | 3 |
| order-specific fields (if used) | 8 ¥ ADD_RECORD_LEN |
| xe "RESERVED"§RESERVED | 1 |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00000010’.

xe "ACK_SEQ"§ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.1.3.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.1.3.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.1.3.1.

xe "VALID_ACK"§VALID_ACK	-	Valid acknowledgement indicator.

See 6.7.1.3.1.

xe "ESN"§ESN	-	Mobile station’s xe "electronic serial number"§electronic serial number.

See 6.7.1.3.1.

xe "RESERVED"§RESERVED	-	Reserved bits.

These bits take the place of the xe "AUTH_MODE "§AUTH_MODE field.

The mobile station shall set this field to ‘00’.

xe "ORDER"§ORDER	-	Order code.

The mobile station shall set this field to the ORDER code (see 6.7.3) for this type of order message.

xe "ADD_RECORD_LEN"§ADD_RECORD_LEN		-	Additional record length.

The mobile station shall set this field to the number of octets in the order-specific fields included in this message.

order-specific fields	-	Order-specific fields.

The mobile station shall include order-specific fields as specified in 6.7.3.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall set this field to ‘0’.

###### 6.7.1.3.2.3 Data Burst Message xe "mobile station, signaling formats, Access Channel, One-Way Data Burst Message"§

When the mobile station sends a xe "Data Burst Message "§Data Burst Message on the Access Channel, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00000011’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 2 |
| xe "MSG_SEQ"§MSG_SEQ | 2 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "VALID_ACK"§VALID_ACK | 1 |
| xe "ESN"§ESN | 32 |
| xe "AUTH_MODE"§AUTH_MODE | 2 |
| xe "AUTHR"§AUTHR | 0 or 18 |
| xe "RANDC"§RANDC | 0 or 8 |
| xe "COUNT"§COUNT | 0 or 6 |
| xe "MSG_NUMBER"§MSG_NUMBER | 8 |
| xe "MIN1"§MIN1 | 24 |
| xe "MIN2"§MIN2 | 10 |
| xe "BURST_TYPE"§BURST_TYPE | 6 |
| xe "NUM_MSGS"§NUM_MSGS | 8 |
| xe "NUM_FIELDS"§NUM_FIELDS | 8 |
| NUM_FIELDS occurrences of the following field: |  |
| xe "CHARi"§CHARi | 8 |
| xe "RESERVED"§RESERVED | 2 |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00000011’.

xe "ACK_SEQ"§ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.1.3.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.1.3.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.1.3.1.

xe "VALID_ACK"§VALID_ACK	-	Valid acknowledgement indicator.

See 6.7.1.3.1.

xe "ESN"§ESN	-	Mobile station’s xe "electronic serial number"§electronic serial number.

See 6.7.1.3.1.

xe "AUTH_MODE"§AUTH_MODE	-	Authentication mode.

See 6.7.1.3.2.

xe "AUTHR"§AUTHR	-	Authentication data.

See 6.7.1.3.2.

xe "RANDC"§RANDC	-	Random challenge value.

See 6.7.1.3.2.

xe "COUNT"§COUNT	-	Call history parameter.

See 6.7.1.3.2.

xe "MSG_NUMBER"§MSG_NUMBER	-	Message number within the data burst stream.

The mobile station shall set this field to the number of this message within the data burst stream.

xe "MIN1"§MIN1	-	First part of the mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN1 value for the MIN sending this message.

xe "MIN2"§MIN2	-	Second part of the mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN2 value for the MIN sending this message.

xe "BURST_TYPE"§BURST_TYPE	-	Data burst type.

The mobile station shall set this field to the value shown in Table 6.7.1.3.2.3-1 for the type of this data burst.

tc "Table 6.7.1.3.2.3-1.  Burst Data Types"\c 	§Table 6.7.1.3.2.3-1.  Burst Data Types

| Value (binary) | Burst Data Type |
| --- | --- |
| 000000 | Unknown burst data type |
| All other burst data type codes are reserved. |  |

xe "NUM_MSG"§NUM_MSG	-	Number of messages in the data burst stream.

The mobile station shall set this field to the number of messages within this data burst stream.

xe "NUM_FIELDS"§NUM_FIELDS	-	Number of characters in this message.

The mobile station shall set this field to the number of CHARi fields included in this message.

xe "CHARi"§CHARi	-	Character.

The mobile station shall include NUM_FIELDS occurrences of this field.  The mobile station shall set these fields to the corresponding octet of the data burst stream.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall set this field to ‘00’.

###### 6.7.1.3.2.4 Origination Message xe "mobile station, signaling formats, Access Channel, Origination Message"§

When the mobile station sends an xe "Origination Message"§Origination Message, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00000100’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 2 |
| xe "MSG_SEQ"§MSG_SEQ | 2 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "VALID_ACK"§VALID_ACK | 1 |
| xe "ESN"§ESN | 32 |
| xe "AUTH_MODE"§AUTH_MODE | 2 |
| xe "AUTHR"§AUTHR | 0 or 18 |
| xe "RANDC"§RANDC | 0 or 8 |
| xe "COUNT"§COUNT | 0 or 6 |
| xe "MIN1"§MIN1 | 24 |
| xe "MIN2"§MIN2 | 10 |
| xe "SCM"§SCM | 8 |
| xe "REQUEST_MODE"§REQUEST_MODE | 3 |
| xe "SPECIAL_MUX"§SPECIAL_MUX | 1 |
| xe "MUX_OPTION"§MUX_OPTION | 0 or 8 |
| xe "SPECIAL_SERVICE"§SPECIAL_SERVICE | 1 |
| xe "SERVICE_OPTION"§SERVICE_OPTION | 0 or 16 |
| xe "DIGIT_MODE"§DIGIT_MODE | 1 |
| xe "NUMBER_TYPE"§NUMBER_TYPE | 0 or 3 |
| xe "NUMBER_PLAN"§NUMBER_PLAN | 0 or 4 |
| xe "MORE_FIELDS"§MORE_FIELDS | 1 |
| xe "NUM_FIELDS"§NUM_FIELDS | 8 |
| NUM_FIELDS occurrences of the following field: |  |
| xe "CHARi"§CHARi | 4 or 8 |
| xe "RESERVED"§RESERVED | 0 - 7 (as needed) |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00000100’.

xe "ACK_SEQ"§ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.1.3.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.1.3.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.1.3.1.

xe "VALID_ACK"§VALID_ACK	-	Valid acknowledgement indicator.

See 6.7.1.3.1.

xe "ESN"§ESN	-	Mobile station’s xe "electronic serial number"§electronic serial number.

See 6.7.1.3.1.

xe "AUTH_MODE"§AUTH_MODE	-	Authentication mode.

See 6.7.1.3.2.

xe "AUTHR"§AUTHR	-	Authentication data.

See 6.7.1.3.2.

xe "RANDC"§RANDC	-	Random challenge value.

See 6.7.1.3.2.

xe "COUNT"§COUNT	-	Call history parameter.

See 6.7.1.3.2.

xe "MIN1"§MIN1	-	First part of the xe "mobile station identification number"§mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN1 value of the MIN originating the call.

xe "MIN2"§MIN2	-	Second part of the xe "mobile station identification number"§mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN2 value of the MIN originating the call.

xe "SCM"§SCM	-	Station class mark.

The mobile station shall set this field to the xe "station class mark "§station class mark of the mobile station.  See 2.3.3.

xe "REQUEST_MODE"§REQUEST_MODE	-	Requested mode code.  The mobile station shall set this field to the value shown in Table 6.7.1.3.2.4-1 corresponding to its current configuration.

tc "Table 6.7.1.3.2.4-1.  REQUEST_MODE Codes"\c 	§Table 6.7.1.3.2.4-1.  REQUEST_MODE Codes

| Value (binary) | Requested Mode |
| --- | --- |
| 001 | CDMA only |
| 010 | Analog only |
| 011 | Either CDMA or analog |
| All other REQUEST_MODE codes are reserved. |  |

xe "SPECIAL_MUX"§SPECIAL_MUX	-	xe "Special multiplex option "§Special multiplex option indicator.

To request a special frame format or xe "multiplex rule"§multiplex rule for this call, the mobile station shall set this field to ‘1’.  To request the default multiplex option (xe "Multiplex Option 1"§Multiplex Option 1) for this call, the mobile station shall set this field to ‘0’.

xe "MUX_OPTION"§MUX_OPTION	-	xe "Multiplex option "§Multiplex option for this call.

If the SPECIAL_MUX field is set to ‘1’, the mobile station shall set this field to the value shown in Table 6.7.1.3.2.4-2 corresponding to the requested multiplex option.  If the SPECIAL_MUX field is set to ‘0’, the mobile station shall omit this field.

tc "Table 6.7.1.3.2.4-2.  Multiplex Options"\c 	§Table 6.7.1.3.2.4-2.  Multiplex Options

| Value (binary) | Multiplex Option |
| --- | --- |
| 00000001 | Multiplex Option 1 |
| All other Multiplex Option codes are reserved. |  |

xe "SPECIAL_SERVICE"§SPECIAL_SERVICE	-	Special service option indicator.

To request a xe "special service option"§special service option, the mobile station shall set this field to ‘1’.  To request the default service option (xe "Service Option 1"§Service Option 1), the mobile station shall set this field to ‘0’.

xe "SERVICE_OPTION"§SERVICE_OPTION	-	Requested xe "service option"§service option for this call.

If the SPECIAL_SERVICE field is set to ‘1’, the mobile station shall set this field to the value shown in Table A-1 corresponding to the requested service option.  If the SPECIAL_SERVICE field is set to ‘0’, the mobile station shall omit this field.

xe "DIGIT_MODE"§DIGIT_MODE	-	Digit mode indicator.

This field indicates whether the dialed digits are 4-bit DTMF codes using the Unknown numbering plan, or 8-bit ASCII codes using a specified numbering plan.

To originate the call using the binary representation of DTMF digits, the mobile station shall set this field to ‘0’.  To originate the call using ASCII characters, the mobile station shall set this field to ‘1’.

NUMBER_TYPE	-	Type of number.

If the DIGIT_MODE field is set to ‘1’, the mobile station shall set this field to the NUMBER_TYPE value shown in Table 6.7.1.3.2.4-3 corresponding to the type of the number as defined in ANSI T1.607 §4.5.9.  If the DIGIT_MODE field is set to ‘0’, the mobile station shall omit this field.

tc "Table 6.7.1.3.2.4-3.  Number Types"\c 	§Table 6.7.1.3.2.4-3.  Number Types

| Description | xe "NUMBER_TYPE"§NUMBER_TYPE (binary) |
| --- | --- |
| Unknown | 000 |
| International number | 001 |
| National number | 010 |
| Network-specific number | 011 |
| Subscriber number | 100 |
| Reserved | 101 |
| Abbreviated number | 110 |
| Reserved for extension | 111 |

xe "NUMBER_PLAN"§NUMBER_PLAN	-	xe "Numbering plan"§Numbering plan.

If the DIGIT_MODE field is set to ‘1’, the mobile station shall set this field to the NUMBER_PLAN value shown in Table 6.7.1.3.2.4-4 corresponding to the requested numbering plan.  If the DIGIT_MODE field is set to ‘0’, the mobile station shall omit this field.

tc "Table 6.7.1.3.2.4-4.  Numbering Plan Identification (DIGIT_MODE =  ‘1’)"\c 	§Table 6.7.1.3.2.4-4.  Numbering Plan Identification (DIGIT_MODE= 1) (See xe "ANSI T1.607 "§ANSI T1.607 §4.5.9)

| Description | xe "NUMBER_PLAN"§NUMBER_PLAN (binary) |
| --- | --- |
| Unknown | 0000 |
| ISDN/Telephony numbering plan (xe "CCITT E.164 "§CCITT E.164 and xe "CCITT E.163)"§CCITT E.163) | 0001 |
| Data numbering plan (xe "CCITT X.121)"§CCITT X.121) | 0011 |
| xe "Telex numbering plan "§Telex numbering plan (xe "CCITT F.69)"§CCITT F.69) | 0100 |
| Private numbering plan | 1001 |
| Reserved for extension | 1111 |
| All other xe "NUMBER_PLAN"§NUMBER_PLAN codes are reserved. |  |

xe "MORE_FIELDS"§MORE_FIELDS	-	More dialed digits indicator.

This field indicates whether additional dialed digits will be sent in a later xe "Origination Continuation Message"§Origination Continuation Message.

If all dialed digits will fit in this message, the mobile station shall set this field to ‘0’.  If not, the mobile station shall set this field to ‘1’.

xe "NUM_FIELDS"§NUM_FIELDS	-	Number of dialed digits in this message.

The mobile station shall set this field to the number of dialed digits included in this message.

xe "CHARi"§CHARi	-	A dialed digit or character.

The mobile station shall include NUM_FIELDS occurrences of this field.  If the DIGIT_MODE field is set to ‘0’, the mobile station shall set each occurrence of this field to the code value shown in Table 6.7.1.3.2.4-5 corresponding to the dialed digit.  If the DIGIT_MODE field is set to ‘1’, the mobile station shall set each occurrence of this field to the ASCII representation corresponding to the dialed digit, as specified in ANSI X3.4, with the most significant bit set to ‘0’.

tc "Table 6.7.1.3.2.4-5.  Representation of DTMF Digits"\c 	§Table 6.7.1.3.2.4-5.  Representation of DTMF Digits

| Digit | Code (binary) | Digit | Code (binary) |
| --- | --- | --- | --- |
| 1 | 0001 | 7 | 0111 |
| 2 | 0010 | 8 | 1000 |
| 3 | 0011 | 9 | 1001 |
| 4 | 0100 | 0 | 1010 |
| 5 | 0101 | * | 1011 |
| 6 | 0110 | # | 1100 |
| All other codes are reserved. |  |  |  |

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall add reserved bits as needed in order to make the length of the entire message equal to an integer number of octets.  The mobile station shall set these bits to ‘0’.

###### 6.7.1.3.2.5 Page Response Message xe "mobile station, signaling formats, Access Channel, Page Response Message"§

When the mobile station sends a xe "Page Response Message"§Page Response Message, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00000101’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 2 |
| xe "MSG_SEQ"§MSG_SEQ | 2 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "VALID_ACK"§VALID_ACK | 1 |
| xe "ESN"§ESN | 32 |
| xe "AUTH_MODE"§AUTH_MODE | 2 |
| xe "AUTHR"§AUTHR | 0 or 18 |
| xe "RANDC"§RANDC | 0 or 8 |
| xe "COUNT"§COUNT | 0 or 6 |
| xe "MIN1"§MIN1 | 24 |
| xe "MIN2"§MIN2 | 10 |
| xe "SCM"§SCM | 8 |
| xe "REQUEST_MODE"§REQUEST_MODE | 3 |
| xe "RESERVED"§RESERVED | 5 |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00000101’.

xe "ACK_SEQ"§ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.1.3.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.1.3.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.1.3.1.

xe "VALID_ACK"§VALID_ACK	-	Valid acknowledgement indicator.

See 6.7.1.3.1.

xe "ESN"§ESN	-	Mobile station’s xe "electronic serial number"§electronic serial number.

See 6.7.1.3.1.

xe "AUTH_MODE"§AUTH_MODE	-	Authentication mode.

See 6.7.1.3.2.

xe "AUTHR"§AUTHR	-	Authentication data.

See 6.7.1.3.2.

xe "RANDC"§RANDC	-	Random challenge value.

See 6.7.1.3.2.

xe "COUNT"§COUNT	-	Call history parameter.

See 6.7.1.3.2.

xe "MIN1"§MIN1	-	First part of the xe "mobile station identification number"§mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN1 value from the xe "Page Message "§Page Message or xe "Slotted Page Message "§Slotted Page Message to which this message is a response.

xe "MIN2"§MIN2	-	Second part of the xe "mobile station identification number"§mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN2 value from the xe "Page Message "§Page Message or xe "Slotted Page Message "§Slotted Page Message to which this message is a response.

xe "SCM"§SCM	-	Station class mark.

The mobile station shall set this field to the xe "station class mark "§station class mark of the mobile station.  See 2.3.3.

xe "REQUEST_MODE"§REQUEST_MODE	-	Requested mode code.  The mobile station shall set this field to the value shown in Table 6.7.1.3.2.4-1 corresponding to its current configuration.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall set this field to ‘00000’.

###### 6.7.1.3.2.6 Registration Update Message xe "mobile station, signaling formats, Access Channel, Registration Update Message"§

When the mobile station sends a xe "Registration Update Message "§Registration Update Message on the Access Channel, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00000110’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 2 |
| xe "MSG_SEQ"§MSG_SEQ | 2 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "VALID_ACK"§VALID_ACK | 1 |
| xe "ESN"§ESN | 32 |
| xe "AUTH_MODE"§AUTH_MODE | 2 |
| xe "AUTHR"§AUTHR | 0 or 18 |
| xe "RANDC"§RANDC | 0 or 8 |
| xe "COUNT"§COUNT | 0 or 6 |
| xe "REG_TYPE"§REG_TYPE | 4 |
| xe "MORE_MINS"§MORE_MINS | 1 |
| SCM | 8 |
| xe "SLOT_CYCLE_INDEX"§SLOT_CYCLE_INDEX | 3 |
| Zero or more occurrences of the following record: |  |
| xe "ADD_DEL"§ADD_DEL | 1 |
| xe "MIN1"§MIN1 | 24 |
| xe "MIN2"§MIN2 | 10 |
| xe "HOME_SID"§HOME_SID | 15 |
| MOB_TERM | 1 |
| xe "RESERVED"§RESERVED | 0-7 (as needed) |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00000110’.

xe "ACK_SEQ"§ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.1.3.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.1.3.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.1.3.1.

xe "VALID_ACK"§VALID_ACK	-	Valid acknowledgement indicator.

See 6.7.1.3.1.

xe "ESN"§ESN	-	Mobile station’s xe "electronic serial number"§electronic serial number.

See 6.7.1.3.1.

xe "AUTH_MODE"§AUTH_MODE	-	Authentication mode.

See 6.7.1.3.2.

xe "AUTHR"§AUTHR	-	Authentication data.

See 6.7.1.3.2.

xe "RANDC"§RANDC	-	Random challenge value.

See 6.7.1.3.2.

xe "COUNT"§COUNT	-	Call history parameter.

See 6.7.1.3.2.

xe "REG_TYPE"§REG_TYPE	-	Registration type.

This field indicates which type of event generated the registration attempt.

The mobile station shall set this field to the REG_TYPE value shown in Table 6.7.1.3.2.1-1 corresponding to the event which caused this registration to occur.

xe "MORE_MINS"§MORE_MINS	-	More MINs to follow indicator.

This indicator specifies whether additional MINs are to be sent in later xe "Registration Update Messages"§Registration Update Messages.

If all MINs to be added or deleted will fit in this message, the mobile station shall set this field to ‘0’.  If not, the mobile station shall set this field to ‘1’.

xe 	".i.SCM"§SCM	-	Station class mark.

The mobile station shall set this field to its xe "station class mark.  See 2.3.3."§station class mark.  See 2.3.3.

SLOT_CYCLE_INDEX;	-	Slot cycle mode.

If the mobile station is operating in non-slotted mode, it shall set this field to ‘000’.  If the mobile station is operating in slotted mode, it shall set this field to the SLOT_CYCLE_INDEX value in use.  See 6.6.2.1.1.

The mobile station shall include one occurrence of the following five-field record for each MIN included in this message.

xe "ADD_DEL"§ADD_DEL	-	Add/Delete indicator.

This field indicates whether the MIN in this record is to be added to or deleted from the list of active MINs for this mobile station.

To request that the MIN in this record be added to the list, the mobile station shall set this field to ‘1’.  To request that the MIN in this record be deleted from the list, the mobile station shall set this field to ‘0’.

xe "MIN1"§MIN1	-	First part of the xe "mobile station identification number"§mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN1 value for the MIN specified by this record.

xe "MIN2"§MIN2	-	Second part of the xe "mobile station identification number"§mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN2 value for the MIN specified by this record.

xe "HOME_SID"§HOME_SID	-	Home xe "system identification"§system identification.

The mobile station shall set this field to the xe "SID "§SID of the home cellular system for the MIN specified by this record.

.	MOB_TERM	-	Mobile terminated calls accepted indicator.

If the MIN specified by this record is configured to accept mobile terminated calls while operating with the current roaming status (see 6.6.5.3), the mobile station shall set this bit to ‘1’.  Otherwise, the mobile station shall set this bit to ‘0’.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall add reserved bits as needed in order to make the length of the entire message equal to an integer number of octets.  The mobile station shall set these bits to ‘0’.

###### 6.7.1.3.2.7 Authentication Challenge Response Message xe "mobile station, signaling formats, Access Channel, Authentication Challenge Response Message"§

When the mobile station sends an xe "Authentication Challenge Response Message "§Authentication Challenge Response Message on the Access Channel, it shall use the following fixed-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00000111’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 2 |
| xe "MSG_SEQ"§MSG_SEQ | 2 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "VALID_ACK"§VALID_ACK | 1 |
| xe "ESN"§ESN | 32 |
| xe "RESERVED"§RESERVED | 2 |
| xe "AUTH_MIN1"§AUTH_MIN1 | 24 |
| xe "AUTH_MIN2"§AUTH_MIN2 | 10 |
| xe "AUTHU"§AUTHU | 0 or 18 |
| xe "RESERVED"§RESERVED | 6 |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00000111’.

xe "ACK_SEQ"§ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.1.3.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.1.3.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.1.3.1.

xe "VALID_ACK"§VALID_ACK	-	Valid acknowledgement indicator.

See 6.7.1.3.1.

xe "ESN"§ESN	-	Mobile station’s xe "electronic serial number"§electronic serial number.

See 6.7.1.3.1.

xe "RESERVED"§RESERVED	-	Reserved bits.

These bits take the place of the xe "AUTH_MODE "§AUTH_MODE field.

The mobile station shall set this field to ‘00’.

xe "AUTH_MIN1"§AUTH_MIN1	-	First part of the xe "mobile station identification number"§mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the value of the MIN1 field from the xe "Authentication Challenge Message "§Authentication Challenge Message to which this message is a response.

xe "AUTH_MIN2"§AUTH_MIN2	-	Second part of the xe "mobile station identification number"§mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the value of the MIN2 field from the xe "Authentication Challenge Message "§Authentication Challenge Message to which this message is a response.

xe "AUTHU"§AUTHU	-	Authentication challenge response.

The mobile station shall set this field as specified in 6.3.12.1.5.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall set this field to ‘000000’.

### 6.7.2 Reverse Traffic Channel xe "mobile station, signaling formats, Reverse Traffic Channel"§

The xe "Reverse Traffic Channel"§Reverse Traffic Channel is used to send signaling information to the base station during Traffic Channel operation.

#### 6.7.2.1 Reverse Traffic Channel Structure

When sending a message on the Reverse Traffic Channel, the mobile station may use any combination of blank-and-burst and dim-and-burst information bit structures (see 6.1.3.3.11) that allocates enough signaling traffic bits to contain the message.  If the number of allocated bits exceeds the number of bits in the message, the mobile station shall transmit the message beginning with the first allocated bit.  In the remaining allocated bits, the mobile station shall transmit padding bits.  The mobile station shall set the padding bits to ‘0’.

#### 6.7.2.2 Reverse Traffic Channel Message Structure xe "mobile station, signaling formats, Reverse Traffic Channel, message structure"§

A Reverse Traffic Channel message capsule consists of a Reverse Traffic Channel message and padding, as shown in Figure 6.7.2.2-1.

Each Traffic Channel message shall consist of a xe "length field"§length field (MSG_LENGTH), a message body, and a xe "CRC"§CRC, in that order.

tc "Figure 6.7.2.2-1.  Reverse Traffic Channel Message Structure"\c §Figure 6.7.2.2-1.  Reverse Traffic Channel Message Structure

##### 6.7.2.2.1 Reverse Traffic Channel MSG_LENGTH Field xe "mobile station, signaling formats, Reverse Traffic Channel, MSG_LENGTH field"§

The mobile station shall set the MSG_LENGTH field of each Reverse Traffic Channel signaling message to the length of the message in octets, including the MSG_LENGTH field, the message body, and the CRC.  The MSG_LENGTH field shall be 8 bits in length.

##### 6.7.2.2.2 Reverse Traffic Channel Signaling Message CRC xe "mobile station, signaling formats, Reverse Traffic Channel, signaling message CRC"§

A 16-bit xe "CRC"§CRC shall be computed for each Reverse Traffic Channel signaling message.  The CRC shall include the xe "MSG_LENGTH"§MSG_LENGTH field and the xe "message body"§message body.  The generator polynomial for the CRC shall be the standard CRC-CCITT polynomial:

g(x) = x16 + x12 + x5 + 1.

The CRC shall be the value computed by the following procedure and the logic shown in Figure 6.7.2.2.2-1:

•	All shift register elements shall be initialized to logical one.

•	The switches shall be set in the up position.

•	The information bit count k shall be defined as 8 + message body length in bits.

•	The register shall be clocked k times, with the length and message body of the message as the k input bits.

•	The switches shall be set in the down position.

•	The register shall be clocked an additional 16 times.

•	The 16 additional output bits shall be the check bits.

•	The bits shall be transmitted in the order in which they are calculated.

tc "Figure 6.7.2.2.2-1.  Reverse Traffic Channel Signaling CRC Calculation"\c §Figure 6.7.2.2.2-1.  Reverse Traffic Channel Signaling CRC Calculation

#### 6.7.2.3 Reverse Traffic Channel Message Body Format xe "mobile station, signaling formats, Reverse Traffic Channel"§

The xe "Reverse Traffic Channel message"§Reverse Traffic Channel messages are summarized in Table 6.7.2.3-1.

tc "Table 6.7.2.3-1.  Reverse Traffic Channel Messages "\c 	§Table 6.7.2.3-1.  Reverse Traffic Channel Messages

| Message Name | Message Type (binary) |
| --- | --- |
| xe "Order Message"§Order Message | 00000001 |
| xe "Authentication Challenge Response Message"§Authentication Challenge Response Message | 00000010 |
| xe "Flash With Information Message"§Flash With Information Message | 00000011 |
| xe "Data Burst Message"§Data Burst Message | 00000100 |
| xe "Pilot Strength Measurement Message"§Pilot Strength Measurement Message | 00000101 |
| xe "Power Measurement Report Message"§Power Measurement Report Message | 00000110 |
| xe "Send Burst DTMF Message"§Send Burst DTMF Message | 00000111 |
| xe "Status Message"§Status Message | 00001000 |
| xe "Origination Continuation Message"§Origination Continuation Message | 00001001 |
| xe "Handoff Completion Message"§Handoff Completion Message | 00001010 |
| xe "Parameters Response Message"§Parameters Response Message | 00001011 |

##### 6.7.2.3.1 Common Fields

###### 6.7.2.3.1.1 Common Acknowledgement Fields xe "mobile station, signaling formats, Reverse Traffic Channel"§

All Reverse Traffic Channel messages share the same three acknowledgement fields:

xe "ACK_SEQ"§ACK_SEQ	-	Acknowledgement sequence number.

The mobile station shall set this field to the value of the MSG_SEQ field from the most recently received Forward Traffic Channel message requiring acknowledgement.  If no such message has been received, the mobile station shall set this field to ‘111’.  See 6.6.4.1.3.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

The mobile station shall set this field to the message sequence number for this message.  See 6.6.4.1.3.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

This field indicates whether this message requires an acknowledgement.

To indicate that this message requires acknowledgement, the mobile station shall set this field to ‘1’.  To indicate that this message does not require acknowledgement, the mobile station shall set this field to ‘0’.

###### 6.7.2.3.1.2 Common Encryption Field xe "mobile station, signaling formats, Reverse Traffic Channel"§

Many Reverse Traffic Channel messages contain the following field:

xe "ENCRYPTED"§ENCRYPTED	-	Message encryption indicator.

If this message is encrypted, the mobile station shall set this field to ‘1’.  If this message is not encrypted, the mobile station shall set this field to ‘0’.  See 6.3.12.2.

##### 6.7.2.3.2 Message Body Contents xe "mobile station, signaling formats, Reverse Traffic Channel, message body"§

The following sections specify the contents of the message body for each message that may be sent on the Reverse Traffic Channel.

###### 6.7.2.3.2.1 Order Messagexe "mobile station, signaling formats, Reverse Traffic Channel, Order Message"§

When the mobile station sends an xe "Order Message "§Order Message on the Reverse Traffic Channel, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00000001’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 3 |
| xe "MSG_SEQ"§MSG_SEQ | 3 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "ENCRYPTED"§ENCRYPTED | 1 |
| xe "ORDER"§ORDER | 6 |
| xe "ADD_RECORD_LEN"§ADD_RECORD_LEN | 3 |
| Order-specific fields (if used) | 8 ¥ ADD_RECORD_LEN |
| xe "RESERVED"§RESERVED | 7 |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00000001’.

ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.2.3.1.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.2.3.1.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.2.3.1.1.

xe "ENCRYPTED"§ENCRYPTED	-	Message encryption indicator.

See 6.7.2.3.1.2.  The mobile station shall set this field to ‘0’ unless encryption is required for this type of order.

xe "ORDER"§ORDER	-	Order code.

The mobile station shall set this field to the ORDER code.  See 6.7.3.

xe "ADD_RECORD_LEN"§ADD_RECORD_LEN	-	Additional record length.

The mobile station shall set this field to the number of octets in the order-specific fields included in this message.

Order-specific fields	-	Order-specific fields.

The mobile station shall include order-specific fields as specified in 6.7.3.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall set this field to ‘0000000’.

###### 6.7.2.3.2.2 Authentication Challenge Response Messagexe "mobile station, signaling formats, Reverse Traffic Channel, Authentication Challenge Response Message"§

When the mobile station sends an xe "Authentication Challenge Response Message "§Authentication Challenge Response Message on the Reverse Traffic Channel, it shall use the following fixed-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00000010’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 3 |
| xe "MSG_SEQ"§MSG_SEQ | 3 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "RESERVED"§RESERVED | 1 |
| xe "AUTHU"§AUTHU | 18 |
| xe "RESERVED"§RESERVED | 6 |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00000010’.

ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.2.3.1.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.2.3.1.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.2.3.1.1.

xe "RESERVED"§RESERVED	-	Reserved bit.

This field takes the place of the ENCRYPTED bit.

The mobile station shall set this field to ‘0’.

xe "AUTHU"§AUTHU	-	Authentication challenge response.

The mobile station shall set this field as specified in 6.3.12.1.5.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall set this field to ‘000000’.

###### 6.7.2.3.2.3 Flash With Information Message xe "mobile station, signaling formats, Reverse Traffic Channel, Flash With Information Message"§

When the mobile station sends a xe "Flash With Information Message"§Flash With Information Message, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00000011’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 3 |
| xe "MSG_SEQ"§MSG_SEQ | 3 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "ENCRYPTED"§ENCRYPTED | 1 |
| One or more occurrences of the following record: |  |
| xe "RECORD_TYPE"§RECORD_TYPE | 8 |
| xe "RECORD_LEN"§RECORD_LEN | 6 |
| xe "Type-specific fields"§Type-specific fields | 8 ¥ RECORD_LEN |
| xe "RESERVED"§RESERVED | 0 - 7 (as needed) |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00000011’.

ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.2.3.1.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.2.3.1.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.2.3.1.1.

xe "ENCRYPTED"§ENCRYPTED	-	Message encryption indicator.

See 6.7.2.3.1.2.

The mobile station shall include one occurrence of the following record for each information record to be included:

xe "RECORD_TYPE"§RECORD_TYPE	-	xe "Information record "§Information record type.

The mobile station shall set this field to the record type code shown in Table 6.7.4-1 corresponding to the type of this information record.

xe "RECORD_LEN"§RECORD_LEN	-	Information record length.

The mobile station shall set this field to the number of octets in the type-specific fields of this record.

Type-specific fields;	-	Type-specific fields.

The mobile station shall set these fields as specified in 6.7.4 for this type of information record.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall add reserved bits as needed in order to make the length of the entire message equal to an integer number of octets.  The mobile station shall set these bits to ‘0’.

###### 6.7.2.3.2.4 Data Burst Message xe "mobile station, signaling formats, Reverse Traffic Channel, One-Way Data Burst Message"§

When the mobile station sends a xe "Data Burst Message"§Data Burst Message on the Reverse Traffic Channel, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00000100’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 3 |
| xe "MSG_SEQ"§MSG_SEQ | 3 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "ENCRYPTED"§ENCRYPTED | 1 |
| xe "MSG_NUMBER"§MSG_NUMBER | 8 |
| xe "MIN1"§MIN1 | 24 |
| xe "MIN2"§MIN2 | 10 |
| xe "BURST_TYPE"§BURST_TYPE | 6 |
| xe "NUM_MSGS"§NUM_MSGS | 8 |
| xe "NUM_FIELDS"§NUM_FIELDS | 8 |
| NUM_FIELDS occurrences of the following field: |  |
| xe "CHARi"§CHARi | 8 |
| xe "RESERVED"§RESERVED | 0 |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00000100’.

ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.2.3.1.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.2.3.1.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.2.3.1.1.

xe "ENCRYPTED"§ENCRYPTED	-	Message encryption indicator.

See 6.7.2.3.1.2.

xe "MSG_NUMBER"§MSG_NUMBER	-	Message number within the data burst stream.

The mobile station shall set this field to the number of this message within the data burst stream.

xe "MIN1"§MIN1	-	First part of the mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN1 value for the MIN sending this message.

xe "MIN2"§MIN2	-	Second part of the mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN2 value for the MIN sending this message.

xe "BURST_TYPE"§BURST_TYPE	-	Data burst type.

The mobile station shall set this field to the value shown in Table 6.7.1.3.2.3-1 for the type of this data burst.

xe "NUM_MSGS"§NUM_MSGS	-	Number of messages in the data burst stream.

The mobile station shall set this field to the number of messages within this data burst stream.

xe "NUM_FIELDS"§NUM_FIELDS	-	Number of characters in this message.

The mobile station shall set this field to the number of CHARi fields included in this message.

xe "CHARi"§CHARi	-	Character.

The mobile station shall include NUM_FIELDS occurrences of this field.  The mobile station shall set these fields to the corresponding octet of the data burst stream.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall omit this field.

###### 6.7.2.3.2.5 Pilot Strength Measurement Message xe "mobile station, signaling formats, Reverse Traffic Channel, Pilot Strength Measurement Message"§

When the mobile station sends a xe "Pilot Strength Measurement Message"§Pilot Strength Measurement Message, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00000101’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 3 |
| xe "MSG_SEQ"§MSG_SEQ | 3 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "RESERVED"§RESERVED | 1 |
| xe "REF_PN"§REF_PN | 9 |
| xe "PILOT_STRENGTH"§PILOT_STRENGTH | 6 |

Zero or more occurrences of the following record:

| xe "PILOT_PN_PHASE"§PILOT_PN_PHASE | 15 |
| --- | --- |
| xe "PILOT_STRENGTH"§PILOT_STRENGTH | 6 |
| xe "KEEP"§KEEP | 1 |
| xe "RESERVED"§RESERVED | 0 - 7 as needed |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00000101’.

ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.2.3.1.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.2.3.1.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.2.3.1.1.

xe "RESERVED"§RESERVED	-	Reserved bit.

This field takes the place of the ENCRYPTED field.

The mobile station shall set this field to '0'.

xe "REF_PN"§REF_PN	-	Time reference PN sequence offset.

The mobile station shall set this field to the PN sequence offset of the pilot used by the mobile station to derive its time reference, relative to the xe "zero offset pilot PN sequence"§zero offset pilot PN sequence, in units of 64 PN chips.

xe "PILOT_STRENGTH"§PILOT_STRENGTH	-	Pilot strength.

The mobile station shall set this field to

Î- 2 ¥ 10 ¥ log10PS _,

where PS is the strength of the pilot used by the mobile station to derive its time reference (see 6.1.5.1), measured as specified in 6.6.6.2.2.  If this value is less than 0, the mobile station shall set this field to 0.  If this value is greater than 63, the mobile station shall set this field to 63.

The mobile station shall include one occurrence of the following three-field record for each pilot in the xe "Active Set "§Active Set and for each pilot in the xe "Candidate Set, other than the pilot identified by the REF_PN field."§Candidate Set, other than the pilot identified by the REF_PN field.

xe "PILOT_PN_PHASE"§PILOT_PN_PHASE	-	Pilot measured phase.

The mobile station shall set this field to the phase of the xe "pilot PN sequence "§pilot PN sequence relative to the xe "zero offset pilot "§zero offset pilot PN sequence of this pilot, in units of one PN chip, as specified in 6.6.6.2.4.

xe "PILOT_STRENGTH"§PILOT_STRENGTH	-	Pilot strength.

The mobile station shall set this field to

Î- 2 ¥ 10 ¥ log10PS _,

where PS is the strength of this pilot, measured as specified in 6.6.6.2.2.  If this value is less than 0, the mobile station shall set this field to 0.  If this value is greater than 63, the mobile station shall set this field to 63.

xe "KEEP"§KEEP	-	Keep pilot indicator.

For pilots in the Active Set, this field indicates whether the mobile station recommends that this pilot be kept in the xe "Active Set "§Active Set or not.  For pilots in the xe "Candidate Set, "§Candidate Set, this field always indicates that the mobile station recommends adding this pilot to the Active Set.

If the strength of this pilot has exceeded T_DROP within the last interval specified by T_TDROP, as specified in 6.6.6.2.3, the mobile station shall set this field to ‘1’.  Otherwise, the mobile station shall set this field to ‘0’.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall add reserved bits as needed in order to make the length of the entire message equal to an integer number of octets.  The mobile station shall set these bits to ‘0’.

###### 6.7.2.3.2.6 Power Measurement Report Message xe "mobile station, signaling formats, Reverse Traffic Channel, Power Measurement Report Message"§

When the mobile station sends a xe "Power Measurement Report Message"§Power Measurement Report Message, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00000110’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 3 |
| xe "MSG_SEQ"§MSG_SEQ | 3 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "RESERVED"§RESERVED | 1 |
| xe "ERRORS_DETECTED"§ERRORS_DETECTED | 5 |
| xe "PWR_MEAS_FRAMES"§PWR_MEAS_FRAMES | 10 |
| xe "LAST_HDM_SEQ"§LAST_HDM_SEQ | 2 |
| xe "NUM_PILOTS"§NUM_PILOTS | 4 |
| NUM_PILOTS occurrences of the following field: |  |
| xe "PILOT_STRENGTH"§PILOT_STRENGTH | 6 |
| xe "RESERVED"§RESERVED | 0 - 7 (as needed) |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00000110’.

ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.2.3.1.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.2.3.1.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.2.3.1.1.

xe "RESERVED"§RESERVED	-	Reserved bit.

This field takes the place of the ENCRYPTED field.

The mobile station shall set this field to '0'.

xe "ERRORS_DETECTED"§ERRORS_DETECTED	-	Number of frame errors detected.

If the number of bad frames (see 6.2.2.2) received in the measurement period is less than or equal to 31, the mobile station shall set this field to that number.  If that number exceeds 31, the mobile station shall set this field to ‘11111’.

xe "PWR_MEAS_FRAMES"§PWR_MEAS_FRAMES	-	Number of xe "Forward Traffic Channel frame"§Forward Traffic Channel frames in the measurement period.

The mobile station shall set this field to the number of Forward Traffic Channel frames in the measurement period (see 6.6.4.1.1).

xe "LAST_HDM_SEQ"§LAST_HDM_SEQ	-	Handoff Direction Message sequence number.

If a Handoff Direction Message has been received during this call, the mobile station shall set this field to the value of the HDM_SEQ field from the Handoff Direction Message that determined the current Active Set.  If no Handoff Direction Message has been received during this call, the mobile station shall set this field to ‘11’.

xe "NUM_PILOTS	-	Number of pilots reported."§NUM_PILOTS-Number of pilots reported.

The mobile station shall set this field to the number of pilots in the current Active Set.

xe "PILOT_STRENGTH"§PILOT_STRENGTH	-	Pilot strength.

The mobile station shall include one occurrence of this field for each pilot in the Active Set.  If the Active Set contains more than one pilot, the mobile station shall include the pilot strengths in the same order as in the Handoff Direction Message that determined the current Active Set.

The mobile station shall set each occurrence of this field to

Î- 2 ¥ 10 ¥ log10PS _,

where PS is the strength of the pilot, measured as specified in 6.6.6.2.2.  If this value is less than 0, the mobile station shall set this field to 0.  If this value is greater than 63, the mobile station shall set this field to 63.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall add reserved bits as needed in order to make the length of the entire message equal to an integer number of octets.  The mobile station shall set these bits to ‘0’.

###### 6.7.2.3.2.7 Send Burst DTMF Message xe "mobile station, signaling formats, Reverse Traffic Channel, Send Burst DTMF Message"§

When the mobile station sends a xe "Send Burst DTMF Message"§Send Burst DTMF Message, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00000111’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 3 |
| xe "MSG_SEQ"§MSG_SEQ | 3 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "ENCRYPTED"§ENCRYPTED | 1 |
| xe "NUM_DIGITS"§NUM_DIGITS | 8 |
| xe "DTMF_ON_LENGTH"§DTMF_ON_LENGTH | 3 |
| xe "DTMF_OFF_LENGTH"§DTMF_OFF_LENGTH | 3 |
| NUM_DIGITS occurrences of the following field: |  |
| xe "DIGITi"§DIGITi | 4 |
| xe "RESERVED"§RESERVED | 0 - 7 (as needed) |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00000111’.

ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.2.3.1.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.2.3.1.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.2.3.1.1.

xe "ENCRYPTED"§ENCRYPTED	-	Message encryption indicator.

See 6.7.2.3.1.2.

xe "NUM_DIGITS"§NUM_DIGITS	-	Number of DTMF digits.

The mobile station shall set this field to the number of DTMF digits included in this message.

xe "DTMF_ON_LENGTH"§DTMF_ON_LENGTH	-	DTMF pulse width code.

The mobile station shall set this field to the DTMF_-ON_LENGTH value shown in Table 6.7.2.3.2.7-1 corresponding to the requested width of DTMF pulses to be generated by the base station.

tc "Table 6.7.2.3.2.7-1.  Recommended DTMF Pulse Width"\c 	§Table 6.7.2.3.2.7-1.  Recommended DTMF Pulse Width

| DTMF_ON_LENGTH Field (binary) | Recommended Pulse Width |
| --- | --- |
| 000 | 95 ms |
| 001 | 150 ms |
| 010 | 200 ms |
| 011 | 250 ms |
| 100 | 300 ms |
| 101 | 350 ms |
| All other DTMF_ON_LENGTH codes are reserved. |  |

xe "DTMF_OFF_LENGTH"§DTMF_OFF_LENGTH	-	DTMF interdigit interval code.

The mobile station shall set this field to the DTMF_OFF_LENGTH value shown in Table 6.7.2.3.2.7-2 corresponding to the requested minimum interval between DTMF pulses to be generated by the base station.

tc "Table 6.7.2.3.2.7-2.  Recommended Minimum Interdigit Interval"\c 	§Table 6.7.2.3.2.7-2.  Recommended Minimum Interdigit Interval

| DTMF_OFF_LENGTH Field (binary) | Recommended Minimum Interdigit Interval |
| --- | --- |
| 000 | 60 ms |
| 001 | 100 ms |
| 010 | 150 ms |
| 011 | 200 ms |
| All other DTMF_OFF_LENGTH codes are reserved. |  |

xe "DIGITi"§DIGITi	-	DTMF digit.

The mobile station shall include one occurrence of this field for each DTMF digit to be generated by the base station.  The mobile station shall set each occurrence of this field to the code value shown in Table 6.7.1.3.2.4-5 corresponding to the dialed digit.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall add reserved bits as needed in order to make the length of the entire message equal to an integer number of octets.  The mobile station shall set these bits to ‘0’.

###### 6.7.2.3.2.8 Status Message xe "mobile station, signaling formats, Reverse Traffic Channel, Status Message"§

When the mobile station sends a xe "Status Message"§Status Message, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE "§MSG_TYPE (‘00001000’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 3 |
| xe "MSG_SEQ"§MSG_SEQ | 3 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "RESERVED"§RESERVED | 1 |
| One or more occurrences of the following record: |  |
| xe "RECORD_TYPE"§RECORD_TYPE | 8 |
| xe "RECORD_LEN "§RECORD_LEN | 6 |
| xe "Type-specific fields"§Type-specific fields | 8 ¥ RECORD_LEN |
| xe "RESERVED"§RESERVED | 0 - 7 (as needed) |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00001000’.

ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.2.3.1.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.2.3.1.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.2.3.1.1.

xe "RESERVED"§RESERVED	-	Reserved bit.

This field takes the place of the ENCRYPTED field.

The mobile station shall set this field to '0'.

The mobile station shall include one occurrence of the following record for each information record to be included:

xe "RECORD_TYPE"§RECORD_TYPE	-	Information record type.

The mobile station shall set this field to the record type value shown in Table 6.7.4-1 corresponding to the type of this information record.

xe "RECORD_LEN"§RECORD_LEN	-	Information record length.

The mobile station shall set this field to the number of octets included in the type-specific fields of this information record.

xe "Type-specific fields"§Type-specific fields	-	Type-specific fields.

The mobile station shall set these fields as specified in 6.7.4 for this type of record.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall add reserved bits as needed in order to make the length of the entire message equal to an integer number of octets.  The mobile station shall set these bits to ‘0’.

###### 6.7.2.3.2.9 Origination Continuation Message

When the mobile station sends an xe "Origination Continuation Message"§Origination Continuation Message, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00001001’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 3 |
| xe "MSG_SEQ"§MSG_SEQ | 3 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "ENCRYPTED"§ENCRYPTED | 1 |
| xe "DIGIT_MODE"§DIGIT_MODE | 1 |
| xe "NUM_FIELDS"§NUM_FIELDS | 8 |
| NUM_FIELDS occurrences of the following field: |  |
| xe "CHARi"§CHARi | 4 or 8 |
| xe "RESERVED"§RESERVED | 0 - 7 (as needed) |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00001001’.

ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.2.3.1.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.2.3.1.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.2.3.1.1.

xe "ENCRYPTED"§ENCRYPTED	-	Message encryption indicator.

See 6.7.2.3.1.2.

xe "DIGIT_MODE"§DIGIT_MODE	-	Digit mode indicator.

The mobile station shall set this field to the DIGIT_MODE value from the xe "Access Channel "§xe Access Channel "Origination Message "§Origination Message for which this message is a continuation.

xe "NUM_FIELDS"§NUM_FIELDS	-	Number of dialed digits in this message.

The mobile station shall set this field to the number of dialed digits included in this message.

xe "CHARi"§CHARi	-	A dialed digit or character.

The mobile station shall include NUM_FIELDS occurrences of this field.  The mobile station shall include occurrences of this field for all dialed digits after those sent in the xe "Access Channel "§xe Access Channel "Origination Message "§Origination Message of which this message is a continuation.  If the DIGIT_MODE field is set to ‘0’, the mobile station shall set each occurrence of this field to the code value shown in Table 6.7.1.3.2.4-5 corresponding to the dialed digit.  If the DIGIT_MODE field is set to ‘1’, the mobile station shall set each occurrence of this field to the ASCII representation corresponding to the dialed digit, as specified in ANSI X3.4, with the most significant bit set to ‘0’.

.	xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall add reserved bits as needed in order to make the length of the entire message equal to an integer number of octets.  The mobile station shall set these bits to ‘0’.

###### 6.7.2.3.2.10 Handoff Completion Message xe "mobile station, signaling formats, Reverse Traffic Channel, Parameters Response Message"§

When the mobile station sends a xe "Handoff Completion Message"§Handoff Completion Message, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00001010’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 3 |
| xe "MSG_SEQ"§MSG_SEQ | 3 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "RESERVED"§RESERVED | 1 |
| xe "LAST_HDM_SEQ"§LAST_HDM_SEQ | 2 |
| xe "NUM_PILOTS"§NUM_PILOTS | 4 |
| NUM_PILOTS occurrences of the following field: |  |
| xe "PILOT_PN"§PILOT_PN | 9 |
| xe "RESERVED"§RESERVED | 0 - 7 (as needed) |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00001010’.

ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.2.3.1.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.2.3.1.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.2.3.1.1.

xe "RESERVED"§RESERVED	-	Reserved bit.

This field takes the place of the ENCRYPTED field.

The mobile station shall set this field to '0'.

xe "LAST_HDM_SEQ"§LAST_HDM_SEQ	-	Handoff Direction Message sequence number.

The mobile station shall set this field to the value of the HDM_SEQ field from the Handoff Direction Message that determined the current Active Set.

xe "NUM_PILOTS	-	Number of pilots reported."§NUM_PILOTS-Number of pilots reported.

The mobile station shall set this field to the number of pilots in the current Active Set.

xe "PILOT_PN"§PILOT_PN	-	Pilot PN sequence offset.

The mobile station shall include one occurrence of this field for each pilot in the current Active Set.  The mobile station shall set this field to the pilot PN sequence offset, relative to the xe "zero offset pilot PN sequence "§zero offset pilot PN sequence in units of 64 PN chips, for this pilot.

.	xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall add reserved bits as needed in order to make the length of the entire message equal to an integer number of octets.  The mobile station shall set these bits to ‘0’.

###### 6.7.2.3.2.11 Parameters Response Message xe "mobile station, signaling formats, Reverse Traffic Channel, Parameters Response Message"§

When the mobile station sends a xe "Parameters Response Message"§Parameters Response Message, it shall use the following variable-length message format:

| Field | Length (bits) |
| --- | --- |
| xe "MSG_TYPE"§MSG_TYPE (‘00001011’) | 8 |
| xe "ACK_SEQ"§ACK_SEQ | 3 |
| xe "MSG_SEQ"§MSG_SEQ | 3 |
| xe "ACK_REQ"§ACK_REQ | 1 |
| xe "ENCRYPTED"§ENCRYPTED | 1 |
| One or more occurrences of the following record: |  |
| xe "PARAMETER_ID"§PARAMETER_ID | 16 |
| xe "PARAMETER_LEN"§PARAMETER_LEN | 10 |
| xe "PARAMETER"§PARAMETER | PARAMETER_LEN + 1 |
| xe "RESERVED"§RESERVED | 0 - 7 (as needed) |

xe "MSG_TYPE"§MSG_TYPE	-	Message type.

The mobile station shall set this field to ‘00001011’.

ACK_SEQ	-	Acknowledgement sequence number.

See 6.7.2.3.1.1.

xe "MSG_SEQ"§MSG_SEQ	-	Message sequence number.

See 6.7.2.3.1.1.

xe "ACK_REQ"§ACK_REQ	-	Acknowledgement required indicator.

See 6.7.2.3.1.1.

xe "ENCRYPTED"§ENCRYPTED	-	Message encryption indicator.

See 6.7.2.3.1.2.

The mobile station shall include one occurrence of the following three-field record for each occurrence of the xe "PARAMETER_ID "§PARAMETER_ID field in the xe "Forward Traffic Channel "§xe Forward Traffic Channel "Retrieve Parameters Message "§Retrieve Parameters Message to which this message is a response.  See Appendix E.

xe "PARAMETER_ID"§PARAMETER_ID	-	Parameter identification.

The mobile station shall set this field to the value of the PARAMETER_ID field for this parameter from the Retrieve Parameters Message to which this message is a response.

PARAMETER_LEN;	-	Parameter length.

The mobile station shall set this field to the length shown in Table E-1 corresponding to this PARAMETER_ID.

xe "PARAMETER"§PARAMETER	-	Parameter value.

The mobile station shall set this parameter as specified in Appendix E for this value of PARAMETER_ID.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall add reserved bits as needed in order to make the length of the entire message equal to an integer number of octets.  The mobile station shall set these bits to ‘0’.

### 6.7.3 Orders xe "mobile station, signaling formats, orders"§

xe "Order Message"§Order Messages are sent by the mobile station on the xe "Access Channel "§Access Channel and on the xe "Reverse Traffic Channel"§Reverse Traffic Channel.  The general format used on the Access Channel is defined in 6.7.1.3.2.2, and the general format used on the Reverse Traffic Channel is defined in 6.7.2.3.2.1.  There are many specific types of xe "Order Message"§Order Message, as shown in Table 6.7.3-1.

The mobile station may send on the Access Channel any type of order shown in Table 6.7.3-1 with a ‘Y’ in the first column, but shall not send on the Access Channel any type of order with an ‘N’ in the first column.  The mobile station may send on the Reverse Traffic Channel any type of order shown in Table 6.7.3-1 with a ‘Y’ in the second column, but shall not send on the Reverse Traffic Channel any type of order with an ‘N’ in the second column.  The mobile station shall be capable of sending all types of orders shown in Table 6.7.3-1 with a ‘Y’ in the sixth column.

An order consists of an order code and order-specific fields.  The order-specific fields consist of an xe "order qualification code"§order qualification code (xe "ORDQ"§ORDQ) and zero or more additional order-specific fields.  The mobile station shall set the xe "ORDER "§ORDER field in the xe "Order Message "§Order Message to the order code shown in Table 6.7.3-1 corresponding to the type of order being sent.

If the order qualification code is ‘00000000’ and there are no other additional fields as shown in Table 6.7.3-1 by an ‘N’ in the fifth column, the mobile station shall include no order-specific fields in the Order Message.  The order qualification code of such a message is implicitly ‘00000000’.

If the order qualification code is not ‘00000000’ and there are no other additional fields as shown in Table 6.7.3-1 by an ‘N’ in the fifth column, the mobile station shall include the order qualification code as the only order-specific field in the Order Message.

If there are other additional fields as shown in Table 6.7.3-1 by a ‘Y’ in the fifth column, the mobile station shall include order-specific fields as specified in the corresponding subsection of this section.

tc "Table 6.7.3-1.  Order and Order Qualification Codes Used on the Reverse Traffic Channel and the Access Channel (Part 1 of 2)"\c 	§Table 6.7.3-1.  Order and Order Qualification Codes Used on the Reverse Traffic Channel and the Access Channel (Part 1 of 2)

| Access Channel Order | Reverse Traffic Channel Order | Order Code, ORDER (binary) | Order Qualification Code, xe "ORDQ"§ORDQ (binary) | More Fields other than ORDQ | SupportReq’d | Name/Function |
| --- | --- | --- | --- | --- | --- | --- |
| Y | Y | 000010 | 00000000 | Y | Y | xe "Base Station Challenge Order"§Base Station Challenge Order |
| Y | Y | 000011 | 00000000 | N | Y | xe "SSD Update Confirmation Order"§SSD Update Confirmation Order |
| Y | Y | 000011 | 00000001 | N | Y | xe "SSD Update Rejection Order "§SSD Update Rejection Order |
| N | Y | 001011 | 00000000 | N | N | xe "Request Analog Service Order"§Request Analog Service Order |
| Y | Y | 010000 | 00000000 | N | Y | xe "Mobile Station Acknowledgement Order"§Mobile Station Acknowledgement Order |
| N | Y | 010011 | 00000000 | Y | N | xe "Service Option Request Order "§Service Option Request Order (see 6.7.3.2) |
| N | Y | 010100 | 00000000 | Y | Y | xe "Service Option Response Order "§Service Option Response Order (see 6.7.3.3) |
| N | Y | 010101 | 00000000 | N | Y | xe "Release Order "§Release Order (normal release) |
| N | Y | 010101 | 00000001 | N | Y | xe "Release Order "§Release Order (with power-down indication) |
| N | Y | 010111 | 00000000 | N | N | xe "Long Code Transition Request Order "§Long Code Transition Request Order (request public) |
| N | Y | 010111 | 00000001 | N | N | xe "Long Code Transitionn Request Order "§Long Code Transitionn Request Order (request private) |
| N | Y | 010111 | 00000010 | N | Y | xe "Long Code Transition Response Order "§Long Code Transition Response Order (use public) |
| N | Y | 010111 | 00000011 | N | N | xe "Long Code Transition Response Order "§Long Code Transition Response Order (use private) |
| N | Y | 011000 | 00000000 | N | Y | xe "Connect Order"§Connect Order |
| N | Y | 011001 | 0000nnnn | N | Y | xe "Continuous DTMF Tone Order "§Continuous DTMF Tone Order (where ‘nnnn’ is the tone per Table 6.7.1.3.2.4-5). |

Table 6.7.3-1.  Order and Order Qualification Codes Used on the Reverse Traffic Channel and the Access Channel (Part 2 of 2)

| Access Channel Order | Reverse Traffic Channel Order | Order Code, ORDER (binary) | Order Qualification Code, xe "ORDQ"§ORDQ (binary) | More Fields other than ORDQ | SupportReq’d | Name/Function |
| --- | --- | --- | --- | --- | --- | --- |
| N | Y | 011001 | 11111111 | N | Y | xe "Continuous DTMF Tone Order "§Continuous DTMF Tone Order (Stop continuous DTMF tone) |
| Y | Y | 011110 | nnnnnnnn | N | N | xe "Local Control Response Order "§Local Control Response Order (specific response as designated by ‘nnnnnnnn’ as determined by each system) |
| Y | Y | 011111 | 00000001 | Y | Y | xe "Mobile Station Reject Order"§Mobile Station Reject Order (unspecified reason; see 6.7.3.4) |
| Y | Y | 011111 | 00000010 | Y | Y | Mobile Station Reject Order (message not accepted in this state; see 6.7.3.4) |
| Y | Y | 011111 | 00000011 | Y | Y | Mobile Station Reject Order (message structure not acceptable; see 6.7.3.4) |
| Y | Y | 011111 | 00000100 | Y | Y | Mobile Station Reject Order (message field not in valid range; see 6.7.3.4) |
| Y | Y | 011111 | 00000101 | Y | Y | Mobile Station Reject Order (message type or order code not understood; see 6.7.3.4) |
| Y | Y | 011111 | 00000110 | Y | Y | Mobile Station Reject Order (message requires a capability that is not supported by the mobile station; see 6.7.3.4) |
| Y | Y | 011111 | 00000111 | Y | Y | Mobile Station Reject Order (message cannot be handled by the current mobile station configuration; see 6.7.3.4) |
| All other codes are reserved. |  |  |  |  |  |  |

#### 6.7.3.1 Base Station Challenge Order xe "mobile station, signaling formats, orders, Base Station Challenge Order"§

When the mobile station sends a xe "Base Station Challenge Order"§Base Station Challenge Order, it shall set the ENCRYPTED field to ‘0’ and shall not encrypt the contents of the message.  The mobile station shall use the following fixed-length format for the order-specific fields:

| Order-Specific Field | Length (bits) |
| --- | --- |
| xe "ORDQ"§ORDQ | 8 |
| xe "RANDBS"§RANDBS | 32 |

xe "ORDQ"§ORDQ	-	Order qualification code.

The mobile station shall set this field to ‘00000000’.

xe "RANDBS"§RANDBS	-	Random challenge data.

The mobile station shall set this field as specified in 6.3.12.1.9.

#### 6.7.3.2 Service Option Request Order xe "mobile station, signaling formats, orders, Service Option Request Order"§

When the mobile station sends a xe "Service Option Request Order"§Service Option Request Order, it shall use the following fixed-length format for the order-specific fields:

| Order-Specific Field | Length (bits) |
| --- | --- |
| xe "ORDQ"§ORDQ | 8 |
| xe "SERVICE_OPTION"§SERVICE_OPTION | 16 |

xe "ORDQ"§ORDQ	-	Order qualification code.

The mobile station shall set this field to ‘00000000’.

xe "SERVICE_OPTION"§SERVICE_OPTION	-	Service option.

The mobile station shall set this field to the service option code shown in Table A-1 corresponding to the requested or alternative service option.

#### 6.7.3.3 Service Option Response Order xe "mobile station, signaling formats, orders, Service Option Response Order"§

When the mobile station sends a xe "Service Option Response Order"§Service Option Response Order, it shall use the following fixed-length format for the order-specific fields:

| Order-Specific Field | Length (bits) |
| --- | --- |
| xe "ORDQ"§ORDQ | 8 |
| xe "SERVICE_OPTION"§SERVICE_OPTION | 16 |

xe "ORDQ"§ORDQ	-	Order qualification code.

The mobile station shall set this field to ‘00000000’.

xe "SERVICE_OPTION"§SERVICE_OPTION	-	Service option.

The mobile station shall set this field to the service option code shown in Table A-1 corresponding to the accepted service option, or to ‘0000000000000000’ to reject the proposed service option.  See 6.6.4.1.2.2.1.

#### 6.7.3.4 Mobile Station Reject Order xe "mobile station, signaling formats, orders, Mobile Station Reject Order"§

The xe "Mobile Station Reject Order"§Mobile Station Reject Order can be sent on either the xe "Access Channel"§Access Channel or the xe "Reverse Traffic Channel"§Reverse Traffic Channel.  The mobile station shall use the following variable-length format for the order-specific fields:

| Order-Specific Field | Length (bits) |
| --- | --- |
| xe "ORDQ"§ORDQ | 8 |
| xe "MSG_TYPE"§MSG_TYPE | 8 |
| xe "ORDER"§ORDER | 0 or 8 |
| xe "ORDQ"§ORDQ | 0 or 8 |

xe "ORDQ"§ORDQ	-	Order qualification code.

The mobile station shall set this field to the ORDQ value shown in Table 6.7.3-1 corresponding for the reason for rejecting the message.

xe "MSG_TYPE"§MSG_TYPE	-	Message type of rejected message.

The mobile station shall set this field to the value of the MSG_TYPE field of the message being rejected.

xe "ORDER"§ORDER	-	Order type of rejected message.

If the rejected message was an order message, the mobile station shall set this field to the value of the ORDER field in the rejected message.  If the rejected message was not an order message, the mobile station shall omit this field.

xe "ORDQ"§ORDQ	-	Order qualification code of rejected message.

If the rejected message was an order message including an ORDQ field, the mobile station shall set this field to the value of the ORDQ field in the rejected message.  If the rejected message was an order message not including an ORDQ field, the mobile station shall set this field to ‘00000000’.  If the rejected message was not an order message, the mobile station shall omit this field.

### 6.7.4 Reverse Traffic Channel Information Records

On the Reverse Traffic Channel, information records may be included in the xe "Flash with Information Message"§Flash with Information Message or the xe "Status Message"§Status Message.  Table 6.7.4-1 lists the information record type values that may be used with each message type.  The following sections describe the contents of each of the record types in detail.

tc "Table 6.7.4-1.  Information Record Types"\c 	§Table 6.7.4-1.  Information Record Types

| Message Type | Information Record | Record Type (binary) |
| --- | --- | --- |
| None | xe "Reserved"§Reserved | 00000001 |
| Flash | xe "Feature Indicator"§Feature Indicator | 00000010 |
| Flash | xe "Keypad Facility"§Keypad Facility | 00000011 |
| Flash | xe "Called Party Number"§Called Party Number | 00000100 |
| Flash | xe "Calling Party Number"§Calling Party Number | 00000101 |
| Status | xe "Identification"§Identification | 00000110 |
| Status | xe "Call Mode"§Call Mode | 00000111 |
| Status | xe "Terminal Information"§Terminal Information | 00001000 |
| Status | xe "MIN Information"§MIN Information | 00001001 |
| Status | xe "Security Status"§Security Status | 00001010 |
| Flash | xe "Connected Number"§Connected Number | 00001011 |
| All other record type values are reserved. |  |  |

#### 6.7.4.1 Feature Indicator

This information record allows the user to invoke supplementary services and features.  The mobile station shall use the following fixed-length format for the type-specific fields:

| Type-Specific Field | Length (bits) |
| --- | --- |
| xe "FEATURE"§FEATURE | 4 |
| xe "RESERVED"§RESERVED | 4 |

xe "FEATURE"§FEATURE	-	Feature identifier.

This field identifies the supplementary service or feature to be invoked.  Definition of the values for this field is left to the manufacturers and system operators.

The mobile station shall set this field to the feature identifier.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall set this field to ‘0000’.

#### 6.7.4.2 Keypad Facility

This information record allows the user to send characters entered via a keyboard or other such terminal.  The mobile station shall use the following variable-length format for the type-specific fields:

| Type-Specific Field | Length (bits) |
| --- | --- |
| One or more occurrences of the following field: |  |
| xe "CHARi"§CHARi | 8 |
| xe "RESERVED"§RESERVED | 0 |

xe "CHARi"§CHARi	-	Character.

The mobile station shall include one occurrence of this field for each character entered.  The mobile station shall set each occurrence of this field to the ASCII representation corresponding to the character entered, as specified in ANSI X3.4, with the most significant bit set to ‘0’.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall omit this field.

#### 6.7.4.3 Called Party Number

This information record identifies the called party's number.  The mobile station shall use the following variable-length format for the type-specific fields:

| Type-Specific Field | Length (bits) |
| --- | --- |
| xe "NUMBER_TYPE"§NUMBER_TYPE | 3 |
| xe "NUMBER_PLAN"§NUMBER_PLAN | 4 |
| Zero or more occurrences of the following field: |  |
| xe "CHARi"§CHARi | 8 |
| xe "RESERVED"§RESERVED | 1 |

xe "NUMBER_TYPE"§NUMBER_TYPE	-	Type of number.

The mobile station shall set this field to the NUMBER_TYPE value shown in Table 6.7.1.3.2.4-3 corresponding to the type of the called number, as defined in ANSI T1.607 §4.5.9.

xe "NUMBER_PLAN"§NUMBER_PLAN	-	Numbering plan.

The mobile station shall set this field to the NUMBER_PLAN value shown in Table 6.7.1.3.2.4-4 corresponding to the numbering plan used for the called number, as defined in ANSI T1.607 §4.5.9.

xe "CHARi"§CHARi	-	Character.

The mobile stations shall include one occurrence of this field for each character in the called number.  The mobile station shall set each occurrence of this field to the ASCII representation corresponding to the character, as specified in ANSI X3.4, with the most significant bit set to ‘0’.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall set this field to ‘0’.

#### 6.7.4.4 Calling Party Number

This information record identifies the calling party's number.  The mobile station shall use the following variable-length format for the type-specific fields:

| Type-Specific Field | Length (bits) |
| --- | --- |
| xe "NUMBER_TYPE"§NUMBER_TYPE | 3 |
| xe "NUMBER_PLAN"§NUMBER_PLAN | 4 |
| xe "PI"§PI | 2 |
| xe "SI"§SI | 2 |
| Zero or more occurrences of the following field: |  |
| xe "CHARi"§CHARi | 8 |
| xe "RESERVED"§RESERVED | 5 |

xe "NUMBER_TYPE"§NUMBER_TYPE	-	Type of number.

The mobile station shall set this field to the NUMBER_TYPE value shown in Table 6.7.1.3.2.4-3 corresponding to the type of the calling number as defined in ANSI T1.607 §4.5.9.

xe "NUMBER_PLAN"§NUMBER_PLAN	-	Numbering plan.

The mobile station shall set this field to the NUMBER_PLAN value shown in Table 6.7.1.3.2.4-4 corresponding to the numbering plan used for the calling number, as defined in ANSI T1.607 §4.5.9.

xe "PI"§PI	-	Presentation indicator.

This field indicates whether or not the calling number should be displayed.

The mobile station shall set this field to the PI value shown in Table 6.7.4.4-1 corresponding to the presentation indicator as defined in ANSI T1.607 §4.5.9.

tc "Table 6.7.4.4-1.  Presentation Indicators"\c 	§Table 6.7.4.4-1.  Presentation Indicators

| Description | xe "PI"§PI (binary) |
| --- | --- |
| Presentation allowed | 00 |
| Presentation restricted | 01 |
| Number not available | 10 |
| Reserved | 11 |

xe "SI"§SI	-	Screening indicator.

This field indicates how the calling number was screened.

The mobile station shall set this field to the SI value shown in Table 6.7.4.4-2 corresponding to the screening indicator value as defined in ANSI T1.607 §4.5.9.

tc "Table 6.7.4.4-2.  Screening Indicators"\c 	§Table 6.7.4.4-2.  Screening Indicators

| Description | xe "SI"§SI (binary) |
| --- | --- |
| User-provided, not screened | 00 |
| User-provided, verified and passed | 01 |
| User-provided, verified and failed | 10 |
| Network-provided | 11 |

xe "CHARi"§CHARi	-	Character.

The mobile stations shall include one occurrence of this field for each character in the calling number.  The mobile station shall set each occurrence of this field to the ASCII representation corresponding to the character, as specified in ANSI X3.4, with the most significant bit set to ‘0’.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall set this field to ‘00000’.

#### 6.7.4.5 Identification

This information record can be included in a xe "Status Message"§Status Message to return the mobile station's ESN and MIN list.  The mobile station shall use the following variable-length format for the type-specific fields:

| Type-Specific Field | Length (bits) |
| --- | --- |
| One or more occurrences of the following record: |  |
| xe "MIN1"§MIN1 | 24 |
| xe "MIN2 "§MIN2 | 10 |
| xe "HOME_SID"§HOME_SID | 15 |
| xe "MOB_TERM"§MOB_TERM | 1 |
| xe "RESERVED"§RESERVED | 0-7 (as needed) |

The mobile station shall include one occurrence of the following two-field record for each MIN it is actively supporting.

xe "MIN1"§MIN1	-	First part of the xe "mobile station identification number"§mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN1 value for the MIN specified by this record.

xe "MIN2"§MIN2	-	Second part of the xe "mobile station identification number"§mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN2 value for the MIN specified by this record.

xe "HOME_SID"§HOME_SID	-	Home xe "system identification"§system identification.

The mobile station shall set this field to the xe "SID "§SID of the home cellular system for the MIN specified by this record.

xe "MOB_TERM"§MOB_TERM	-	Mobile terminated calls accepted indicator.

If the MIN specified by this record is configured to accept mobile terminated calls while operating with the current roaming status (see 6.6.5.3), the mobile station shall set this bit to ‘1’.  Otherwise, the mobile station shall set this bit to ‘0’.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall add reserved bits as needed in order to make the length of the entire message equal to an integer number of octets.  The mobile station shall set these bits to ‘0’.

#### 6.7.4.6 Call Mode

This information record can be included in a xe "Status Message"§Status Message to return the mobile station's preferred call mode and call-related information.  The mobile station shall use the following fixed-length format for the type-specific fields:

| Type-Specific Field | Length (bits) |
| --- | --- |
| xe "ORIG_MODE"§ORIG_MODE | 1 |
| xe "PRI_SERVICE"§PRI_SERVICE | 16 |
| xe "SEC_SERVICE"§SEC_SERVICE | 16 |
| xe "RESERVED"§RESERVED | 7 |

xe "ORIG_MODE"§ORIG_MODE	-	Origination mode indicator.

If the current call is a mobile-originated call, the mobile station shall set this field to ‘0’.  If the current call is a mobile-terminated call, the mobile station shall set this field to ‘1’.

xe "PRI_SERVICE"§PRI_SERVICE	-	Primary service option.

The mobile station shall set this field to the value shown in Table A-1 corresponding to the current primary service option.  If no primary service option is active, the mobile station shall set this field to ‘0000000000000000’.

xe "SEC_SERVICE"§SEC_SERVICE	-	Secondary service option.

The mobile station shall set this field to the value shown in Table A-1 corresponding to the current secondary service option.  If no secondary service option is active, the mobile station shall set this field to ‘0000000000000000’.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall set this field to ‘0000000’.

#### 6.7.4.7 Terminal Information

This information record can be included in a xe "Status Message"§Status Message to return configuration information about the mobile station.  The mobile station shall use the following variable-length format for the type-specific fields:

| Type-Specific Field | Length (bits) |
| --- | --- |
| xe "MOB_CAI_REV"§MOB_CAI_REV | 8 |
| xe "MOB_MFG_CODE"§MOB_MFG_CODE | 8 |
| xe "MOB_MODEL"§MOB_MODEL | 8 |
| xe "MOB_FIRM_REV"§MOB_FIRM_REV | 16 |
| xe "SCM"§SCM | 8 |
| xe "LOCAL_CTRL"§LOCAL_CTRL | 1 |
| xe "SLOT_CYCLE_INDEX"§SLOT_CYCLE_INDEX | 3 |
| One or more occurrences of the following field: |  |
| xe "SERVICE_OPTION"§SERVICE_OPTION | 16 |
| xe "RESERVED"§RESERVED | 4 |

xe "MOB_CAI_REV"§MOB_CAI_REV	-	CAI revision of the mobile station.

The mobile station shall set this field to ‘00000001’.

xe "MOB_MFG_CODE"§MOB_MFG_CODE	-	Manufacturer code.

This field identifies the manufacturer of the mobile station.

The mobile station shall set this field to the manufacturer code assigned to its manufacturer.

xe "MOB_MODEL"§MOB_MODEL	-	Model number.

This number is assigned by the manufacturer for a particular model.

The mobile station shall set this field to the model number assigned by the manufacturer for this mobile station.

xe "MOB_FIRM_REV"§MOB_FIRM_REV	-	Firmware revision number.

This number is assigned by the manufacturer for a particular firmware version.

The mobile station shall set this field to the revision number assigned by the manufacturer for the firmware version running in this mobile station.

xe "SCM"§SCM	-	Station class mark.

The mobile station shall set this field to its xe "station class mark.  See 2.3.3."§station class mark.  See 2.3.3.

xe "LOCAL_CTRL"§LOCAL_CTRL	-	Local control indicator.

If local control is enabled, the mobile station shall set this field to ‘1’.  If local control is disabled, the mobile station shall set this field to ‘0’.

xe "SLOT_CYCLE_INDEX"§SLOT_CYCLE_INDEX	-	Slot cycle mode.

If the mobile station is operating in non-slotted mode, it shall set this field to ‘000’.  If the mobile station is operating in slotted mode, it shall set this field to the SLOT_CYCLE_INDEX value in use.  See 6.6.2.1.1.

xe "SERVICE_OPTION"§SERVICE_OPTION	-	Supported service option.

The mobile station shall include one occurrence of this field for each service option supported by the mobile station.  The mobile station shall set this field to the value shown in Table A-1 corresponding to the supported service option.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall set this field to ‘0000’.

#### 6.7.4.8 MIN Information

This information record can be included in a xe "Status Message"§Status Message to return MIN-related configuration information about the mobile station.  Each record of this type returns information for one MIN.  The mobile station shall use the following variable-length format for the type-specific fields:

| Type-Specific Field | Length (bits) |
| --- | --- |
| xe "MIN1"§MIN1 | 24 |
| xe "MIN2"§MIN2 | 10 |
| xe "ACCOLC"§ACCOLC | 4 |
| xe "MOB_TERM_HOME"§MOB_TERM_HOME | 1 |
| xe "MOB_TERM_FOR_SID"§MOB_TERM_FOR_SID | 1 |
| xe "MOB_TERM_FOR_NID"§MOB_TERM_FOR_NID | 1 |
| xe "HOME_SID"§HOME_SID | 15 |
| Zero or more occurrences of the following record: |  |
| xe "SID"§SID | 15 |
| xe "NID"§NID | 16 |
| xe "RESERVED"§RESERVED | 0-7 (as needed) |

xe "MIN1"§MIN1	-	First part of the xe "mobile station identification number"§mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN1 value for the MIN specified by this record.

xe "MIN2"§MIN2	-	Second part of the xe "mobile station identification number"§mobile station identification number (xe "MIN"§MIN).

The mobile station shall set this field to the MIN2 value for the MIN specified by this record.

xe "ACCOLC"§ACCOLC	-	Overload class.

The mobile station shall set this field to the access overload class assigned to the mobile station for this MIN.

xe "MOB_TERM_HOME"§MOB_TERM_HOME	-	Home registration enable indicator.

If the mobile station is configured to receive mobile station terminated calls when using a home (SID,NID) pair, the mobile station shall set this field to ‘1’.  Otherwise, the mobile station shall set this field to ‘0’.  See 6.6.5.3.

xe "MOB_TERM_FOR_SID"§MOB_TERM_FOR_SID	-	Foreign SID roaming registration enable indicator.

If the mobile is configured to receive mobile station terminated calls when it is a foreign SID roamer, the mobile station shall set this field to ‘1’.  Otherwise, the mobile station shall set this field to ‘0’.  See 6.6.5.3.

xe "MOB_TERM_FOR_NID"§MOB_TERM_FOR_NID	-	Foreign NID roaming registration enable indicator.

If the mobile is configured to receive mobile station terminated calls when it is a foreign NID roamer, the mobile station shall set this field to ‘1’.  Otherwise, the mobile station shall set this field to ‘0’.  See 6.6.5.3.

xe "HOME_SID"§HOME_SID	-	Home xe "system identification"§system identification.

The mobile station shall set this field to the xe "SID "§SID of the home cellular system for the MIN specified by this record.

The mobile station shall include one occurrence of the following two-field record for each home (SID, NID) pair for this MIN:

xe "SID"§SID	-	xe "System "§System identification.

The mobile station shall set this field to the SID value for this (SID, NID) pair.

xe "NID"§NID	-	xe "Network"§Network identification.

The mobile station shall set this field to the NID value for this (SID, NID) pair.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall add reserved bits as needed in order to make the length of the entire information record equal to an integer number of octets.  The mobile station shall set these bits to ‘0’.

#### 6.7.4.9 Security Status

This information record can be included in a xe "Status Message"§Status Message to return the authentication, encryption, and voice privacy modes of the mobile station.  The mobile station shall use the following fixed-length format for the type-specific fields:

| Type-Specific Field | Length (bits) |
| --- | --- |
| xe "AUTH_MODE"§AUTH_MODE | 2 |
| xe "ENCRYPT_MODE"§ENCRYPT_MODE | 4 |
| xe "PRIVATE_LCM"§PRIVATE_LCM | 1 |
| xe "RESERVED"§RESERVED | 1 |

xe "AUTH_MODE"§AUTH_MODE	-	Authentication mode.

If the mobile station provided standard authentication information at the initiation of this call, the mobile station shall set this field to ‘01’.  Otherwise, the mobile station shall set this field to ‘00’.  All other values are reserved.

xe "ENCRYPT_MODE"§ENCRYPT_MODE	-	Message encryption mode.

The mobile station shall set this field to the value shown in Table 7.7.2.3.2.8-2 corresponding to the message encryption mode currently in use for this call.

xe "PRIVATE_LCM"§PRIVATE_LCM	-	Private long code mask indicator.

If the mobile station is using the private long code mask for this call, the mobile station shall set this field to ‘1’.  If the mobile station is using the public long code mask for this call, the mobile station shall set this field to ‘0’.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall set this field to ‘0’.

#### 6.7.4.10 Connected Number

This information record identifies the responding party to a call.  The mobile station shall use the following variable-length format for the type-specific fields:

| Type-Specific Field | Length (bits) |
| --- | --- |
| xe "NUMBER_TYPE"§NUMBER_TYPE | 3 |
| xe "NUMBER_PLAN"§NUMBER_PLAN | 4 |
| xe "PI"§PI | 2 |
| xe "SI"§SI | 2 |
| Zero or more occurrences of the following field: |  |
| xe "CHARi"§CHARi | 8 |
| xe "RESERVED"§RESERVED | 1 |

xe "NUMBER_TYPE"§NUMBER_TYPE	-	Type of number.

The mobile station shall set this field to the NUMBER_TYPE value shown in Table 6.7.1.3.2.4-3 corresponding to the type of the connected number as defined in ANSI T1.607 §4.5.9.

xe "NUMBER_PLAN"§NUMBER_PLAN	-	Numbering plan.

The mobile station shall set this field to the NUMBER_PLAN value shown in Table 6.7.1.3.2.4-4 corresponding to the numbering plan used for the connected number, as defined in ANSI T1.607 §4.5.9.

xe "PI"§PI	-	Presentation indicator.

This field indicates whether or not the connected number should be displayed.  The mobile station shall set this field to the PI value shown in Table 6.7.4.4-1 corresponding to the presentation indicator as defined in ANSI T1.607 §4.5.9.

xe "SI"§SI	-	Screening indicator.

This field indicates how the connected number was screened.  The mobile station shall set this field to the SI value shown in Table 6.7.4.4-2 corresponding to the screening indicator value as defined in ANSI T1.607 §4.5.9.

xe "CHARi"§CHARi	-	Character.

The mobile station shall include one occurrence of this field for each character in the connected number.  The mobile station shall set each occurrence of this field to the ASCII representation corresponding to the character, as specified in ANSI X3.4, with the most significant bit set to ‘0’.

xe "RESERVED"§RESERVED	-	Reserved bits.

The mobile station shall set this field to ‘0’.

No text.

The purpose of having two parameters is to distinguish between their use.  If INIT_PWR were 0, then NOM_PWR is the correction that should provide the correct received power at the base station.  INIT_PWR is the adjustment that is made to the first Access Channel probe so that it should be received at somewhat less than the required signal power.  This conservatism partially compensates for occasional, partially decorrelated path losses between the Forward CDMA Channel and the Reverse CDMA Channel.

The terms (-mean input power (dBm) - 73 + 0.5 ¥ xe "NOM_PWR "§NOM_PWR (dB) ) can be thought of as an open loop estimate of the required power for an Access Channel transmission.  The terms (0.5 ¥ xe "INIT_PWR "§INIT_PWR (dB) + the sum of all xe "access probe"§access probe corrections) can be thought of as the closed loop correction for Access Channel transmissions with the loop being through the base station.  The total is an open estimate of the initial required transmit power on the Reverse Traffic Channel.

At the full data rate, every power control bit is valid.

The purpose of the 0.25 dB steps in the table is to provide finer control of the power level when averaged over several power control bits.

In the first case, the accumulated value would be 1.0 dB + 0.75 dB equalling 1.75 dB.  If truncation is used, the previous output level would be 1.0 dB, and the new output level would be 1.5 dB.  In the second case, the accumulated value would be 1.25 dB + 0.75 dB which equals 2.0 dB.  If truncation is used, the previous output level would be 1.0 dB, and the new output level would be 2.0 dB.

For example, one timeslot since the last valid power control bit means that the previous timeslot contained a valid power control bit.

This is a bit-reversed readout of the row addresses.  If there is a binary counter c4c3c2c1c0, counting from 0 through 31, and n is a 5-bit binary number, n = a4a3a2a1a0, where a4=c0, a3=c1, a2=c2, a1=c3, a0=c4, then the row address is given by n+1.

In order to randomize the position of the data bursts, only 8 bits are strictly necessary.  The algorithm described here uses 14 bits to assure that the slots used for data transmission at the quarter rate are a subset of the slots used at the half rate and that the slots used at the one-eighth rate are a subset of the slots used at the quarter rate.

This permutation prevents high correlation between long codes corresponding to consecutive ESNs.

The frame quality indicator supports two functions at the receiver.  The first function is to determine the transmission rate of the frame; the second is to determine whether the frame is in error.

The multiplex option is the same on both the Forward Traffic Channel and the Reverse Traffic Channel.

Certain service options, which can be connected to the multiplex sublayer, can satisfactorily handle some bit errors.

PAG_3 counts those messages processed by layer 3.

A typical mobile station implementation can assume that the mobile station is synchronized with the Paging Channel message stream until several successive messages do not have good CRCs.  At this point, the mobile station can search for an SOM bit that is a ‘1’.  PAG_4 counts these events.

Transmit waveform quality can be viewed as the output of a normalized ideal matched filter.  The range of values for the transmit waveform quality is from 1.0 for a perfect CDMA waveform to 0.0 for a non-CDMA signal.  As an example, a mobile station with a 0.5 dB degradation in its transmit waveform would have a quality, r, of 10(-0.5/10) = 0.89.

Manufacturers can achieve these availability numbers through a combination of self-tests, reliability analysis, and reliability tests.

The requirement can also be stated as follows:  A manufacturer’s mobile station cannot transmit and exceed the minimum performance requirements by more than the specified transmit waveform degradation with greater than the specified probability at any point in its service life.  No more than two percent of a manufacturer’s mobile stations can have up to a 0.5 dB degradation; no more than 0.1% can have up to a 3.5 dB degradation; and no more than 0.001% can have up to a 10.0 dB degradation.

These messages do not have an ACK_REQ field.

Separate sequence numbers are used for messages requiring acknowledgement and messages not requiring acknowledgement on the Paging Channel.

If the mobile station user has entered a particular MIN, e.g., prior to an origination, then the mobile station should indicate whether that MIN is roaming.

A persistence test is not needed for response access attempts, because the base station controls the arrival rate of response messages directly by controlling the rate at which it transmits messages requiring responses.

This increases the probability that the base station will be able to separately demodulate transmissions from multiple mobile stations in the same Access Channel slot, especially when many mobile stations are at a similar range from the base station.  Use of a non-random algorithm for PN randomization permits the base station to separate the PN randomization from the actual propagation delay from the mobile station, so it can accurately estimate the timing of Reverse Traffic Channel transmissions from the mobile station.

The P calculations for PSIST(n), n=0, 1, ... 9 imply taking the number 2 exponentiated by a floating point number.  The entire process can be implemented using integer arithmetic.  Since the random variable zn in 6.6.7.2 ranges from 0 through 231 - 2, the persistence calculations can be implemented with sufficient accuracy in the range from 0 to 231.  The scaled probability value, P, can be written as: 231 ¥ 2-ÎPSIST(n)/4_ ¥ 2-[PSIST(n)-4¥ÎPSIST(n)/4_]/4.  The second factor can be implemented by shifting; the third factor consists of only four values that can be implemented by a table lookup.

For example, if the mobile station uses a 2 second slot cycle, then it may increment the counter by 10 every time it becomes active.

BASE_LAT and BASE_LONG are given in units of 1/4 seconds.  BASE_LAT/14400 and BASE_LONG/14400 are in units of degrees.  There are several simple methods that provide the required accuracy and avoid explicitly taking the cosine and the square root.

The indicator C_REG_ENABLED does not govern parameter-change registration.

Individual systems may or may not allow these capabilities.  The management of these capabilities is a function of the base station and system operator.

This system is where the Home Location Register (HLR) for this MIN is located.

For example, suppose a mobile station has the following SID, NID list (2, 3) (2, 0) (3, 1).  If  the base station (SID, NID) pair is (2, 3) then the mobile station is not roaming because the (SID, NID) pair is in the list.  If the base station (SID, NID) pair is (2, 7) then the mobile station is a foreign NID roamer because the SID 2 is in the list, but the (SID, NID) pair (2, 7) is not in the list.  If the base station (SID, NID) pair is (4, 0) then the mobile station is a foreign SID roamer because SID 4 is not in the list.

The table defines the entire search range.  For example, SRCH_WIN_As = 7 corresponds to a 28 PN chip search window or ±14 PN chips around the search window center.

This formula is adapted from Knuth, D. N., Sorting and Searching, vol. 3 of The Art of Computer Programming, 3 vols., (Reading, MA: Addison-Wesley, 1973), pp. 508-513.

This generator has full period, ranging over all integers between 1 and m-1; the values 0 and m are never produced. Several suitable implementations can be found in Park, Stephen K. and  Miller, Keith W., “Random Number Generators:  Good Ones are Hard to Find,” Communications of the ACM, vol. 31, no. 10, October 1988, pp. 1192-1201.

Initialization of the register to ones causes the CRC for all-zero data to be non-zero.

Initialization of the register to ones causes the CRC for all-zero data to be non-zero.