Introduction

SIM68

Version 2.20

Welcome  to the latest release of SIM68,  the  Motorola  MC68HC11

simulator  designed    to    allow  the    product  engineer  and

programmer to run and  debug  68HC11 .S19 object code  on  a  IBM

PC, or equivalent.

SIM68  allows you to load and actually watch your object code run

in  a manner  similar to the format of the BUFFALO  monitor,  but

with a  full screen display on an IBM compatible PC.

Below is a list of some of the SIM68 programming features:

- Set and alter program breakpoints

- Enable a full time display of memory location contents  during

program execution (single bytes)

- Enable the display of 32 bytes of contiguous memory, ie useful

for examining buffers and caches

- A CPU cycle stop watch which allows you  to  perform  actual

CPU cycle timing of program loops

- A memory mapping feature displaying the actual block of  memory

that code has been loaded into

- A feature that allows the modification of register and  memory

location contents

- Full support of all 68HC11 Op Codes, for a non-test mode system

- The  ability to run a user defined assembler and  editor  from

within SIM68

- The ability to simulate all of the MC68HC11 interrupts at  any

point during program's execution

- SIM68 now displays source code instead of disassembled object

code.  This feature is based on the presence of an AS11

(Motorola's freeware Assembler) list file ie.

AS11 test.asm -l > test.lst

- The ability to display and modify memory contents  dynamically

during program execution

- EEPROM programming and emulation

- Improved  run time simulation speed,  ie faster screen updates

and code interpretation

- Logging of simulator session to disk or printer

- Support of timer and PACC interrupts (new)

- Use of full context sensitive help screens (new)

- The ability to save and restore previous SIM68 sessions

Disclaimer

No warranties of any kind are expressed or implied by  myself  or

this product.   I assume no responsibility for any damages caused

as a result of using this package.  I hereby commit SIM68 version

2.20 into the public domain,  for use in private  and  commercial

engineering environments.

The Future

The  SIM68  simulator  package  will continue to grow and evolve,

gaining the  ability   to  emulate  more  of  the  true  internal

functions  of  the Motorola 68HC11 and other mirco-controllers.

The    SIM68   package is written in the C  programming  language

(using Borland's  Turbo-C++ compiler)  and    assembler.      The

source  code  is available  for  sale.   I  must stress that this

is   an  ever  growing software   product  that  will  eventually

be a commercially  offered  product.

If  you  have any problems or suggestions please write to  me  at

the following address:

Perry J. Fisch

24 Barellan Avenue

Carlingford, NSW  2018

Australia

or

Perry J. Fisch

1553 Sugarwood Circle

Winter Park, FL  32792

USA

The Catch...

A fee of $35.00 (Australian) $25.00 (US), paid via check made

payable to Perry Fisch and mailed to either of the above addresses,

will enable you to receive complete program documentation, help

sub-system, I/O and A/D options and place you on a mailing list so

that we can notify you of future program upgrades.

Please understand, you now have a fully functional SIM68 68HC11

program simulator, however, there are more than 100 key combinations

and other options which are fully desribed in the documentation.

You can do productive work without the documentation, but the

printed docs will make life a little easier for you...

The 34,000+ lines of SIM68 source code are available for

$275.00 Australian and $200 US.

If you like how SIM68 works,  and find it useful I  suggest  that

you  register.    I am sorry that I must ask for money,  however,

this simple project that I started a while ago has turned into  a

monster, consuming vast amounts of my time.

System Requirements

The  current  SIM68  package (Version 2.20) requires a minimum of

512Kb or memory,  monochrome or color monitor,  and  an  optional

printer.   Note: an EGA or VGA monitor is required to support the

I/O and A/D simulator modules.

As  stated above the 512 Kb of memory is what SIM68 needs to load

and operate,   any additional memory will be used to support  the

external editor and assembler.

Help Sub-system

This release (the release you now have) is shipped with an

incomplete help system.  The registered package will have the full

300+ pages of 'on-line' SIM68 help and 'advise'.

I/O Device Simulator

This release also comes with a demo of the SIM68 I/O and A/D state

simulators.  The registered package will have the full working

modules.

Installing SIM68

To install the MC68HC11 simulator please run the setup program by

typing:

setup-68.exe <cr>

note: <cr> means carriage return or enter

The setup package will allow you to specify the source and

destination drives and directories and will ensure that the progam

is installed correctly.

Running SIM68

After typing sim68 at the DOS prompt you will be presented with a

menu of six choices:

1) Run SIM68

2) Change Working Directory

3) Run User Defined Editor

4) Run User Defined Assembler

5) Configure SIM68

6) Exit to DOS

Note  this is not a literal list of options,  but I hope it helps

convey the meaning of the options.

Option 1 is obviously the work horse of the system, this is where

the user S19 code is actually run.   After  the  simulator  loads

your  S19  file  into it's memory you will see the working screen

with a display of register contents and a  comment  window.    At

this  point  hit  the  F1  key  and  the package will give a good

overview of all of it's abilities. (sample help screen)

From  this  point  forward you are  on  your  own.    I'm  in  the

middle   of version 2.3 and a Windows 3.0 based MC68332 simulator

and I've got a lot of work to do,  not to mention the  fact that I

wrote SIM68 to be a tool  not a  product, and I've  got   alot  of

work  to  do  on my 68HC11 systems.

Good luck,  and I  hope  this proves useful you.   Please  let me

know if  you have any problems with the package, I really do want

to   make this a useful product,  not to mention make a few bucks

too!

Perry Fisch