CHEAP AND EASY REAL-WORLD CONTROL FUNCTIONS FROM YOUR PC

Here's a cheap and easy way to make an IBM-compatable PC perform

real-world control functions using NO special hardware, and the

software you are already familiar with.

LPTREAD is a program that allows you to read directly from

individual pins of a parallel printer port, thereby using the

printer port as a data acquisition device for external equipment.

LPTREAD punches a predefined character string through the

keyboard buffer, one string for each of the eight data pins on

the parallel port, just as if it had been typed at the keyboard

whenever the associated pin is connected to the port's ground

pins (pins 19 through 25).

This makes it possible to have the PC monitor up to eight

external switch-closure type devices like alarm systems,

laboratory monitors, process control switches, or pushbuttons.

You can even connect the port to any TTL level device driver

circuit.

LPTREAD version 2.0 addresses five pre-mapped printer port

addresses, although these may have different physical addresses

on different computers. Your LPT1 might be someone elses LPT2. To

see a list of available printer port addresses and their mapping

numbers, type "LPREAD" with no parameters.

The calling protocol is this:

LPTREAD n NR

where n  := printer port address, 1 through 5, and [NR] is an

optional parameter which tells LPTREAD to operate in Non-Resident

mode.

Obviously, fiddling with your computer hardware is a little

risky. However, the LPT card is usually very resilient and

difficult to damage. Just be careful to observe all normal

electronic safeguards.

DEFINING KEYBOARD MACROS

Each of the eight data bits on the LPT port can be assigned a

key sequence of up to 40 bytes. This can include imbedded

carriage returns and control characters as well as alphanumerics.

The keyboard definition file, LPTREAD.DEF contains the macros.

LPTREAD.DEF is an ASCII flat file with eight variable length

records delimited with a carriage return/line feed pair. You can

create and modify this file using most word processors which do

not embed invisible control characters in the text (i.e. Wordstar

in Nondocument mode).

Following is an example of a valid LPTREAD.DEF file. Each text

line is a macro assigned to the associated data line, one through

eight. Semicolons are interpreted as chr(13) (a carriage return).

All lines should be flush-left in the file (no leading spaces,

unless you want them as part of the macro).

A;123;

B;

Jerry Garcia;

M;The Front Gate is Open!;

M;Someone's at the back door.;

1

2

Q;CD/;MENU.BAT

LPTREAD is Copyright (c) 1992 by Rob Cosgrove and is distributed

by Precision Data Corporation in Memphis, Tennessee. You are

hereby granted a limited license to use and distribute LPTREAD

and this documentation non-commercially, as long as you 1) don't

charge money for it, and 2) don't modify LPTREAD.COM to eliminate

the copyright message.

We can custom-modify LPTREAD for your application for our

regular programming rate of $70.00/hour.

We will also sell you limited rights to the source code, if you

want to modify it yourself. Source code costs $250, the only

limit being that you can't sell it as a stand-alone product to

compete in any way with LPTREAD itself. Otherwise, feel free to

include it in your own applications as often as you want.

LPTREAD is written for Turbo Pascal version 3.01.

If you send us a registration fee of $20.00 you will receive a

commercial version of LPTREAD, as well as a demo version of

LPTWRITE.COM (see the hype below).  We'll also put you on our

mailing list for future announcements of similar neat stuff.

LPTWRITE is great for adding real-world control functions to

ANY program without difficult real-world programming.

LPTWRITE.COM is a useful little program which allows you to

individually control up to eight external devices like relays,

LEDs, or TTL level devices by shelling out of your application,

and running LPTWRITE with parameters telling it which devices to

activate.

Check out the DEMO version of LPTWRITE, which contains complete

documentation and a simple schematic for the interface circuit.

Send $35.00 and receive the commercial versions of both programs.

We at PDC are available for consulting and programming using this

and other inovative technology for industrial control, data

aquisition, telemetry, and QC testing.

Precision Data Corporation

654 South White Station Road

Memphis, TN  38117

(901) 682-0732

CONTROL HARDWARE SCHEMATIC

for use with

LPTREAD.COM

Parallel Port

Connector

-----------------

Function    Pin

_________________

-----

DATA 0    2 ---------------------------->   <--------+

|

-----        |

DATA 1    3 ---------------------------->   <--------+

|

-----        |

DATA 2    4 ---------------------------->   <--------+

|

-----        |

DATA 3    5 ---------------------------->   <--------+

|

-----        |

DATA 4    6 ---------------------------->   <--------+

|

-----        |

DATA 5    7 ---------------------------->   <--------+

|

-----        |

DATA 6    8 ---------------------------->   <--------+

|

-----        |

DATA 7    9 ---------------------------->   <--------+

|

|

|

Ground   19 -----------------------------------------+

-----

-->   <--   :=   switch

You may find that you need to place 10K ohm pull-up resistors

between each of pins 2 through 9 and pin 17 (+ 5 VDC) to keep the

data pins from 'floating'.

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 4,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.