corr.doc

CORR.COM will correct the BIOS clock by the number of seconds

(up to 59) given as argument n on the command line:

CORR n

where n can be one or two digits. If the correction is meant

to decrement the clock, a minus sign should precede the number.

If the argument n is omitted, the program will prompt for it.

I use this program to correct the BIOS clock so that CLOCK.COM

each day shows correct time within one second. The BIOS clock on

my PC slows about 4 seconds a day, so I have my AUTOEXEC.BAT

execute CORR 4. To prevent AUTOEXEC.BAT from adjusting the

clock more than once a day, e.g. when I for some reason have to

reboot that same day, I use the once-a-day routines described

in PC Magazine August 1988 page 432. I will repeat those routines

here. I keep my batch files in a subdirectory called \bat, and

you must adapt the following routines to your own environment:

As part of my AUTOEXEC.BAT setup the following statements are

executed:

cd\bat

echo | more | date > readdate.bat

command /c readdate

READDATE.BAT will typically read:

Current date is Wed  1-11-1989

Enter new date (mm-dd-yy):

and "Current" will be conceived by the PC as a command.

In my \bat subdirectory I have a batch file called CURRENT.BAT

which lists like this:

echo off

if exist %4.xxx goto end

rename *.xxx %4.xxx

corr 4

:end

and the execution of this batch file command will result in

a dummy file called, say, 1-11-198.xxx, but only once a day.

If you reboot the same day, the program will jump to :end.

This is how I accomplish the once-a-day adjustment of my clock.

If you would like to include an after-weekend adjustment, add

the following statements right after the corr 4 statement:

if not %3==Mon goto end

corr m

where m is the necessary adjustment.

If you like CORR.COM you are requested to send a check for

$5 drawn on a U.S. bank to

Gunnar Richwell

Olav Kyrresgt. 2

N-0273 Oslo 2

NORWAY

If you would like to get the assembly source code for the program,

or get a minor custom change, send instead a check for $10 drawn on

a U.S. bank. See TIME.DOC for price discounts when you buy more

than one source code.

Feedback and suggestions are welcome, of course. The program was

developed on an NCR PC8 IBM-compatible AT machine, DOS ver. 3.1.

Companion programs are SETTIME.COM, TIMEDIFF.COM, and SETSAME.COM:

SETTIME.COM will set the BIOS clock.

TIMEDIFF.COM gives the time difference between the BIOS and

DOS clocks.

SETSAME.COM will set the DOS clock equal to the BIOS clock.