Documentation

for

Palet.com, Release 2

Overview

--------

Palet.com is a TSR (Terminate, but Stay Resident) program that ties into int

10h and traps (or allows to pass) any changes to the values stored in the EGA

registers. It occupies only 448 bytes of memory (exclusive of memory taken up

by its environment). The program will not reload, but will pass any parameters

to the resident portion when you call it subsequent times.

Command line instructions

palet

-----

Entering this will display a usage screen to refresh your memory, it will not

install the resident portion of palet.

palet d       (reset to default)

--------

Entering this will install (if not already installed) the TSR portion of palet

and write the default values stored within palet to the EGA registers. This is

a good way to "clean everything up" after playing with setting the EGA regis-

ters with the next command. The 'd' command should precede any other parameters

on the command line.

palet o1 o2 .... o16    (set EGA register values)

--------------------

Entering this will set any or all of the sixteen EGA palette registers. The

numbers o1, o2, etc. are  two digit octal (sorry) numbers that represent one

of the 64 valid values for the EGA colors. The numbers may be separated by

a space, or you may run all digits together (in which case any single digit

must be preceded by a zero, ie.: 7 4 16 is same as: 070416). It is not nec-

essary to enter values for registers above the ones you wish to change. Values

for registers you wish to remain unchanged can be replaced with a period, ie.:

palet ..7....2    will change registers 3 and 8, and leave others

unchanged.

This command will change the EGA registers and by default, will set the 'X'

switch, which will allow other programs to change the EGA registers also (see

the exclude and exclude text commands).

palet e    (exclude changes)

-------

Entering this will set the exclude changes switch. Palet will now prevent any

attempt to change the values in the EGA registers, no matter what display mode

is set (text or graphics). If you have programs that display in graphics mode,

but don't provide a way to set the colors used, then this is a way to do it.

Some trial and error will usually show you which EGA registers the program is

using so that you can set the registers to suit your preference. Generally, the

easiest way to use palet for this is in a batch file, ie.:

Page 1

echo off

palet o1 o2 .....o16 e

program            (program to run)

palet d

This file will set the registers of the EGA, make the change exclusive and load

the program to run. Upon exiting the program, palet will set the EGA registers

to the default value, leaving the exclude switch set.

palet t     (exclude text color changes)

-------

This is a variation on the exclude command, as it will only trap text mode

color changes, ie. attempts to change EGA register values while the display is

in graphics mode will go through.

palet x    (turn off exclusions)

-------

This command will turn off e or t exclusions, and allow other software to reset

the EGA registers.

Command usage

-------------

All commands can be combined in any way, with the exception of the 'd' command,

which when used, should precede all other parameters on the command line.

examples:

--------

palet ...4...16e    will write the octal values 4 and 16 (4 and 14 dec-

imal) to the 4th and 8th registers, and set the

exclude switch.

palet d..7....2t    will write default values to 16 registers, then

change the 3rd and 8th registers, and set the text

exclude switch.

palet dx	    will write default values to 16 registers, and turn

off the exclude switches.

Of course it would be silly to combine 'x' with 't' or 'e' in the same call.

-----------------

Page 2

Technical

---------

When palet is called, it first checks for DOS 2.0 or higher, and the existence

of the EGA, then 'hooks' itself into int 10h (video interrupt). EGA registers

are reset (if requested in the parameters), and palet's internal switches are

set (if specified in the parameters). Palet then removes most of itself from

memory, leaving behind the TSR portion that monitors all calls to int 10h.

A flag is used within palet to monitor the current display mode, this way

palet can determine whether or not to allow changes to the EGA registers to

pass (based on the 'E' and 'T' switch settings).

Any display mode function call to the EGA will reset all EGA registers to the

system default values. Palet will allow the display mode change to go through,

then check the 'T' and 'E' switch settings, if the display mode has been

changed to graphics, and the 'T' switch is set, then palet will allow any EGA

register changes to pass, until the display mode is changed back to text. On

the other hand, if the 'E' switch is set, then palet will immediately write the

values stored in it's buffer area to the EGA registers, and trap any attempts

to change those values. Any EGA register changes initiated by a call to palet

will be allowed to pass with any switch settings (palet knows when it is called

by itself).

A table of palet's default values is contained within the program code (out-

side of the resident portion). By carefully patching palet with debug, you can

customize the default values to your preference. DOS normally uses the 8th EGA

register for its text color (white), and the 1st register for its background

color; the 3rd EGA register normally contains the value for green. If you pre-

fer green text on black, you can switch the values of the 3rd and 8th EGA reg-

isters (this way you still have white available, albeit in the 3rd EGA regis-

ter). Use debug to patch the locations below:

address        value now (hex)    EGA register

----------------------------------------------

051eh		0h            		1

051fh		1h                      2

0520h 		2h 			3

0521h		3h                      4

0522h		4h                      5

0523h		5h			6

0524h	       14h			7

0525h		7h			8

0526h	       38h			9

0527h	       39h		       10

0528h	       3ah		       11

0529h	       3bh		       12

052ah	       3ch		       13

052bh	       3dh		       14

052ch	       3eh		       15

052dh	       3fh		       16

example:  (next page)

Page 3

debug palet.com

-e 051e

xxxx:051E  00.{enter new value here, hex number between 0 and 3F}

-w

-q

Do this carefully, and do not change values beyond 052dh.

Please note that programs which write directly to the EGA registers (and not

using int 10h) will be able to defeat palet. Programs like this are considered

'ill-behaved' since they don't use standard DOS or EGA BIOS calls, but address

the hardware directly.

Post Script

-----------

Palet.com, Release 2 is a public domain program, no claims are made to its

effectiveness or reliability. The original palette.com has been in use for

some time with no problems, Release 2 should also cause no problems.

Palet.com has evolved to it's current level through the joint efforts of

Charles Lazo III and Tim Worley, whose collaboration began with an exchange

of e-mail on CompuServe. Help support the 'hacker ethic', report any bugs, mod-

ifications, or suggestions for improvements to:

Charles Lazo III			Tim Worley

P.O. Box 452		 or		1314 Poplar St.

Hohenwald, TN  38462			Charlottesville, VA  22901

CIS ID [72210,17]			CIS ID [71336,730]

Palet can remain in continuing development if you so desire, but let's make it

a joint effort.

Page 4

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  The P(s)L cannot de-

bug programs over the telephone.

Disks in the P(s)L are updated monthly, so if you did not get

this disk  directly from the P(s)L,  you should be aware that

the files in this set may no  longer be the current versions.

For a copy of the latest monthly software library newsletter

and a list of the 1,000+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

(713) 721-6104 or (713) 721-5205