HCFORMAT V1.3 (c) 1992,1993 IMSL Software

INTRODUCTION

------------

The HCFORMAT utility formats floppy disks with capacities ranging from

20% to 128% higher than the standard DOS FORMAT.  These high-capacity

floppy disk formats can be used on any PC with high-density 3.5" or

5.25" drives.  The highest capacity formats provided are:

1.804 Mbyte format on 3.5" HD disks, instead of the normal 1.44 Mb

(22 sectors per track, 82 tracks)

1.476 Mbyte format on 5.25" HD disks, instead of the normal 1.2 Mb

(18 sectors per track, 82 tracks)

984 Kbyte format on 3.5" DD disks, instead of the normal 720 Kb

(12 sectors per track, 82 tracks)

820 Kbyte format on 5.25" DD disks, instead of the normal 360 Kb

(10 sectors per track, 82 tracks)

Other formats provided are:

1.44 Mbyte format on 5.25" HD disks, compatible with the 1.44 Mb

format of 3.5" HD disks (18 sectors per track, 80 tracks)

720 Kbyte format on 5.25" DD disks, compatible with the 720 Kb

format of 3.5" DD disks (9 sectors per track, 80 tracks)

1.76 Mbyte format on 3.5" HD disks, limited to 80 tracks for universal

drive compatibility (22 sectors per track, 80 tracks)

960 Kbyte format on 3.5" DD disks, limited to 80 tracks for universal

drive compatibility (12 sectors per track, 80 tracks)

820 Kbyte format on 3.5" DD disks, limited to standard density for

better media compatibility (10 sectors per track, 82 tracks)

800 Kbyte format on either 5.25" or 3.5" DD disks, for a little

additional capacity over the 720 Kb format without exceeding track

or density specifications (10 sectors per track, 80 tracks)

These higher capacities are obtained primarily by making better use of

the unformatted disk capacity than DOS FORMAT does, in ways which are

safe and fully supported by the drives and the diskette BIOS.  However,

the highest-capacity formats make use of techniques which slightly

exceed the drive or media specifications.  These techniques (identified

below) are usually safe, and are used by default, but can be turned off

if desired.  HCFORMAT has been thoroughly tested through several

generations of DOS, with many different PCs and floppy drives, and

works reliably on all known systems.

These formats do not use any data compression - you can use Doublespace,

Stacker, or PKZIP on top of these capacities.  The ability of Stacker to

create self-mounting floppy disks can be used, along with the similar

ability of HCFORMAT, to create floppy disks of up to 3.6 Mb capacity

which can be used on any system.

The HC TSR driver must be installed by HCFORMAT to read or write most of

the high-capacity disk formats (see the TSR command below).

PACKAGE CONTENTS

----------------

HCFLOPPY.DOC - this file

HCFORMAT.EXE - main utility

COPYANY.EXE - disk copy utility

FORMATTING WITH HCFORMAT

------------------------

The high-capacity disk formats are created by the utility HCFORMAT. The

syntax is very similar to the standard DOS FORMAT commmand:

HCFORMAT d: [/F:xxxx] | [/L] [/3] [/T:tt]  [/[+|-]M]  [/Q]

where:

d: is the drive containing the disk to format (A: or B:)

/F:xxxx optionally specifies the format capacity, where xxxx is:

1.804 (or 1804) for 1.804 Mb format on a 3.5" HD disk

1.76 (or 1760) for 1.76 Mb format on a 3.5" HD disk

1.476 (or 1476) for 1.476 Mb format on a 5.25" HD disk

1.44 (or 1440) for 1.44 Mb format on a 5.25" HD disk

984 for 984 Kb format on a 3.5" DD disk

960 for 960 Kb format on a 3.5" DD disk

820 for 820 Kb format on a 3.5" or 5.25" DD disk

800 for 800 Kb format on a 3.5" or 5.25" DD disk

720 for 720 Kb format on a 5.25" DD disk

The default format is the highest capacity supported by the

drive type (1.804 Mb for 3.5" or 1.476 Mb for 5.25")

/L specifies that a low-density (DD) disk is to be formatted,

changing the default formats to 984 Kb for 3.5" or 820 Kb for

5.25"

/3 specifies that a 5.25" disk is to be formatted for 3.5"

compatibility (1.44 Mb for a high-density disk, 720 Kb for a

low-density disk)

/T:tt specifies the number of tracks to be formatted (minimum

80). HCFORMAT uses 82 by default. The DOS standard is 80.

/Q specifies a quick re-format of a previously-formatted disk,

re-initializing only the DOS FAT and root directory to erase

the disk

/M formats the disk as self-mountable (defined below)

/+M makes an existing high-capacity disk self-mountable

/-M removes self-mountable format, reversing the effect of /+M

Use either /F:xxxx or a combination of the /L /3 /T switches to

specify the format. The /F:xxxx switch always overrides the /L /3 /T

switches if specified. /3 always formats 80 tracks, overriding /T if

specified.

The switches /Q, /+M, and /-M can only be used on previously-formatted

disks, and each should be the only switch on the command line if

specified.  The switches /F:xxxx, /L, /3, /T, and /M can only be used

when formatting new disks or unconditionally re-formatting disks, and

can be combined in any order subject to the override rules above.

Drive letters and switches are not case-sensitive.

Example:

>HCFORMAT A: /F:1.44

HCFORMAT V1.2 (c) 1992,1993 IMSL Software

Formatting A: as 1440 Kb

Insert disk to be formatted and press Enter to proceed ...

Cyl c Hd h  (counts while formatting)

Done

>

HCFORMAT low-level formats the disk, verifying as it formats.  If an

error is detected, HCFORMAT will report it and continue.  HCFORMAT does

NOT mark faulty sectors as Bad in the FAT like DOS FORMAT - it is

recommended that faulty disks not be used, or else they should be

thoroughly tested and marked with a separate disk test utility like

Norton Disk Doctor or PC Tools which can do a proper job of it.

When low-level formatting is complete, HCFORMAT initializes the boot

sector, FAT, and root directory for DOS.

If /Q (Quick format) is specified, the disk is assumed to be already

formatted, and the low-level format step is skipped.  The existing

format type is read from the disk, and only the DOS FAT and root

directory initialization is performed.	HCFORMAT /Q will Quick Format

any floppy disk with valid boot sector parameters, including standard

DOS disks and self-mountable high-capacity disks.  However, the HC TSR

driver must be installed to Quick Format HC disks with capacities of

1.44 Mb or higher.

Self-mountable disk format is described in a separate section below.

HCFORMAT will create only the specified high-capacity formats - DOS

FORMAT should be used for all other standard disk formats.  HCFORMAT

will not disallow any combination of formats and drive types - it will

only display a warning message (for example, HCFORMAT will write the

5.25" 1.44 Mb format on a 3.5" disk after displaying a warning, but this

is NOT the same 1.44 Mb low-level format as normally used on a 3.5"

disk).

The default formats use 82 tracks, which is the practical maximum on

most disks and drives.	The /T:tt switch can be used to specify any

number of tracks from the DOS standard of 80 up to whatever maximum you

want to try on a particular disk and drive.  This can produce format

capacities slightly different from the specific formats quoted here

(e.g. 830 instead of 820), but the actual capacity is always

correctly displayed by HCFORMAT and correctly handled by DOS.

However, the /F switch accepts only the specific 80-track and 82-track

format capacities listed here.

HCFORMAT supplies a variable DOS V4.0+ volume serial number, but not a

volume label (use the DOS LABEL utility afterward if you want a volume

label).  HCFORMAT does not support installing bootable system files on

the floppy disk (you can do this with the DOS SYS command, but only the

800/820 Kb 3.5" and, on some systems, the 960/984 Kb non-self-mountable

format are bootable).

HCFORMAT does not save Unformat information, but if you run a separate

FAT preservation utility such as DOS V5.0 MIRROR, PC Tools MIRROR, or

Norton IMAGE on the floppy disk before doing a Quick format, it is

possible to recover the contents by running the corresponding UNFORMAT

utility immediately afterward.	You can create a "SafeFormat" batch file

or command line macro to always perform MIRROR before HCFORMAT /Q if you

prefer.

As with standard DOS formats, you should never attempt to use a format

requiring high-density (HD) media with double-density (DD) media.

HCFORMAT always displays a reminder message to this effect when

high-density media is required.  Attempting to format double-density

media as high density will probably result in a large number of read

errors or verification errors while formatting. You should specify the

/L switch with DD media.

HCFORMAT supports only physical floppy drives 0 and 1 (A: and B:).

TECHNICAL INFORMATION ON DISK FORMATS

-------------------------------------

HCFORMAT uses the following physical format techniques in various

combinations:

- smaller gaps between sectors to fit more sectors on a track

- 80 tracks (single-step) instead of 40 on 5.25" DD disks

- 1024-byte sector size (instead of 512) to reduce the amount of sector

overhead (gaps, headers, and trailers) on the track

- 20% higher recording density on 3.5" DD disks by using the higher data

rate intended for 5.25" DD disks

- 82 tracks instead of 80, allowed by almost all drives and disks

- multi-pass formatting to place two different physical formats on the

same track to support self-mounting

HCFORMAT also makes us of the following DOS format variations:

- single-sector clusters instead of double-sector clusters on DD disks

to waste less space per file

- smaller root directory on 800/820 Kb disks, mainly to allow

self-mounting

- "virtual" FAT2 on self-mounting 1.44-1.804 Mb disks

DOS is able to accept almost any disk format which fits its basic model

of a fixed track/head/sector organization and 512-byte data sector size.

This includes the HC formats, with the condition that 1024-byte sectors

must be translated to equivalent 512-byte sectors for DOS.  However, DOS

always leaves it up to the diskette BIOS to handle setting the

appropriate drive hardware parameters.	While any standard PC BIOS is

able to handle all the allowable variations in drive parameters if told

what to do, the BIOS normally makes certain assumptions based on

long-established DOS disk formats.  These assumptions must be explicitly

overridden if you want to do something different.  This is the case with

the HC formats which require non-standard combinations of data rate,

track stepping, and sector size.

The 1.76+ Mbyte formats on 3.5" HD disks, and the 1.44+ Mbyte formats on

5.25" HD disks, use a 1024-byte sector size instead of the standard DOS

512-byte sector size.  Attempting to read a disk in one of these formats

without the HC TSR driver installed will normally result in a "Sector

not found" error.  When the HC TSR driver is installed, the disk will

appear to DOS to have normal 512-byte sectors, with twice the actual

number of sectors per track.

The 720 Kbyte format on 5.25" DD disks uses 80 tracks instead of 40, and

is identical to the 3.5" DD 720 Kb format (and to the old quad-density

5.25" disk format skipped over by IBM).  Attempting to read a disk in

this format without the HC TSR driver installed will appear to succeed

initially, but will usually result in a "Sector not found" error when

files beyond the first cylinder are accessed.  When the HC TSR driver is

installed, the drive is set to the correct 80-track mode on the initial

access.

The 800 Kbyte format on either 3.5" or 5.25" DD disks uses 10 sectors

per track instead of 9. DOS correctly handles this difference, and no

additional steps are required. The 5.25" 800 Kb disks still need the

HC TSR driver installed to set 80-track mode, but the 3.5" 800 Kb disks

can be read by DOS unaided, and do not require the driver.

The 960 Kbyte format on 3.5" DD disks uses 12 sectors per track instead

of 9.  This format is written using the higher 300 Kpbs data rate

intended for 5.25" HD drives which rotate at 360 rpm, instead of the

normal 250 Kbps rate intended for 300 rpm 3.5" drives.  Most BIOS

versions automatically recognize the data rate on the disk regardless of

physical disk type, but a few do not (see compatibility notes below).

Also, while some BIOS versions correctly assume that this is an 80-track

format, others incorrectly assume 40 tracks.  If the BIOS recognizes the

300 Kbps data rate and assumes 80 tracks, the 960 Kb disk is directly

readable by DOS unaided.  Otherwise, the HC TSR driver is required to

select 80-track mode and/or the correct data rate.

Note that the use of 300 Kpbs data rate instead of 250 Kbps for the 960

Kb format pushes the DD media to a 20% higher density than normal, but

it seems to be reliable (unlike trying to use the 500 Kbps HD data rate

on DD media, which seldom works and is never reliable).

The highest-capacity format for each media type (1.804 Mb, 1.476 Mb,

984 Mb and 820 Mb) uses 82 tracks instead of 80.  Standard floppy drives

do not limit the heads to stop at the 80th track - the heads are free to

keep stepping until they hit the head stop (you will hear a clunking

sound if this happens).  Since the head movement range is designed a

little larger than necessary in order to allow for drive variations, it

is usually possible to fit in at least 82 tracks instead of the standard

80.  DOS and the diskette BIOS don't care how many tracks there, as long

as the number of tracks is specified correctly in the boot sector

parameters of the disk.

ADDITIONAL FUNCTIONS OF HCFORMAT

--------------------------------

HCFORMAT also has several other functions, invoked by the following

keyword commands:

HCFORMAT TSR

ON | OFF

STATUS

[/?]

where:

TSR loads the HC TSR driver

OFF turns the HC TSR driver off (disables translation)

ON  turns the HC TSR driver back on (re-enables translation)

STATUS displays the HC TSR driver status and BIOS drive status

/? (or no parameter) displays command line syntax help

Keywords can be abbreviated to the minimum unique length, and are not

case-sensitive.

TSR:

The command HCFORMAT TSR installs the resident HC driver, which occupies

a little over 2 Kbytes of memory.  This command should normally be

placed in the AUTOEXEC.BAT file to load the driver every time the system

boots.

If the HC TSR driver is already loaded, you will receive an error

message.

Once installed, the driver operates automatically with any of the high-

capacity formats, and is fully transparent to standard disk formats.

The HC TSR driver is compatible with all programs which access the

floppy drives through the BIOS, including Windows 3.1, SMARTDRV, and

Stacker.  It is also compatible with most disk utilities, including

Norton Utilities and PC Tools, with a few sensible exceptions (see

Compatibility Notes below).

The HC TSR driver can be loaded HIGH, but note that while it occupies

only a little over 2 Kb when resident, HCFORMAT requires up to 20 Kb

to load initially (or you can load the driver with MOUNT.EXE instead of

HCFORMAT as described below, in which case it requires only about 3 Kb

to load initially).

If you receive the following non-fatal warning message when loading the

HC TSR driver:

Warning - Internal buffer moved up to 64K physical memory boundary

HC TSR should be loaded at a different address to save memory

it indicates that the driver's internal 1024-byte disk I/O buffer would

have crossed a 64K physical memory boundary at the address where it was

loaded, which is not allowed by the hardware DMA controller.  The driver

has therefore moved its buffer up to start at the boundary, wasting a

few bytes of unusable memory.  You can probably avoid this by simply

re-arranging the order in which TSRs are loaded.  The operation of the

HC TSR driver will be unaffected in any case.

ON/OFF:

The HC TSR driver can be turned off (disabled) and back on (enabled)

with these commands.  This is only necessary if you wanted to view or

access HC disks without the driver's translation, or if you suspect some

interference with another disk utiltity (both unlikely).

The HC TSR driver is On (enabled) when initially loaded.  It cannot be

unloaded from memory unless you use a separate TSR unloading utility,

but it can be temporarily disabled by the OFF command.	When the OFF

command is issued, a flag is set in the driver which causes it to pass

all disk I/O commands through without modification.  Issuing the ON

command clears this flag and causes the driver to resume normal

processing.  The On command also causes a drive reset and re-read

in order to insure that any disks currently in the drives are correctly

recognized.

If you turn off the HC TSR driver, or remove it from memory with a TSR

unloading utility, any 720+ Kb DD disks currently in the drives can

continue to be accessed normally until they are removed.  However, any

attempt to access any 1.44+ Mb HC disks left in the drives will

result in a "Sector not found" error.

STATUS:

The command HCFORMAT STATUS displays the current status of the HC TSR

driver (Not installed/On/Off) and the driver version number.  It also

shows the BIOS-reported disk type and status of each of the floppy

disks.

The disk types which may be reported are:

5.25" 360 Kb DD

5.25" 1.2 Mb HD

5.25" 720+ Kb HC 80-track

5.25" 1.44+ Mb HC 1024-byte sectors

3.5" 720+ Kb DD

3.5" 960+ Kb HC 300 Kbps

3.5" 1.44 Mb HD

3.5" 1.76+ Mb HC 1024-byte sectors

Disks with 1024-byte sectors which are in self-mountable format have

the word "Mounted" appended to their type.

Example:

>HCFORMAT STATUS

HC TSR V1.2 On

A: 5.25" 1.2 Mb HD

B: 3.5" 1.76+ Mb HC 1024-byte sectors Mounted

Drives which have not yet been accessed, or do not exist as physical

drives, or are not high-density floppy drives are shown as "n/a".

Drives from which the disk has been removed continue to show

their status as of the last disk access. The STATUS command does not

cause HCFORMAT to re-access the disk.

The STATUS command shows physical disk type reported by the BIOS and the

HC TSR driver, not the disk capacity reported by DOS.  Neither the BIOS

nor the HC TSR driver distinguish between similar formats such as the

720 Kb, 800 Kb and 820 Kb formats, so these are simply shown as "720+

Kb". 3.5" disks in the 800/820 Kb format are not reported as HC type,

since they do not require any translation by the HC TSR driver.

SELF-MOUNTABLE FORMAT

---------------------

A high-capacity disk normally cannot be read by DOS on a system without

the HC TSR driver installed (with the exception of the 3.5" 800/820 Kb

format, and sometimes the 960/984 Kb format, depending on the BIOS).

This is obviously a problem if you ever need to read your disks on

another system which may not have the driver installed.  The solution is

to place a loadable copy of the driver on the high-capacity disk itself,

and install it on the destination system directly from the high-capacity

disk in a "self-mounting" process.

The /M switch causes HCFORMAT to place a loadable copy of the HC TSR

driver called MOUNT.EXE on the disk when it is formatted.  Running

MOUNT.EXE from the floppy disk installs a copy of the HC TSR driver

which is identical to that installed by the HCFORMAT TSR command. A

disk with MOUNT.EXE on it in a loadable format is called Self-Mountable.

In the case of 720-820 Kb 5.25" HC disks, or 960/984 Kb 3.5" HC disks,

no special disk formatting required to make them self-mountable - the

file MOUNT.EXE is simply copied to the disk as the first file, and the

disk can be mounted by running MOUNT.EXE from the floppy disk.

MOUNT.EXE must be the first physical file on the disk and must be small

enough to fit entirely on cylinder 0.  MOUNT.EXE lacks the formatting

and command functions of HCFORMAT, and is therefore much smaller.

MOUNT.EXE is marked as a hidden/read-only file on the disk, and

therefore does not appear in a standard directory listing.

The /+M switch can be used to make an existing 720-984 Kb disk self-

mountable by placing MOUNT.EXE at the beginning of the disk, even if

existing files already occupy that space.  Any existing files occupying

the beginning of the disk are safely moved to make room for MOUNT.EXE.

An appropriate error message will be displayed if a file named MOUNT.EXE

is already found on the disk, or if the disk is full and there is no

room for MOUNT.EXE (there must be 2 Kb of space free). If you have a

full 720-960 Kb disk which you want to make self-mountable, you must

reformat it as the next higher capacity to make room for MOUNT.EXE.

If you subsequently want to make the disk non-mountable, use the /-M

switch of HCFORMAT.  Alternatively, you can simply remove the read-only

protection and un-hide MOUNT.EXE with the command:

ATTRIB -R -H A:MOUNT.EXE

and then delete the file MOUNT.EXE.

Remember, if you try to access files on a 720-820 KB 5.25" high-capacity

disk (or a 960 Kb 3.5" disk on some systems) without installing the HC

TSR driver, the operation will fail with a disk error at the first

attempt to read beyond cylinder 0.  MOUNT.EXE is able to load

successfully because it is small enough to fit entirely on cylinder 0.

Self-mounting formats with 1024-byte sectors:

Things are a little more complicated with the 1.44-1.804 Mb high-density

HC formats.  Since they use 1024-byte sectors, they cannot be read at

all by DOS without the HC TSR driver installed.  Therefore, some special

formatting on the first track is required to make them self-mountable on

another system.

When used with high-capacity 1.44-1.804 Mb floppy disks, the /M

switch of HCFORMAT reformats track 0 of the disk to have two separate

areas in different formats.  Part of the track is a standard DOS format

with 512-byte sectors, and MOUNT.EXE is placed in this readable area.

The other part of the track is a high-capacity format with 1024-byte

sectors, where the standard high-capacity boot sector and FAT are

placed.  None of the disk capacity is lost by this formatting (however,

DOS must be tricked a little by the HC TSR driver to get away with this!).

When read by DOS without the HC TSR driver installed, the Mountable disk

will initially appear to be a DOS disk of very small capacity (8 sectors

total), labelled "HC DISK V1", with a single file called MOUNT.EXE

occupying the entire disk.  The directory listing contains a message

with instructions to run MOUNT instead of file names:

Volume in drive A is HC Disk V1

Directory of  A:\

1This is		  0

2a high-		  0

3capacty		  0

4disk.		  0

5Run		  0

6MOUNT		  0

7to		  0

8access.		  0

MOUNT    EXE     2048  17-09-93

9 File(s) 	0 bytes free

When you run MOUNT from the floppy disk, it installs the HC TSR driver.

The disk will then be automatically mounted and appear as a normal high-

capacity 1.44-1.804 Mb disk, with no trace of MOUNT on it.  This process

is known as "mounting" the disk, and the disk is then reported as

"Mounted" by the STATUS command.  If the HC TSR driver is already

installed, self-mountable disks are automatically mounted on the initial

access, and there is no need to run MOUNT.  There is no indication that

such a disk is mounted unless you run HCFORMAT STATUS.	You can use

these disks in exactly the same way as non-mountable HC disks.

Only the normal high-capacity disk will appear as long as the HC TSR

driver is operating, even if the disk is removed and reloaded later.

Any other self-mountable disks inserted will also be mounted

automatically.	If you remove or turn off the HC TSR driver, the mounted

disk will again appear in its original form, showing MOUNT.EXE and the

directory instructions as the only contents.  No changes are written to

the disk by mounting or unmounting it.

The /+M switch of HCFORMAT can be used to make an existing 1.44-1.804 Mb

HC disk self-mountable by reformatting track 0.  Any existing disk

contents are preserved.  The conversion can be reversed by the /-M

switch, restoring the normal non-mountable high-capacity format.  Using

/+M on a high-capacity disk which is already in self-mountable format,

or /-M on a high-capacity disk which is not in self-mountable format,

has no effect.

FALLBACK PROCEDURES

-------------------

If you ever need to read a non-self-mountable 720-984 Kb HC disk on

a system without the HC TSR driver, and the driver is not available,

there is a simple way to temporarily mount these disks using the DOS

DEBUG utility or BASIC.  You must change the BIOS media state byte at

memory location 40:90 hex (for drive A:) or 40:91 hex (for drive B:)

to select the correct drive mode (single-step/80-track and 300 Kbps

data rate).  This fix will last as long as the disk stays in the drive.

With DEBUG:

> DEBUG

-e 40:90 57		(substitute 40:91 for drive B:)

-q			(quit DEBUG)

With BASIC:

DEFSEG	&H40

OUT &H90 &H57		(substitute 91 for drive B:)

You can write the three numbers "40:90 57" on the disk label if you want

to remember them.

Additional fallback for 960/984 Kb disks:

If you get a "General failure" error trying to read a self-mountable

960/984 Kb disk on a machine without the HC TSR driver, the problem is

that the BIOS is not automatically trying the 300 Kbps data rate.  The

above DEBUG/BASIC fallback procedure will fix it, but there is also

another way to get DOS to load MOUNT from the disk:  enter the machine's

Setup mode, change the floppy drive type from 3.5" 1.44 Mb to 5.25" 1.2

Mb, and re-boot.  The BIOS will then assume that the drive is a 5.25"

1.2 Mb drive, and will automatically try the 300 Kbps data rate.  Don't

forget to change the drive type back after loading MOUNT.

COPYANY

-------

The 1.44 Mbyte and 720 Kb formats on 5.25" disks appear to DOS to be

exactly the same as for 3.5" disks, and allow disk copying back and

forth. There is one problem:  DOS DISKCOPY recognizes different

physical drive types and will refuse to copy from a 3.5" drive to a

5.25" drive (although the opposite direction works, and DISKCOMP will

work as long as the drives are specified in the correct order).

The COPYANY utility is provided to avoid this problem with DISKCOPY.

COPYANY will copy any floppy disk exactly to any other disk in a

compatible format without worrying about the drive type.  Compatible

means that the number of tracks and the number of sectors per track of

the destination disk must be equal to (or greater than) the source disk.

Both source and destination disks must be pre-formatted, and the source

and destination drives must be different.

DOS DISKCOPY will handle copying in the same drive or between compatible

drives, even for high-capacity formats as long as the HC TSR driver is

installed.

In addition, COPYANY will automatically handle disks in high-capacity

format (1024-byte sector format or 80-track DD format) even without the

HC TSR driver installed.  This provides a disk copy capability in the

event that the driver is not installed on the host system.

Syntax: >COPYANY B: A:	  (order is source: dest:)

COPYANY V1.0 (c) 1992 IMSL Software

Copying B: to A: - Press Enter when ready to proceed ...

Copying 1440 Kb

Cyl c Hd h	  (counts while copying)

Done

>

COPYANY will not reproduce a self-mountable copy of a 1.44 Mb

disk.  It will copy the high-capacity portion of the disk if the disk is

mounted.  The resulting copy can then be converted with HCFORMAT /+M if

desired.  COPYANY cannot copy the unmounted disk.

PROBLEMS/COMPATIBILITY NOTES

----------------------------

Compatibility with DOS versions and different types of PCs:

HCFORMAT has been extensively tested with PCDOS and MSDOS V3.3 through

V6.0, on a variety of 286/386/486 machines including IBM, Compaq,

Toshiba, AST, Zenith, clones with various AMI and Phoenix BIOS versions,

and on systems with various combinations of floppy and hard disks.  The

utilities have been made compatible with the few quirks found, and

should operate reliably on any machine.

DOS problems:

The HC TSR driver is dependent on one major assumption about the way

that DOS operates:  DOS always tests for a hardware disk change

indication prior to accessing the disk, and reads the new disk

parameters from the boot sector if a change is detected.  The driver

relies on this action to detect when a HC disk requiring translation is

inserted.  This is a very safe assumption, since DOS is designed to

operate this way.  However, there are a few cases where programs or DOS

itself may violate this assumption.  In particular, DOS may fail to

re-read the type of disk from the boot sector after a disk change

indication if the same disk is re-inserted.  This will result in a disk

read failure if you remove and re-insert a disk with 1024-byte

sectors, since the disk change indication causes the HC TSR driver to

stop translating the disk format until the new disk type is read.

If this should happen, simply remove and re-insert the disk, and then

do a DIR listing on it (always a good idea when inserting a new HC

disk).	An example of a program bypassing the disk type check after a

disk change is probably the bug in PKZIP V2.04C reported below.

Some older versions of DOS (like Zenith DOS V3.3+) fail to recheck the

disk parameters between operations. This can cause strange effects

(like reporting the wrong amount of free space on the disk) if you

have issued a command which changes the disk parameters without

physically switching disks (e.g. MOUNT, or HCFORMAT /Q). Issuing a

CHKDSK command after such operations avoids problems with these older

versions of DOS.

BIOS variations:

A minor BIOS quirk which you may notice is that the floppy disk controller

on some machines returns an error code reported by DOS as "General failure"

rather than the correct "Sector not found" when attempting to read a

disk with 1024-byte sectors without the HC TSR driver installed.

HCFORMAT therefore assumes that the BIOS might return almost any error

code when initially accessing such a disk in order to work on all

machines.

Another variation between machines is whether or not the BIOS will

handle 960/984 Kb disks without the HC TSR driver installed.  The BIOS

on most machines automatically tries all 3 possible data rates

(500/300/250 Kbps) on each newly loaded disk to find the correct one,

regardless of physical drive type.  However, on some machines the BIOS

does not do this (e.g.	AMI and Phoenix 386 BIOS clones), and the disk

is not readable without the driver to select the correct data rate.

Even when the data rate is selected correctly, the BIOS does not know

whether there should be 40 or 80 tracks on this type of disk - some

machines correctly assume 80 tracks (e.g. Zenith, AST), while others

incorrectly assume 40 tracks (e.g. Compaq, Phoenix 286 BIOS clones).

If the BIOS assumes 80 tracks, DOS can read the disk without

translation.  If not, then the HC TSR driver is required to set 80-track

mode, and the disk must be made self-mountable if you may want to use it

on another such system which does not have the driver installed.

If the BIOS of the target machine does not automatically recognize the

300 Kbps data rate on a 3.5" 960/984 Kb disk, the disk cannot self-mount

on that machine (while it might be possible to create a unique

self-mounting 960/984 Kb format for such machines, it's really too much

trouble since they are the exception).	The fallback procedures

described above for changing the drive media state byte or drive type

will work around the problem if you are ever stuck needing to read the

disk on such a machine without the HC TSR driver.

Most BIOS and DOS versions need the HC TSR driver to select 80-track

mode for 720-820 Kb 5.25" disks, but there are a few BIOS versions

around which do so on their own, whether by accident or because they are

smarter.  On these machines you may find that the 720-820 Kb 5.25" disks

are readable without the driver.

Disk Repair Utilities (e.g. Norton Disk Doctor):

Most disk utilities accept the high capacity formats without problems.

However, disk repair utilities like Norton Disk Doctor should not be

allowed to attempt FAT repairs on high capacity diskettes.  Since they

do not recognize the specific format, they may assume that the FAT is

damaged and attempt to "repair" it by converting it to one of the

standard DOS formats.  (For that matter, I have not found these disk

repair utilities to be intelligent enough to turn loose on any disk.)

The best defence against disk problems is to run a FAT backup utiltity

like MIRROR or IMAGE on the disk - you can then recover the files if the

disk is damaged by quick-formatting it and running the corresponding FAT

recovery utiltiy.

PKZIP V2.04C/V2.04E

PKZIP version 2.04C had a problem with non-standard disks when using its

new option to span multiple disks with a single ZIP file.  The problem

appears as a "Sector not found" error when loading the 3rd or 4th disk

of a high-capacity backup set.	Apparently PKZIP bypasses DOS and

assumes that new disks are the same format as the previous one, instead

of following the normal practice of reading the boot sector to determine

the disk format.  The maintenance release 2.04E of PKZIP (which quickly

followed the inital 2.04C release of version 2) fixed this problem.

Using DOS disks in an Apple MacIntosh:

Newer Macs can read and write the DOS 1.44 Mb HD floppy disk format using

utilities such as Apple File Exchange or DOS Mounter.  The HC formats

are NOT compatible with these utilities, and cannot be used to exchange

data with a Mac.

TIPS

----

Running HCFORMAT under Windows:

Make sure that you load the HC TSR driver BEFORE starting Windows;

otherwise it will be loaded only for a single temporary DOS session.

Making multi-disk sets self-mounting:

Only the first disk of a multi-disk set need be made self-mounting - all

subsequent disks will be automatically mounted once the HC TSR driver is

installed from the first disk.

Maximum number of tracks:

Many drives will successfully read and format 83 or 84 tracks, but the

lowest common denominator appears to be 82 (the HCFORMAT default).

Experiment if you like, but best to stick to 82 maximum for

compatibility between drives.

Compatibility with untested machines:

If you are trying an HCFORMAT disk on a new machine or a different

version of DOS, it is a good idea to check out the disk with CHKDSK

and DIR after inserting it, MOUNTing it, reformatting it, or

quick-formatting it. As noted above under Compatibility Notes, there

are some quirks to beware of with older versions of DOS.

RELEASE HISTORY

---------------

V0.9 - Beta test

V1.0 - Initial release

- BIOS/FDC variations between different machines accomodated

V1.1 - HCMOUNT combined into MOUNT, separate UNMOUNT utility added

- HCFORMAT aborts immediately if disk is write-protected

- A few other improved error messages

V1.2 - 960 Kb format added, other disk formats unchanged from previous

versions (note: V1.0 or V1.1 of the HC TSR driver will correctly

handle 960 Kb disks)

- HCTSR, MOUNT, and UNMOUNT combined into HCFORMAT to make a single

program with multiple functions; only COPYANY is still separate

- The HC TSR driver now incorporates disk mounting (formerly found

in MOUNT.EXE only); this makes the driver slightly larger, but

makes it easier to use self-mountable disks transparently (note:

MOUNT.EXE V1.2 is not identical to earlier versions of MOUNT.EXE

which may be found on existing self-mountable disks, but is

compatible)

- TSR On/Off commands added to HCFORMAT to replace the

corresponding functions of MOUNT/UNMOUNT; Off command disables

the TSR entirely (rather than just for self-mountable disks),

allowing some internal simplifications

- Status display command has been added to HCFORMAT to check status

of the TSR and drive format translation (note: the STATUS command

shows V1.0 or V1.1 of the HC TSR driver as "V1.0", and cannot

display the drive format translation with this version of the

TSR)

- HCFORMAT can now format new self-mountable disks in one step, and

can make existing 720/800/960 Kb disks self-mountable without

removing the existing contents (note slight change in /M switch

syntax)

- Floppy drives now reset when enabling the HC TSR driver

V1.3 - 82-track formats added (1.804/1.476 Mb and 984/820 Kb)

- /T:tt switch to adjust number of tracks

- simplified formatting options added: /L /3

- DD formats now use single-sector cluster size (except 720 Kb

format remains with 2-sector cluster size for 3.5"/5.25"

compatibility)

- root directory size is reduced on 800/820 Kb disks to allow space

on cylinder 0 for MOUNT.EXE in self-mounting format