PBS - Plate Bending Solutions

General Introduction

PBS is the COPAN/II module that analyzes LAMCON cross-sections

for strength and stiffness in pure plate bending configurations.

The edge fixity is simply supported on all four edges.   The

plate X-Y dimensions are input by the user and should be limited

to aspect ratios of less than 5.  PBS handles evenly distributed

pressures which act normal to the plate surface.

The laminate to plate orientation may be set to any angle. If an

angle is not given, then the default value of 0 degrees is used.

This means the 0 degree axis of the laminate runs parallel to the

1st side of the plate.

In addition to bending solutions, PBS Version 1.0 includes a

buckling routine to produce a edge force density.   This value

does not refer to face buckling on sandwich structures but rather

general buckling of an isotropic plate.

PBS is supplied as a executable COM file although it can be

called from the ACCESS menu.    To run PBS from DOS, enter BS and

hit the <CR> key.  (Even though the module name is PBS, we have

given it the name BS on the disk.)

PBS is not copy protected.  This is true even on the lower priced

version which uses a copy-protection scheme.  This means that PBS

can be used without having a master disk in drive A.  (for those

of you with the less expensive model)

A suggestion for floppy disk users

If  you  will  be using PBS with a single  or  dual  floppy

system,  or  with  a lap-top machine with 3.5"  drives,  we

suggest  that  you  set  up a virtural  RAM  disk  for  PBS

calculations.

When solving the stiffness matrix, PBS will be reading the

material file as well as making stratch calculations on

disk.   To the user, this will be transparent as the

disk files are erased as soon as PBS is done using them.

Because of this,  a floppy system may not be desireable due

to all of the disk spinning that will take place.   Another

disadvantage  with  the  DC powered lap-tops is that disk

access draws quite a bit more power from the already small

reserve.

A  remedy  to this situation is to set up what is called a

virtural RAM disk.   With most machines, this can be done

using a DOS program called VDISK.COM, which is usually

defined in a CONFIG.SYS file.  eg DEVICE=VDISK.COM *64

#

The amount of memory that you reserve for the VDISK is

based on the amount you have available.  However, if

possible, we suggest at least 64K.   This will give you

enough room to copy PBS and the .PRP files as well as write

.OUT  files to it.    A batch file can be created to assist

with this repetitive task.   If you do not know how to make

a batch file, check out your DOS manual.

Now, when you are running calculations, PBS will read and

write to the virtural disk.   The operations will be

absolutely quiet and a ton faster.   In addition to saving

your disk drives from overload, your battery pack will allow

you a few extra hours of COPANing.

Starting-Up PBS

Operating PBS is simple.

You will load PBS using one of two methods.   Which one you use

will be determined by how you are using the COPAN/II system.

The ACCESS Method

After ACCESS has been loaded, move the menu pointer to PBS.

After positioning the pointer to PBS,  hit the <CR>.   The screen

should  blank out for a few seconds.   The opening screen of PBS

should then be visible.  If not, read on.

Troubleshooting the ACCESS System

For some reason, PBS does not load via ACCESS on all claimed

compatibles.   Obviously, this is due to some of the subtle

hardware differences between machines.   It stands to reason that

if the ACCESS system loaded,  PBS should kick-in with no problem.

Unfortunately, this is not always the case.   Should the DOS

prompt appear after your PBS request from ACCESS,  reload ACCESS

and try it a second time.   Possibly the shell partition was not

properly reserved on the first attempt.

NOTE:   If you have been using your machine for other memory

resident programs, be sure to do a three finger reboot before

loading ACCESS.

If after the reboot and reload, ACCESS can still not call PBS, it

might be safe to assume that your machine is not 100%  comptible

with COPAN/II.   Not to worry however, you can run PBS directly

from DOS.

#

Running From DOS

If you are using the ACCESS System,  you need not bother yourself

with the following instructions.   The exception of course is if

you were not able to load PBS from ACCESS due to hardware

incompatiblity problems.

To load PBS from the DOS prompt, enter BS.  A few examples are

given:

A>BS   or   B>BS   or   C>BS   or   C\COPAN>BS

Be sure to hit the <CR> after you have typed this.

How to Operate PBS

Analyzing composite plates for strength and stiffness is quick

and easy with PBS.   The following is a list of the information

which you must provide in order for a calculation to occur.

1.  LAMCON file name

2.  Plate dimensions

3.  Fiber Reference Orientation (optional)

4.  Loading in a uniform pressure

Using PBS keywords

When using PBS, it will be necessary to enter specific keywords

in order to produce the required prompts.   Each keyword is

abbreviated with a certain number of leading characters.   For

example, to exit, you would type EXI.   To load a laminate, you

would key in LAM.  Upper or lower case is fine.  If you enter too

many or not enough characters,  PBS will clear the input line and

redisplay the happy blinker.   As a guide, all keywords are

displayed on the 25th row of the screen with the necessary

leading characters in UPPER case.

Entering a LAMCON file

The first order of business is to call in a LAMCON property file.

When PBS loads, the opening screen will appear.  The screen

format resembles PCAT and LAMCON in that a rectangular box is

produced on top of a list of available keywords.   One noteworthy

difference is that the keywords must be entered manually.   They

are not assigned to the function keys.

#

To enter a LAMCON file,  type in the keyword LAM when the happy

blinker is visible.  This will produce the following prompt.

Prompt 1 --->      Enter LAMCON / .PRP Laminate:

PBS is waiting for a the name of an existing LAMCON file.   You

should not enter the .PRP as only .PRP files can be entered.  PBS

will attach this file extension for you.  The reason it does not

say --->  Enter LAMCON file:  is because PBS does not assume that

the  file  was created with LAMCON.   It is possible for you to

build and analyze an ASCII .PRP file without using the LAMCON

module.

If you decide to cancel without entry, press the <CR> with

nothing entered.

After entering a valid LAMCON file name,  PBS will read the file

and produce a status box in the upper left hand corner of the

screen.  This box will always be present to remind you which

laminate file is in memory.   The information in the LAM box is

the JOB (file name),  the total ply quantity and the overall

laminate thickness.

If the file shown in the LAM box is not correct,  you may type in

LAM again and prompt 1 will be redisplayed.   It is then possible

to load in a different LAMCON file.   If you have forgotten the

name of the file,  you will have to exit from PBS using the EXI

keyword.  The directory may then be reviewed from DOS or from the

ACCESS program using the [Esc] key function.

Defining Plate Dimensions

The plate thickness will have been predefined based on your

material selections from LAMCON.  This leaves only the length and

width to be entered.  The units in which the X-Y values should be

entered will be displayed just above the happy blinker.

To activate the necessary prompt,  key in the code,  PLA from the

happy blinker.  Don't forget the <CR>.

Prompt 2  --->    Enter Plate Dimensions (### ###)

Enter the values that you wish your plate to assume.   Keep in

mind the format required here.  PBS will be expecting both values

to be entered on the same line separated with one or more blank

spaces.   If you should press return with only a single value

entered, or if you forgot the blank space, PBS will let you know.

#

A PBS Modeling Hint

When doing a ply-by-ply stress analysis,  keep the plate

dimensions equal, that is to say, analyze square plates.

Using aspect ratios other greater than four, with certain

laminate cross-sections, can sometimes produce "uncertain"

stress results.

For this reason, we recommend that you get in a habit of

using square plates for stress problems.   If you are only

concerned with laminate bending results, aspect ratios  of

up to five produce very reliable solutions.  Ratios higher

than five are accomodated but the theory of plates is being

stretched a bit (No pun intended) so be careful.

After the plate dimensions have been entered and accepted by PBS,

a second status box is displayed just below the LAM box.  The

information given keeps you abreast of your plate size and the

orientation of the laminate with respect to the plate side.

Modifying the Laminate Angle

The laminate angle will automatically default to a value of 0

degrees AFTER the plate dimensions are entered.   This default

value will be displayed in the PLA status box which is located

about one third of the way down the left side of the screen.

To change this 0 to a non-zero, enter the first three characters

of the keyword ANGLE.  This will produce the following prompt:

Prompt 3 --->   Laminate angle to first side?

At this prompt, you should then enter a value between 0 and 90.

This value will then be displayed in the PLA status box.

Why Use the Laminate ANGle?

The ANG feature is used to orient your LAMCON cross-section

with the plate that you are analyzing.   Remember that the

section you constructed with LAMCON has no physical

references with which to align the fibers.   The ANG option

allows you to define a reference axis.

#

Suppose you created a LAMCON file with all fibers running

at 0 degrees.   Then, in PBS, you defined a plate with X-Y

dimensions of 40 and 60 inches.   It is obvious that the

alignment of the unidirectional fibers within the plate

will have a substantial impact on the plate stiffness.

If the ANG option is not used when calculating this plate,

the fibers will be aligned parallel to the short side of

the plate.    To rotate the orientation so that the fibers

run  perpendicular to the short side,  a ANG value of 90

would be entered.

Consider this example as well.   A LAMCON file is defined

with a series of 0/90 roving plies.    The file is then

submitted to PBS to determine the amount of bending under a

specific pressure.   The same laminate can now be rotated

45 degrees within the plate to make a direct comparison to

between the 0/90 orientation and the double bias

configuration.

Entering a Bending Load

The final requirement for producing PBS results is the load

entry.  In this version, the load is always an evenly distributed

pressure acting normal to the plate surface.   A normal force

vector is perpendicular to the plane of the plate.

The units in which the pressure should be entered will be

displayed just above the happy blinker.   They will appear in  a

reverse video format immediately after the LAMCON file is read.

To enter a pressure, invoke the prompt with the key code BEN LOA

It is necessary to have a space between the BEN and the LOA

Prompt 4 --->     Enter a uniform pressure:

Any real value will be accepted.  When the entry is complete, PBS

will display the boundary conditions (simply supported) and the

pressure that will be applied.

#

Submitting the LAMCON File for Processing

To produce results, the file must now be submitted to PBS for

processing.   This final task is accomplished with the following

key code: BEN SOL.  This is an abbreviation for Bending Solution.

After this command is entered, and if all required input data has

been supplied,  PBS will respond with a prompt for an output file

name.  This  is an ASCII file that PBS will use for storing the

calculation results.  If you do not enter a name, PBS will assume

that you do not want the output saved on disk.  In this case, all

output will be routed to the screen; data will not be saved.

Quick Printer Summary

If all that you require is a quick summary, the print-screen

function  can be used AFTER the results  have  been

written to the monitor.   A [Ctrl] [PrtSc] will send all

text to LPT1, which is the primary parallel printer port.

More Words About the Output File

By supplying an output file name, you will be creating a type of

outbox for all subsequent results.   Each time that you BEN SOL a

laminate file,  PBS will ask you if you wish to add to the outbox.

The prompt will appear as:

Prompt 5 --->   Append output file (Y/N)?

By pressing the <CR> without a typed response, PBS will assume

you wish to append the outbox.   Pressing the [N] key and hitting

the <CR> will suspend output to the file.    On the next

calculation, the prompt will again appear.  This way, you may be

selective with the information that is compiled to the outbox.

With this concept, it is possible to write all information to a

single report.   Rather than having to "search and collect"  all

the necessary single output files, one report can be reviewed and

acted upon.

Reviewing Output Files

The output files created with PBS are formatted  ASCII  files.

This means that the file on disk is in the same layout as you

would see on a 80 column sheet of paper.   The data contained by

any PBS output file can be reviewed with either an editor or via

a line printer.

#

(1)  Using a Full Screen Editor

If you have your own favorite editor such as  WORDSTAR,  open a

non-document file to review COPAN/II text files.

(2)  Outputting Text Files to the Printer

The output files created with PBS can be routed directly to  the

line printer for hard copy review.   This can be done a number of

different ways.

The quick way out is to use the DOS function called PRINT.  The

major disadvantage with PRINT is that COPAN/II files are not page

formatted.  This means that a ^L character is not embedded in the

text which causes the printer to do a form feed.   What you will

end up with when using PRINT is your output printed across the

form perforations.   If you must use PRINT,  you could always go

into the text, and insert some ^L characters.  This would cause

the printer to page feed at each occurence.   Of course, the

better way is to use LINER.

Using LINER to Print Text Files

A smarter option is using the COPAN/II module called LINER.  This

is a separate module which has been designed specifically to

print, fully page formatted, COPAN/II text files.  This program

can be loaded via ACCESS.    Turn to the section of this

manual  that addresses the operation of LINER.

Summary of a .OUT File

All of the information that is presented on the video screen is

also written to the output file.   The disk output file also

contains a stiffness matrix of equivalent material.   This

information is available only when a output file is requested.

Generally speaking, the output file consists of three parts.  The

first section is the calculation parameters and laminate file

details such as name, thickness,  x-y dimensions, fiber/plate

orientation, loading and so on.

The second section contains the matrix information.  Two matrices

are given for each laminate cross-section.   The matrices are

printed for the first calculation.    Then for subsequent

calculations on the same laminate, the matrix data is not printed

if the matrix values do not change.    For example, if a laminate

file is calculated several times with the pressure different on

each run, the matrix information will be printed only once.  When

a change is made to the laminate that cause the stiffness matrix

to be recalculated, the new values will be written.

#

The third and final section, of each output report is the bending

and ply stress data.  A maximum plate displacement is provided

along with three maximum stress values per ply.

Maximum plate deflection will occur at the middle of the plate.

The stress information is defined as Sig-1,  Sig-2 and  SHEAR.

The short side of the plate is sigma 1 so it follows that the in-

plane  stresses  for Sig-1 are in  that  direction.   Sigma-2  is

obviously   the  in-plane  stresses  in  the  direction  of   the

longitudinal  edge.   SHEAR is a little different than what you

might expect.

With normal plate theory,  the subject of shear stress through

the thickness is given all too little attention.   With

isotropics, this is no problem,  as shear strength is usually

high enough to not cause any problems.   This is certainly not

the case with low to medium density  plastic foam cores.   Rather

than ignore this,  we have built in a custom routine which

produces realistic core shear results.   This alogrithm was

developed using a mainframe finite element processor which

incorporated a true composite element.  By make multiple runs

with this system,  we have calculated our own table of

coefficients for laminated sandwich plates.

#

A Word to Our Wise Users

While we are very pleased with the results,  it is not possible

for us to say that the calculated shear values will be identical

to the stress levels that your core may experience within any

given structure.   We feel obligated to make this statement

because of the inherent variability of composite constructions.

The Story of a Sap

In discussing this, it brings to mind the small story of how a

finite element processor was given its name.   The program, known

as SAP, was one of the early mainframe codes that gave engineers

a real taste for full powered designs.   For many years, users

referred to the program and, quite unfortunately, the meaning

behind the name was forgotten.   What took its place was a not so

clever acronym for Structural Analysis Program.

The original meaning of SAP was just that, sap.   You know, the

noun sap, as in without brains, idiot; your basic dumbshit.  It

was given that name to always remind engineers that computer

programs, especially the complex ones, are dangerous tools unless

used by thinking - reasoning - scrutinizing humans.   This one is

no different.   Never assume that a calculation is anything other

than just a mathematical approximation.    What you can assume

quite confidently,  is that #error always exists# and its your job,

as the responsible technician, to determine if the amount of

error is acceptable.

#

COPAN/II Plate Bending Benchmark

In order to verity that PBS is producing reliable results,  a

benchmark calculation can be run using the following plate

bending problem:

Plate dimensions:   78.771" x 78.771"   (2000mm  x  2000mm)

Thickness       :   .4726"              (12mm)

Plate Stiffness :   3.04 E+07           (21,000 daN/mm^2)

Poisson ratio   :   .30

Uniform Pressure:   .29 lbf/in^2        (200 daN/M^2)

Edge Fixity     :   Simply Supported

The  results  obtained  with  COPAN/II  are  compared  with   the

analytical  solution  for  maximum  deflection as  given  in  the

Therory of Plates and Shells by S. Timoshenko (Ref 1)

Maximum Deflection

Theory    COPAN/II    % Error

======    ========    =======

.1540"     .1546"      .3896

(3.912)    (3.925)

This benchmark should be used to periodically ensure that PBS is

producing predictable results with your specific   hardware

configuration.   If the solution  that your machine produces is

significantly different from the value given above - you've got a

problem.

Note:   A file has been included on one of the COPAN/II diskettes

which is contains the above properties.   All you will have to do

is enter the plate dimensions and loading.

The file name is BENCH.PRP

REF 1. Timoshenko - Woinowsky - Krieger.

Theory of Plates and Shells, 2nd Edition, New York 1969