POWERPAK

Version  1.0

Power Distribution Design Software

for

Electric Utility Engineers and Consultants

by

George Kwascha, PE

copyright 1984, 1985

POWERPAK software has been placed in the public domain for free

distribution.  The  programs  are provided on as "as is"  basis

without warranty of any kind,  expressed or implied,  including

but  not limited to implied warranties of  merchantability  and

application  for  a particular purpose.  The user  assumes  all

responsibility  for  damages of any sort arising from  the  use

and/or application,  direct or indirect,  of PowerPak software.

This  software  is  not  a  replacement  for  good  engineering

practices.

TABLE OF CONTENTS

page

Introduction .............................................  3

System Files .............................................  4

Preparing Your PowerPak System Disk ......................  5

Running PowerPak .........................................  5

General Instructions .....................................  6

OPTION 1- Conductor Tables ...............................  7

OPTION 2- Voltage Drop Calculations ......................  9

OPTION 3- Secondary Fault Current Calculations ........... 11

OPTION 4- Transformer Tables ............................. 12

Future Enhancements and Updates .......................... 14

Software Registration .................................... 15

Invoice and Feedback Form ................................ 16

2

------------

INTRODUCTION

------------

PowerPak  has  been designed for use by  the  Electric  Utility

Distribution Engineer or Consultanting Engineer with the intent

of simplifying basic design calculations necessary in providing

electric  utility  service.  This preliminary version  provides

tools for secondary voltages calculations. Future versions will

support primary voltage circuit calculation tools.

PowerPak  allows the user to create a set of  parameter  tables

for  secondary conductors and transformers.  This set of tables

is  utilized  by PowerPak to make calculations easier  for  the

user   by   eliminating  the  need  to   constantly   look   up

specifications for voltage drop and fault current calculations.

PowerPak  allows the user to perform individual voltage drop or

fault current calculations or create tabular outputs.  This  is

further   discussed   in  the   documentation.   Voltage   drop

calculations  can  be  done  with  or  without  motor  starting

criteria.  Secondary  fault current calculations can be done at

the  point  of  delivery,  whether it  be  at  the  transformer

connections or at the end of the service conductor.

PowerPak  is  menu-driven  for ease of use  with  user  prompts

highlighted  at the bottom of each screen.  All data is entered

on  formated  screens.  PowerPak is designed to be run  from  a

single  floppy  system with a minimum of 64K of memory  with  a

color/graphics adaptor.

3

------------

SYSTEM FILES

------------

PowerPak  is configured to run self-contained on a floppy disk.

The floppy disk can reside in either drive and the program will

function properly as long as the resident drive is the  default

drive.  PowerPak  can also be operated from a hard disk as long

as  all  files  are  in the same  sub-directory  and  the  sub-

directory is the default directory.

The following files come with the PowerPak 1.0 disk:

README       - initial instruction & help file

POWERPAK.DOC - program documentation in ASCII format

POWERPAK.TXT - program documentation in WordStar format

POWERPAK.COM - main control and loading module

MAINMENU.CHN - main menu

CABLSPEC.CHN - secondary conductor data table access program

TXSPECS.CHN  - transformer data table access program

SECVDROP.CHN - secondary voltage drop loading program

CABLDROP.CHN - secondary conductor voltage drop calculator

TXDROP.CHN   - transformer voltage drop calculator

CABLMOTR.CHN - secondary conductor voltage drop with motor

starting calculation

TXMOTR.CHN   - transformer voltage drop with motor starting

calculation

STLTDROP.CHN - street light design voltage drop calculator

STLIGHT.PIC  - street light calculator input screen

CABLE.DAT    - secondary conductor data file

TRANSF.DAT   - transformer data file

It  is  recommended  that  the user  format  a  new  disk  with

operating  system and then transfer all the above files to this

new disk before using the program. Store your original PowerPak

disk in a safe place.

4

-----------------------------------

PREPARING YOUR POWERPAK SYSTEM DISK

-----------------------------------

The  first  step  in preparing your  PowerPak  system  disk  is

formatting  a new disk.  Please follow instructions in your DOS

manual, if you are not already familiar with this procedure. Be

sure to format this new disk with the   /S  option.  This  will

also  transfer the DOS system onto your new disk.  With the DOS

system  on  your disk,  you will be able to  boot  your  system

directly from the PowerPak disk.

The  next step in this process is to tranfer all PowerPak files

to your newly formatted disk.  Again,  refer to your DOS manual

if you are not familiar with this procedure. Using the COPY *.*

command is recommended.

For your information,  the original PowerPak disk comes on an 8

sector formatted disk to allow compatibility with all  versions

of DOS. The disk is double-sided.

----------------

RUNNING POWERPAK

----------------

To  start  PowerPak,  simply type POWERPAK from the A>  prompt.

Note that your PowerPak disk must be in drive A: throughout the

duration of your session.

You  may wish to create a Batch file or and AUTOEXEC.BAT  file.

Refer  to  your DOS manual if you are not familiar  with  batch

files.  When creating a batch file,  simply include POWERPAK as

your last command.

5

--------------------

GENERAL INSTRUCTIONS

--------------------

1.  All  menus require single keystroke entry when choosing  an

option. Options are numbered anywhere from 0 to 9.

2.  Except for the Main Menu,  all single keystroke options are

bracketed with {  } on menu screens.

3.  There  are several standard prompts that will appear at the

bottom of the screen:

Field  {#} - hit the appropriate number that  corresponds  with

the field that you wish to correct.

{+} - hitting  the  plus sign will increment the  record

forward by one.

{-} - hitting  the minus sign will increment the  record

backwards by one.

{M} - hitting  the  "M"  key  will  return  you  to  the

previous menu.

{E} - hitting  the "E"  key will allow you  to  view  or

print the entire file.

{P} - hitting  the "P" key will specify that any  output

be redirected to the printer.

{S} - hitting  the "S" key will specify that any  output

be directed to the screen.

{space} - hitting the space bar will scroll window data.

{RETURN} - hitting  the Return or Enter key will perform  the

action specified in the prompt.  These will vary from screen to

screen.

4.  Hitting the {ESC} key from most prompts in the program will

return you to the previous menu.

5.  The  only other type of data entry prompt is found  in  the

conductor  and transformer data files,  where the  --->  prompt

asks you to enter a code.  Since this code can be more than one

character in length,  it will be necessary for you to terminate

your entry by hitting the RETURN or ENTER key.

6. Use {SHIFT} + {PrtSc} to dump screen results to your printer.

6

-------------------------------------

MAIN MENU OPTION 1 - CONDUCTOR TABLES

-------------------------------------

The  Conductor  Data file can be accessed by choosing option  1

from  the  Main Menu.  You will be presented with a  menu  that

allows you to Add,  Edit,  Delete and Print/View your conductor

records.

A sample data file is provided on the disk.  If your conductors

do  not match the ones specified in this sample file,  you  may

change the appropriate conductors one by one or you may  delete

the  CABLE.DAT  file and start your own file.  The Edit  option

allows you to change data in each record. The Add option allows

you to add records to the file. If the file does not exist, the

Add option will create the file automatically.  Option 4 is not

functional at this time.

{1} ADD CONDUCTOR DATA

When  this  option is chosen you will be presented with a  data

input  screen that allows you to enter the following  conductor

parameters:

Conductor Code- a unique code assigned by the user.  This  code

can  be up to 3 characters in length,  comprised of numbers  or

letters.  It  is used to retrieve conductor data records.  This

value is treated as a string variable.

Cable Type- 20 spaces are provided for the user to describe the

type of conductor.

Phase Conductor Size-   9 spaces are provided for the  user  to

record the size of the phase conductor.

Rated Amps-  3 spaces are provided to denote the rated  current

carrying capacity of your cable.

Phase Cond Resistance-  specify the phase conductor  resistance

value in ohms/1000 feet. 6 spaces are provided.

Phase Cond Reactance-   specify the phase  conductor  reactance

value in ohms/1000 feet. 6 spaces are provided.

Neutral Conductor Size-  9 spaces are provided for the user  to

record  the size of the neutral conductor.  If the size is  the

same as the phase conductor, just hit the RETURN key.

Neutral Cond Resistance-   specify neutral conductor resistance

value in ohms/1000 feet.  6 spaces are provided.  If this value

is the same as the phase conductor, just hit the RETURN key.

7

Neutral  Cond Reactance-  specify neutral  conductor  reactance

value in ohms/1000 feet. If this value is the same as the phase

conductor, just hit the RETURN key.

{2} EDIT CONDUCTOR DATA

When  this  option  is  chosen you will  be  prompted  for  the

conductor code that you wish to edit or you can scroll the file

forward   or  backward  by  hitting  the  plus  and  minus  key

respectively.  When  you wish to edit a field on the  displayed

screen,  type  the number of the field and then  re-enter  your

data.

{3} DELETE CONDUCTOR DATA

When  this  option  is  chosen you will  be  prompted  for  the

conductor code that you wish to delete.

The  Delete  option does not actually delete  the  record.  Any

record that is deleted by the user is only marked for deletion.

The next record that is added is written into the record  space

of your deleted record.

{5} PRINT/VIEW CONDUCTOR RECORDS

When this option is chosen you will be prompted for a conductor

code.  If you type a plus sign, the records will be scrolled on

the  screen.  Typing the letter "E" specifies the entire  file.

When specifying the entire file,  you will also be prompted for

screen or printer output.

{6} RETURN TO MAIN MENU

This option returns you to the Main Menu.

8

--------------------------------

MAIN MENU OPTION 2- VOLTAGE DROP

--------------------------------

The  Voltage  Drop  calculation  option is  comprised  of  five

different types of calculations.

{1} Service Cable Voltage Drop- calculates the voltage drop  on

secondary  conductors  under  normal  loading  conditions.  The

conductors must be in the conductor data file.

{2} Transformer Voltage Drop-  calculates  the voltage drop  in

the  transformer  core under normal  loading  conditions.  This

value should be added to the service cable voltage drop to  get

the  total drop seen by the associated load.  The  transformers

must be in the data file.

The two sections above have the same basic screen input design.

A sub-menu will allow you to choose whether you wish to perform

one calculation for a particular situation or whether you  wish

to  create a set of voltage drop tables for various parameters.

The  table  creation feature is a very  valuable  feature  that

allows you set up a complete set of tables for each transformer

and conductor with different ranges of footage and load.

The  single calculation option will prompt you for a  conductor

or transformer code by displaying a window of the data file and

allowing  you  to  scroll through the file searching  for  your

conductor or transformer. You will be prompted for power factor

and  load values.  The conductor calculation will also  require

the  entry  of the number of  phases,  voltage,  and  conductor

length  in  feet.  It will also ask you to  specify  single  or

parallel conductors.

The cable option will calculate the line-to-neutral and line-to

line voltage drop in volts and percent.  The transformer option

will  calculate the voltage drop in percent for the given  load

and  for full load.  The full load value can be used as a worst

case   value  if  there  are  other  loads  connected  to   the

transformer.

The   respective  table  calculation  option  will  need   some

additional  values.  The cable option will requires a  starting

load value with an increment. For example, if you wish to start

your table at 10 KW and increment it by 5 KW, you would specify

the starting load at 10 KW and load increment at 5  KW.  Values

for  starting  length and length of increment in feet are  also

needed.  The transformer option only requires the starting load

and load increment values.

{3} Cable Voltage Drop for  Motor Starting-   calculates  the

voltage  drop associated with motor starting only.

9

{4} Transformer Voltage Drop for Motor Starting- calculates the

voltage  drop associated with motor starting only.  This  value

may be added to the Cable Motor Starting value to get the total

drop due to starting motors.

The two sections above have the same basic screen input design

as  the voltage drop calculations without motor  starting.  The

only  additional  parameter needed is a value for the  starting

amps of the motor or motors.  This value is usually a nameplate

value  called locked-rotor amps.  An approximate value  of  six

times the normal current can also be used.

{5}  Street Light Voltage Drop- a specialized section  designed

to calculate voltage drops for street light design.

This  option  presents you with a graphic representation  of  a

street  light circuit,  showing cable impedance  and  luminaire

locations.  A  table allowing input of footage,  conductor code

and number of luminaires at each point is displayed.  Enter the

secondary voltage at the prompt.  Enter the load per  luminaire

next.  This  program assumes that all luminaires in use are the

same.  Enter the luminaire load value in VA, not KVA! Next, the

program will allow you to scroll through the conductor file  to

pick  out  your  cable size.  Enter the conductor code  of  the

segment  in  column  one.   Enter the number of  feet  in  each

segment  in the second column.  The third column  requests  the

number  of  luminaires  that are connected at  that  point  and

beyond. For example, if one luminaire is connected at points 1,

2,  and  3,  then you specify 3 luminaires at point 0 to 1.  At

point 1 to 2 you specify 2 luminaires and 1 luminaire at  point

2 to 3.

After  the  entry  of  each  line  segment,  the  program  will

calculate  and display the voltage drop,  the actual voltage at

that end point, and the total percent drop at the end point.

A suggestion ...  if you have more than 10 luminaires,  you can

combine  cable segments and luminaires into one point near  the

voltage  source.  In this manner you will still be able to  get

the voltage drop values toward the end of the circuit.

10

----------------------------------

MAIN MENU OPTION 3 - FAULT CURRENT

----------------------------------

Option 3 will allow you to perform secondary fault caluclations

with ease. You can make single calculations or create tables of

fault  currents  for each type of conductor in the  data  file.

This  is  done  in  much the same manner as  described  in  the

Voltage Drop sections.

You  will  need to specify a transformer code and  an  optional

conductor code and conductor length.  This is done in the  same

manner  as  described  in the previous  sections  by  scrolling

through the data files in the window displays. If a fault value

is needed at the transformer spades only, then a conductor code

should not be entered. If a conductor code is entered, then you

must  specify  a conductor length of zero feet,  otherwise  the

calculation will include the secondary conductor and the  fault

current value given, will be at the end of the conductor.

A  table  of  fault currents is  generated  for  conductors  of

various  lengths only,  with a particular transformer  type.  A

table  of  various transformers cannot be  generated.  Use  the

single calculation option to do this.

11

---------------------------------------

MAIN MENU OPTION 4 - TRANSFORMER TABLES

---------------------------------------

The Transformer Data file can be accessed by choosing option  4

from  the  Main Menu.  You will be presented with a  menu  that

allows you to Add,  Edit,  Delete and Print/View your conductor

records.

A sample data file is provided on the disk.  If your conductors

do  not match the ones specified in this sample file,  you  may

change the appropriate conductors one by one or you may  delete

the TRANSF.DAT  file and start your own file.  The Edit  option

allows you to change data in each record. The Add option allows

you to add records to the file. If the file does not exist, the

Add option will create the file automatically.  Option 4 is not

functional at this time.

{1} ADD TRAMSFORMER DATA

When  this  option is chosen you will be presented with a  data

input screen that allows you to enter the following transformer

parameters:

Transformer Code- a unique code assigned by the user. This code

can  be up to 3 characters in length,  comprised of numbers  or

letters.  It is used to retrieve transformer data records. This

value is treated as a string variable.

Transformer  #- 5 spaces are provided for a transformer  number

if necessary.

Transformer  Type- 20  spaces  are provided  for  the  user  to

describe the type of transformer, ie. aerial or pad-mounted.

#  phases- specify "1" for single phase or "3" for three  phase

transformers.

TX  Size (KVA)- 4 spaces are provided to record the transformer

KVA.

TX  Impedance  (%)- 5 spaces are provided for  the  transformer

impedance value in percent.

Nameplate Voltage Rating- 32 spaces are provided to record  the

nameplate voltage information.

Secondary  Volts  (line-line)- record the secondary voltage  in

volts.  5 spaces are provided.  Line-to-line or  phase-to-phase

voltage must be used. This value is used in the calculations.

Total  Losses  (Watts)- 5  spaces  are provided  to  enter  the

transformer  total full load losses expressed  in  watts.  This

12

value includes no load core loss and full load copper loss.

No  Load  Loasses (Watts)- 5 spaces are provided to  enter  the

transformer no load core loss in Watts.

{2} EDIT TRAMSFORMER DATA

When  this  option  is  chosen you will  be  prompted  for  the

transformer  code  that you wish to edit or you can scroll  the

file  forward  or backward by hitting the plus  and  minus  key

respectively.  When  you wish to edit a field on the  displayed

screen,  type  the  number of the field and then re-enter  your

data.

{3} DELETE TRANSFORMER DATA

When  this  option  is  chosen you will  be  prompted  for  the

transformer code that you wish to delete.

The  Delete  option does not actually delete  the  record.  Any

record that is deleted by the user is only marked for deletion.

The next record that is added is written into the record  space

of your deleted record.

{5} PRINT/VIEW TRANSFORMER RECORDS

When  this  option  is  chosen  you  will  be  prompted  for  a

transformer code.  If you type a plus sign, the records will be

scrolled  on the screen.  Typing the letter "E"  specifies  the

entire file.  When specifying the entire file, you will also be

prompted for screen or printer output.

{6} RETURN TO MAIN MENU

This option returns you to the Main Menu.

13

-------------------------------

FUTURE ENHANCEMENTS AND UPDATES

-------------------------------

PowerPak   is  distributed  under  a  modified   user-supported

shareware  concept.  Please read the following statement  under

software registration for a complete explanation. The author is

relying  on  the  users  of PowerPak to  make  suggestions  for

improvements and enhancements that they would like to see.

The author will notify all registered users any time  revisions

are  made to PowerPak.  Registered users may then obtain a copy

of the updated disk for a $5.00 fee (subject to  change).  This

fee  is to cover the cost of disk media,  postage and handling.

The  user  may  also choose to send a blank  disk  with  proper

enclosed  return  postage  for a free updated  copy.  The  disk

mailer must be re-usable.

The author has spent a great deal of his own time writing  this

program,  with hopes of benefiting from user contributions. The

software  availability  of  good inexpensive  programs  in  the

electric  utility  area is limited.  PowerPak is an attempt  to

change  this.  With  your  help  and  support,  maybe,  we  can

encourage  other engineers to write and release their  programs

to  the  realm  of User-Supported  Shareware.  Help  make  this

experiemnt a success and send a donation.

If  you  have written useful engineering software and  wish  to

participate in this experiment please let us know.  Wordstar is

a registered trademark of MicroPro International.

14

---------------------

SOFTWARE REGISTRATION

---------------------

PowerPak  is  distributed  under  a  modified  version  of  the

shareware  concept.  PowerPak  has been released to the  public

domain for free distribution.  Users are encouraged to copy the

program for use by others. This includes use by individuals and

companies.  This  program  may  not  be  sold  without  written

permission  of  the  author,  with  one  exception.  Non-profit

organizations,   such  as  computer  clubs,   user-groups   and

electronic bulletin boards may sell this software as long as it

is for no more than $6.00. Source code is not available.

The  author  will  support  PowerPak  users  for  a  one   time

contribution of $10.00. Support includes notification of future

updates,  corrections  of any "bugs" in the software,  and  any

relevant  written correspondence that requires a written reply.

All users,  however,  are encouraged to supply the author  with

any  suggestions  for future improvements to this software  and

any  other comments that do not require a written reply by  the

author. The author will not supply copies of PowerPak without a

contribution of $10.00 for user-support and an additional $5.00

for  postage,  handling  and disk media.  Free copies  of  this

software can be obtained from your colleagues,  computer clubs,

user groups or electronic bulletin boards. If you have obtained

a  free  copy of this software and wish to  register  with  the

author please send your contribution of $10.00 only. Please use

the attached invoice form for all correspondence.

15

------------------------------

INVOICE AND USER FEEDBACK FORM

------------------------------

Name: __________________________________________

Company: _______________________________________

Address: _______________________________________

City: ________________ State: ____ Zip: ________

===============================================================

Item Description                          | Price Ea |   Price

===============================================================

|          |

PowerPak 1.0 Registration fee             |  $10.00  |  $

|          |

PowerPak 1.0 software on 5" disk          |  $ 5.00  |  $

postage included                         |          |

-----------------------------------------------------|---------

Total               |

===============================================================

Make checks payable to: Information Management Solutions, Inc

Send To: Information Management Solutions, Inc

P.O. Box 13263    Durham, N.C.  27709

List  any  comments or suggestions for future  improvements  to

PowerPak below. Use reverse side if necessary. Your interest is

appreciated.

16

#

#rest i