FFT-256 Documentation

Copyright 1991 by Harry Garland, Ph.D.

1) Introduction

FFT-256 is a software package that makes it easy for you to find the

spectral content (i.e. the sine wave or "Fourier" components) of a

waveform or set of data.  The program achieves this by taking

the 256-point Fast Fourier Transform (FFT) of your waveform or data.

This determines the amplitude and phase of 128 sinusoidal waves

that, when added together, exactly match your original data or waveform.

Both the amplitude and phase of these 128 Fourier components can be

displayed at your console.  You can also list the amplitude

and phase of these components to your printer or to a disk file.

To illustrate graphically that the 128 Fourier waves do indeed sum to

your original waveform, you can invoke the "Synthesize" option

to watch your waveform materialize as the 128 Fourier components

are added together.

There are seven files in this package:

FFT256.exe      The main program

FFT256.doc      This documentation file

Wave1.wve       Example waveform #1

Wave2.wve       Example waveform #2

Wave3.wve       Example waveform #3

Wave4.wve       Example waveform #4

Wave5.wve       Example waveform #5

2) Example waveforms

To view the example waveforms, press 1 from the Main Menu to "Begin

program."  From the Program Menu press 3 to select "Disk file."  To

select Wave1.wve simply type Wave1 followed by a carriage return.

The waveform will then be displayed on the screen.

Each of the five example waveforms is described below:

2.1) Wave1

Wave1 is a waveform described by the formula y = sin (2*pi*16*t/256).

After selecting this waveform you can view its spectral content by

pressing "F" to perform the FFT.  From the spectral plot you can see

that there is a single sine wave component, representing the 16th

harmonic of the fundamental.  The fundamental frequency is defined

here as the frequency corresponding to one cycle over 256 samples.

The 16th harmonic then has 16 cycles over the 256 samples.  In

general the formula for harmonic number "h" is given by

sin (2*pi*h*t/256).

While viewing the amplitude spectrum you can press P to view the

phase or "shift" of each of the spectral components relative to

a cosine wave.  Notice that the 16th harmonic is shifted by 90

degrees.  This is because there is a 90 degree phase difference

between a sine wave and a cosine wave.

2.2) Wave2

Wave2 is designed to illustrate the superposition or addition of

sinusoidal waveforms.  Wave2 is defined by the formula:

(1/2) * sin(2*pi*16*t/256) + (-1/2) * cos(2*pi*32*t/256)

After loading this waveform, press F to take the FFT and view the

spectrum.  Notice that both the 16th harmonic and the 32nd harmonic

appear in the Fourier amplitude spectrum.  Press P to view the

phase angles of the Fourier components.  The 16th harmonic is a

pure sine wave with a 90 degree phase shift.  The 32nd harmonic has

a 180 degree phase shift because the cosine component of the waveform

is multiplied by a negative number.

Wave2 is a composite waveform formed by the superposition of two

component waveforms that interact to produce a more complicated

waveform.  The FFT, however, effectively resolves this complexity

and successfully extracts the two component waveforms from the

composite waveform.

2.3) Wave3

Wave3 illustrates the effect of multiplying two sinusoidal waveforms.

This waveform is defined by the formula:

y = cos(2*pi*64*t/256) * sin(2*pi*60*t/256)

In taking the FFT of this waveform there is a surprise!  Even though

we are multiplying the 64th harmonic by the 60th harmonic, NEITHER

of these harmonics appear in the resultant spectrum!  Instead we have

the sum and the difference frequencies (sometimes called the sidebands.)

The amplitude of each of these is just half that of the original waves.

Notice also the phase of these two components (by pressing P while

viewing the amplitude spectrum.)  The 4th harmonic is shifted -90 degrees

from a cosine wave (i.e. it is the negative of a sine wave) and the

124th harmonic is shifted 90 degrees from a cosine wave (i.e. it is

a sine wave.)  In other words the mathematical expression for our

original wave, namely:

y = cos(2*pi*64*t/256) * sin(2*pi*60*t/256)

is mathematically the same as:

= .5 * sin(2*pi*124*t/256) - .5 * sin(2*pi*4*t/256)

and in fact this identity can be proven true from trigonometry.

It is always amazing to me that the Fourier transform somehow

"knows" the trig identities!

2.4) Wave4

Wave4 is a pulse waveform.  Because of the symmetry of this pulse, it

is composed of only cosine wave components (i.e. the phase of each

component is either 0 degrees or 180 degrees.)  Notice the undulating

pattern of harmonics so typical of the pulse waveform (the mathematical

formula for these harmonic amplitudes is described in my tutorial

program "Fourier-7" described in section 4.0 below.)

It is interesting to see the rectangular pulse as it is synthesized

from its harmonic components.  While viewing the spectrum amplitude

display, select the Synthesize option by pressing S.

2.5) Wave5

Wave5 is an example of a waveform made up of a strong sinusoid component

and a large amount of superimposed noise.  The formula for this waveform

is given by:

y = .5 * sin(2*pi*16*t/256) + .5 * cos (t^3)

where cos(t^3) is our contrived way to generate a bounded uncorrrelated

noise source.  This waveform can be used to illustrate one practical

application of the FFT, namely recovering a signal that has been

corrupted by noise.  The amplitude spectrum of this signal shows very

clearly the dominant sinusoidal component, even though this would be

very difficult to discern accurately by looking at the time domain

waveform alone.  This technique has many applications, such as the

accurate determination of the frequency of the doppler shift in an

ultrasonic blood flow meter.

3) Your own waveforms

Having seen how these example waveforms work, you can experiment with

your own waveforms or data.  I have tried to make it as easy as possible

to enter your own data or wave information.  You have three options,

selectable from the Program Menu.

3.1) Cursor entry

Press 1 from the Program Menu to enter one cycle (256 data points) of

your waveform using the cursor keys on your keyboard.  These 256 data

points correspond to time samples from t=0 to t=255.  Once you enter

the last data point (t=255) you may choose to save your waveform to

a file on your disk (press S to save), take the FFT of your waveform

(by pressing F), abandon the waveform and enter a new one (Press N), or

return to the Program Menu (Press X).

3.2) Math formula

Press 2 from the Program Menu to enter a mathematical expression to

define your waveform. You will be prompted with "y = " after which

you can enter a mathematical expression of the variable t.  The

program will then compute and plot the value of your expression for

256 values of t ranging from 0 to 255.

The following functions and operators are supported:

ABS      Absolute Value          AND       Logical AND

ARCCOS   Arc Cosine              ARCCOSH   Arc Hyperbolic Cosine

ARCCOT   Arc Cotangent           ARCCOTH   Arc Hyperbolic Cotangent

ARCCSC   Arc Cosecant            ARCCSCH   Arc Hyperbolic Cosecant

ARCTANH  Arc Hyperbolic Tangent  ARCSEC    Arc Secant

ARCSECH  Arc Hyperbolic Secant   ARCSIN    Arc Sine

ARCSINH  Arc Hyperbolic Sine     ATN       Arc Tangent

CLG      Common Log (base 10)    COS       Cosine

COT      Cotangent               COTH      Hyperbolic Cotangent

CSC      Cosecant                CSCH      Hyperbolic Cosecant

EXP      Exp                     LOG       Natural Log (base e)

NOT      Logical NOT             OR        Logical OR

SINH     Hyperbolic Sine         SECH      Hyperbolic Secant

SEC      Secant                  SIN       Sine

SQR      Square Root             TAN       Tangent

TANH     Hyperbolic Tangent      !         Factorial

^        Exponentiation          *         Multiplication

/        Division                +         Addition

-        Subtraction             >         Greater than

<        Less than               =         Equal

The mathematical constants pi and e are also recognized.

Logical operations take the value of -1 when true, and 0 when

false.  For example, the following formula would take the value

of 0 for t greater than or equal to 64 and the value of -t/64 for

t less than 64:

y = (t<64) * t/64

Sine waves corresponding to the harmonics of the Fourier series

are represented by:

y = SIN(2*pi*h*t/256)

where h is the harmonic number (you must substitute a number for

the letter h in the above expression.)

Once you have entered your formula, you can save your formula

and the resultant waveform to a disk file by selecting the Save

option (press S).  Press F to take the FFT of your waveform. Press

N to enter a new formula.  Press X to return to the program menu.

3.3) Disk file

Press 1 from the Program Menu to read data that is stored in a disk

file.  A disk file can be created by invoking the Save option,

described above, after entering data using the cursor keys or after

producing a waveform from a mathematical formula.

This disk file can also be created by another program (such as a data

acquisition program) or you can create the disk file yourself using a

text editor, such as EDIT in DOS 5.0.  The format of the disk file is

simply an ASCII list of 256 numbers, each separated by a carriage

return.  If you wish to add a one-line title or description of your

file you can do so by adding this to the end of your list of numbers.

Each of the 256 numbers is called a record.  If you add a title it is

the 257th record.  A carriage return must separate the 256th number from

the title.  There should be no carriage return after the title.  The

title is optional.

When you Save a waveform that you created as a mathematical formula,

the formula itself is recorded as the title in the 257th record of the

disk file.  If you want to change this title, you can use a text editor

to do so.  When doing this, be careful to not insert a carriage return

at the end of the title.

4.0) License information

FFT-256 is a shareware program that may be distributed freely for

evaluation purposes.  The program may be registered for use by sending

$10 to the author at the address below.  I also have an extended

version of this program available for sale.  This program, FFT-1024,

gives you the option of working with 128, 256, 512, or 1024 sample

points.

If you would like to purchase FFT-1024 the price is $19.95 (which

includes postage.)  To order the 3.5 inch disk version order

FFT1024-S.  To order the 5.25 inch disk version, order FFT1024-L.

I also have an interactive tutorial program available that teaches

the priciples of Fourier analysis and synthesis through interactive

examples and on-line tutorial text.  The price for this package is

$24.95.  To order this tutorial software package on 3.5 inch disk

order Fourier7-S.  To order on a 5.25 inch disk order Fourier7-L.

The price of $24.95 for Fourier7 includes postage.

Please send your order to:

Harry Garland

27555 Purissima Rd.

Los Altos Hills, California

94022

Thank you.

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 3,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.