===========================================================

ELDA v2.1               (c)1994 Brian E. Kelly

===========================================================

Enhanced Line Diagnostics Analyser.  E.L.D.A.

What it does:

-------------

ELDA is a small program which sends commands to a modem

to request diagnostic information about its last call. The

data returned from the modem is then filtered and either

displayed or written to a logging file.

Although originaly intended to be used with a Wildcat! BBS

using US Robotics Dual Standard modems, it will work with

almost all other BBS and compatible modem types. It can

also be run as a stand-alone program or in conjunction with

dial out communications programs.

Why I wrote it:

---------------

I run a multi-node Wildcat! BBS under OS/2. Sometimes I get

complaints that the system is running slow or the predicted

time for file downloads is inaccurate. I have not been able

to replicate the problem myself and therefore have to assume

that poor telephone lines are responsible. I use U.S.Robotics

modems which have an "ARQ" indicator on their front panels

and I note that sometimes this flickers while a user is

connected. The flicker tells me that an error has been

detected and is being corrected. These errors are hidden from

the user because the modem asks for a block of data to be

repeated and only passes it through its DTE connector when

all the errors have been removed. However, this resending

takes time and gives the user the impression that the BBS

is running slower than it should. All USR modems have an

in-built facility to report details of the last call by

sending the ATI6 command to them. By sending this command

and disecting the reply before the BBS resets the modem it

is possible to see what really did happen during the call.

How to use it:

--------------

The program expects to run at a DOS prompt, the prompt in

an OS/2 DOS session or in a batch file. The syntax is:

ELDA <com port> <baud rate> <irq> <TERM _or_ logging file name>

All parameters must be provided but entering 0 for the baud

rate or IRQ will not change the ports present settings.

If a com port other than 1, 2, 3 or 4 is specified or an IRQ

that is already in use, an error will be reported and the

program will terminate.

The baud rate can be 300, 600, 1200, 2400, 4800, 9600, 19200

38400, 57600 or 115200. If a value of zero is entered, the

divider register in the UART is left untouched and the port

remains	at whatever speed it was previously used at. For BBS use,

leave the value	at zero as the DTE speed should be kept locked

anyway.

Any valid logging file name can be used. You can enter a full

path and name or just the name itself. In no path is given,

the file will be written in the current directory. The file is

opened in 'append' mode so the log can be added to the end of

an existing file. If the file doesn't exist it will be created.

I give the name of the activity log for the Wildcat! nodes it

runs on so the information is written after the call details.

If you find the activity log becomes too large to handle, you

can log to a different file. The date, time and port number

are logged to allow matching against your activity log files.

DOS treats devices and files in the same way, if the log is

written to "CON" it will appear on the screen. Specifying "PRN"

will send it to the parallel printer port. Be very careful if

you send it to a serial device, writing to the com port you

are monitoring will cause a black hole to form and suck you in.

If the word "TERM" is entered instead of a file name, the

program becomes a very simple ASCII terminal. This is to help

when configuring the modem or checking its responses. I do not

recommend using it to make outgoing calls via the modem, it

may work but the risk is yours. You can exit from terminal

mode by pressing the [Esc] key.

Examples: (taken from my own system )

--------------------------------------

QM.BAT   (runs QmodemPro)

@echo off		<- don't want to see all this

c:			<- QmodemPro is on C: drive

cd \qmpro		<- the QmodemPro home directory

qmpro /nologo %1	<- quick start

elda 2 38400 3 con	<- report to screen when finished

POSTCALL.RUN (logs details after a Wildcat 4 BBS call)

elda 1 38400 4 \wildcat\activity.1

(there is a postcall.run in each nodes work directory, some use

non-standard IRQ lines so I specify all the parameters just for

consistency. You could also call it POSTCALL.BAT in the home

directory and use

"elda %wcportid% 0 0 \wildcat\activity.%wcnodeid%"

but you must specify the apropriate environment variables and

paths correctly if you do this)

Modem types:

------------

Modems manufactured by US Robotics and Practical Peripherals

both return connection details on request. Other manufacturers

probably also do the same but each has their own way of quoting

the results. I have provided two sample config files called

USR.CFG and PPI.CFG, you must rename one of these to ELDA.CFG

before running the program. You can have a different modem on

each port but you must take care that the correct .CFG is used

or the log file will be incomplete or incorrect.

The Commands file, ELDA.CMD:

----------------------------

Is simply a file containing the command that make your modem

reveal the last connection details. For USR and PPI modems the

normal diagnostic command is ATI6. On the USR DS V34 ready

model the command to return VFC information is ATI11. If your

modem requires a different command you can change it with a

text editor. Instructions are in the files header section.

The Configuration file, ELDA.CFG:

---------------------------------

This is a text file containing all the parameters that can

be returned by the modem. The file must be in the same

directory as ELDA is executed from. On a Wildcat! system this

will probably be the "wcwork\node*" directory for the node

being logged. By using this location, a different configuration

can be defined for each node.

Use any text editor to modify the the files contents. Any line

starting with a ";" (semi-colon) is ignored, delete the ";" to

make the line active. Only one parameter is allowed on each line.

Note there are lines for "Equalisation" and "Equalization" as both

spellings are used depending on the modems intended country

of use. If new parameters are introduced in later models

they can be added to the config file, there is no limit to the

files length.

The special entry "[VFast]" is used to separate any response

which is applicable to speeds of 21,600 bauds or faster. Lines

following "[VFast]" are ignored unless a fast speed is reported

by the modem.

The text in the config file must exactly match that which the modem

reports. It is case sensitive and spaces between words are important.

What you should get:

--------------------

is something like this-

�---- Connection report from ELDA v2.1  on Sat Aug 06 00:17:11 1994  --Ŀ

� Chars sent               247                                          �

� Chars Received           179828                                       �

� Chars lost               0                                            �

� Octets sent              310                                          �

� Octets Received          146657                                       �

� Blocks sent              103                                          �

� Blocks Received          2044                                         �

� Blocks resent            0                                            �

� Retrains Requested       0                                            �

� Retrains Granted         0                                            �

� Line Reversals           0                                            �

� Blers                    2                                            �

� Link Timeouts            0                                            �

� Link Naks                0                                            �

� Data Compression         V42BIS 2048/32                               �

� Equalisation             Long                                         �

� Equalization             Not Reported                                 �

� Fallback                 Disabled                                     �

� Protocol                 LAPM                                         �

� Speed                    21600                                        �

� Last Call                00:04:52                                     �

� Disconnect Reason        is DISC Received                             �

� Modulation               V.FC                                         �

� Carrier Freq (Hz)        1920/1920                                    �

� Symbol Rate              3200/3200                                    �

� Trellis Code             32S-2D/32S-2D                                �

� Nonlinear Encoding       OFF                                          �

� Precoding                ON                                           �

� Shaping                  OFF                                          �

� Preemphasis (-dB)        7/4                                          �

� Recv/Xmit Level (-dB)    35/18                                        �

� Roundtrip Delay (msec)   18                                           �

�-------------------------------------------------------[Port 2]--------�

This is taken from an actual call and with all the lines in

the config file active. The modem used was a USR Dual Standard

V34 ready model. The call was in V.FC mode, be aware that some

lines are meaningless unless in HST mode.

The report is probably much longer than you would ever need

but editing the config file would of course make it smaller.

The box closes after the "Disconnect Reason" line if V.FC isn't

detected.

Exit codes and error messages:

------------------------------

ELDA displays help whenever it cannot run for some reason.

It also sets the DOS errorlevel to a non-zero value if an

error occurs. Zero errorlevel indicates normal operation.

Likely errors and messages are -

[1] Command line syntax was wrong.

[2] There was an error opening the ELDA.CMD file.

[3] There was an error opening the ELDA.CFG file.

[4] The Log file could not be created or updated.

[5] No "OK" from modem after 30 seconds after Link diag request.

[6] No "OK" from modem after 30 seconds after Fast diag request.

[7] An invalid speed was detected or specified in command line.

Other exit codes refer to internal problems with the com ports

or UART chips, if you see them please report as many details as

possible about your system so I can investigate. Unless you are

using a 16550 UART I suggest you keep speeds down below 19200 or

errors are inevitable, this is not a limitation of ELDA, it is

a processing speed problem with PCs.

Feedback:

---------

I hope you find ELDA meets your requirements. Suggestions for

future versions are always welcomed.

I do not have access to	a PPI modem to try its version of config

file so please let me know if it suffers problems. I am indebted

to Ray Tackett for his help with the PPI responses.

I have tested the program and used it on my own BBS, it seems

to run fine. If you find any bugs I would like to hear about

them as quickly as possible. My BBS and CIS numbers are later

in this text. I also monitor MSI supportnet and the US Robotics

modem net but may take up to 7 days to get a reply back to you.

Registration:

-------------

Try it out and if you think it deserves some of your hard earned

cash, send a small donation toward my BBS running costs.

I suggest $10, 5 UK pounds or its equivalent in your own currency.

I can accept cheques (checks) from UK banks only. For other

currencies, put paper cash in an envelope and mail it to me. Make

sure the contents can't be seen through	the envelope. I will try

to confirm its arrival by Email if you give me a return address.

If you make a donation it will also cover any future releases of

the program.

I consider continual use without registration to be unethical and

dishonest. If I catch you doing so, I will send a gremlin to crash

your hard disk.

Disclaimer:

-----------

(sorry about this but I have to say it)

The program is released without warranty or guarantee of any

kind. The author will not be held liable for damage caused by

this program or as consequence of it being used on any system.

It is released in good faith and is believed to be bug free.

Acknowledgements:

-----------------

The program is written in Borland Turbo C++ version 3.0.

Serial routines are from PCL4C by Marshallsoft Computing Inc.

Future Plans:

-------------

Logging Caller ID (ANI) would be rather neat but my local telco

doesn't offer it so theres no way I could verify it worked.

It would mean another section in the .CMD file to specify the

"AT" command to report the callers number. If enough people

ask for it I'll add code to support it.

Contacting me:

--------------

My BBS details are -

BetWiXt BBS. +44 1633 614765  (from the UK it is 01633 614765)

U.S. Robotics Dual Standard V34/VF.C modems - 2 lines.

My CIS number is -  71141,2757

My mail address is -

Brian E. Kelly.

12 Cotswold Way

Risca

Gwent.  NP1 6QT

United Kingdom.

BBS note:

---------

BetWiXt has been running since 1990. It started in Palatine, IL.

but moved with me to South Wales, UK in 1991.

It concentrates on programming files and utilities and has sections

covering Ham radio and television. It is adopted by the B.A.T.C.

(British Amateur Television Club.) to support their activities.

Limited access on the first call but full access is given within

48 hours subject to confirmation of log-in details.

Release History:

-------------------

V1.00	Released as USRATI6. Support for US Robotics modems only.

( 3 July 1994 )

V2.00	Rewritten and released as Enhanced Line Diagnostic Analyser.

Added support for 115200 baud rate and non-standard IRQs.

V.FC / V34 functions added.

Support for PPI modems added.

Improved error trapping and reporting.

Faster operation at high DTE speeds and timeout bug fixed

at 300 baud. ( 29 July 1994 )

V2.10	Timeout increased slightly to allow 300 baud DCE to send whole

report before giving up hope. This was a bug in V2.00 which I

don't believe anyone spotted, maybe nobody uses 300 baud any

more!

Sends an extra "AT <CR>" before requesting link information.

Later versions of USR firmware seem to need this to allow the

modem to determine DCE speed if DTR was dropped to cut the last

caller off. The problem did not exhibit itself on earlier USR

code but on recent updates the modem and PC could try talking

at different speeds causing a timeout after 30 seconds.