SPECIAL INFORMATION FOR USERS OF OS456 OPTOELECTRONICS

SCANCAT's interface is optimized for basically 386 machines.

If you have a slower or faster machine you made need to make

a one time adjustment. This timing adjustment controls the

squelch and signla strenght readout from the PRO-2005/6.

To be sure everything is working properly, we would recommend

checking to be sure the "squlech" detection is properly set.

Simply choose any of the scanning functions at the "A" menu

(1,2, or 5) and after the program starts scanning, open the

radio's squelch knob by turning it completely counter clockwise

until you hear audio, and see the "signal strength" indicator

on the program's radio graphic.

IF YOU GET A SIGNAL STRENGTH INDICATION, then no further

adjustment should be necessary.

IF NO INDICATION is displayed press either the "[" of "]"

keys, which are located to the right of the "P" key and above

the "ENTER" key. These keys will let you adjust the "timing"

necessary for the computer to read the squelch status coming

from the radio, and optimize the speed in which it is decoded.

Pressing the SHIFT and these two keys will speed up the timing

set process. ONCE the SIGNAL STRENGTH IS INDICATED, no further

adjustment will be necessary.

You can optimize the scanning speed at any time by using this

simple process.

INDEX OF TOPICS

INTRODUCTION .........................X

GETTING STARTED ......................X

SETTING UP SCANCAT ...................X

SCANNING AND CONTROLLING THE RADIO....X

SINGLE FREQUENCY INPUT ...............X

SCAN BETWEEN TWO FREQUENCIES .........X

LOGGING ..............................X

DISK FILE SCANNING ...................X

DISK FILE SETUPS/EDITING/UTILITIES ...X

SPECTRUM ANALYSIS ....................X

FREQUENCY BYPASS/SPECIAL OPTIONS .....X

DISK FILE UTILITIES ..................X

QUIKTERM COMMUNICATIONS PROGRAM ......X

TUTORIAL .............................X

IF YOU HAVE PROBLEMS .................X

SPECIAL THANKS

and

Appreciation to:

. J.D. Alexander

. Guy Atkins

. Art Heely

. Jerry Henley

. Jim Marciniak

. Buddy Marshall

. Tim Schira

. Keith Thomas

. Ed Lentz

Who helped us to develop, BETA test and even LOAN us their radios in

some cases, in order to help us produce this program.  Our heart felt

thanks goes out to you, guys. You saved us countless hours (and

dollars), and made us lots of new  friends.

. Introduction

Thanks for purchasing SCANCAT DEMO. We are sure that you will be more

and more impressed with the versatility of our program the longer that

you use it.  Another Ham (NS5Z) and myself (N5JMZ) co-authored this

computer program. We originally wrote it to control a FRG-9600 Yaesu,

but it has now evolved into the multi- radio supported SCANCAT that

you are using.

One of SCANCAT's strong points is that this ONE PROGRAM will

control ANY of the below listed radios. In addition, any file created

by one radio can be used by another, as long as the modes are

compatible.  Sca ncat even "translates" the unavailable modes not

recognized to a mode compatible with the radio in use.

SCANCAT presently supports the following radios:

AOR-3000 Communications Receiver

M ost ICOM including R-71 and R-7000

Japan Radio NRD-525, NRD-535

Most Kenwoods including TS-440, R-5000, TS-950

Yaesu FT-757GX, FT-757GXII, FT-767GXII, FRG-8800

Drake R-8

AOR-2500

SCANCAT allows complete control of all radio functions that are

addressed to the radio's microprocessor. This enables you to perform

many functions that are just not available by manually controlling the

radio.  PLUS, with most Icom radios that have a remote control

recorder jack, SCANCAT supports full squelch detect and stop-o-signal

through an interface cable using the joystick port of your computer.

SCANCAT allows you to:

1. Enter any one frequency and increment up and down.

2. Enter any two frequencies and scan between them with

. ANY increment and VARIABLE time delay and pause.

3. Scan a file of frequencies, search the file by a description or

*wildcards*.

4. Create frequency files and save to disk and/or send to the printer.

5. Create 30 personal scanning frequency BANDS (presets) for Short

.  Wave, aircraft etc., including increment and mode. ( Mode selection

is not supported on the FT-757.) Ten are included in a supplied

file.  6. You can read from the Kenwood, Icom, AOR-3000, NRD-

525/535 and R8 radios all of their memory channels including VFO

splits and modes and save them to a SCANCAT disk file that can

later be retrieved and re-loaded into the radio.

7. You can fully autoload all of these radios memories from any

SCANCAT disk file including mode and VFO split, if appropriate, in

less than 90 seconds. ( Great for mobile trips.)

8. AOR-3000 radios have memory upload and download similar to the

.  KENWOOD and ICOMS but can also save to the disk file a relative

signal strength and spectrum analysis.

9. Run a SPECTRUM ANALYSIS of your frequency searches, and

displays a graphic representation of your analysis.

SCANCAT includes ON-LINE HELP and implements KEY-STROKE accessed

POP-UP windows, making SCANCAT extremely user friendly. SCANCAT has

been extensively "Beta-Tested" in our effort to create an easy-to-use

and bug-free program.

PLUS:

SCANCAT users can create unlimited SCANCAT disk files with 400 frequencies

in each file! The frequency records can be SORTED by ANY field including

DESCRIPTION, FREQUENCY, ETC.  While you are scanning between two frequencies

you can access several PRE- PROGRAMMED "SPLIT-OFFSETS" such as Marine

Telephone. The user only has to press a key to log or add any frequency to the

current disk file complete with optional description and comments. In under one

second SCANCAT can check for description matches or frequency duplicates.

(Great for Field Day or contest  logging.)

Computer Aided Technologies can also supply files for immediate use including

POP-COMM's 278 English Short Wave broadcast frequencies, Maritime, and

the TOP 200 Utility Frequencies. For the Kenwood, Icom, NRD-525/535,

AOR-3000 and R8 radios there is a loader for their memories with

"splits" for the Kenwood Split VFO's.  Check on the GENIE, The General

Electric News and Information Service, Radio & Electronics BBS for

free downloads.

Lastly, SCANCAT has a built-in, integrated communications program

enabling the user to access a TNC (terminal node controller) or a

digital data decoder.  It features dual windows and a split screen for

SENDING and RECEIVING . The logging of frequencies from the terminal

is supported.

.     SYSTEM REQUIREMENTS

The required hardware for SCANCAT is an IBM/PC compatible computer

with at least 512K of RAM, 1 disk drive, and a minimum of 1 serial

port. Color screens are supported but YOU DO NOT NEED A COLOR MONITOR

OR GRAPHICS CARD.  If you are using the joystick interface cable for

Icom radios, you must have a 15-pin dual in-line game port output.

Computer Aided Tech. does not supply the radio interface (the small

box that connects between the radio and the computer), but they are

available from most Ham Radio suppliers for around $90.00.  The AOR-

3000 radio does not require an external interface.

@PAGE BREAK =

. Getting Started

O.K,  let's get started. The first thing that you should do is to

copy your master diskette onto either another floppy or your hard

drive. SCANCAT is not copy protected so the simplest thing to do is:

A. Floppy System:

1. Place your master diskette in Drive A

2. Place a blank disk in Drive B

3. At the "DOS prompt type:  "Diskcopy A: B: and press  "ENTER"

B. Hard Disk System:

1. Place your master diskette in Drive A (or B)

2. At the "C" prompt, type

(If source is floppy A:) "INSTALLA"

(If source is floppy B:: "INSTALLB"

All your files will be copied to your Hard Drive in the directory

SCANCAT, and a batch file named "SCANCAT.bat" will be placed in your

root directory.

To  run SCANCAT simply type "SCANCAT" and press "ENTER". You will be

greeted with the SCANCAT logo.  After pressing any key the MAIN MENU

will appear.  (See Figure 1)

. Main Menu

At the Main Menu you have the following choices:

( R ) Set for (radio selected):

. This pops up a window that lets you choose between the different

radios supported by SCANCAT. See the introduction for the list or

simply press the letter for your radio.  The last choice on the RADIO

pop-up menu permits the  manual selection of an ICOM radio that might

not be on the      supported list.  The radio's address MUST BE

ENTERED IN      DECIMAL.  Refer to your owners manual for the proper

numerical address.

( 1 ) Input Single Frequency:

. This allows input of a single frequency and immediately changes the

radio's dial to that frequency and  mode.

( 2 ) Scan between frequencies:

. This allows the input of two frequencies between which the radio

will scan.

( 3 ) Disk File Scan Setups:

. This selects the SCANCAT file utility menu from which the user can

select a file, edit a file or create a new file of frequencies.  Also

there are utilities for loading and reading the radio's memory

channels, and for creating      automated loader files for just about

any frequency range. You can print out the database files to a printer

or disk file.

( 5 ) Scan Diskfile:

. This begins the scanning of the disk file selected by option (3) and

also will tell you how many records are contained in that particular

file.

( 6 ) Set Time Zone/Date and Internal Computer Clock:

. Enables the user to set the internal computer clock and select the

time zone in which you live for proper setting of the SCANCAT UTC

clock.

( 7 ) Interface Comm. Parameters:

. Default parameters are COM1:4800,N,8,2 for most radios.  For Icom

and Japan Radio, COM1:1200,N,8,2 is common.  This selection is a two-

step process.

. 1. This shows the present configuration of the parameters for

addressing          the interface  between the radio and the computer.

. 2.  It next prompts for setting the Quick Term Communications Port

parameters.

( C ) Quick Term Communication Program:

. This is an integrated terminal program for operation of the

frequency and scanning features of SCANCAT "simultaneously"  with a

TNC or digital data decoder. The simultaneous feature actually

switches between the control of the radio  and a TNC. This feature

requires that the computer system have two serial ports.

( ALT-C ) Color Setups:

. This allows the user to select the colors most suitable to your

system.  Press "ALT-C" and follow the menu.

( E ) Exit:

. This is the only recommended exit from the program.  You are always

given a last chance in case of a mis-key, as the user is asked, "Are

you sure?". Each time you exit, SCANCAT saves the changes to a current

disk file, if you wish, and then the working parameters such as the

radio in use, the disk file name currently in use, etc.  Be patient

while the program closes its files if you are using a floppy  system.

.  SETTING UP SCANCAT

. Setting the SCANCAT Clock

If you are starting SCANCAT for the first time, you should set

the UTC clock function.  Press ( 6 ) and then enter the information at

the prompts that is appropriate for the date and time, plus the letter

for the time zone in which you live.

. Setting Colors

If you wish to change the SCANCAT "colors", the next thing to do

is set up the colors that are used for each screen. Press  ( ALT-C

) and a window will pop up. Pressing 1-2-3-4-5-6-8 changes the colors.

The final prompt will enable you to save the existing color setups.

. Choosing Your Radio

Now you are ready to use SCANCAT. First press ( R ) to select the

radio that you will be using.  A pop-up window will show all the

supported radios. By pressing the appropriate key, the radio is

selected.

. SCANNING AND CONTROLLING THE RADIO

The instructions below are in the same order as the menu

selections from the Main Menu.

. Entering A Single Frequency

Press (1) from the main menu.

.    This selects "Single Frequency" input.  Type in a single

frequency that is within the range of the receiver you are using. An

input beyond the radio's range may not affect the program but the

radio will not accept the information that is out of range, and the

program may not respond.

. A. Pressing the (UP/DOWN ARROWS) will increment the original

frequency by the increment you have selected.

.

. B. Look at the menu bar at the bottom of the screen. Each command's

first letter is highlighted for the  function are available at this

screen.

.

. C. The logging features are available while operating the single

frequency option.  Please refer to the section on logging for full

details.

.

. D. If you should want to try some other frequency, simply press

(ENTER) and the prompts will appear again for frequency, mode and

increment.

. SCANNING  AND  CONTROLLING THE RADIO

. Scan Between Two Frequencies

Pressing  ( 2 ) at the main menu allows the  user to set up

SCANCAT for scanning between two frequencies.  Once the radio graphic

appears, the pop-up prompt is seen that says:

.  "Press "P" for presets or enter lower frequency"

Entering a "P", instead of a frequency, pulls up the menu for the

PRESET frequencies then pressing any key that contains a description

automatically starts the scanning process at the preset frequency

range, mode and increment.

OR:

You can enter the lower, then the upper frequency, along with the Mode

and Increment, at which point SCANCAT starts the receiver scanning.

(See

Figure 2)

. Controlling Scan Rate

( Left-Arrow  )

. �  Slows the DELAY betweenchanges.

( Right-Arrow )

. � Speeds up the DELAY  between changes .

( SPACE  BAR ) or ( INS )

. Toggles  between PAUSE  of scanning and RESUME of scanning.

. Pressing (ENTER) also resumes scanning.

( ENTER     )

. Resumes scanning if in pause.

. COMMAND KEYS  Available from the Radio Screen

( UP-ARROW  ) IF IN PAUSE

. Moves frequency up by the amount of               the increment.

( DWN-ARROW ) IF IN PAUSE

. Moves the frequency down by the               amount of the

increment.

( C )  Communications

. Immediate entry  into the Quickterm Module while scanning.

( D ) Program  DUPLEX OFFSET

. Used for listening to Marine transmissions.  There are many default

offsets for these marine bands that come with SCANCAT.

( F )  - Functions (Special functions)

( L ) - LOGGING

. Turns the "LOG" on and selects several options, as described in the

logging section.

( M ) - MANUAL LOGING

. Manually Logs active frequency to current memory file.

( R )   AUTOREAD

(Kenwood and Icom only). Enables the radio's dial to be changed

manually by the operator and the changes will be reflected immediately

on the computer's screen.  This function works only while in PAUSE.

( T ) - TWEAK

. Enables the fine-tuning of frequencies that are already in a disk

file.

( W ) - HUGE NUMBERS�

. Toggles (ON/OFF) - Enables a "HUGE NUMBERS" window

( O )       "FLIPS" active frequency to the LOWER duplex offset.

( - )         "FLIPS" active frequency to the LOWER duplex offset.

( + )      "FLIPS" active frequency to the  HIGHER duplex offset.

( F-1 )       Brings up the HELP SCREEN.

( F-2 )     Pops up the MODE "Help" screen.

( F-3 ) - COMMENTS

. Toggles the display of the COMMENTS on and off. (ENTER ) Starts over

the input for a new  frequency .

( ESC )       Universal exit to the previous menu.

. LOGGING

The logging functions are available while scanning from any area

EXCEPT when you press F-9 or F-10 to access the direct access

highlight bar "pop-up window" for viewing and editing.

Pressing the (L) pops up a window with options for

creating/logging your active frequencies to a memory file for saving

to a disk.

. LOGGING OPTIONS:

0. Turns off any logging that has been set.

1. Turns on logging.

2. Logging with Manual inputs.

3. Auto logging - Dupe frequency checks.

4. Manual logging- duplicate description prompts.

5. Manual logging- duplicate frequency prompts.

IF YOU DO NOT HAVE A RADIO THAT ENABLES SIGNAL              DETECTION,

pressing (M) while at a frequency will log it to the disk memory file,

as if the radio DETECTED the signal.

. SCANNING AND CONTROLLING THE RADIO

. SCAN A DISK FILE

If there is no file in memory, or, if you want to change to

another file, press ( 3 ) at the main menu.  Then at the "Disk File

Utilities" menu press  a ( 1 ) to select the file from the displayed

directory.  For practice select the file "TOP20.FRQ". After it is

retrieved, the Secondary menu will then reappear. Press ( 5 ) which

takes you to the scanning display and begins the scanning.

�There are also several command keys available in addition to the keys

used in the "Scanning between two frequencies " section.

. Command keys available while scanning a disk file

( E )  Edit -

( I )  Increment

( S )  Search

Most of the COMMAND KEY functions from the previous scanning modes

also function in the disk file scanning mode.

. SPECTRUM ANALYSIS

This function is only available if your radio supports signal

detection or you are using an ICOM/JOYSTICK INTERFACE CABLE. As of

this writing, the AOR-2500, AOR-3000, NRD-535, R8, and Icoms with

remote recorder jack are supported. There are two totally  independent

Spectrum Analysis functions: One scanning between two frequencies, and

the other for using the logging function in a disk file.

. WHILE SCANNING BETWEEN TWO FREQUENCIES

Spectrum analysis with SCANCAT system a graphic chart of the

signal strength or number of "Hits" versus the frequency spectrum

under analysis.  The frequency spectrum is determined at the time you

select the two frequencies to scan between.  While scanning between

two frequencies, press ( A ) for analysis. The lower half of the

screen will be covered with a pop-up window that graphs the signal

strength on the vertical part of the graph, and the frequency on the

lower or horizontal part of the graph.

. The commands that control the Spectrum Graph are:

( A ) Toggles on/off the spectrum display

( CTRL-Right arrow )  and  ( CTRL-Left arrow  )

( CTRL-S ) or ( CTRL-H )

( / ) - PRINTER

.  **  NOTE **

A. Each time you change scanning parameters or change scanning

frequencies, a pop-up message  prompts you to erase the existing

spectrum analysis.

B. All normal operations or scanning functions may take place

independent or this function. However due to programming constraints,

the "HUGE" numbers window is disabled during display of the spectrum

analy sis window.

. SPECTRUM  ANALYSIS  WHILE VIEWING/EDITING DISK FILE RECORDS

The spectrum analysis takes on a different picture when using the

logging functions of the AOR-3000, NRD-535, R8 and Kenwood 450/950 you

log hits while scanning the disk file, the strength and number of

accumulated hits are recorded for that particular diskfile frequency

in the diskfile memory file.

Place the highlight bar over a record that shows a number of hits, and

press the period ("."). The spectrum graph will pop up and the display

will detail the strength vertically for each of the first 40 hits.

. FREQUENCY BYPASS / SPECIAL SCANNING OPTIONS

. FREQUENCY BYPASS

If you are using a radio that supports signal detection, you

will undoubtedly encounter "birdies". These are carriers that may be

generated internally by the radio or your computer. They can also be

full-time carrier broadcast stations. If this option is chosen,

SCANCAT will search for a bypass frequency each time it changes to

another frequency. If it is present in the bypass table, SCANCAT will

automatically advance to the next increment or frequency.

. SPECIAL OPTIONS  POP-UP MENU

The Special Option menu is accessed by pressing ( F ). The SPECIAL

FEATURES are the last several items in the Frequency Bypass Window.

Most of these options are only usable if the radio you are using

supports signal detection through SCANCAT.

. SPECIAL FEATURES COMMAND KEYS

(  T  )   Priority sample rate

. Input a value from 1 second to 99 seconds.

( U ) Continuous Scanning

. Permits the scanning to resume after a predetermined time

( W ) Time stamp ON/OFF. Toggles logging time stamp to diskfile.

( Y ) Squelch delay

. Enables the scanning to delay for a predetermined time AFTER the

signal is lost.

( Z ) Joystick Squelch Detect ON/OFF.

. This is a "must" feature if  you have several radios hooked up to

SCANCAT at the same time.  SCANCAT has no way of knowing that the

joystick is sensing a true signal or one from a radio that may be

hooked up but not selected. This will solve this conflict.

. DISK FILE UTILITIES

The ultimate  advantage of a computer scanning system is the

ability to store large amounts of information, then selectively

retrieve that information.  One of SCANCAT'S premier attributes is its

ability to store and retrieve many types of disk files. In the

Appendix, there is a listing of spectrum usage from 500 Khz. to 1270

Mhz. as an aide in setting up disk files. We recommend you create many

small files as opposed to one B-I-I-I-G-G file, dividing your files

into categories, such as:

Short Wave Broadcast ( 5 Khz. Spacing )

Public Safety

RTTY (even into different modes)

Marine

Aviation

Other Utility

Cellular

Files, for the most part, are COMPATIBLE with any other radio you

might own. For obvious reasons, a VHF frequency will not have any

effect on a HF-only radio, and vice versa. Some radios have exotic

modes that are not supported on another radio. We have made every

attempt to reconcile these differences with "software filters" that

look for modes not supported and change them to one that is.

. 1.  Create / Read Disk Files

This selection brings up the directory of the path or disk you are in.

At the bottom of the screen you will also notice the options:

1. CD\Path - Change the path on your hard disk to another directory or

to another disk.

2. KILL - Delete a file - use the full filename and extension.

3. RENAME - Enables you to rename a file - use the full filename and

extension.

4. DIR - followed by a dos convention such as "*.frq" or "*.*" will

let you see a directory of the path you are in.

Entering a filename in the prompt box will select a new file. If the

filename is not in existence, the filename will create a new file by

that name when the new records are saved.

. 2. Preset Scan band / Edit file

The preset frequencies are actually TWO frequencies that you can

program the computer to scan between when in the "Scan between two

frequencies" section.

.

. 3. Select/Edit "Filename"

The following keystrokes are in alphabetical order and may be

performed by pressing:

( A )   ADD A RECORD

( C ) COMMENT

( D )      DELETE

( E )        EDIT

( F )Turns ALL records OFF

( I ) INSERT

O )  Turns ALL records ON

( P )  PRINT

( S ) SORT

( T ) Bandwidth/AGC or Time stamp

( W ) Wildtag

( ARROW ) - Scrolls the display of records up and down by lines.

(PGUP/DN) - Scrolls the display  upand down by pages.

( HOME )  - Go to top of the file.

( END )  -  Go to end of the file.

( ENTER )

. Selects/deselects (Turns ON/OFF) records to be scanned.  Press enter

when the highlight bar is over the record to be selected.  The bar

then moves to the next lower record.

4. Create file of incremented frequencies

. M. Merge file with existing file.

Simply loads a newer file with the file already in memory and

combines the two, up to a maximum of 400 records total per file. The

filename changes to the last file read in from the disk.

. R. Read radio's memory to file.

Enables you to store the radio's memory contents in a disk file

for later retrieval. The disk file that is in memory when you choose

this option will be erased. Therefore, when you first access this

function, the disk file save prompt will pop up, giving you an

opportunity to save the existing disk file before it's erasure. The

prompts are very simple, and except for the AOR-3000, there is only

one bank of memories. On the AOR-3000, pressing the (SPACE BAR)

repeatedly will change the bank to the one you want to load. The only

indication you have of which bank is being accessed is by viewing the

red light on the panel of the AOR-3000.  A pop-up status window at the

lower part of the screen will give you a visual progress of the

reading process.

. L. Load radio with "Filename"

This will load the radio with the FIRST RECORDS contained in the

existing disk file. If the radio has for instance 100 memories, then

the records 0-99 will be loaded into memories 0-99 of the radio. If

the disk file contains more than the limit of memories in the radio,

then only that number will be loaded.

. S. Load records selectively

This gives you the option to load only CERTAIN records into

the radio's memory. ONLY RECORDS THAT SHOW "ON" will be loaded.

. Communications Module

SCANCAT has a built-in communications terminal program that will

allow the user to access a TNC  (terminal node controller) or a

digital data decoder such as the PK-232, MFJ-1278 or the Universal M-

7000.  The user with two serial ports can configure the terminal

program to one serial port and SCANCAT operations to the other. The

user can then very rapidly change back and forth between pressing "C"

either while at the MAIN MENU or from any scanning function.  You are

presented with a split-screen terminal window at the bottom is a small

input window.

The top window is for data received from the controller and the bottom

screen echoes data typed from the keyboard.

The user has several options from this screen. They are:

( INS   ) -

. Toggles the keyboard input between immediate and "buffered".

( Alt-C ) - Clears the screen

. In case of a disruption (usually caused by receiving unprintable

control characters from the terminal.)

( Alt L ) - Manual Logging Function.

.  While receiving data from the controller the user  can log an entry

to the active SCANCAT file.  After pressing (Alt L) the user will be

asked to:

( Alt P ) - Toggles the printer on and off.

( Alt Q ) - Quit and back to the  main menu.

( Alt-S ) - Spooling

. Send the TNC's output to a disk file of the received data.  A

secondary prompt will be asked for the Disk  filename.

( Alt-V ) - Lets you view a file

. (From a diskfile) on the communications screen. This could, for

instance, be a spooled file from a previous run.

(  ESC  ) - Exits to the main menu.

. Operation with Universal M-7000

The user who has a Universal M-7000 is indeed fortunate to have

both  very fine digitial data decoding equipment and SCANCAT's

QuickTerm module to overcome the two biggest deficiencies of the

unit: The M-7000 will not save received data to a file and the system

printer can't be used easily with the M-7000 without an A/B printer

switch.

. Brief Tutorial

. A session using the TOP20.FRQ file as an example.

From the main menu select the disk file utilities menu. When you

are at the disk file menu, press a ( 1 ) and the screen will clear and

the file names on the disk/path will appear.  You will notice at the

bottom the prompts to either KILL or RENAME a file. These are handy

options, and use the same convention as the DOS commands "DEL"

and "RENAME".  Be sure you use the full file name including the

extension.

("RENAME   OLDFILE.EXT   NEWFILE.EXT")

("KILL FILENAME.EXT"             )

Select a file OR if you wish to change DRIVES or PATHS, type in

"CD\" and the desired device/path .

A. EXAMPLE   "CD\SCANCAT" -

Would change the PATH to "SCANCAT" on whichever

drive is active.

B. EXAMPLE   "CD\B:"  )  -

Would change the  DRIVE to  "B:" and would

override the \SCANCAT Path

C. EXAMPLE   "CD C:\SCANCAT" -

Would change the DRIVE to "C:" AND the  Path to

Scancat on the C: drive all in one operation.

Once the file is chosen, you will be placed at the secondary

menu, Disk file Utilities.

1. Press  ( ESC )  and return to the main menu.

2. Press ( 5 ) to go to the  scanner screen.

. The program will automatically start scanning at the default rate of

about .05 seconds delay per record. Press the left or right  ARROWS

several times to get the feel of slowing or speeding up the scanning

process.  (If you have a radio that       supports scan stop on

signal, the scanning may stop if a signal is detected.  Press the

(SPACE-BAR) and the scanning will pause.  The up-down ARROW keys now

will move you one  record forward or backward at each key press.

Press the  SPACE BAR once again and the scanning will resume.

3. Press the  ( S ) for search

. A prompt will pop up and ask you for a search criteria to look for

in the DESCRIPTION field. TYPE IN "VOA".  ANY part of an input will

look at the record's description and if found anywhere in that field

will scan that frequency. If you are in PAUSE, the first record you

come to will again pause until another key is pressed.  In this case

the VOA frequencies  will appear one after another.  Try the search

with "MARINE" or "SHIP" or "AIR".

. 4.  Press the  ( L ) for logging.

.  AS you press it, several  different options will appear.

. 0 - Logging is OFF

. 1 - Auto-Logging ON - No dupe checks without a prompt for input,.

. 2 - Logging with MANUAL input.

. 3 - Auto Logging- Duplicate freq. checks.

. This function will log signal detection unattended.  You can go off

for hours and come back and find all the signals that were detected

and how many times. You even have a log of the last time the signal

was detected.

. The next two are the same as manual EXCEPT they will search the

DESCRIPTION or FREQUENCY field for duplicates and let  you decide if a

duplicate is found if you wish to continue or abort the logging..

. 4 - Manual Logging - Dupe description checks.

. 5 - Manual Logging - Dupe frequency checks.

. This is an excellent way to look for call signs in a field day

exercise, or if scanning utility bands, to keep from logging the same

frequency over and over again.

. 5.  PRESS an "O" for offset.

.  You will notice the frequency changed lower by a default offset.

These presets are present for all marine and 10

mete r amateur bands, so you can hear both sides of a Marine radio-

telephone linkup.  The  "+" and  "-" signs  will also change the

offset either upward or downward.  If another offset is desired, press

<<D>>  and a duplex  prompt will let you type in any other offset.

. HINTS FOR YOUR SCANNING ENJOYMENT

1. We have attempted to put every command key in the HELP SCREENS

(Help screens not available on DEMO COPY)

that is possible. Pull these screens down whenever you need

information or are looking for a new function. Also the abbreviated

menus at the bottom highlight the most commonly used keys.  There are

THREE sub-menus at the bottom of the screen that appear whenever you

press the ALT, CTRL or RIGHT SHIFT keys and remain until these keys

are released.

2. If you are using disk files, and have a signal-detecting radio,

first turn OFF all the records using the global "F" key while  in the

disk-file utilities menu. Then go to the scanning menu. Since ALL

records are off, only the first one will appear in the scanning

window.  (not scanning). Press (S)earch and type in an "*" (wildcard

for ALL). Turn on the logging option #3.  Now every time a signal is

detected, that frequency will be turned back ON, for scanning, in the

database. After a few hours running, the only records turned on will

be the ACTIVE ones. Save this file for the fastest access of ACTIVE

channels in your area.  (Cellular for example).

3. When you create your disk files be sure and save the increment that

applies for the frequency spectrum you are listening to. Then if you

come across an adjacent channel while scanning, the increment can be

used immediately by pressing the PAGE UP/DOWN keys to move off the

channel.

4. Certain features may slow down the scanning process. The frequency

bypass option must search each of the frequencies before going on to

the next scanned item.  To increase scanning speed disable this

function when not necessary. You may also like the "HUGE" numbers, but

they really slow things down.

Some radios scan faster in the lower frequencies than they do the VHF-

UHF spectrum (AOR-3000). The scan rate is also faster if you are not

mixing wide ranges of frequencies. Keep all of one spectrum in one

file.

ON MOST COMPUTERS BUT NOT ALL, PRESSING THE CONTROL - ALT - AND THE

RIGHT-SHIFT KEY bring up alternate menus. These, by themselves, do not

change a function, but merely serve as a help-aid.

The rest we leave to you and your experimentation. Happy scanning.  If

there are features you would like to see incorporated,  please drop us

a line. If your idea is used in a future update, we will send you a

free upgrade.

. Thanks again for choosing.....

. S C A N C A T

. P R O B L E M S

.

. WE CANNOT EMPHASIZE THIS ENOUGH.  90% OF ALL CUSTOMER RELATED

PROBLEMS  PERTAIN TO INCORRECT SETTINGS OR HOOKUPS  OF THE SERIAL

PORT.

PROBLEM : COMPUTER SCANS BUT RADIO DOES NOTHING

SOLUTION: Check your "COMM" settings.  (Baud  & etc).  Also check the

RADIO setting.  Be  sure it is the correct radio.  RESELECT the radio

you are using from the Main menu. Also the Radio you were using last

is saved when you exit SCANCAT.  This may not be the radio you are now

using.

For instance: Each time you EXIT, the COMM PORT setting is saved as

well as the RADIO used during that session.

. "Most" factory interfaces use  "COM1:4800,N,8,1"

PROBLEM : Sluggish Scanning on the R-7000

SOLUTION 1: Make sure that VSC (Variable Scan Rate) is not activated

by depressing the front panel button.

PROBLEM : Sluggish scanning on the AOR-2500

NO SOLUTION : The Aor-2500 manual states that a 1/15 second delay MUST

be placed between EACH  ChARACTER sent to the radio. Most commands

require 4 or more characters. The frequency requires 8.

PROBLEM : ERROR MESSAGES - DEVICE NOT READY

SOLUTION 1: Comm settings could be wrong, or  set for wrong port

n umber, baud, parity etc.

SOLUTION 2: Make sure radio interface is  plugged in & turned on.

PROBLEM : TNC WON'T RESPOND. SCREEN STAYS BLANK in comm program.

SOLUTION 1: Be sure the TNC/computer cable is  plugged in and the TNC

is on.

SOLUTION 2: Check Comm settings from the Main menu.  Make sure

por t # is correct  as well as baud and other settings.

SOLUTION 3: Go to main menu. Turn off the TNC. Reenter the

communi cations program and press either "*" or the key several times

to re-initialize the TNC.  DEFAULT/ usual settings are: COM1:4800,n,8,

1

. ===========================

. If  an error persists & the error message box pops up ---

. ===========================

1. PRESS "?" AT THE ERROR MESSAGE AND WRITE  DOWN THE NUMBERS THAT

APPEAR IN PLACE OF THE MESSAGE.  DROP US A LINE - or call our Order

Desk. OR IF YOU WISH, SEND THE ORIGINAL DISK AND WE WILL SEND YOU

ANOTHER DISK WITH THE "bug" FIXED.  N/C PPD.

Most "BUGS" are really differences of opinion on what the programmer

had in mind for a function and what the user expects the program to

do. True bugs are things that shouldn't happen and prevent the program

from running, hang or crash the system. We never make mistakes, ....

well, hardly ever.  If you encounter an error, try several times, and

before faulting the program, REBOOT, without TSR's present, and see if

it persists. TURN OFF your radio's power and turn back on.

SCANCAT IS A COMPLEX PROGRAM.  Many different radios are supported

from within this one program.  We have attempted to make the files as

compatible as possible for all radios, however there are obviously

modes and frequency ranges that are not compatible or present with all

radios.  Exceeding these compatibilities could cause your computer to

"hang" or do strange things to the radio being addressed.  The

incorrect baud rates  can send messages to some radios that can cause

re-boot or turn off.

IF YOU HAVE AN 8087 CHIP INSTALLED and if your system crashes before

even getting up, reboot, and from the DOS prompt, type

. "set 87=off".

This will turn off the math chip and let the program run without

crashing.

==================================================================

We are presently using CATEGORY 15 TOPIC 9 on the GENIE bulletin

board available in most major cities. There are SCANCAT utilities and

SCANCAT frequency files on this BBS free for the cost of a download.

We welcome you to visit and chat.

Jim Springer

Computer Aided Technologies

P.O. Box 18292.

Shreveport, LA 71138

Order Desk Phone 318-636-1234

FAX 318-686-0449