========================================================

FLASHLED.COM - FlashLED v1.01

"Disc Drive Activity at Your Finger Tips"

(C)opyright 1991 - Seagate Technology, Inc.

The first name in disc drives.

written by R. Dimick and B. Rudock

Seagate Tech Support BBS (408)438-8771

========================================================

=====Intro==================

FlashLED is a very small TSR (Terminate and Stay Resident in memory)

utility that uses the LED's (Light Emitting Diode) found on AT

keyboards and, optionally, the system speaker to indicate disc drive

activity.

Normally, the activity lights found on the front faceplates of the

disc drive and/or the system chasis are more than adequate for

displaying activity, however, many of today's smaller drives are

mounted deep within the systems, obscuring any view of the drive's own

LED (if it even has one). Also, many computer systems are of the

"tower" type, taking them off the desk, which requires the user to

look down at the floor for activity lights. In either case, FlashLED,

can help by putting the activity light "at your finger tips."

=====Syntax=Summary========

The command line syntax of FlashLED is:

FLASHLED [/options]

The default behavior, if no command line arguments are specified, is

to prompt the user for manual selection of the options.  This manual

mode, since it requires user input, is not really appropriate for an

AUTOEXEC.BAT. Therefore, if you would like the program to load

automatically, via the AUTOEXEC.BAT, it is recommeded that you supply

the [/options].

| The option switches are:

|

|    Select ONE of the keyboard LED options (or not) :

|       /N   Num Lock

|       /C   Caps Lock

|       /S   Scroll Lock (default)

|       /K   Kill keyboard LED activity (FlashLED will not

|             indicate activity on the keyboard, so, hopefully,

|             you will be selecting the speaker option [/P])

|

|    Additionally, select ONE of the disc drive options:

|       /F   Floppy drives only

|       /H   Hard drives only (default)

|       /B   Both floppy and hard disc drives

|

|    Optionally select one of these, if you like:

|       /P   sPeaker "click" with drive activity (low

|             frequency is READ, high frequency is WRITE)

|       /?   Display all of these options to the screen,

|             without loading the program. (overrides any

|             other command line arguments)

Multiple command line argument switches may be specified together or

seperately. The following examples are functionally equivalent:

FLASHLED /S /H /P

flashled /shp  (DOS nor FlashLED is case sensitive)

(this space ^ must be there)

=====Outline================

An brief outline of how FlashLED works is as follows:

A. FlashLED Disc Drive Activity from Keyboard LED program

1. Display program instructions and take user input of

configuration parameters

a) determine the current status of the keyboard LED's and save

that status then turn off all the LED's

b) wait for the user's selection of which keyboard LED should

indicate the activity (Num, Caps or Scroll Lock)

c) If it's not an LED that was ALREADY in use before the

program was started, then great, otherwise default to the

Scroll Lock LED.

d) wait for the user's selection of which drive devices (hard,

floppy or both) should reflect activity to the LED

e) wait for user's selection on activity "clicking" the speaker

2. terminate and stay resident

3. program "watches" for activity on DOS interrupt 13 (disk

services)

a) if "observed", then LED and/or speaker is turned on

b) when INT 13 concludes, LED is turned off after a few DOS

INT 1C timer ticks (18.2 ticks per second)

4. program removed from memory by a system reset or TSR

management utility

=====App=Notes==============

Some Application Notes:

1. The effect of the program on the performance of the system is

negligible.

2. When the speaker "click" sound option is activated, other

programs which beep on errors will usually get their chance but then

the beep frequency is lowered again to the level defined by FlashLED.

3. The use of FlashLED on XT's is disallowed, since keyboard LED's

are not defined in that host environment.

4. In Windows (tm), the keyboard LED activity is suspended but the

speaker click still works. When Windows is exited, the keyboard LED

activity returns.

5. Since FlashLED uses INT 1C (timer tick), some TSR screen clocks

may not allow FlashLED to work properly. Try reversing the loading

order of the TSR's.

6. When disk caching software is in use, FlashLED may indicate

activity when data is drawn from the cache (AKA a cache hit).

7. There will be times when the keyboard activity LED doesn't seem

to keep on pace with the activity of the drive. This is due to the

fact that the bios keyboard services are given LOW priority; since the

system figures that the user is probably the slowest component in the

system, it doesn't come looking as often. As a result, FlashLED will

tend to "even out" the rapid flicker of an busy drive's activity light

and show a steady, even glow on the LED. If the speaker option [/p]

has been selected, the "click" will easily keep up with the activity.

8. As mentioned earlier, FlashLED is a tiny TSR (less than 700

bytes). When a TSR is kept in memory, the total amount of memory space

that it takes is the sum of itself and the DOS environment active at

the time of the load.  Since many of us use a "PATH=" which is max'd

out, a "PROMPT=" which is complex and a "SET=" for everything under

the sun, our environment size gets pretty BIG!  To minimize the

environment "impact" on the TSR, try getting the TSR's loaded and

tucked in, first, then build up the environment variables.

9. FlashLED will not see the activity of a network drive since

different interrupts are being used.

10. Some SCSI host adapters are bundled with device drivers which do

not use INT 13, in which case the FlashLED program will not work.

This may also be true for some partitioning software.

11. If the program is started with /shp (scroll lock, hard drives

only, speaker on), and then loaded again with /cf (caps lock, floppy

only) the LED's will switch between scroll and caps, depending on hard

or floppy drive access.

=====Rev=Hist===============

Revision History:

10/91   v1.0   In-house release.

12/91   v1.1   Public release. Added speaker click, command

line options.

=====Legal=Disclaimer=======

LICENSE AGREEMENT

Seagate provides the accompanying object code software ("Software")

and nonexclusively licenses its use on the following terms and

conditions. The Software is copyrighted by Seagate.  YOU ASSUME FULL

RESPONSIBILITY FOR THE SELECTION OF THE SOFTWARE TO ACHIEVE YOUR

INTENDED PURPOSES, FOR THE PROPER INSTALLATION AND USE.  SEAGATE DOES

NOT WARRANT THAT THE SOFTWARE WILL MEET YOUR REQUIREMENTS, THAT THE

SOFTWARE IS FIT FOR ANY PARTICULAR PURPOSE OR THAT THE USE OF THE

SOFTWARE WILL BE ERROR FREE.  SEAGATE EXPRESSLY DISCLAIMS ALL

WARRANTIES, WHETHER ORAL OR WRITTEN, EXPRESSED OR IMPLIED, INCLUDING

WITHOUT LIMITATION WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A

PARTICULAR PURPOSE.  IN NO EVENT WILL SEAGATE BE LIABLE TO YOU, YOUR

CUSTOMERS OR OTHER USERS FOR ANY INDIRECT, INCIDENTAL, CONSEQUENTIAL,

SPECIAL OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE

USE OR INABILITY TO USE THE SOFTWARE.

End of License agreement.

=====Other==================

Available on the Seagate Tech Support BBS (408)438-8771:

Specifications and jumper drawings for all Seagate Disc Drives and

Controllers.

Reprints of Installation Guides.

Troubleshooting essays.

FINDTYPE - Utility which displays bios drive type table and matches a

Seagate model to the best drive type.  Also prints complete

specifications lists and much more!

FINDINIT - Utility for Seagate controllers and host adapters that have

onboard bios. Queries the system to determine bios memory address and

initiates low-level format.

DESK REFERENCE - Hypertext data system for all Seagate products,

troubleshooting, other OEM phone numbers and much, much more. A must

for dealers who do a fair amount of support for Seagate products.

-=EOF: FLASHLED.DOC=-

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 4,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.