HRAMDISK & LRAMDISK

HRAMDISK.SYS is a RAM disk that loads into segments D and E, between

the video buffers and the system ROM.  It was the basis for the PC

Magazine POWER USER column in the summer of 1985.

If you are using my EZDOS program, HRAMDISK.SYS already has the

DTEST code loaded.  This means that you can monitor disk activity

across system warm boots.  Also, this means that the contents of the

RAM disk are preserved when the system is rebooted.

HRAMDISK will initialize system memory if you have memory installed

beyond the mother board switch settings.  In the CONFIG.SYS file,

you need to add a parameter telling HRAMDISK how much memory to

initialize (the real top of RAM).  No reboot will be done, so you

need to use my LRAMDISK program to do that.  This will give you two

non-volatile RAM disks for the price of one.

LRAMDISK.SYS will do the system reboot after it installs itself

above the BIOS top of memory pointer.  With these RAM disk programs,

plus EZDOS, you have abbreviated commands, disk statistics that

continue across system warm starts, and RAM disks that preserve

their contents across system warm starts.

HRAMDISK is installed with a line in the CONFIG.SYS file that

includes the driver name and the amount of system memory installed

below the display buffer.  It should only be used in systems with

memory installed in segments D and E. The result is a 127K RAM disk

and the EZDOS DTEST code.

LRAMDISK is installed with a line in the CONFIG.SYS file that

contains its name, followed by parameters for the disk size, cluster

size, number of directory entries, and amount of system memory below

the display adapters.  My CONFIG.SYS looks like this:

DEVICE=HRAMDISK.SYS 704

DEVICE=LRAMDISK 128 128 32 704

This gives me drive D: in high memory with 127K and drive E: with

128K above the DOS area, which extends to 704K on my system.