RoInfo                                                         Page 1

�����������������������������������������Ŀ

�  RoInfo Hard Drive Information Program  �

�                                         �

�             August 12, 1988             �

�                                         �

�              Revision 1.0               �

�������������������������������������������

RoInfo and this document (c) 1988

/-RoRoWare-/

by Robert Sink, Jr.

The Sixteen Bit Exchange BBS

(703) 342-6386

WARRANTY

The  author  makes  no warranty of any kind, expressed or implied, including

without limitation, any warranties or merchantability and/or fitness  for  a

particular purpose.  The author shall not be liable for any damages, whether

direct,  indirect,  special  or consequential arising from a failure of this

program to operate in the manner desired by the user.  The author shall  not

be liable for any damage to data or property which may be caused directly or

indirectly by the use of this program.

IN NO EVENT WILL THE AUTHOR BE LIABLE TO  YOU  FOR  ANY  DAMAGES,  INCLUDING

ANY  LOST  PROFITS,  LOST  SAVINGS  OR  OTHER  INCIDENTAL  OR  CONSEQUENTIAL

DAMAGES ARISING OUT OF YOUR USE OR INABILITY TO  USE  THE  PROGRAM,  OR  FOR

ANY CLAIM BY ANY OTHER PARTY.

RoInfo                                                         Page 2

Explanation of RoInfo:

RoInfo was  designed  as  a  guide  to ease in the setup/use of hard drives.

RoInfo provides total information on two of the largest and most  used  hard

drive  brands  today  -  Seagate and Miniscribe.  Not to mention RoInfo also

gives  technical  assistance  numbers  in  case  you  need  to  contact  the

companies,  minor definitions of all the encoding methods (such as MFM, RLL,

etc..) And also provides the "DEBUG  FORMAT  ADDRESS  CODES"  for  low-level

formatting.

Technical information about RoInfo:

RoInfo  was  written  in  Turbo  Pascal*  4.0, using TTT (Technojock's Turbo

Toolkit) for various  windowing  and  cosmetical  enhancements.   RoInfo  in

it's  entirety  is  a  very simple program providing the user with quick and

easy  reference  to  perhaps  some  of  the  most   needed   and   forgotten

information  facing  the  continual  use  of hard drives around the country.

Definitions of terms used in RoInfo:

Since there was alot of information on these drives and so little  space  to

put  it,  I  found  myself  making  abbreviations  for the statistics on the

table.  Although the program  gives  a  fairly  understandable  key,  it  is

going to be explained to you here on a more understandable level.

Model   -  This can explain a considerable amount to you  about  the  drive.

Such  as size, disk capacity, series, etc..  (explained  further

on..)

Disks   -  Platters,  these  are the actual "storing" parts of a hard drive.

These spin and are read and wrote to by the heads.

Heads   -  Heads read  and  write  information  to  the drive by reading the

platters (disks). There are usually two heads per every disk (one

reads the top, the other the bottom).

AcsTime -  Access  time.  This is a general  average of how fast the head is

able to read the data from the spinning media (disks).

RoInfo                                                         Page 3

(definitions continued...)

Trk2Trk -  Track to track access  time.   This  is similar  to  the  regular

access  time,  but  this is a judgement of how fast the heads can

read sequential data (track to track).   This  value  is  usually

considerably lower than the regular access time value.

AvgPwr  -  Average power consumption by this particular drive.

Size    -  This is usually defined by "HH" or "FH".   HH being  half height,

or FH being full height (two half height drives).

MTBF    -  Mean   time   between  failure.   This  is  how  long  the  drive

can  go  before  a  failure  in  the  hard  drive  is   possilble

(measured in hours, and considering the drive gets normal use).

ActTyp  -  Actuator  type.   The  actuator  is the device that positions the

heads over the media.  The actuator usually  plays  an  important

part in the access time of the drive.

EncMtd  -  Encoding  method  that the drive is rated for.  Such as MFM, RLL,

ESDI, or SCSI.

SctrPt  -  What  the  hard drive's sector per track rating is.  This is more

dependant of the controller than it is the hard drive.

AutoPk  -  Merely tells if the drive will auto-park its  heads if  power  is

removed  or  if  it is not supported, then the hard drive parking

procedure must be assisted by a software sequence or program.

Media   -  Gives a list of  the  coating  and/or oxides  found on  the  hard

drive platters.  Explains what kind of controller is most optimum

for this hard drive as well.

Megs  -    Explains the disks  formatted  capacity (i.e.  free space after a

high level format.)

Help in understanding what the model number of a drive indicates:

Miniscribe information:

Miniscribe drives consist of 4 or 5 digit model numbers  and  in  there,  it

explains the disks' size, and capacity - as follows..

6128

����

������������> Indicates the drive's unformatted capacity.

������������> Indicates the drive's physical size FH or HH.

Drives  beginning  with  3's  are half height, drives beginning with 6's are

full height, and drives beginning with 8's are 3.5's.

RoInfo                                                          Page 4

(understanding drive model numbers continued...)

Seagate information:

Seagate  drives  consist  of  5-7  digit  model  numbers  and  in  there, it

explains the disks' size, revision, and/or RLL, MFM or SCSI compatibility.

ST238R

������

������������> Indicates the drive is RLL compatible.

������������> Indicates drive's unformatted capacity.

������������> Indicates the drive's physical size FH or HH.

������������> Indicates "Seagate Technology".

Drives that have "N" prefixed to the end of  their  model  number  are  SCSI

compatible drives such as "ST225N".

Drives  that  end  in  "-1"  or  any other numerical value indicate revision

number such as "ST251-1" which is a revision of the ST251  that  is  10  ms.

faster than the older ST251.

Seagate  drives  beginning  with  a  "ST1"  are 3.5" drives.  Seagate Drives

beginning  with  a  "ST2"  are  half  height  drives.   And  Seagate  drives

beginning with a "ST4" are full height drives.

Notes from the author:

This  program's  main  purpose  is to provide the most clear-cut information

concerning hard drives as possible.  Though we feel the information  on  the

two  drive  brands  (Miniscribe  and Seagate) is complete and as accurate as

can be, there is  no  reason  why  other  drive  manufacturers  couldn't  be

included  in  future  releases.  If you happen to have some information (any

information) that  could  help  us  out,  we'd  definitely  appreciate  your

help.   If  you  find any of the statistic and/or information here or within

to be no longer accurate or inacurate, we would appreciate  you  letting  us

know as soon as possible.  We may be reached on the following BBSs:

The Sixteen Bit Exchange BBS

24 hrs/day  7 days/week

(703) 342-6386  -  my BBS

PhotoStar BBS

24 hrs/day  7 days/week

(703) 774-4667

Bedlam BBS

24 hrs/day  7 days/week

(703) 890-5733

RoInfo                                                         Page 5

Acknowledgments:

I'd  like  to  thank  Mike  Overacker  -  SysOp  of  the  PhotoStar  BBS for

providing some of the information in  compiling  this  list  of  statistics.

And hope he'll have other help in the future (other than parties)...

Robert Sink - SysOp of the Sixteen Bit Exchange

RoRoware

* Turbo Pascal is a registered trademark of Borland International, Inc.

[end of RoInfo document]

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  The P(s)L cannot de-

bug programs over the telephone.

Disks in the P(s)L are updated monthly, so if you did not get

this disk  directly from the P(s)L,  you should be aware that

the files in this set may no  longer be the current versions.

For a copy of the latest monthly software library newsletter

and a list of the 1,400+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

(713) 665-7017