BIOSCKSM - v1.2 10/15/88, Charlie Stampf, N2CJ

BIOSCKSM is a program to aid in the modification of the ROM BIOS in a

PC/AT computer.  I had a need to add new information to the hard disk

parameter table in BIOS.  I quickly found that changing anything would upset

the checksum routine that occured during the Power On Self Test (POST).  If

the routine finds changes it sends a HALT instruction to the CPU, stopping

everything.  There had to be a solution.  I found the BIOS checksum routine

after searching the listing contained in the PC/AT technical reference.  I

noted that test can be defeated by simply replacing the offending HALT with a

NOP instruction.  I would then be able to add my new disk data to the table.

I then began to realize that I would defeat a routine that might

possibly ferret out some future problems.  I couldnt rest with that threat

each time I powered up the computer.  So I began to search for a fix.

I used the program GETROM  to extract and split the BIOS code into two

modules as required by the hardware.  The code is contained in chips U27 and

U47 on the AT mother board.  U27 contains the even numbered bytes while U47

contains the odd.  I used an editor to change the HALT to a NOP at offset

019EH.  Then I added the data for my new disk parameters.  It turns out that

the last byte in the BIOS contains a value that when added to all other bytes

returns a result of zero.  All that is necessary, is to figure the correct

value, replace it and off you go.  Well, my hexadecimal mathematics is not too

good.  I decided to try my changes before restoring the checksum routine.  I

burned two EPROMS with the new code and inserted them into the machine.   Lo

and behold, everything worked as expected.

Now on to restoring the checksum routine.  It was easier to write

BIOSCKSM than to go through all the arithmetic required.  Besides, I enjoy

assembler and jumped to the challenge.  I wanted the program to give me the

value needed to replace the checksum byte in BIOS.

When BIOSCKSM is first invoked, it checks to see if the BIOS had been

changed to defeat the checksum.  It looks for the added NOP instruction.  If

it is not found the program displays a message to that effect and aborts.  If

the NOP is found, a checksum is determined by adding all the byte values of

the BIOS.  Then BIOSCKSM calculates the correct value for tha last byte

location, the offset byte.  Both values are displayed on the screen.  With

this information, all that is required is to create new EPROMs, insert them

into the computer and power up.  If all goes well, the POST should complete as

usual.

I realize that messing around in the BIOS is not a task for all.

Only true hackers would make the effort.  However, this was a good exercise

and gave me the opportunity to learn a bit about the inner workings of

the BIOS.  I hope you find BIOSCKSM useful.

NOTE:

The neat little utility GETROM can be found on many BBSs and on CSERVE.

Unfortunately the ARC file as I obtained it, did not contain an author's name,

so credit cannot be given.  But, thank you anyway.