BW-ONLY PROGRAM

Written by: Marshall Dudley

BW-ONLY is a memory resident program which checks all mode commands

going to the BIOS, and if they are color mode commands (such as co80

or co40) replaces them with the same monochrome (B/W) commands.  Thus

if you have a monochrome composite monitor, and an IBM or other brand

color graphics adapter which does NOT have a second composite jack for

driving monochrome, this program will turn off and keep off the color

subcarrier. The subcarrier, if driving a monochrome monitor, tends to

make the display unreadable.

To install: Simply log the drive which BW-ONLY is on and type in BW-ONLY,

or include it in your batch file to get installed on power on.

The screen should clear and you will immediately be in the B/W

equivalent of whatever mode you were in previously.  Attempts to

change back to the color mode will now be futile.

De-install: ALT/CTL/DEL. Obviously this deinstalls everything but that's the

way it goes with memory resident programs.

Problems:   Some programs do not turn on the color through the BIOS, but

write directly to the video chip.  Obviously, this program will

not be able to overide such methods.  It appears that QMODEM

does this on the main screen if you don't have a window opened.

However, if you can define the screen colors, as in QMODEM, then

defining the colors as only black, white and gray will force the

subcarrier off anyway.

This program should install with other memory resident programs

independent of the order of installation. If problems arise then

try installing in a different order.

Since there are some sadistic people which will hack a program

and try to destroy your data by uploading other people's programs

which they have hacked, I am torn between uploading only the .COM

file to make this more difficult, or both the .COM and .ASM files

so the user can verify what is going on and that he doesn't have a

hacked version.  I feel that knowledge solves most everything so

I am including the .ASM file.  It should assemble (with exe2bin)

to the included .COM file exactly.

Rights:     This program is copyrighted.  It is illegal to hack it and upload

it.  However, feel free to give copies away, and upload to your

hearts desire, unmodified copies, provided they include all 3 files

(.COM, .ASM, .DOC).  Copies may not be sold, except for a nominal

charge to cover media cost.  If you encounter any problems I can

be reached on Compuserve ID# 72416,3357 or written to at:

Marshall Dudley

12402 W. kingsgate Dr.

Knoxville, Tn. 37922

Warantee:   None.  The user assumes sole responsibility in determining if this

program is suitable for his requirements.  the author is not liable

for any losses thought to been caused by this program.  It is the

user's responsibility to determine that he does not have a hacked

copy, and that the program is used appropriately.

Hopefully this makes your computing a little easier until you can get that RGB

monitor I'm sure you want.

BW-ONLY Version 2, written by Marshall Dudley

Several months ago I wrote BW-ONLY to allow one to use programs which

set the CGA into a color mode, with a CGA card and a composite monitor.

I have since found out that a number of programs, most notably ANSI.SYS

set the mode to color without going through BIOS.  The status word which

was last sent to the CGA is suppose to be in 40:65h, but ANSI doesn't

change that when it goes to color mode.  Therefore, version 2 of the

program resets the mode to black and white mode, via hardware on almost every

call of int 10.  Since this could conceivable cause some problems for other

programs, I have included both BW-ONLY version 1 and BW-ONLY version 2 in

this file.  If version 1 works for you I would use it,  if not then try

version 2.

Note that on some CGA boards, different colors have the same intensity.  Thus

you may find that a program which puts say red writing on a green background

will be all white.  This is hardware dependent, and if you find out which

colors are the same, a trap could be written into this program to translate

them to contrasting intensities.  I have not done this as virtually every

brand of CGA board would require a different translation.