�����������������������������������������������������������������Ŀ

�      *** One, Two, and Three Stage TRINOMIAL Sampling ***       �

�                                                                 �

�               A Quality Assurance Training Tool:                �

�        Statistics Committee of the QA Section of the PMA.       �

�                                                                 �

�        by Bob Obenchain, Manager, Nonclinical Statistics        �

�               Glaxo, MIS Statistical Services.                  �

�������������������������������������������������������������������

TABLE-OF-CONTENTS

=================

NOMAJORS.EXE.................................Executable Module

NOMAJORS.OUT.................................ASCII Output File

<NOMAJORS.IN...............ASCII Batch Input (DOS Redirection)

PROMPTS and DEFAULTS..................Minimum & Maximum Values

NOMAJRUN.BAT...........................Input & Output Examples

NOMAJORS.C..............................C-language Source Code

*********************************************************************

NOMAJORS.EXE

NOMAJORS calculates the expected percentage of acceptable lots

in single, double, or triple stage TRINOMIAL sampling in which

acceptance implies that NO MAJOR Nonconformances were observed.

Testing of each unit sampled is assumed to yield one of three

possible results...

Observance of a Conformance to Specification

(say, within 25% of a target value), or

Observance of a Minor Nonconformance

(say, deviation from target by 25% to 35%), or

Observance of a Major Nonconformance

(say, deviation from target by more than 35%).

Furthermore, NOMAJORS assumes that...

the observance of a single MAJOR Nonconformance implies

rejection of the lot, and

a one, two, or three stage acceptance sampling plan for MINOR

Nonconformances will be specified.

NOMAJORS.EXE is an executable module for IBM-compatible personal

computers.  It was compiled using Microsoft C, version 5.0.

NOMAJORS.EXE thus provides automatic detection (and use or

emulation) of an 8087/80287/80387 numeric math coprocessor.  If

your PC has a math chip, NOMAJORS should run much faster.  If

your PC does NOT have a math chip, NOMAJORS will still run and

should yield identical numerical results.

NOMAJORS.OUT

This is the default filename for the ASCII summary file of inputs

and outputs from NOMAJORS.EXE.  You may specify an alternative

name and/or extension for this file as the first argument to the

NOMAJORS invocation on the DOS command line...

Prompt> nomajors myname.ext

You can read/print this file after the program terminates, so you do

not need to make hand-written notes during execution.

<NOMAJORS.IN

NOMAJORS accepts ACSII batch input via "redirection" (from a file

to the keyboard console) specified on the DOS command line.  You

simply specify the batch filename preceded by the < (less than)

symbol as the last argument to the NOMAJORS invocation...

Prompt> nomajors myname.ext <batch.in

or

Prompt> nomajors <batch.in

The format for a batch INPUT file is similar to that of a NOMAJORS

OUTPUT file, except that

* lines that do not start with a number should not be present,

and

* only the first number on each line of the file is utilized as

input.  This number may be either an integer or a floating

point number - but must be separated from other information

on the same line by at least one white space (blank or tab.)

The example input file distributed with NOMAJORS is USP_CAP.IN,

for analysis of the USP (two-stage) Capsule Acceptance Plan.

One way to create an input file would be to use the invocation

Prompt> nomajors trial.out

to run NOMAJORS in prompt mode, saving calculations for, say, 50

different pairings of Major and Minor % NonConforming incoming

for inspection in a file named TRIAL.OUT.

Now suppose you edited TRIAL.OUT, changing sample sizes or

accept and/or reject numbers for Minor Nonconformances, deleting

lines that do not start with a number, and separating the % Major

NonConf and % Minor NonConf values - placing them on successive

lines of the file.  Name the new file, say, TRIAL.IN.  Then

an invocation of the form

Prompt> nomajors trial.new <trial.in

could evaluate a new trinomial sampling plan at, say, the same

50 Major-Minor % NonConf pairings that you used in your first

run.

PROMPTS and DEFAULTS

...Type responses to prompts using the number keys along the

top row of the keyboard, and then press the ENTER key.

...If you press the ENTER key without typing a value, your

response will be assigned to the DEFAULT value.

...The prompts are listed below with minimum, default, and

maximum allowed values.

Enter your First Stage Sample Size: n1

Minimum Sample Size =     2

Default Sample Size =    50

Maximum Sample Size = 5,000

Enter your First Stage MINOR Nonconformance Accept Number: a1

Minimum Acceptance Number =     -1

(i.e. lot acceptance impossible at stage 1

even if zero nonconformances are observed.)

Default Acceptance Number =      0

Maximum Acceptance Number =      min(n1,999)

Enter your First Stage MINOR Nonconformance Reject Number: r1

Minimum Reject Number =      max(a1+1,1)

(i.e. single stage sampling)

Default Reject Number =      max(a1+1,1)

Maximum Reject Number =      n1

Enter your Second Stage Sample Size: n2

Minimum Sample Size =     1

Default Sample Size =    50

Maximum Sample Size = 5,000

Enter your Second Stage MINOR Nonconformance Accept Number: a2

Minimum Acceptance Number =      a1

Default Acceptance Number =      a1

Maximum Acceptance Number =      min(n1+n2,999)

Enter your Second Stage MINOR Nonconformance Reject Number: r2

Minimum Reject Number =      a2+1

(i.e. two stage sampling)

Default Reject Number =      a2+1

Maximum Reject Number =      n1+n2

Enter your Third Stage Sample Size: n3

Minimum Sample Size =     1

Default Sample Size =    50

Maximum Sample Size = 5,000

Enter your Final MINOR Nonconformance Accept Number:  a3

Minimum Acceptance Number =      r2 - 1

Default Acceptance Number =      r2 - 1

Maximum Acceptance Number =      min(n1+n2+n3,999)

Specify Testing Efficiency (Percentage):

Minimum Testing Efficiency =    10%

Default Testing Efficiency =   100%

Maximum Testing Efficiency =   100%

YIELD Calculations...

...The user is repeatedly prompted to specify pairs of

major-minor nonconformance percentages of units within lots.

These are the values of quality (% NonConf) "incoming" to

the acceptance sampling inspection.

...Enter values between 0.0% and 100.0%.

...Enter any negative value (say, -1) for % MAJOR

Nonconforming to halt execution of NOMAJORS.EXE.

...NOMAJORS.EXE will calculate the "Yield Percentage"

corresponding to each specified incoming % NonConf pairing.

This Yield % is 100 times the Probability of Lot Acceptance,

which defines the Operating Characteristic Curve for the

trinomial sampling plan.

NOMAJRUN.BAT...

...This DOS batch command file contains "nomajors %1.out <%1.in"

and is invoked by entering a command line of the form...

Prompt> nomajrun xyz

where XYZ is a DOS filename (8 characters maximum) with

neither a period (.) nor a 3 character filename extension.

NOMAJORS then creates (or overwrites) file XYZ.OUT using a

file named XYZ.IN, which must already exist, as input.  If

file XYZ.IN cannot be found, XYZ.OUT is not created and no

calculations are performed.

NOMAJORS.C

This is an ASCII file of C-language source code for NOMAJORS.  The

only external functions called by NOMAJORS are the logarithm and

exponential functions, so the code should be highly portable.  For

example, to compile and run the program on a VAX under VMS, upload

NOMAJORS.C to the VAX in ASCII mode and enter the commands...

VAX Prompt> cc nomajors

VAX Prompt> link nomajors

VAX Prompt> run nomajors

*********************** End of NOMAJORS.DOC *************************

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  The P(s)L cannot de-

bug programs over the telephone.

Disks in the P(s)L are updated monthly, so if you did not get

this disk  directly from the P(s)L,  you should be aware that

the files in this set may no  longer be the current versions.

For a copy of the latest monthly software library newsletter

and a list of the 1,800+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

(713) 665-7017