�������������������������������������������������������������������������

��������                                  ������ ��� ��� ������ ��   ��

�  ��  � ��� ��� ������   �����   �����  ���   � ��   �� ��   � �� ����

��    ��   �� ��  ��� ��� ��� ��� ��� ��      ��   �� ��     �����

��    ��   �� ��  ��� ��  ��� ��   �� ��      ������� ����   ���

��    ��   �� ������  ������  ��   �� ��      ��   �� ��     �����

��    ��� ��� �� ���  ��  ��� ��� ��� ���   � ��   �� ��   � �� ����

����    �����  ��� ��  ������   �����   ������ ��� ��� ������ ��   ��

�������������������������������������������������������������������������

Version 2.10a

By Steve Blinch of Mikerosoft Productions

�������������������������������������������������������������������������

�������������������������������������������������������������������������

Intro

�������������������������������������������������������������������������

TurboCHEK  is a  callback verification  system designed to  ensure

BBS security.  The  idea is  that  if a  user is  using a  fake  phone

number, he won't be  able to get  access to  your BBS,  and cause  any

problems there -- and if he's using his REAL phone number and  causing

problems, even under a fake name, it's  not too difficult to  find out

his REAL name, and take actions against him/her.  TurboCHEK also stops

users from using more than one username on your BBS, as it will  auto-

matically search your userbase for duplicate phone numbers.

�������������������������������������������������������������������������

TurboCHEK Warranty/disclaimer

�������������������������������������������������������������������������

Prior  to it's  release, TurboCHEK was  tested  on an IBM '486/33,

'386/33, '386/20, and a '286/12.  We  found no  errors, and  TurboCHEK

successfully called back  every user  that we tried it on.  What we're

trying to say is that it works just great for us.

But,  there  IS  the  possibility  that it might not work for you.

So, if you use it, and it  doesn't  perform as well  as it did on  our

systems,  you take  full  responsibility for  anything  that  happens,

including any problems or damages that may occur due to the use of it.

In  no  event  shall  Mikerosoft  Productions, Steve  Blinch,  Michael

Helliker, or any other member of our crew be  liable to you or  anyone

else  for any  damages or costs,  including, but not  limited to,  any

lost profits, lost savings, lost income, lost information, loss of the

ability to use TurboCHEK, or other incidental or consequential damages

arising out of the use or inability to use TurboCHEK.

�������������������������������������������������������������������������

Configuring TurboCHEK

�������������������������������������������������������������������������

TurboCHEK now comes with a setup program, TCSETUP.EXE, as  opposed

to the old .CTL file method of configuration.  Since some of the items

in the configuration might be a bit obscure, here they are explained a

bit more clearly:

Main Menu

����������������������������������������������������������������������

BBS Software

��������������

Not too difficult -- The BBS software that you're running: SuperBBS

or RemoteAccess.

BBS Path

��������������

The full path to your BBS files (USERS.BBS, TRASHCAN.CTL, etc).  In

a multinode system, this would be your common path.

Log Style

��������������

How  should  TurboCHEK  write to  your log  file:  SuperBBS  style,

RemoteAccess Compact, or RemoteAccess Expanded.

Log Filename

��������������

The name of your log file.  This is written to either the BBS  path

(in a single node  system) or the  node directory  (in a  multinode

system).

Bad Phone Number Handling

���������������������������

If a user has a phone number which you have placed in the bad phone

numbers list, what action should be taken?

a) Return to the BBS

b) Hang up on user

c) Put the user's name in the TRASHCAN.CTL file in your BBS direc-

tory (or common directory) & hangup, therefore locking  his/her

username out of your BBS.

Dupe Phone Number Handling

����������������������������

If a user has a phone number which is already being used by another

user on your BBS, what action should be taken?

a) Return to the BBS

b) Hang up on user

c) Put the user's name in the TRASHCAN.CTL file in your BBS direc-

tory (or common directory) & hangup, therefore locking  his/her

username out of your BBS.

Dialing Menu

����������������������������������������������������������������������

Delay Before Dial

�������������������

After disconnecting, some modems require a pause of a second or two

to recuperate.  That's what this number is for.

Wait for Connect

������������������

After dialing the number, how long should  TurboCHEK sit there  and

wait patiently for a connection?

Dial Attempts

���������������

How many times should TurboCHEK dial  the user's  phone number  and

wait for a connect if it does not connect on the first attempt?

Number Menu

����������������������������������������������������������������������

Local Area Code

�����������������

Your area code, the one  people have to dial  when they're  calling

you long distance (i.e. in ###-xxx-xxxx and (###) xxx-xxxx, ###  is

the area code).

Local Prefixes

����������������

A dial prefix is the first 3 digits of a seven-digit phone  number,

i.e. in xxx-###-xxxx and (xxx) ###-xxxx, ### is the prefix.  If you

can find ALL the dial prefixes  which are  local to you, and  enter

them into TurboCHEK's  setup, TurboCHEK  will be able to tell auto-

matically  who is a  long distance caller and who is not (using the

area code as well) and decide whether or not to call the user back.

If you cannot find out all of the prefixes,  or there are too  many

to enter,  this option can be disabled and the user will simply  be

asked whether or not  he is long  distance.  (Don't  worry, if  the

user lies, it  still won't  cost you anything  as the call won't go

through anyway, without the 1- in front of it!)

The defaults are for Abbotsford, B.C., Canada.

Bad Phone Numbers

�������������������

If you want to avoid a  certain user, enter his phone number  here.

When he calls, he will not be verified, and an action will be taken

(he will either be returned to the BBS, hung up on, or  locked out,

depending on what option you chose back in the Main Menu).

Toggle Menu

����������������������������������������������������������������������

Save Password In Comment Field

��������������������������������

As a security feature in RemoteAccess, the makers decided it  would

be a good  idea to  lock up  the  users' passwords nice and  tight.

Notice how you can NEVER see the users' passwords in RAMGR?  Due to

interest  by  SysOps in  their  users'  passwords,  I've added this

option  which puts the user's password in the comment field, there-

fore allowing the SysOp to view it thru RAMGR.  Of course, this  is

of no benefit to SBBS users, but hey, they should switch to RA any-

way!! :-)

Check Dupe Numbers

��������������������

Should TurboCHEK check to see if the user's phone number(s)  is/are

already in use?  This is useful in case the user changes his  phone

number just before verification, but after logging on.

Verify Long Distance Users

����������������������������

Only use this if you're rich!  If this is  enabled, TurboCHEK  will

place long  distance  phone calls (charged to YOU) to  verify  long

distance  users.  It  is *NOT*  recommended that  you  enable  this

option, for obvious reasons!

Ask User If He/She Is Calling Long Distance

���������������������������������������������

If there are too many dial  prefixes for your calling area, or  you

just  don't feel  like  typing them  in, just  enable this  option.

TurboCHEK will then ask the user whether  or not he/she is  calling

long distance.

Color Menu

����������������������������������������������������������������������

Bar Color

�����������

The color of the boxes & a few other things.

Prompt Color

��������������

The color of prompts, menu items, etc.

Text Color

������������

The color of variable text (username, phone numbers, etc.) &  inst-

ructions given to the user.

Hilite Color

��������������

The color of text used to point out specific information.

�������������������������������������������������������������������������

Setting up TurboCHEK for use with RA and SuperBBS

�������������������������������������������������������������������������

Create a batch file containing the following:

@Echo Off

CD\RA\TCHEK

TCHEK.EXE [parameters]

CD\RA

[parameters] would be one or more of the following:

/N?        Specify node number, ? is the number itself

/P<path>   Specify node directory (for EXITINFO.BBS, etc.)

/1         By default, TurboCHEK reads DORINFO?.DEF, where ?

is the node number -- using /1 forces DORINFO1.DEF

no matter what node the user is on

/!         Read DOOR.SYS instead of DORINFO?.DEF

/S???      Multitaskers: Release timeslice every ???/100 of a second

/O         Do not verify user, only ask password (intended for RA

SysOps to get their users' passwords into the comment

field, even if the user has already been verified).

/Fxyz      Only works with /O parameter, sets a certain flag if the

user enters the correct password.  "x" is the flag letter

(A, B, C, or D), "y" is the flag number (1 to 8), and "z"

is the status (- or X).  See section below explaining this.

(when using parameters, "-" or "/" can be used, it doesn't matter)

We used the following menu items when testing TurboCHEK:

RemoteAccess [tested under version 2.02]

Type: 7

Data: *C /C D:\RA\TCHEK\TCHEK.BAT *N

(You may need to add a *M at the end if you run outta memory!)

SuperBBS [tested under version 1.17-3]

Type: 7

Data: *C /C D:\SBBS\TCHEK\TCHEK.BAT *N *E

(You may need to add a *S at the end if you run outta memory!)

Concord [tested under version O.O1-G]

Type: Run External Program

Data: *C /C D:\CONCORD\TCHEK\TCHEK.BAT *N *D2- *D3-

(You may need to add a *S<minmemkb> at the end if you run outta

memory!)

�������������������������������������������������������������������������

RA: Password-To-Comment Field, users who have already been verified

�������������������������������������������������������������������������

If you're using RA, and just switching to TurboCHEK, you probably  have

several (hundred?) users verified already.  Rather than force each  one

to use the CBV again, you can do the following to ask the password ONLY

and place it in the user's comment field, NO callback required:

In Your Batch File (D:\RA\TCHEK\TCHEKPWD.BAT)

@Echo Off

CD\RA\TCHEK

TCHEK -PD:\RA\NODE%1 -N%1 -O -FC8X

In Your Menu (preferrably the first option in the TOP menu)

Type: 7

Data: *C /C D:\RA\TCHEK\TCHEKPWD.BAT *N

Flag: Flag C8 *MUST* be set to "O" (i.e. C might be "-------O")

This way, if RA sees that flag C8 is NOT set, TurboCHEK will be run and

the user will  have to  enter his/her password.  If he/she  enters  the

correct  password,  flag C8 will be set, therefore  stopping  TurboCHEK

from being run again.

This might seem complicated,  and if it does, don't  worry... Just  set

everything up as shown here and it'll work just fine!

*NOTE*  C8 is just an example, you can use any flag you want, but which

ever one you use MUST be set to "-" for every user in the user-

base!

�������������������������������������������������������������������������

Version History

�������������������������������������������������������������������������

[2.10a] � Added Concord support

� Added password-to-comment option for RA

� Setup program, TCSETUP.EXE now added

� Text control file TCHEK.CTL replaced with binary TCHEK.CFG

� Added specific bad number & dupe number consequences (Return to

BBS, Hang up, or Lockout user)

� Bad fone number check now checks both voice & data, not just

the one that the user chooses to call back at

� Added file sharing (nasty share violations started to annoy

me! :-))

� Added dupe fone number check

� Got rid of DDI support.. 'twas a good idea, but it was just too

damned hard to use :-)

� Did some internal stuff... fixed a few cosmetic errors... and

a bunch of other little buglets... :-)

� Fixed bug in which xxxyyyy style phone numbers were formatted

as xxx-yyy-y

� Fixed bug in which long distance verification would not  work

even if the SysOp configured it to!

� TurboCHEK no longer dials the area code before the phone num-

ber, except in long distance calls

� Fixed bug in which 1- phone numbers would not be handled

� Added new codeword  file TCHEK.CWD,  from which all  the user

codewords will be read (if file exists, that is)

� Fixed  bug with  REALLY  long user-locations & most  SuperBBS

phone numbers being shown incorrectly

[2.00a] � Due to WAY too many bugs &  problems (not to  mention  losing

the source  code!),  I decided  to  completely  rewrite  this

version from scratch.  Everything's new, and should work MUCH

better than the previous versions!!

[1.00b] � Never released this sucker  to the public -- the only one  to

get it was Mikerosoft BBS!

[1.00a] � Fixed bug where TurboCHEK would not call the user back  again

if he was not reached on the first call

� Many previous  versions were  created,  however, none of them

worked... this does, though... <for us, at least!>

�������������������������������������������������������������������������

End of TCHEK.DOC

�������������������������������������������������������������������������