HPFilter

Rip Toren

POB 674

Columbia MD 21045

12 Jan 1987

So... you just got the fancy HP LaserJet+ printer that will allow you

to publish your own version of whatever. You have just spent about 3

hours reading the operator's manual, as well as the technical

reference. And you still aren't sure what this hummer will do.

In short:

1)   It will not print out the extended IBM PC character set. You get a

bunch of European characters that just don't convey the same

impact.

2)   You can't seem to get anything that resembles BOLD printing.

3)   And that compressed printing will only work in portrait (up and

down) and not in landscape (side to side) where it would be really

useful.

HPFilter can solve only the first two of those problems for you.

The concept is that HPFilter installs itself in the interrupt 17 chain,

so that it has a chance to look at each character as it is sent to the

printer.

If a code between 179 and 223 (these are the line drawing and block

characters) is seen, a substitute string is sent in its place. This

string is a graphic definition to the printer to create the desired

character as a set of individual line segments.

In the case of BOLDing, the character is printed a second time, with

the registration offset by 1/300th of an inch in both the vertical and

horizontal directions. This may be considered FATtening, rather than

bolding, but it's better than nothing.

<<INSTALLATION>>

Simply execute HPFilter from the command line. It will take up a bit

less than 4k bytes. If you want to disable HPFilter, simply run the

program again. Each invocation flips HPFilter on and off.

To activate the BOLDing, you will have to insert a code into your file

to turn it on and then off. To turn BOLDing on, send the codes

<ESC>+'G'. That is in hexadecimal <1B 47> or <27 71> in decimal. To

turn the BOLDING off, use <ESC>+'H' or <ESC>+'E'. The later code is a

reset to the LaserJet.

If by chance you leave HPFilter active, while printing to a printer

other than the HP LaserJet, expect a bunch of gibberish for those

graphic characters.

<<SOURCE>>

The source has been included so that you may add any additional

characters that you might want. In the definitions for the substitution

strings, you will see a number of periods (.). These are ingnored, and

not passed to the printer. They just make things easier to line up and

see what is going on in the commands. You may obviously delete them to

compact the size a bit. To rebuild the application,

>masm hpfilter,hpfilter;

>link hpfilter,hpfilter;

(you will get a STACK error - ignore)

>exe2bin hpfilter hpfilter.com

The lines that are drawn are 3/300th of an inch. The are laid out on a

grid of 30x50 pels. This is consistant with the examples in the HP

Tecnical Reference manual.

<<DISCLAIMER>>

This is the first assembly program that I have ever written in the

INTEL 80x8x family. It is oversized, and can be tightened up with

subroutine calls. But figuring out where the assembler and linker would

place things was too confusing.  The basic structure was inspired by an

article and program by Jeff Garbers in the September 1984 issue of PC

Tech Journal. He said his program was a good basis for further work,

and he was correct. Thanx.

If you have any comments, or useful HP LaserJet utilities, I would love

to hear from you at the above address, or leave a message on Vince

Castelli's CHESAPEAKE BBS (301) 267 4930 addressed to RIP TOREN.