------------------------------------------

ASCII to POSTSCRIPT Translator Release 0.6

by Johannes Geenen

------------------------------------------

This is release 0.6 of my ASCII to Postscript translator driver

PSDRIVER. It is a true DOS device driver that redirects output from a

DOS printing device (LPTx) to a BIOS printer port and translates input

into Postscript statements. PSDRIVER support a big subset of the IBM PC

character set including national character sets and most IBM graphic

characters. It support a subset of the EPSON FX printer code sequences,

too.

PSDRIVER has a DOS command line interface to review installation status

and change default behavior of PSDRIVER's output. This command line

interface is bound together with the device driver to an .EXE file.

PSDRIVER was developed with Borland's TASM and C++ under DOS 6.2

environment and tested under DOS 5.0, 6.0 and 6.2, Novell DOS 7.0 and

the DOS shell of OS/2 2.1.

It allows output on network printers (tested with our BANYAN VINES 4.11

and 5.53 LAN) and may be used under WINDOWS for output through the

LPTx.DOS devices. I am using Mannesmann Tally MT906PS for testing, but

PSDRIVER uses no printer specific Postscript extensions and should work

on any Postscript printer.

PSDRIVER occupies less than 9k in memory.

Installation

------------

Although PSDRIVER is named PSDRIVER.EXE, it is a true DOS device driver

and has to be installed during boot procedure from the CONFIG.SYS file.

To install PSDRIVER, simply add a line to your CONFIG.SYS file:

DEVICE = <Your_Drive_And_Path>\PSDRIVER.EXE

where "<Your_Drive_And_Path>" is the location of PSDRIVER.EXE, i.e.

DEVICE = C:\UTIL\STARTUP\PSDRIVER.EXE

PSDRIVER may be loaded high using the DEVICEHIGH statement instead of

DEVICE.

On default, PSDRIVER hooks onto the LPT3 device and redirects output

after translation to the first BIOS printer port. PSDRIVER assumes an A4

page size (297*210mm) and will print in portrait orientation. Several

command line options may be used during installation to change this

default behavior:

/D<device-name>   The default LPT3 may be changed using the /D

switch to a different device. The device name

is allowed to have at maximum 8 characters

that will be converted to upper case letters.

/P<printer-port>  Specify the desired printer port (1, 2, or 3

are valid printer port IDs) for the trans-

lated output.

/S<page-size>     Specify one of the following digits:

1 - Letter size

2 - Legal size

3 - A4 size (default)

/O<page-orient>   Specify

1   for portrait orientation

2   for landscape orientation

/L<left-margin>   Specify left margin in characters;

current character density will be used

/M<code>          Specify Multiplex interrupt 2Fh code that

will be used by PSDRIVER command line

(default EFh). Use a two character hex value.

(See troubleshooting)

For example, if you are using letter size paper with your Postscript

printer, the printer is attached to the third printer port (or a network

redirector hooks redirects output to the second printer port to a

network printer), and you want to use LPT2 for ASCII output on your

Postscript printer, insert the following DEVICE statement into your

CONFIG.SYS file:

DEVICE=PSDRIVER.EXE /DLPT2 /P3 /S1

Option switches may be either in small or captial letters. Do not insert

spaces between switch and value.

Usage

-----

PSDRIVER does not change the behavior of the original printer port

(unless you map PSDRIVER onto the same device and printer port). Print

your Postscript files to the original port, and your ASCII files to the

re- directed port. PSDRIVER will do the rest:

- On the device open call it sends the internal Postscript header

to the printer.

- It captures each character, converts it to the equivalent character

of the postscript character set or issues Postscript procedure

calls to the printer.

- On the device close call it performs a last showpage to eject the

last page and terminates the Postscript job.

When calling PSDRIVER from DOS command line, it will report the

installation status of PSDRIVER.

You may use command line switches to change the default behavior of

PSDRIVER for subsequent outputs. Try PSDRIVER /V for complete

installation info or use one (ore more) of the following switches (do

not add spaces between switch and parameter):

/P<printer-port>  Specify the desired printer port (1, 2, or 3

are valid printer port IDs) for the trans-

lated output.

/S<page-size>     Specify one of the following digits:

1 - Letter size

2 - Legal size

3 - A4 size (default)

/O<page-orient>   Specify

1   for portrait orientation

2   for landscape orientation

/L<left-margin>   Specify left margin in characters;

current character density will be used

/M<code>          Specify Multiplex interrupt 2Fh code that

will be used by PSDRIVER command line to

retrieve data from installed PSDRIVER

(default EFh). Use a two character hex value.

Specify a value of ? to allow the PSDRIVER

command line to search MUX codes between

DBh and FFh.  (See troubleshooting)

Supported Printer Control Codes

-------------------------------

PSDRIVER supports a basic subset of the EPSON FX printer control code set.

This allows you to change font sizes and styles, line density, page length,

etc. This is a complete list of ESCAPE sequences and control characters

supported by PSDRIVER:

BS      08h     Backspace (go back one character cell)

HT      09h     Tab (fixed tabstops every 8th character cell)

LF      0ah     Line Feed (advance to next line)

FF      0ch     Form Feed (eject page)

CR      0dh     Carriage Return (go back to start of line)

SI      0fh     Set compressed character pitch (17cpi)

DC2     12h     Reset compressed character pitch

ESC SI          Set compressed character pitch (17cpi)

ESC 0           Set line density: 8 lpi

ESC 2           Set line density: 6 lpi

ESC 4           Set italic font style

ESC 5           Reset italic font style

ESC E           Set bold font style

ESC F           Reset bold font style

ESC M           Set Elite character pitch (12cpi)

ESC P           Set Pica character pitch (10cpi)

ESC T           Set Super-/Subscript off

ESC - <0>       Set Underline off

ESC - <1>       Set Underline on

ESC S <0>       Set subscript on

ESC S <1>       Set superscript on

ESC 3 <v>       Set line density to <v>/180 inch

ESC A <v>       Set line density to <v>/60 inch

ESC C <v>       Set page length to <v> lines of current line density

ESC l <v>       Set left margin in current character density

Troubleshooting

---------------

Some users reported problems with the command line interface of

PSDRIVER: Calling PSDRIVER /v from DOS displays just garbage, not the

true driver state. I think, interferences between PSDRIVER and other

installed programs (i.e. the Netware shell or drivers) caused this

problems. With version 0.6, I enhanced the communication interface to

PSDRIVER.

If your PSDRIVER installation continues to malfunction under DOS,

WINDOWS or OS/2 DOS-box please try out the following hints:

*  PSDRIVER is installed during CONFIG.SYS (you see the installation

message) but the DOS command PSDRIVER /v reports that PSDRIVER isn't

installed.

May be, another resident program uses the same MUX-function as

PSDRIVER. On default, PSDRIVER answers on an interrupt call to the

multiplex interrupt 0x2f:

mov    ah, 0efh

xor    al, al

int    02fh

with the return values 'G' in register AH and 'N' in register AL. The

other resident program seems to intercept the EFh code, too and

responds in a different way.

Modify PSDRIVER's MUX-code using the /M command line option. Specify

a different code as a two character hex value. Codes between 00 and

DA should be avoided because they are reserved by DOS.

The command line version of PSDRIVER tries the EF code first. If you

have specified a different code on CONFIG.SYS command line, you have

to specify the same value on PSDRIVER's DOS interface, too. You may

use the query function of PSDRIVER's interface (specify /M?) to allow

PSDRIVEr to check all codes above DAh.

*  PSDRIVER reports garbage after issuing the PSDRIVER /v command.

It seems that a program uses the same technique as PSDRIVER for

communication. (It uses the EF00 code and returns 'GN'). In this

case try a different MUX code on installation and command line

interface.

History

-------

0.1   (Sep 09, 1993)    First release; bug free but with limited

character set and without control code support

0.2   (Oct 20, 1993)    Removed code overhead, added control code support

0.3   (Jan 16, 1994)    Reduced font size to allow printing of 80

characters on one line, enhanced subset of

recognized characters; first version with

.EXE layout

0.4   (Feb 20, 1994)    Modified device driver interface, support of

different page sizes and orientations; first

implementation of DOS command line interface.

0.5   (Apr 20, 1994)    Fixed bug: No longer resetting line density

on start of a new page

Fixed bug: Evaluation of multiple command

line arguments

Fixed bug: ESC-E starts bold printing and

ESC-F stops it (thanks to Edward Skvorc).

Added Feature: /L (set left margin)

0.6   (Jun 05, 1994)    Modified communication interface between

installed PSDRIVER and command line interface

Added switch: /M (set MUX code);

Added left margin escape code

Known Problems

--------------

The last page of output is ejected only, if the application closes the

printer port on termination of the print job. If the application doesn't

close the print device, the last page is lost forever... A programmable

timer for PSDRIVER will fix this problem. The next release of PSDRIVER

will have this feature.

Future Enhancements

-------------------

The next release will add a user programmable interface to PSDRIVER.

A windowed configuration and installation program for PSDRIVER is under

development.

The Author

----------

I'm network administrator of a Banyan Vines WAN in a german mobile phone

company (Mannesmann Mobilfunk). I developed PSDRIVER to meet my personal

requirements: printing source codes, E-Mail and ASCII reports. PSDRIVER

is my first assembler program (and my first device driver implementation,

too).

Please feel free to contact me if you have questions, bug reports or

suggestions.

Send mail to

Johannes Geenen

Schwannstr. 21

D-41460 Neuss

Germany

or contact me on Compuserve (ID 100137,3124).